3GPP TSG-RAN WG1 Meeting #103-e	Tdoc R1-2009090

e-Meeting, October 26th – November 13th 2020

Agenda Item:	8.15.3

Source:	Ericsson

Title:	On evaluation assumptions for eMTC and NB-IoT based NTN

Document for:	Discussion

# 1	Introduction

In RAN#86, a Rel-17 Study Item (SI) on IoT NTN [1] was approved to evaluate the feasibility of NTN for NB-IoT and eMTC. It was agreed to use the existing work on NR NTN captured in TR 38.821 [2] as a baseline. The first objective of this study is as follows.

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

In this contribution, we discuss evaluation assumptions to support evaluations needed to verify the feasibility of NTN for eMTC and NB-IoT.

# 2	Evaluations for IoT NTN

## 2.1	Evaluation targets

eMTC and NB-IoT were carefully studied at the initial design stage to ensure that they could meet the various design targets, see 3GPP TR 36.888 [3] and TR 45.820 [4]. Later, in Release 15, eMTC/NB-IoT performance was further evaluated to show that they fulfill the 3GPP massive MTC [5] and the IMT-2020 5G performance requirements [6][7].

To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for NB-IoT and eMTC in the new context of NTN, while accounting for factors such as the additional complexity, cost, latency and power consumption associated with GNSS operation.

Additionally, the study item description clearly indicates that device types with PC3 and PC5 are in the scope and thus it will be necessary to evaluate both device types.

To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for eMTC and NB-IoT in the new context of NTN, taking into account factors such as the additional complexity,  cost, and power consumption associated with GNSS operation.

In Rel-17 IOT NTN SI, evaluate eMTC and NB-IoT in the context of NTN at least for the following targets: (1) coverage performance through link budget analysis; (2) supported device density; (3) complexity and cost of equipping eMTC/NB-IoT devices with NTN capability; (4) power consumption performance of eMTC/NB-IoT devices with NTN connectivity; and (5) latency performance of eMTC/NB-IoT devices in NTN systems.

## 2.2	Evaluation methodology and preliminary evaluation results

In this section, we discuss evaluation methodology and present preliminary evaluation results for the coverage performance.

### 2.2.1	Coverage performance

A link budget analysis is needed to evaluate the coverage performance of IoT NTN. The maximum coupling loss (MCL) of the channels has traditionally been used as a metric for coverage performance evaluation.

During the initial design stage of eMTC/NB-IoT [3][4] as well as for 3GPP mMTC [5] and IMT-2020 5G performance requirements [6][7], the MCL of the coverage-limiting channel was used as a metric for coverage performance evaluation.

As agreed in the WID, the existing work [2] should be used as a baseline. Therefore, we suggest using the simulation scenarios considered in [2] for evaluating IoT NTN link budgets.

In Rel-16 NR NTN SI [2], two sets (Set 1 and Set 2) of system-level simulation scenarios were agreed and Set 1 was prioritized.

As the scope of IoT NTN SI is limited to sub 6 GHz frequency bands, only the simulation scenarios in [2] that consider the S-band (i.e., 2 GHz nominal frequency) are relevant.

In Rel-17 IOT NTN SI, the MCL should be provided for eMTC and NB-IoT based on the simulation scenarios considered in [2] and the relevant Set 1 simulation scenarios should be prioritized.

Both NB-IoT and eMTC support different granularities for uplink (UL) frequency resource allocation. For instance, NB-IoT supports an UL allocation of 1-tone (15 kHz), 3-tones (45 kHz), 6-tones (90 kHz) or 12-tones (180 kHz). Similarly, eMTC supports an UL allocation of 2-out-of-3-tones, 3-tones (45 kHz), 6-tones (90 kHz) or 12-tones (180 kHz). Moreover, the smaller the allocation of resources in the frequency-domain, the longer the transmission duration in the time-domain. Therefore, we need to determine which of the legacy time-frequency allocations NB-IoT/eMTC can support in different NTN scenarios.

The time-frequency UL resource allocations for NB-IoT/eMTC that are feasible in NTN scenarios should be determined based on link budget calculations.

In Appendix A1.	Link budgets, we provide the preliminary link budget results for both LEO and GEO satellites for certain relevant Set 1 simulation scenarios.

### 2.2.2	Connection density evaluation

In [5], the connection density was defined as the number of devices per unit area that fulfil a target quality of service (QoS) requirement to ensure a packet drop rate of 1% for a certain packet size and packet arrival rate. For IMT-2020 connection density requirement for massive IoT devices [6], a target connection density of 1 million devices per km2 was to be supported for the specified set of scenarios. In these scenarios, the inter-site distances of 500 m and 1732 m were considered.

The cell sizes in NTN are significantly larger than those considered while evaluating eMTC/NB-IoT connection density requirement in terrestrial cellular networks.

In [10], a projected device density for IoT NTN based on the population data in England was presented. For a 40 km NTN cell diameter, it was estimated that around 500,000 devices within the satellite coverage area of 1257 km will need to be supported in rural areas. This translates to a target device density of less than 400 devices per km2. For dense urban areas, the target device density was estimated to be around 26,000 devices per km2. The average target device density was estimated to be around 6600 devices per km2.

There is a stark difference between the projected device density range of 400 to 26000 devices per km2 and the device density of 100,0000 devices per km2 considered for connection density requirement.

Revisit the target connection density requirement as well as the connection density evaluation assumptions for IoT NTN.

### 2.2.3	Complexity evaluation

A low device complexity was one of the key design targets for NB-IoT and eMTC devices. For this reason, the device complexity for various proposals was evaluated in [4] in terms of silicon area estimate (including on-chip memory), relative area of RF and baseband functions, list of external components, relative cost, etc. GNSS-equipped operation, however, was not envisioned while designing NB-IoT/eMTC. Therefore, the complexity and cost of IoT NTN devices need to be estimated to account for GNSS.

GNSS-assisted operation will increase the complexity, cost and power consumption of eMTC and NB-IoT devices.

Evaluate the impact of GNSS-assisted operation on NB-IoT and eMTC device complexity.

### 2.2.4	Energy consumption evaluation

Based on energy consumption evaluation, NB-IoT/eMTC have been shown to have a battery life of at least 10 years with a 5 Wh battery under certain assumptions about the device activity pattern. Specifically, in [3], battery life was evaluated for a mobile-originated data transfer of 200 bytes UL data followed by 20 bytes DL data per day for a MCL of 164 dB. As IoT NTN devices are expected to support GNSS operation, the device battery life calculation needs to be revisited.

GNSS-assisted operation is expected to increase the power consumption of IoT NTN devices.

Evaluate the battery life supported by eMTC and NB-IoT devices while accounting for the increased power consumption due to GNSS-assisted operation in IoT NTN.

### 2.2.5	Latency evaluation

The latency evaluation for IoT NTN needs to account for additional delays due to large link distances as well as GNSS operation.

The large propagation delay in NTN as well as GNSS-assisted operation is expected to significantly increase the supported latency for eMTC/NB-IoT NTN.

To minimize the effort, we propose adopting the latency evaluation methodology in [5][3] for massive IoT scenario. The transmission latency was measured from the time a device in its most battery-efficient state decides to send an uplink application layer packet until the time the packet is ready at the base-station to be dispatched to the core network. It included the time for downlink synchronization and random access. It was compared against a requirement of 10 s assuming a packet size of 105 bytes defined at the physical layer (including MAC, PDCP and RRC overheads).

Use the evaluation methodology considered in [5] for evaluating latency performance of eMTC and NB-IoT for NTN.

To begin the discussion, the latency evaluation formula for IoT NTN can be expressed as

T=TGNSS+Tsync+TRA+Tdata+Tmisc

where TGNSS is the time needed to acquire timing/frequency compensation information using GNSS, Tsync is the time needed to acquire DL synchronization, TRA is the time needed to perform random access, Tdata is the time needed to transmit data on the uplink and Tmisc models other miscellaneous time delays. For the preliminary evaluation, we propose following the assumptions in Table 1.

Table 1 Preliminary assumptions for latency evaluation for IoT NTN.

|  | Delay variable | Comments | Initial evaluation assumptions |
| --- | --- | --- | --- |
| 1 | TGNSS | This will vary depending on cold, warm or hot start. | Evaluate latency for each of these options. |
| 2 | Tsync | This includes time for acquiring (N)PSS, (N)SSS, and (N)PBCH. | Target 1% BLER for synchronization channels and 10% BLER for other channels. |
| 3 | TRA | This includes time for random access. | Use 4-step RACH. |
| 4 | Tdata | This counts the time starting from the instant when UL data is transmitted until it is received successfully at the eNB. | Needed only for RRC resume. Not needed for early data transmission (EDT). |
| 5 | Tmisc | To account for other delays such as wait times between transmission and reception. | If RTT is not explicitly included in 2-4, then it can be accounted for here. |
| 6 | Coupling loss | The latency calculation will vary depending on the coupling loss. | Use maximum coupling loss (MCL). |

Evaluate the latency performance of eMTC/NB-IoT while accounting for the large propagation delay, GNSS operation and NTN-specific link budget.

# Conclusion

In the previous sections we made the following observations:

Observation 1	To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for eMTC and NB-IoT in the new context of NTN, taking into account factors such as the additional complexity,  cost, and power consumption associated with GNSS operation.

Observation 2	During the initial design stage of eMTC/NB-IoT [3][4] as well as for 3GPP mMTC [5] and IMT-2020 5G performance requirements [6][7], the MCL of the coverage-limiting channel was used as a metric for coverage performance evaluation.

Observation 3	In Rel-16 NR NTN SI [2], two sets (Set 1 and Set 2) of system-level simulation scenarios were agreed and Set 1 was prioritized.

Observation 4	As the scope of IoT NTN SI is limited to sub 6 GHz frequency bands, only the simulation scenarios in [2] that consider the S-band (i.e., 2 GHz nominal frequency) are relevant.

Observation 5	The cell sizes in NTN are significantly larger than those considered while evaluating eMTC/NB-IoT connection density requirement in terrestrial cellular networks.

Observation 6	There is a stark difference between the projected device density range of 400 to 26000 devices per km2 and the device density of 100,0000 devices per km2 considered for connection density requirement.

Observation 7	GNSS-assisted operation will increase the complexity, cost and power consumption of eMTC and NB-IoT devices.

Observation 8	GNSS-assisted operation is expected to increase the power consumption of IoT NTN devices.

Observation 9	The large propagation delay in NTN as well as GNSS-assisted operation is expected to significantly increase the supported latency for eMTC/NB-IoT NTN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In Rel-17 IOT NTN SI, evaluate eMTC and NB-IoT in the context of NTN at least for the following targets: (1) coverage performance through link budget analysis; (2) supported device density; (3) complexity and cost of equipping eMTC/NB-IoT devices with NTN capability; (4) power consumption performance of eMTC/NB-IoT devices with NTN connectivity; and (5) latency performance of eMTC/NB-IoT devices in NTN systems.

Proposal 2	In Rel-17 IOT NTN SI, the MCL should be provided for eMTC and NB-IoT based on the simulation scenarios considered in [2] and the relevant Set 1 simulation scenarios should be prioritized.

Proposal 3	The time-frequency UL resource allocations for NB-IoT/eMTC that are feasible in NTN scenarios should be determined based on link budget calculations.

Proposal 4	Revisit the target connection density requirement as well as the connection density evaluation assumptions for IoT NTN.

Proposal 5	Evaluate the impact of GNSS-assisted operation on NB-IoT and eMTC device complexity.

Proposal 6	Evaluate the battery life supported by eMTC and NB-IoT devices while accounting for the increased power consumption due to GNSS-assisted operation in IoT NTN.

Proposal 7	Use the evaluation methodology considered in [5] for evaluating latency performance of eMTC and NB-IoT for NTN.

Proposal 8	Evaluate the latency performance of eMTC/NB-IoT while accounting for the large propagation delay, GNSS operation and NTN-specific link budget.

# References

RP-193235, Study on NB-IoT/eMTC support for Non-terrestrial Network, MediaTek Inc., RAN#86, Dec 2019.

TR 38.821, Solutions for NR to support Non-terrestrial Networks (NTN), 3GPP, V16.0.0, Jan 2016.

TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, 3GPP, V12.0.0, June 2013.

TR 45.820, Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT), 3GPP, V13.1.0, November 2015.

3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies,” version 16.0.0, July 2020.

ITU-R, Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, October 2017.

ITU-R, Report ITU-R M.2411-0 “Requirements, evaluation criteria and submission template for the development of IMT-2020”, ITU-R.

A. Hoglund, J. Bergman, X. Lin, O. Liberg, A. Ratilainen, H. S. Razaghi, T. Tirronen, E. Yavuz, "Overview of 3GPP Release 14 Further Enhanced MTC," IEEE Communications Standards Magazine, vol. 2, no. 2, pp. 84-89, June 2018.

Y.P.E. Wang, X. Lin, A. Adhikary, A. Grovlen, Y. Sui, Y. Blankenship, J. Bergman, and H.S. Razaghi, "A Primer on 3GPP Narrowband Internet of Things," IEEE Communications Magazine, vol. 55, no. 3, pp. 117-123, March 2017.

R2-1901404, IoT Device Density Models for Various Environments, Vodafone, RAN2#105, Athens 2019.

# Appendix

## A1.	Link budgets

We provide the link budgets for the parameters in Table 2.

Table 2 Parameters for link budget calculation.

| Simulation scenario | Set 1 |
| --- | --- |
| Frequency band | S-band |
| Carrier frequency | 2 GHz |
| Frequency reuse | 1 |

Table 3 Link budget for 600 km LEO satellite for a 30° elevation angle.

| System | NB-IoT |  | eMTC |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TX: EIRP/spot [dBm] | 56.6 | 23.0 | 64.3 | 23.0 |
| RX: G/T [dB/T] | -31.6 | 1.1 | -31.6 | 1.1 |
| Bandwidth [Hz] | 1.80E+05 | 1.80E+05 | 1.08E+06 | 1.80E+05 |
| Free space path loss (PL) [dB] | 159.1 | 159.1 | 159.1 | 159.1 |
| Atmospheric loss (LA) | 0.1 | 0.1 | 0.1 | 0.1 |
| Shadow fading margin (SF) [dB] | 3 | 3 | 3 | 3 |
| Scintillation loss (SL) [dB] | 2.2 | 2.2 | 2.2 | 2.2 |
| Polarization loss [dB] | 0 | 0 | 0 | 0 |
| Additional losses (AD) [dB] | 0 | 0 | 0 | 0 |
| Target SNR [dB] | 6.6 | 5.8 | 6.6 | 5.8 |

Table 4 Link budget for 1200 km LEO satellite for a 30° elevation angle.

| System | NB-IoT |  | eMTC |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TX: EIRP/spot [dBm] | 62.6 | 23.0 | 70.3 | 23.0 |
| RX: G/T [dB/T] | -31.6 | 1.1 | -31.6 | 1.1 |
| Bandwidth [Hz] | 1.80E+05 | 1.80E+05 | 1.08E+06 | 1.80E+05 |
| Free space path loss (PL) [dB] | 164.5 | 164.5 | 164.5 | 164.5 |
| Atmospheric loss (LA) | 0.1 | 0.1 | 0.1 | 0.1 |
| Shadow fading margin (SF) [dB] | 3 | 3 | 3 | 3 |
| Scintillation loss (SL) [dB] | 2.2 | 2.2 | 2.2 | 2.2 |
| Polarization loss [dB] | 0 | 0 | 0 | 0 |
| Additional losses (AD) [dB] | 0 | 0 | 0 | 0 |
| Target SNR [dB] | 7.2 | 0.4 | 7.2 | 0.4 |

Table 5 Link budget for GEO satellite for an elevation angle of 10°.

| System | NB-IoT |  | eMTC |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TX: EIRP/spot [dBm] | 81.6 | 23.0 | 89.3 | 23.0 |
| RX: G/T [dB/T] | -31.6 | 19.0 | -31.6 | 19.0 |
| Bandwidth [Hz] | 180000.0 | 180000.0 | 1080000.0 | 180000.0 |
| Free space path loss (PL) [dB] | 190.63 | 190.63 | 190.63 | 190.63 |
| Atmospheric loss (LA) | 0.190 | 0.190 | 0.190 | 0.190 |
| Shadow fading margin (SF) [dB] | 3 | 3 | 3 | 3 |
| Scintillation loss (SL) [dB] | 2.2 | 2.2 | 2.2 | 2.2 |
| Polarization loss [dB] | 0 | 0 | 0 | 0 |
| Additional losses (AD) [dB] | 0 | 0 | 0 | 0 |
| Target SNR [dB] | -0.04 | -7.97 | -0.04 | -7.97 |