3GPP TSG RAN WG1 #103-e                                                                                            R1-2009128

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.11.2.1

Source:	Robert Bosch GmbH

Title:	Sidelink Resource Allocation to Reduce Power Consumption

Document for:	Discussion

# Introduction

In RAN#86 meetings, a new Rel-17 WI for sidelink enhancements was approved with two main motivations, power saving and enhanced reliability/reduced latency. The objectives related to power saving of this WID were further adapted in RAN#88e in order to meet the shifted timeline due to current pandemic, where the following is stated:

In Rel-17 WI sidelink enhancements, the following objectives are approved [1]:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

This contribution provides high-level discussions and motivation for resource allocation enhancement considering power saving for sidelink. More specifically, we are considering the automotive community concerns regarding sidelink power saving enhancements that can be achieved in Rel-17.

# Resource allocation enhancement

In NR-V2X Rel-16, several enhancements are introduced including advanced control channel design, physical layer structure, physical layer procedure including HARQ/CSI feedbacks, and multiple resource allocation enhancements for both periodic and aperiodic traffic. However, Rel-16 has been developed without power saving in mind as V2X is the main focus of this release. This also means that some use cases that requires power saving or operation of battery limited UEs may not efficiently perform with Rel-16. Therefore, power saving solutions for sidelink, including resource allocation enhancements (RAN1 and RAN2) and DRX (RAN2), are in the focus of Rel-17. In the following sub-sections we will discuss some potential issues that need to be considered in this WI.

## Multiple UE capabilities for sidelink power saving

In the justification of this WID [1], it was stated that this work should strive for specifying solutions for power saving in Rel-17 for, at least, vulnerable road users (VRUs) in V2X use cases where power consumption in the UEs needs to be minimized. At least for VRU use cases, there could be different equipped VRU devices with different power constraints and requirements [2]. For example, an equipped walking pedestrian with a limited battery UE is different from an equipped electrical-lightweight mobility devices with a relatively higher battery capacity, e.g., e-scooter, e-bike, e-motorcycle, etc.. Furthermore, these e- lightweight mobility devices may have higher interaction –for a longer time- with the vehicular traffic in busy roads than that of walking pedestrians. Thereby, participation in more advanced VRU use cases is expected from these devices.

Let us consider the e-bike as a specific example to clarify what different UE capability we are expecting. For instance, if an e-bike wants to conduct sidelink VRU communication, there could be at least two options:

- Option 1: the e-bike (itself) is equipped with an NR-sidelink capable communication module, or

- Option 2: the bike rider is equipped with a smart phone (e.g., connected to the e-bike), where the smartphone is capable of VRU sidelink communication and not the e-bike.

In Option 1, it is clear that the sidelink communication is used only for the e-bike mobility use cases. In Option 2, the smart phone should be able to send and receive VRU messages. If a smart phone is connected to an e-bike controller and e-bike charger (if it exist), then the same smart phone is expected to experience different mobility use cases than when it is only used by a walking pedestrian. In this case, the same UE should be able to switch between, at least, two different capabilities based on its status (e.g., connected to an e-bike or not).

Figure 1: A vehicle that communicates with two vulnerable road users with different power constraints

Observation 1: For VRU sidelink communication, it is expected that multiple equipped VRU devices may have different power constrains, requirements, and capabilities

Proposal 1: Rel-17 sidelink power saving design should consider different power capabilities for different VRU UEs or for the same VRU UE

## Power saving using limited sensing

The first objective in in Rel-17 sidelink enhancements WID opts for specifying solutions to enable battery-driven UEs to perform sidelink operations with a reduced power consumption. The solutions fit many use cases including battery driven V2X use cases (e.g., parking vehicles and vulnerable road user (VRU) like pedestrian UEs, e-Bikes, etc.), commercial use cases, and public safety. In LTE and NR sidelink communication, long term sensing may be a main reason to drain UEs batteries. Therefore, our main focus here is to limit sensing capabilities in these cases.

### Transmission using Random selection

In this WID [1], it is approved to consider LTE Rel-14 sidelink random selection and partial sensing solution as baseline for power saving; however, without precluding introducing new solutions to reduce power consumption for sensing.  In LTE Rel-14, both random selection and partial sensing are configurable, for the same or different resource pool. For random selection to reduce interference, a UE can be requested to perform periodic transmission for the randomly selected resources. In this case, this can be easily detected by the partially/long-term sensing nearby UEs, i.e., reducing interference. Therefore, if the number of pedestrian UEs are limited and their traffic patterns are rather periodic, the performance can be acceptable.

However, if the number of pedestrian UEs, performing random selection, increases, interference among the vulnerable users cannot be easily mitigated, which degrades the performance. Additionally, aperiodic traffic is also encouraged in NR sidelink, which may limit the performance of the randomly selecting UEs if they are not able to decode HARQ feedbacks or SCI reservations, i.e., to perform re-evaluation and pre-emption.

Observation 2: For LTE sidelink random selection, the traffic is assumed to be mainly periodic

Proposal 2: Enhance LTE aperiodic and periodic random selection for power limited UEs by allowing at least re-evaluation, preemption and HARQ feedback

### Partial sensing and DRX

In LTE sidelink, partial sensing was introduced to allow UEs to sense only limited number of subframes within the sensing window. It assumed that power saving UEs (e.g., pedestrian UE (P-UE)) are only performing sensing periodically and on a sub-sensing window. The periodicity P may be selected based on the packet delay budget (PDB) and latency requirement. The width of the sub-sensing window may be configured; however, the location is set based on each UE on its own. This mechanism was designed having in mind a TX-only (P-UE) communicating with an always active UEs (vehicles).

Figure 2: Partial sensing in LTE for P-UEs

As observed, LTE partial sensing may be sufficient for selecting candidate resources for a TX-only VRU/P-UEs. Therefore, the DRX definition was not required to save power for P-UE in LTE. However, NR sidelink VRU UEs will need to perform sidelink PSCCH and PSSCH reception as well.  In order to save power in this case, DRX mechanism needs to be exploited either in-coverage or in out-of-coverage.

In this WI, the DRX definitions and its wake-up time alignments should be considered by RAN2 only. However, we also see an impact on RAN1 as, at least, the DRX ON/OFF durations (needed for RX) need to be adapted to the partial sensing definition in RAN1 (which is needed for TX candidate resource selection), or vice versa. In other words, a UE should conduct the sensing and RX decoding according to its DRX configuration.

Therefore, we propose considering a joint DRX and partial sensing definition at least for the TX/RX capable VRU. Hence, the DRX cycle adaption may also involve partial sub-sensing capability.

Observation 3: For sidelink power saving, partial sensing mechanism may only fit TX-only UEs

Proposal 3: Consider a unified partial-sensing and DRX definition, where a UE preforms sensing only during the DRX active timers

### Partial sensing modification

#### Partial sensing for aperiodic transmission

Since the traffic assumption in LTE was assumed to be mainly periodic and sensing window has one limited size, this assumption used to work. However, in NR-V2X, two different window sizes are configured, a short one of 100ms, e.g., for aperiodic traffic, and a long one of 1000+100ms, e.g., for periodic traffics. Therefore, partial sensing need to be adapted in this case, allowing also for aperiodic transmission or mixed (aperiodic and periodic) transmissions. Additionally, NR sidelink VRU (power saving UEs) should also consider NR sidelink enhancements to have a better reception capability for aperiodic transmission, e.g., by exploiting HARQ feedback, CSI feedback, flexible SCI reservation, re-evaluation capability, and pre-emption,.

Observation 4: In LTE sidelink partial sensing, UEs are assumed to be TX-UEs and have periodic traffic

Proposal 4: Enhance LTE partial selection to allow aperiodic traffic and utilize Rel-16 enhancements including feedback decoding, re-evaluation, and pre-emption

#### Adaptive partial sensing and RX activity

In NR sidelink, a power saving UE may be requested to perform advanced VRU communication (either TX or RX). Hence, these UEs may need to extend it sub-sensing window and/or its reception activity timers. In this case, the length of the sub-sensing window (i.e., also DRX On Duration) and the inactive sensing period (i.e., also DRX OFF Duration) can be adaptively selected, e.g., between a maximum and a minimum configured number of slots.

Adaptation may be triggered by some measurements within a certain evaluation phase/period. These measurements could be, e.g., the VRU transmission rate, received scheduling assignments (SA), channel busy ratio (CBR) of the shared resources, amount of available resources, etc. For example, if the UE needs to select fewer candidate resource where there is more resources available, the VRU can reduce its sub-sensing window/DRX_on time. Another example, if a UE is approaching a spot where the channel is more congested (with a high CBR value) and the SAs have high RSRP values. In this case, the UE may be approaching a dense vehicular activity and it may need to expand its sub-sensing window/DRX_on time for better reception conditions.

Figure 3 depicts an example where a UE may select a sub-sensing window (size & time-location) for either a long sensing/evaluation period (for periodic traffic with a sub-sensing period) or a short evaluation period (for aperiodic traffic). The UE may extend or reduce the sub-sensing window based on the aforementioned parameters. Newly selected sub-sensing window(s) may partially overlap with the old ones to guarantee enough history for resource (re)selection. The UE may also select one or more sub-sensing windows for periodic and/or one-shot transmission.

Figure 3: Possible adaptive wake-up durations (e.g., N1 ≠ N2≠ Ni) and sleep times

Observation 5: NR sidelink VRU UE needs to transmit and receive different VRU services with different traffic patterns/requirements

Proposal 5: Rel-17 NR sidelink should study an adaptive partial sensing/DRX for periodic and aperiodic transmission based on channel and traffic conditions

## Flexible frequency search space within a sidelink BWP

One obvious issues in Rel-16 sidelink (which is also extended to Rel-17), is having a fixed bandwidth part (BWP) per each carrier, which cannot be flexibly adapted for sidelink power saving . Additionally, during Rel-16 discussions, an idea for segregating resource pools based on QoS/priorities was not concluded as it may fragment the resources [3]. Therefore, also segregating resource pools based on VRU and V2X services may not be easily considered. Moreover, we also understand that (at least for some use-cases) VRUs need to share the same resources with V2X devices, e.g., to receive advanced V2X messages (cooperative awareness messages (CAM), cooperative perception messages (CPM), etc.) [2].

For Rel-17 sidelink enhancements, we propose to have a flexibly configured frequency sidelink search space within a fixed BWP (in a carrier); at least a minimum and a maximum frequency search spaces could be configured. The maximum search space can be the maximum configured BWP subchannels or the maximum suchannels of the configured resource pools. Wherein, the minimum search space can be configured as a sidelink common search space (SCSS) in frequency domain. This SCSS may allow easier communication among all VRU classes and vehicles. The minimum frequency sidelink search space can be configured as, e.g., multiple adjacent subchannels, which should be common along all overlapping and non-overlapping configured resource pools.

Based on the VRU UE required services and its power capability, the UE may adapt its search space between these two configured values. For example, the UE may have a limited search space in one instant and may extend it if it figures out, e.g., high congestions or decodes more SA with relatively high RSRP. In this case, the UE may conduct it is approaching a spot with vehicular traffic that may need attention, i.e., more receiving capability.

Once the UE selects an optimized frequency search space and an optimized partial-sensing/DRX ON duration (as described in Section 2.2.3), the UE will have only a limited PSCCH/PSSCH decoding occasions. This will limit the power needed for monitoring the whole time and frequency search space.

Observation 6: NR sidelink does not have an adaptive sidelink BWP and does not configure a way to limit the frequency search space

Proposal 6: we propose to have an adaptive frequency search space based on the channel or the VRU traffic conditions

## Coexistence with Rel-16 UEs

In this WI, it is assumed that Rel-16 and Rel-17 UEs can coexist in the same resource pool. Additionally, it was agreed to minimize physical layer impacts to allow both forward and backward compatibility. Therefore, it is important to agree that Rel-17 power saving enhancements can preserve this need. Moreover, in automotive requirements, a Rel-17 VRU needs to decode messages (e.g., at least basic safety messages (from Rel-16 UEs, e.g., vehicles). Therefore, in our power saving design may need to minimize physical layer impacts to easily achieve such a goal for Rel-16 and Rel-17 co-existence.

Additionally, as described above, multiple VRUs with multiple capabilities need to coexist together with V2X communication despite their selected sensing approach and/or their DRX activity. Coexistence here means a mutual communication between all devices, i.e., even between some VRU as defined in some scenarios in [2].

Proposal 7: Enhanced sidelink power saving design needs to minimize physical layer impacts to allow better coexistence between Rel-16 and Rel-17 UEs and between Rel-17 with different power saving capabilities.

## Considering Rel-17 Enhancements

Rel-17 Enhancements requires additionally DRX adaptation and aligning wake-up time among communicating, but power saving, UEs. Even though DRX should be handled in RAN2, in our view it is also important to design a sidelink power saving resource allocation mechanism that aligns DRX cycles to allow better communication between UEs with different DRX cycles.

In other agenda item, resource allocation enhancements exploiting inter-UE coordination is studied. However, in our view, if it will be agreeable to specify inter-UE coordination, it is important to consider a design that utilize such a coordination to allow coordinating active and inactive times between UEs. For example, a UE with a high power capacity may assist other UEs to adapt their partial sensing/DRX time and frequency search space .

Proposal 8: Enhanced sidelink power saving utilizing inter-UE coordination for aligning DRX timers alignment and frequency search spaces

# Conclusions

In this contribution the following observation have been made:

Observation 1: For VRU sidelink communication, it is expected that multiple equipped VRU devices may have different power constrains, requirements, and capabilities

Observation 2: For LTE sidelink random selection, the traffic is assumed to be mainly periodic

Observation 3: For sidelink power saving, partial sensing mechanism may only fit TX-only UEs

Observation 4: In LTE sidelink partial sensing, UEs are assumed to be TX-UEs and have periodic traffic

Observation 5: NR sidelink VRU UE needs to transmit and receive different VRU services with different traffic patterns/requirements

Observation 6: NR sidelink does not have an adaptive sidelink BWP and does not configure a way to limit the frequency search space

In this contribution the following proposals have been made:

Proposal 1: Rel-17 sidelink power saving design should consider different power capabilities for different VRU UEs or for the same VRU UE

Proposal 2: Enhance LTE aperiodic and periodic random selection for power limited UEs by allowing at least re-evaluation, preemption and HARQ feedback

Proposal 3: Consider a unified partial-sensing and DRX definition, where a UE preforms sensing only during the DRX active timers

Proposal 4: Enhance LTE partial selection to allow aperiodic traffic and utilize Rel-16 enhancements including feedback decoding, re-evaluation, and pre-emption

Proposal 5: Rel-17 NR sidelink should study an adaptive partial sensing/DRX for periodic and aperiodic transmission based on channel and traffic conditions

Proposal 6: we propose to have an adaptive frequency search space based on the channel or the VRU traffic conditions

Proposal 7: Enhanced sidelink power saving design needs to minimize physical layer impacts to allow better coexistence between Rel-16 and Rel-17 UEs and between Rel-17 with different power saving capabilities.

Proposal 8: Enhanced sidelink power saving utilizing inter-UE coordination for aligning DRX timers alignment and frequency search spaces

# References

- RP-201385, “NR Sidelink enhancement”, Work Item Description, Electronic Meeting, June 29 - July 3, 2020.

- ETSI TS 103 300 V2.1.1 (2020-05) Intelligent Transport System (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2

- R1-2004664, “Summary#2 for AI 7.2.4.6 QoS management for sidelink,”	Moderator (Nokia), RAN WG1 #101-e, May 25th – June 5th, 2020