3GPP TSG RAN WG1 #103-e	R1-2009129

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Enhancements on Multi-beam Operation

Agenda Item:	8.1.1

Document for:	Discussion and Decision

# Background

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed [1] and the revised WID was agreed at RAN#89-e [2]. In the WID, the following objectives were agreed for multi-beam enhancement.

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In RAN1#102-e meeting, some agreements were made for SRS enhancement. In this contribution, we share our views on multi-beam enhancement.

# Discussions

### TCI for DL and UL

In the last meeting, the following agreements were made for joint TCI for DL and UL.

- [Issue 1] For Rel.17 NR FeMIMO, on the unified TCI framework - Support joint TCI for DL and UL based on and analogous to Rel.15/16 DL TCI framework - The term “TCI” at least comprises a TCI state that includes at least one source RS to provide a reference (UE assumption) for determining QCL and/or spatial filter - The source reference signal(s) in M TCIs provide common QCL information at least for UE-dedicated reception on PDSCH and all or subset of CORESETs in a CC - FFS: Optionally this common QCL information can also apply to CSI-RS resource for CSI, CSI-RS resource for BM, and CSI-RS for tracking - FFS: Applicability on PDSCH includes PDSCH default beam - Working Assumption: Select between M=1 and M>=1 - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC, - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions - FFS: applicability of this UL TX spatial filter to SRS configured for beam management (BM) - FFS: PUSCH port determination based on the TCI, e.g., to be mapped with SRS ports analogous to Rel.15/16 - Working Assumption: Select between N=1 and N>=1 - FFS: extension to common QCL information applied to only some of the CORESETs or PUCCH resources in a CC, e.g. for mTRP - FFS: When used for the purpose of joint beam indication for UL and DL, whether a joint TCI pool for DL and UL dedicated for the purpose is used, or the same TCI pool as that used for the purpose of separate DL/UL beam indication is used - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - FFS (RAN1#103-e): The supported number of active TCI states considering factors such as multi-TRP and issue 6 - FFS (RAN1#103-e): Applicable QCL types, and co-existence with DL TCI and spatial relation indication in Rel.15/16 - In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives for accommodating the case of separate beam indication for UL and DL - Alt1. Utilize the joint TCI to include references for both DL and UL beams - Alt2. Utilize two separate TCI states, one for DL and one for UL. The TCI state for the DL is the same as agreed in 1a. The TCI state for the UL can be newly introduced. - Alt 2-1: The UL TCI state is taken from the same pool of TCI states as the DL TCI state - Alt 2-2: The UL TCI state is taken from another pool of TCI states than the DL TCI state - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - Note: This may be related to issue 5 as well as other reasons for different TCIs such as network flexibility/scheduling

Regarding the number of TCIs for joint TCI, for single active beam operation, M = 1 and N = 1 are enough but to support multiple active beam operation, we slightly prefer M > 1 and N > 1.

Proposal 1: For DL TCI and UL TCI, RAN1 should take M > 1 and N > 1 for DL channels and UL channels respectively.

In the last meeting, some companies have proposed alignment of default beam for PDSCH and PDCCH. In the current specification, the default PDSCH beam is determined by a TCI state for the lowest CORESET ID in the latest slot on the active DL BWP of the CC. On the other hand, for PUSCH, PUCCH, and SRS, a TCI state for the lowest CORESET ID on the active DL BWP of the CC. Therefore, we should align the default beam before the discussion of applicability of the default beam.

Proposal 2: Align the default beam for physical channels and signals to facilitate common beam operation.

### Dynamic TCI update

The following agreements were made for SRS capacity enhancement.

- [Issue 3] For Rel.17 NR FeMIMO, on dynamic TCI state update signaling medium: - In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives: - Alt1. DCI - Alt2. MAC CE - Note: Combination between DCI and MAC CE for, e.g. different use cases or control information partitioning can also be considered - Note: The study should consider factors such as feasibility for pertinent use cases, performance (based on at least the agreed EVM), overhead (including PDCCH capacity), latency, flexibility, reliability including the support of retransmission - Note: This may be related to outcome of issue 1a), 1b), and 6a) - In RAN1#103-e, depending on the outcome of 3a), identify candidates for more detailed design issues for the dynamic TCI state update such as - Exact content - Signaling format - Reliability aspects including the support of retransmission - Extensions, including the support of UE-group (in contrast to UE-dedicated) signaling

In our view, the signaling of beam switch should be short as much as possible. The current specifications, the activation time is long and this issue has been pointed out by some companies in the last meeting. Therefore, we prefer Alt. 1

Proposal 3: Dynamic indication by DCI is preferred

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: For DL TCI and UL TCI, RAN1 should take M > 1 and N > 1 for DL channels and UL channels respectively.

Proposal 2: Align the default beam for physical channels and signals to facilitate common beam operation.

Proposal 3: Dynamic indication by DCI is preferred

# References

- RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN#86, December 2019

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, RAN#89-e, September 2020.

- R1-2005842, Ericsson, “Enhancements to multi-beam operation”, RAN1#102-e, August 2020.

- R1-2006951, NTT DOCOMO, “Discussion on multi-beam operation”, RAN1#102-e, August 2020.