3GPP TSG-RAN WG1 Meeting #103-e	R1-2009251

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.1.2.1

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

Document for:	Discussion/Decision

# Introduction

The Rel-17 WID for further enhancements on MIMO (FeMIMO) is approved [1], which includes the following objective:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

In Rel. 16 mTRP enhancements, the focus was mostly on PDSCH reliability enhancements while reliability for PDCCH, PUCCH, and PUSCH was not taken into account explicitly due to lack of time. In order to ensure overall reliability of the system, it is important to ensure that both data and control for both downlink and uplink are reliable. In this contribution, we discuss the aspects related to reliability and robustness of PDCCH, PUCCH, and PUSCH separately.

# PDCCH

In RAN1 #102e, the following were agreed related to PDCCH enhancements:

Agreement

To enable a PDCCH transmission with two TCI states, study pros and cons of the following alternatives:

- Alt 1: One CORESET with two active TCI states

- Alt 2: One SS set associated with two different CORESETs

- Alt 3: Two SS sets associated with corresponding CORESETs

- At least the following aspects can be considered: multiplexing schemes (TDM / FDM/ SFN / combined schemes), BD/CCE limits, overbooking, CCE-REG mapping, PDCCH candidate CCEs (i.e. hashing function), CORESET / SS set configurations, and other procedural impacts.

Agreement

For non-SFN based mTRP PDCCH reliability enhancements, study the following options:

- Option 1 (no repetition): One encoding / rate matching for a PDCCH with two TCI states

- Option 2 (repetition): Encoding / rate matching is based on one repetition, and the same coded bits are repeated for the other repetition. Each repetition has the same number of CCEs and coded bits, and corresponds to the same DCI payload.

  - Study both intra-slot repetition and inter-slot repetition

- Option 3 (multi-chance): Separate DCIs that schedule the same PDSCH /PUSCH /RS/TB/etc. or result in the same outcome.

  - Study both cases of DCIs in the same slot and DCIs in different slots

Note 1: Companies are encouraged to evaluate the different options based on agreed LLS assumptions for possible down-selection in RAN1#103-e.

Note 2: The actual encoding / rate matching chain for PDCCH polar coding (i.e. 38.212 Sections 5.3.1 / 5.4.1 / 7.3.3 / 7.3.4) is not changed in the options above.

Agreement

For mTRP PDCCH reliability enhancements, study the following multiplexing schemes

- TDM : Two sets of symbols of the transmitted PDCCH / two non-overlapping (in time) transmitted PDCCH repetitions / non-overlapping (in time) multi-chance transmitted PDCCH are associated with different TCI states

  - Aspects and specification impacts related to intra-slot vs inter-slot to be discussed

- FDM : Two sets of REG bundles / CCEs of the transmitted PDCCH / two non-overlapping (in frequency) transmitted PDCCH repetitions / non-overlapping (in frequency) multi-chance transmitted PDCCH are associated with different TCI states

- SFN : PDCCH DMRS is associated with two TCI states in all REGs/CCEs of the PDCCH

  - Note: There is dependency between this scheme and AI 2d (HST-SFN )

- Note: Combinations of the schemes are not precluded, and they can be discussed at a later stage.

Agreement

For Alt 1 (one CORESET with two active TCI states), study the following

- Alt 1-1: One PDCCH candidate (in a given SS set) is associated with both TCI states of the CORESET.

- Alt 1-2: Two sets of PDCCH candidates (in a given SS set) are associated with the two TCI states of the CORESET, respectively

- Alt 1-3: Two sets of PDCCH candidates are associated with two corresponding SS sets, where both SS sets are associated with the CORESET and each SS set is associated with only one TCI state of the CORESET

- Note 1: A set of PDCCH candidates contain a single or multiple PDCCH candidates, and a PDCCH candidate in a set corresponds to a repetition or chance

- Note 2: How one or more PDCCH candidates are counted for monitoring (for BD limit) is FFS

  - The note is applicable also to other alternatives

Agreement

For Alt 1-2/1-3/2/3, study the following

- Case 1: Two (or more) PDCCH candidates are explicitly linked together (UE knows the linking before decoding)

  - FFS: How the explicit linkage is derived/determined by the UE

- Case 2: Two (or more) PDCCH candidates are not explicitly linked together (UE does not know the linking before decoding)

  - FFS: How the UE knows the linkage after decoding

Figure 1 provides illustrative examples for the five different alternatives (Alts 1-1, 1-2, 1-3, 2, and 3) described in the agreements above.

Figure 1: Illustration of different alternatives.

Figure 2 shows different options (Options 1, 2, and 3) defined in the agreement above.

Figure 2: Illustrations of different options.

In the reminder of this section, we discuss the following:

- Performance evaluation of different options.

- Valid combinations of different options, alternatives, and cases.

- Specification impact analysis.

- Overall pros and cons.

- FDM and TDM for Alt 1-1 and Option 1.

## Performance Evaluations of Different Options

In this section, we compare the performance of the three options above (Options 1, 2, and 3) under various scenarios. In the simulations, FDM scheme is assumed but the observations are general and applicable also to the case of TDM scheme. Simulation parameters are summarized in Table 3, which follow the agreement on EVM for PDCCH.

We compare the performance of single-TRP PDCCH, multi-TRP PDCCH assuming Option 1 (no repetition), multi-TRP PDCCH assuming Option 2 (repetition), and multi-TRP PDCCH assuming Option 3 (multi-chance) under the following scenarios for AL=2 (2 CCEs) and AL=8 (8 CCEs):

- PL delta=0dB and no blockage (Figure 3).

- PL delta=6dB and no blockage (Figure 4).

- Blockage with p=0.1 and x=10dB (Figure 5).

For multi-TRP schemes (for Options 1-3), FDM scheme is used, and even REG bundles are transmitted from the first TRP and odd REG bundles are transmitted from the second TRP. For mTRP Option 2 and Option 3, AL=2 consists of two AL=1, and AL=8 consists of two AL=4 for fair comparison, and UE soft combines (chase combining) the two repetitions before decoding. The performance comparisons are shown in Figure 3, Figure 4, and Figure 5. Additional simulation results are provided in Section 8 (Appendix II).

Figure 3: PL delta=0dB and no blockage for AL=2 and AL=8.

Figure 4: PL delta=6dB and no blockage for AL=2 and AL=8.

Figure 5: Blockage with p=0.1 and x=10dB for AL=2 and AL=8.

As it can be observed from the Figures above, no repetition (Option 1) provides better performance compared with repetition (Option 2) when there is no blockage and when coding rate is not very low (i.e., for AL=2). This is due to the better coding gain in Option 1 compared to Option 2. Even in the case of blockage when AL=2, Option 1 is slightly outperforming Option 2. In other scenarios, including in the case of blockage and when code rate is low (i.e., AL=8), the performance is the same for both Option 1 and Option 2.

Observation 1: Option 1 (no repetition) performs better than Option 2 (repetition) when coding rate is high. Otherwise (when there is blockage and/or coding rate is low), the performance of both Option 1 and Option 2 is similar.

Also, it is worth noting that Option 1 is more flexible as the number of coded bits transmitted with different TCI states do not need to be the same whereas in Option 2 same aggregation level should be used for the two repetitions. While not shown in the plots above, this can be beneficial in some cases, e.g., when PL delta is not negligible but is known by the network.

Observation 2: Option 1 (no repetition) is more flexible than Option 2 (repetition) in terms of number of coded bits transmitted from each TRP.

With respect to multi-chance (Option 3), as it can be seen from the Figures above, it not only clearly underperforms compared to Option 1 and Option 2 in most scenarios, but also its performance is worse than sTRP in some cases, e.g., when PL delta=6dB and AL=2. For the case of AL=8 and blockage, it performs similarly to Option 1 and Option 2. It is important to ensure that the scheme used for PDCCH is robust in various different scenarios and not just in a particular narrow situation. Hence, Option 3 should not be further considered as it does not allow a UE to perform soft-combining and results in waste of resources in most scenarios.

Observation 3: Option 3 (multi-chance) underperforms compared to Option 1 (no repetition) and Option 2 (repetition) in most scenarios, and even underperforms compared to single-TRP in some cases. The performance of Option 3 is only reasonable in the case of large AL and blockage.

Proposal 1: Option 3 (multi-chance) should not be considered further.

Based on the above simulation results and observations, and also considering the specification impact discussed in detail in Section ‎2.3, we propose:

Proposal 2: Support Option 1 (no repetition) for PDCCH reliability enhancements.

## Valid Combinations of Different Options, Alternatives, and Cases

Considering different alternatives, options, and cases agreed in RAN1 #102e, we first need to identify which combinations make sense from specification support point of view. Note that in Alt 1-1, a PDCCH candidate is self-contained whereas Alts 1-2, 1-3, 2, and 3 requires at least two PDCCH candidates. Hence, Option 1 (no repetition) can be achieved with Alt 1-1 while Option 2 (repetition) and Option 3 (multi-chance) can be achieved with Alts 1-2, 1-3, 2, and 3. This is because each PDCCH candidate can contain one repetition (in Option 2) or one DCI (in Option 3).

With respect to Case 1 (explicit linking) vs Case 2 (no explicit linking), it is clear that Case 1 is required for Option 2 (repetition), or otherwise, there will be no difference between Option 2 and Option 3 from UE point of view. Hence, when considering the combinations of different Alts, Options, and Cases, we can limit it to the following possibilities:

Observation 4: The following combinations can be considered:

- Combination 1: Alt 1-1 + Option 1

- Combination 2: Other Alts + Option 2 + Case 1

- Combination 3: Other Alts + Option 3 + Case 1

- Combination 4: Other Alts + Option 3 + Case 2

Next, we analyse specification impact as well as pros and cons for each of the four combinations above.

## Specification Impact Analysis

Possible specification impacts can be summarized as below. For each specification impact, it is also mentioned that to which combination (or which Alt/Option/Case) it applies. At the end of this section, specification impacts are summarized in Table 1.

Issues related to configuration / association of TCI states and/or PDCCH candidates:

- Issue A1: Configuring / activating two TCI states for a CORESET (e.g. MAC-CE to indicate two active TCI states for a CORESET).

  - Applicable to Alt 1-1, 1-2, 1-3.

- Issue A2: Associating a REG bundle with a TCI state

  - Applicable to Combination 1 with FDM

- Issue A3: Associating a symbol of a CORESET with a TCI state

  - Applicable to Combination 1 with TDM

- Issue A4: Associating a PDCCH candidate or a SS set with one TCI state of the CORESET (out of two active TCI states)

  - Applicable to Alt 1-2 and 1-3.

- Issue A5: Configuring one SS set with 2 CORESETs

  - Applicable to Alt 2.

- Issue A6: Linking two or more PDCCH candidates (by configuration or by a rule)

  - Applicable to Case 1, i.e., Combinations 2 and 3

Issues related to PDCCH procedures:

- Issue B1: CCE-REG mapping to ensure that each PDCCH candidate includes REG bundles from both TCI states. See more details in Section ‎2.5.

  - Applicable to Combination 1 with FDM

- Issue B2: Precoding granularity is defined within the symbols with the same TCI state. See more details in Section ‎2.5.

  - Applicable to Combination 1 with TDM

- Issue B3: BD limit: For the case that UE supports soft-combining, additional blind decoding is required for counting toward the limit

  - Applicable to Combination 2

- Issue B4: Overbooking: When additional BD limits are counted, the order of considering PDCCH candidate for monitoring for overbooking requires change when a BD is defined in two different SS sets

  - Applicable to Combination 2 when Alt 1-3 or Alt 3 is used (two SS sets are used)

- Issue B5: Hashing function changes: Defining CCEs for PDCCH candidates in one SS set that is associated with 2 CORESETs

  - Applicable to Alt 2

- Issue B6: Default QCL assumption: When lowest CORESET ID that the UE monitors in the latest slot is a CORESET with 2 active TCI state, which one should be assumed for QCL assumption of a PDSCH when scheduling offset is less than timeDurationForQCL.

  - Applicable to Alt 1-1, 1-2, 1-3

The following issues are related to scheduling info / restriction procedures when two different PDCCH candidates are utilized. The issues arise due to the fact that gNB is not aware of which PDCCH candidates are decoded by the UE while in current specification the corresponding scheduling info / restriction is a function of time or frequency property of the detected DCI:

- Issue C1: Rate matching PDSCH around scheduling DCI resources, i.e., to avoid ambiguity, UE needs to rate match around both PDCCH candidates irrespective of which one is detected:

  - Applicable to Combination 2 and 3 (for intra-slot).

  - Not easily solvable for Combination 4 resulting in scheduling restriction (scheduling DCI should not overlap with PDSCH resources). This is because PDCCH candidates are not linked.

Figure 6: Illustration of Issue C1.

- Issue C2: Implicit PUCCH resource determination for the first PUCCH resource set, where start CCE an number of CCEs in the CORESET play a role (in addition to PRI) in PUCCH resource determination, i.e., how does the UE determine  and  in the following formula in 38.213:

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in scheduling restriction (two PDCCH candidates can be used only if they both result in the same PUCCH resource determination based on the above formula). This is because PDCCH candidates are not linked.

- Issue C3: Earliest time a PDSCH with mapping type B can start relative to the scheduling DCI, i.e., how the following restriction in 38.214 should be modified to avoid ambiguity in case UE does not detect one of the PDCCH candidates

The UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation.

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in scheduling restriction (PDSCH cannot start early). This is because PDCCH candidates are not linked.

Figure 7: Illustration for Issue C3.

- Issue C4: Reference for SLIV when “ReferenceofSLIV-ForDCIFormat1_2” is configured and K0=0, i.e., how the following rule in 38.214 should be modified to avoid ambiguity in case UE does not detect one of the PDCCH candidates

if configured with ReferenceofSLIV-ForDCIFormat1_2, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected;

  - Applicable to Combination 2.

    - Note that it is potentially ok for Combination 3 and 4 since a different TDRA row can be indicated in the second PDCCH candidate (since DCI content can be different in Option 3).

- Issue C5: Determination of scheduling offset for “timeDurationForQCL”, i.e., UE and gNB should be on the same page with respect to when default beam is used even when one of the PDCCH candidates is not detected at the UE

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in scheduling restriction (PDSCH cannot be start in certain duration to avoid the ambiguity). This is because PDCCH candidates are not linked.

Figure 8: Illustration for Issue C5.

- Issue C6: K0 / K1 indication in TDRA field of the DCI when different PDCCH candidates are in different slots, i.e., how to determine the reference for counting K0 / K1 slots for unambiguous determination irrespective of which PDCCH candidate is detected

  - Applicable to Combination 2 (for inter-slot)

    - Note that it is potentially ok for Combination 3 and 4 since a different TDRA row can be indicated in the second PDCCH candidate (since DCI content can be different in Option 3).

- Issue C7: Interruption time for BWP switching: How to unambiguously determine the start of the interruption time for BWP switching irrespective of which PDCCH candidate is detected by the UE, i.e., how the following rule in 38.213 should be modified.

the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

  - Applicable to Combination 2.

- Issue C8: Out-of-order / in-order definition: Certain rules are needed as to how the out-of-order scheduling is defined when a PDCCH consists of multiple PDCCH candidates scheduling the same PDSCH / PUSCH while UE may detect all or some of the PDCCH candidates. This is illustrated in Figure 9 in which the first DCI scheduling PDSCH1 / PUSCH1 consists of two PDCCH candidates of which only the first one is detected, and the second DCI scheduling PDSCH2 / PUSCH2 consists of two PDCCH candidates of which only the second one is detected. The question is then whether this should treated as out-of-order from UE’s perspective.

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in scheduling restriction (worst case assumption by gNB). This is because PDCCH candidates are not linked.

Figure 9: Illustration for Issue C8.

- Issue C9: CORESETPoolIndex related issues: When two PDCCH candidates are in different CORESETs associated with different CORESETPoolIndex values, a rule is required to determine one unique CORESETPoolIndex value (e.g. for HARQ-Ack, PDSCH scrambling, CRS rate matching, etc.) irrespective of  which PDCCH candidate is actually detected by the UE

  - Applicable to Combination 2 and 3 (with Alt2 or Alt3, i.e., when the two PDCCH candidates are in different CORESETs).

  - Not easily solvable for Combination 4 resulting in scheduling restriction (for multi-DCI based mTRP). This is because PDCCH candidates are not linked.

- Issue C10: drx-InactivityTimer for DRX: UE and gNB should be on the same page as to when to start the drx-InactivityTimer irrespective of which PDCCH candidate is detected by the UE (note that in Rel. 15/16, UE starts the drx-InactivityTimer in the first symbol after a detected DCI that schedules a new transmission).

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in mismatch between UE and gNB. This is because PDCCH candidates are not linked.

Figure 10: Illustration of issue C10.

- Issue C11: SFI indication in DCI format 2_0: How to uniquely determine which set of slots the SFI -index field value is applied to irrespective of which PDCCH candidate is detected, i.e., how the following rule in 38.213 should be modified:

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0.

  - Applicable to Combination 2 (for inter-slot)

    - Note that it is potentially ok for Combination 3 and 4 since a different SFI index can be indicated in the second PDCCH candidate (since DCI content can be different in Option 3).

- Issue C12: Search space set group switching by DCI format 2_0: How to uniquely determine when to switch the SS set group irrespective of which PDCCH candidate is detected (note that in Rel. 16, UE switches the SS set group at a first slot that is at least Pswitch symbols after the last symbol of the detected DCI).

  - Applicable to Combination 2 and 3.

  - Not easily solvable for Combination 4 resulting in mismatch between UE and gNB. This is because PDCCH candidates are not linked.

- Issue C13: Interrupted transmission indication by DCI format 2_1: How to unambiguously determine the set of symbols that the interrupted transmission indication is applied to irrespective of which PDCCH candidate is detected.

  - Applicable to Combination 2

    - Note that it is potentially ok for Combination 3 and 4 since a different field value can be indicated for interruption transmission indication in the second PDCCH candidate (since DCI content can be different in Option 3).

Figure 11: Illustration for Issue C13.

- Issue C14: Uplink cancelation indication by DCI format 2_4: How to unambiguously determine the set of symbols that the uplink cancelation indication is applied to irrespective of which PDCCH candidate is detected, i.e., how the following rule should be modified:

the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming …

  - Applicable to Combination 2

    - Note that it is potentially ok for Combination 3 and 4 since a different field value can be indicated for UL transmission cancelation indication in the second PDCCH candidate (since DCI content can be different in Option 3).

- Issue C15: TPC command in DCI formats 2_2 and 2_3: UE needs to apply the TPC command one time even if more than one PDCCH candidate is detected.

  - Applicable to Combination 4: E.g. UE needs to respect only one TPC commands in a slot (for intra-slot), or UE needs to ignore one of the two TPC commands in consecutive slots if two TPC commands are detected in two respective DCIs (for inter-slot)

    - Note that it is potentially ok for Combinations 2 and 3, as the two PDCCH candidates are linked, and UE is aware of the fact that the detected DCI coveys the same information.

- Issue C16: DAI operation for DL DCI for Type-2 HARQ-Ack codebook: In Rel. 15, cDAI is defined as the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion.

  - For Combination 2, a modification of DAI definition is required since the two PDCCH candidates may not belong to the same PDCCH monitoring occasion while the DAI value should be the same. For example, PDCCH monitoring occasion corresponding to the first candidate should be used as reference for DAI definition. This is illustrated in the left-hand side of Figure 12.

  - For Combination 4, up to two positions in the HARQ-Ack codebook is required for the same scheduled PDSCH since the two PDCCH candidates are not linked, and a the two DCIs can indicate different cDAI/tDAI values depending on their respective PDCCH monitoring occasions and other DCIs sent by the network (e.g. in other CCs). This is illustrated in the right-hand side of Figure 12.

  - For combination 3: Either the first change (mentioned for Combination 2) or the second change (mentioned for Combination 4) above is required.

Figure 12: Illustration for Issue C16.

Table 1: Specification impact summary.

| Spec impact? | Combination 1: Alt 1-1 + Option 1 | Combination 2: Other Alts + Option 2 + Case 1 | Combination 3: Other Alts + Option 3 + Case 1 | Combination 4: Other Alts + Option 3 + Case 2 |
| --- | --- | --- | --- | --- |
| Issue A1 | Yes | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 |
| Issue A2 | Yes (for FDM) | No | No | No |
| Issue A3 | Yes (for TDM) | No | No | No |
| Issue A4 | No | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 |
| Issue A5 | No | Yes: Alt 2 No: Alts 1-2 / 2 / 3 | Yes: Alt 2 No: Alts 1-2 / 2 / 3 | Yes: Alt 2 No: Alts 1-2 / 2 / 3 |
| Issue A6 | No | Yes | Yes | No |
| Issue B1 | Yes (for FDM) | No | No | No |
| Issue B2 | Yes (for TDM) | No | No | No |
| Issue B3 | No | Yes | No | No |
| Issue B4 | No | Yes: Alt 1-3 / 3 No: Alts 1-2 / 2 | No | No |
| Issue B5 | No | Yes: Alt 2 No: Alts 1-2 / 2 / 3 | Yes: Alt 2 No: Alts 1-2 / 2 / 3 | Yes: Alt 2 No: Alts 1-2 / 2 / 3 |
| Issue B6 | Yes | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 | Yes: Alts 1-2 / 1-3 No: Alts 2 / 3 |
| Issue C1 | No | Yes | Yes | Not easily solvable |
| Issue C2 | No | Yes | Yes | Not easily solvable |
| Issue C3 | No | Yes | Yes | Not easily solvable |
| Issue C4 | No | Yes | No | No |
| Issue C5 | No | Yes | Yes | Not easily solvable |
| Issue C6 | No | Yes (inter-slot) | No | No |
| Issue C7 | No | Yes | No | No |
| Issue C8 | No | Yes | Yes | Not easily solvable |
| Issue C9 | No | Yes: Alts 2 / 3 No: Alts 1-2 / 1-3 | Yes: Alts 2 / 3 No: Alts 1-2 / 1-3 | Not easily solvable: Alts 2 / 3 No: Alts 1-2 / 1-3 |
| Issue C10 | No | Yes | Yes | Not easily solvable |
| Issue C11 | No | Yes (inter-slot) | No | No |
| Issue C12 | No | Yes | Yes | Not easily solvable |
| Issue C13 | No | Yes | No | No |
| Issue C14 | No | Yes | No | No |
| Issue C15 | No | No | No | Yes |
| Issue C16 | No | Yes | Yes | Yes |

Based on the discussions in this section, we can observe that

Observation 5: In terms of specification impact, for the issues discussed above

- Combination 1 (Alt 1-1 + Option 1) has considerably smaller specification impact compared to other combinations.

- Combination 2 (Other Alts + Option 3 + Case 1) and Combination 3 (Other Alts + Option 3 + Case 1) have significant specification impact. Compared to Combination 2, Combination 3 has slightly smaller specification impact.

  - For Combination 2 and 3, specification impact is slightly smaller when Alt 3 is used compared to Alt 1-2, 1-3, and 2.

- Certain issues are not easily solvable for Combination 4 (Other Alts + Option 3 + Case 2) resulting in major scheduling limitations and/or mismatch between UE and gNB.

## Overall Pros and Cons

In this section, we consider different aspects discussed in Sections ‎2.1, ‎2.2, and ‎2.3, and provide an overall comparison in Table 2. With respect to UE implementation, it is evident that Combination 1 has the smallest UE complexity as decoding of two PDCCH candidates is not required and separate RE mapping for decoding one DCI is not needed. On the other hand, Combination 2 has the largest impact on UE complexity as separate decoding, soft combining, and separate RE mapping is required. Compared to Combination 2, Combinations 3 and 4 do not require soft combining, but they still require separate RE mapping for the two PDCCH candidates. In addition, Combination 3 and Combination 2 require UE to take into account PDCCH candidate pairing / linking, and interpret the decoded DCI in certain ways based on the pairing as discussed in Section ‎2.3 in details.

Table 2: Overall Pros / Cons of different Combinations

|  | Performance | Specification Impact | UE complexity |
| --- | --- | --- | --- |
| Combination 1: Alt 1-1 + Option 1 | Good | Substantially smaller specification impact compared to other combinations. In addition, it can easily support SFN case if needed. | Small |
| Combination 2: Other Alts + Option 2 + Case 1 | Good: Similar to Combination 1 in most cases, but slightly underperforms when coding rate is high | Significant specification impact. Among the alternatives, Alt 3 results in smaller specification impact. | Largest |
| Combination 3: Other Alts + Option 3 + Case 1 | Not good: Significant under-performance when coding rate is high or there is no blockage. Only acceptable when coding rate is small and there is blockage | Large specification impact (slightly smaller compared to Combination 2). Among the alternatives, Alt 3 results in smaller specification impact. | Large |
| Combination 4: Other Alts + Option 3 + Case 2 | Not good: Significant under-performance when coding rate is high or there is no blockage. Only acceptable when coding rate is small and there is blockage | Certain issues are not easily solvable (since PDCCH candidates are not linked) resulting in significant scheduling restrictions. | Medium |

Based on the discussions above, we propose:

Proposal 3: Support Combination 1 (Alt 1-1 + Option 1).

## FDM and TDM for Alt 1-1 and Option 1

In this section, we discuss more details related to Issues B1 and B2 (as well as Issues A2 and A3) that were briefly mentioned in Section  ‎2.3 for Combination 1 (Alt 1-1 + Option 1) in the case of FDM and TDM multiplexing schemes.

For FDM scheme, each REG bundle of the CORESET should be associated with one TCI state out of the two TCI states of the CORESET. The association can be based on a fixed rule or can be based on RRC configuration. Following similar principle as PDSCH FDM scheme in Rel. 16, a simple fixed rule can be based on either a first half/second half splitting rule (appropriate for the case of “precoderGranularity=allContiguousRBs”), or based on even/odd REG bundles splitting rule (appropriate for the case of “precoderGranularity=sameAsREG-bundle”). This is illustrated in Figure 13.

Figure 13: Association of REG bundles with TCI states for FDM scheme.

Proposal 4: For FDM scheme in Alt 1-1, study how to associate each REG bundle with a TCI state, including first/second half or even/odd REG bundle splitting rules.

Once the association is determined, especially if the association is not explicitly configured by RRC, separate interleaving is required for REG bundles to ensure that each PDCCH candidate has REG bundles from both TCI states. The interleaving function itself follows Rel. 15 formula but it is done separately for the REG bundles associated with the first TCI state and for the REG bundles associated with the second TCI state, and CCE j consists of REG bundles from the first set or REG bundles from the second set.

Proposal 5: For CCE-to-REG mapping in FDM scheme in Alt 1-1, the REG bundles associated with different TCI states are separately interleaved using the Rel. 15 interleaving formula, and each CCE j is mapped to REG bundles from the first set or REG bundles from the second set.

For TDM scheme, each symbol of the CORESET should be associated with one TCI state out of the two TCI states of the CORESET. Similar to the case of FDM discussed above, the association can be based on a fixed rule or can be based on RRC configuration. For TDM, the CORESET duration should be obviously two or three symbols. When CORESET has two symbols, the association is uniquely determined. When CORESET has three symbols, a simple rule, e.g., first two symbols associated with first TCI state and the last symbol associated with the second TCI state, should be sufficient.

In addition, for the case of TDM, precoding granularity should be defined within the symbols with the same TCI state as mentioned in Section  ‎2.3. This is due to the fact that REG bundles are defined first in time and second in frequency. In fact, there is no need to change the definition of REGs, REG bundles, and CCE-to-REG mapping. Instead, the precoding assumption should be clarified for the TDM scheme for both cases of “precoderGranularity=allContiguousRBs” and “precoderGranularity=sameAsREG-bundle”. This is illustrated in Figure 14.

Figure 14: Precoding assumption clarification for TDM scheme.

Proposal 6: For TDM scheme in Alt 1-1, precoding granularity is defined within the symbols with the same TCI state for both cases of “precoderGranularity=allContiguousRBs” and “precoderGranularity=sameAsREG-bundle”.

# PUCCH

In this section, we discuss multi-beam PUCCH transmission for both cases of inter-slot and intra-slot. In RAN1 #102e, the following related to PUCCH enhancements were agreed:

Agreement

To improve reliability and robustness for PUCCH using multi-TRP and/or multi-panel, consider all PUCCH formats.

Agreement

To enable TDMed PUCCH transmission with different beams, support configuring/activating of multiple PUCCH Spatial Relation Info. RAN1 shall further study the exact schemes considering the following aspects,

- Method of configuration/activation of multiple spatial relation info

- Use of the same PUCCH resource or different PUCCH resource for PUCCH transmission

- Mapping between PUCCH repetition/symbol and spatial relation info among multiple PUCCH repetitions / multiple PUCCH symbols.

Agreement

For configuration/indication of the number of PUCCH repetitions, RAN1 shall further study the following,

- Alt.1: Use Rel-15 like framework

- Alt.2: Dynamic indication of the number of PUCCH repetitions

Agreement

For multi-TRP PUCCH transmission, further investigate required power control enhancement.

Agreement

Support TDMed PUCCH scheme(s) to improve reliability and robustness for PUCCH using multi-TRP and/or multi-panel. Study the following alternatives,

- Alt.1: supporting both inter-slot repetition and intra-slot repetition / intra-slot beam hopping.

- Alt.2: supporting only inter-slot repetition

- Note1: It is not precluded to study the use of multiple PUCCH resources to repeat the same UCI in both inter-slot repetition and intra-slot repetition.

- Note2: The alternatives are clarified as below,

  - inter-slot repetition: One PUCCH resource carries UCI , another one or more PUCCH resources or the same PUCCH resource in another one or more slots carries a repetition of the UCI .

  - intra-slot repetition: One PUCCH resource carries UCI , another one or more PUCCH resources or the same PUCCH resource in another one or more sub-slots carries a repetition of the UCI

  - intra-slot beam hopping: UCI is transmitted in one PUCCH resource in which different sets of symbols have different beams

Given that reliability and latency aspects are both important, and following the same principle as Rel. 16 PDSCH enhancements, our view is that both inter-slot and intra-slot PUCCH enhancements should be supported. At the same time, some of the PUCCH enhancements such as enabling 2 beams / 2 sets of power control parameters for a given PUCCH resource can be common to both cases of inter-slot and intra-slot cases.

Proposal 7: For mTRP TDMed PUCCH enhancements, support both inter-slot and intra-slot cases (Alt1).

In the next two sections, we look at the specific enhancements for inter-slot and intra-slot PUCCH cases.

## Inter-Slot Multi-Beam PUCCH

In Rel. 15, we can have inter-slot PUCCH repetition configured with “nrofSlots” as part of “PUCCH-FormatConfig” that can be defined for PUCCH formats 1, 3, or 4. However, the same uplink beam or spatial relation info is used across all nrofSlots slots. This is because the same PUCCH resource is used for repetition across slots, and PUCCH spatial relation info is configured / activated per PUCCH resource.

To enable inter-slot PUCCH repetition with different beams using Rel. 15 PUCCH repetition mechanisms, one easy way is to allow one PUCCH resource to be configured / activated with two PUCCH-SpatialRelationInfoId’s. For PUCCH, this will automatically also take care of different power control parameters including P0, PL RS and closed loop index as they are all defined within PUCCH-SpatialRelationInfoId as shown below:

Proposal 8: To enable two beams and two sets of power control parameters for inter-slot PUCCH repetition, support activating two PUCCH-SpatialRelationInfoId’s for a given PUCCH resource.

With respect to indication of number of repetitions, it is beneficial to allow more dynamic indication compared to Rel. 15 approach. In Rel. 15, both nrofSlots and interslotFrequencyHopping are configured per PUCCH format, which means that all the PUCCH resources configured with that PUCCH format have the same number of repetitions and are the same with respect to inter-slot frequency hopping being enabled/disabled. If these two are configured per PUCCH resource, dynamic indication becomes possible by exiting PRI field in the DCI. This is a simple change and enables more flexible/dynamic signaling compared to Rel. 15 PUCCH repetition.

Proposal 9: Support configuring both nrofSlots and interslotFrequencyHopping per PUCCH resource to enable more dynamic and flexible signalling.

With respect to mapping between repetitions and beams, both cyclic mapping and sequential mapping should be allowed as illustrated in Figure 15 (similar to inter-slot PDSCH repetition in Rel. 16, i.e., Scheme 4) when the same PUCCH resource is activated with two PUCCH-SpatialRelationInfoId’s. One of the cyclic mapping or sequential mapping can be configured per PUCCH resource.

Figure 15: Cyclic mapping and sequential mapping between repetitions and beams for inter-slot PUCCH repetition.

Proposal 10: Support configuring cyclic mapping or sequential mapping per PUCCH resource.

In addition, when inter-slot frequency hopping is enabled for inter-slot PUCCH repetition with two PUCCH-SpatialRelationInfoId’s, frequency hopping should be performed among the repetitions with the same beam to ensure that both beam diversity and frequency diversity are achieved. Otherwise, repetitions with a given beam may not go through both frequency hops, and frequency diversity may not be achieved. Figure 16 illustrates the case in which four PUCCH repetitions have different beams (for both cases of cyclic mapping and sequential mapping) and inter-slot frequency hopping is enabled.

Figure 16: Inter-slot frequency hopping for inter-slot PUCCH repetition with different beams.

Proposal 11: When inter-slot frequency hopping is enabled for inter-slot PUCCH repetition with two PUCCH-SpatialRelationInfoId’s, frequency hopping is performed among the repetitions with the same beam.

## Intra-Slot Multi-Beam PUCCH

For intra-slot PUCCH transmission with 2 beams, a new procedure is required as Rel. 15 does not support intra-slot PUCCH repetition. Three alternatives to enable intra-slot multi-beam PUCCH transmission are described below and illustrated in Figure 17:

- Alternative 1: Reusing intra-slot frequency hopping mechanisms to enable beam-hopping within one PUCCH resource.

- Alternative 2: PUCCH repetition in the same PUCCH resource in different sub-slots, where repetitions have different beams.

- Alternative 3: Allowing PUCCH repetition in two different non-overlapping PUCCH resources in a given slot, where the two PUCCH resources are configured / activated with different beams.

Figure 17: Different alternatives for intra-slot multi-beam PUCCH transmission.

In Alternative 1, PUCCH duration of a given PUCCH resource is divided into two parts corresponding to the two beam-hops. In this case, most of the frequency-hopping procedures in Rel. 15 can be reused including number of symbols in the first / second beam-hops, and number of DMRS symbols and locations. Note that intra-slot frequency hopping is supported for all PUCCH formats in Rel. 15. However, Rel. 15 only allows one active spatial relation info / power control parameters per one PUCCH resource. Interestingly, the enhancements required to enable beam-hopping within the same PUCCH resource for the case of intra-slot in Alternative 1 are the same as those required for inter-slot PUCCH repetition discussed in Section 3.1. That is, in both cases, it is required to activated two PUCCH-SpatialRelationInfoId’s for one PUCCH resource, which also addresses different power control parameters.

Observation 6: Activation of two PUCCH-SpatialRelationInfoId’s for one PUCCH resource discussed in Section 3.1 for the purpose of inter-slot PUCCH repetition, can also serve the purpose of beam hopping within one PUCCH resource (Alternative 1) for the case of intra-slot.

In Alternative 2, the same PUCCH resource is used in multiple sub-slots for PUCCH repetition with different beams. In Rel. 16, sub-slot based PUCCH transmission is introduced. However, the enhancements are only for the purpose of HARQ-Ack transmission. Specifically, Rel. 16 enhancements only address indication of K1 in units of sub-slots. In particular, Rel. 16 does not support sub-slot based PUCCH repetition. In addition, there is no notion of sub-slot for periodic PUCCH, e.g., for CSI transmission, as the periodicity and offset are still defined in units of slots. Compared to Alternative 1, Alternative 2 requires more specification changes since as a first step, regular (i.e. single beam) sub-slot based PUCCH repetition needs to be enabled.

In Alternative 3, two different PUCCH resources (non-overlapping) are used for intra-slot PUCCH repetition. The two PUCCH resources should have the same number of control REs in the case of PUCCH formats 2, 3, and 4 (that support more than 2 bits) so that the same coded bits are transmitted in the second PUCCH resource as a repetition of the first one. Otherwise, soft-combining cannot be done at the receiver, especially for polar code (UCI bits > 11 bits), where the same mother code length should be ensured. In addition, the specification impact for Alternative 3 is more compared with both Alternative 1 and Alternative 2. For example, PRI indication in DCI (including PUCCH resource selection within a PUCCH resource set based on PRI and payload size) may require some change as PRI should indicate two PUCCH resources. As another example, UCI multiplexing rules may need to change when one or both PUCCH resources overlap with another PUCCH transmission.

Proposal 12: For intra-slot multi-beam PUCCH enhancements, support intra-slot beam hopping (Alternative 1).

# PUSCH

In RAN1 #102e, the following were agreed related to PUSCH enhancements:

Agreement

Further study M-TRP CG PUSCH reliability enhancements in Rel-17.

Agreement

For M-TRP PUSCH reliability enhancement, support single DCI based PUSCH transmission/repetition scheme(s).

- Further study multi-DCI based PUSCH transmission/repetition scheme(s) to identify potential gains and required enhancements.

- Note: This agreement does not reflect any prioritization of single DCI based PUSCH transmission/repetition over multi-DCI based PUSCH transmission/repetition. Ran1 can further discuss that in the next meeting.

Agreement

For single DCI based M-TRP PUSCH reliability enhancement, support TDMed PUSCH repetition scheme(s) based on Rel-16 PUSCH repetition Type A and Type B.

- Further study PUSCH transmission without repetition as a potential candidate M-TRP PUSCH scheme

Agreement

To support single DCI based M-TRP PUSCH repetition scheme(s), up to two beams are supported. RAN1 shall further study the details considering,

- Codebook based and non-codebook based PUSCH

- Enhancements on SRI/TPMI/power control parameters/any other

Note1: Companies are encouraged to provide additional details on how above enhancements are applied to different PUSCH repetitions (e.g. mapping between PUSCH repetitions and beams)

Note2: Studying enhancements/aspects related to TA is not precluded.

Agreement

On the mapping between PUSCH repetitions and beams in single DCI based multi-TRP PUSCH repetition Type A and Type B, further study the following,

- For both PUSCH repetition Type A and B, how the beams are mapped to different PUSCH repetitions (or slots/frequency hops),

  - Alt.1: cyclical mapping pattern (the first and second beam are applied to the first and second PUSCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUSCH repetitions).

  - Alt.2: sequential mapping pattern (the first beam is applied to the first and second PUSCH repetitions, and the second beam is applied to the third and fourth PUSCH repetitions, and the same beam mapping pattern continues to the remaining PUSCH repetitions).

  - Alt.3: Half-Half pattern (the first beam is applied to the first half of PUSCH repetitions, and the second beam is applied to the second half of PUSCH repetitions)

  - Alt.34: Other variants (e.g. configurable mapping patterns)

  - Note1: For PUSCH repetition type B, the variants considering slot level beam mapping with the same mapping principals (replacing repetition with slot) in Alt.1/2/3 are also included.

  - Note2: For PUSCH repetition type A and B with frequency hopping, the variants considering frequency hop level beam mapping with the same mapping principals (replacing repetition with frequency hop) in Alt.1/2/3 can also be studied further. Final selection of such schemes also depends on the number of beams allowed per PUSCH repetition.

- For PUSCH repetition Type B, which repetition type that the beams shall consider for the mapping,

  - Alt.1: beams are mapped to the nominal repetitions

  - Alt.2: beams are mapped to the actual repetitions

  - Alt.3: beams are mapped to different slots (not in the granularity of actual/nominal repetition)

  - Alt.4: Other variants

- Consider additional requirements on switching gap(s) between two PUSCH repetitions towards different TRPs considering beam switching latency aspects.

- Note: use of the above solutions to multi-DCI based PUSCH repetition and TDMed PUSCH transmission without repetition (when there are agreed to support) is not precluded.

For the case of PUSCH, the focus should be on PUSCH repetition type A and PUSCH repetition Type B using the single-DCI framework. While multi-DCI based framework is also beneficial, RAN1 should first make good progress on single-DCI based approach before agreeing to support multi-DCI based approach as the required enhancements may require a reasonable amount of time. Also, PUSCH transmission without repetition should be deprioritized as there is already a framework for intra-slot PUSCH repetition in Rel. 16 (i.e., PUSCH repetition Type B), and the required enhancements are more or less only related to beam / ULPC / TPMI enhancements. Note that this is not the case of PUCCH enhancements.

In this section, we discuss the following aspects related to single-DCI based PUSCH repetitions targeted toward different TRPs

- SRI / UL beam indication

- UL power control enhancements

- TPMI indication

- Mapping between repetitions and UL beams, frequency hops and RV values.

- Link-level simulation results

## SRI / UL Beam(s) Indication

For codebook-based PUSCH transmission, SRI field of the UL DCI indicates one SRS resource from which UL beam is determined. For non-codebook-based PUSCH transmission, SRI field of the UL DCI indicates one or more SRS resources, where the number of SRS resources determine the rank of the PUSCH. Also, when associated CSI-RS is configured for the SRS resource set with usage set to “nonCodebook” (for UE to determine SRS precoding based on the associated CSI-RS), the SRS resources in the SRS resource set should not be configured with “SpatialRelationInfo”. In addition, in both cases, UE can be configured with only one SRS resource set with usage set to “codebook” or “nonCodebook”. These restrictions are not suitable for enabling multiple PUSCH repetitions with different beams. In particular, allowing for only a single SRS resource set has the following limitations:

- SRS power control parameters are configured per SRS resource set (and not per SRS resource). Hence, all SRS resources within the SRS resource set have the same power, which is not suitable for multi-TRP PUSCH enhancement as UL beam is indicated based on indication of one or more SRS resources. That is, the underlying beam for PUSCH is based on beam of corresponding SRS resources, but the SRS resources cannot have different power control parameters.

- For the case of non-codebook based PUSCH, associated CSI-RS resource is configured per SRS resource set. Hence, the precoding of SRS resources (which is used also for precoding PUSCH based on indicated SRS resource) can be only based on received signal from one TRP that transmits the associated CSI-RS resource.

Observation 7: Current restriction of only one SRS resource set with usage set to “codebook” or “nonCodebook” has the following fundamental limitations in the case of multi-TRP PUSCH enhancements

- SRS power control parameters are configured per SRS resource set (and not per SRS resource). Hence, the SRS resources cannot have different power control parameters even though the underlying beam for PUSCH is based on beam of corresponding SRS resources.

- When usage is set to “nonCodebook”, associated CSI-RS resource is configured per SRS resource set. Hence, the precoding of SRS resources and consequently the precoding of PUSCH can be only based on received signal from one TRP that transmits the associated CSI-RS resource.

In order to address the issues and limitations above, as well as to indicate two beams in the DCI, it should be allowed to configure two or more SRS resource sets, and DCI indicating up to 2 SRS resource sets as part of SRI field or a new UL DCI field. This is conceptually illustrated in Figure 18.

Figure 18: Indication of two SRS resource sets by the DCI.

To be able to dynamically switch between single-TRP and multi-TRP, it should be possible to either indicate one SRS resource set or two SRS resource sets in the DCI. The indication of SRS resources within one or two selected SRS resource sets can follow the Rel. 15 approach for both codebook-based and non-codebook-based UL transmission. Note that this approach can not only address the UL beam indication, but also is suitable for uplink power control enhancements required for PUSCH repetition with different beams as discussed in more details in Section ‎4.2.

Proposal 13: Support two or more SRS resource sets to be configured with usage set to “codebook” or “nonCodebook”, and UL DCI to indicate one or two SRS resource sets for UL beam(s) indication.

## Power Control Parameters

In Rel. 15, ULPC for PUSCH is based on SRI codepoints. In particular, a list of SRI-PUSCH mapping can be configured, each member of the list has an id (e.g., sri-PUSCH-PowerControlId: 0,…,15) as well as corresponding ULPC parameters such as PL RS, P0 and alpha, and closed loop index. Then, sri-PUSCH-PowerControlId is used as a codepoint of the SRI field in the DCI. That is, SRI codepoint value indicates sri-PUSCH-PowerControlId, and hence, the set of ULPC parameters.

For single-DCI based multi-TRP PUSCH enhancements, DCI needs to indicate two sets of ULPC parameters. If multiple SRS resource sets as discussed in Section ‎4.1 is introduced, then each “SRI-PUSCH-PowerControl” can be configured with a “sri-resource-setId” (in addition to “sri-PUSCH-PowerControlId”), as shown below:

SRI-PUSCH-PowerControl ::=          SEQUENCE {

sri-resource-setId					INTEGER {0,1,2,…}

sri-PUSCH-PowerControlId            	SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id   PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId             	P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex           	ENUMERATED { i0, i1 }

}

Then, a DCI that indicates two SRS resource sets (and includes two corresponding SRIs), points to two sets of ULPC parameters by the corresponding SRI codepoints. Note that this approach does not requires additional DCI overhead beyond the overhead that is required due to indication of two UL beams as discussed in Section ‎4.1.

Proposal 14: Support configuring each “SRI-PUSCH-PowerControl” with a “sri-resource-setId”. When DCI indicates two SRS resource sets, it points to two sets of ULPC parameters by the corresponding SRI codepoints.

Furthermore, if the two sets of ULPC parameters are associated with different closed loop index values, how to interpret the TPC command in the UL DCI or whether to include two TPC commands in the UL DCI should be discussed. In Rel. 15/16, TPC command field in UL DCI consists of two bits indicating one of the 4 possible values. In the case of two closed loop indices, one approach is to keep one TPC command and apply it to both closed loop indices. Another approach is to add another two bits for separate TPC command indication. Given that single-DCI based multi-TRP PUSCH enhancements already requires some increase in UL DCI overhead, we slightly prefer the first approach. Note that UL DCI indicating one UL beam or DCI format 2_2 (group-common TPC command for PUSCH) can anyway be used to control the two TPC of the two closed loop index separately when needed.

Proposal 15: Support one of the following options for the case that the two sets of ULPC parameters are associated with different closed loop index values:

- Option 1: The TPC command indicated in the UL DCI is applied to both closed loop index values.

- Option 2: Two TPC commands are indicated in the UL DCI.

## TPMI Indication

For codebook-based PUSCH transmission, the UL DCI field “Precoding information and number of layers” indicated both TPMI as well as number of layers. In Rel. 15/16, for both cases of PUSCH repetition type A and PUSCH repetition type B, all repetitions are transmitted with the same precoding (same TPMI) and all repetitions have the same number of layers. In our view, separate TPMIs are needed when different repetitions are targeted toward different TRPs as the channel and hence precoding matrix are different. However, different number of layers may not be needed for two reasons. First, allowing for different number of layers in different PUSCH repetitions further increases the DCI overhead (as two “Antenna ports” fields also become necessary). Second, in Rel. 16 for the case of multi-TRP PDSCH repetition (TDMSchemeA and inter-slot PDSCH repetition), all repetitions have the same number of layers, and UL should be consistent with the Rel. 16 DL design.

Proposal 16: For codebook-based PUSCH transmission, support indicating two TPMIs to be used for repetitions targeted toward different TRPs while all repetitions have the same number of layers.

## Mapping Repetitions to UL Beams, Frequency Hops, and RVs

Similar to PDSCH repetition scheme 4 introduced in Rel. 16, both cyclic mapping and sequential mapping should be supported for PUSCH repetition type A. Furthermore, for PUSCH repetition type B, the mapping between repetitions and beams should be based on nominal repetitions, and not actual repetitions. This is because in Rel. 16, UL power control procedures is based on nominal repetitions. For example, power control parameters are defined for a PUSCH transmission occasion in 38.213, where it is mentioned that a PUSCH transmission occasion is a nominal repetition. Also, to determine the BPRE value for power control, the assumption is no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B.

Proposal 17: Support RRC configuration between cyclic mapping and sequential mapping for both PUSCH repetition Type A and PUSCH repetition Type B for mapping between repetitions and two UL beams / two ULPC parameters / two TPMIs (RRC configuration between Alt1 and Alt2 in the agreement in RAN1 #102e)

- For PUSCH repetition Type B, the mapping is based on nominal repetitions (Alt 1 in the agreement in RAN1 #102e).

Furthermore, it should be discussed how to apply frequency hopping in the following cases:

- PUSCH Repetition Type A with different beams

  - Intra-slot (or intra-repetition) frequency hopping

  - Inter-slot (or inter-repetition) frequency hopping

- PUSCH Repetition Type B with different beams

  - Inter-repetition frequency hopping

  - Inter-slot frequency hopping

For the case of intra-slot (or intra-repetition) frequency hopping for PUSCH repetition Type B, no changes are required as frequency hopping is performed for each repetition irrespective of the beam. Similarly, for the case of intra-slot frequency hopping for PUSCH repetition Type B, no changes are required as frequency hopping is performed for different slots irrespective of the beam of the repetitions in a given slot. However, for the case of inter-repetition beam hopping for both PUSCH repetition Type A and PUSCH repetition Type B, it should be ensured that frequency hopping is performed among the repetitions with the same beam. Otherwise, all repetitions with a given beam might be in the same frequency hop, and hence not achieving frequency diversity for a given beam. This is illustrated in Figure 19 and Figure 20.

Figure 19: Inter-repetition frequency hopping for PUSCH repetition Type A with different beams.

Figure 20: Inter-repetition frequency hopping for PUSCH repetition Type B with different beams.

Proposal 18: For inter-repetition frequency hopping with PUSCH repetition Type A or Type B, frequency hopping is performed among the repetitions with the same beam.

Finally, for mapping repetitions to RV, an approach similar to Rel. 16 PDSCH scheme 4 can be considered. This is because if RV sequence is determined irrespective of the UL beam, the balance between coding gain versus self-decodibility cannot be controlled by the gNB. This is illustrated for PUSCH in Figure 23 in Section ‎4.5 for the case of no blockage and blockage, where for two repetitions, the RV sequence [0,2] is better in the absence of blockage and RV sequence [0,0] is better when there is blockage if coding rate is high. Hence, RVSeqOffset can be configured where RV for the repetitions with the second beam are determined by an additional shifting operation. Note that for PUSCH repetition Type B, the mapping between repetitions and RVs is based on actual repetitions.

Proposal 19: For mapping between repetitions and RV values in both cases of PUSCH repetition Type A or Type B, RV sequence is considered for the (actual) repetitions with the first beam. RVSeqOffset can be configured where RV values for the (actual) repetitions with the second beam are determined by an additional shifting operation.

## Link-Level Simulation Results

In this section, we first evaluate different receiver assumptions in the case of one transmission / one repetition. That is, UE transmits one repetition/transmission, but both TRPs receive the same repetition/transmission. Note that the simulation is for FR1. In the case of FR2, if one repetition with one beam is transmitted, the received signal may be weak at one or both TRP depending on if a wide beam or narrow beam is used. In this set-up, we consider the following 4 different receiver assumptions:

- Joint demodulation: Each TRP has 4Rx antennas, and demodulation is based on received signal on all 8Rx antennas across both TRPs.

- Soft combining: Demodulation is done separately at each TRP, but the results (e.g. LLRs) are combined before decoding. The decoding is done jointly after soft combining.

- Selection diversity: Each TRP demodulates and decodes separately. However, the TB is assumed to be correctly decoded if at least one TRP successfully decodes.

- Single-TRP: Only the first TRP attempts to decode the TB.

The simulation assumptions are summarized in Table 4. The performance is illustrated in Figure 21 and Figure 22 for MCS=4 (QPSK) and MCS=5 (16QAM) from Table 5.1.3.1-2 of 38.214, respectively, in the case no blockage and in the case of blockage. As it can be seen from the Figures, selection diversity underperforms soft combining and joint demodulation when there is no blockage. Also, joint demodulation and soft combining are the same for QPSK, but joint demodulation is slightly better for 16QAM.

Figure 21: Different receiver assumption (QPSK).

Figure 22: Different receiver assumptions (16QAM).

Observation 8: Joint demodulation and soft combining have the same performance in the case of QPSK, while joint demodulation can perform slightly better for higher modulation orders.

Next, we consider the case of 2 repetitions, where the first repetition is received only by the first TRP, and the second repetition is received only by the second TRP. The two repetitions are soft-combined before decoding. Two RV sequences [0, 0] and [0, 2] are used for each of the two cases of no blockage and blockage. As a reference, the performance of single-TRP (one repetition received only by the first TRP) is also illustrated. Figure 23 shows the performance for MCS=0 (QPSK, coding rate=0.12) and MCS=4 (QPSK, coding rate=0.59).

Figure 23: PUSCH with 2 repetitions with RV seq. [0,0] and [0,2].

As it can be observed from Figure 23, in the case of low coding rate (MCS=0), there is no difference between RV sequence [0,0] and [0,2]. However, in the case of high coding rate (MCS=4), the RV sequence [0,2] outperforms when there is no blockage, while RV sequence [0,0] outperforms when there is blockage. This is because when coding rate is high, RV sequence [0,2] achieves better coding gain in the absence of blockage, while RV=2 is not self-decodable when the first TRP is blocked in the presence of blockage.

Observation 9: Best RV sequence to apply across different repetitions targeted toward different TRPs depends on presence/absence of blockage (and the blockage level) as well as the coding rate / MCS.

# Conclusion

For PDCCH enhancements, we observe / propose:

Observation 1: Option 1 (no repetition) performs better than Option 2 (repetition) when coding rate is high. Otherwise (when there is blockage and/or coding rate is low), the performance of both Option 1 and Option 2 is similar.

Observation 2: Option 1 (no repetition) is more flexible than Option 2 (repetition) in terms of number of coded bits transmitted from each TRP.

Observation 3: Option 3 (multi-chance) underperforms compared to Option 1 (no repetition) and Option 2 (repetition) in most scenarios, and even underperforms compared to single-TRP in some cases. The performance of Option 3 is only reasonable in the case of large AL and blockage.

Observation 4: The following combinations can be considered:

- Combination 1: Alt 1-1 + Option 1

- Combination 2: Other Alts + Option 2 + Case 1

- Combination 3: Other Alts + Option 3 + Case 1

- Combination 4: Other Alts + Option 3 + Case 2

Observation 5: In terms of specification impact, for the issues discussed above

- Combination 1 (Alt 1-1 + Option 1) has considerably smaller specification impact compared to other combinations.

- Combination 2 (Other Alts + Option 3 + Case 1) and Combination 3 (Other Alts + Option 3 + Case 1) have significant specification impact. Compared to Combination 2, Combination 3 has slightly smaller specification impact.

  - For Combination 2 and 3, specification impact is slightly smaller when Alt 3 is used compared to Alt 1-2, 1-3, and 2.

- Certain issues are not easily solvable for Combination 4 (Other Alts + Option 3 + Case 2) resulting in major scheduling limitations and/or mismatch between UE and gNB.

Proposal 1: Option 3 (multi-chance) should not be considered further.

Proposal 2: Support Option 1 (no repetition) for PDCCH reliability enhancements.

Proposal 3: Support Combination 1 (Alt 1-1 + Option 1).

Proposal 4: For FDM scheme in Alt 1-1, study how to associate each REG bundle with a TCI state, including first/second half or even/odd REG bundle splitting rules.

Proposal 5: For CCE-to-REG mapping in FDM scheme in Alt 1-1, the REG bundles associated with different TCI states are separately interleaved using the Rel. 15 interleaving formula, and each CCE j is mapped to REG bundles from the first set or REG bundles from the second set.

Proposal 6: For TDM scheme in Alt 1-1, precoding granularity is defined within the symbols with the same TCI state for both cases of “precoderGranularity=allContiguousRBs” and “precoderGranularity=sameAsREG-bundle”.

For PUCCH enhancements, we observe / propose:

Observation 6: Activation of two PUCCH-SpatialRelationInfoId’s for one PUCCH resource discussed in Section 3.1 for the purpose of inter-slot PUCCH repetition, can also serve the purpose of beam hopping within one PUCCH resource (Alternative 1) for the case of intra-slot.

Proposal 7: For mTRP TDMed PUCCH enhancements, support both inter-slot and intra-slot cases (Alt1).

Proposal 8: To enable two beams and two sets of power control parameters for inter-slot PUCCH repetition, support activating two PUCCH-SpatialRelationInfoId’s for a given PUCCH resource.

Proposal 9: Support configuring both nrofSlots and interslotFrequencyHopping per PUCCH resource to enable more dynamic and flexible signalling.

Proposal 10: Support configuring cyclic mapping or sequential mapping per PUCCH resource.

Proposal 11: When inter-slot frequency hopping is enabled for inter-slot PUCCH repetition with two PUCCH-SpatialRelationInfoId’s, frequency hopping is performed among the repetitions with the same beam.

Proposal 12: For intra-slot multi-beam PUCCH enhancements, support intra-slot beam hopping (Alternative 1).

For PUSCH enhancements, we observe / propose:

Observation 7: Current restriction of only one SRS resource set with usage set to “codebook” or “nonCodebook” has the following fundamental limitations in the case of multi-TRP PUSCH enhancements

- SRS power control parameters are configured per SRS resource set (and not per SRS resource). Hence, the SRS resources cannot have different power control parameters even though the underlying beam for PUSCH is based on beam of corresponding SRS resources.

- When usage is set to “nonCodebook”, associated CSI-RS resource is configured per SRS resource set. Hence, the precoding of SRS resources and consequently the precoding of PUSCH can be only based on received signal from one TRP that transmits the associated CSI-RS resource.

Observation 8: Joint demodulation and soft combining have the same performance in the case of QPSK, while joint demodulation can perform slightly better for higher modulation orders.

Observation 9: Best RV sequence to apply across different repetitions targeted toward different TRPs depends on presence/absence of blockage (and the blockage level) as well as the coding rate / MCS.

Proposal 13: Support two or more SRS resource sets to be configured with usage set to “codebook” or “nonCodebook”, and UL DCI to indicate one or two SRS resource sets for UL beam(s) indication.

Proposal 14: Support configuring each “SRI-PUSCH-PowerControl” with a “sri-resource-setId”. When DCI indicates two SRS resource sets, it points to two sets of ULPC parameters by the corresponding SRI codepoints.

Proposal 15: Support one of the following options for the case that the two sets of ULPC parameters are associated with different closed loop index values:

- Option 1: The TPC command indicated in the UL DCI is applied to both closed loop index values.

- Option 2: Two TPC commands are indicated in the UL DCI.

Proposal 16: For codebook-based PUSCH transmission, support indicating two TPMIs to be used for repetitions targeted toward different TRPs while all repetitions have the same number of layers.

Proposal 17: Support RRC configuration between cyclic mapping and sequential mapping for both PUSCH repetition Type A and PUSCH repetition Type B for mapping between repetitions and two UL beams / two ULPC parameters / two TPMIs (RRC configuration between Alt1 and Alt2 in the agreement in RAN1 #102e)

- For PUSCH repetition Type B, the mapping is based on nominal repetitions (Alt 1 in the agreement in RAN1 #102e).

Proposal 18: For inter-repetition frequency hopping with PUSCH repetition Type A or Type B, frequency hopping is performed among the repetitions with the same beam.

Proposal 19: For mapping between repetitions and RV values in both cases of PUSCH repetition Type A or Type B, RV sequence is considered for the (actual) repetitions with the first beam. RVSeqOffset can be configured where RV values for the (actual) repetitions with the second beam are determined by an additional shifting operation.

- References

- RP-193133, New WID: Further enhancements on MIMO for NR, Samsung

- Appendix I: Simulation Assumptions

Table 3: LLS simulation assumptions and parameters for PDCCH.

| Parameter | Value |
| --- | --- |
| Number of TRPs | 2 |
| Channel model | TDL-C; 100ns RMS Delay Spread in 4GHz. |
| PL delta | {0,6} dB |
| Blockage | Blockage model from Rel-16 (x dB power offset with probability p), x and p mentioned for each plot separately when applicable. |
| Num Tx_Ant at each TRP | 4 |
| Num UE Rx_Ant | 4 |
| CCE-REG mapping | nonInterleaved, REG bundle size =6. |
| Channel estimation | Practical (MMSE) |
| number of symbols of the CORESET | 1 symbol |
| DCI payload | 40+24(CRC)=64 bits. |
| Precoding assumptions | Precoding cycling, precoder granularity=REG bundle |

Table 4: LLS simulation assumptions and parameters for PUSCH.

| Parameter | Value |
| --- | --- |
| Number of TRPs | 2 |
| Channel model | TDL-C; 100ns RMS Delay Spread in 4GHz. |
| PL delta | 0 dB |
| Blockage | Blockage model from Rel-16 (10 dB power offset with probability 0.1). |
| Num Rx_Ant at each TRP | 4 |
| Num UE Tx_Ant | 4Tx antennas with fixed open-loop precoding [1 1 1 1]. |
| Number of layers | 1 |
| Channel estimation | Practical (MMSE) |
| RB size | 256 |
| PUSCH length | 4 symbols, DMRS in the first symbol. |

- Appendix II: Additional Simulation Results

First, we look at the extreme blocking case were probability of blocking is 1, and either the first half or second half (randomly) of the REGs are blocked by 10, 20, 40 dB and compare the performance of Option 1 (no repetition) vs Option 2 (repetition) with AL=8, where baseline is AL=4 without blocking. Figure 24 shows the performance comparison for this setup. For Option 2 (repetition), AL=8 consists of two AL=4, and UE soft combines (chase combining) the two repetitions before decoding. Simulation assumptions are the same summarized in Table 3 except that REG bundle size=2 is assumed.

Figure 24: Extreme Blockage of half of REGs with AL=8 (Case 1 and Case 2) vs no blockage with AL=4.

As it can observed from Figure 24, extreme blocking of half of the REGs for AL=8 can result in 1~2dB performance loss compared with AL=4 without blocking, but there is not much performance different between “no repetition” (Case 1) vs “repetition” (Case2). This in fact shows that Polar coding is very robust against blockage as even with 40dB blockage (with P=1), performance degradation is not significant. In addition, blockage impacts both Option 1 and Option 2 roughly equally if UE does not perform separate decoding of REG bundles for a given PDCCH candidate.

In addition, the following Figures compare the performance of sTRP, mTRP Option 1 and mTRP Option 2 as additional data points for different scenarios. REG bundle size is assumed to be 2 REGs in the Figures below while other assumptions are the same as of the simulation results in Section ‎2.1, and similar observation can be seen from the Figures.

Figure 25: PL delta=0dB and no blockage for AL=2 and AL=8.

Figure 26: PL delta=6dB and no blockage for AL=2 and AL=8.

Figure 27: Blockage with p=0.1 and x=10dB for AL=2 and AL=8.