3GPP TSG RAN WG1 Meeting #103-e	R1- 2009271

Oct. 26th – Nov. 13th, 2020

Agenda item:	8.11.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation Methodology for Power Saving in Sidelink

Document for:	Discussion/Decision

# Introduction

In RANP #86, it was agreed that:

“The objective of this work item is to specify radio solutions that can enhance NR sidelink for the V2X, public safety and commercial use cases.

1. Sidelink evaluation methodology update: Define evaluation assumption and performance metric for power saving by reusing TR 36.843 and/or TR 38.840 (to be completed by RAN#88) [RAN1]

- Note: TR 37.885 is reused for the other evaluation assumption and performance metric. Vehicle dropping model B and antenna option 2 shall be a more realistic baseline for highway and urban grid scenarios.”

In this contribution, we discuss NR SL power consumption modeling; and evaluation assumptions and methodologies for NR V2X and public safety use cases.

# Sidelink Power Consumption Modelling

During RAN1 #102e, the following agreements on modelling SL power consumption were reached:

Agreements:

- For reference configuration for power consumption model,

  - 14 SL symbols in a slot (including AGC and TX-RX switching period)

  - SL sub-carrier spacing (SCS)

    - 30 kHz SCS for FR1

  - SL BWP size

    - 100 MHz for FR1

  - 2 OFDM symbols for PSCCH (excluding AGC symbol)

  - TX antenna  port (AP)

    - 1 TX AP for FR1

  - RX AP

    - 4 RX APs for FR1

  - TX power of {0 dBm, 23 dBm} for FR1

- Note that FR2 is not precluded as an optional/additional reference configuration, and companies are encouraged to provide power consumption model for FR2.

- Note that 15 kHz SCS is not precluded as an optional/additional reference configuration, and companies are encouraged to provide power consumption model for 15 kHz SCS.

Agreements:

- For evaluation, the followings are baseline

  - 2 RX APs

  - 1 TX AP

  - 40 MHz for SL BWP size

- Note that parameters or cases other than baseline is not precluded for evaluation, and companies are encouraged to provide the assumptions in details.

Agreements:

- For power consumption scaling for adaptation,

  - (Working assumption) Scaling of SL BWP size adaptation in RX perspective

  - X MHz is (0.4 +0.6*(X-20)/80)*100 MHz

  - Scaling for SL BWP size adaptation in TX perspective

  - No scaling

  - Scaling for RX AP adaptation for FR 1

  - 2 RX is 0.7*4 Rx power

- Note that scaling for adaptation on other parameters is not precluded for power consumption model, and companies are encouraged to provide the assumptions in details.

Agreements:

- For power consumption level,

  - Reuse three states of “Sleep” specified in TR38.840 including transition time/energy consumption

  - (working assumption) For “PSCCH/PSSCH RX”,

    - In non-PSFCH-slot (i.e., the number of PSCCH/PSSCH symbols is 13),

      - the power consumption level is the same as that of “PDCCH+PDSCH”

  - For power consumption level of “PSCCH/PSSCH TX”

    - In non-PSFCH-slot (i.e. the number of PSCCH/PSSCH symbols is 13),

      - the power consumption level is the same as that of “UL” for long PUCCH or PUSCH

  - For power consumption level of “1st SCI/2nd SCI RX”,

    - the power consumption level is [0.7]* power consumption level of “PSCCH/PSSCH RX”

  - For power consumption level of “PSFCH TX”,

    - the power consumption level is [0.3]*power consumption level of “UL” for long PUCCH or PUSCH

  - (Working assumption) For power consumption level of “PSFCH RX”,

    - the power consumption level is power consumption level of “PDCCH-only” for cross-slot scheduling

  - For power consumption level of “S-SSB TX” (in 13 symbol duration),

    - the power consumption level is the same as power consumption level of “UL” for (long PUCCH or PUSCH)

  - For power consumption level of “S-SSB RX”,

    - the power consumption level is [1.5]*power consumption level of “Uu SSB-processing”

  - The power consumption level of “GNSS-processing” is 8

  - When the synch reference source is gNB, reuse power consumption level of “Uu SSB processing”

  - Power consumption level of “SL-CSI-RS processing” is not separately defined

- Note that power consumption level of other Power states is not precluded, and companies are encouraged to provide the assumptions in details.

Considering the BW scaling of 0.55 and the RX antenna scaling of 0.7, the total scaling factor for Uu power consumption is 0.385. The reference powers after scaling are captured in the table below:

Table 1: A UE power consumption model for FR1 with BW = 40MHz and 2Rx antennas

| Power State | Characteristics | Relative Power | After scaling |
| --- | --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) | 1 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 50 Based on Rel. 16 power saving WI, any power consumption level below 45, corresponding to micro sleep, should be replaced by 50. |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 115.5 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) | 250 (0 dBm) 700 (23 dBm) |

Based on the abovementioned power consumption levels, the SL power consumption levels, as agreed in the last meeting, can be summarized as follows:

Table 2: Rx power consumption levels for SL with BW = 40MHz and 2Rx antennas

| Rx State | Reference power |
| --- | --- |
| Only PSCCH/PSSCH (in a non-PSFCH slot) | 115.5 |
| Only SCI-1/SCI2 | 0.7*PSCCH/PSSCH = 80.85 |
| Only PSFCH | = PDCCH only for x-slot scheduling = 27 which should be mapped to 50 |

Table 3: Tx power consumption levels for SL BW = 40MHz and 1Tx antenna

| Tx State | Reference power | Comments |
| --- | --- | --- |
| Only PSCCH/PSSCH (in a non-PSFCH slot) | 250 (0dBm) 700 (23dBm) |  |
| Only PSFCH | With interpolation: 102 (0dBm) 285 (23dBm) | In Rel. 16, a scaling factor of 0.3 was used to determine the power of short PUCCH with a single symbol. For PSFCH, 3 symbols (2+1 for AGC) should be assumed. A better approach then is to interpolate between the power needed for a 1-symbol PUCCH and 14-symbol PSCCH/PSSCH. |

Next, we present the power consumption levels for some of the Rx and Tx states which were not considered in the RAN1 agreement so far. The power consumption levels are captured in the following two tables:

Table 4: Power consumption levels for the remaining SL Rx states with BW = 40MHz and 2Rx antennas

| Rx State | Reference power | Comments |
| --- | --- | --- |
| PSCCH/PSSCH + PSFCH | 115.5 | Approximated by PSCCH/PSSCH |
| PSCCH/PSSCH only in PSFCH slot | 91.7 | 10 symbols used for reception Interpolation between the power of PSCCH/PSSCH and PSFCH only. |
| Sensing only (SCI-1) | 50 | Approximated by the power for PDCCH only with BD reduction; Assuming 10 PSCCH decoding: Max(0.28*50 + 0.72*0.7*50,50) |
| Sensing only + PSFCH Rx | 85.75 | Interpolation between SCI1 only and PSCCH/PSSCH. Assuming 9 symbols is used (3 for PSCCH+3 for buffering + 2 for PSFCH + 1gap when RF is ON.) This is under the assumption that 5 symbols is enough for micro-sleep. |

Table 5: Power consumption levels for the remaining SL Tx states with BW = 40MHz and 1Tx antenna

| Tx State | Reference power | Comments |
| --- | --- | --- |
| PSCCH/PSSCH + PSFCH | 250 (0dBm) 700 (23dBm) | The same as PSCCH/PSSCH + PSFCH |
| PSCCH/PSSCH in PSFCH slot | 196 (0dBm) 550 (23dBm) | Interpolation assuming a 10-symbol transmission |

Finally, it should be noted that the above analysis assumes that the modulation order of 256QAM is used. However, for both V2X and public safety use cases, the modulation orders are expected to be equal or smaller than 64QAM. Hence, RAN1 should identify scaling factors to account for the impact of the modulation order on the overall SL power consumption.

Proposal 1: RAN1 should identify scaling factors to account for the impact of the modulation orders below 256QAM on the overall SL power consumption.

# Evaluation Assumptions and Methodologies for V2X

In this section, we discuss evaluation assumptions for power savings in V2X applications based on [6] and [7].

## Evaluation Scenarios

Urban scenarios are more likely to have a mix of power-sensitive and always-on UEs than highway scenarios. Therefore, they could provide better insights into how a power saving scheme would perform in deployment.

Observation 1: Urban scenarios can provide better insights into power saving evaluations for V2X applications.

## UE Drop and Mobility Modelling

The vehicular UE drop and mobility model from [7] can be reused; and for pedestrian UEs, modelling from [8] can be reused with the addition of 1000 pedestrians as a simulation assumption to better reflect pedestrian density.

Proposal 2: Introduce 1000 as the number of pedestrian UEs for power saving evaluations for V2X applications.

## Traffic Model

The traffic models from [7] can be reused, but discussion is needed on what inter-packet arrival times and what packet sizes to use for traffic to and from power-sensitive UE. In our view, the following traffic model could be added to address both V2P and P2V traffic:

- Aperiodic traffic

  - Model 3:

    - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms.

    - Packet size: uniformly random in the range between 200 and 800 bytes with the quantization step of 200 bytes.

    - Latency requirement: 100ms

Proposal 3: Introduce the following additional traffic model for power saving evaluations for V2X applications:

- Aperiodic traffic

  - Model 3:

    - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms

    - Packet size: uniformly random in the range between 200 and 800 bytes with the quantization step of 200 bytes.

    - Latency requirement: 100ms

Other than the details of traffic models, it is important to discuss how to mix traffic models. Some evaluations will contain two types of UEs: always-on UEs (e.g. vehicles, RSUs, …) and power-sensitive UEs (e.g. pedestrians). It is reasonable to consider the use of different traffic types (V2V, V2P, and P2V) and associated models simultaneously in such an evaluation.

Proposal 4: Use a mix of traffic models for power saving evaluations with a mix of UE types for V2X applications.

# Evaluation Assumptions and Methodologies for Public Safety

In this section, we first, based on [2], propose a set of evaluation assumptions for public safety.

## Deployment Layouts

The following options can be considered:

- Urban macro layout with 1732m ISD (Option 5 from [2])

- Urban macro layout with 500m ISD (Option 3 from [2])

Both layouts are based on a hexagonal grid with 7 or 19 macro sites; each macro site consists of 3 cells (sectors). One of the two layouts (e.g., the first one) can be considered as the basis for the evaluations, while the other one is optional.

Additional details on the deployment scenarios are given in the table below:

| PARAMETER | VALUE |
| --- | --- |
| Carrier Frequency | 700MHz |
| System Bandwidth | 10MHz |
| Wraparound | Considering an OOC scenario, a wraparound of the layouts should be considered. |
| UE Mobility | 3kmph and 60kmph (optional) |
| Maximum TX Power | 23dBm or 31 dBm |
| Number of Antennas | 1T2R |
| Antenna Gain | 0dBi |
| Noise figure | 9dB |
| Number of UEs participating in a D2D communication session | Unicast: 2 Groupcast: 10 (One is a TX UE) Broadcast: One TX, a random number of RX-UEs as a result of association phase |
| Average number of communication sessions per cell | Unicast: 12 Groupcast: 3 (One is a TX UE) Broadcast: 3 or other optional values Note: These are average values since the TX and RX-UE distribution is uniform over the whole layout of 7 or 19 sites and session initiator TX UEs do not have to be equally distributed to cells. |
| Uniform Drop (for Option 3 and 5 of [2]) | All UEs are randomly and uniformly dropped throughout the layout. All UEs are outdoor and no buildings are dropped. |
| Indoor-Outdoor mixture of UEs (for Option 3 and 5 of [2]) | - Drop exactly 2 buildings in each cell area such that they are not overlapping. - Drop 2/3 of UEs inside the buildings and 1/3 uniformly over the layout. - By naming some of the outdoor UEs as virtually indoor UEs (if the ratio of indoor UEs is less than 80 %) maintain the indoor-to-outdoor ratio as 4/1. |
| Option 3 | Same as Option 5. |
| Minimum distance between UEs | >= 3m |
| Minimum association RSRP for D2D | TBD Note: RAN1 should clearly define how the RSRP is measured. In particular, which Tx power and transmission BW should be considered. |

## Dropping and Association for Different Cast Types

The following guidelines can be considered for UE dropping and association:

- Only one cast type per simulation is assumed.

- The traffic is always unidirectional from a pre-defined source UE to other UE(s). The pre-defined source is always the first randomly selected UE in all cast types.

- The association RSRP threshold is calculated by taking only the transmit power, the large scale pathloss between the communicating UEs, and the shadowing into account.

- For unicast: 150 UEs are dropped per cell. One UE is selected randomly within all UEs on the layout as the TX-UE and corresponding unicast pair, the RX-UE, is selected within the remaining UEs such that it satisfies the RSRP association requirement. 12 such pairs are chosen on average per cell.

- For groupcast: 150 UEs are dropped per cell. One UE is selected randomly within all UEs on the layout as the TX-UE. 9 RX-UEs are associated to this TX-UE using the RSRP threshold. 3 such groups are chosen on average per cell.

- For broadcast: 32 UEs are dropped per cell. One UE is selected randomly within all UEs on the layout as the TX-UE. Continue until 3 TX-UEs per cell (on average) are selected. For all remaining RX-UEs, associate a potential RX-UE to each one of TX-UEs using the RSRP threshold. There is no limit on the number of sessions that a given RX-UE listens to except the total number of TX-UEs. Similarly, there is no defined number for the associations of a TX-UE.

## Channel Models

| Model/Parameter | Value/Formulation |
| --- | --- |
| Outdoor-to-outdoor |  |
| Pathloss | Winner+ B1 (Urban micro model) with antenna heights 1.5m and hexagonal layout. |
| LOS probability | Winner II with B1 model given in Table 4-7 of [9] |
| Shadowing | Log-normal with std = 7 dB, i.i.d. |
| Fast Fading Model | ITU-R IMT Urban micro LOS and NLOS (Annex 1.3.2) [3]. |
| Outdoor-to-indoor |  |
| Pathloss | Winner+ B1 (Urban micro model) with additional penetration loss of 20 dB, indoor loss of 0.5d_in, antenna heights 1.5m, and hexagonal layout for LOS path. For NLOS path, subtraction of 0.8h_MS from the LOS value above. Note: d_in is the indoor UE’s distance from the wall on the path to the outdoor UE. h_MS is the antenna height of the UE. |
| LOS probability | ITU-R IMT Urban micro on Table A1-3 [3]. |
| Shadowing | Log-normal with std = 7 dB, i.i.d. |
| Fast Fading Model | ITU-R IMT Urban micro O2I (Annex 1.3.2 [3]) |
| Indoor-to-indoor |  |
| Pathloss | InH (indoor hotzone) model in [4], Table A2.1.1.5-1. |
| LOS probability | ITU-R IMT Urban micro on Table A1-3 [3]. |
| Shadowing | Log-normal with std = 3 dB for LOS UEs in the same building Log-normal with std = 4 dB for NLOS UEs in the same building Log-normal with std = 10 dB for UEs in different building For all cases, shadowing i.i.d. |
| Fast Fading Model | ITU-R IMT InH (Annex 1.3.2) for both LOS and NLOS [3]. |

## Public Safety Requirements

In [5], the KPIs for some of the mission critical services (such as push-to-talk, video and data) are listed:

| Packet Delay Budget | Packet Error Rate | Example Services |
| --- | --- | --- |
| 75ms | 10-2 | Mission Critical user plane Push-to-Talk voice (e.g., MCPTT) |
| 100ms | 10-3 | Mission Critical Video |
| 60ms | 10-6 | Mission Critical delay sensitive signaling (e.g., MC-PTT signaling) |
| 200ms | 10-6 | Mission Critical Data (e.g. Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.)) |

As is evident from the table above, the requirements are diverse. Based on these set of requirements as a starting point, RAN1 should adopt a set that could cover different public safety use cases. In addition to the PDBs and packet error rates, the expected communication range and the number of users in each group around an incident area should also be decided for evaluations.

Proposal 5: For performance evaluation of public safety services, RAN1 should select a set of traffic patterns and requirements in terms of latency, reliability, range and number of supported UEs that can cover a diverse set of use cases.

## Public Safety System Level Evaluation Results

A sample scenario is considered for NR sidelink public safety performance evaluations based on a selection over the diverse set of parameters listed in the preceding sections. The scenario of interest covers the broadcast MCPTT voice service and the system parameters for which a selection is made within the possible values are provided in the table.

| PARAMETER | VALUE |
| --- | --- |
| Wraparound | Considered |
| UE Mobility | 3kmph |
| Maximum TX Power | 31 dBm |
| Number of Antennas | 1T2R |
| Antenna Gain | 3dBi |
| Cast Type | Broadcast |
| Number of UEs participating in a D2D communication session | One TX, a random number of RX-UEs as a result of association phase |
| Average number of communication sessions per cell | 3 TX UEs per cell on average |
| Layout | Option 5 with ISD = 1732m |
| Number of Sites | 7 sites each with 3 cells |
| Drop Type | All UEs are randomly and uniformly dropped throughout the layout. All UEs are outdoor and no buildings are dropped. |
| Minimum association RSRP for D2D | -107 dBm for nominal scenario can change for observing association characteristics. |
| Packet Delay Budget | 75ms |
| Packet Error Rate | < 10-2 |
| Traffic Type | VoIP traffic with 20 ms voice payload of 41 bytes. Alternating speech and talk spurt events with exponentially distributed durations of average length 2.5 and 0.833 seconds, respectively. |
| Number of transmissions per VoIP payload | 1,2, or 4 transmissions for different evaluations, where all the re-transmissions following the first transmission are blind. |

System level evaluation is done for a network of 672 UEs (32 of which are randomly selected as broadcasting TX UEs). Three separate simulations are run such that a minimum 3250, 2300, and 1450 distinct packets (VoIP payloads) are transmitted by a TX UE for 1 TX, 2 TX, and 4 TX test cases.

The association for RX UEs follows the RSRP threshold check, where a viable RSRP threshold of -107 dBm is selected after conducting experiments on successful decoding event of the PSSCH transmitted under the deteriorating effects of pathloss, shadow fading, and interference resulting from the other broadcasting UEs in the network. In particular, since the payload for VoIP traffic is small, the transmissions on PSSCH are QPSK modulated with a relatively low code rate, which allows for selection of lower RSRP threshold.

The association for the given parameters in this section can be investigated from different viewpoints. One way of identifying the association performance would be by observing the cumulative distribution function (CDF) of the distances for which RX UEs are able to be associated with a broadcasting TX UE. Figure 1 provides the CDF for successfully associated link distances. It can be deduced that an RX UE have to be in close proximity of a TX UE for a successful association. In particular, for the selected association RSRP threshold of -107 dBm, %90 of the RX UEs are within 1 km distance from a TX UE. One also should consider the fact that an RX UE can receive from more than one broadcast sessions as long as it is associated to them.

Figure 1. CDF of distance for associated RX UEs’ distances to the TX UEs

A second way of observing the association characteristics would be by plotting the association probability versus the distance of an RX UE to a TX UE. This second method is shown in Figure 2. For an RX UE that is at a distance of 580 meters, the probability of association to a given TX UE is 0.5. This can also be interpreted as follows: For a public safety device trying to reach potential RX devices, more than half of the devices are not accessible via VoIP service at a distance larger than 580 meters.

Observation 2: For public safety MCPTT voice service, a broadcasting TX UE can only establish communication up to half of the RX UEs when the RX UE distance is larger than 580 meters. This proportion of accessible RX UEs is lower for larger distances.

Figure 2. Association probability at a distance d from the TX UE.

Another possible metric for observing the association performance would be based on the number of RX UEs listening to a broadcast session. Figure 3 demonstrates the CDF of the number of associated RX UEs over 63 TX UEs active in the network. It is identified that most of the broadcast sessions can maintain connection with 50 to 55 RX UEs, while %90 of the TX UEs observe less than or equal to 60 RX UEs associated to them.

Observation 3: For public safety MCPTT voice service, a broadcasting TX UE is expected to have a group size around 50-55 RX UEs. The maximum groups size is limited to 65 for this scenario.

Figure 3. Number of associations for a broadcasting TX UE.

One of the most important metrics for the public safety VoIP service applications would be the successful packet reception for the RX UEs that are associated to a broadcast session. As given on the Public Safety Requirements table, MCPTT voice data transmission necessitates PER < 10-2 or packet reception ratio (PRR) > 0.99. We identify the PRR performance with respect to the associated link distance in Figure 4. Different number of re-transmissions are compared with respect to successful decoding of VoIP payloads and a successful decoding is defined as at least one correct PSSCH decoding event out of N transmissions of a transport block (TB) for N TX/TB scheme. Single transmission (1Tx/TB with no re-transmissions) is limited to less than 200 meters of communication distance with acceptable VoIP performance of PRR > 0.99, even for the associated UEs. On the other hand, 4 TX/TB scheme satisfies the reliability requirement of associated RX UEs well over 2000 meters. It should be stressed once more that although high PRR is satisfied up to communication distances of 2000 meters for associated UEs, the association probability is well below 0.1 at distances above 1000 meters. This is further exemplified on the same figure with the right axis representing the total number of RX associations for UEs within a given distance bin. As an example, for the distance bin centred at 1860 meters (from 1800m to 1920m), the number of RX associations is only 6. This also explains the rather non-monotonic behaviour of the PRR curves in spite of the large number of VoIP payloads generated per TX UE.

Observation 4: For RX UEs associated with a broadcast session utilizing public safety MCPTT voice service, PRR > 0.99 can be satisfied up to 2 km distance by using 4 repetitions per VoIP payload.

Figure 4. PRR performance for different number of re-transmissions vs distance bins on the left axis. The number of RX UEs associations vs distance bins on the right axis.

# Conclusion

Observation 1: Urban scenarios can provide better insights into power saving evaluations for V2X applications.

Observation 2: For public safety MCPTT voice service, a broadcasting TX UE can only establish communication up to half of the RX UEs when the RX UE distance is larger than 580 meters. This proportion of accessible RX UEs is lower for larger distances.

Observation 3: For public safety MCPTT voice service, a broadcasting TX UE is expected to have a group size around 50-55 RX UEs. The maximum groups size is limited to 65 for this scenario.

Observation 4: For RX UEs associated with a broadcast session utilizing public safety MCPTT voice service, PRR > 0.99 can be satisfied up to 2 km distance by using 4 repetitions per VoIP payload.

Observation 2: For public safety MCPTT voice service, a broadcasting TX UE can only establish communication up to half of the RX UEs when the RX UE distance is larger than 580 meters. This proportion of accessible RX UEs is lower for larger distances.

Observation 3: For public safety MCPTT voice service, a broadcasting TX UE is expected to have a group size around 50-55 RX UEs. The maximum groups size is limited to 65 for this scenario.

Observation 4: For RX UEs associated with a broadcast session utilizing public safety MCPTT voice service, PRR > 0.99 can be satisfied up to 2 km distance by using 4 repetitions per VoIP payload.

Proposal 1: RAN1 should identify scaling factors to account for the impact of the modulation orders below 256QAM on the overall SL power consumption.

Proposal 2: Introduce 1000 as the number of pedestrian UEs for power saving evaluations for V2X applications.

Proposal 3: Introduce the following additional traffic model for power saving evaluations for V2X applications:

Proposal 4: Use a mix of traffic models for power saving evaluations with a mix of UE types for V2X applications.

Proposal 5: For performance evaluation of public safety services, RAN1 should select a set of traffic patterns and requirements in terms of latency, reliability, range and number of supported UEs that can cover a diverse set of use cases.

# References

- 3GPP TS 38.840, “Study on user equipment (UE) power saving in NR (Release 16)”

- 3GPP TS 36.843, “Study on LTE device to device proximity services (Release 12)”

- ITU-R M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-advanced”, accessed at https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2135-1-2009-PDF-E.pdf

- 3GPP TS 36.814, “Further advancements for E-UTRA physical layer aspects (Release 9)”

- 3GPP TS 23.501, “System architecture for the 5G system (5GS); stage 2 (Release 16)”

- 3GPP TS 38.885, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”

- 3GPP TS 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR (Release 15)”

- 3GPP TS 36.885, “Study on LTE-based V2X Services (Release 14)”

- IST-4-027756 WINNER II, D1.1.2 V1.2, “WINNER II Channel Models: Part I Channel Models”