3GPP TSG-RAN WG1 Meeting #103-e	R1-2009302

eMeeting, October 26th – November 13th, 2020

Agenda Item:	8.10.2

Source:	Ericsson

Title:	Timing, CLI and power control in enhanced IAB

Document for:	Discussion, Decision

# Introduction

In RAN#88, the following objectives of the IAB work item regarding IAB duplexing enhancements were approved [1]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including: - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx). - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing. - Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In RAN1#102e, the following agreements were made [2]:

Agreement - Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx) - Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx) - RAN1 should strive to minimize specification impact due to this feature - FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)

This contribution addresses Case-6 and Case-7 timing configurations required to allow simultaneous transmission on child and parent links using the same carrier frequencies and their challenges.

We also study different interference types that operation of IAB-nodes can cause to other RAN nodes, depending on whether transmissions take place in UL or DL slots and discuss power control for IAB-MTs and IAB-DUs.

# Timing

In Rel-16, a method was specified to enable Case-1 timing aligned DL transmission operation [3][4].

In order to enable simultaneous transmission on child and parent links, i.e. transmission in upstream and downstream (to avoid the terminology of UL and DL), respectively, these two transmissions should preferably also be time aligned. A similar argument can be made for the case of simultaneous reception on child and parent links. Such additional timing coordination, i.e., in addition to the Case-1 timing aligned operation, can be enabled by IAB Rel-17 supporting the timing configurations known as Case-6 and Case-7, for simultaneous transmission and reception, respectively [4]. In fact, is has been agreed to support both timing configurations [2].

Figure 1 shows the transmit and receive timing for Case-6 (left) and Case-7 (right) timing configuration, respectively, for a simple IAB network consisting of a donor/parent node (P), an IAB-node (N), a child node (C) with a Rel-15 UE (UE) connected to the IAB-node N. The propagation delay between parent node P and IAB-node N is denoted as T_PN, the delay between the IAB-node N and the child node C is denoted as T_NC, and the access link delay is denoted as T_UE.

Figure 1: Transmit and receive timing for Case-6 (left, DL and UL Tx timing aligned) and Case-7 (right, DL and UL Rx timing aligned) timing configurations. The propagation delays are chosen specifically for each case to illustrate critical aspects.

As can be seen in Figure 1, for Case-6, the UL Tx timing is set to be aligned with the DL Tx timing; for Case-7, the UL Rx timing is set to be aligned with the DL Rx timing.

It is observed from Figure 1 that for Case-6, the UL reception timing of an IAB-node is delayed due to the propagation delay on the child backhaul link (T_NC). For Case-7, the UL reception timing of an IAB-node is delayed due to the propagation delay on the parent backhaul link (T_PN).

For Case-6, the UL reception of an IAB-node is delayed due to the propagation delay of the child backhaul link and
for Case-7, the UL reception of an IAB-node is delayed due to the propagation delay of the parent backhaul link.

Figure 1 furthermore presents that for Case-7, a child node may have a negative timing advance (TA), if the propagation delay on a parent link (T_PN) is more than twice the propagation delay on a child link (T_NC), i.e., if T_PN > 2*T_NC. That relation was already identified during the IAB Study Item phase.

The conditions when a UE will have a negative TA are as follows:

Case-6:	The child link has twice the propagation delay of the UE access link (T_NC > 2*T_UE), which is likely, if a UE is somehow in between the IAB-node and child node but connected to the IAB-node cell.

Case-7:	The parent link has twice the propagation delay of the UE access link (T_PN > 2*T_UE).

For both Case-6 and Case-7, a Rel-15 UE can have a negative timing advance for certain link conditions.

As a result of the above, involving Rel-15 UEs during simultaneous transmission or reception on the IAB backhaul will be complicated. In principle, NR (contrary to LTE) does not have a (minimum) limitation on the TA and, effectively, a negative TA can be achieved by successively reducing the TA, starting from a positive value. However, this is not true for a Rel-15 UE during RA, when the UE (and also MT) always starts with a non-negative TA for the preamble transmission and subsequently TA is updated with a non-negative value in RAR. As a consequence, Rel-15 (and Rel-16) UEs may not be able to complete RA to an IAB-node operating in a Case-6 or Case-7 timing configuration on access links.

A UE connected to an IAB-node that operates with timing aligned transmission or reception, may require a negative TA.

NR does not support Rel-15/16 UEs, connecting to an IAB-node operating with timing aligned transmission or reception, having a negative TA already during the RA phase. And as a consequence, such UEs might not be able to complete RA to IAB-node operating in a Case-6 or Case-7 timing configuration on access links.

The fact that a UE with a negative initial TA may not be able to perform/complete RA leads us to the conclusion that simultaneous transmission or reception must be limited to only involve backhaul links.

Specification on simultaneous transmission or reception is limited to only consider operation on backhaul links.

RAN4 has investigated, though for the sake of interference and deployment requirements, two scenarios of upstream/downstream transmission configurations [5]. Scenario 1 is summarized as

- IAB-MT using the UL parts of the TDD pattern for IAB transmission

- IAB-MT using the DL parts of the TDD pattern for IAB reception

- IAB Parent using the DL parts of the TDD pattern for IAB transmission

- IAB Parent using the UL parts of the TDD pattern for IAB reception

Correspondingly, Scenario 2 is summarized as

- IAB-MT using the DL parts of the TDD pattern for either transmission or reception of backhaul link

- IAB Donor using the DL parts of the TDD pattern for either transmission or reception of backhaul link

Scenario 1 assumes a traditional transmission configuration, with all upstream and downstream transmissions in UL and DL slots, respectively. Scenario 2 assumes that an MT transmits its UL data in DL transmission slots; this is the only difference. Operation on access links is still according to Case-1, as in the first scenario.

One advantage of Scenario 2 is that simultaneous transmission or simultaneous reception will not impact the timing requirements of Rel-15 UEs, since IAB BH link transmission and reception is limited to DL slots in TDD patterns, since UE transmissions only occur in UL, i.e., access and IAB BH links are timing-wise separated.

Adopt Scenario 2 as defined in RAN4 (i.e., transmissions on BH links only in DL time slots) as baseline for studies related to simultaneous transmission or reception of IAB-nodes.

In Rel-16, RAN1 has specified an OTA method to enable Case-1 timing alignment based on timing advance information on the parent backhaul uplink and additional information about parent node internal uplink/downlink timing arrangements (T_delta). However, in a timing configuration as Case-7, an IAB-node is setting its own UL reception timing. Therefore, no additional signaling is required to align an IAB-node’s UL reception timing to its DL reception timing.

In Case-7, no additional signaling is required to align the UL reception timing of an IAB-node to its DL reception timing.

In Rel-16, support for Case-1 timing alignment was achieved by using meta-information of 4 timestamps. Two timestamps on the IAB-node side (TA as the difference between DL Rx and UL Tx) and the difference of two timestamps on parent node side (T_delta as the difference between DL Tx and UL Rx). Technically, no restrictions apply on the magnitudes of the timestamp differences or the relative location within periodic system frames. It should also work for a positive T_delta (as for Case-6, whereas T_delta in Rel-16 is always negative), potentially resulting in a negative TA on the IAB-node side.

As for Case-1, it is crucial in Case-6 for the IAB-node or its parent IAB-node to know the propagation delay Tprop between the parent and IAB-node. In the following, we show that the relation of Tprop, TA and T_delta (or any equivalent) also holds for any, including Case-6, timing configuration and is therefore universal.

If TA and a T_delta are defined as for Case-1, and we further introduce the term T corresponding to -2*T_delta as interpreted in Rel-16 agreements for brevity, then the following relations apply

- T: UL_RX_p + T = DL_TX_p

- TA: UL_TX_c + TA = DL_RX_c,

and if reception timings are delayed relative to their respective transmission timings (a matter of physics)

- UL_TX_c + Tprop = UL_RX_p

- DL_TX_p + Tprop = DL_Rx_c,

where UL_RX_p  and DL_TX_p denote the UL reception and DL transmission timing at parent IAB-node, respectively, and UL_TX_c  and DL_RX_c denote the UL transmission and DL reception timing at the IAB-node, respectively, it follows that 2*Tprop + T = TA (or Tprop=TA/2+T_delta, using the Rel-16 agreement notation). This is an equivalent relation to the one in Rel-16. We note that no restriction is made on UL_RX_p relative to the downlink transmission timing DL_TX_p at the parent IAB-node.

As already mentioned, in Case-6, the Rel-16 equivalent of T_delta would be positive, as the UL reception always takes place after the DL transmission (see Figure 3). Thus, the same method, providing information about the DL transmission and UL reception timing relation on parent node side, can be used for Case-6 as is used for Case-1 timing alignment specified in Rel-16. The information about this relation can apply the same granularity, it only requires a new range (in extension and sign) of T_delta. This requires involving RAN4 to propose a new T_delta range for Case-6.

In Case-6, a T_delta-based OTA timing alignment does not require any principal change of method and signaling type compared to Case-1, only a new range specification.

Adapt T_delta based Rel-16 Case-1 timing alignment as baseline for Case-6 timing synchronization.

# Cross link interference

In terms of transmission capabilities, such as power allocation, beamforming, etc., an IAB-node will have the same properties as ordinary gNBs, and it is not expected to differ apart from its backhauling capabilities. Thus, it should be expected that interference, possibly caused by an IAB-node, to any node in the RAN should not have more impact than interference caused by an ordinary gNB. Therefore, from an interference consideration, assessing IAB-nodes should be done in relation to if the same node instead is an ordinary gNB.

Interference caused by operation of IAB-nodes is assessed in relation to the interference if the IAB-node was an ordinary gNB.

Figure 2: Examples of possible strong interference to IAB and non-IAB RAN nodes by IAB-MT or IAB-DU transmission in an UL slot.

In the following, we study different interference types that an IAB network operation can cause to neighbor RAN nodes. Below we address the two cases of IAB transmissions in UL and DL, respectively.

## Interference from IAB transmissions in UL slots

Figure 2 shows examples of possibly strong interference types in case an IAB-node transmits in an UL time slot. As can be seen in these examples, any IAB transmission competing with a UE transmission in an UL slot can have severe impact on the performance of the UEs both inside and outside the IAB network. For instance, with an upstream transmission of IAB-MT to the parent IAB-DU in an UL slot, there is a risk that the signal transmitted by a UE to a neighbor gNB (or, the parent IAB-DU itself) will be highly affected by interference as shown in the left side of Figure 2. This is due to considerably higher transmit power of the IAB-MT compared a UE and possibly reduced transmit dynamic range. Similarly, severe interference due to, e.g., IAB-DU transmissions in UL slots can be expected, as presented in the right side of Figure 2. Here, the most obvious victims can be found in the non-IAB part of the network (operating on the same carrier frequency) where the IAB-nodes are deployed. However, the victims of interference outside the IAB network are also the networks on adjacent (or nearby) spectrum to that used by the IAB network since adjacent networks would typically have a coordinated UL/DL TDD pattern.

It should be noted that the interference arising from a substantially stronger signal superpositioned onto a weaker UE signal may not only be manifested as a reduced MCS. Due to the large differences in transmit power, it may be that the dynamic range of the receiving node is not able to resolve both signals. In this case, it will not be possible to at all detect the weaker UE which may have consequences, e.g., for the UE to perform random access or other fundamental operations.

IAB-nodes transmitting in UL slots may jeopardize fundamental network functions on both the same and adjacent carriers.

Figure 3: Examples of typical interference by IAB-MT or IAB-DU transmission in a DL slot.

## Interference from IAB transmissions in DL slots

With IAB-nodes transmitting in DL slots (see Figure 3 for different examples) any RAN node or UEs served by, e.g., a gNB outside the IAB network will experience “typical” interference, in the sense that the transmission of an IAB-node during the DL parts of the TDD pattern appears like a transmission from a typical RAN node transmitting during DL slots (which we observe as standard operation). Any interference due to such a transmission can be considered of no different nature than other DL transmissions, for which there are deployment requirements or recommendations to control the negative effects. Note that, such an argument holds not only for wide-area IAB-nodes but also for local-area IAB-nodes. This is because the transmission capabilities, e.g., transmit power, of a local-area IAB-node may range between a UE to a wide-area IAB, and the coverage of the node will vary accordingly. Hence, within the coverage area of a node the interference from a typical IAB-node will be similar.

IAB-nodes transmitting in DL slots will mostly limit interference to within the IAB network.

The above discussion and observations lead us to the following proposal:

Limit IAB-node transmissions to DL slots.

Interestingly, such a setup with IAB transmission limited to DL transmission falls back to Scenario 2 as defined by RAN4 (see [5]). Therefore, we propose further to adopt Scenario 2 for Rel-17.

Adopt RAN4’s Scenario 2 for Rel-17.

# Power control

The discussion about power control can be divided into the more common UL and UE-like power control in the IAB-MT and the uncommon DL power control in the IAB-DU. Since the two are substantially different, we discuss them separately below.

## IAB-MT power control

The rationale for UL power control is similar to traditional UEs – devices in proximity to the serving node should be able to coexist with devices at the cell edge. Although IAB-nodes are not UEs, some resemblance to the above requirements can be found in the RAN4 specs. RAN4 has agreed on two different IAB-MT classes, the wide are and the local area IAB-nodes. The use case for the wide area IAB-node is that it is an independent IAB-node providing its own coverage, requiring a long distance backhaul link to connect to its parent IAB-node, see Figure 4. For such a deployment, little power control is required, which is also why RAN4 limited the dynamic range of the wide are IAB-MT to 5 dB.

Figure 4: Deployment scenario for wide area IAB-nodes.

The use case for the local area IAB-node, on the other hand, is to boost capacity within an already existing  cell served by a donor or parent IAB-node, see Figure 5. For this scenario it is apparent that the IAB-node may require a higher dynamic range in order to manage more variable deployments, e.g., closer to or further away from the donor or parent IAB-node. Consequently, RAN4 specified a 10 dB dynamic range requirement for this case.

Figure 5: Deployment scenario for local area IAB-nodes.

In addition to the different scenarios, different generations of IAB-nodes may have varying capabilities to control transmit power. For example, an IAB-node using analogue BF may not have individual control of its antenna elements whereas a future IAB-node using digital BF may have. Additionally, the first generation of IAB-nodes, based on existing HW, may have little power control functionality. Hence, the spec should allow for different capabilities of power control.

The specification should allow for different IAB-MT power control capabilities, considering existing and future HW architectures.

## IAB-DU power control

On its DU-side, the IAB-node takes the role as a base station. For that reason, on the one hand, it should provide a constant output power to achieve predictable coverage. That implies that the coverage providing broadcast signals and channels will not use power control. Instead, power control is limited to a subset of all channels, essentially unicast PDCCH and PDSCH. Such a selective power control requirement will put further requirements on the IAB-DU in that rapid power control may be required to allow transmit power changes from one slot to another or even from one symbol to another. Such requirements are presently not feasible and likely will not be, since power amplifiers typically require at least a few µs for changes to stabilize. Switching power levels would hence be associated with reduced network capacity due to the symbols that would be left unused during the adjustment.

Power control in DL broadcast signals and channels will affect cell coverage.

To suit all IAB-DU power control requirements would necessitate slot-by-slot or symbol-by-symbol power control which is infeasible.

On the other hand, it would clearly be beneficial from a multiplexing perspective with some DL power control, e.g., to allow simultaneous reception in an IAB-node between a wide area parent IAB-node and a local area IAB-node or a UE. While it has been explained in the two first sections in this contribution why including UEs in the simultaneous communications schemes to be specified should be avoided, allowing it for local area IAB-node is certainly within the scope of the WID. For that reason, introducing some optional DL power control may be beneficial for some use cases.

DL power control is optional for IAB-DU.

Further study DL power control for PDCCH and PDSCH.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For Case-6, the UL reception of an IAB-node is delayed due to the propagation delay of the child backhaul link and for Case-7, the UL reception of an IAB-node is delayed due to the propagation delay of the parent backhaul link.

Observation 2	For both Case-6 and Case-7, a Rel-15 UE can have a negative timing advance for certain link conditions.

Observation 3	A UE connected to an IAB-node that operates with timing aligned transmission or reception, may require a negative TA.

Observation 4	NR does not support Rel-15/16 UEs, connecting to an IAB-node operating with timing aligned transmission or reception, having a negative TA already during the RA phase. And as a consequence, such UEs might not be able to complete RA to IAB-node operating in a Case-6 or Case-7 timing configuration on access links.

Observation 5	In Case-7, no additional signaling is required to align the UL reception timing of an IAB-node to its DL reception timing.

Observation 6	In Case-6, a T_delta-based OTA timing alignment does not require any principal change of method and signaling type compared to Case-1, only a new range specification.

Observation 7	IAB-nodes transmitting in UL slots may jeopardize fundamental network functions on both the same and adjacent carriers.

Observation 8	IAB-nodes transmitting in DL slots will mostly limit interference to within the IAB network.

Observation 9	Power control in DL broadcast signals and channels will affect cell coverage.

Observation 10	To suit all IAB-DU power control requirements would necessitate slot-by-slot or symbol-by-symbol power control which is infeasible.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Specification on simultaneous transmission or reception is limited to only consider operation on backhaul links.

Proposal 2	Adopt Scenario 2 as defined in RAN4 (i.e., transmissions on BH links only in DL time slots) as baseline for studies related to simultaneous transmission or reception of IAB-nodes.

Proposal 3	Adapt T_delta based Rel-16 Case-1 timing alignment as baseline for Case-6 timing synchronization.

Proposal 4	Interference caused by operation of IAB-nodes is assessed in relation to the interference if the IAB-node was an ordinary gNB.

Proposal 5	Limit IAB-node transmissions to DL slots.

Proposal 6	Adopt RAN4’s Scenario 2 for Rel-17.

Proposal 7	The specification should allow for different IAB-MT power control capabilities, considering existing and future HW architectures.

Proposal 8	DL power control is optional for IAB-DU.

Proposal 9	Further study DL power control for PDCCH and PDSCH.

# References

RP-201293, “Enhancements to Integrated Access and Backhaul for NR,” Qualcomm, RAN Plenary #88-e, June 2020.

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102e, August 2020.

3GPP TS 38.213, V16.1.0, Physical layer procedures for control.

3GPP TR 38.874, V16.0.0, Study on Integrated Access and Backhaul.

R4-1905105, “Summary of the IAB scenario”, Ericsson, RAN4 #90bis, April 2019.

R1-2005953, “Measurement results for IAB multi-panel transmission and reception”, AT&T, 3GPP TSG RAN WG1 #102-e, August 2020.