3GPP TSG RAN WG1 Meeting #103-e	R1-2009425

# E-meeting, 26 October – 13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0550 | rev | - | Current version: | V16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on interference randomization for NB-IoT SPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2020-10-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#90, the following in R1-1714639 was agreed: - The interference randomization technique for NPDCCH is applied in the following cases: - For Type-2 and Type1-CSS, in non-anchor carriers. - For USS, is enabled by RRC configuration. - For Type-1A and Type-2A, always. According to the agreement, the interference randomization is applied to NPDCCH in USS when enabled by RRC. The NPDCCH associated with SPS C-RNTI is mapped on USS, however, current spec omits it from interference randomization. Besides, the cross reference of interferenceRandomisationConfig is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that NPDCCH associated with SPS C-RNTI applies interference randomization if enabled by RRC configuration, i.e. interferenceRandomisationConfig. - Correct the cross reference of interferenceRandomisationConfig from [11] to [9]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreement in R1-1714639 is incompletely captured, letting aside the case where NPDCCH is associated with SPS C-RNTI, which is also mapped on USS. The performance of NPDCCH associated with SPS C-RNTI will be degraded. The number of NPDCCH blind decodes is increased if NPDCCH associated with C-RNTI is with interference randomization and SPS C-RNTI is not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.4, 10.2.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 10.2.3.4	Mapping to resource elements

#### < Unchanged parts are omitted >

For frame structure type 1,

-	for NPDCCH associated with RA-RNTI, TC-RNTI or

-	for P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or

-	for NPDCCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or

-	for NPDCCH associated with PUR-RNTI/G-RNTI/ SC-RNTI, or

-	for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [9], or

for frame structure type 2,

each complex-valued symbol , shall be multiplied with ,where

where the scrambling sequence  is given by clause 7.2 and shall be initialized at the start of each subframe with .

#### < Unchanged parts are omitted >

#### < Unchanged parts are omitted >

#### 10.2.5.5	Mapping to resource elements

The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS, and

-	except in a special subframe when NPDCCH is transmitted in more than one subframe, they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4],

-	in addition, for frame structure Type 2,

-	in a special subframe where the NPDCCH is transmitted in one subframe, they are in DwPTS

-	in a special subframe where the NPDCCH is transmitted in more than one subframe, they are not NRS locations when the subframe is not a special subframe.

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. Denote  as the complex-valued symbols that are mapped to resource elements meeting the criteria above in subframe ns/2, with the insertion of <NIL> elements in the locations of resource elements which are not part of the NCCE(s) assigned for the NPDCCH transmission.

If the NPDCCH is transmitted in more than one subframe, the resource elements in a special subframe that are not part of DwPTS are counted but not used in the mapping. When l= NsymbDL-5, NsymbDL-4, the resource elements in a special subframe assumed by the UE for NRSs are counted but not used in the mapping if the NPDCCH is transmitted in more than one subframe.

For frame structure type 1,

-	for NPDCCH associated with RA-RNTI, TC-RNTI or

-	for P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or

-	for NPDCCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or

-	for NPDCCH associated with PUR-RNTI/G-RNTI/ SC-RNTI, or

-	for NPDCCH associated with C-RNTI or SPS C-RNTI when interferenceRandomisationConfig is used according to [9], or

for frame structure type 2,

each complex-valued symbol , shall be multiplied with ,where

where the scrambling sequence  is given by clause 7.2 and shall be initialized at the start of each subframe with .

The NPDCCH transmission can be configured by higher layers with transmissions gaps where the NPDCCH transmission is postponed. The configuration is the same as described for NPDSCH in clause 10.2.3.4.

The UE shall not expect NPDCCH in subframe  if it is not a NB-IoT downlink subframe. In case of NPDCCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDCCH transmission is postponed until the next NB-IoT downlink subframe.

#### < Unchanged parts are omitted >