| 3GPP TSG-RAN WG1 Meeting #103 e-Meeting, October 26th – Nov 13th, 2020 |  | R1-2009451 |
| --- | --- | --- |

Agenda item:	8.8.2.2

Source: 	EURECOM, Sophia Antipolis

Title: 	Low-PAPR Sequence-Based Approaches for PUCCH Coverage Enhancement

Document for:	Discussion/Decision

# Introduction

In this contribution, we shall discuss means to improve the transmission waveforms for PUCCH in order to enhance coverage. We provide three concrete examples for 4, 11 and 22 bit transmission which can lead to substantial coverage enhancements. Three different schemes are proposed depending on the number of allocated PRBs and payload size. All proposed strategies provide significant coverage enhancement over current Release-15 PUCCH Format 3 transmission even when near optimal receivers are used for the latter. The proposed schemes do not make use of DMRS and are amenable to low-complexity receivers. One particular configuration of the proposed schemes achieves 0 dB PAPR.  The moderate payload scheme in this set of proposals reuses the existing Release-15 Polar coding methods and only adapts the modulation component to allow for non-coherent reception and low-PAPR.

# Low-PAPR Transmission Strategies in NR

NR/LTE Waveforms such as the PRACH and PUCCH  are designed for transmitting small payloads (tens of bits) with low probability of error at very low signal-to-noise ratios. This is achieved both by optimizing the energy efficiency of the transmit waveform through (a) minimizing peak-to-average power ratio (PAPR) and (b) design of strong structured coding strategies permitting low-complexity detection algorithms.

In terms of the precise use-case of the PUCCH, we are primarily concerned with scenarios where the transmitted waveform makes use of prior information regarding synchronization with respect to the time window of the gNodeB receiver. The current PUCCH transmission formats in 5G NR are summarized in terms of the their high-level characteristics in Table 1.  We see that the low-PAPR configurations with multiple-symbols are formats 1,3 and 4.

While PUCCH Format 0 has low PAPR, it seems not to have been designed for coverage because of the limitation to transmission in 1 or 2 symbols and thus is energy-limited. For the sake of completeness and comparison with the proposals in this correspondence, the Format 0 scheme is shown in Figure 1. The message index m is combined with a pseudo-random integer derived from system and user-specific parameters to select a low-PAPR quaternary sequence from a set of optimized sequences and to choose a cyclic time-shift of the selected sequence.

PUCCH Format 2 has neither low-PAPR nor multi-symbol transmission for coverage and likely to be used primarily in small-cell configurations and as such it is not a scheme which should be considered as a baseline for improvement.

Format 1 uses low-PAPR sequences similar to Format 0 but without a message dependent cyclic time-shift. The sequence is modulated in amplitude depending on the 1 or 2 bit message and repeated across L-LDMRS symbols, where LDMRS is the number of OFDM symbols corresponding to the so-called demodulation reference signals (DMRS). The DMRS symbols are not message dependent and can be used by the gNodeB receiver to resolve channel uncertainty through explicit channel estimation or more general methods based on joint detection and estimation. This format exhibits a bounded PAPR less than 2.8 dB.

Formats 3 and 4 use DFT-precoded QPSK or π/2-BPSK for low-PAPR rather than optimized quaternary sequences. For the case of π/2-BPSK the current formats provide a PAPR of 4.78dB at a 1% outage for randomized payloads. The PAPR values reported here were determined by the authors using well-known numerical techniques.

Similar to Format 1, Formats 3 and 4 also make use of 1,2 or 4 DMRS which are similar payload-independent signals. The transmission scheme is shown in Figure 2. In terms of channel coding, Formats 3 and 4 make use of classical length 32 Reed-Muller codes for 3-11 bit payloads[38.212]. The 32-bit encoded bit stream is shortened or lengthened with rate-matching which amounts to truncation or repetition to fit the available resources. For more than 11 bits, the 3GPP polar code [TS 38.212] with interleaving and a different rate-matching procedure is used. Following rate-matching and interleaving, the bit stream is scrambled using a pseudo-random sequence prior to modulation, in both cases. Format 4 also includes a time-domain orthogonal spreading function on top of the modulation to allow for multiple-acesss.

The decoding algorithms used in the current PUCCH formats are typically sub-optimal reduced-complexity approaches consisting of least-squares channel estimation based on the DMRS signals followed by quasi-coherent detection and soft-combining of repetitions from rate matching. These two operations yield soft-decisions for each of the coded bits. In the case of the RM code, a permutation is then applied to the 32 received soft-decisions to recuperate the bi-orthogonal structure of the code which is then amenable to reduced-complexity decoding via a fast Hadamard Transform. For the polar-coded case, the soft-decisions are passed to one of the reduced-complexity polar decoding algorithms available in the scientific literature. It should be noted that in both cases these algorithms are sub-optimal because of the detection procedure involving separate channel estimation and quasi-coherent detection. A fully non-coherent receiver using the DMRS signals could also be the chosen implementation method, at the expense of an increase in complexity, especially for the RM coded case with longer bit lengths.

With respect to the identified NR PUCCH formats, the current proposal provides methods for significantly improving the probability of correct detection compared to existing solutions for PUCCH in extreme coverage scenarios which are characterized by low signal-to-noise ratio. Moreover, the proposed methods allow for mitigation of inter-cell interference and, optionally, two levels of protection for different types of payload information aggregated in a common message. Specifically, we propose methods which do not use DMRS signals and are applicable to payload lengths up to 16 bits. Moreover, the methods exhibit strictly bounded and lower or equal PAPR to PUCCH formats 0 and 1. In one particular configuration (single resource element transmission) 0 dB PAPR is achieved.

Figure 1: 5G NR PUCCH Format 0 (L=1,2)

Sequence

selector

sequence

generation

m=0,1,…,2B-1

rl(m), l=0,1,…,L-1

System/user parameters

Figure 2: 5G NR PUCCH3/4 Transmission format

Binary code (including Rate-matching)

m=0,1,…,2B-1

Modulation

(π/2 –BPSK, QPSK)

rl(m),l=0,…,L-1

System/user parameters

12K-point

DFT

| PUCCH format | Transmission method | PAPR dB |
| --- | --- | --- |
| 0 | 1-3 bit orthogonal transmission, cyclic time-shifts of a low-PAPR quaternary sequence, L=1,2, K=1 | 2.42 – 2.80 dB |
| 1 | 1-2 bit payload, orthogonal transmission (for 1-bit), non-orthogonal transmission (for 2-bit), low-PAPR frequency-domain sequences with orthogonal spreading in time, use of demodulation reference signals (DMRS), L=4,5,…,14, K=1 | 2.42 – 2.80 dB |
| 2 | >2 bit payload, non-orthogonal transmission, binary code, binary rate-matching, QPSK modulation, L=1,2, arbitrary K | 9.1 dB at 1% outage, 6.66 dB average |
| 3 | >2 bit payload, non-orthogonal transmission, binary code, binary rate-matching, π/2-BPSK, QPSK modulation, DFT precoding, L=4,5,…,11, K=2α3β5γ | QPSK : 6.30dB at 1% outage, 4.28dB average π/2-BPSK : 4.78dB at 1% outage, 3.28 dB average |
| 4 | >2 bit payload, non-orthogonal transmission, binary code, binary rate-matching, π/2-BPSK, QPSK modulation, DFT precoding, orthogonal time-spreading, L=4,5,…,11, K=1 | QPSK : 6.30dB at 1% outage, 4.28dB average π/2-BPSK : 4.78dB at 1% outage, 3.28 dB average |

Table 1 : Comparison of PUCCH Transmission formats

# Proposal for block lengths less than 16 bits

The coding construction considered here consists of the concatenation of a sequence of low peak-to-average power ratio vectors, rl(m), l=0,1,…, L-1, of dimension 12K which depend on both the message and the symbol index.

split

Signal code

sequence

selection/

generator

m=0,1,…,2B-1

rl(m), l=0,1,…,L-1

wlm,l=0,…,L-1

combinor

rl(m)wlm,l=0,…,L-1

Time index

frequency index

System/user parameters

This is similar in spirit to NR 5G PUCCH format 0, although generalized to different sequences providing potentially lower PAPR and providing the means for extending the transmission over multiple symbols. Furthermore, the concatenation with a structure signal code provides a means to increase the number of payload bits compared to NR 5G PUCCH format 0. The overall code structure is shown in Figure 3 where the B bits are optionally split into two components of size B0 and B1, such that B=B0+B1. The overall bitstream is used as input to a low peak-to-average power ratio (PAPR) frequency-domain sequence selection procedure and a time-domain overlay signal-space code. The partition of the input bitstream is meant to provide the possibility for two levels of error protection, whereby the decoding process for the  B0  bits has a lower error probability than the decoding process of the B1 bits.

Figure 3: Low-PAPR Concatenated Coding Scheme

The sequence of L low-PAPR vectors are used to signal in the K PRBs and L symbols potentially across multiple time slots. The sequence of low-PAPR vectors should include randomization of the resource elements both in the form of using different vectors in each symbol and also using different frequency positions if the vectors have less non-zero elements than the total number of dimensions.

The sequence of L vectors are multiplied by an outer code sequence wl(m) with wl(m) =1  ∀l,m to yield the overall coded sequence as xk,l(m)=  rk,l(m) wlm for each symbol and resource element.

## Sequence Selection

The vectors rl(m) are firstly designed so that they minimize the peak-to-average power ratio of the resulting time waveform after OFDM modulation

The second criterion for choosing the low-PAPR vectors is to limit the number of non-zero elements in order to both minimize complexity in the detection of the overall code sequence x(m) and also to mitigate the effects of multipath propagation. In the 3GPP specification 38.211, sequences of length 6,12,18 and 24 are proposed using vectors with elements in the quaternary set (QPSK modulation), ejπq/4,q=-3,-1,1,3.  The 3GPP sequences are augmented in the current proposal to include new minimum PAPR sequences of length 3 and 6 in an 8-ary set, ejπq/4,q=0,1,2,3,4,5,6,7, shown respectively in Tables 1 and 2. No two sequences in the tables are cyclic shifts of each other when transformed by inverse discrete Fourier transforms of order 3 (Table 1) and 6 (Table 2). The length 6 sequences have identical and lesser or equal PAPR than the 3GPP sequences (Table 5.2.2.2-1  in 38.211) which do not all have the same PAPR. We also consider the simplest case where the elements of rl(m) have only one non-zero element set to 1. This corresponds to a single resource element used among the 12K and results in the minimum complexity receiver for the overall code sequence. Furthermore, it provides the lowest PAPR (0dB) and could be used for extreme coverage scenarios.

| index | 8-ary sequence |
| --- | --- |
| 0 | (0,0,4) |
| 1 | (0,1,6) |
| 2 | (0,2,0) |
| 3 | (0,3,2) |
| 4 | (0,4,4) |
| 5 | (0,5,6) |
| 6 | (0,6,0) |
| 7 | (0,7,2) |

Table 2: Minimal-PAPR sequences of length 3 (PAPR 2.22 dB)

| Index | 8-ary sequence | Index | 8-ary sequence |
| --- | --- | --- | --- |
| 0 | (0,0,1,4,6,3) | 16 | (0,4,1,0,6,7) |
| 1 | (0,0,3,5,1,6) | 17 | (0,4,3,1,1,2) |
| 2 | (0,0,5,3,7,2) | 18 | (0,4,5,7,7,6) |
| 3 | (0,0,7,4,2,5) | 19 | (0,4,7,0,2,1) |
| 4 | (0,1,1,7,6,2) | 20 | (0,5,1,3,6,6) |
| 5 | (0,1,3,7,2,0) | 21 | (0,5,3,3,2,4) |
| 6 | (0,1,5,0,5,3) | 22 | (0,5,5,4,5,7) |
| 7 | (0,1,7,6,3,7) | 23 | (0,5,7,2,3,3) |
| 8 | (0,2,1,1,7,4) | 24 | (0,6,1,5,7,0) |
| 9 | (0,2,3,2,2,7) | 25 | (0,6,3,6,2,3) |
| 10 | (0,2,5,2,6,5) | 26 | (0,6,5,6,6,1) |
| 11 | (0,2,7,3,1,0) | 27 | (0,6,7,7,1,4) |
| 12 | (0,3,1,6,5,5) | 28 | (0,7,1,2,5,1) |
| 13 | (0,3,3,4,3,1) | 29 | (0,7,3,0,3,5) |
| 14 | (0,3,5,5,6,4) | 30 | (0,7,5,1,6,0) |
| 15 | (0,3,7,5,2,2) | 31 | (0,7,7,1,2,6) |

Table 3: Minimal-PAPR sequences of length 6 (PAPR 2.32 dB)

Let K0∈{1,3,6,12,24) be the number of non-zero carriers in the rl(m0) vectors. Although limited here to 24 dimensions, longer sequences can also be used if needed. The transmitted sequence in each OFDM symbol is indexed by two parameters (kl,0(m),kl,1(m)). The frequency offset is in the range kl,0(m)=0,1,…,12KK0-1 and corresponds to the first non-zero element in the vector. The parameter kl,1(m) corresponds to a cyclic-shift in the range kl,1(m)=0,1,…,K0-1. Let rl0 be the base sequence used for symbol l, the transmitted sequence is then given by

rl,k(m)=rl,k-k00ej2πkl,1(m)(k-K0)/K0kl,0(m)≤k<kl,0(m)+K00otherwise         (1)

In the simple case where K0=1, rl0=1 corresponding to a single non-zero resource element for symbol l.

The choice of (kl,0(m),kl,1(m)) should be dependent on the transmitted message but also scrambled by a pseudo-random sequence initialized by system parameters such as the cell identification number, user-dependent higher-layer configuration or user identification numbers. This will ensure both whitening of the spectral characteristics of the transmitted waveform as well as exploiting frequency diversity to fading and interference through frequency hopping via kl,0. For the purpose of this discussion, we can consider that

kl,0(m)=(k0m+kl,0')mod12KK0  ,kl,1(m)=(k1m+kl,1') mod K0

where kl,n' are the pseudo-random components that are symbol and system parameter dependent, while knm are the message dependent components.

## Signal Code

Two possibilities are considered in this proposal depending on the combinatorial configuration of the resource grid and number of bits B. In the case where 12KL≥2B, there are sufficient signaling dimensions to allow for orthogonal transmission and an L-dimensional DFT-based orthogonal code is used for the  wl(m) sequence. There are two sub-cases depending on the B and K0 described below.

When there are insufficient signaling dimensions to allow for orthogonal transmission we make use of an (L,B1) non-coherent code as described in [1] (first patent) to generate the  wl(m) sequence as described below.

In both cases we choose B0=log212K and without loss of generality assume that the B0 bits  correspond to the least-significant bits of the message  m . Below we specialize to the two individual cases.

### Orthogonal outer-code (12KL≥2B)

We consider two sub-cases.

- (Orthogonal A). The sub-case satisfies B0≤B≤B0+log2L the orthogonal outer code consists of any combination of 2B-B0 L-dimensional vectors from the set {ωL: ωl,n=ej2πln/L,l=0,1,…,L-1, n=0,1,…,L-1}. In this case, we set k0m=m mod 2B02log2K0, k1m= m mod 2B0mod K0 and wlm is selected as element m2-B0 from the chosen set.

- (Orthogonal B). For B>B0+log2L we set k0m=m mod 12K2log2K0, k1m=(m mod 12K) mod K0 and choose any combination of 2B12K  L-dimensional vectors from ωL. wlm is selected as element m12K from the chosen set.

Note that in the Orthogonal A sub-case, the message-dependent frequency-shift (k0m) is independent of the B1bits. In the Orthogonal B sub-case, the message-dependent frequency-shift (k0m) depends on the B1bits, but still allows for detection of the B0 bits independently of the B1 bits without ambiguity. In both cases, both unequal error protection and reduced-complexity decoding can be exploited, even if L and 12K are not powers of 2.

### Non-coherent code (12KL<2B)

In this case we set k0m=m mod 2B02log2K0 and choose wlm as codeword m2-B0 from the (L, B-B0) linear code in a binary or quaternary alphabet (BPSK/QPSK) . The codes are described in the standard way via their generator matrix parametrized by (L,B')

GL,B'= 100…0010⋯0001⋯0⋮⋮⋯⋱000001 g0,0⋯g0,L-B'-1g1,0…g1,L-B'-1g2,0…g2,L-B'-1⋮⋮⋮gB-1,0…gB'-1,L-B'-1

For BPSK the input, output and generator matrix elements are binary and addition and multiplication are assumed to be modulo 2. For QPSK, the input is binary but transformed to the integers modulo 4 and all required arithmetic operations (addition, subtraction and multiplication) along with the generator matrix elements are assumed to be modulo 4. The QPSK coded-modulation scheme is based on [2] and adapted to the current low-spectral efficiency scenario. For both BPSK and QPSK, the encoding process takes a B-bit information vector u and generates an N-dimensional coded vector, c, as c=u'G, where u'=u for BPSK. For QPSK and even values of B’ , u’(i)=2u(2i)+u2i+1. For QPSK and odd values of B’

u’i=2u2i+u2i+1, 0≤i<B'22u(2i)i=B'2

The codes are designed to minimize the pairwise correlations of the resulting sequences and can be found numerically by a reduced-complexity algorithm.

Similar to the orthogonal case, detection of the B0 bits is possible independently of the B1 bits without ambiguity and will thus allow for both reduced-complexity decoding and unequal error protection.

## Unequal Error Protection and Reduced-Complexity Receivers

Firstly by construction (when B0<B1), the error probability for the first B0 bits can be made lower, offering unequal error protection. It should be noted that this is purely an optional feature but can be used for reduced-complexity receivers. A two-step receiver, which first decodes B0 by non-coherently combining the L received symbols can significantly reduce computational complexity, especially in the case of the larger payload sizes.

Consider the general case of the detection problem

yl,p=Hl,prl(m)wlm+zl,p,l=0,…,L-1

Where Hl,p=diag(Hl,k,p) where Hl,k,p is the complex channel response for OFDM symbol l and subcarrier k on antenna port p. A near-optimal receiver for a completely unknown channel realization is

m=argmaxmp=0P-1l=0L-1yl,prlH(m)wl*m2                             (2)

This receiver is used in the simulations that will follow, both for the proposed scheme and the current Release 15 coding strategy.

Alternatively, we propose a simpler low-complexity receiver, which first detects the B0 bits and then using this hypothesis separately detects the remaining B1 bits.

Specifically, consider first the case B≤B0+log2L, corresponding to the Orthogonal A scheme or the non-coherent code for the non-orthogonal case. The simplified detector for the first B0 bits is

m0=argmaxm0=0,1,…,2B0-1p=0P-1l=0L-1yl,prlH(m0)2                           (3)

The remaining bits are then computed as

m1=argmaxm1=0,1,…,2B1-1p=0P-1l=0L-1yl,prlH(m0)wl*m12B02                                     (4)

and the overall hypothesis is m=m0+m12B0. From a complexity standpoint, in the first step, the number of complex multiplications  is P2B0LK0, while in the second it will be P2B1L. The total for P=1,K=1,L=14, K0=3, B0=3, B1=8 is 3920 complex multiplications. This should be compared to the full receiver in (2) which would require 344064 complex multiplications. Performance can be increased by creating a list of several hypotheses in addition to the one corresponding to the maximum in (2) at the expense of increasing the second term governing the complexity. For example, with two hypotheses, the total complexity would rise to 7504 complex multiplications.

For the Orthogonal B case the two-step detector is slightly modified as:

m0'=argmaxm0=0,1,…,12K-1p=0P-1l=0L-1yl,prlH(m0)2,   m0= m0'mod2B0                       (5)

m1=argmaxm1=0,1,…,2B1-1p=0P-1l=0L-1yl,prlH(m0')wl*m12B02                                     (6)

## Simulation Results and Discussion

We now provide some initial simulation results to show the benefit of the proposed techniques in comparison to Release-15 baseline configurations. These will be calibrated with reported simulations in ongoing 3GPP contributions. The simulations use the following assumptions: 14 OFDM symbols, 1 PRB,TDL-C NLOS channel model, 2 and 4 antenna configuration with independent and identically distributed realizations on the antenna ports (i.e. no correlation modelling). The simulations use a 2.6 GHz model with a 100 MHz BWP and 10Hz Doppler. Both the proposed schemes and the baseline use Frequency-hopping on opposite ends of the BWP. Rel-15 baseline comparison is PUCCH 3 with π/2-BPSK modulation and advanced receivers, namely non-coherent joint detection/estimation as in eq.(2), to allow the comparison to be as fair as possible.

### 4-bit transmission over 14 symbols (1 PRB)

The lowest PAPR transmission (0 dB PAPR) configuration uses 3-bits to select a single-RE sequence (K0=1) for  rl(m) and 1 bit to select the time-domain sequence wlm over 7 OFDM symbols.  With frequency-hopping, this 7 symbol signl is repeated in the second half of the of the slot.

In Figure 4 we see an improvement of 1-1.5 dB at 1% BLER. It should be noted that the 1% PAPR outage for this PUCCH 3 configuration is 4.78 dB. This corresponds to an overall improvement of the uplink link budget of around 6 dB for the lowest PAPR configuration. When using a PUCCH 0 type sequence (K0=12) the overall improvement would be reduced to 3.2-3.6 dB because of the increased PAPR.

Figure 4:  4-Bit Transmission, 14 Symbols, 1 PRB. BLER Comparison with Rel15 Waveform

### 11-bit transmission over 14 symbols (1PRB)

Figure 5: 11-bit Transmission, TDL-C NLOS 300ns delay-spread, 14 symbols , 1 PRB, frequency hopping

In this scenario we show the potential for using the proposed coding construction to provide unequal error-protection in addition to enhanced coverage. We employed the 3-bit low-PAPR sequence as in the previous case with K0=1 combined with the (7,8) signal code generated by

G=1000233010023300102330001012

The overall code sequence was obtained by using a permuted input to the same code for the second half-slot. In the current simulation we reversed the order of the systematic symbols which amounts to reversing the rows of G.

Similar to the 4-bit case, we see that the more highly protected 3 bits provide around 6 dB coverage gain compared  to the Rel 15 strategy when we include PAPR improvement. The overall performance exhibits around a 4.8 dB improvement when we include PAPR reduction. The BLER is virtually the same as the Rel15 code, which is not really surprising since they have identical maximum pairwise correlation magnitudes. In general, for other bit widths the BLER is either improved or at par with the current Rel-15 solution, so potential coverage gains will always be significant because of the PAPR reduction. To see this consider a LOS configuration (TDL-D with 30 ns delay-spread ) using the same system parameters aside from frequency hopping. Here the signal code is the (14,8) code

G=10001330120012010013302012010010302011102300011020102213

which yields around 0.5dB coding gain compared to PUCCH format 3 for around 5.3 dB PAPR improvement in the overall link budget.

Figure 6: 11-bit Transmission, TDL-D LOS 30ns delay-spread, 14 symbols, 1 PRB, without frequency hopping

# Concatenated Coding Strategy for Moderate Payload Sizes

binary code

sequence

selection

m=0,1,…,2B-1

rl(m0), l=0,1,…,L-1

binary interleaver

Figure 7 Non-coherent Serial concatenation of a binary code with low-PAPR sequences

In a similar fashion to the case of the strategies in Section 3 we consider a B-bit transmission with low-PAPR sequences but now as a serial concatenation of a binary code, and in the nominal configuration the 3GPP polar code, a binary interleaver and a mapping to a low-PAPR sequence. This scheme is depicted in Figure 7. Note that there are also no demodulation reference signals (DMRS) in this transmission strategy. The output is a similar to the earlier cases consisting of 12KL resource elements. The number of bits output from the binary code and interleaver is E=Llog212K. The binary code output is denoted by fi,i=0,1,…,E-1 and the binary interleaver output by gi,i=0,1,…,E-1. In the case of the 3GPP polar code (Section 5.3.1,38.212), the code includes the polar encoder code, polar rate-matching procedure and the final interleaving ([Section 5.4.2.2, 38.212]) converting N=2ncoded bits into E bits which are input to an additional interleaver prior to the sequence selection unit.  The additional interleaver should be chosen to ensure that more than one bit in a group of log212K adjacent coded bits do not jointly contribute to a particular low-PAPR sequence.The interleaver mapping for the sake of comparison with existing methods is

gi=fimodELL+iL/E                                   (7)

The mapping of coded bits to low-PAPR sequences is as follows. Let ml∈{0,1,…,2EL-1} be the E/L –bit representation of E/L adjacent output bits from the gi sequence the sequence in symbol l, namely gn+lEL, n=0,1,…,EL-1. We choose the low-PAPR sequence according to the same procedure as in eq.(1), with m replaced by ml.

Payload sizes will vary as a function of the chosen code rate of the polar code and rate matching parameters. If we bound the overall code rate of the binary code to around .5, we show the typical payload sizes (exluding the 6 or 11-bit CRC) along overall code rate in parentheses for nominal parameters L=14,28 and K=1,2,3,4,5,6 in Table 4

| K | L=14 | L=28 |
| --- | --- | --- |
| 1 | 15 (.125) | 31 (.125) |
| 2 | 17 (.083) | 45 (.083) |
| 3 | 24 (.069) | 59 (.069) |
| 6 | 31 (.042) | 73 (.042) |

Table 4 : Input bit length (including CRC) and overall spectral efficiency for rate .5 binary code

The choice of rate .5 is to guarantee a diversity order of 2 when using frequency-hopping. In order to allow for longer payloads, we admit the use of transmission over longer symbols periods (e.g. 2 slots) and more than one PRB.

A basic receiver for this strategy consists of a non-coherent correlator followed by a log-likelihood ratio (LLR) generator to feed to the channel decoder. One particular implementation of the correlator and LLR generation for the E/L bits in OFDM symbol l is

λl,n=maxm=0,1,…,2EL-1 s.t.  m2-n=1p=0P-1l=0L-1yl,prlH(m)2-maxm=0,1,…,2EL-1 s.t.  m2-n=0p=0P-1l=0L-1yl,prlH(m)2,l=0,1,…,L-1,n=0,1,…,EL-1  (7)

The E LLRs are deinterleaved by inverting (7) and fed to the channel decoder input in the conventional sense.

## Simulation Results

We show the performance for a 22-bit payload using the concatenated transmission strategy with K0=1 in Figure 8 where we see a 2-2.5 dB coding loss over the Release 15 PUCCH 3 transmission (π2 – BPSK) using the same LOS and NLOS configurations as in Section 3. Because of the PAPR reduction of 4.8 dB, the overall link budget improvement is therefore greater than 2 dB for 1% BLER. The concatenated scheme uses the LLR computation in eq.7 while that of the Release 15 strategy uses a quasi coherent receiver based on least-squares channel estimation derived from the DMRS symbols. The simulation reported here assumes only 2 DMRS symbols (i.e. one DMRS per hop). A more advanced non-coherent block receiver using the DMRS symbols similar to eq.7 could also be employed to compute the bit metrics, for instance by computing LLRs jointly across 7 symbols (in each frequency hop) separately for each resource element. This would come at the expense of a non-negligible complexity increase.

We are currently investigating similar DMRS-less techniques with higher coding gain at the expense of increased PAPR in order to determine whether overall coverage enhancements of more than 2dB can be attained similar to the cases described in earlier sections.

Figure 8: 22-bit payload, TDL-C NLOS 300ns delay-spread, 14 symbols , 1 PRB, frequency hopping

# Interference Mitigation and Multiple-Access

The transmission method described in this proposal has the added by-product of providing significant mitigation for inter-cell interference where similar PUCCH transmissions from adjacent cells are received by the gNodeB . This is the case when the transmission scheme is configured with a small number of non-zero elements (1,3,6) in the low-PAPR sequences and consequently many dimensions for frequency-hopping. If the cell-specific parameters provide randomization of the hopping sequence, interference from adjacent cells will not fall on the same frequency resources as the target transmission with high probability and will thus not greatly affect the probability of correct detection. Moreover, because of the bursty nature of the transmission scheme, it will also be tolerant to very high interference levels.  The same method could equally be applied for multiple-access within the same cell through appropriately chosen hopping patterns by the gNodeB. This would be similar to the orthogonal spreading sequence approach used in PUCCH format 4 in the current 5G NR specifications.

# Conclusion

This contribution discusses some potential channel coding technique for providing coverage enhancement for PUCCH for payload sizes less than 16 bits. For longer payload sizes we discuss a concatenated strategy which combines the Release 15 polar code with a low-PAPR sequence generator. The nominal configuration for low-PAPR uses a single resource element per OFDM symbol. The techniques are applicable with existing or new low-PAPR sequences designed for multiple non-zero resource elements per OFDM symbol.

In general, we believe that for coverage-limited scenarios short-block length codes should be designed for non-coherent detection and should not make use of DMRS in the transmitted waveforms. In particular, we make the following proposals:

Proposal 1: for appropriate payload sizes (less than 9 bits) and depending on the number of allocated PRBs for PUCCH, “DMRS-less” orthogonal signal sets should be used instead of the current short-block length coding methods combined with DMRS since there will ALWAYS be a non-negligible coding gain over the existing methods. These orthogonal signal sets should also be designed for low PAPR and low decoding complexity. Section 3 discusses one such orthogonal coding scheme which can achieve 0dB PAPR.

Proposal 2: for other payload sizes less than 17 bits where orthogonal transmission is impossible and depending on the number of allocated PRBs for PUCCH, non-orthogonal strategies should be devised combining low-PAPR sequences and structured codes for coding gain. These codes should have pairwise complex correlation magnitude as small as possible. Section 3 discusses such non-orthogonal schemes which can achieve 0 dB PAPR.

Proposal 3: Consider unequal error protection (UEP) for aggregated UCI payloads in the code design process. Section 3 discusses such UEP possibilities.

Proposal 4: For moderate payload sizes consider concatenated coded-modulation without DMRS which uses the existing Release 15 Polar Code mapped to low-PAPR sequences. Section 4 discusses one such scheme which provides significant coverage enhancement (more than 2 dB) over Release 15 techniques for PUCCH payload above 11 bits.

# Reference

- 3GPP 38.212, Multiplexing and Channel Coding, V16.2.0, June, 2020

- Knopp, R. and Leib, H. ,”M-ary Phase Coding for the Non-coherent AWGN Channel,” IEEE Transactions on Information Theory, vol. 40, no. 6, Nov. 1994.