3GPP TSG RAN WG1 #103-e														R1-2009554

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.12.3

Source: 			Moderator (BBC)

Title:	Summary # 3 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states

Document for:	Discussion and Decision

# Introduction

RAN1#103-e is the first meeting that discusses the AI 8.12.3 on Basic functions for broadcast/multicast for RRC_IDLE/ RRC_INACTIVE UEs. The information of the email thread on this topic and the check points on the discussion provided by RAN1 Chairman is shown below:

[103-e-NR-MBS-03] Email discussion/approval for basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs– David (BBC)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

A summary of the analysis and key issues identified from the technical inputs to this meeting to AI 8.12.3 can be found in R1-2009465 (Inbox).

The discussion on prioritisation of issues to be discussed at RAN1#103-e, with the most recent proposals, can be found in R1-2009553 (Inbox).

Section 2 includes (most stable) high priority issues for discussion at RAN1#103-e. For each issue the analysis from R1-2009465 is included from convenience. Each Issue includes Initial FL proposals where companies are welcomed to provide their inputs.

Section 3 includes (most stable) medium priority issues for discussion at RAN1#103-e, only after 2nd check point. For each issue the analysis from R1-2009465 is included from convenience.

Please use the “Navigation Pane” to quickly find the proposals and the different rounds of discussions in this document.

Section 4 will include any agreements reached from the discussions.

# Discussion on High Priority Issues

## Issue 1: Group scheduling for UEs in RRC_IDLE/ RRC_INACTIVE states

This issue has been discussed/mentioned in 7 tdocs (8 companies). At RAN1#102-e it was agreed that for RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. This issue addresses group scheduling for UEs in RRC_IDLE/ RRC_INACTIVE states.

In particular:

In [3, Chengdu TD Tech Ltd] for periodic scheduling, it proposed to support that the PDCCH with CRC scrambled with SPS G-RNTI is used to activate/de-activate the SPS PDSCH resource. For dynamic scheduling, it is proposed to support that the PDCCH with CRC scrambled with G-RNTI is transmitted with the allocated PDCCH resource and the PDSCH with G-RNTI used in the bit scrambling is transmitted with the allocated PDSCH resource.

[6, CMCC] proposes the support of group-common PDCCH based group scheduling for RRC_IDLE/ RRC_INACTIVE UEs for MBS service reception.

In [7, LGE] group scheduling of MBS data is discussed. Here it is discussed that a UE should be provided one or more search space sets in which the UE monitors PDCCH candidates to detect DCI with Group RNTI.

In [8, Samsung] it is proposed that high layer signalling delivers CORESET for multicast and group-specific RNTI.

In [12, Lenovo, MotoM] it is proposed to support a group-common DCI to schedule a group-common PDSCH for MBS for Idle/Inactive UEs.

[13, Intel] proposes that RRC_IDLE/INACTIVE UEs can be configured with a common MBS frequency resource and MBS CORESET within the common frequency resource by SIB where it can monitor the group common PDCCH within a CSS.

In [16, Ericsson] it is observed that for Idle/Inactive UEs to receive the RRC Connected multicast transmission they need to reuse the RRC connected configuration, e.g. using the same G-RNTI. UEs in Idle/Inactive mode could use this to monitor the G-RNTI-based PDCCH and get the scheduling of the G-RNTI-based PDSCH, using the same processing as Connected UEs.

In the contributions above, the 7 inputs (8 companies) propose and/or discuss the support of group-common PDCCH based group scheduling for UEs in RRC_IDLE/RRC_INACTIVE sates.

Based on the above the FL makes the following proposal:

### Initial FL proposal for Issue 1

Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| (Included by FL from Phase 1 discussions) Nokia | (Included by FL from Phase 1 discussions) - Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. - FFS: Whether a group-common PDSCH can be configured to be used by both idle and connected mode UEs simultaneously? |
| (Included by FL from Phase 1 discussions) Ericsson | (Included by FL from Phase 1 discussions) Regarding Issue 1 it would be good to clarify if the Idle/Inactive UEs may use the same G-RNTI as the one used by RRC connected UEs or a different G-RNTI. As we propose in our RAN1 contribution R1- 2009307 it should be possible to use the same transmission for UEs in all RRC states, provided they are configured in RRC Connected (this is to better support exceptional cases in RRC Connected with congestion). As also others have mentioned, we wish to point out that there is a dependency with the RAN2 about this topic. |
| Samsung | Support Proposal 1 above. |
| LG | We are fine with Proposal 1. The same PTM transmission scheme should be supported by a UE in any RRC state. |
| Apple | We are ok with Proposal 1. If the MBS service is intent for both idle and connected UE, e.g., broadcast service, the same RNTI is applied by idle and connected UE. |
| Huawei/HiSilicon | OK with the proposal and fine with the FFS added by Nokia. |
| Lenovo, Motorola Mobility | We are OK with FL proposal. FFS added by Nokia is also fine with us. |
| CATT | We are OK with the FL’s proposal 1. |
| ZTE | We are fine with proposal 1. We are also fine to clarify the FFS point added by Nokia above. |
| vivo | We are OK with the FL’s proposal 1. |
| Moderator | Thanks for the inputs. The FFS from Nokia has support from various previous inputs and also addresses comments from Ericsson. We could continue the discussion assuming the FFS can be included in the proposal. The Moderator was also wondering whether the FFS should also include the group-common PDCCH only for consistency with the text of the proposal as below: Proposal 1(under revision): For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. - FFS: Whether a group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected modes UEs simultaneously. |
| CMCC | We are OK with FL’s proposal 1. Regarding the FSS, RAN2 has made the agreements as the following in last Friday, For Rel-17, R2 specifies two modes: 1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD) 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions. R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS. From the agreements, we can see that the delivery modes are different for different RRC states, i.e., delivery mode 1 is for RRC_CONNECTED mode UE, and delivery mode 2 is for RRC_IDLE/INACTIVE UE. From this perspective, the group scheduling mechanisms may be different for RRC_CONNECTED state and RRC_IDLE/INACTIVE states. We are open to further discuss the FFS suggested by Nokia, but we think we should not add it at the current proposal, and this discussion is also related to RAN2 decision. |
| LG | We wonder if CMCC’s observation on the following RAN2 agreement is correct. 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). We think that ‘also’ means the UE in INACTIVE/IDLE as well as CONNECTED can receive MBS data. Thus, the same MBS data can be received by a UE in any state, depending on configuration and capability. UE in connected mode would not need to go to INACTIVE/IDLE in order to receive broadcast sessions. |
| NOKIA | The latest update of Proposal-1 (under revision) from FL is OK for us. |
| Qualcomm | Based on RAN2 latest agreement, mode 1 is for multicast services to be received by CONN UEs only. The mode 2 is for broadcast services to be applied to UEs in any states. Therefore, we suggest: Proposal 1(under revision): For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. - For broadcast services, same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected UEs. |
| Futurewei | OK with the proposal |
| Moderator | Thank you for the inputs. Thank you also CMCC for shading the latest RAN2 agreements. Based on the inputs above, the Initial FL proposal for Issue 1, i.e. “Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.” seems to be acceptable by all the inputs above. During the discussion above, a potential FFS included as sub bullet of Proposal 1 has been discussed with the following form: FFS: whether a group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected modes UEs simultaneously. Building on top of the discussion on the FFS above, CMCC has shared with the group the following RAN2 agreements: For Rel-17, R2 specifies two modes: 1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD) 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions. R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS. However, there are concerns on the FFS above by one company on two aspects: - whether using the same group-common RNTI for both idle/inactive and connected UE states is a RAN2 discussion; and, - interpretations from different companies of the latest RAN2 agreements. One view is that broadcast sessions can be received by both idle/inactive and connected UEs and another view is that broadcast sessions can only be received by idle/inactive UEs. Since the disagreement between companies mainly lays with the FFS and on the interpretations on the RAN2 agreements on broadcast support, the FL proposes to remove the FFS (or the modification proposed by Qualcomm) from Proposal 1 and move this discussion on the contents of the FFS to Issue 3 (Broadcast support). Therefore, Proposal 1 would be unmodified compared to the Initial FL proposal for Issue 1. |

### Second round FL proposal for Issue 1 [unchanged]

Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

Please use the table below to indicate whether your company has any concerns about the proposal above, or any other views you may want to make:

| company | comments |
| --- | --- |
| Intel | We are ok with FL proposal. We agree with LG that RAN2 agreement implies that mode 2 can be received by both CONNECTED and IDLE/INACTIVE UEs. |
| Huawei/HiSilicon | Ok with the proposal. |
| Lenovo, Motorola Mobility | We are OK with this proposal. |
| Moderator | This proposal has been discussed and agreed with the minor modification of the FFS at the online GTW session on Tuesday 10 November 2020. Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. - FFS details The discussion in this Issue 1 for this meeting is therefore finished. Thank you all for your contributions. |

## Issue 3: Broadcast support by UEs in RRC_IDLE/RRC_INACTIVE state

This issue has been discussed/mentioned in 5 tdocs (7 companies). The inputs argue whether both multicast and broadcast type of services should supported/considered for Ues in RRC_IDLE/RRC_INACTIVE states.

In particular:

[5, CATT, CBN] proposes that it is necessary for RRC_IDLE/RRC_INACTIVE Ues to receive MBS services in NR at least for broadcast service.

In [11, Nokia, NSB] first discusses that supporting broadcast only in RRC_IDLE/INACTIVE mode or support multicast only in RRC_CONNECTED mode may simplify the specification work on RAN2 design a lot by considering the limited working timeframe in Rel-17 MBS. Based on this in [11] it is observed that it is still not clear whether there is the need to support both broadcast and multicast services in RRC_IDLE/INACTIVE mode.

In [14, Convida] two proposal are done.  The first proposal supports using broadcast to deliver PTM transmission in RRC_INACTIVE state and in RRC_IDLE, while the second proposes that further study is needed to evaluate the benefits of using multicast to deliver PTM transmission in RRC_INACTIVE state and in RRC_IDLE state.

[15, Qualcomm] makes two proposals. For the first proposal, multicast reception with high reliable QoS requirement is not supported for Ues in RRC_IDLE/RRC_INACTIVE states. For the second proposal, broadcast reception with no specific QoS requirement can be supported for Ues in RRC_IDLE/INACTIVE/CONNECTED states.

[16, Ericsson] proposes to study the reception of the same multicast service by Ues in all RRC states for the case when Ues have first been configured in RRC Connected.

In the inputs above for Ues in RRC_IDLE/ RRC_INACTIVE states: there are 3 inputs (4 companies) that support broadcast reception, there is 1 input (1 company) that proposes that multicast reception is not supported, and there are 3 inputs (3 companies) that either ask whether multicast reception is needed or propose that further studies are done for the reception of multicast.

Based on the above the FL makes the following initial proposals.

### Initial FL proposals for Issue 3

Proposal 3: broadcast service reception can be supported for Ues in RRC_IDLE/INACTIVE/CONNECTED states.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Samsung | Support Proposal 3 above. |
| LG | We are fine with Proposal 3. |
| Apple | We are fine with Proposal 3. |
| Huawei/HiSilicon | Fine with the proposal in general. However, it is unclear what RAN1 impact is per this proposal. |
| Lenovo, Motorola Mobility | We are OK with FL proposal. |
| CATT | We are OK with the proposal 3. |
| ZTE | We are fine with Proposal 3. |
| Vivo | We are fine with Proposal 3. |
| Moderator | Thank you for the inputs. @Huawei/HiSilicon, do you think we should add some further clarifying text for this proposal? Thanks. |
| CMCC | Not support. First we think the definition of broadcast service is in application layer, we only discus broadcast session and multicast session in RAN. Second, RAN2 and has achieved the following agreements in last Friday. For Rel-17, R2 specifies two modes: 1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD) 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions. R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS. As the comment in proposal 1, delivery modes are different for different RRC states, i.e., delivery mode 1 is for RRC_CONNECTED mode UE, and delivery mode 2 is for RRC_IDLE/INACTIVE UE. RAN2 also assumes the delivery mode 2 is used for broadcast session, that means RRC_IDLE/INACTIVE Ues can receive broadcast session, which has the same meaning of broadcast service reception can be supported for Ues in RRC_IDLE/INACTIVE states in the proposal. As for RRC_CONNECTED state, from current RAN2 agreements, the delivery mode 1 is used only for multicast sessions, but cannot support broadcast session i.e., broadcast service. Therefore, we think this proposal has been discussed in RAN2. In addition, the leader group of this sub-agenda is RAN2, we should not waste time on the discussion on the same topic in RAN1. |
| LG | We are generally agree that RAN2 should make decision on this issue. However, we still wonder if the RAN2 agreement prevents connected Ues to receive broadcast sessions. |
| NOKIA | We are fine with Proposal 3 |
| Qualcomm | We support Proposal 3. The RAN2 agreement is clear that the delivery mode is different for multicast sessions and broadcast sessions. For broadcast sessions, same delivery mode 2 can be applied to Ues in any states. We think RAN1 can assume Proposal 3 is aligned with RAN2 agreement. |
| Futurewei | OK with the proposal. |
| Moderator | Thank you all for the inputs. Although there has been wide support on the Initial FL proposal for Issue 3 (11 inputs 13 companies) there are concerns from one company. Similarly to the discussion on Initial FL proposal for Issue 1, there disagreements mainly are on the interpretations on RAN2 agreements. To try to address the concerns, the FL proposes the following revision to Proposal 3, taking also into account the comment received in the email reflector from Huawei/HiSilicon: Proposal 3-rev1: RAN1 assumes that broadcast sessions can be supported by Ues in RRC_IDLE/INACTIVE/CONNECTED states. - same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected Ues The text above includes the word “assumes” to be aligned with RAN2 agreements and also includes some text from Qualcomm from Issue 1 discussion. This new text tries to address the company’s concerns while giving further guidance for more discussions at next RAN1 meeting. |

### Second round FL proposals for Issue 3

Proposal 3-rev1: RAN1 assumes that broadcast sessions can be supported by Ues in RRC_IDLE/INACTIVE/CONNECTED states.

- same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected Ues.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Intel | From RAN1 perspective, we think multicast or broadcast mode can be transparent i.e., RAN2 service modes are enough. It is clear from RAN2 agreement that both CONNECTED and IDLE/INACTIVE Ues can receive mode 2 transmissions (brodcast). If transparent, the main bullet may not be necessary for RAN1 agreement. The sub-bullet can be further discussed and should be the main proposal in this section. |
| Huawei/HiSilicon | We may not need the main bullet and the sub-bullet for this proposal might be sufficient. |
| CMCC | As the comment in 1st round, we don’t need this proposal in RAN1 and it’s up to RAN2’s decision. As the sub-bullet, the agreement about proposal 1 in GTW has covered this issue. Therefore, we don’t need to discuss this issue any more. |
| Lenovo, Motorola Mobility | The main bullet seems not necessary. For the sub-bullet, we made some slight revisions for clearer. Same group-common PDCCH/PDSCH can be configured to be used transmitted to by both idle/inactive and connected UEs. |
| Moderator | Thank you again for your contributions. It seems that there is a sustained concern by one company on discussing this Issue 3 on Broadcast support. At the GTW on Tuesday the 10th of November, it was also mentioned whether it could still be possible to discuss the use of the same group-common PDCCH/PDSCH by both idle/inactive and connected UEs. The discussion was wrap up as part of the “FFS details”. At this point it seems to the FL that, given that this discussion has already highlighted the RAN2 agreements on this subject in RAN1, it would therefore seem more productive to postpone this discussion to other meetings where further contributions could be submitted to the next RAN1 meetings. The FL would like to hear comments from other companies whether there are strong views we should still work towards an agreement for a study on same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected UEs. |
| Qualcomm | Based on RAN2 agreement we can discuss the sub-bullet, which is more related with RAN1 issues. We suggest the following changes: Proposal 3-rev1: - For broadcast reception, same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected UEs. |
| NOKIA,NSB | We are fine with the above proposal from the FL to postpone this discussion For the sub-bullet, we have the following revise proposal - Same group-common PDCCH/PDSCH can be configured to be used accessed by both idle/inactive and connected UEs for broadcast reception. |
| Convida | In broadcast, we think that the idle/inactive UEs and the connected UEs may share the same group-common PDCCH/PDSCH in some use cases. However, mandating such behaviour seems very restrictive as the idle/inactive UE can only operate with limited frequency resource. It is not clear to us what is the intention of the discussion here. Some clarifications would be appreciated. |
| Moderator | Thank you for the inputs. @ Convida, thank you for the comments. Regarding the text in Proposal 3-rev1, the FL’s interpretation is that because the proposal has the wording “assumes” (aligned to RAN2 wording) this would serve as a working assumption to RAN1. The motivation to agree on such a proposal would be to provide some scope for technical discussion at next RAN1 meetings. It is also FL’s interpretation that current wording of the sub-bullet (i.e. same group-common PDCCH/PDSCH can be configured to be used by both idle/inactive and connected UEs) would not preclude other configurations (i.e. it can be configured). However, given the inputs from other companies this proposal will be changed as below. Based on the inputs above, there are different views on whether we should or should not continue discussing this issue in this RAN1 meeting. The FL thinks that it would be good to hear the comments from more companies. However, also based on the comments above, Proposal3-rev1 is modified as follows to include the comments that the proposal can focus only on the sub-bullet that deals with RAN1 issues and some wording suggestions. Proposal 3-rev2: RAN1 assumes that for broadcast reception, the same group-common PDCCH/PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. |

### Third round FL proposals for Issue 3

Proposal 3-rev2: RAN1 assumes that for broadcast reception, the same group-common PDCCH/PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs.

Please use the table below to provide your company’s views on Proposal 3-rev2 and/or on whether this discussion should be postponed to next meetings:

| company | comments |
| --- | --- |
| Intel | While we understand the intent of this proposal, the meaning of broadcast reception from RAN1 perspective is unclear for us. In our understanding, this should be transparent. Therefore, we suggest the following: Proposal 3-rev2: RAN1 assumes that for broadcast reception, the same group-common PDCCH and the corresponding scheduled PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. |
| LG | We are fine with Proposal 3-rev2. We already used ‘multicast’ in other agreements. Thus, it seems fine to use ‘broadcast reception’. In addition, we suggest to add ‘at least for PTM transmission scheme 1’. |
| vivo | We are generally fine with Proposal 3-rev2. |
| ZTE | We are fine with the FL proposal. |
| CMCC | There are two cases about the relationship between UE-specific BWP of RRC_CONNECTED UE and the common frequency resource for RRC_IDLE/INACTIVE UEs: - Case1: The UE-specific BWP of RRC_CONNECTED UE contains the common frequency resource for RRC_IDLE/INACTIVE UEs In this case, we think the same group-common PDCCH/PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. - Case 2: The UE-specific BWP of RRC_CONNECTED UE not contains or only partial overlaps with the common frequency resource for RRC_IDLE/INACTIVE UEs There two options for gNB transmitting the group-common PDCCH/PDSCH. - Opt 1. gNB transmit two group-common PDCCH/PDSCHs for the same broadcast session/service, which one is in the UE-specific BWP of RRC_CONNECTED UE, another is in the common frequency resource for RRC_IDLE/INACTIVE UEs. - Opt 2. gNB only transmit one group-common PDCCH/PDSCH in the common frequency resource for RRC_IDLE/INACTIVE UEs, but RRC_CONNECTED UE need BWP switch to receive the broadcast session/service. Therefore, we want to clarify in case 2, the proposal 3-rev2 is required gNB can only take Opt 1 or it’s up to gNB’s implementation to adopt both Opt 1 or Opt 2. |
| Apple | We are generally fine with this proposal. For CMCC’s question, our understanding is Opt2 is more sensible. We are open to discuss two options. |
| CATT | We are generally OK with proposal 3-rev2 by FL. The modification proposed by Intel also makes the proposal clearer. But we do not see the necessary to add “at least for PTM transmission scheme 1”, since this proposal is not related to transmission scheme. Furthermore, the appropriate transmission scheme will be used whenever it is determined after discussion in another AI 8.12.1. |
| Samsung | Agree. |
| Spreadtrum | We are generally fine with Proposal 3-rev2. |
| Lenovo, Motorola Mobility | We are fine with the FL proposal. |
| Huawei/HiSilicon | Suggests changing to the following: Proposal 3-rev3: From physical layer perspective, RAN1 assumes that for broadcast reception, the same group-common PDCCH and the corresponding scheduled PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. When we say “can be”, it basically means one implementation possibility. Hence, to CMCC’s question, in my understanding, for this particular implementation possibility, maybe only case 1 make more sense. |
| Ericsson | We agree with the FL proposal. |
| NOKIA, NSB | We are fine with the current proposal from FL. |
| Convida | @ Moderator: Thank you for the explanation. We are fine with the proposal if the intention is not to mandate this behaviour. As the current wording is ‘the same group-common PDCCH/PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs’, it is not clear to us. We would like to clarify whether the behaviour is mandated or not. We are OK with the proposal if the behaviour is not mandated. |
| Qualcomm | We prefer to keep ‘for broadcast’ here to differentiate it with multicast for RRC_CONN UEs (discussed in 8.12.1). Our understanding of this agreement is to maximize the commonality between CONN states and IDLE/INACTIVE states for broadcast reception based on WI guideline. Regarding CMCC’s questions, we think case 1 is the main use case. If case 2 happens to some UE, Option 2 can work. Whether to use option 1 or unicast PDCCH/PDSCH for the CONN UE can be further discussed. |
| Moderator | Thank you for the inputs. The FL interpretation of the following RAN2 agreements (copied from CMCC response to previous rounds) for delivery mode 2 used for broadcast sessions are working assumptions. For Rel-17, R2 specifies two modes: 1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD) 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions. R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS. Hence, The FL proposes that the we work towards a working assumption. The following revision of Proposal 3-rev2 tries to include the inputs in this round of discussion. Proposal 3-rev3: (Working assumption) From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. - FFS: the scenario when the UE-specific BWP of RRC_CONNECTED UE does not contain (or it only partially overlaps) with the common frequency resource of the group-common PDCCH/PDSCH of RRC_IDLE/INACTIVE UEs. @Intel, thank you for the input. Your clarifications have been included, but the term broadcast reception has been maintained as per comments from other companies. @LG, thank you for the input. We keep the term “broadcast reception” but I have not included your suggestion to add “PTM transmission scheme 1” due to concerns from other companies. @CMCC, thank you for the input. To address your comment, the FFS on case 2 scenario has been included to allow companies to discuss different options at the next RAN1 meetings. @CATT, thank you for the input. your suggestions have been taking into account. @Huawei/HiSilicon, thank you for the input. your suggestions have been included. @Convida, thank you for the input. As per Huawei/HiSilicon comments, the FL interpretation is that “can be” is one implementation possibility. Furthermore, the proposal now also includes the working assumption status. @Qualcomm, thank you for the input. Your suggestion to keep the term broadcast has been included. |

### Fourth round FL proposals for Issue 3

Proposal 3-rev3: (Working assumption) From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled PDSCH can be transmitted to both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs.

- FFS: the scenario when the UE-specific BWP of RRC_CONNECTED UE does not contain (or it only partially overlaps) with the common frequency resource of the group-common PDCCH/PDSCH of RRC_IDLE/INACTIVE UEs.

Please use the table below to provide your company’s views on Proposal 3-rev3:

| company | comments |
| --- | --- |
| ZTE | We are generally fine with the proposal. But it seems the current proposal is formulated from the network’s perspective, is it possible to formulate it from the UE’s perspective like below since the spec is written from UE’s perspective? Proposal 3-rev3: (Working assumption) From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled PDSCH can be received by both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED UEs. - FFS: the scenario when the UE-specific BWP of RRC_CONNECTED UE does not contain (or it only partially overlaps) with the common frequency resource of the group-common PDCCH/PDSCH of RRC_IDLE/INACTIVE UEs. |
| Moderator | A revision of Proposal 3-rev3 has been agreed at online GTW session. Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs. - FFS details. The discussion of this issue in this RAN1#103-e meeting is therefore finished. |

## Issue 4: MBS frequency resources of UEs in RRC_IDLE/ RRC_INACTIVE states

This issue has been discussed in 14 tdocs (17 companies). This issue addresses the MBS frequency resource for Ues in RRC_IDLE/ RRC_INACTIVE states.

In particular:

[2, Huawei, HiSilicon] proposes that for broadcast (SIB and/or MCCH) based mechanism, the frequency resource of MBS for UE in RRC_IDLE/ RRC_INACTIVE states can be the initial BWP or configured via SIB or MCCH.

In [3, Chengdu TD Tech Ltd] it is first proposed that the frequency range [F1, F2] with the initial DL BWP contained can be set to provide the MBS for II-Ues (II-Ues stand for Ues in RRC_IDLE/ RRC_INACTIVE states). The frequency range [F1, F2] can be divided into several II-UE BWPs to lower the power consumption of II-Ues with each II-UE BWP containing the initial DL BWP. [13] also proposes that one or several secondary II-UE BWPs can be configured outside the frequency range [F1, F2] to meet the bandwidth requirement for MBS for II-UE. Each secondary II-UE BWP can not only provide the MBS for II-Ues, but they can also act as the initial DL BWP.

[4, VIVO] proposes for MBS transmission bandwidth part, the following cases are supported for Rel-17 MBS. Case 1: The MBS transmission is included within the initial BWP. Case 2: The initial BWP is included within the MBS transmission bandwidth.

[5, CATT, CBN] states that for RRC_IDLE/RRC_INACTIVE Ues, the CORESET#0 or the initial BWP can be configured into the mixed BWP.  [5] also suggests that each UE should only support one common frequency resource to simplify the operation.

[6, CMCC] discusses that if Rel-17 MBS service provided for RRC_IDLE/RRC_INACTIVE UE(s) is still in initial BWP, the bandwidth may be not enough, especially considering SSB, CORESET 0 and PDSCH associated with SI-RNTI have occupied many resources of initial BWP. Therefore, [6] proposes to further study whether to support a larger BWP used for MBS service reception for RRC_IDLE/RRC_INACTIVE Ues.

In [7, LGE] it is proposed to support MBS transmission/reception based on a new MBS specific BWP as well as the initial/active BWP.

[8, Samsung] discusses that to allow enough resources for multicast, larger bandwidth is allocated for multicast BWP. Not to have BWP switching between multicast BWP and initial BWP, it is better to have CORESET#0 included in multicast BWP.  [8] makes the proposal that a separate BWP is configured for multicast.

[10, ZTE] proposes that NR MBS supports configuring a MBS-specific DL BWP for MBS transmission for RRC_IDLE/INACTIVE Ues via SIBx or multicast control information (e.g., carried on MCCH).

[11, Nokia, NSB] first proposes to discuss if there is enough capacity for SC-MCCH in the initial BWP for RRC_IDLE/INACTIVE Ues. [11] also observes that for avoiding BWP switching, the SC-MTCH (i.e. group common PDSCH) and SC-MCCH shall be provided in the same BWP. The final proposal in [11] is to discuss how to support BWP operation for RRC_IDLE/INACTIVE Ues.

[12, Lenovo, MotoM] first proposes that the group-common DCI is transmitted in CORESET 0 and the group-common PDSCH for MBS is transmitted within initial DL BWP. [12] further provides details on RB numbering and the payload size of frequency domain resource assignment (FDRA) indicator in the DCI.

[13, Intel] proposes that RRC_IDLE/INACTIVE Ues can be configured with a common MBS frequency resource and MBS CORESET within the common frequency resource by SIB where it can monitor the group common PDCCH within a CSS.

[14, Convida] states that in unicast scenario, the initial BWP is used in RRC idle/inactive to support paging delivery, system information delivery, etc. However, if we further consider supporting MBS, the initial BWP may be not enough anymore. Therefore [14] proposes that enhancements on BWP operation are needed to support MBS for Ues in RRC_IDLE/RRC_INACTIVE states.

[15, Qualcomm] proposes that the broadcast BWP can be same as initial BWP by default. FFS more than one Broadcast BWP and/or different Broadcast BWP for MCCH/MTCH(s).

In [16, Ericsson] it first observed that in RRC Idle/Inactive the UE is required to monitor signalling, such as SIB1 and paging, in the initial BWP. [16] also observes that to avoid BWP switching in RRC Idle/Inactive, when receiving MBS, an RRC Idle/Inactive UE may be configured with a single BWP. Finally, [16] also observes that when an RRC Idle/Inactive UE is configured with a second BWP (may be same as for unicast), different from the initial BWP, and the initial BWP is a subset of that second BWP, the UE may both receive MBS data and monitor the initial BWP ignalling without BWP switching.

In the inputs above regarding the common frequency resource to receive MBS data for Ues in RRC_IDLE/ RRC_INACTIVE states: 7 inputs (10 companies) support that it can be the initial BWP, 8 inputs (9 companies) support to define a specific MBS BWP, 4 inputs (4 companies) discuss that a specific BWP containing the initial BWP can avoid BWP switching, while 3 inputs (4 companies) propose/suggest to study enhancements to the initial BWP.

Based on the above the FL makes the following initial proposals.

### Initial FL proposals for Issue 4

Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the initial BWP can be the default common frequency resource to receive MBS data.

- FFS: one/more MBS specific BWPs where to avoid BWP switching the MBS specific BWP(s) can contain the initial BWP.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| (Included by FL from Phase 1 discussions) Nokia | (Included by FL from Phase 1 discussions) - For RRC_IDLE/RRC_INACTIVE Ues, if the initial BWP is considered as the default common frequency resource to receive MBS data, discuss whether there is enough capacity in initial BWP to contain MBS related MCCH and/or MTCH traffic - If there is NOT enough capacity in initial BWP for RRC_IDLE/INACTIVE UEs identified - FFS: How to handle the issue, i.e. one/more MBS specific BWPs where to avoid BWP switching the MBS specific BWP(s) can contain the initial BWP. |
| (Included by FL from Phase 1 discussions) Intel | (Included by FL from Phase 1 discussions) On issue 4, there can be some dependence on the discussion for RRC_CONNECTED mode Ues i.e., if BWP framework is reused or a so-called MBS frequency region (as discussed in R1-2009000) can be defined which requires lesser configuration than BWP. In this case, the RRC_IDLE/INACTIVE Ues may not be confined to only the initial active BWP and can be switched to the common MBS frequency resource via SIB. Furthermore, we tend to agree with the observation from Nokia above, that there may be need for wide BW compared to initial BWP. But the flexibility of switching by configuration can be left up to the network. Additionally, based on the common frequency resource design for RRC_CONNECTED Ues, it may be possible to have similar design for the IDLE/INACTIVE case as well. Therefore, we prefer the following wording: - Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the initial BWP can be the default common frequency resource to receive MBS data. - FFS: one/more MBS specific common frequency resource(s) can be configured which may or may not contain the initial BWP. - FFS: configuration details of common frequency resource when it is different from initial BWP |
| Samsung | It seems that what “default” means should be clarified. Also, we suggest to change “MBS data” to “PDSCH for MBS”. |
| LG | We also prefer to clarify the meaning of ‘default’ which, in our view, means that UE receives group common PDCCH/PDSCH in the initial BWP unless any MBS specific BWP is configured. We suggest to change ‘MBS data’ to ‘group common PDCCH/PDSCH’. We prefer to change the FFS to: “FFS: one or more MBS specific BWPs which may or may not contain the initial BWP” |
| Apple | If the capacity of the initial BWP is the concern already, it’s pre-mature to set it as default BWP. |
| Huawei/HiSilicon | The bandwidth of initial BWP can be configured to be larger than CORESET0. The revision from Intel can be ok in general, but we prefer the resource for IDLE/INACTIVE covers the resource for CORESET0, so as not to switch the BWP when receiving SIB/Paging and MBS in IDLE/INACTIVE. |
| Lenovo, Motorola Mobility | MBS specific BWP is still in discussion. We prefer to add below FFS: - Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the initial BWP can be the default common frequency resource to receive MBS data. - FFS: one/more MBS specific common frequency resource(s) can be configured. - FFS: configuration details of common frequency resource |
| CATT | We have few clarification/comments for this proposal. - The main bullet is supportive. It is the simplest method for IDLE Ues receiving broadcast services without any specific configurations on the frequency resource in addition of initial BWP. The main bullet can be explained by case 1a in the figure below (Please correct me if my understanding is not correct). - For the FFS in sub-bullet: the current FFS is understood like Case 1b in the figure below, which means more than one MBS frequency resources can contain the initial BWP. However, we are not supporting this mechanism, since we are not sure how it consumes standardization/specification effort, and it is not a simple way. - The most general way is Case 1c that is shown in the figure below. Besides the main bullet, a MBS common frequency resource can be configured within the initial BWP for broadcast service reception, which is similar with the design principle in AI 8.12.1 of RRC_CONNECTED group scheduling. Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the initial BWP can be the default common frequency resource to receive MBS data. FFS: one MBS specific common frequency resource(s) where to avoid BWP switching the MBS specific BWP(s) can be contained the initial BWP. |
| ZTE | We suggest the following update for clarifying the meaning of ‘default’, Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, MBS specific BWP can be configured for receiving PDCCH/PDSCH for MBS. If MBS specific BWP is not configured, the initial BWP can be the default common frequency resource to receive MBS dataPDCCH/PDSCH for MBS. - FFS: the MBS specific BWP(s) contain the initial BWP. - FFS: configuring one/more than one MBS specific BWPs where to avoid BWP switching the MBS specific BWP(s) can contain the initial BWP. |
| Vivo | We prefer to use same terminology, i.e., MBS frequency region as that in the discussion for RRC_CONNECTED Ues, which can reuse the BWP framework or not. We agree with Apple’s comment that it’s pre-mature to set the initial BWP as default BWP. |
| CMCC | We are generally fine with the proposal, but whether to define MBS specific BWP (option 2A) or a MBS common frequency resource (option 2B) for common frequency resource for group-common PDCCH/PDSCH is still discussing in AI 8.12.1, therefore, we suggest use the same definition as in RRC_CONNECTED mode. Regarding the FFS, generally we think the common frequency resource should contain the initial BWP to avoid the BWP switching: Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the initial BWP can be the default common frequency resource for group-common PDCCH / PDSCH to receive MBS data. FFS: one/more common frequency resources which contain initial BWP MBS specific BWPs where to avoid BWP switching between common frequency resources and the MBS specific BWP(s) can contain the initial BWP. |
| Qualcomm | Several comments: - The ‘default’ means what the IDLE/INACTIVE UE may assume if there is no explicit configuration. - For ‘MBS data’, we prefer a more general wording to consider MBS control/data on PDSCH. If Proposal 1 is agreed, we can use GC-PDCCH/PDSCH. - We are fine to use common frequency resource before deciding Opt2A and Opt2B. - For IDLE/INACTIVE Ues, there is no concept of BWP switching. On top of CMCC’s, we suggest changing Proposal 5 as: Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH / PDSCH to receive MBS data. FFS: whether to configure one/more common frequency resources which contain initial BWP MBS specific BWPs. |
| Futurewei | We are OK with proposal and above suggestions from CMCC/QC. Proposal 5: for RRC_IDLE/RRC_INACTIVE Ues, the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH / PDSCH to receive MBS data. But no need for FFS bullet now, we can come back to it later. |
| Moderator | Thank you for your inputs. There are concerns from companies on whether there will be enough capacity in the initial BWP. However, other companies discuss that the initial BWP can be configured to be larger than CORESET0 that should remove the issue. There were also questions on clarifying the meaning of default common frequency resource, which has been clarified by some companies. Companies have also discussed that using the same terminology as for the connected state case is preferred. The FL proposes the following revision 1 to Proposal 5. Proposal 5-rev1: For RRC_IDLE/RRC_INACTIVE Ues, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: whether to configure one/more common frequency resources which may or may not contain the initial BWP - FFS: configuration details of common frequency resource when it is different from initial BWP |

### Second round FL proposals for Issue 4

Proposal 5-rev1: For RRC_IDLE/RRC_INACTIVE Ues, define/configure common frequency resource for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: whether to configure one/more common frequency resources which may or may not contain the initial BWP

- FFS: configuration details of common frequency resource when it is different from initial BWP

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Intel | Support this proposal. It’s a good way forward to address concerns raised so far. |
| Huawei/HiSilicon | Regarding the first FFS, the common frequency resources if configured should contain the initial BWP to avoid BWP switching and don’t see the scenario of the common frequency resource does not contain the initial BWP. |
| CATT | The updated version is still not quite clear to us, so I would like to ask for clarification on the current version of proposal 5-rev1 for better understanding. - For the first sub-bullet: UE is using initial BWP (including all the configurations for initial BWP) to receive group-common PDCCH/PDSCH. Is this correct understanding? Figure Case 1a as below. - For the second sub-bullet: if my understanding is correct in the figure Case 1b below, then may I ask the technical reason why we should have this FFS? Because from my understanding on this FFS, it states all of the configuration possibilities of relationship between MBS frequency resource and initial BWP, which means there is no necessary to add such a bullet to say FFS everything. - What’s more, our comments/proposal in case 1c is not addressed in the updated proposal 5. We would like to add another FFS to include case 1c. - For the third sub-bullet, what does “different from initial BWP” mean? Does it mean the MBS common frequency resource is configured confined in the initial BWP but with different configurations (e.g. parameters) like Case 1c below, or does it mean MBS frequency resource is a separate configuration without any overlap on frequency domain, like Case 1d? We would like to suggest updating the proposal as: Proposal 5-rev1: For RRC_IDLE/RRC_INACTIVE Ues, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: whether to configure one/more common frequency resources which may or may not contain the initial BWP - FFS: configuration details of common frequency resource when it is different from initial BWP - FFS: whether to configure one/more common frequency resources confined within the initial BWP |
| CMCC | Similar view as Huawei, we don’t see the motivation to define the common frequency resource not contain the initial BWP. |
| Lenovo, Motorola Mobility | Generally it is fine with us. One following comment from our side is if the specific common frequency resource is configured, should it be confined within the initial BWP? |
| Moderator | Thank you for your inputs. Some clarifications from FL side. @Huawei/HiSilicon and CMCC, the first FFS (whether to configure one/more common frequency resources which may or may not contain the initial BWP) including the possibility or not containing the initial BWP was to include the explicit comments on this from Intel and LG form previous round of discussion. However, given the tdocs submitted to this meeting and the inputs from the previous round, it seems that there is more support that the common frequency resources if configured containing the initial BWP. @LG and Intel, do you have strong views on whether the FFS should include the possibility that the common frequency resource is configured not containing the initial BWP? @CATT (thank you for providing the figures!) - regarding question 1: this is also FL’s understanding. - regarding question 2: - your figure 1b provided in this round of discussion represents the situation in the first FFS (whether to configure one/more common frequency resources which may or may not contain the initial BWP). - regarding your proposal on figure 1c, one question, would case 1c be a subset of case 1a? My understanding was that if the common frequency resource was configured as the initial BWP, the specific resource blocks scheduled for transmission within the initial BWP would be up to the network implementation. Please do let me know if my understanding is correct. - regarding your question 3: FL understanding is that this would address the situation of Case 1b, in your figures above in this round of discussion. @Lenovo, Motorola Mobility: I think the previous discussion also touches on your question on this round. @All: assuming the following frequency resource configurations in the figure below (which I borrow from CATT – thanks!): - The first sub bullet in Proposal 5-rev1 (the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.) is represented by Case 1a in the figure above. - The second sub bullet in Proposal 5-rev1 (FFS: whether to configure one/more common frequency resources which may or may not contain the initial BWP) could be changed to (FFS: whether to configure one/more common frequency resources which may or may not containing the initial BWP) if there are not strong concerns with this. - if there are concerns, the FL would propose a generic FFS that would allow to study different options for next meeting. - The third sub bullet in Proposal 5-rev1 (FFS: configuration details of common frequency resource when it is different from initial BWP) would consider details of configuration of Case 1b. - Whether to also include explicit mention of Case 1c, depends on the clarifications from CATT. Thank you. |
| Qualcomm | We are fine with FL’s suggestion to keep ‘containing the initial BWP’ only in the 2nd FFS. Regarding Case 1c, it seems not necessary to further limit the scheduling flexibility of GC-PDCCH/PDSCH since initial BWP is common to UEs. |
| NOKIA, NSB | Thanks for the clarifications from the FL. Generally, we are fine with the current proposal. |
| Convida | We are OK with the firs bullet (i.e., case 1a) as the fallback/default mode when a specific common frequency resource is not configured. For other cases (e.g., case 1b, 1c, 1d), since this is the first meeting we discuss the MBS for RRC_IDLE/RRC_INACTIVE UEs and we are listing all the different cases here for the purpose of further study, we would prefer to include all the options and cases in the proposal. |
| Moderator | Thank you for your inputs. Based on the discussion in this round: - the main bullet of Proposal 5-rev1 (i.e., For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH) seems acceptable to companies. - the first sub-bullet of Proposal 5-rev1 (i.e., the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.) seems acceptable to companies. - the discussion on the two last sub-bullets addressing the FFS are less stable. Based on the comments above the FL proposes the following revision to Proposal 5-rev1. This proposal tries to include both views that the configured common frequency resources can contain the initial BWP and/or can be contained within the initial BWP. Since this is an FFS, it should provide sufficient guidance for technical contributions at next meetings while not being too restrictive at this stage. Proposal 5-rev2: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: whether to configure one/more common frequency resources which may or may not containing the initial BWP and/or contained within the initial BWP. - FFS: configuration details of common frequency resource when it is different from initial BWP. |

### Third round FL proposals for Issue 4

Proposal 5-rev2: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: whether to configure one/more common frequency resources containing the initial BWP and/or contained within the initial BWP.

- FFS: configuration details of common frequency resource when it is different from initial BWP

Please provide your company’s views on the Proposal 5-rev2 in the table below:

| company | comments |
| --- | --- |
| Intel | In the second FFS, the motivation of configuring one or more frequency resource containing the initial BWP is not clear. The FFS should only be for frequency resources contained within the initial BWP. - FFS: whether to configure one/more common frequency resources containing the initial BWP and/or contained within the initial BWP. The second FFS can be further modified as follows: - FFS: configuration details of common frequency resource when it is different from when it is not contained within the initial BWP |
| LG | We are fine with Proposal 5-rev2. In addition, considering the working assumption, we propose to add ‘FFS: whether the common frequency resource is defined as an MBS specific BWP or an ‘MBS frequency region’. |
| vivo | Similar as the discussion for RRC CONNECTED UEs, we can focus on how to define/configure common frequency resource for group-common PDCCH/PDSCH first and leave it open “if a specific common frequency resource is not configured”. We propose to add “FFS: the relation of this common frequency resource and initial BWP, i.e., the common frequency resource containing the initial BWP, or contained within the initial BWP.” |
| ZTE | Minor update. We propose to separate the first FFS point into two FFS points because it contains two different items. Proposal 5-rev2: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: whether to configure one/more common frequency resources - FFS: whether the common frequency resource contains the initial BWP and/or is contained within the initial BWP. - FFS: configuration details of common frequency resource when it is different from initial BWP |
| CMCC | We suggest use “confined” in place of “contained”, which is aligned with the working assumption of common frequency resource for RRC_CONNECTED state Proposal 5-rev2: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: whether to configure one/more common frequency resources containing the initial BWP and/or contained confined within the initial BWP. - FFS: configuration details of common frequency resource when it is different from initial BWP |
| Apple | We share the similar view as vivo, first we need to agree whether define the common frequency resource, then to discuss the relationship between the common frequency resource and initial BWP. |
| CATT | Thanks FL for the clarifications and good explanations. - To answer the question during second round from FL “would case 1c be a subset of case 1a?”: My understanding is it depends on whether the MBS common frequency resource is configured or not. My understanding is that case 1c is not a subset of case 1a. Please also correct if my understanding is wrong. - The first sub-bullet said that “if a specific common frequency resource is not configured”, that means the initial BWP (including the configurations) is used for reception of broadcast; in other words, there is no specific BWP configuration for MBS existing. - For case 1c, there is a configuration of frequency resource for MBS within the initial BWP, and the range can be small or large. When the range of MBS frequency resource equals to initial BWP, it looks like they are the same, but the configurations may not be the same. - If the configuration of broadcast common frequency resource and the configuration of initial BWP are all same except the frequency range, then case 1c can be considered as a sub-set of case 1a. This is the reason why I think it depends on the definition and configurations. The current version of updated proposal by FL seems OK to us. |
| Samsung | We are generally fine with the proposal. But we would like to suggest to change as below. - the UE may assume uses the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. This is because, there will be configuration on MBS. So, if there is no frequency resource, then the UE will use the initial BWP, not just assume. All configuration will be related to the initial BWP in that case, so the UE “needs” to use the initial BWP for MBS. |
| Spreadtrum | We are generally fine with the proposal. Shared the same view as CMCC that it is better to change the second “contained” to “confined”, two cases of common frequency resource should be further discussed. |
| Lenovo, Motorola Mobility | Generally, we are fine with the FL proposal. Agree with LG and CMCC’s modifications. |
| Huawei/HiSilicon | The revision from CMCC looks good to us. |
| Moderator | Thank you for the inputs so far. @All, I want to point out that most of the discussion is around FFS. I think that while it is useful to have precise wording that can help discussion at next meetings, we are getting close to the end of the meeting so we will need to come to an agreement soon. Hence, the FL would propose to revise Proposal 5-rev2 as follows (there are companies still to comment, but to make some progress we make an update at this stage – hope this is fine): Proposal 5-rev3: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume uses the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of this common frequency resource and initial BWP, i.e., the common frequency resource containing the initial BWP, or confined within the initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource when it is different from initial BWP The new Proposal 5-rev3 tries to include the inputs above as follows while being general to include various points of view (hopefully): @Intel, thanks for the suggestions. - your first comment regarding the FFS in Proposal 5-rev2 (whether to configure one/more common frequency resources containing the initial BWP and/or contained within the initial BWP) combines the proposals from different companies in previous rounds of discussions where the common frequency resource contains the initial BWP or is confined within the initial BWP. Various companies expressed their preference for a common frequency resource that contains the initial BWP. One of the motivations for such an approach is to avoid BW switching. Also, it felt useful to have an FFS, at this stage of the discussion, with options in the table so companies can come back at next meetings with more detail proposals. Hence, in the Proposal 5-rev3 I have kept the two options. I hope this is fine. - Regarding your last comments, on the FFS of Proposal 5-rev2 (configuration details of common frequency resource when it is different from initial BWP), this FFS has been updated so it is more general while including the views from other companies on definition aspects of the common frequency resource. @LG, thanks for the comments, I have included your point in the last FFS of Proposal 5-rev3, although I have included it in a more general form to also avoid having too many FFS. Hope this is fine. @vivo, thanks for the comment which has also been included in the revised proposal. @ZTE, thanks for the comment which has been included in the revised proposal @CMCC, thanks for the comment, which seems to have support since it aligns wording, so this has been included. @CATT, thanks for your clarifications! Hopefully the current wording of the proposal intends to be general enough so the different options can be studied for the next meeting. @ Samsung, thanks for the comment which has been included, if there are no strong concerns from other companies, we can keep it. |

### Fourth round FL proposals for Issue 4

Proposal 5-rev3: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH.

- the UE uses the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of this common frequency resource and initial BWP, i.e., the common frequency resource containing the initial BWP, or confined within the initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Please use the table below to indicate whether your company has any concerns about the proposal above, or any other views you may want to make:

| company | comments |
| --- | --- |
| Ericsson | OK |
| NOKIA, NSB | We are fine with the current high-level proposal from the FL |
| LG | We can live with the Proposal 5-rev3. |
| Convida | We are OK with the proposal. |
| Qualcomm | We prefer not to say ‘UE uses’. It is not mandatory for RRC_IDLE/INACTIVE UEs to use. |
| Intel | @FL: Thank you for the explanations and taking our view into account. We are ok with the current proposal. |
| Moderator | Thank you for the inputs. @ Samsung, based on the comment raised by Qualcomm and given that the previous wording was widely supported, the FL proposes to revert back to “may assume” instead of “uses”. Hope this is acceptable. Hence, the revision of Proposal 5-rev3 would be as follows: Proposal 5-rev4: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH. - the UE may assume uses the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of this common frequency resource and initial BWP, i.e., the common frequency resource containing the initial BWP, or confined within the initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource |

### Fifth round FL proposals for Issue 4

Proposal 5-rev4: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of this common frequency resource and initial BWP, i.e., the common frequency resource containing the initial BWP, or confined within the initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Please use the table below to indicate whether your company has any concerns about the proposal above, or any other views you may want to make:

| company | comments |
| --- | --- |
| ZTE | We are ok with the proposal. |
| Moderator | A revision of Proposal 5-rev4 has been agreed at online GTW session. Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of the common frequency resource(s) (if configured) and initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource The discussion of this issue in this RAN1#103-e meeting is therefore finished. |

## Issue 6: Beam sweeping for UEs in RRC_IDLE/ RRC_INACTIVE states

This issue has been discussed in 8 tdocs (10 companies). This issue address beam sweeping mechanisms for MBS for Ues in RRC_IDLE/ RRC_INACTIVE states.

In particular:

[2, Huawei, HiSilicon] discusses that beam sweeping mechanism is widely used for common channel transmissions in NR, such as SSB, SI and paging, in order to guarantee the coverage requirement. For example, one paging occasion contains multiple sub-occasions each linked with corresponding SSB index, so a UE only needs to monitor a subset of the sub-occasions for paging. [2] proposes that beam sweeping mechanism and the corresponding monitoring behaviour for multicast scheduling should be considered.

[3, Chengdu TD Tech Ltd] proposes that the beams for transmitting the SS/PBCH Block are used to transmit the PTM bearer.

In [4, VIVO] proposes that the MBS PDCCH monitoring occasion is associated with SSB and that the UE is only required to monitor the MBS PDCCH monitoring occasion associated with at least one SSB.

[5, CATT, CBN] first proposes that multi-beam operation is supported for Rel-17 MBS transmission. [5] also proposes that for saving the frequency resource, the indications of PTP/PTM mode can be based on per-SSB.

[6, CMCC] proposes that the association between SSB indexes and monitoring occasions for group-common PDCCH scheduling MBS PDSCH can be defined to support beam sweeping in RRC_IDLE/RRC_INACTIVE states.

[10, ZTE] observes that for Rel-15 paging mechanism different Ues are separated into different Pos, while for Rel-15 SIBx mechanism different SI messages are separated into different SI-windows. [10] compares paging and SIBx mechanisms and based on their analysis, the SIBx mechanism is more appropriate to Rel-17 broadcast. Hence, [10] proposes to consider beam sweeping mechanism for NR Rel-15 SIBx transmission as the starting point for Rel-17 broadcast.

[14, Convida] proposes that beam sweeping mechanism can be considered for supporting MBS for Ues in RRC_IDLE/RRC_INACTIVE states. The details can be for further study.

[15, Qualcomm] proposes that the UE may assume that PDCCH/PDSCH for MCCH is QCL’d with SSB or TRS if configured in SIB for broadcast reception.

In the inputs above for Ues in RRC_IDLE/ RRC_INACTIVE states: 8 inputs (10 companies) propose/consider the support of beam sweeping for MBS, 6 inputs (8 companies) discuss UE monitoring occasions are associated with a subset of the total SSB indexes, 1 input/company proposes that the UE monitoring and association is based on beam sweeping SIBx procedures, 1 input/company proposes that QCL assumptions based on SSB or TRS are configurable, and 1 input (2 companies) proposes that the PTP/PTM indications can be done per SSB.

Based on the above the FL makes the following initial proposals.

### Initial FL proposal for Issue 6

Proposal 9: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for Rel-17 MBS transmissions.

- FFS: UE monitoring occasions are associated with a subset of the total SSB indexes.

- FFS: UE monitoring and association is based on beam sweeping SIBx/paging procedures.

- FFS: QCL assumption based on SSB or TRS based on SIB configuration.

- FFS: PTP/PTM indications can be done per SSB.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Samsung | Before agreeing the proposal, can we clarify “MBS transmissions”? Does it include PDCCH for MBS or PDSCH for MBS or both? |
| LG | We are fine with Proposal 9. We could change ‘MBS transmission’ to ‘group common PDCCH/PDSCH’ for clarity. |
| Huawei/HiSilicon | Suggested revision: Proposal 9: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for Rel-17 MBS transmissions. - FFS: UE monitoring occasions are associated with a subset of the total SSB indexes. - FFS: UE monitoring and association is based on beam sweeping for SIBx/paging procedures. - FFS: QCL assumption based on SSB or TRS based on SIB configuration. - FFS: PTP/PTM indications can be done per SSB. Reasons: Keeping one of the first two sub-bullets is sufficient and maybe the first one is more accurate. The motivation of QCL to TRS is unclear. PTP/PTM needs clarification. I know they are FFS, but the point should be clear so as to further study. |
| CATT | We are generally OK with this proposal. For the last FFS, the PTP/PTM indications can be done per SSB based on gNB’s indication to IDLE UE, e.g. the total number of CONNECTED Ues. For example, the PTP mode is applied in an SSB when there are few CONNETDED Ues, and the IDLE/INACTIVE Ues in this SSB will firstly switch from IDLE to CONNEDTED state to receive the PTP mode services. At this time, all UE in this SSB are in CONNECTED state. Thus, gNB gets to know the total number of Ues of receiving the service in PTP mode. Currently, if the total number of Ues increases largely by comparing with previous scenario, gNB update this SSB by switching it from PTP mode to PTM mode for transmitting MBS services. Thus, we suggest that the last FFS can be changed as: FFS: PTP/PTM indications can be done per SSB based on gNB’s indication, e.g. the total number of RRC_CONNECTED Ues. |
| ZTE | We are generally fine with Proposal 9. Our understanding of Rel-17 MBS transmission is PDCCH/PDSCH for MBS. |
| Vivo | We are generally OK with this proposal. We are also fine to clarify the “MBS transmissions”. |
| Moderator | Thanks for the inputs. Although more discussion is needed for the proposal in general, it may be helpful to clarify at this point “MBS transmissions” to “group-common PDCCH/PDSCH” to check whether this would be more represent better the views of the companies. |
| CMCC | Similar view as the above companies, the MBS transmission should be clarified as group-common PDCCH/PDSCH. Regarding the FFS, the first FFS should also contain the case associated with all SSB indexes, in addition, the first FFS has contained the second FFS, because the UE monitoring and associations of SIBx/paging are also associated with SSB. The third FFS can be removed as there is no TRS configuration for RRC_IDLE/INCTIVE UE in Rel-15/16, and we don’t see the motivation to introduce it. Regarding the last FFS, we don’t have PTP/PTM definition in RAN1, we only agree the definition of PTP transmission, PTM transmission scheme 1 and scheme 2 for convenience of discussion, and whether to support PTP transmission / PTM transmission scheme 2 for RRC_CONNECTED UE has not have an agreement now, the similar situation is also applied to RRC_IDLE/INACTVE UE, thus we suggest remove the last three FFS. Proposal 9: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH / PDSCH Rel-17 MBS transmissions. - FFS: UE monitoring occasions are associated with all SSB indexes or a subset of the total SSB indexes. - FFS: UE monitoring and association is based on beam sweeping SIBx/paging procedures. - FFS: QCL assumption based on SSB or TRS based on SIB configuration. FFS: PTP/PTM indications can be done per SSB. |
| NOKIA | We agree with the above modification proposal from CMCC |
| Qualcomm | If Proposal 1 is accepted, we agree with LGE to change ‘MBS transmission’ to ‘GC-PDCCH/PDSCH’. We prefer to keep second and third FFS. However, the first and last FFS are not clear to us. We prefer to delete them. - For the first FFS, we think how the IDLE/INACTIVE Ues monitor GC-PDCCH/PDSCH is up to UE implementation. - For the third FFS, the TRS is to be introduced for IDLE/INACTIVE Ues in Rel17. It can be considered for MBS reception of IDLE/INACTIVE Ues as well. It is a far-fetched reason to object it just because Rel15/16 is not supporting TRS. - For the last FFS, PTP indication is not possible for RRC_IDLE/INACTIVE Ues since the UE may be moving and the network is not aware of UE condition. |
| Moderator | Thank you for the inputs. From the comments above, it seems that supporting beam sweeping for RRC_IDLE/ RRC_INACTIVE Ues is agreeable by all contributing companies. It is also supported that “MBS transmissions” is clarified to “group-common PDCCH / PDSCH”. There are disagreements on the FFS sub bullets. We may need more discussion to clarify concerns from different companies. Based on the above, the FL proposes the following revision 1 to Proposal 9: Proposal 9-rev1: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH Rel-17 MBS transmissions. - FFS: UE monitoring occasions are associated with all SSB indexes or a subset of the total SSB indexes. - FFS: UE monitoring and association is based on beam sweeping SIBx/paging procedures. - FFS: QCL assumption based on SSB or TRS based on SIB configuration. - FFS: PTP/PTM indications can be done per SSB based on gNB’s indication, e.g. the total number of RRC_CONNECTED Ues. The FL proposes another round of discussion focusing on clarifying the FFS or to check whether clarifications provided addresses concerns raised by companies: - companies check comments from Qualcomm regarding first FFS on monitoring is up to UE implementation; - Qualcomm to clarify further the motivation for second FFS (QCL assumption based on SSB or TRS based on SIB configuration) to address concerns raised. - CATT to clarify for third FFS (PTP/PTM indications can be done per SSB based on gNB’s indication, e.g. the total number of RRC_CONNECTED Ues) concerns raised by companies. Thank you for your continued contributions. |

### Second round FL proposal for Issue 6

Proposal 9-rev1: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

- FFS: UE monitoring occasions are associated with all SSB indexes or a subset of the total SSB indexes.

- FFS: QCL assumption based on SSB or TRS based on SIB configuration.

- FFS: PTP/PTM indications can be done per SSB based on gNB’s indication, e.g. the total number of RRC_CONNECTED Ues.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Intel | The main bullet is OK For the FFS points, we prefer to have a more generic formulation at this early stage rather than list the current options i.e., FFS: Details for support of beam sweeping for GC-PDCCH/PDSCH. |
| Huawei/HiSilicon | Beam sweeping for IDLE/INACTIVE implies defining the monitoring occasions associated with SSB, so don’t see why the first sub-bullet is FFS and don’t see the necessity to FFS on other sub-bullets. If controversial, we could be ok to list a single FFS as Intel suggested. |
| CATT | We are OK with the current version of proposal 9 and the FFS sub-bullets. |
| CMCC | Agree with Intel, just keep a general FFS, all the details can be discussed in next meeting. |
| Lenovo, Motorola Mobility | We are OK with the main bullet. Maybe we can replace all the sub-bullets with FFS details. |
| Qualcomm | ok to keep a general FFS and delay discussion of details to next meeting. |
| NOKIA, NSB | We are fine with the current proposal, and details can be discussed in the coming meeting as suggested also by CMCC and Qualcomm |
| Convida | We are OK with the main bullet and share with other companies’ view to have generic sub-bullet(s). |
| Moderator | Thank you all for your inputs. From the discussion so far in the two rounds of comments supporting beam sweeping for RRC_IDLE/ RRC_INACTIVE Ues is stable. Giving the comments above, it seems an acceptable way forward to leave a generic FFS that allows further technical discussions on the next RAN1 meetings. Hence, the FL proposes the following revision to Proposal 9-rev1. Proposal 9-rev2: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH. - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH. - FFS: UE monitoring occasions are associated with all SSB indexes or a subset of the total SSB indexes. - FFS: QCL assumption based on SSB or TRS based on SIB configuration. - FFS: PTP/PTM indications can be done per SSB based on gNB’s indication, e.g. the total number of RRC_CONNECTED Ues. |

### Third round FL proposal for Issue 6

Proposal 9-rev2: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

- FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Please use the table below to indicate whether your company has any concerns about the proposal above, or any other views you may want to make:

| company | comments |
| --- | --- |
| Intel | OK with Proposal 9-rev2 |
| LG | We are fine with Proposal 9-rev2. In addition, we suggest to add ‘FFS: for RRC_CONNECTED Ues’. |
| Vivo | OK with Proposal 9-rev2 |
| ZTE | Ok with the FL proposal. |
| CMCC | OK |
| Apple | We are generally OK with this proposal, the FFS sub-bullet is redundant . |
| CATT | We are OK with the current version of proposal 9-rev2. |
| Samsung | Agree. |
| Spreadtrum | We are ok with Proposal 9-rev2 |
| Lenovo, Motorola Mobility | We are fine with the FL proposal. |
| Huawei/HiSilicon | Support. |
| Moderator | Thank you for the inputs. @ LG, thank you for the suggestion. I wanted to propose that, if you do not have very strong views on this suggestion, we do not include the FFS for connected UEs for this proposal. Since this proposal is focusing on idle/inactive UEs, it may be convenient for progress to discuss such a FFS at next meetings in this AI if companies are interested. Hope this is makes sense. Thank you. |
| NOKIA, NSB | We are fine with Proposal 9-rev2 |
| Convida | We are OK with the proposal. |
| Qualcomm | We are fine with current wording. |
| Moderator | Thank you for inputs. The FL considers this proposal is stable and ready for agreement. Therefore, the Proposal 9-rev2 is unchanged. |

### Fourth round FL proposal for Issue 6

Proposal 9-rev2 [unchanged]: for RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

- FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Please use the table below to indicate whether your company has any strong concerns about the proposal above:

| company | comments |
| --- | --- |
| Moderator | Proposal 9-rev2 has been agreed by email. Agreements: - For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH. - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH. |

# Discussion on Medium Priority Issues (only after 2nd check point)

## Issue 7: PDCCH Search Space for UEs in RRC_IDLE/ RRC_INACTIVE states

This issue has been discussed in 5 tdocs (6 companies). This issue addresses initial aspects of the PDCCH common search spaces for Ues in RRC_IDLE/ RRC_INACTIVE states.

In particular:

[2, Huawei, HiSilicon] discusses that control resource set (CORESET) and related common search space (CSS) set for multicast scheduling should be configured jointly or separately for MCCH and MTCH. If not, the default could be CORESET0 and Type0 CSS set.

[4, vivo] proposes that the MBS-specific CORESET/searchingSpace can be configured.

[6, CMCC] proposes that CSS is supported for group-common PDCCH monitoring in RRC_IDLE/ RRC_INACTIVE states. [6] also proposes that further study whether to define a new CSS type or reuse current CSS type.

[7, LGE] proposes that assuming that RAN2 introduces a new MBMS SIB, if UE is interested to receive a MBS TB, UE monitors PDCCH for Type0A-PDCCH CSS set to detect a DCI with SI-RNTI and receive MBMS SIB on PDSCH for a serving frequency. [7] also proposes that assuming that RAN2 introduces a MCCH, if UE is interested to receive a MBS TB, UE monitors PDCCH for a PDCCH CSS set to detect a DCI with SC-RNTI and receive MCCH information on PDSCH where FFS on which type of PDCCH CSS set is used for MCCH, if supported. [7] also proposes to discuss whether to define a new type CSS set or a new MBS specific SS set (e.g. MSS) for group scheduling of MBS data.

[15, Qualcomm] proposes that a new type of CSS is defined as the SS of PDCCH for MCCH, configured by SIB.

In the inputs above for Ues in RRC_IDLE/ RRC_INACTIVE states: 5 inputs (6 companies) support the definition/study of new CSS types, and 3 inputs (4 companies) discuss/propose to reuse existing CSS types.

Based on the above the FL makes the following initial proposals.

### Initial FL proposal for Issue 7

Proposal 10: for RRC_IDLE/RRC_INACTIVE Ues, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS types and/or define a new CSS type.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Intel | Seems OK |
| CMCC | OK |
| CATT | We are generally fine with the proposal. The CSS configuration in IDLE/INACTIVE Ues should keep consistent with that of CONNECTED Ues. We would like to suggest updating the FFS as: - FFS: The configuration of CSS can be the same with RRC_CONNECTED Ues. |
| Lenovo, Motorola Mobility | We are fine with the FL proposal. |
| Huawei/HiSilicon | What is the reason to have “and” in the FFS sub-bullet? |
| Moderator | Thanks for the inputs. @Huawei/HiSilicon, the intention to have “and” was to leave the door open in case there were proposal that wanted to include for idle/inactive configuration both an existing CSS type and new CSS types. But if companies prefer to remove the “and” this is fine from FL point of view of course. |
| NOKIA,NSB | We are fine with the FL proposal. |
| LG | We are fine with this proposal. |
| Convida | We are OK with the proposal. |
| Qualcomm | Ok to us |
| Moderator | Thank you for the inputs. Since there was a suggestion from CATT, the FL proposes a revision of Proposal 10: Proposal 10-rev1: for RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH. - FFS: reuse current CSS types and/or define a new CSS type. - FFS: the CSS configuration of the CSS can be the same as the CSS configuration of with RRC_CONNECTED UEs. @CATT, thanks for the suggestion, which has been included. I just did some modifications of the wording to try to make it clearer. |

### Second round FL proposal for Issue 7

Proposal 10-rev1: for RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS types and/or define a new CSS type.

- FFS: the CSS configuration can be the same as the CSS configuration of RRC_CONNECTED UEs.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| ZTE | Regarding the second FFS, our understanding is that it applies to at least broadcast for RRC_CONNECTED UEs. We are not sure whether it can be applied to multicast for RRC_CONNECTED UEs yet. Thus, we would like to update it as following. - FFS: the CSS configuration can be the same as the CSS configuration of RRC_CONNECTED UEs at least for broadcast. |
| Moderator | A revision of Proposal 10-rev1 has been agreed at online GTW session. Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH. - FFS: reuse current CSS type, define a new CSS type, etc. - FFS other details. The discussion of this issue in this RAN1#103-e meeting is therefore finished. |

## Issue 8: CORESET for UEs in RRC_IDLE/ RRC_INACTIVE states

This issue has been discussed in 7 tdocs (9 companies). This issue addresses whether new and/or existing control resource set (CORESET) are used for UEs in RRC_IDLE/ RRC_INACTIVE states.

In particular:

[2, Huawei, HiSilicon] discusses that control resource set (CORESET) and related common search space (CSS) set for multicast scheduling should be configured jointly or separately for MCCH and MTCH. If not, the default could be CORESET0 and Type0 CSS set.

[3, Chengdu TD Tech Ltd] proposes that at least one CORESET and SS group needs to be configured for each II-UE (II-UES stand for idle/inactive state UEs) BWP or each secondary II-UE BWP.

[4, vivo] proposes that the MBS-specific CORESET/searchingSpace can be configured.

[8, Samsung] proposes that high layer signalling also delivers CORESET for multicast and group-specific RNTI.

[12, Lenovo, MotoM] proposes that the group-common DCI is transmitted in CORESET 0 and the group-common PDSCH for MBS is transmitted within initial DL BWP.

[13, Intel] proposes that RRC_IDLE/INACTIVE UEs can be configured with a common MBS frequency resource and MBS CORESET within the common frequency resource by SIB where it can monitor the group common PDCCH within a CSS.

[15, Qualcomm] discusses that CORESET for MCCH can be same as CORESET0 by default. It is possible to configure the CORESET for MCCH by SIB. CORESET for brodcast MTCH can be CORESET0 or configured by MCCH within the Broadcast BWP. [15] proposes that CORESET of PDCCH for MCCH can be same as CORESET0 by default.

In the inputs above for UEs in RRC_IDLE/ RRC_INACTIVE states:7 inputs (9) companies discuss/support defining a MBS CORESET, where 3 inputs (4 companies) state that the MBS coreset could be the CORESET0.

Based on the above the FL makes the following initial proposals.

### Initial FL proposal for Issue 8

Proposal 11: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of PDCCH can be CORESET0 by default.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| Intel | Similar to discussion in 8.12.1, it would be better to state what is meant by default i.e., when MBS-specific CORESET is not configured within the MBS frequency resource. Note that for default to apply, the MBS frequency resource should contain the initial BWP. |
| CMCC | It is not clear the usage of CORESET for RRC_IDLE/RRC_INACTIVE UEs. We think there are potential three types of PDCCH may be discussed: 1) group-common PDCCH for scheduling group-common PDSCH 2) PDCCH scheduling MCCH, if MCCH is supported by RAN2 3) PDCCH scheduling SIBx, if SIBx is supported by RAN2 to configure the PDCCH scheduling MCCH Since, the last two types of PDCCH has not been discussed in RAN2, we should first focus on the first type. In addition, we think the CORESET configuration is also related to the common frequency resource for RRC_IDLE/INACTIVE UEs. - If the common frequency resource equals with initial BWP, the CORESET for group-common PDCCH used for scheduling group-common PDSCH is CORESET 0. - If the common frequency resource not equals with initial BWP, a MBS-Specific CORESET can be configured. Since, whether to support common frequency resource not equals with initial BWP is still under discussion, we think this issue can be postpone in the further meeting. Or we suggest take the following proposal at this stage: CORESET0 is used for group-common PDCCH for RRC_IDLE/INACTIVE UEs, if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP. |
| Lenovo, Motorola Mobility | We are fine with the FL proposal. |
| Huawei/HiSilicon | Proposal from CMCC can be a starting point and FFS CORESET when a common frequency resource for group-common PDCCH/PDSCH is not initial DL BWP. |
| NOKIA, NSB | Agree with CMCC, the CORESET configuration may depend on what kind of PDCCH payload that is going to be conveyed. Also how to configured the common frequency resource for RRC_IDLE/INACTIVE and the relation with initial BWP need to be clarified first before we can make further discussion on this issue. |
| LG | We generally agree with CMCC. We propose to postpone RAN1 discussion on this proposal. |
| Convida | We are generally fine with the proposal. We think the word ‘default’ here may raise the similar issue discussed in issue 4. We suggest to reword the proposal as: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of PDCCH can be CORESET0 by default, if an MBS-specific CORESET is not configured. |
| Qualcomm | Understand CMCC’s concern. But it’s not clear whether the CORESET can be configured or not when initial BWP is the common frequency resource. If no MBS CORESET is configured, UE assumes CORESET0 as default. Therefore, we suggest the following modification Proposal 11: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of group common PDCCH can be CORESET0 by default if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP and no MBS specific CORESET is configured. |
| Moderator | Thank you for the inputs. Based on the inputs, the FL proposes a revision to Proposal 11 as follows (this revision uses the wording from Qualcomm and Huawei/HiSilicon since it seems to address comments from CMCC): Proposal 11-rev1: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of group common PDCCH can be CORESET0 by default if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP and no MBS-specific CORESET is configured. - FFS: configuration and definition details of the MBS-specific CORESET @Intel, thanks for input. I think the current wording may clarify the “default” word. @CMCC, thank you for your detailed input. I think the current wording includes your proposal plus additional clarifications. @Huawei/HiSilicon, thanks for input. This proposal uses CMCC as starting point as per your suggestion and also included the FFS. The FFS tries to be more general and cover more common frequency options @Nokia, NSB, thanks for input. The points from CMCC have hopefully been included in this revision of the proposal that also includes FFS to allow further clarifications and next meetings. @LG, thanks for input. the points from CMCC have hopefully been included in this revision. The FL proposes to see how the discussion evolves and if there is sustained concern this can be moved to next meetings. @Convida, thanks for the input. the new proposal tries to address you comment. @Qualcomm, your proposal has been used as starting point. |

### Second round FL proposal for Issue 8

Proposal 11-rev1: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of group common PDCCH can be CORESET0 by default if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP and no MBS-specific CORESET is configured.

- FFS: configuration and definition details of the MBS-specific CORESET

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| ZTE | It seems there is still one scenario missing in the proposal. If the common frequency resource is configured and it contains the initial DL BWP and no MBS-specific CORESET is configured, then CORESET0 can still be used by default. Thus, we would like to propose the following update. Proposal 11-rev1: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of group common PDCCH can be CORESET0 by default - if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP and no MBS-specific CORESET is configured. - If the common frequency resource for group-common PDCCH/PDSCH contains the initial DL BWP and no MBS-specific CORESET is configured. - FFS: configuration and definition details of the MBS-specific CORESET |
| vivo | We are fine Proposal 11-rev1. Regarding t ZTE’s update, we have different views. From our understanding, if the common frequency resource for group-common PDCCH/PDSCH is configured different from initial DL BWP, it may be mandatory to configure MBS-specific CORESET in that case. We prefer to leave it for further study instead of making decision now. |
| Huawei/HiSilicon | Share the similar view with ZTE that CORESET0 can still be used when no other CORESET is configured as long as the common frequency resource containing the CORESET0. The point of this proposal should not be “MBS-specific CORESET is defined” but rather the point should be another CORESET can be configured because CORESET0 should be applicable by default. Whether defining a MBS-specific CORESET or whether it is another new CORESET name does not matter. Hence, I suggest changing it as follows: Proposal 11-rev1: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured for PTM scheme 1 if the common frequency resource for group-common PDCCH/PDSCH contains initial DL BWP. If the CORESET is not configured, CORESET0 is used by default. - FFS: configuration details of the CORESET for PTM scheme 1. |
| Lenovo, Motorola Mobility | Since the CORESET for monitoring group-common PDCCH is CORESET 0 and CORESET 0 is a common CORESET, it is not accurate to say CORESET 0 is a default MBS-specific CORESET. We support the proposals from Huawei which addresses our concern. |
| CATT | For proposal 11-rev1, we share the similar view with vivo in general that we can leave it in FFS as proposed by FL. The design and relationship of common frequency resource and initial BWP are still not defined/determined, we cannot make decision before that. I think the intention of the FFS here proposed by FL is provide a guidance for future study and contribution discussion. We support the FL’s proposal. Proposal 11-rev1: for RRC_IDLE/RRC_INACTIVE UEs, an MBS-specific CORESET is defined where the CORESET of group common PDCCH can be CORESET0 by default if the common frequency resource for group-common PDCCH/PDSCH is initial DL BWP and no MBS-specific CORESET is configured. - FFS: configuration and definition details of the MBS-specific CORESET |
| Moderator | Thank you for the comments. Given the inputs above the FL proposes the following revision to Proposal 11-rev1 Proposal 11-rev2: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP. - FFS: configuration details of the CORESET for group-common PDCCH/PDSCH. The revised proposal above tries to capture the minimum common points from the comments raised by companies. Before discussing the points from each company is worth noting here the agreement for Issue 4 in this meeting: Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of the common frequency resource(s) (if configured) and initial BWP. - FFS: whether to configure one/more common frequency resources - FFS: configuration and definition details of the common frequency resource @ZTE, thanks for input. Given concerns from vivo and CATT, and given that it is still not agreed whether a common frequency resource will be configured containing the initial BWP, I would propose to leave the possibility that CORESET0 is used for a common frequency resource containing the initial BWP for discussion at next meetings when frequency resource discussion has moved forward. However, your points about defining a CORESET for group-common PDCCH/PDSCH and using CORESET0 at least for the initial BWP are included. @vivo, thanks for input. the proposal above includes your views. @Huawei/HiSilicon, thank you for input. Similar comments as the ones provided to ZTE. Your comments about not discussing an MBS-specific CORESET have been included. I have also removed the term PTM scheme 1 since, in my understanding, although it has been agreed for AI 8.12.1, in such agreement the text mentions for RRC_CONNECTED UEs, so I thought it may be more convenient to write group-common PDCCH/ PDSCH to avoid any misunderstanding. @Lenovo, Motorola, thank you for input. As discussed with Huawei and ZTE, this proposal tries to agree the minimum common points from the comments raised by companies. @CATT, thanks for input, your comments have been included. |

### Third round FL proposal for Issue 8

Proposal 11-rev2: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH.

Please use the table below to indicate if your company has concerns about above proposal:

| company | comments |
| --- | --- |
| Qualcomm | Agree with FL’s suggestion to leave the other possible cases to next meeting. We suggest some minor change on the wording about the CORESET configuration. Hope it is the common understanding. Proposal 11-rev2: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP. FFS: configuration details of the CORESET for group-common PDCCH/PDSCH |
| Moderator | Thanks Qualcomm for the input. Given this input the FL proposes the revision of Proposal 11-rev2 for final consideration: Proposal 11-rev3: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP. - FFS: configuration details of the CORESET for group-common PDCCH/PDSCH |

### Fourth round FL proposal for Issue 8

Proposal 11-rev3: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

Please use the table below to indicate if your company has concerns and/or wants to do further modifications, and note that in this case the discussion of this proposal would be left for next meetings:

| company | comments |
| --- | --- |
| LG | We are fine if you relocate “If the CORESET is not configured,” as follows: Proposal 11-rev3: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. If the CORESET is not configured, CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured. - FFS: configuration details of the CORESET for group-common PDCCH/PDSCH |
| CMCC | OK with LG’s version. |
| Moderator | The update to Proposal 11-rev3 from LG was agreed by email as Proposal 11-rev3-update: Agreement: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured. - FFS: configuration details of the CORESET for group-common PDCCH/PDSCH |

# Summary

The following are the agreements achieved during RAN1#103-e meeting.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

- FFS details

Agreements:

- For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

  - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs.

- FFS details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS type, define a new CSS type, etc.

- FFS other details.

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

# References

- RP-201038	Revised Work Item on NR Multicast and Broadcast Services, Huawei, HiSilicon

- R1-2007564	Discussion on multicast support for IDLE/INACTIVE UEs, Huawei, HiSilicon

- R1-2007639	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs, CHENGDU TD TECH LTD.

- R1-2007693	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues, vivo

- R1-2007837	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, 	CATT, CBN

- R1-2008036	Discussion on NR MBS in RRC_IDLE/RRC_INACTIVE states, CMCC

- R1-2008066	Basic function for broadcast/multicast, LG Electronics

- R1-2008194	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, Samsung

- R1-2008244	Discussion on enhancements for IDLE and INACTIVE state UEs, OPPO

- R1-2008828	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs, ZTE

- R1-2008884	Basic Functions for Broadcast / Multicast for RRC_IDLE / RRC_INACTIVE UEs, Nokia, Nokia Shanghai Bell

- R1-2008928	Basic functions for broadcast/multicast in idle/inactive states, Lenovo, Motorola Mobility

- R1-2009002	NR-MBS for RRC_IDLE/INACTIVE UEs, Intel Corporation

- R1-2009167	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs, Convida Wireless

- R1-2009276	Discussion on broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, Qualcomm Incorporated

- R1-2009307	Support for NR multicast reception in RRC Inactive/Idle, Ericsson