3GPP TSG RAN WG1 Meeting #103-e	R1-2009730

E-Meeting, October 26 – November 13, 2020

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #3 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for NB-IoT and LTE-MTC [1] includes an objective to support 16-QAM for unicast in UL and DL in NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

This documents provides the proposals and summary of discussions of the following email discussion according to the inputs [2-10]

[103-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Round 3 proposals

## Issue 1: The TBS table for DL

The following has been achieved:

Working Assumption

The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

Please input your comments regarding the following:

- Whether the working assumption can be confirmed

- Your preference of the entry (I_SF = 0, I_TBS=16) and reasons

- Your preference of the entry (I_SF = 1, I_TBS=14) and reasons

- Your preference of the entry (I_SF = 4, I_TBS=21) and reasons

| Companies | Comments |
| --- | --- |
| Ericsson | - Working assumption: Although based on our simulation results the WA can be confirmed, we respect the fact that other companies need to time to check. - Entry (I_SF = 0, I_TBS=16): We prefer TBS = 296 bits as to avoid a performance crossing issue with respect to TBS = 336 bits located in the next adjacent row (i.e., ITBS = 17). - Entry (I_SF = 1, I_TBS=14): We prefer TBS = 536 bits as to avoid a performance overlapping issue with respect to TBS = 600 bits located in the next adjacent row (i.e., ITBS = 15). - Entry (I_SF = 4, I_TBS=21): We prefer TBS = 2536 bits as to have the possibility of transmitting the max TBS in Rel-16 with half of the time-domain resources when 16-QAM is used. |
| Qualcomm | Our thoughts were to keep the WA till the next meeting to have some more time to verify the results. We do not have a strong view on the numbers in square brackets. |
| Nokia, NSB | We are OK to confirm the working assumption. We have the following preferences – - (I_SF = 0, I_TBS=16) – 328 bits - (I_SF = 1, I_TBS=14) – 552 bits - (I_SF = 4, I_TBS=21) – 2472 bits We prefer to use values from the existing LTE table. |
| Lenovo, MotoM | We share the same view as Nokia and OK to confirm the working assumption. We have the following preferences – - (I_SF = 0, I_TBS=16) – 328 bits - (I_SF = 1, I_TBS=14) – 552 bits - (I_SF = 4, I_TBS=21) – 2472 bits We prefer to use values from the existing LTE table. |
| ZTE,sanechips | We are OK to confirm the working assumption and values from the legacy LTE table are preferred for us. |
| HW, HiSilicon | We are OK to confirm the working assumption. We have the following preferences – - (I_SF = 0, I_TBS=16) – 328 bits - (I_SF = 1, I_TBS=14) – 552 bits - (I_SF = 4, I_TBS=21) – 2472 bits We prefer to use values from the existing LTE table. |
| MTK | We keep the same view as Nokia and Lenovo, MotoM,ZTE,HW |

According to the comments, the following is proposed:

Proposal: the following working assumptions is updated as below

Working Assumption

The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

## Issue 2: The breaking point from QPSK to 16QAM

The following has been achieved:

Working Assumption

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

Please input your comments regarding the following:

- Whether the working assumption can be confirmed

- Whether the brackets of [17] can be removed

| Companies | Comments |
| --- | --- |
| Ericsson | - Working assumption: Similar comment as for issue 1, although based on our simulation results the WA can be confirmed (including removing the brackets of [17]), we respect the fact that other companies need to time to check. The WA states “TBS entries between 14 ..and 21 …”, if there is a chance it would be good to indicate that 14 and 21 refer to indices by stating “TBS entries between I_TBS = 14 ..and I_TBS = 21 …”. |
| Qualcomm | Same answer as issue 1 |
| Nokia, NSB | We are OK to confirm the working assumption. We support removing the bracket from [17]. |
| Leonvo, MotoM | We are OK to confirm the working assumption. We support removing the bracket from [17]. |
| ZTE,sanechips | In LTE, the modulation order is switch to 16QAM at TBS9 (1544 bits for 10PRBs) which has the code rate of round 0.6 for QPSK modulation. And TBS 9 is overlapped in two adjacent MCS entries with QPSK and 16QAM. This is because the design of switching point considers both AWGN and fading channel. In AWGN channel, for a TBS, low-order modulation has better performance than high-order modulation. However, in a fading channel, the performance of a MCS with lower modulation order and higher code rate may be worse than that of a MCS with higher modulation order and lower code rate because some deep fading data is more difficult to correct at very high code rate. In WA, the switching point is set to TBS14 only based on AWGN simulation results for NB-IoT. The code rate of TBS 14 is round 0.9 for QPSK modulation, which is significantly larger than that of LTE switching point. It may not be conducive to transmission performance. In our opinion, the modulation switching point for NB-IoT should follow LTE design principle. That is, the modulation switching point also should be based on fading channel. According to our simulation results, TBS 12 (2280 bits for I_SF=7) can be switch to 16QAM for guard-band and standalone deployment. And TBS 10 (1736 bits for I_SF=7) can be switch to 16QAM for in-band deployment. So, we propose to modify the work assumption as following: - For standalone and guardband deployments, the downlink TBS entries between [14, 12] (TBS of [2856, 2280] for I_SF=7) and 21 are used for 16QAM. - For inband deployments, the downlink TBS entries between [11, 10] (TBS of [2024, 1736] for I_SF=7) and [17] are used for 16QAM. |
| HW, HiSilicon | We are OK to confirm the working assumption. We support removing the bracket from [17]. |
| MTK | We are ok to confirm the WA. ZTE’s analysis is reasonable. For simplicity, we are a little bit preferable that TBS entries between14 and 21 are used for 16QAM. Thus R17 and legacy terminal keep same performance when TBS entry is less than 14. |

## Issue 3: The TBS table for UL

The following has been achieved:

Working Assumption

The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Please input your comments regarding the following:

- Whether the working assumption can be confirmed

- Your preference of the entry (I_SF = 5, I_TBS=19) and reasons

- Your preference of the entry (I_SF = 6, I_TBS=15) and reasons

- Your preference of the entry (I_SF = 6, I_TBS=16) and reasons

| Companies | Comments |
| --- | --- |
| Ericsson | - Working assumption: Yet again, although based on our simulation results the WA can be confirmed, we respect the fact that other companies need to time to check. - There is something wrong with “(I_SF = 5, I_TBS=19), (I_SF = 6, I_TBS=15), (I_SF = 6, I_TBS=16)” it seems the FL meant “(I_TBS = 19, I_RU=5), (I_TBS = 15, I_RU=6), (I_TBS = 16, I_RU=6)” - Entry (I_TBS = 19, I_RU=5): TBS = 2536 bits is ok to have since according to our results no overlapping nor performance crossing issue have been observed. - Entry (I_TBS = 15, I_RU=6): We prefer in this entry to have a TBS = 2472 bits as to be able to use under it (i.e., in the next row) the larger transport block size that is allowed for UL. - Entry (I_TBS = 16, I_RU=6): TBS = 2536 bits is ok to have if the entry in the row above it (I_TBS = 15, I_RU=6) uses TBS = 2472 bits. |
| Qualcomm | Same answer as issue 1. |
| Nokia, NSB | We are OK to confirm the working assumption. Our preferences are – - (I_TBS = 19, I_RU=5) – 2536 bits is OK - (I_TBS = 15, I_RU=6) – 2472 bits - (I_TBS = 16, I_RU=6) – 2536 bits is OK |
| Lenovo, MotoM | We are OK to remove all brackets and (I_TBS = 15, I_RU=6) – 2472 bits |
| ZTE,sanechips | We are OK with the WA. For I_TBS = 15 and 16 for I_RU=6, 2536 is very close to 2472. This results in the code rate gap or SNR gap between TBS15 and 16 in this column is obvious smaller than that in other columns. So we prefer to have a TBS = 2536 bits in I_TBS = 15, I_RU=6. And the entry of I_TBS = 16, I_RU=6 is removed. |
| Huawei, HiSilicon | We are OK to confirm the working assumption and share similar preferences with Nokia. |
| MTK | We are ok to the WA and need more time to check which entry is preferable. |

According to the comments, the following is proposed:

Proposal: the following working assumptions is updated as below

Working Assumption

The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

## Issue 4: The MCS field

As the following agreement on repetition has been achieved:

Agreement

Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

Based on discussions, the following is proposed:

Possible Agreement

The MCS field for 16QAM is supported without increasing the DCI size.

Please input your comments

| Companies | Comments |
| --- | --- |
| Ericsson | At this point it seems hard to know if we are going to be able to fulfill the potential agreement 2, if in the end supporting 2 repetitions is found beneficial. We prefer to wait, and still leave open those DCI design aspects. |
| Qualcomm | We are OK with the agreement, but as Ericsson mentioned, it would be better to solve the repetition case first. Given the discussion this week, however, it seems complicated to agree (maybe companies need to go back and run some evaluations). |
| Nokia, NSB | We prefer to wait on this one |
| Lenovo, MotoM | We want to clarify that the MCS field for support 16QAM is still 4 bit? Or do you mean the DCI size will not increase for support 16QAM for downlink. |
| ZTE,sanechips | OK with the agreement. |
| HW, HiSilicon | We are OK with this agreement and whether 2 repetitions is beneficial or not requires more evaluations to be supported. And we are OK with this possible agreement. |
| MTK | We prefer to no repetition for 16QAM. Thus we are ok with the agreement. |

## Issue 5: Applicability of 16QAM for uplink

Based on the comments, the following is proposed:

Possible Agreement: 16QAM can be used at least for multi-tone transmission with 12 subcarriers.

- FFS: 3 and 6 subcarriers.

Please input your comments

| Companies | Comments |
| --- | --- |
| Ericsson | Ok. About the FFS, certainly these days we have been producing simulation results for 3 and 6 subcarriers and we found that the performance of 3/6 subcarriers is very close to the performance of 12 subcarriers, so it seems ok to support 3 and 6 subcarriers for 16-QAM (we can already share the results if according with the FL there is time for it). |
| Qualcomm | OK with the proposal. |
| Nokia, NSB | OK with the proposal |
| Lenovo, MotoM | OK with the proposal, and OK to support 3,6,12 tones. |
| ZTE,sanechips | OK with the proposal. |
| HW, HiSilicon | OK with the proposal. We also support 3,6 subcarriers because it is beneficial in terms of scheduling. |
| MTK | OK with the proposal. |

## Issue 6: soft buffer size

Based on the comments, it is proposed to discuss the soft buffer size for 16QAM-capable UEs. Based on the proposals, the following is proposed:

Possible Agreement: The soft buffer size is doubled with respect to QPSK.

Please input your comments

| Companies | Comments |
| --- | --- |
| Ericsson | We prefer not to make any premature agreement on the soft buffer size without having had a proper technical discussion on it. We prefer to treat this topic till the next meeting. |
| Qualcomm | We think the agreement is the only alternative. The soft buffer size in the DL for NBIOT is given by the number of REs x bits/RE, which is doubled with respect to the previous release (not that NBIOT does not have RV in the downlink). The only way the soft buffer size could be increased further is to have N_SF > 10. |
| Nokia, NSB | We have similar view as Qualcomm but probably better to consider this issue in the next meeting. |
| Lenovo, MotoM | Prefer to discuss in next meeting. |
| ZTE,sanechips | OK with the proposal. |
| HW, HiSilicon | We are OK with this possible agreement since it is an automatic consequence for the introduction of 16QAM. |
| MTK | We prefer to treat it in next meeting. |

Issues

Support of 16QAM for NB-IoT downlink

### Issue 1: The maximum TBS to support 16-QAM for unicast in DL for standalone and guardband.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 1: 5736 bits with ISF = 7 (i.e. option 3) is the maximum TBS to support 16-QAM for unicast in DL for standalone and guard-band deployments. |
| [3] | Proposal 4: For stand-alone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits for both 1-HARQ and 2-HARQ UEs. |
| [4] | Observation 1: For guard-band/standalone deployment, the TBS of 5736 bits cannot be applied for 2Tx antennas since the code rate of 5736 exceeds the upper limit of 0.932 when 2 NRS ports are configured. Proposal 1: 4968 bits with ISF=7 can be defined as the maximum TBS for DL 16QAM in guard-band/standalone deployment. |
| [5] | Proposal 2: For all operation modes, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7. |
| [6] | Observation 1 On the new max TBS to be supported for 16-QAM in DL, “Option 1: 4968 bits with ISF =7” seems to be a better choice as to avoid using a TBS that has not been previously used in the standard, or going beyond twice the max TBS in Rel-16. Proposal 1 The maximum TBS to support 16-QAM for unicast in DL for stand-alone and guard-band deployments is “Option 1: 4968 bits with ISF =7”. |
| [8] | Proposal 1: The maximum TBS for DL 16-QAM is 5736. - Target a maximum code rate of ~0.9 for all cases (deployment scenarios and N_SF) |
| [9] | Proposal 5: For all deployment scenarios, the max DL TBS should be 4968. |

For the following options:

- Option 1: 4968 bits with ISF=7

  - Nokia, Nokia Shanghai Bell, ZTE, Lenovo, Motorola Mobility, Ericsson, Sierra Wireless

- Option 2: 5072 bits with ISF=7

- Option 3: 5736 bits with ISF=7

  - Huawei, HiSilicon, Qualcomm,

Based on the intpus, the following is proposed:

Proposal 1: At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is down-selected from following options in RAN1#103e:

- Option 1: 4968 bits with ISF=7

- Option 3: 5736 bits with ISF=7

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Option 1: 4968 bits with ISF=7” seems to be a better choice as to avoid going beyond twice the max TBS in Rel-16 (as it happens with Option 3), which won’t be in line with the increase in the number of bits per M-ary symbols introduced by 16-QAM with respect to QPSK. |
| Qualcomm | We do not see the need to go with Option 1. For standalone/guard band, Option 3 is attainable. Note that in LTE, when we introduce a new higher peak data rate, there is always the notion of adding this MCS that works in ideal conditions. A possible compromise would be to add an RRC parameter to switch between these two (e.g. similar to entries 26/26A in the LTE TBS table) |
| Mediatek | We have no strong view on this. In view of cost, option1 seems to be better. |

The following has been achieved in this meeting:

Agreement

At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7.

### Issue 2: The maximum TBS to support 16-QAM for unicast in DL for inband.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 5: For in-band deployments, the maximum TBS to support 16-QAM in DL is 3624 bits. |
| [3] | Proposal 5: For in-band deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits for both 1-HARQ and 2-HARQ UEs. |
| [4] | Proposal 6: In-band uses the same DL TBS table as guard-band/standalone for DL 16QAM. - TBS configured for in-band should be less than or equal to TBS16 i.e. 3240 bits with ISF=7. |
| [6] | Proposal 3 For in-band deployments, the TBS/MCS Table to support 16-QAM is a sub-case of the TBS/MCS Table used for stand-alone and guard-band deployments. • The entries for 16-QAM ranges from a TBS = 176 bits to TBS = 3624 as to keep below 0.88 the achievable code rates in in-band deployments. |

Based on the inputs, there are following options for maximum TBS for inband deployment:

- Option 1: 3624 bit

  - Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

- Option 2: 3240 bit

  - ZTE,

Based on the majority view, the following is proposed:

Proposal 2: For inband deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | We believe that is possible to have Option 1 as max TBS for in-band deployments since it will be suitable when there are “2 NRS ports and 2 CRS ports”, whereas for dealing with the scenario pointed out by ZTE (i.e., 2 NRS ports and 4 CRS ports), then the eNB won’t schedule the UE with I_TBS = 17. |
| Qualcomm | After we agree on Issue#1, we can agree on the maximum code rate and then decide on this. |

The following has been achieved in this meeting:

Agreement

For inband deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits (ISF=7).

### Issue 3: The TBS table.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Table 1 An example of TBS table for DL 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. 22; 520; 1064; 1608; 2152; 2664; 3240; 4264; 5352. 23; 552; 1128; 1736; 2280; 2856; 3496; 4584; 5736. Proposal 3: Adopt table 1 as the TBS design to support 16-QAM in DL. Proposal 2: I_SF>7 for 16-QAM is not supported. |
| [3] | Table 1. DL TBS table with 16-QAM. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. Proposal 6: Keep all existing MCS values in the TBS table and extend the TBS table to support 16-QAM. The TBS table for DL 16-QAM is given in Table 1. |
| [4] | Table 2: Extended TBS entries for DL 16QAM 0; 1; 2; 3; 4; 5; 6; 7. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. Proposal 2: TBS table for DL 16QAM can be expanded to 0~21 based on Table 16.4.1.5.1-1 of [2]. - Reuse the existing TBS 0~13 in Table 16.4.1.5.1-1 - Reuse TBS 14~21 in LTE TBS table |
| [6] | Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 16-QAM only; 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2536; 2984; 4008; 4968. Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 16-QAM only; 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; -; -; -; -; -; -; -; -. 19; -; -; -; -; -; -; -; -. 20; -; -; -; -; -; -; -; -. 21; -; -; -; -; -; -; -; -. |
| [9] | Proposal 1: TBS table(s) should be designed to support data rates of at least 180 kbps for all deployment scenarios (i.e. in-band, guard band, stand-alone) Proposal 2: The same TBS table should be used for all deployment scenarios Proposal 3: For the SA/GB scenario, the TBS table should be designed such that the code rate is always <= 0.85. Proposal 4: To support the in-band scenario, the maximum number of SFs (NSF) per TBS should be 15 (i.e. up from 10). ITBS; ISF (NSF). 0 (1); 1 (2); 2 (4); 3 (6); 4 (8); 5 (10); 6 (12); 8 (15). 0; 16; 32; 88; 152; 208; 256; 328; 392. 1; 24; 56; 144; 208; 256; 344; 424; 520. 2; 32; 72; 176; 256; 328; 424; 520; 648. 3; 40; 104; 208; 328; 440; 568; 680; 872. 4; 56; 120; 256; 408; 552; 696; 840; 1064. 5; 72; 144; 328; 504; 680; 872; 1032; 1320. 6; 328; 176; 392; 600; 808; 1032; 1224; 1544. 7; 104; 224; 472; 712; 968; 1224; 1480; 1800. 8; 120; 256; 536; 808; 1096; 1384; 1672; 2088. 9; 136; 296; 616; 936; 1256; 1544; 1864; 2344. 10; 144; 328; 680; 1032; 1384; 1736; 2088; 2664. 11; 176; 376; 776; 1192; 1608; 2024; 2408; 2984. 12; 208; 440; 904; 1352; 1800; 2280; 2728; 3368. 13; 224; 488; 1000; 1544; 2024; 2536; 3112; 3880. 14; 256; 552; 1128; 1736; 2280; 2856; 3496; 4264. 15; 280; 600; 1224; 1800; 2472; 3112; 3624; 4584. 16; 328; 632; 1288; 1928; 2600; 3240; 3880; 4968. 17; 336; 696; 1416; 2152; 2856; 3624; 4392. 18; 376; 776; 1544; 2344; 3112; 4008; 4776. 19; 408; 840; 1736; 2600; 3496; 4264. 20; 440; 904; 1864; 2792; 3752; 4584. 21; 488; 1000; 1992; 2984; 4008; 4968. Proposal 6: Adopt the above TBS table to support max TBS =4968 for all deployment scenarios (in-band, standalone, guard band) |

Based on the inputs, the following is proposed:

Proposal 3: to support 16QAM for NB-IoT DL, the following TBS indices are introduced

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS for I_SF > 7

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Proposal 3 should say somewhere to which deployment mode(s) it refers to, but more importantly before discussing the TBS/MCS table we need to know the outcome of proposal 1 and 2. |
| Qualcomm | As we explained in our contribution, agreeing on the MCS/TBS table can be done offline after the following decisions are taken: - Maximum TBS / code rate. - Deployments to be supported. - Switching point. Based on our previous experience, this should not be controversial. |
| ZTE,sanechips | Since the TBS 4968bits for the DL has been supported, the above table now can be introduced for stand-alone and guard-band deployments. However, it is noted the should be no less than 17 for in-band deployment. As for the switching point, from our point view, it shall be discussed in the MCS table design. |
| Huawei, HiSilicon | It is clear that inband deployment can not support all ITBS indexes in the above table due to code rate, thus it needs to clarify for inband mode. |
| Nokia, NSB | We are fine with the FL’s proposal. We agree the above new rows are for stand-alone and guard-band deployment. For in-band, we think the same table can be used and we just limit the applicable rows. |

The following has been achieved:

Working Assumption

The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

### Issue 4: The breaking point from QPSK to 16QAM.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [3] | Proposal 7: For stand-alone and guard-band deployments, the break-point for 16-QAM in DL is I_TBS ≥ 14 based on Table 1. Proposal 8: For in-band deployment, the break-point for 16-QAM in DL is I_TBS ≥ 11 based on Table 1. |
| [4] | Observation 2: For NPDSCH in guard-band/standalone deployment, 16QAM performance is slightly better than QPSK performance at TBS= 2024 with ISF=7, i.e. TBS 11. Proposal 5: TBS 11 or 12 could be adopted as DL 16QAM switching point for guard-band/standalone deployment. Observation 3: For NPDSCH in in-band deployment, 16QAM performance is better than QPSK performance at TBS=1736 with ISF=7, i.e. TBS 10. Proposal 7: TBS 10, i.e. 1736 bits with ISF=7, could be adopted as DL 16QAM switching point for in-band deployment. |
| [6] | - The break point between different modulation schemes The average SINR difference at the breaking-point is ITBS14_16QAMavg-to-ITBS13_QPSKavg = abs(8.94 dB – 8.28 dB) ⁓ 0.66 dB. - The break point between different modulation schemes The average SINR difference at the breaking-point is ITBS11_16QAMavg-to-ITBS10_QPSKavg = abs(8.98 dB – 8.08 dB) ⁓ 0.9 dB. |
| [8] | Observation 1: The breakpoint between QPSK and 16-QAM for DL is at approximately at 1.8 (total bits)/#REs (code rate of 0.9 for QPSK) Observation 2: For a given TBS, the optimum modulation scheme is different for different deployment scenarios. Proposal 2: For downlink, specify different MCS/TBS tables for different deployment scenarios (to optimize the switching point between 16-QAM and QPSK). The switching point between QPSK and 16-QAM is at approximately at (total bits)/#REs ≈1.65 Consider at least the following scenarios: - In-band with [3] symbol control, 2/4 port CRS - Guard-band/standalone with 0 symbol control |

On the switching point from QPSK to 16QAM for standalone and guardband deployments, there are following options:

  - The TBS entries of 14 (TBS of 2856 for I_SF=7) and above are used for 16QAM

    - Nokia, Nokia Shanghai Bell, Ericsson, MediaTek

  - The TBS entries of [11 (TBS of 2024 for I_SF=7) or 12 (TBS of 2280 for I_SF=7)] and above are used for 16QAM

    - ZTE

For inband deployment, there are following options:

  - The TBS entries of 11 (TBS of 2024 for I_SF=7) and above are used for 16QAM

    - Nokia, Nokia Shanghai Bell, Ericsson

  - The TBS entries of 10 (TBS of 1736 for I_SF=7) and above are used for 16QAM

    - ZTE,

In addition, [8] also proposed that the switching point if at approximately at (total bits)/#REs≈1.65.

As not many companies provides Thus it is proposed:

Proposal 4: Different switching points are used for standalone/guradband and inband deployments.

  - FFS the details of the switching point.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Proposal 4 is fine for us as starting point. As a side comment, we noticed that in the table summarizing the inputs from different companies, our input was missed since we used as a metric the average SINR gap as to select the breaking point (e.g., For stand-alone and guard-band deployments: The average SINR difference at the breaking-point is ITBS14_16QAMavg-to-ITBS13_QPSKavg = abs(8.94 dB – 8.28 dB) ⁓ 0.66 dB). |
| Qualcomm | It is unclear what “switching points” means. If it is a “switching point” based on code rate, it should be the same for all deployments. If it is based on TBS entry, it would be different for different deployments. I think we agree on this. |

The following has been achieved:

Agreement

Different breaking points (QPSK16QAM) are used for standalone/guardband and inband deployments.

  - FFS the details of the breaking point.

As most inputs on the breaking point depends on proposed TBS tables by companies, the breaking point of different deployments can be discussed after the TBS table is decided.

Observation 1: the breaking point from QPSK to 16QAM can be discussed after TBS table is decided.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| ZTE,sanechips | Regarding the simulation assumption for switching point evaluation, the fading channel with 1Tx/2Tx shall be assumed for more realistic simulation at least. |
| Huawei, HiSilicon | Fine with this observation. For simulation assumption, generally AWGN is assumed e.g. in LTE based on our understanding. |
| Mediatek | There is a clear definition on “switching point” that it indicates a SINR point beyond which 16QAM will has much better BLER than QPSK. Agree with AWGN is assumed for this simulation. |
| Nokia, NSB | We are fine with the FL’s observation. We are OK to use AWGN for the simulations. |

With the proposed TBS, the following is proposed for switching point:

Proposal 4A: For standalone and guardband deployments, the TBS entries of 14 (TBS of 2856 for I_SF=7) and above are used for 16QAM.

Proposal 4B: For inband deployments, the TBS entries of 11 (TBS of 2024 for I_SF=7) and above are used for 16QAM.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

The following has been achieved:

Working Assumption

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

### Issue 5: Applicability

The following are proposed on scheduling of TBS and modulation:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 4: Repetition is not supported for 16-QAM in DL. |
| [3] | Proposal 11: Support 16-QAM with repetition in the DL. Proposal 12: Support modulation adjustment to QPSK when 16-QAM is scheduled with repetition in the DL. |
| [4] | Observation 4: For DL, 16QAM with 2 repetitions can still provide a higher peak data rate compared to QPSK. But 16QAM may show worse data rate after the number of repetitions increases to 4. Proposal 8: At least 2 repetitions should be supported for DL 16QAM. |
| [7] | Observation: Repetition for 16QAM has much worse performance under AWGN. |
| [8] | Proposal 5: DL 16-QAM is only applicable for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI. - At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS. Proposal 6: 16-QAM NPDSCH is only supported for R=1. - FFS whether to support the new TBSs with QPSK and increased RU |

As there are diverse views, the following is proposed to collect more views

Proposal 5: Further study on the following to support 16QAM for NB-IoT DL:

  - Whether repetition can be used for 16QAM in DL.

  - Whether DL 16QAM is only used for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI from USS.

  - Whether to support the new TBS with QPSK and increased RU.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | It seems to us that whether to support 16-QAM with or without repetitions, should have its own discussion. |
| Qualcomm | - Whether repetition can be used for 16QAM in DL. No - Whether DL 16QAM is only used for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI from USS. Yes - Whether to support the new TBS with QPSK and increased RU. Need further discussion |
| ZTE,sanechips | We are fine with the proposal. |
| Huawei, HiSilicon | For 2nd sub-bullet, we are fine. For 3rd sub-bullet it is clear that the soft buffer size may be increased if RU is increased. |
| Nokia, NSB | We are fine with the proposal |

On whether repetition can be used for 16QAM, there are following options:

  - Repetition is not supported for 16-QAM in DL

    - Huawei, HiSilicon, MediaTek, Qualcomm

  - Support 16-QAM with repetition in the DL

    - Nokia, Nokia Shanghai Bell, ZTE (at least 2 repetitions)

Based on the inputs, the following is proposed:

Proposal 5A: further study whether repetition can be supported for 16QAM in DL according to data rates.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| ZTE,sanechips | Yes. For DL, 16QAM with 2 repetitions can still provide a higher peak data rate than the QPSK. So, 16QAM with 2 repetitions can be considered. |
| Huawei, HiSilicon | We prefer to not consider since 16QAM is generally used for good coverage UEs. But we are open to further discuss if there is some gains. And we think there is no need to consider >2 repetitions anymore. |
| Mediatek | As our simulation result indicated, 16QAM with repetition is much worse under AWGN at least. It’s maybe better that all of us provide simulation under fading channel to do final decision. We prefer to 16QAM without repetition. |
| Nokia, NSB | We support using 16QAM with repetition in order to increase data rate for UEs that may not have sufficient SNR to use 16QAM |

The following has been achieved:

Agreement

Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

Support of 16QAM for NB-IoT uplink

### Issue 6: The TBS/MCS table to support 16-QAM for unicast in UL.

There are following proposals on TBS design of 16-QAM for UL unicast

| Sourcing | proposals |
| --- | --- |
| [2] | Table 4 An example of TBS table for UL 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1000. 7; 104; 224; 328; 472; 584; 712; 1000; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1000; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 1000; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2472. 16; 328; 632; 968; 1288; 1608; 1928; 2536. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152; 2536. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2536. Proposal 6: Adopt table 4 as the TBS design to support 16-QAM in UL. |
| [3] | Table 2. UL TBS table with 16-QAM. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2472. 16; 328; 632; 968; 1288; 1608; 1928. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2472. Proposal 13: Keep all existing MCS values in the TBS table and extend the TBS table to support 16-QAM. The TBS table for UL 16-QAM is given in Table 2. |
| [4] | Proposal 10: For UL 16QAM, TBS table can be extended to up to 2536 bits for each IRU. Table 6: TBS table for UL 16QAM 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 696. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 328; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 712; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1000; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2536. 16; 328; 632; 968; 1288; 1608; 1928. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152; 2536. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2536. |
| [5] | Proposal 8: Support 16QAM for NPUSCH needs further study: - Option1: Extend TBS table and generate modulation, TBS and MCS table. - Option2: Reinterpret the number of resource unit for modulation order of 16QAM. |
| [6] | Proposal 8 The TBS/MCS Table to support 16-QAM in UL consists of: - All legacy QPSK entries. - The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 280 bits to TBS = 4968, with only two changes: o In the above-mentioned TBS range, the TBS > 2536 are excluded as to be compliant with the objective of preserving for UL the max TBS for Rel-16. o To transmit the max Rel-16 TBS with half of the time domain resources replacing TBS = 2472 bits by TBS = 2536 bits. |
| [8] | Proposal 12: RAN1 to discuss whether to introduce one or more “implicit MCS” for retransmissions in the MCS table for UL 16-QAM. |

Based on the inputs, the following can be proposed:

Proposal 6: to support 16QAM for NB-IoT UL, at least the following TBS indices are introduced

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | [2472,2536] |  |  |  |

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | It is probably good to clarify as part of proposal 6 that the highlighted inputs in proposal 6 are meant to be removed. Apart from that, and given that for UL max TBS is already know from the WID, proposal 6 looks fine with its embedded place holders (i.e., []). |
| ZTE,sanechips | Agreed |
| Huawei, HiSilicon | Fine with this proposal. We support 2536 in ITBS index of 16,19,21 as the code rate is similar with others in same row. |
| Nokia, NSB | We are fine with the FL’s proposal |

The following has been achieved:

Working Assumption

The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

### Issue 7: The breaking point from QPSK to 16QAM.

There are following proposals on TBS design of 16-QAM for UL unicast

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 15: The break-point for 16-QAM in UL is I_TBS ≥ 14 based on Table 2. |

As not many companies provide input, the following is proposed:

Proposal 7: further study on the breaking point from QPSK to 16QAM for NB-IoT UL.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | If proposal 6 proceeds further, then somehow it provides a breaking point unless one of the TBS entries used for QPSK will be also used for 16-QAM. |
| ZTE | Fine with the proposal. BTW, for the UL, to obtain the breaking point by evaluation, the fading channel should be assumed for more realistic simulation at least. |
| Huawei, HiSilicon | Fine. For simulation assumption, generally AWGN is assumed e.g. in LTE based on our understanding. |
| Mediatek | Agree AWGN is assumed for this simulation. |
| Nokia, NSB | We are fine with the FL’s proposal |

### Issue 8: Applicability

The following are proposed on scheduling of TBS and modulation:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 7: Repetition is not supported for 16-QAM in UL. Proposal 8: 16-QAM can be used for NPUSCH with at least multi-tone 3, 6 and 12 subcarriers. |
| [3] | Proposal 16: FFS whether to Support 16-QAM with repetition in the UL. Proposal 17: 16-QAM is not supported for sub-PRB allocation. |
| [4] | Proposal 9: UL 16QAM should be supported for 3/6/12 subcarriers allocation. |
| [6] | Observation 2 Although 16-QAM requires a high SNR and resource allocations < 12 subcarriers are mainly targeted towards low SNR regimes (especially single-tone allocations), still there are scenarios (e.g., NPRACH and NPUSCH coexistence) where multi-tone allocations could benefit from a higher order modulation. Proposal 7 The support of 16-QAM in UL is only for NPUSCH Format 1 using both full-PRB allocations and multi-tone allocations consisting of 6 and 3 allocated subcarriers. |
| [8] | Proposal 15: UL 16-QAM is applicable for NPUSCH scheduled from a DCI with CRC scrambled by C-RNTI. - At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS. - FFS: Applicability of 16-QAM for PUR or EDT. Proposal 16: UL 16-QAM is applicable at least to NPUSCH with full-PRB allocations. FFS NPUSCH with sub-PRB allocations. |

On the application of 16QAM to full PRB/sub-PRB allocations, there are following options:

  - Multi-tone transmission with 3, 6 and 12 subcarriers

    - Huawei, HiSilicon, ZTE, Ericsson,

  - Multi-tone transmission with only 12 subcarriers

    - Nokia, Nokia Shanghai Bell,

  - FFS on sub-PRB allocation

    - Qualcomm

Therefore, the following is proposed based on the input:

Proposal 8: RAN1 to further discuss on the applicability to different number of subcarriers.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with a further discussion on it. |
| ZTE,sanechips | Actually, based on the above opinions, the 12 subcarrier can be supported firstly. FFS on 3 and 6 subcarriers. |
| Huawei, HiSilicon | We support 3,6,12 subcarriers because it is beneficial in terms of scheduling. And we are also fine to further discuss. |
| Nokia, NSB | We are fine with the FL’s proposal |

DCI

### Issue 9: MCS field.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 9: The introduction of 16-QAM shall not increase the NPDCCH blind decodes. Proposal 10: The introduction of 16-QAM shall avoid increasing DCI size. |
| [3] | Proposal 9: The size of the MCS field in DCI N1 in UE-specific search space is increased to 5 bits. Proposal 14: The size of the MCS field in DCI N0 in UE-specific search space is increased to 5 bits. |
| [4] | Proposal 3: 4-bit MCS table could be defined for DL 16QAM in guard-band/standalone deployment. Table 3: Possible 4-bit MCS table for DL 16QAM MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 2. 2; 2; 4. 3; 2; 6. 4; 2; 8. 5; 2; 10. 6; 4; 12. 7; 4; 13. 8; 4; 14. 9; 4; 15. 10; 4; 16. 11; 4; 17. 12; 4; 18. 13; 4; 19. 14; 4; 20. 15; 4; 21. Proposal 4: 4-bit MCS table for DL 16QAM can be defined based on the MCS entries of Rel-16 NB-IoT. - Remove the existing 6 MCS entries - Add new 8 MCS entries Proposal 11: 5-bit MCS table could be used for UL 16QAM. - MCS 0~13 correspond to TBS 0~13 with QPSK modulation - MCS 14~24 correspond to TBS 11~21 with 16QAM modulation Table 7: Possible 5-bit MCS table for UL 16QAM MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 1. 2; 2; 2. 3; 2; 3. 4; 2; 4. 5; 2; 5. 6; 2; 6. 7; 2; 7. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12; 2; 12. 13; 2; 13. 14; 4; 11. 15; 4; 12. 16; 4; 13. 17; 4; 14. 18; 4; 15. 19; 4; 16. 20; 4; 17. 21; 4; 18. 22; 4; 19. 23; 4; 20. 24; 4; 21. 25~31; reserved; reserved. Proposal 12: The most significant bit of the subcarrier indication field can be used for UL 5-bit MCS indication. |
| [5] | Proposal 3: To support 16QAM of NPDSCH, the MCS field in DCI format N1 is enlarged, optimized or reinterpreted, which needs further discussion. |
| [7] | MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 1. 2; 2; 2. 3; 2; 3. 4; 2; 4. 5; 2; 5. 6; 2; 6. 7; 2; 7. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12; 2; 12. 13; 2; 13. 14; 4; 13. 15; 4; 14. 16; 4; 15. 17; 4; 16. 18; 4; 17. 19; 4; 18. 20; 4; 19. 21; 4; 20. 22; 4; 21. Table 2: DL MCS table for NB-IOT 16QAM Proposal: 5-bit MCS should be adopted and Table 2 should be DL MCS table |
| [9] | Proposal 7: Increase the Modulation and coding scheme DCI field from 4 to 5 bits |

On the size of the MCS field, there are following options:

  - 4 bits as legacy

    - Huawei, HiSilicon, ZTE (for DL),

  - 5 bits

    - Nokia, Nokia Shanghai Bell, ZTE (for UL), MediaTek, Sierra Wireless, Ericsson

Based on the majority view, the following is proposed

Proposal 9: The MCS field to support 16QAM is increased from 4bits to 5 bits.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Although there seems to be a trend towards 5-bits, it seems that we need to know the outcome of proposal 1, 2 and 3 for DL (and proposal 6 for UL) as to know whether 4-bits or 5-bitsd will be required. |
| Qualcomm | If we restrict the number of repetitions with 16-QAM to 1, probably we do not need to introduce additional bits, since some DCI codepoints can be reused. |
| ZTE,sanechips | For the DL, 4 bits MCS field was proposed since we do not want to increase the DCI size. We are also fine with the 5bits MCS scheme, if the DCI size is not increased. So maybe the modified proposal can be shown as following: Proposal 9: The MCS field to support 16QAM is increased from 4bits to 5 bits without increasing the DCI size. For the UL, 5 bits MCS field is OK with us, since the reserved bits can be re-purposed. |
| Huawei, HiSilicon | We prefer to not increase the DCI size, so we are fine to firstly agree the principle to not increase the DCI size. For reusing the repetition field, it may have impact to legacy QPSK scheduling since it limits the number of repetitions for QPSK. There may have problems when channel condition becomes worse. Note that in NB-IoT there is no CSS for fallback. |
| MTK | Yes, agree with there is no increase to the DCI size, how to support 5bit MCS needs FFS. |
| Nokia, NSB | We are fine with the FL’s proposal. We think it is fine to increase the MCS field size from 4 to 5 bits (i.e. to support up to 32 rows). We understand it may be possible to do this without increasing the DCI size and we can discuss further. |

Based on the comments, the following is proposed:

Proposal 9A: The MCS field to support 16QAM is increased from 4bits to 5 bits without increasing the DCI size.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Downlink power allocation to support 16QAM

### Issue 10: Power allocation.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 11: The ratio of NPDSCH EPRE to NRS EPRE should be signaled for the following cases: - NPDSCH in symbols without NRS and CRS - NPDSCH in symbols with CRS (only for “In-band” deployment) - NPDSCH in symbols with NRS |
| [4] | Proposal 13: The same total transmit power is set for each OFDM symbol for Rel-17 DL power allocation. Proposal 14: Only a new higher layer parameter Pn which indicates the ratio of NPDSCH EPRE and NRS EPRE in symbols with NRS needs to be introduced for Rel-17 DL power allocation. |
| [5] | Proposal 5: Network should semi-statically configure three types of NPDSCH EPRE separately. |
| [6] | Proposal 4 The data-to-power ratios for 16-QAM in DL, uses as baseline the DL power control definitions in LTE  Stand-alone and Guard-band deployments: - Type A refers to the NPDSCH symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB] - Type B refers to the NPDSCH symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]  In-band deployments: - Type A refers to the NPDSCH symbols without NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB] - Type B refers to the NPDSCH symbols with NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB] - Type C refers to the NPDSCH symbols without NRS and with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB] Where: ρ_a = PA [dB] PB is the index that refers to the linear ratio between ρ_b and ρ_a PC is the index that refers to the linear ratio between ρ_c and ρ_a FFS: signaling details of ρ_a, ρ_b, ρ_c. or PB, PC. |
| [8] | Observation 3: In NB-IoT, the power level change of NPDSCH relative to NRS does not have impact on legacy NPDSCH with QPSK. This does not hold anymore with 16-QAM NPDSCH. Proposal 8: Define three different levels ρA,ρB,ρC of EPRE of NPDSCH with respect to EPRE of NRS: - ρA: Applicable to NPDSCH in symbols with NRS. - ρB: Applicable to NPDSCH in symbols with CRS (required for in-band NB-IoT only). - ρC: Applicable to NPDSCH in symbols without NRS and CRS. Proposal 9: RAN1 to decide among the following alternatives: - Alt1: Rel-16 NRS power levels are kept (ρA=ρC). - Alt2: An additional “power boost” value for NRS is introduced (ρA≠ρC). Proposal 10: The UE derives the values of ρA, ρB, ρC implicitly based on - Power boost value for NRS (if introduced) - NRS and CRS relative power level. - Number of NRS and CRS ports. |

Based on the input, the following is proposed:

Proposal 10: The signal of ration of NPDSCH EPRE to NRS EPRE for following cases explicitly or implicitly are supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | NRS EPRE is different for anchor and non-anchor carriers, it would be good to clarify if 16-QAM will be supported for non-anchor carriers. Moreover, proposal 10 says “The signal of ration”, is it meant to say the “the signaling of the power ratios …”? “for the following cases are supported” Another bullet at the bottom can be added to capture the “explicitly or implicitly” aspect, for example: - FFS: Signaling details. including if all or which cases are supported explicitly or implicitly. |
| Qualcomm | We agree. We also agree with the editorial changes from Ericsson. |

According to the comments, the proposal is updated as below:

Proposal 10A: The signaling of power rations of NPDSCH EPRE to NRS EPRE for the following cases explicitly or implicitly are supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| ZTE,sanechips | Seems OK |
| Huawei, HiSilicon | fine |
| Nokia, NSB | We are fine with the FL’s proposal |

The following has been achieved:

Agreement

Explicit or implicit signaling of power ratios of NPDSCH EPRE to NRS EPRE for the following cases is supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Configuration and capability

### Issue 11: capability

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 1: Support of 16-QAM by UE is indicated by UE capability signalling. |
| [5] | Proposal 1: Introduce UE capability signaling for the support of 16QAM for unicast NPDSCH. Proposal 6: Introduce UE capability signaling for the support of 16QAM for unicast NPUSCH. |
| [8] | Proposal 4: Introduce a unicast RRC parameter to enable 16-QAM for NPDSCH. Introduce an optional UE capability indicating support of DL 16-QAM. Proposal 11: Introduce a unicast RRC parameter to enable 16-QAM for NPUSCH. Introduce an optional UE capability indicating support of UL 16-QAM. |

Based on the input, the following is proposed:

Proposal 11: Support of 16QAM of DL and UL are indicated by an optional UE capability signaling separately.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Our interpretation of proposal 11 is that it decouples DL from UL, in that case it would be clearer to express it as Nokia did last time using separate statements: “Introduce an optional UE capability indicating support of DL 16-QAM.” “Introduce an optional UE capability indicating support of UL 16-QAM” |
| Qualcomm | We agree with separate parameters for UL / DL. |
| ZTE,sanechips | Agreed. |
| Huawei, HiSilicon | Agree, we also agree the changes from Ericsson |
| Mediatek | Agreed. |
| Nokia, NSB | Agreed. And also support Ericsson’s proposal to have separate statements for UL and DL capability. |

The following has been achieved:

Agreement (Proposal 11)

For 16-QAM in NB-IoT, separate optional UE capabilities for UL and DL are supported:

- The support of 16QAM in DL is indicated by an optional UE capability signaling.

- The support of 16QAM in UL is indicated by an optional UE capability signaling.

### Issue 12: configuration

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 2: 16-QAM feature is configured (e.g. enabled/disabled) via UE-specific RRC signalling. Proposal 3: Consider separate 16-QAM capability signalling and configuration for UL and DL. |
| [5] | Proposal 4: The configuration of 16QAM for NPDSCH can be enabled/disabled by eNB through RRC signaling. Proposal 7: The configuration of 16QAM for NPUSCH can be enabled/disabled by eNB through RRC signaling. |
| [8] | Proposal 4: Introduce a unicast RRC parameter to enable 16-QAM for NPDSCH. Introduce an optional UE capability indicating support of DL 16-QAM. Proposal 11: Introduce a unicast RRC parameter to enable 16-QAM for NPUSCH. Introduce an optional UE capability indicating support of UL 16-QAM. |

Based on the input, the following is proposed:

Proposal 12: 16QAM for UL and DL are configured by UE-specific RRC signaling separately.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | Similar comment as for proposal 11, it would be clearer to use separate statements. |
| Qualcomm | We agree with separate parameters for UL/DL. |
| ZTE,sanechips | Agreed. |
| Huawei, HiSilicon | agree |
| Mediatek | Agreed. |
| Nokia, NSB | We agree with the proposal and support Ericsson’s suggestion to have separate statements. |

The following has been achieved:

Agreement (Proposal 12)

For 16-QAM in NB-IoT, separate UE-specific RRC signaling for UL and DL are supported:

- 16QAM for UL is configured by UE-specific RRC signaling.

- 16QAM for DL is configured by UE-specific RRC signaling.

Channel quality reporting

### Issue 13: Channel quality reporting

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 10: Study further channel quality reporting to support 16-QAM in DL. In the WID, one of the objective is to extend the NB-IoT channel quality reporting based on the framework of Rel-14-16, to support 16-QAM in DL. We see two potential areas here – - Extend the channel quality report to cover the relevant spectral efficiency range for 16-QAM. - Extend the channel quality report to UE in CONNECTED mode (in addition to Msg3). |
| [4] | Proposal 15: For Rel-17 channel quality report, the CQI table needs to be discussed in RAN1. |
| [6] | Proposal 5 The CQI reporting definition to support 16-QAM in DL is as in clause 7.2.3 of TS 36.213 for LTE-MTC with the corresponding updates to adapt it to NB-IoT. Proposal 6 The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.213 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL. - The NPDCCH and NPDSCH repetition level is equal to 1. o candidateRep-M is reported when the SINR is suitable for 16-QAM with ITBS =A/D.  FFS: A for stand-alone and guard-band deployments, and D for in-band deployments. o candidateRep-N is reported when the SINR is suitable for 16-QAM with ITBS =B/E.  FFS: B for stand-alone and guard-band deployments, and E for in-band deployments. o candidateRep-O is reported when the SINR is suitable for 16-QAM with ITBS =C/F.  FFS: C for stand-alone and guard-band deployments, and F for in-band deployments. |
| [10] | Observation 1: In current NB-IoT, the channel quality reporting in Msg3 and connected mode are NPDCCH repetition level reporting. Observation 2: For UE in good coverage and hence not needing repetition, the repetition-level based channel quality reporting does not convey sufficiently fine-grained channel quality information. Proposal 1: Finer NB-IoT channel quality reporting is supported to provide sufficient channel quality information in good coverage, particularly for 16-QAM. Proposal 2: Re-purpose the channel quality reporting field in Msg3 and MAC CE to support finer channel quality reporting. |

As the channel quality report depends on the discussion of max DL TBS, application on repetition etc, the following is proposed:

Observation 2: The channel quality report depends on discussion of DL TBS and needs further discussion.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |
| Ericsson | The channel quality reporting topic is highly dependent on whether 16-QAM is going to use only 1 repetition or 1 and more repetitions. That is why for proposal 5 we mentioned that it seems to us that whether to support 16-QAM with or without repetitions, should have its own discussion. |
| ZTE,sanechips | The channel quality report with respect to CQI is beneficial to improve DL data rate and we agree on the further CQI discussion. Further, we hope that the CQI design can reuse the existing design in LTE system to avoid massive efforts. |
| Huawei, HiSilicon | We are fine to further discuss the channel quality report. |
| Mediatek | As we proposed in last meeting, reporting based on MAC has large latency, we should consider the L1 reporting mechanism. |

Others

### Issue 16: Others

There are also other proposals as below:

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 18: 16-QAM can be supported together with DL/UL multi-TB scheduling in unicast. Proposal 19: 16-QAM can be supported together with PUR. Proposal 20: 16-QAM is not supported for UL EDT. |
| [4] | Proposal 16: Soft buffer size needs to be specified based on maximum TBS for DL 16QAM. |
| [8] | Proposal 7: Do not introduce LBRM for 16-QAM. The soft buffer size is doubled with respect to QPSK. Proposal 14: RAN1 to consider adding an additional power control parameter to allow for increased power with 16-QAM (e.g. similar to ΔTF) |

Please input your comments if you think any proposed listed in this section (2.7) or any other issue can be discussed in this meeting:

| Companies | Comments |
| --- | --- |
| Ericsson | From the topics summarized under Issue 16, it seems that the “soft buffer” size topic is more fundamental. If there is time, it would be good to collect initial views on it. |
| Qualcomm | We also think soft buffer size / LBRM is very fundamental. |
| ZTE,sanechips | For NB-IoT, the UE soft buffer size issue is important which is related to the cost and complexity. We agree to collect the initial views. |
| Huawei, HiSilicon | Share same views as companies that we can discuss the soft buffer size if time is allowed in this meeting. |
| Mediatek | Proposal18 is relatively simple, Proposal19 needs more consideration on signaling. |
| Nokia, NSB | We are also supportive to discuss soft buffer size if time allows |

Summary

- References

- RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.

- R1-2007618	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2008073	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2008697	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2008920	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2008930	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2008969	Further considerations on support of 16QAM for NB-IOT	MediaTek Inc.

- R1-2009112	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2009125	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

- R1-2007620	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon