3GPP TSG RAN WG1 #103e                                                   R1-2009762

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	Discussion on DL PRS and SRS for Positioning Measurement Time ([103-e-NR-ePos-02])

Agenda item:	8.5.2

Document for:	Discussion and Decision

- Introduction

RAN WG2 has provided reply LS to RAN WG1 on latency analysis of NR positioning protocols in [1]. In summary document it was indicated that RAN WG2 is waiting for RAN WG1 outcome on DL PRS and SRS for positioning measurement times [2] (please refer to highlighted in yellow part in table below):

Table 1: Latency Components Label; Latency [ms]; Description. Processing Latencies. TUEProc-RRCReconf; 10; RRC Reconfiguration processing. TUEProc-RRCDLInfo; 5; RRC DL information transfer. TUEProc-RRCULInfo; 2-5; RRC UL information transfer. TUEProc-RRCLocationMeas; 2-5; RRC Location Measurement Indication. TUEProc-LPPCapab; 10-20; LPP Provide Capabilities. TUEProc-LPPAssi; 10; LPP Provide Assistance Data. TUEProc-LPPLocationRe; 5; LPP Request/Provide Location Information. TUEProc-MAC-SRSAct; 1-3; MAC-CE SRS Activation/Deactivation. TgNBProc-RRC; 3; RRC Processing. TgNBProc-NRPPa; 3; NRPPa Processing. TgNBProc-NAS/LPP; 3; NAS/LPP Processing. TAMFProc; 3; AMF Processing. TLMFProc; 3; LMF Processing. Signalling Propagation Delays between Nodes. TUE-gNB; 0-0.5. TgNB-AMF; 3-10. TAMF-LMF; 1-10. TAMF-GMLC; 3-10. Positioning Measurement Latencies. TLMF-Calc; 2-30; Position Calculation latency. TDL-Meas; Wait for RAN1; PHY DL-PRS measurement time; best possible case. TUL-Meas; Wait for RAN1; PHY UL-PRS measurement time; assume the same value as for DL-PRS..

- DL PRS Measurement Time

The following latency components need to be considered for DL PRS measurement time:

- DL PRS alignment time / MG alignment time

  - This time depends on DL PRS periodicity and MG alignment time

  - The following DL PRS periodicities are supported in Rel.16 –{4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240} 2μ

  - The following MG periods are supported in Rel.16 {20, 40, 80, 160} ms

  - Considering that the minimum MG time is 20ms it is reasonable to assume 20ms in a worst case scenario

- DL PRS transmission time

  - Depends on DL PRS resource configuration

    - ~0.5 ms can be assumed for dense scenarios

- DL PRS measurement time (measurement period requirements)

  - This time depends on UE capability and DL PRS configuration settings. It is defined in TS 38.133 in Section 9.9.2 for RSTD (please refer to Table 1 below) and 9.9.3 for RSRP

Table 1: UE measurement period requirements for DL PRS RSTD

TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast where: NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2 NRxBeam,i = [8]. CSSFPRS,i is the carrier-specific scaling factor for the positioning frequency layer i as defined in clause 9.1.5.2 as CSSFwithin_gap,i. Nsample is the number of PRS RSTD samples and Nsample= [4]. Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + LPRS,i , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi. LPRS,i is the time duration as defined in clause 5.1.6.5 of TS 38.214 [26, 5.1.6.5]. NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot. {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14]. N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14].

  - Main Assumptions for Analysis

The following assumptions are made to estimate measurement period:

- NRxBeam, i=1, - the number of RX beams is 1 at UE (FR1)

- NPRS,islotN'LPRS,iN=1, - UE can process all DL PRS resources in a slot

- Nsample = 4, - the measurement time is 4 DL PRS periods

- (N, T) = (0.5ms, 8ms)

- DL PRS periodicity and MGRP is 20ms => Tavailable_PRS, i =20ms

TPRS-RSTD,i=Nsample-1*Teffect,i+Tlast=3*Teffect,i+Tlast

Tlast = Ti + LPRS,i  = 0.5ms + 8ms,

Teffect,i = TiTavailable_PRS,i*TavailablePRS,i=20 ms

TPRS-RSTD,i = 68.5ms

  - Summary for DL PRS Measurement Time Analysis

- DL PRS alignment time/MG alignment time ~4-20ms (depending of DL PRS periodicity 4ms or or 20ms – worst case is assumed)

- DL PRS transmission time

  - Included in RAN4 analysis for measurement period requirements

- DL PRS measurements (measurement period requirements)

  - ~68,5 ms (CSSF = 1, NRxBeam, i=1, Nsample = 4, DL PRS Periodicity - 20ms)

- In total overall latency for DL PRS measurement time is 72.5 – 88.5 ms

  - Discussion Round #1

Proposal 1-1

- RAN1 to conclude that DL PRS measurement time for Rel.16 is within 70.5-88.5ms range depending on DL PRS configuration settings

Companies are invited to comment on proposal above

| Company | Comments |
| --- | --- |
| vivo | For the proposal, we suggest modifying as below - RAN1 to conclude that DL PRS measurement time for Rel.16 is within [70.5-88.5ms] range under certain depending on DL PRS configuration settings For the Main Assumptions for Analysis，we have some doubts： - What means UE can process all DL PRS resources in a slot, it means no matter how many resources in a slot can be processed, or only resources within processing capacity are allocated? - NPRS,islotN'LPRS,iN=1, - UE can process all DL PRS resources in a slot， - NRxBeam, i=1 needed to be add in below bracket - ~68,5 ms (CSSF = 1, NRxBeam, i=1, Nsample = 4, DL PRS Periodicity - 20ms) |
| Qualcomm | With regards to the following: - DL PRS alignment time/MG alignment time ~4-20ms (depending of DL PRS periodicity 4ms or or 20ms – worst case is assumed) Our understanding si that MG is always needed, so the aligment time should be considered as 20 msec in the best configuration scenario. |
| OPPO | The major contribution to the PRS latency caculated above is from the assumption of Nsample = 4 PRS RSTD samples. One more sample would contribute 20ms more to the latency. With Nsample = 1, we would have TPRS-RSTD,i = 8.5 ms and total overall latency for DL PRS measurement time is 12.5ms ~28.5ms. Furthermore, as specified in 133, the total RSTD time of L frequency layers are calculated as: TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i Apparently, here the calculation assumes only one freqyency layer. We shall to clarify those critical assumpptions in the obersvation. So suggest to include one note in the observation as follows: Proposal 1 - RAN1 to conclude that DL PRS measurement time for Rel.16 is within 70.5-88.5ms range depending on DL PRS configuration settings - Note: Assume one frequency layer and 4 PRS RSTD samples. |
| vivo2 | A small modification for aligning the result between proposal and summary - RAN1 to conclude that DL PRS measurement time for Rel.16 is within 720.5-88.5ms range depending on DL PRS configuration settings |

  - Discussion Round #2

Proposal 1-2

- Capture conclusion in RAN1 chair notes to facilitate RAN2 work on latency analysis of NR positioning protocols:

- Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings

    - Note: The following assumptions are made

      - One DL PRS frequency layer in FR1

      - CSSF = 1

      - NRxBeam, i = 1,

      - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods)

      - Both DL PRS and MGRP are equal to 20ms

      - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms)

| Company | Comments |
| --- | --- |
| CATT | In general, we support Proposal 1-2 but we prefer to put the range of DL PRS measurement time instead of 88.5ms in the proposal, as shown in below: - Capture conclusion in RAN1 chair notes to facilitate RAN2 work on latency analysis of NR positioning protocols: - Estimated DL PRS measurement time in Rel.16 is within 72.5-88.5ms range depending on DL PRS configuration settings - Note: The following assumptions are made - One DL PRS frequency layer - CSSF = 1 - NRxBeam, i = 1, - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods) - Both DL PRS Periodicity and MGRP equal to 20ms - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms) |
| Nokia/NSB | We will oppose this proposal if it will require RAN1 to send an LS on this topic. It was already discussed during GTW and most companies seemed opposed to sending a reply LS. If other companies insist on discussing this topic perhaps we can capture an observation in the TR but we do not see this as critical with such limited time remaining. |
| Huawei/HiSilicon | Sorry for the late response. We are generally OK with the conclusion, but I guess the NRxBeam, i = 1 is only valid for FR1, and thus we propose to modify the following condition: - One DL PRS frequency layer in FR1 |
| Qualcomm | To Nokia: We can capture it as a conclusion in the TR and not send an LS We agree with the suggestion from HW/HiSi. We are still not convinced on adding the range. We suggested to put the 88.5 assumign that the slot-offset of the configured MG is the worst-case scenario (i.e. with a periodicity of 20 msec, in the worst case scenario, when the UE has finished with the decoding of the MG configuration, the earliest MG instance is 20 msec away). I acknolowedge that it may depend on the scheduling decision, and a bit of “luck” whether the decoding of the RRC message is happening fast-enough for the UE to “catch” the first MG instance, but in the conservative case, we expect a serving gNB to send an MG configuration that has a slot offset that has enough time margin with respect to the PDSCH that carries the configuration (due to the fact that the RRC decoding by the UE has some intrinsic uncertainty; there is no tight timeline lime MAC-CE or DCI scheduling). Therefore, we still prefer to say 88.5 msec. Also, in many scenarios the PRS measurement time can be longer than this, depending on the configuration, so I am wondering whether we need to change the “is” to “can” and add a word “minimum”. E.g.: - Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings. - Note: The following assumptions are made - One DL PRS frequency layer in FR1 - CSSF = 1 - NRxBeam, i = 1, - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods) - Both DL PRS Periodicity and MGRP equal to 20ms - Configured DL PRS resources are within a UE DL PRS processing capacity (N,T) = (0.5ms, 8ms) |
| LG | Either way of modifications by CATT or Qualcomm is generally fine, but we have a minor comment. In proposal 1-1, we said value range such that 70.5-88.5, so we would like to check if it is okay to add “minimum” for 88.5 ms. |

- SRS for Positioning Measurement Time

The SRS for positioning measurement time is not defined by RAN4 WG. In general, it includes similar components (alignment time, transmission time, gNB measurement time). Overall, this latency component depends on numerology and resource configuration settings. The minimum SRS for positioning periodicity is 1 slot. In addition, aperiodic SRS is supported. It means that alignment time and transmission time can be less than 1ms. The gNB SRS for positioning measurement time in the order of [5]ms can be assumed. In summary, it can be concluded that SRS for positioning measurement time can be less than [6] ms.

  - Discussion Round #1

Proposal 2-1

- Capture conclusion in RAN1 chair notes to facilitate RAN2 work on latency analysis of NR positioning protocols:

- SRS for positioning measurement time less than 6 ms can be achieved under certain SRS for positioning configuration settings

    - Note: The following assumptions are made

      - SRS for positioning alignement time 0.5 ms

      - SRS for positioning transmission time 0.5ms

      - SRS for positioning processing time 5 ms

- Note: Considering UL link budget and interference on SRS for positioning signals, the longer transmission time may be needed that will further increase SRS for positioning measurement time

Companies are invited to comment on proposal above and the view on SRS for positioning measurement time

| Company | Comments |
| --- | --- |
| CATT | In gerenal , we support Proposal 2-1. The assumptions to derive the SRS for positioning measurement time maybe can be listed here as a note, just like Proposal 1-2. |
| Nokia/NSB | We will oppose this proposal if it will require RAN1 to send an LS on this topic. It was already discussed during GTW and most companies seemed opposed to sending a reply LS. If other companies insist on discussing this topic perhaps we can capture an observation in the TR but we do not see this as critical with such limited time remaining. |
| Huawei/HiSilicon | Support in general. |
| Qualcomm | Wouldn’t multiple SRS instances be needed here also? Why a UE requires 4 instances minimum, whereas a gNB would be OK with one? I understand that there are no requirements for gNBs, but still this does not mean that in Ran1 we should make unrealistic assumptions. It is actually likely that a gNB would need even more than 4 samples because of UL interference (no muting) and worse coverage than DL. For fairness, we can assume 4 samples though. With regards to the periodicity, I understand that SRS can have very low periodicity, but not all slots are “UL” at least in TDD. Making a reasonable assumption for a frame, like “DDDU” would be more appropriate. In this case, and for 30 KHz SCS, the SRS would span over 8 msec. Then, a gNB would need to do finalize the processing ([2-4] msec?). Then the overall minimum time seems to be more close to [10-12] msec. |

- Summary

In this contribution, we have provided the intermediate status of the RAN WG1 discussion on DL PRS and SRS for positioning measurement time. The main purpose of this analysis is to facilitate study of the Rel.16 NR positioning protocols latency performed by the RAN WG2. It is expected, that proposals made in this contribution are to be further discussed during the GTW session on NR positioning.

- References

- R2-2010873, Reply LS on Latency of NR Positioning Protocols, RAN WG2

- R2-2010872, Summary of Latency Results, Intel Corporation

- 3GPP TS 38.133 “Requirements for support of radio resource management” V16.5.0 (2020-09)