3GPP TSG RAN WG1 Meeting #103-e	R1-2009809

e-Meeting, October 26th – November 13th, 2020

Source:		Moderator (SoftBank)

Title:	Summary of [103-e-NR-CovEnh-02] A.I. 8.8.1.1 baseline coverage performance using LLS for FR1

Agenda Item:	8.8.1.1

Document for:	Information

Introduction

This document summarizes the contributions submitted to 8.8.1.1 and 8.8.3 (related to FR1) and the email discussion held in RAN1#103-e. The email discussion will be held based on the Chairman’s guidance.

[103-e-NR-CovEnh-02] Eamil discussion for FR1 coverage performance – Yusuke (Softbank)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

It was announced by RAN1 char that this email discussion was closed on 16th November.

Guidance after the GTW session on 26th October

For the the 1st checkpoint, FL plans to progress the following high priority topics:

- Section 2.2  [H] Target metrics and values for bottleneck identification

- Section 2.3.1 [H] How to aggregate evaluation results from different companies and limit number of scenarios of interest for bottleneck identification (common for FR1 and FR2)

Even though we had an intensive discussion at the GTW session on 27th October, FL would like to gather the companies’ views again because they have not documented yet, and some companies have not have chance to speak up due to the limited time of discussion.

The deadline of the 1st round of discussion is 20:00 UTC of 27th October (Tue).

Guidance before the GTW session on 28th October

After the 1st round of email discussion, FL sees that we can progress the discussion based on the following steps:

(Step 1)

  - Selection of scenarios to derive representative values.

  - Conclude the derivation of representative value (handling of outlier etc.)

(Step 2)

  - Decision on performance metrics and values for further analysis.

    - Including the preparation of the TP how the target value is derived (if necessary)

  - Bottleneck identification by using the metrics and representative values

(Step 3)

  - Discussion on additional observations, which were not covered by the representative data analysis

  - Resolution of remaining issues

The definition of steps may be modified depending on the discussion situations. Also the relationship between each email discussion and each step will be provided later.

The details second round of email discussion will be announced after the GTW session on 28th October.

Guidance after the GTW session on 29th October

Companies are encouraged to provide your views on the FL observation/proposal highlighted in light-blue in the following sections:

2.1 [M] Finalization of parameters and values

2.2 [H] Target metrics and values for bottleneck identification

2.3.1 [H] How to aggregate evaluation results from different companies and limit number of scenarios of interest for bottleneck identification (common for FR1 and FR2)

The deadline of this round of email discussion is 17:00 UTC of 2nd November (Mon) .

Guidance after the GTW session on 3rd November

Companies are encouraged to provide your views on the FL observation/proposal highlighted in light blue in the following sections:

2.2.1.	[L] Confirmation of the working assumption on pathloss formula

2.2.2.	[H] Details of Scenario dependent targets

2.2.3. 	[H] Details of Service dependent targets

2.2.4.	[H] Details of targets based on relative difference between channels

2.2.5.	[H] Interaction among three target metrics for bottleneck identification

The deadline of this round of email discussion is 11:59am of UTC on 5th of November (Thursday)

Guidance after the GTW session on 4th November

Companies are encouraged to provide your views on the FL observation/proposal highlighted in light blue in the following sections:

2.2.1.	[L] Confirmation of the working assumption on pathloss formula

2.2.2.	[H] Details of Scenario dependent targets

2.2.3. 	[H] Details of Service dependent targets

2.2.4.	[H] Details of targets based on relative difference between channels

2.2.5.	[H] Interaction among three target metrics for bottleneck identification

The deadline of this round of email discussion is 11:59am of UTC on 5th of November (Thursday)

Guidance at the beginning of the 3rd meeting week

Companies are encouraged to review the updated FL proposal in the sections:

High priority (early decision expected)

- 2.2.2[H] Details of Scenario dependent targets

- 2.2.3[H] Details of Service dependent targets

- 2.2.4[H] Details of targets based on relative difference between channels

- 2.2.5[H] Interaction among three target metrics for bottleneck identification

Medium priority

The FL proposal will be affected by the decision of high priority items, but early review by companies are expected.

- 2.5.1[M] Summary of the contributions submitted to RAN1#103-e

- 2.5.2[M] Bottleneck identification by using representative values

Low priority

The FL proposal will be affected by the decision of high priority items, and the discussion on the section has less or no impact to others.

- 2.3.3[L] Observation for Urban 4GHz TDD scenario

- 2.3.4[L] Observation for Urban 2.6GHz TDD scenario

- 2.3.5[L] Observation for Rural 4GHz TDD NLOS O2I scenario

- 2.3.6[L] Observation for Rural 2.6GHz TDD NLOS O2I scenario

- 2.3.7[L] Observation for Rural 2GHz FDD NLOS O2I scenario

- 2.3.8[L] Observation for Rural 700MHz FDD NLOS O2I scenario

- 2.3.9[L] 2nd priority: Rural 700MHz with long distance FDD LOS O2O scenario

- 2.3.10[L] 2nd priority: Rural 4GHz with long distance TDD LOS O2O scenario

- 2.3.11[L] 2nd priority, Observation not using representative values

The FL plans to discuss the high priority items in the GTW session on Tuesday. Companies are encouraged to review the sections, and provide their comments (especially concerns) by 7:00pm UTC of 10th November (Tuesday)

Guidance after the GTW session on 10th November

Companies are invited to provide their views on the following sections:

High priority topics: important to finalize the SI

- 2.2.4[H] Details of targets based on relative difference between channels

- 2.5.2[H] Bottleneck identification by using representative values

Medium priority topics: important but will not block the progress of other topics

- 2.3.3[M] Observation for Urban 4GHz TDD scenario

- 2.3.4[M] Observation for Urban 2.6GHz TDD scenario

- 2.3.5[M] Observation for Rural 4GHz TDD NLOS O2I scenario

- 2.3.6[M] Observation for Rural 2.6GHz TDD NLOS O2I scenario

- 2.3.7[M] Observation for Rural 2GHz FDD NLOS O2I scenario

- 2.3.8[M] Observation for Rural 700MHz FDD NLOS O2I scenario

Lop priority topics: best effort basis

- 2.2.6[L] Handling of 2nd priority scenarios

- 2.3.9[L] 2nd priority: Rural 700MHz with long distance FDD LOS O2O scenario

- 2.3.10[L] 2nd priority: Rural 4GHz with long distance TDD LOS O2O scenario

- 2.3.11[L] 2nd priority, Observation not using representative values

IMPORTANT NOTICE:

FL has a strong concern on the progress of the discussion in 2.2.4 (relative difference). FL thinks this discussion should be concluded as soon as possible hopefully in 24 hours. Otherwise, FL cannot prepare observations and proposals on the subsequent discussions. Please see more details in that section.

Please provide your views until 3pm UTC of 11th November.

Guidance after the GTW session on 12th November

Companies are encouraged to share the views on the following sections:

2.2.6, 2.3.10: handling of 4GHz rural long distance scenario

2.3.2 – 2.3.8: Observation by using 3 metrics

2.3.11: observation by 2nd priority items

2.5.2.1-2.5.2.3: bottleneck identification

The deadline for this round of the email discussion is 4:00 am UTC of 13th November.

Issues for discussion

[Closed] Finalization of parameters and values

The following issues were identified by the companies’ contributions.

Table 2.1-1. Companies’ proposals on the parameters

| Topic | Company Tdoc No | Proposal |
| --- | --- | --- |
| (1) MCS and RB | vivo R1-2007683 | A fixed MCS/RB allocation is preferred for PUSCH evaluation. |
|  | Intel R1-2007957 | • For link budget analysis, it is more desirable to align simulation assumptions for PDSCH and PUSCH, especially MCS, TBS, number of PRBs and DMRS configuration for various deployment scenarios. - Consider Table 1 and Table 2 as a starting point of discussion for link level simulation assumptions in FR1 and FR2 for NR coverage enhancement. |
| (2) Antenna gain correction factor | Vivo R1-2007683 | The correction factor for gNB Tx BF gain for broadcast channel, should be considered in link budget template. - In FR1, the correction factor is about 8dB for carrier frequency other than 700MHz; 0dB for 700MHz. - In FR2, the correction factor is about 8dB and 5dB in urban and indoor scenarios, respectively. |
|  | ZTE R1-2007741 | Consider the antenna gain correction factors in Table 1 for link budget calculation in FR1. |
|  | vivo R1-2007678 | Proposal 1: The correction factor for gNB BF gain for broadcast channel should be considered in link budget template. - In FR1, the correction factor is about 8dB for carrier frequency other than 700MHz; - 0dB for 700MHz. |
|  | vivo R1-2007683 | The gNB antenna gain loss, due to tilt angle from the boresight direction of antenna pannel, is about 2.65dB for FR1, and about 3.07dB and 3.48dB for FR2 in Urban and Indoor, respectively. |
|  | Apple R1-2008478 | Make an agreement on the value of antenna gain correction factor Δ1 and Δ2. |
| (3) Target error rate | Ericsson R1-2008422 | •Enhancements for PRACH performance should provide gains at a miss detection rate of 10% •Enhancements for CSI performance on PUCCH or PUSCH should provide gains at an initial BLER of 10% |
|  | Sierra Wireless R1-2007931 | The rural PUSCH baseline configuration should be with HARQ enabled and without restrictions on iBLER |
| (4) VoIP latency requirement | InterDigital R1-2008487 | - Study the optimum number of HARQ processes, number of repetitions and number of maximum retransmission of a bundle within 50ms and 100ms latency requirement for evaluation for TDD VoIP - Use multiple number of HARQ processes for evaluation of baseline performance and enhancement for uplink FR1 TDD VoIP - For TDD, assume configured grant and follow RV sequence labeling and HARQ process numbering |
| (5) DMRS configuration for PUCCH | ZTE R1-2007741 | Support additional DMRS (i.e. 4 DMRS symbols for 14-OS PUCCH) for link level simulation for PUCCH Format 3 for FR1 |
| (6) Penetration margin | ZTE R1-2007741 | Consider the penetration margins for different O2I cases in Table 2 for link budget calculation in FR1. |
| (7) PUSCH repetition | Sierra Wireless R1-2007931 | The rural PUSCH baseline configuration should use an aggregation factor (i.e. repeats) of 4 or 8 |
| (8) PUSCH frequency hopping | Sierra Wireless R1-2007931 | The rural PUSCH baseline configuration should use inter-slot frequency hopping. |
| (9) Channel Estimation for PUSCH | Sierra Wireless R1-2007931 | The rural PUSCH baseline configuration should be with practical channel estimation • FFS: on configuration details (e.g. maximum time and frequency averaging) |
| (10) Tx/Rx loss | OPPO R1-2008269 | We prefer the option 2 for Cable, connector, combiner and body losses in LB template. For other rows without agreement, we can reuse the values for IMT-2020 self-evaluation. |
| (11) PUCCH F1 for HARQ-ACK for Msg.4 | ZTE R1-2007741 | For link level simulation of PUCCH format 1 with 1-bit HARQ-ACK for Msg4 in FR1, reuse the simulation assumptions of normal PUCCH with assuming no PUCCH repetition. |

Considering the fact that this is the last meeting to conclude this study item, it would not be a good idea to re-open the discussion on evaluation assumption and values because it requires performing link level simulation again to obtain required SINR. Even when it has no impact on the LLS, all the companies are required to update the template, which requires huge efforts to the companies.

Given these reasons, FL strongly recommends that the parameters/values/configurations suggested in Table 2.1-1 should be treated as “reported by companies” manner, which means RAN1 will not spend time at this meeting on the resolution of these issues.

Please provide your view on the FL suggestion above if you have a concern on it.

| Company | Comments |
| --- | --- |
| CATT | We agree with FL’s suggestion |
| Nokia/NSB | Support FL’s suggestion. |
| Intel | Support FL’s suggestion. |
| Apple | With FL’s suggestion, it means we give up the try to converge the results from different companies, even it’s not relevant to the simulation, such as the parameters in the link budget. This is not our preference, but considering the progress, we can live with it. |
| Ericsson | Support feature lead’s suggestion. However, as conclusions are drawn we think this does not preclude addressing the relevance of different settings, scenarios, etc. |
| Xiaomi | We agree with it. |

FL perspective on 28th October

FL would like to propose nothing new on top of the observation above given the situation that the companies can live with the FL proposal.

As for the handling of simulation results with different settings, it seems two companies has a different understanding. From FL point of view, this issue can be handled case-by-case basis. However, FL would like to explain the principle for such things to alleviate the concern by companies.

- The discussion priority is decided based on the companies’ interest. Therefore, the evaluation results with “major” assumptions will get a higher priority.

- Even when the evaluation results are derived from “minor” assumptions, the opportunities for discussion shall be given equally. However, we should keep in mind that the discussion tends to be longer with a lack of consensus, and eventually the whole progress of the discussion will be bad. Therefore, I (as a FL) need to set lower priority for such topics, and encourage offline conversation.

This discussion is kept open until the end of second round of discussion.

Updated FL perspective on 29th October

Given the agreement for FR2, FL believes that the same proposal is applicable for FR1. Therefore, FL would like to propose the following:

FL proposal:

- All the parameters/values/configurations related to FR1 modelling for which an agreement has not been reached among companies prior to RAN1 #103-e, will be henceforth treated according to the “reported by companies” principle. RAN1 will not spend further time during RAN1 #103-e on the resolution of these issues.

Please provide your view on the FL suggestion above if you have a concern on it.

| Company | Comments |
| --- | --- |

Summary of the companies’ view:

Since no concern was raised for the FL proposal above, it can be considered as an offline consensus.

Final summary of the discussion

The FL proposal above was agreed in the GTW session on 3rd November. Therefore, the discussion on this section is closed.

Target metrics and values for bottleneck identification

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

At RAN1#102e meeting, the following conclusion was made for the target metrics and values:

Companies have provided their views by their contributions, and they are summarized in Table 2.2-1:

1. Scenario dependent targets, e.g., ISD/MPL

Table 2.2-1. Proposals on target ISD values

|  | Urban FR1 | Rural FR1 | Rural L.D. FR1 | Dense Urban FR2 | Suburban FR2 | Indoor FR2 |
| --- | --- | --- | --- | --- | --- | --- |
| R1-2007746 ZTE | 500m | 1732m or 5000m | 12km or 30km | 200m |  | 12BSs per 120m x 50m |
| R1-2007678 vivo | 350m |  |  |  |  |  |
| R1-2007877 CATT | 400m | 1732m | 12km | 200m | 200m | 20m |
| R1-2007952 Intel | 400m (MIL 4GHz O2I=148.7) | 1732m (MIL 4GHz O2I=149.2) (MIL 4GHz O2O=147.2) 5000m (MIL 700MHz O2I=149.4) (MIL 700MHz O2O=147.4) | 15km (MIL 144.7) 30km (MIL 156.7) |  |  |  |
| R1-2007993 CTC | 500m (MPL=121.75) | 1732m (MPL 4GHz=131.57) 1732m (MPL 2GHz=12.55) | 30km (MPL=142.91) |  |  |  |

Note: other target values for FR2 might be proposed under 8.8.1.2. The values for FR2 here are just for information

2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

R1-2007746 ZTE

  - For service dependent targets, adopt 147dB + 10 * log 10( N/k ) - Δ1 as the target MCL for different services at least in FR1.

    - The number of TxRU N = 64 and the number of Tx/Rx chains k = 2 or 4 for 2.6 GHz and 4 GHz, and N = k for other frequency carriers.

R1-2008380 SoftBank

  - Confirm the target MCL value of 147 dB for FR1 VoIP under the following condition:

    - Rural with 700MHz and/or 2GHz senarios

    - 2 trannsmit TxRUs and 2 receive TxRUs for BS

    - 1 Tx chain and 2 Rx chains for UE

    - Zero receiver interference density

      - Even when non-zero value can be used, it is not needed to adjust the target value

    - Zero recevier implementation margin

      - Even when non-zero value can be used, it is not needed to adjust the target value

3. Relative difference between channels, e.g, MIL(/[MCL])

Table 2.2-1. Proposals on criteria for bottleneck identification

| Company | Preferred metric | Criteria to identify bottlenecks |
| --- | --- | --- |
| R1-2007746 ZTE | MIL | Worse than PBCH |
| R1-2007877 CATT | MIL or MCL | Worst and 2nd worst |
| R1-2008624 Qualcomm | MCL | Worse than control coverage |
| R1-2008667 NTT DOCOMO | MIL |  |
| R1-2008024 CMCC | MIL or MCL |  |
| R1-2008478 Apple |  | PDCCH for CSS is used as the benchmark |

The input from companies is not enough to make a decision due to the following reasons.

For, Scenario dependent targets, e.g., ISD/MPL

  - The proposals by companies are not aligned yet: different values are proposed, and we still have multiple options for a scenario.

  - It is desirable to convert the value to MPL or MIL, but it has been done by only two companies.

For Service dependent targets, e.g., [MCL=147] dB for VoIP

  - From companies evaluation results, it was found that most of the channels cannot satisfy MCL of 147dB

    - This is because the results have used non-zero value for interference and Rx margin, which is not aligned with the derivation of 147dB

    - It would not be good approach to request resubmission of additional link budget table with zero interference and Rx margin

    - Therefore, it is required to reconsider the value of 147dB taking into account the companies’ simulation assumptions

For Relative difference between channels, e.g, MIL(/[MCL])

  - Use of MIL got majority support, but not enough to make the final decision.

  - It is not easy to decide the criteria to identify bottlenecks without seeing the final form of link budget. The level of enhancement should be realistic, but it is not clear at this moment.

- Given the analysis above, FL would like to propose the following guidance for the next step discussion:

The proponents of scenario-dependent targets are requested to come up with a single target value for each scenario

  - It is also requested to provide either MIL or MPL value corresponding to the target ISD value.

The proponent of service-dependent targets are requested to prepare an updated MCL value with non-zero interference and Rx margin, which are widely used by companies

  - Considering this target is valid only for a limited number of scenarios, it is recommended to merge this requirement to other target metric(s).

For Relative difference between channels,

  - Discuss and decide how to identify the bottleneck channels

    - Option 1-1: worse than PBCH

    - Option 1-2: x-th worst channel (x=e.g.2)

    - Option 1-3: worse than control coverage (i.e. worst control channel)

    - Option 1-4: do not decide in RAN1

      - RAN1 will capture a comprehensive analysis in the TR, and RAN will make a decision when a subsequent WI is started.

    - Option 1-5: Other solution

  - Discuss and decide which metric to be used

    - Option 2-1: MCL

    - Option 2-2: MIL

Start the next step discussion after the issues in section 2.3.1 is concluded.

Please provide your view on the FL guidance.

| Company | Comments |
| --- | --- |
| SoftBank | Regarding MCL=[147]dB, we have re-evaluate the requirement taking into account Rx loss and interference value used for IMT-2020 self-evaluation (-165.7dBm/Hz for row No (15) and 2 dB for row No (20) respectively). It is fond that the requirement can be relaxed to 139.2dB instead of 147dB. Also, we are fine to focus on Rural 700MHz for simplicity. We are basically fine with merging this requirement to other metric, but the discussion should be done later, e.g. after checking the representative values. |
| CATT | - First of all, we want to clarify whether all of 3 target metrics including - scenario-dependent targets, service-dependent targets and Relative difference - between channels need be used or we can down-select 1 or 2 targets metric for - bottleneck identification. In our view, if all of three target metrics are used, work load - is very heavy so it is better to down-select 1 or 2 target metrics for bottleneck identification. MIL can be used for the metric of evaluation performance because MIL can consider more factors on antenna gain than MCL. |
| NTT DOCOMO | We are fine with the FL guidance, and we prefer to use the relative value for the target metric, since the absolute value may not reflect the real environment / deployments that have large difference over scenarios and variability. We also support to use Option 1-2 for the target performance derivation. We prefer to use Option 2-2 : MIL for the target metric, since it’s better to include the antenna gain including beamforming gain in the link budget analysis. |
| ZTE | We are fine with the guidance for scenario-dependent targets and service-dependent targets. For the relative way, we prefer Option 1-1. It’s reasonable for a cell to serve a UE as long as it can successfully detects the PBCH and try to access the network. For Option 1-3, it needs to further clarify which control channel it refers to here. We have concerns on Option 1-4. Per our understanding, RAN1 should be responsible for the decision here. If RAN1 failed to make any recommendation, we don’t believe it would have enough time in RAN to discuss the bottleneck channels based on the detailed targets and evaluation results provided in RAN1. In addition, it needs to clarify if multiple targets are defined. How should we draw the conclusion? Would a channel be bottleneck channel as long as it cannot meet one of the target defined? For the metric, we prefer Option 2-2 since MIL could reflect the difference between unicast and broadcast channel for all scenarios. |
| Panasonic | We are fine with FL guidance. On relative difference between channels, at least MIL would be better metric than MCL as mentioned by CATT, DOCOMO, and ZTE. On how to identify the bottleneck channels, although we no strong preference, Option 1-4 might be realistic approach. |
| OPPO | We prefer to use the relative value for the target metric. - Discuss and decide how to identify the bottleneck channels: - We prefer the Option 1-2: x-th worst channel. Maybe the value of x can be 3 if the values of 1st and 2nd worst channel are very close. - Discuss and decide which metric to be used - We prefer the Option 2-2: MIL Evaluation results from different companies are different because of the different parameters/values/configurations. It requires huge efforts if companies need to update the template or obtain required SINR according a new evaluation assumption and values. In this case, according to a scenario-dependent targets (MPL) or service-dependent targets (MCL), the values of gap may be different with the company's different results. Although the results of different companies are different in absolute value, the relationship of different channels is basically the same. We prefer to use MIL because it can consider more factors on antenna gain than MCL. |
| vivo | Scenario dependent target is preferred. And the values provided in TR 38.913, if available, can be considered as starting point. For Urban, since we do not have clear definition on whether it is macro urban (500m) or dense urban (200m), we suggest to use a mean value, that is 350m. Urban FR1; Rural FR1; Rural L.D. FR1; Dense Urban FR2; Suburban FR2; Indoor FR2. Target ISD; 350m; 1732m; 12km; 200m; 20m. MPL; 115.7dB (4GHz) 111.96dB (2.6GHz); 131.57dB (4GHz) 127.83dB (2.6GHz) 125.55dB (2GHz) 116.44dB (700MHz); 127dB (700MHz); 123.08dB; 94.03dB. MIL; 149.51dB (4GHz control) 146.43dB (4GHz data) 145.77dB (2.6GHz control) 142.69dB (2.6GHz data); 149.02dB (4GHz control O2O) 145.7dB (4GHz data O2O) 154.55dB (4GHz control O2I) 151.23dB (4GHz data O2I) 145.28dB (2.6GHz control O2O) 141.96dB (2.6GHz data O2O) 150.81dB (2.6GHz control O2I) 147.49dB (2.6GHz data O2I) 143dB (2GHz control O2O) 139.68dB (2GHz data O2O) 146.5dB (2GHz control O2I) 143.18dB (2GHz data O2I) 133.89dB (700MHz control O2O) 130.57dB (700MHz data O2O) 137.39dB (700MHz control O2I) 134.07dB (700MHz data O2I); 144.06dB (700MHz control O2O) 140.79dB (700MHz data O2O) 147.56dB (700MHz control O2I) 144.29dB (700MHz data O2I); 140.15dB (control O2O) 136.93dB (data O2O) 154.08dB (control O2I) 150.86dB (data O2O); 102.53dB (control) 99.23dB (data). For channels that may be determined as bottleneck using the relative difference metric. It is also possible that even the worst channel can also meet the target ISD requirement. For example, the target ISD is 20m, as recommended in TR 38.913, and it can be fulfilled even considering the channel with worst coverage. Hence, any ‘bottleneck’ channels identified using this metric, which may still fulfill the target ISD, should not be considered. If consensus cannot be reached, we suggest also remain the metric and identified bottleneck channels ‘companies report’, and select the bottleneck channels with the most consensus for further enhancements. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Whatever the metrics chosen, the resulting ISD that we get from the results is an important factor that defines how good the deployments are. We prefer scenario dependent targets, for instance 30km ISD for rural with long distance must be achievable. |
| CMCC | In our contribution of R1-2003970, 400m ISD for urban scenario and 1732m ISD for rural scenario were proposed for the absolute targets of FR1. And these values could also be used as a reference for the MPL. Between the MIL and MCL, MIL is slightly preferred since the impact of antenna gains is taken into account. And due to that there are not many references for the absolute value of MIL, the relative comparison of MIL to identify the bottleneck channels is preferred. And for the option 1-1~1-5, option 1-2 is preferred for identify the bottleneck channels. And X that equals 3 or 4 is acceptable. Option 1-4 is not preferred since RAN1 have enough information to make the decision. Extend the discussion and the decision to the RAN plenary is not a good choice for the progress. |
| Nokia/NSB | We are fine with FL’s guidance. We think service-dependent targets can be considered to VoIP, as discussed since the beginning of the SI. Concerning the discussion on relative and absolute targets. From our perspective, priority should be given to a rational approach. If absolute targets are chosen as the single target, or one of the targets, then we are fine with vivo’s suggestion of setting ISD targets according to TS 38.913. Channels which cannot meet the target should be improved. If relative difference targets, for which we have a slight preference, are chosen, then Option 1-1. In this regard, we agree with ZTE’s argument in support of Option 1-1. Option 1-3 could also be considered, however we are not sure it is as meaningful as Option 1-1, given that PBCH is the “entry door” to the cell and we may want to make sure all the channels can deliver a coverage which is as close as possible to the one of PBCH. The most appropriate metric in case of relative targets would seem MIL, for the reasons other companies mentioned, hence our preference is for Option 2-2. |
| Intel | We slightly prefer the scenario dependent target for analysis, which is based on target ISD value. This can give intuitive insights on what would be the bottleneck channels that need coverage enhancement. As mentioned by Vivo, if relative difference based metric is applied, it may be possible that the identified bottleneck channels may still meet the cell coverage. For relative difference, it is not clear to us whether we can use option 1-1 as performance metric. PBCH is typically designed for the worst case scenario. We do not expect all the channels would meet the link budget performance as PBCH, which is unrealistic. For Option 1-3, we are not sure whether this is for DL or UL control coverage, which should be clarified in the discussion. We do not prefer Option 1-4 as this should be the task in RAN1 to determine the target performance and identify the bottlenecks. Further, for relative difference, we prefer Option 2-2: MIL as this includes the reasonable antenna gain. |
| Qualcomm | - As stated from the beginning, our preference is to use relative comparisons and not rely on absolute metrics. There is a wide variation in the assumptions made across companies that drawing an absolute metric is going to be near impossible. We also do not want our analysis to be limited to any particular assumption on ISDs or network deployments. To this end we propose the following: - Characterize the coverage of RACH procedure and identify the bottleneck channels in the RACH procedure. - Compare uplink and downlink control channel coverage and identify the bottleneck control link in each scenario. - Compare uplink and downlink data coverage with control channel coverage and identify bottlenecks. Data channel enhancements are dependent on service rate requirements and tend to fall short of control channel coverage. - If the above procedure is adopted, then MCL or MIL can serve as the metrics of choice. |
| Apple | Our preference is the scenario dependent target, the ISD value can input from operators. We believe this makes the study more senses considering the requirements from operator’s real network deployments. - For MIL based performance metric, if absolute MIL is applied, we prefer it is derived from ISD implicitly. For relative MIL, the PDCCH for CSS can be considered. Choosing one or two worst channels could not be the better choice, these channels may not be the bottleneck in the real network without considering the cell radius. |
| Ericsson | For scenario dependent targets: We are fine with the FL’s proposal, with the exception that MIL or MPL value from system level simulations should be able to be reported according to the setups used by companies in their simulations. System level simulations have to be rerun with parameters like ISD. For bottleneck identification, we prefer relative MIL, computed as the difference between a reference and target channel. Companies’ results should be internally consistent, and so this seems the most accurate way to determine a bottleneck. Therefore, we prefer option 1-1. Other options are also possible if they can be consistent among companies. |
| Xiaomi | We prefer the scenario dependent target for analysis, which is based on target ISD value and at least MPL value corresponding to the ISD value should be provided. From our point of view, MPL is fully consideration on the antenna gain, implementation margin and various types of losses caused by the actual transmission environment. And the operators can input the ISD based on its typical network deployment. By compared the baseline performance and the target pathloss, it is more intuitive and precise to find the coverage gap. |

Is Summary of the 1st round of email discussion

Comment on the procedure

  - One company concerns on the workload if we adopt all the 3 metrics

    - FL response: this comment is definitely valid. However, it is not so easy to down-select at this stage given the interest from the companies. FL would like to propose to keep these 3 metrics at this stage. Also, FL believes 3 metrics are doable if we can limit the number of scenarios, which is discussed under section 2.3.1.

  - One companies questioned what happens if contradiction is found among the metrics

    - FL response: FL’s understanding is that the concerning channel will be identified as bottleneck if it cannot satisfy either of the metrics because metrics are basically linked with operators’ requirement. Needless to say, RAN1 should avoid setting unrealistic target, though. Companies may have different understanding and hence more discussion will be needed.

  - Handling of evaluation results with SLS

    - FL wonders if the evaluation results by SLS can be put the same table as LLS because the procedure to derive the value is completely different. Probably different table needs to be prepared for LLS and SLS which allows additional information of the evaluation assumptions etc.

Details of 3 metrics

  - For, Scenario dependent targets, e.g., ISD/MPL

    - Updated proposal by one company (see table below, and the proposal for FR2 will be treated under A.I. 8.8.1.2)

|  | Urban FR1 | Rural FR1 | Rural L.D. FR1 |
| --- | --- | --- | --- |
| Target ISD | 350m | 1732m | 12km |
| MPL | 115.7dB (4GHz) 111.96dB (2.6GHz) | 131.57dB (4GHz) 127.83dB (2.6GHz) 125.55dB (2GHz) 116.44dB (700MHz) | 127dB (700MHz) |
| MIL | 149.51dB (4GHz control) 146.43dB (4GHz data) 145.77dB (2.6GHz control) 142.69dB (2.6GHz data) | 149.02dB (4GHz control O2O) 145.7dB (4GHz data O2O) 154.55dB (4GHz control O2I) 151.23dB (4GHz data O2I) 145.28dB (2.6GHz control O2O) 141.96dB (2.6GHz data O2O) 150.81dB (2.6GHz control O2I) 147.49dB (2.6GHz data O2I) 143dB (2GHz control O2O) 139.68dB (2GHz data O2O) 146.5dB (2GHz control O2I) 143.18dB (2GHz data O2I) 133.89dB (700MHz control O2O) 130.57dB (700MHz data O2O) 137.39dB (700MHz control O2I) 134.07dB (700MHz data O2I) | 144.06dB (700MHz control O2O) 140.79dB (700MHz data O2O) 147.56dB (700MHz control O2I) 144.29dB (700MHz data O2I) |

    - One company proposed to employ 30km ISD for rural long distance

    - One company proposed to employ 400m ISD for urban

    - One company proposed to use MIL derived from ISD

      - From FL point of view, use of MPL would be better in terms of the efficiency of discussion. Most of the companies has already computed the MPL values by using their own assumptions. Meanwhile, calculation of MIL from ISD requires common parameters for its derivation, which is not aligned with our agreement in the previous meeting.

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - 139.2dB is proposed instead of 147dB

    - Focus on Rural 700MHz for simplicity

    - Merging with other metric will be discussed later

  - For Relative difference between channels, e.g, MIL(/[MCL])

    - Choice of MIL and MCL

      - 9 companies explicitly supported MIL because it can consider antenna and beamforming gain. No concern was raised for MIL

    - Criteria for bottleneck identification

      - Option 1-1: worse than PBCH

        - Supported by 3 companies while 1 company have a concern because the performance of SSB may be “too good” compared with the worst channel.

      - Option 1-2: x-th worst channel (x=e.g.2)

        - Supported by one company

      - Option 1-3: worse than control coverage

        - One company showed their support, but we need more discussion which control channel would be the target

        - One company proposed to use PDCCH for CSS for bottleneck identification

      - Option 1-4: do not decide in RAN1

        - Two companies showed their concern while one company thinks it might be realistic. This means that no company is positive for this option.

      - Option 1-5 others

        - One company proposed the following procedure for bottleneck identification

          - Bottleneck identification in RACH procedure

          - Comparison among UL and DL control channels

          - Comparison UL/DL data and control channel, which depend on the service requirement

        - FL thinks this approach is reasonable from technical point of view. However, this kind of multi-step discussion is not good to accelerate the discussion. We can keep this proposal in mind, and discuss further at the next stage.

Update after GTW session on 29th October

The methodology for representative value was agreed at the GTW session on 28th October. FL has prepared some tables (for potential 1st & 2nd priority scenarios) with representative values for further analysis of the necessity/justification each performance metrics. The results are shown in Tables 2.2-2 to 2.2-8 below. In addition, FL observation is incorporated just below each table.

Table 2.2-2 Urban 4GHz TDD

FL observation on performance metric for Urban 4GHz TDD

  - MIL of SSB, PDCCH and PUCCH F1 1 are too high as a reference channel. It may require aggressive enhancements for bottleneck channel(s).

  - X-th worst channel requires a big enhancement for the worst channel i.e. PUSCH.

  - ISD of 500m may be slightly aggressive considering the necessary enhancements for PUSCH.

Table 2.2-3 Urban 2.6GHz TDD

FL observation on performance metric for Urban 2.6GHz TDD

  - MIL of SSB, PDCCH and PUCCH F1 are too high as a reference channel. It may require aggressive enhancements for many channels.

  - ISD of 400m looks an appropriate target for enhancements.

Table 2.2-4 Rural 4GHz TDD NLOS O2I

FL observation on performance metric

  - MIL of SSB, PDCCH and PUCCH F1 are too high as a reference channel. It may require aggressive enhancements for most of the channels.

  - The worst channel +6dB and X-th worst channel gives a good guidance for enhancements.

  - ISD of 1732m looks slightly aggressive considering the number of channels/signals enhancements required

Table 2.2-5 Rural 700MHz FDD NLOS O2I

FL observation on performance metric

  - Service based metric (for MCL) gives different observation from other metrics because PUSCH for eMBB is not the worst channel anymore.

  - MIL of SSB and PDSCH are too high as a reference channel while PUCCH F1 can be a good reference.

  - The worst channel +6dB and X-th worst channel gives a good guidance for enhancements.

  - ISD of 1732m is not an appropriate target because it is too easy.

Table 2.2-6 Rural 2.6GHz TDD NLOS O2I

FL observation on performance metric

  - MIL of SSB, PDCCH and PUSCH F1 are too high as a reference channel. It requires aggressive enhancements for many channels.

  - The worst channel +6dB gives a good guidance for enhancements.

  - It is not clear if X-th worst channel is a good reference when there are many missing channels (i.e. for this case, PUCCH F3 w/ 22 bit is invalid) due to lack of samples

  - ISD of 1732m is a reasonable requirement.

Table 2.2-7 Rural with long distance 700MHz FDD LOS O2O

FL observation on performance metric

  - MIL of SSB and PDCCH are too high as a reference channel for relative MIL difference. PUCCH might be a reasonable reference (but slightly challenging)

  - The worst channel +6dB and X-th worst channel gives a good guidance for enhancements.

  - ISD of 12km is an easy requirement while 30km is quite challenging.

Table 2.2-8 Rural 2GHz FDD NLOS O2I

FL observation on performance metric

  - MIL of SSB, PDCCH and PUCCH Format 1 are too high as a reference channel

  - The worst channel +6dB and X-th worst channel gives a good guidance for enhancements.

  - ISD of 1732m seems a reasonable requirement.

Given the summary above, FL would like to propose the following for the next step discussion. (The text in red is the difference from the previous version)

FL proposal:

  - For, Scenario dependent targets, e.g., ISD/MPL

      - MPL computed from ISD will be used for the bottleneck identification.

      - Further discussion is necessary for the target ISD value for FR1. The value should be chosen considering the operator’s requirement & technical feasibility. The down selection will be done at the next step

        - Urban: 350m vs 400m vs 500m (chose one value at the next step)

        - Rural: 1732 m

        - Rural long distance: 12km vs 30km (chose one value at the next step)

      - Scenarios and target ISD below can be used to draw observation

        - Urban 4GHz TDD – [350m or 400m]

        - Urban 2.6GHz TDD – 400m

        - [Rural 4GHz TDD NLOS O2I – 1723m FL note: more discussion is necessary on the feasibility]

        - Rural 2.6 GHz TDD NLOS O2I – 1723m

        - Rural 2 GHz TDD NLOS O2I – 1723m

        - [Rural 700MHz FDD NLOS O2I - 1732m – FL note: 1732m is so easy that the necessity is not clear]

        - [Rural with long distance 700MHz FDD LOS O2O, Neither 12km nor 30km looks appropriate.]

      - Further discussion is necessary how to capture the evaluation results with SLS

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - Agree to 139.2dB is used for Rural 700MHz scenario for making observation for the next step discussion

  - For Relative difference between channels

    - Use MIL is used to derive relative differential values for the next step discussion

    - Relative differential value is defined by the following formula, and used to draw observations Companies are encouraged to continue their investigation on the criteria to identify bottlenecks taking the summary above into consideration

      - (MIL of target channel) – (MIL of the worst channel + X dB)

      - X is chosen considering the realistic value achieved by potential enhancement(s).

    - Relative differential value is applicable to all scenarios

  - Next step discussion will be started after the discussion in 2.3.1 is concluded.

  - FFS: interaction among Scenario dependent targets, Service dependent targets and Relative difference

Companies are invited to provide their views below:

| Company | Comments |
| --- | --- |
| vivo | For urban scenario, we suggest 350m is used as the target ISD. Since, the gap between coverage of PUSCH and target ISD is still significant, about 6-8dB for 4GHz, based on our simulation. An even higher target seems not rational. Target ISD of 30km is too hard to fulfil rural with long distance coverage, only a few unicast downlink channels can reach this requirement, thus ISD of 12km may be more reasonable. As proposed by FL, rural with long distance is considered as 2nd priority in the following proposal. We suggest not to spend too much effort on the target ISD on this scenario. For relative difference between channels, we are OK with FL proposal. |
| China Telecom | We prefer scenario dependent targets, e.g., ISD/MPL. For urban 4GHz TDD, we suggest [400m or 500m] as the target ISD. Add suggest to delete brackets for rural 4GHz TDD NLOS O2I-1732m. |
| CATT | We want to clarify whether the performance evaluation assumptions for all of samples are the same or not when representative value for each channel is calculated. Calculated representative value for each channel won’t be accurate if the performance evaluation assumptions for all of samples are different because the performance evaluation gap between enhanced technology such as frequency hopping/repetition and non-enhancement technology is very large. For MPL, we suggest to use unified target value (1st priority 400m) both Urban 2.6GHz and Urban 4GHz. For other FL proposals, we are fine with FL proposals. |
| Intel | We are in general fine with the FL’s proposal. For urban at 4GHz, we prefer to consider 400m as target ISD. For rural with long distance, we think 12km ISD would be reasonable target. However, if this is agreed as 2nd priority, we can also deprioritize this. We also think target ISD of 1732m may be too optimistic for Rural 4GHz TDD NLOS O2I. A reasonable target ISD needs to be decided accordingly. Further, as we commented during GTW sessions, for different scenario, absolute and relative performance target may draw completely different observations. For example, for Rural 700MHz FDD NLOS O2I, metric with gap from worst channel + 6dB would indicate that many uplink channels need to be enhanced as MIL for many uplink channels is quite close, while all the channels can meet metric with target ISD of 1732m. How would we draw conclusions for these scenarios? It would be good to clarify. |
| ZTE | - Similar to CTC, we suggest 400m and 500m as the target ISD for both 4 GHz and 2.6 GHz in urban scenario, and remove brackets for rural 4GHz TDD NLOS O2I-1732m. - For Rural 700MHz FDD NLOS O2I and Rural with long distance 700MHz FDD LOS O2O, we agree that a more appropriate ISD should be defined with preferred values from operators. If we cannot reach consensus on these values, we may need to de-prioritize these scenarios. - It should be ‘Rural 2 GHz TDD FDD NLOS O2I – 1723m’ - We’d like to clarify whether the service dependent target is only for PUSCH/PDSCH carrying VoIP or could apply all channels/signals. |
| NTT DOCOMO | We are generally fine with the FL proposal. For relative value, we think that “X” may not be necessary. When we may down select the target performance, we still prefer the reference value approach, since the absolute value may not reflect the real environment / deployments that have large difference over scenarios and variability. In addition, the parameters for MPL derivation is not defined, so that we wonder how to handle the absolute MPL values and target performance for the different assumptions. |
| Qualcomm | For relative comparisons, we propose an alternate approach. We prefer to draw three insights, each one mapped to the three sub-agendas under discussion for enhancements (8.8.2.1,8.8.2.2, 8.8.2.3). In particular, we wish to - Identify bottleneck channels in RACH procedure, i.e., compare coverage of msg1(PRACH), msg2 (PDCCH/PDSCH), msg3(PUSCH), and msg4(PDCCH/PDSCH). - Identify bottlenecks in control channel coverage, i.e., compare coverage of broadcast PDCCH vs PUCCH. This gives us an idea of control coverage that is independent of service rate requirements. - Identify bottlenecks in data channel coverage. Compare control channel coverage with data channel coverage. This approach we believe sidesteps the need to identify another parameter X in the FL proposal, while also giving us more insight on what aspects to focus on. For ISD-based approach, for urban scenarios, can we please retain 500m? Its is important to stress coverage for these scenarios. Also, for rural 700MHz scenario, can we use 4-5km ISD to stress coverage? |
| Ericsson | We are in general OK with the FL proposal, but feel strongly that reliability metrics need to be reported for the target metric values. We think it is crucial to understand the reliability of each representative metric when using them for decisions. Therefore, we suggest that a variability metric such as standard deviation and e.g. a number of reported values be provided with each representative metric. Similar comment to Qualcomm regarding stressing coverage with the larger ISDs, we also would like to have 700m ISD for Urban 4 GHz, 3km ISD for Rural Macro 4GHz, and 7km ISD for 700 MHz Rural Macro. Also, we think ZTE’s comment to clarify what channels apply to the service dependent target is a good one. |
| Samsung | We support FL proposal with the following comments: 1) Prefer target ISD = 400m for urban 4GHz TDD to align with urban 2.6GHz TDD. In addition, to stay focused, no additional ISD value (as proposed QC/Ericsson) would be preferred. 2) Current FL proposal is: - Scenario dependent targets (MPL/ISD) bottleneck identification - Service dependent targets (MCL) making observation - Relative difference to draw observations For the consistency among the targets, we suggest the following edits: - MPL computed from ISD will be used for the bottleneck identificationto draw observations. |
| Huawei, Hisilicon | Fine with FL’s proposal. - For urban at 4GHz, we prefer 400m as the target ISD. - For Rural 4GHz, it seems better to put brackets on the ISD=1732m instead of the whole scenario because the concerns seems about the value of ISD instead of Rural 4GHz TDD (NLoS O2I). |
| CMCC | Generally fine with FL’s proposal. We support the proposal of 400m ISD for Urban 2.6GHz TDD scenario. Also we propose to remove the bracket for Rural 700MHz FDD NLOS O2I, which is also identified as the 1st priority scenario in previous discussion with enough contributed samples. If no consensus is reached for the rural ISD at last, we suggest the original proposed 1732m ISD to be the baseline and draw the observations. Even though the 1732m ISD is so easy to be satisfied. From our understanding for the ISD of 700MHz rural scenario, too large ISD or too optimistic coverage of 700MHz is not preferred. For the parameters of MPL, it was agreed that IMT-2020 values are as a starting point and companies could also use other values and report. |
| Nokia/NSB | Agree with Samsung’s comment 1). Having said this, and given the tight relationship between ISD target and network planning, we think priority should be given to operator’s inputs/views, especially if agreement between companies is not easy to find. We also observe the same lack of consistency among targets highlighted by Samsung’s comment 2). However, before suggesting a text change (which could very well be what Samsung suggests) we would like to ask FL the current formulation is the results of a typo or of a specific intention. In the latter case, is there any specific reason for this choice? We would also suggest the following edits (in blue) for the relative difference approach to avoid possible confusion on the “target channel” terminology. - For Relative difference between channels - Use MIL is used to derive relative differential values for the next step discussion - For each channel, relative differential value is defined by the following formula, and used to draw observations Companies are encouraged to continue their investigation on the criteria to identify bottlenecks taking the summary above into consideration - (MIL of target the channel) – (MIL of the worst channel + X dB) - X is chosen considering the realistic value achieved by potential enhancement(s). - Relative differential value is applicable to all scenarios |

- Summary of the companies’ views:

Specific value(s) for ISD target

  - For urban scenario - Quite diverse view on the target value

    - 350m - vivo

    - 400m - CATT, Intel, Samsung, Huawei, Samsung

    - 400 or 500m - CTC, ZTE

    - 500m - Qualcomm

    - 700m (for 4GHz) – Ericsson

    - FL perspective: 400m would be a good target given the support by operators.

  - For rural scenario,

    - Support 1732m for 4GHz - CTC, ZTE

    - 1732m for 4GHz is optimistic - Intel

    - Remove square bracket for 700MHz and discuss more about the value – Huawei, CMCC

    - 3km for 4GHz – Ericsson

    - 7km for 700MHz - Ericsson

    - 4-5km for 700MHz - Qualcomm

    - FL perspective: Considering the feasibility, [1732]m is agreeable for 2.6Ghz and 4GHz given the preference by a operator, and [4]km could be a good target for 700MHz if operators are OK for it.

  - For rural with long distance scenario

    - a concern was raised for rural long distance on the “appropriate” target value

    - 30km is too challenging, and it should be low priority - vivo

    - 12km preferred, but can be deprioritize - Intel

    - Deprioritize this scenario if appropriate target cannot be set - ZTE

    - 4GHz TDD is important, and 12km and/or 30km – IITH

    - FL perspective: keep square bracket for whole proposal for now. Operators are encouraged to come up with a single value for target value. Otherwise, FL would like to propose not to consider this scenario for drawing observation/bottleneck identification

  - Other comments

    - priority should be given to operator’s inputs/views – Nokia

    - FL perspective: Agree. This comment is considered in the FL perspectives above.

Channels for service dependent target

  - Clarification needed which channels/signals are valid for VoIP - ZTE, Ericsson

  - FL perspective: This is definitely a valid point, and can be included in the discussion topic.

Relative difference between channels

  - X (i.e. the offset value from the worst channel) may not be necessary (DOCOCMO)

  - 3step approach is preferred for bottleneck identification (RACH  control  data)

  - FL perspective: The discussion was not so active at this moment, and we can keep the alternative (FL proposal and Qualcomm’s proposal) at this stage. Further details will be discussed later.

Interaction between the metrics

  - Should be clarified (intel)

  - Prefer reference value approach (DOCOMO)

  - FL perspective: The last bullet of the FL proposal covers this intention. We can add some options to clarify what we should do next.

Others comments from companies

  - Concern on the reliability of the representative values due to the different evaluation assumptions - CATT

    - FL perspective: This topic is related to “categorization” discussed in section 2.3.1 . To avoid the duplication, FL would like to suggest to discuss this issue under 2.3.1.

  - Typo correction - ZTE

    - Rural 2 GHz TDD FDD NLOS O2I – 1723m

    - FL perspective: this typo will be corrected.

  - Alignment on “draw observation” vs “bottleneck identification” among three targets

    - Should be consistent among the targets (Samsung, Nokia)

    - FL perspective: Agree with Samsung and Nokia. Actually, this is not the intention by FL. FL would like to treat these 3 targets equally at this stage.

  - Necessity of information showing “reliability”

    - Variability metric such as standard deviation, and number of reported values (Ericsson)

    - FL perspective: Agree. Meanwhile, we should avoid too much information because it may lead to the confusion. From FL point of view, standard deviation, number of samples can be shown with representative value.

      - However, this is more relevant discussion in section 2.3.1. A FL proposal related to this topic will be made under that section.

Other comments from FL

  - FL noticed that we have no agreement on the formula to convert ISD to MPL. This needs to be agreed first to have a common understanding among companies.

- Given the companies’ views and FL perspective above, FL would like to propose the following.

Updated FL proposal:

  - For, Scenario dependent targets, e.g., ISD/MPL

      - MPL is used to compute ISD values computed from ISD will be used for the bottleneck identification.

      - The following formula is used to convert the ISD value to MPL value:

        - For urban scenarios,

          - Target MPL dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97-0.6∙hUT-1.5, where W=20m is the average street width, h=20m is the average building height, hBS=25m is the BS antenna height, hUT=1.5m is the UT antenna height, fc is the carrier frequency and d3D is the target range calculated by ISD/sqrt(3).

        - For rural scenarios,

          - Target MPL dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97,where W=20m, h=5m, hBS=35m and hUT=1.5m.

        - For rural with long distance scenarios,

          - Target MPL dB=20∙log1040∙π∙dBP∙fc3+min0.03∙h1.72,10∙log10dBP-min0.044∙h1.72,14.77+0.002∙log10h∙dBP+40∙log10d3DdBP,where W=20m, h=5m, dBP=2∙π∙hBS∙hUT∙fcc, hBS=35m, hUT=1.5m and c=3∙108m/s.

      - Scenarios and target ISD below can be used to draw observation

        - Urban 4GHz TDD – [350m or 400m]

        - Urban 2.6GHz TDD – 400m

        - Rural 4GHz TDD NLOS O2I – [1732] m - [FL note: discussion needed if 1732 is agreeable from the feasibility point of view]

        - Rural 2.6 GHz TDD NLOS O2I – 1732m

        - Rural 2 GHz FDD TDD NLOS O2I – 1732m

        - Rural 700MHz FDD NLOS O2I – [4000]m – [FL note: need confirmation by companies if 4000m is acceptable]

        - [Rural with long distance 700MHz FDD LOS O2O – FL note: operators’ input is highly encouraged. Otherwise, no ISD target can be set for this scenario]

        - [Rural with long distance 4GHz TDD LOS O2O – FL note: this issue should be discussed under section 2.3.1 first]

      - Further discussion is necessary how to capture the evaluation results with SLS

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - 139.2dB is used for Rural 700MHz scenario for drawing making observation

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - [PUCCH format 3 with 22bits]

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

  - For Relative difference between channels

    - MIL is used to derive relative differential values

    - For each channel, rRelative differential value is defined by the following approaches formula, and used to draw observations

      - Option A-1: (MIL of target the channel) – (MIL of the worst channel + X dB)

        - X is chosen considering the realistic value achieved by potential enhancement(s).

      - Option A-2: apply the following 3-step approach [FL note: companies’ views on this option have not been collected yet.]

        - Identify bottleneck channels in RACH procedure, i.e., compare coverage of msg1(PRACH), msg2 (PDCCH/PDSCH), msg3(PUSCH), and msg4(PDCCH/PDSCH).

        - Identify bottlenecks in control channel coverage, i.e., compare coverage of broadcast PDCCH vs PUCCH. This gives us an idea of control coverage that is independent of service rate requirements.

        - Identify bottlenecks in data channel coverage. Compare control channel coverage with data channel coverage.

    - Relative differential value is applicable to all scenarios

  - FFS: interaction among Scenario dependent targets, Service dependent targets and Relative difference for bottleneck identification

    - Option B-1: use scenario dependent targets only

    - Option B-2: use relative difference between channels only

    - Option B-3: use all the 3 targets, e.g.

      - Absolute targets (i.e. scenario dependent targets and service dependent target) are used to filter the channels/signals, i.e. can be a bottleneck or not.

      - Apply relative difference to the filtered channels/signals, which do not meet the absolute targets, to estimate the necessary amount of enhancements (in dB)

    - Other options are not precluded

    - FL note: the selection of the option will be performed in the next round of email discussion.

Please note that the figures in the proposal is not subject to the agreement. They are a part of FL note and to be removed when it is captured in the chairman’s note.

Companies are invited to provide their views on the FL proposals:

| Company | Comments |
| --- | --- |
| CMCC | Generally fine with 1st and 2nd main bullets. More discussions are needed for the 3rd and 4th main bullets (the relative differences and FFS point) . For the formula used to derive ISD to MPL, our understanding is that the intention is to clarify the calculation procedure from ISD to MPL and is not focused on the detailed parameters of the function. Then, an additional information about the reference of the function could provide more confidence and avoid the detailed discussion about the function itself. This is a minor revision. As in the previous round discussion, we support the 2.6GHz Urban ISD of 400m. For the rural 700MHz FDD NLOS O2I, we also support the FL’s proposal of 4km ISD. But we also would like to propose adding the observations based on 3km ISD, which is also provided by the FL. And the observation of 3km is easy and clear to be captured. The rationale behind this proposal is that both 3km and 4km are typical ISD from our observations. And the observation from 3km ISD is valuable but a little bit different from that of 4km. For the service dependent targets, the PUCCH format 3 with 22bits was agreed in the previous meeting for the evaluation assumptions. It is not clear why it is in the bracket now. For the option A-1, if the X presents the achievable gains through the enhancments, then does the function of option A-1 represent how much gap are left ? From my understanding, the targets of relative difference are first to identify bottlenecks and then identify the gap. A third step is to see how much coverage could be enhanced. For the option A-2, we don’t see much difference between 3 step procedure and finding bottlenecks among the channels. Since any channel could not fulfil the coverage requirement would make the system not work or not fulfil the service requirements. For both option A-1 and A-2 we do not have a strong view. And we are open to more discussions. For the interaction among scnearios and service dependent targets, since we have to provide a unified solution from the perspective of specification, all the scenarios and services should be considered. And how many channels should be enhanced or how many solutions should be considered depends on the workload of the group. That is our initial thinking. More discussions are needed. Or when we have a clearer view of how many channels needs enhancements, we could have a easier discussion on this. For the last comment/note above the table, I think the figures are proper enough to be captured in the TR. The figures or the tables above could give the explanation of how we draw the observations. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Scenario specific targets based on MPL are to be used. Rural with 4 GHz Long distance target coverage of 30km is achievable as shown in our contributions. This input comes from an operator. |

Proposal after GTW session on 3rd November

Given the agreements on the GWS session and the feedback from companies, FL would like to update the proposals. Since the proposals are becoming bigger and bigger, FL sees the necessity to divide the proposals into 4 parts for easier tracking of the discussions.

The discussion of this section is closed. The necessary discussion is continues under the sub-sections.

[Closed] Confirmation of the working assumption on pathloss formula

The agreement on ISD/MPL conversion includes one working assumption.

Agreements:

  - For, Scenario dependent targets, e.g., ISD/MPL

- < Unnecessary part omitted >

        - For rural with long distance scenarios (working assumption)

The source of this formula is Report ITU-R M.2412.

Companies are encouraged to provide their view if there is a concern on this WA. If no comment is provided, the WA is automatically confirmed.

| Company | Comments |
| --- | --- |
| CATT | We agree with FL proposal. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | The proposal is fine. |
| China Telecom | We are fine with FL proposal. |
| CMCC | Fine with the proposal. As our online comments, M.2412 defines two models for each scenario, such as RMA_A and RMA_B. Which specific model is adopted in the calculation should be noted or clarified to facilitate the further check. And this proposal could also apply to the calculation of the other rural and urban scenario. |
| Huawei, Hisilicon | Support. Besides, one question to FL on the agreed formula for Urban scenario, it seems to us that the last sentence about d3d is not a part of M.2412 as we discussed yesterday. Because in M.2412, it is illustrated by the figure below, where d3d = ISD*2/3. Can we replace it with the one from M.2412? Urban case: |

Summary of the email discussion

Given the companies views above, no issue was identified for the working assumption itself. On the other hand, two additional issues are pointed out:

  - Clarification of model used for these formulae

  - Definition of d_3D, which is also discussed in FR2

For the former issue, to save the discussion time at GTW, FL would like to propose to lease it to the TR editors. For the latter issue, we can come back for FR1 to avoid the duplicated discussion for FR2.

FL proposal:

  - Confirm the Working Assumption on the pathloss formula for rural with long distance scenarios.

  - TR editor will take care of the detailed description regarding which model in the reference document is used for these formulae.

Companies are invited to provide their views, if they have a concern. Also, please note that this proposal aims at the email approval, and will not be treated in the GTW sessions .

| Company | Comments |
| --- | --- |

Summary of the discussion

Since no concern was raised, it is deemed that the proposal is stable.

FL proposal:

  - Confirm the Working Assumption on the pathloss formula for rural with long distance scenarios.

  - TR editor will take care of the detailed description regarding which model in the reference document is used for these formulae.

FL would like to ask Chairman to formally approve the proposal above.

[Closed] Details of Scenario dependent targets

Given the discussion so far, RAN1 need to finalize the target ISD values. The FL proposal is updated as follows:

Updated FL proposal

  - For, Scenario dependent targets, e.g., ISD/MPL

    - For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

      - [FL note: It seems from the discussions that companies want to use multiple ISD values for observation. However FL would like to propose not to do that for bottleneck identification. ]

    - Target ISD values for each scenario are as follows:

        - Urban 4GHz TDD – [350] 400, 500 and [700]m for observation and 400m for bottleneck identification

        - Urban 2.6GHz TDD – [350], 400m, 500 and [700]m for observation and 400m for bottleneck identification

          - [FL note: there are proposals to use common value for 4 and 2.6GHz]

        - Rural 4GHz TDD NLOS O2I – 1732 and [3000] m for observation and 1732m for bottleneck identification

          - [FL note: discussion needed if 1732 is agreeable from the feasibility point of view]

        - Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 700MHz FDD NLOS O2I – [3000], 4000 and [7000]m for observation and 4000m for bottleneck identification

          - [FL note: need confirmation by companies if 4000m is acceptable]

        - Rural with long distance 700MHz FDD LOS O2O – [12], 20 and [30]km for observation and 20km for bottleneck identification

          - FL note: Operators’ proposal is highly appreciated. 20km target might be a good target.

          - FL note: this is a 2nd priority scenario

        - Rural with long distance 4GHz TDD LOS O2O – 12, 30km for observation and 12km for bottleneck identification

          - FL note: a company has proposed 30km, but 12km looks a reasonable target.

          - FL note: this is a 2nd priority scenario

- For information, the current summary of achievable ISD for each scenario is found in the tables below.

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | We suggest using unified value for each scenario and propose 400 m for urban, 1732m for Rural and 12km for Rural with long distance. |
| IITM, IITH, CEWIT, Reliance Jio, Tejas Networks | We should always target to improve the coverage as much as possible since this exercise won’t be done over and over again. Considering this we must set better targets and not easy targets. Further, we show how to achieve 30km in our contribution and it is indeed feasible. Considering the above we propose Rural 4GHz TDD NLOS: 3km Rural 4GHz with long distance TDD LOS: 30 km |
| China Telecom | We support FL proposal and suggest to delete the values in brackets for scenarios with taking burden into consideration. |
| Qualcomm | For Urban 4GHz, we can go with 500m to stress coverage. For Rural 700M NLOS, we can go with 4000m to stress coverage. |
| Samsung | Agree with CATT’s proposal. We suggest using a single target ISD value to identify bottleneck channels for each representative scenarios (400 m for Urban, 1732 m for Rural and 12 km for Rural with long distance). |
| Ericsson | OK with FL proposal, except that absolute coverage targets are not used for bottleneck identification unless they are specifically requested by operators. Given the variability of the results, it is difficult to use scenario specific absolute targets for bottleneck identification. Also, I understand the current discussion as seeing what ISDs are feasible given the results we have. If we set the ISDs to be roughly X dB away from a bottleneck, this seems essentially the same as using differential values. Therefore, our suggestion would be to take into account any ISD targets specifically requested by operators, and see how well the targets are met (taking into account the limits of our methodology). |
| Intel | For Rural 4GHz TDD NLOS O2I, with 1732m target ISD, all the uplink channels need to be enhanced. Further, with 24dBm/MHz Tx power in DL, almost all the DL channels need to be enhanced. We are not sure whether this is realistic assumption for the target ISD with O2I. Considering the large penetration loss for O2I, it may be good to consider O2O for Rural 4GHz TDD NLOS, with 1732m target ISD. |
| ZTE | For bottleneck identification for Urban scenario, we prefer to use 500m as defined in IMT-2020. We also think operator’s demand should be considered. |
| vivo | Agree with CATT’s proposal. |
| CMCC | We support to use multiple ISD for observation and single ISD for bottleneck identification. Since multiple ISDs are only used for observation, so we can accept to remove the bracket of ISD in 2.6GHz urban. And 400m ISD in 2.6GHz urban for bottleneck identification is fine to us. For the Rural 700MHz FDD O2I, we do not see the need of observation based on 7km ISD. |
| Huawei, Hisilicon | Regarding Rural long distance, agree with CATT that 12km ISD is big enough. It is unclear why 20km ISD is typical for bottleneck determination. |

Summary of the email discussion

No company raised a concern to introduce multiple ISD to draw observations.

As for target ISD value:

  - For observation: no concern was raised except 7km for Rural 700MHz FDD O2I. FL proposal is agreeable by excluding this value.

  - For bottleneck identification, companies view is quite diverse for some scenarios:

    - Urban 4GHz TDD

      - 400m – support: CATT, CTC, Samsung, Ericsson (if requested by operator), vivo

      - 500m – support: Qualcomm, ZTE

      - FL perspective: 400m is the majority. No strong request for 500 from operators

    - Urban 2.6GHz TDD

      - 400m - support CATT, CTC, Samsung, Ericsson (if requested by operator), vivo, CMCC

      - 500m – support:  ZTE

      - FL perspective: 400m is the majority, including operators

    - Rural 4GHz TDD NLOS O2I

      - 1732m

        - Support: CATT, CTC, Samsung, Ericsson (if requested by operator), vivo

        - Concern: Intel

      - 3000m – Support: IITH

      - FL perspective: 1732m is the majority, but more discussion is necessary. While 3000m is proposed for bottleneck identification, FL has a big concern to adopt such a big challenge (as a consequence, all the channels needs to be enhanced)

    - Rural 2.6 GHz TDD NLOS O2I

      - 1732m – Support: CATT, CTC, Samsung, Ericsson (if requested by operator), vivo

      - FL perspective: 1732m is agreeable

    - Rural 2 GHz FDD NLOS O2I

      - 1732m – Support CATT, CTC, Samsung, Ericsson (if requested by operator), vivo

      - FL perspective: 1732m is agreeable

    - Rural 700MHz FDD NLOS O2I

      - 1732m – Support - CATT, Samsung, vivo

      - 4000m – Support – CMCC, CTC, Qualcomm, Ericsson (if requested by operator)

      - FL perspective: slight majority for 4000m, which two operators support

    - (2nd priority) Rural with long distance 700MHz FDD LOS O2O

      - 12km

        - Support: CATT, Samsung, vivo, Huawei

      - 20km

        - Support: CTC, Ericsson (if requested by operator)

        - Concern: Huawei

      - FL perspective: no clear majority while an operator support 2000m (strictly speaking, they support FL proposal). Note it would not be necessary to agree on 12km because no coverage bottleneck foreseen.

    - (2nd priority) Rural with long distance 4GHz TDD LOS O2O –

      - 12km – Support: CATT, CTC, Samsung, Ericsson (if requested by operator), vivo

      - 30km – Support: IITH

      - FL perspective: 12km is the majority, but more discussion is necessary. However, it is not sure if 12km is a strong need from operators.

    - Note:

      - 3companies (CATT, Samsung, vivo) mentioned that the target ISD should be unified as much as possible

      - One company (Ericsson) mentioned that absolute coverage targets are not used for bottleneck identification unless they are specifically requested by operators.

        - FL perspective: Fully agree. Given the remaining time in RAN1#103-e,

      - One company (Intel) mentioned that O2O for Rural 4GHz TDD NLOS with 1732m target ISD would be better for evaluation

Given the companies view and the analysis above, FL would like to update the proposal as follows:

Updated FL proposal

  - For, Scenario dependent targets, e.g., ISD/MPL

    - For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

    - Target ISD values for each scenario are as follows:

        - Urban 4GHz TDD – 350 400, 500 and 700m for observation and 400m for bottleneck identification

        - Urban 2.6GHz TDD – 350, 400, 500 and 700m for observation and 400m for bottleneck identification

          - [FL note: need for the confirmation by companies who support 500m for bottleneck identification for 4 and 2.6GHz. However, operators’ view is 400m]

        - Rural 4GHz TDD NLOS O2I – 1732 and 3000 m for observation and 1732m with 33dBm/MHz BS transmit power for bottleneck identification

          - [FL note: discussion needed if 1732m for bottleneck identification is agreeable from the feasibility point of view. Also one company supporting 3000m can accept it. Also, it should be discussed if 1732m requirement applies to low power BS (24dBm/MHz) because SSB will also be the bottleneck (which is quite challenging!)]

        - Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 700MHz FDD NLOS O2I – 3000, and 4000m for observation and 4000m for bottleneck identification

          - [FL note: 4000m can be adopted considering the request from operators]

        - Rural with long distance 700MHz FDD LOS O2O – 12, 20 and 30km for observation and bottleneck identification is not performed for this scenario

          - [FL note: No strong opinion from operators for the bottleneck identification with 20km. We can drop it for the sake of progress]

        - Rural with long distance 4GHz TDD LOS O2O – 12, 30km for observation and bottleneck identification is not performed for this scenario

          - [FL note: 12km for bottleneck identification is majority support. However, FL wonders if it gives us a new insight only PUSCH is valid. One company strongly supports 30km. However, FL have a big concern for it because the evaluation results revealed that coverage enhancements are quite challenging]

Companies are invited to provide their views on the updated FL proposal above. As requested by companies, operators’ input is highly appreciated.

| Company | Comments |
| --- | --- |
| CATT | From ▪ Rural 4GHz TDD NLOS O2I, based on our performance evaluation results, SSB won’t be bottleneck with gBS (24dBm/MHz). For Rural 700MHz FDD NLOS O2I, the motivation of target ISD on 3km and 4km isn’t clear. We suggest using unified target ISD (1732m) for all of rural scenarios. For Rural with long distance, 12km should be made as baseline for target ISD. |
| China Telecom | Firstly, we have no strong view on Rural with long distance. And we suggest to delete the values in brackets for urban 4GHz TDD and Rural 700MHz FDD scenarios, or there will be too many observations and bottleneck identification work to be done. Urban 4GHz TDD – 350 400, 500 and 700m for observation and 400m for bottleneck identification Rural 700MHz FDD NLOS O2I – [3000], and 4000 and [7000]m for observation and 4000m for bottleneck identification |
| Intel | We still have concerns if it is feasible for Rural 4GHz TDD NLOS O2I, with 1732m target ISD. As mentioned previously, all the uplink channels need to be enhanced. Further, with 24dBm/MHz Tx power in DL, almost all the DL channels need to be enhanced. We are not sure whether this is realistic assumption for the target ISD with O2I. |
| Samsung | For Rural 4GHz TDD NLOS O2I, we are fine with 33dBm/MHz BS transmit power for bottleneck identification |
| vivo | We are fine with target ISD of urban and rural scenarios. For rural long distance, we tend to agree with FL’s assessment, that coverage enhancement for ISD of 30km is too challenging, in our view ISD of 12km can be baseline, and 20km need further discussion |
| CMCC | As mentioned in the last round, 3km and 4km are proposed to be observed from our side. And this is aligned with the FL’s guidance and most other cases. We see no reason to remove the 3km ISD from Rural 700MHz FDD NLOS O2I. |

Summary of the discussion

Given the agreements in the GTW session on 10th November (see section 4.4 for more details), the discussion for 1st priority scenarios was finalized. Therefore, the discussion on this section is closed.

The discussion on 2nd priority scenarios can be discussed in section 2.2.6.

[Closed] Details of Service dependent targets

During the email discussion, an issue was identified, i.e. which channels need to be evaluated for the VoIP requirements. Given the feedback from the companies, FL would like to propose the following:

Updated FL proposal

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - 139.2dB is used for Rural 700MHz scenario for drawing observation

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - PUCCH format 3 with 22bits

        - [FL note: one company proposed to keep it.]

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | 139.2dB for Service dependent targets of VOIP under Rural 700MHz scenario need be clarified because this value in Rural 700MHz scenario is different with that in other scenarios. |
| Apple | If we adopt 139.2dB for MCL based performance metric and ISD=4km for MPL based performance metric, the identified bottleneck channels are different, MCL based could have additional bottleneck for preamble format 0 and PUCCH format 3 with 11bits. So how to handle the difference between different performance metric? |
| Qualcomm | Not too sure why VoIP is being treated separately. I think its best to retain all the channels we agreed to study during the last online session. |
| Ericsson | Ok with the proposal, except that for VoIP, we see no need for 22bits CSI. |
| ZTE | We are fine with the proposal. A clarification question: It only mentions the target is for drawing observation in current proposal. Should we also use this target for bottleneck identification? |

Summary of the email discussion:

Necessity of clarification how 139.2 is derived

  - FL perspective: Agree. FL will propose the link budget table, which can be captured in Annex of the TR

Necessity of inclusion PUCCH format 3 with 22bit payload for VoIP evaluation

  - In the previous round of the email discussion, one company support to include it. However, in this round, one company mentioned not to include it.

  - FL perspective: Resolution in GTW session would be preferable.

Necessity to mention about bottleneck identification

  - FL perspective: Agree. This is a typo, and the same description as other target metric should apply.

Other comments:

  - FL perspective: FL has already provided the answer using email thread.

- Given the companies’ views and FL perspective above, FL would like to update the proposal as follows:

Updated FL proposal

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - 139.2dB is used for Rural 700MHz scenario for drawing observation and can be used to identify bottlenecks (if applicable)

      - Captured the following table, which shows how the target value (i.e. 139.2dB) is derived, in Annex of the TR.

| Transmitter |  |
| --- | --- |
| (2a) Number of transmit chains modelled in LLS | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 3840000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: For FR1 downlink, (3b) should satisfy the following: For 4GHz frequency, 24 and 33 For 2.6 GHz frequency, 33 For 700MH and 2GHz frequency, 36 Note: For FR2 downlink, the following should be satisfied: 40 dBm for 100 MHz Urban scenario, 23 dBm for 100 MHz Indoor scenario. Note: no PSD constraint for uplink | 17.16 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. for uplink, (3a) = (3c) | 3840000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 |
| Receiver |  |
| (10a) Number of [receive TxRUs] Note: this row is void (empty) for downlink | 2 |
| (10b) Number of receive chains modelled in LLS | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 |
| (13) Receiver noise figure (dB) | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -98.2 |
| (19) Required SNR (dB) | 5 |
| (20) Receiver implementation margin (dB) | 2 |
| (21) H-ARQ gain (dB) or Process gain for UMTS Note: Only applicable if HARQ is not considered in LLS, | 25 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -116.2 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 139.2 |

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - PUCCH format 3 with 22bits

        - [FL note: final confirmation via GTW is recommended, but FL recommends to drop PUCCH format 3 with 22bits to reduce the workload.]

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | Regarding 3b, for 2.6GHz PSD should be 24 and 33 dBm/MHz based on the previous agreement in the last meeting. PUCCH format 3 with 22bits need be kept |
| Qualcomm | Questions to the FL regarding the calculation of target MCL: - Could we know why the BW is assumed to be 3840000? How many RBs are being assumed here? - Regarding antenna gains at the BS, for the 700M band, we assumed 16 antenna elements (row 10) mapped to 2 or 4 receive chains. We will need to take this into account when computing antenna gains as it impacts row 11. Also, I think per antenna gain (row 11c) was commonly understood to be around 8 dBi. - Is entry for row 21 correct? Is it 25? - How is the required SNR being computed here? Is it a desired value? Also, we should retain PUCCH with 22 bits. There is no reason to drop it just because we are targeting VoIP. When the agreement to study PUCCH was made, we did not impose any restriction on the scenarios/services. Its important to retain this to be compliant with the earlier agreements. The only channel that can be dropped is PUSCH for eMBB. |
| Intel | We share similar view as CATT and QC that PUCCH format 3 with 22 bits needs to be considered for this case. If we have sufficient number of data/simulation results, it is better to include this. |
| Samsung | Targeting VoIP service (it is clear that this proposal is for service dependent target), we don’t see the needs for large UCI payload with both 11 and 22 bits. |
| CMCC | Support to keep PUCCH format 3 with 22 bits |

Summary of the discussion

Given the agreements in the GTW session on 10th November (see section 4.4 for more details), the discussion service dependent metric was finalized. Therefore, the discussion on this section is closed.

[Closed] Details of targets based on relative difference between channels

As discussed in Section 2.2, companies has no clear understanding on the criteria how the relative differential value is used to draw observation and to identify bottlenecks. The table below is captured in section 2.2, but shown again for the companies’ convenience.

| Company | Preferred metric | Criteria to identify bottlenecks |
| --- | --- | --- |
| R1-2007746 ZTE | MIL | Worse than PBCH |
| R1-2007877 CATT | MIL or MCL | Worst and 2nd worst |
| R1-2008624 Qualcomm | MCL | Worse than control coverage |
| R1-2008667 NTT DOCOMO | MIL |  |
| R1-2008024 CMCC | MIL or MCL |  |
| R1-2008478 Apple |  | PDCCH for CSS is used as the benchmark |

After the analyses by FL, it was observed that X-th worst approach is not so easy to apply because the 2nd or 3rd worst channel is not always the same. In addition, The fixed channel for comparison such as PBCH or PDCCH is not good as well because the performance of these channels are basically quite good. Given this situation, FL proposed two options (A-1 and A-2 below) but more discussion is obviously needed.

Updated FL proposal

  - For Relative difference between channels

    - MIL is used to derive relative differential values

    - For each channel, relative differential value is defined by the following formula, and used to draw observations and to identify the bottlenecks (if applicable)

      - Option A-1: (MIL of the channel) – (MIL of the worst channel + X dB)

        - X is chosen considering the realistic value achieved by potential enhancement(s).

      - Option A-2: apply the following 3-step approach [FL note: companies’ views on this option have not been collected yet.]

        - Identify bottleneck channels in RACH procedure, i.e., compare coverage of msg1(PRACH), msg2 (PDCCH/PDSCH), msg3(PUSCH), and msg4(PDCCH/PDSCH).

        - Identify bottlenecks in control channel coverage, i.e., compare coverage of broadcast PDCCH vs PUCCH. This gives us an idea of control coverage that is independent of service rate requirements.

        - Identify bottlenecks in data channel coverage. Compare control channel coverage with data channel coverage.

      - Option A-3: Other approach

    - Relative differential value is applicable to all scenarios

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | We prefer to Option A-1 and we need further study how to determine XdB value |
| InterDigital | We prefer Option A-1 with X=0. As we explained in Section 2.2.5, if all 3 targets are used to identify the global bottleneck channel, we can simplify the relative difference analysis by setting X=0. The joint analysis, using all 3 targets as we explained in Section 2.2.5, will filter out unnecessary enhancements; we do not need to worry about the value for X in the relative analysis. |
| Apple | Option A-1 is simple and preferred, the value of X can be studied further, if the coverage gap is over 10dB, X will be larger than 0. |
| Qualcomm | Option A-2. Operators need this input. Option A-1 is not sufficiently informative, and choice of X is hard to nail down. A-2 also sets us up nicely for the next stage where observations from 8.8.1 are used to drive discussions in 8.8.2. We have 150+ pages of data available to us, and we must make sure to get maximum insight from this data. A-1 is too simplistic and doesn’t do full justice to the rich data set we have at our disposal. |
| Samsung | Our preference is Option A-1. |
| Ericsson | Option A-1. We think the bottlenecks should be considered as a whole to deliver a target service, rather than piecewise. |
| Intel | We prefer Option A-1. |
| ZTE | No strong preference for now. Option A-1 looks simple while it may be hard to converge on the X value. Option A-2 could provide more insights for us. For Option A-2, a few comments from our side: - For the first step, Msg4 HARQ-ACK should be added. - For the second step, our understanding is PUCCH with 2, 11, 22 bits are all included. - For the third step, it’s better to clarify what the control channel is. Does it includes all control channels, i.e., unicast/broadcast PDCCH and PUCCH with 2/11/22 bits? |
| Vivo | Option A-1. |
| CMCC | Support the 1st and 3rd bullets in the proposal. As mentioned in the last round, we have some questions for both options. For the option A-2, we do not think there is a need of 3 step procedure. Finding bottlenecks from all channels is more straightforward. For the option A-1, we kind of understand that the (MIL of the worst channel + X dB) is drawing a new baseline for the coverage and how many dB needed for further enhancement of other channels could be observed. Then, how to determine the value of X needs more discussion. For example, based on the observations from above tables, PUSCH of eMBB is the most limited channel in the urban scenarios. And the achievable improvement of PUSCH eMBB could be very limited, since all the uplink resources may be used in the baseline performance. Option A-1 is slightly preferred though more discussions are needed. |
| Huawei, HiSilicon | In the main bullet, it is better to clarify the purpose to derive relative difference between channels. Regarding the last bullet “Relative differential value is applicable to all scenarios”, a clarification seems needed. Which scenario (Urban, Rural, TDD/FDD) is used to derive the relative difference value so that the relative difference value is assumed to be applicable to all scenario? |

Summary of the email discussion

Option A-1: supported by 8 companies (including slight preference)

  - Benefits expressed by companies:

    - Simplicity

  - Concerns expressed by companies

    - Necessity of determination of X

      - X=0 is proposed by a company. If all the metric can be used for bottleneck identification as discussed in section 2.2.5, use of X>0 will be unclear.

      - X>0, and it needs further study, and might be hard to determine.

    - Too simple: this option cannot make full use of large amount of data

Option A-2: supported by 1 company

  - Benefits expressed by companies:

    - It could give us more insight

  - Concerns expressed by companies:

    - Coverage should be determined by checking whole channels

    - More clarification necessary, which channels are included/excluded

    - Complicated

Other comments

  - One company support 1st and 3rd proposal

  - The purpose of deriving relative difference between channels should be clarified

  - The intention of “Relative differential value is applicable to all scenarios” should be clarified.

FL perspective:

  - FL agrees the following comments made by companies:

    - Complexity of option A-2: FL fully agrees. As shown in the table below, this option requires case-by-case analysis, which is not easy for FL to prepare the text proposal. It might be difficult to finish everything until the end of RAN1#103e

    - It would be difficult to determine X value: Considering the fact that no companies proposed specific value of X during this round of email discussion, FL is afraid that X cannot be decided until the end of RAN1#103e.

    - Value of X wouldn’t be useful depending on the methodology of bottleneck identification: This comment is reasonable. This means that the discussion on X here wouldn’t help the efficient discussion in RAN1.

Given the companies view and FL perspective above, FL would like to propose the following.

Updated FL Proposal:

  - For Relative difference between channels

    - MIL is used to derive relative differential values, and relative difference is used to estimate the necessary amount of enhancements for potential bottleneck channel(s) in dB.

      - [FL note: this addition is a request from a company]

    - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

    - For each channel, relative differential value is defined by the following formula, and used to draw observations and to identify the bottlenecks (if applicable)

      - Option A-1: (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + X dB)

        - Option A-1a: X=3 or 6

          - [FL note: For this case, observation will be drawn like: “Channel A is the worst channel in terms of MIL, and the coverage bottleneck for the given scenario. If enhancement of XdB is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit: Channel B – X2 dB enhancement, Channel C- X3 dB enhancement etc…”
Values of 3 or 6 dB come from the discussion in section 8.8.2.1. FL estimates the maximum gain by the combination of new functionalities. FL also understands that these values are very controversial]

        - Option A-1b: X=0

          - [FL note: For this case, observation will be drawn like: “Channel A is the worst channel in terms of MIL, and the coverage bottleneck for the given scenario. If only one channel is arrowed for enhancement in Rel-17, an enhancement of X1 dB at maximum would be useful to maximize the coverage. If two channels are arrowed for enhancement in Rel-17, an enhancement of X2 dB for channel A and X3 dB for channel B, which is the 2nd worst channel, at maximum would be useful to maximize the coverage, etc…”, this means that we anyway perform “X-th worst” appoach.

        - [FL note: FL preference is option A-1a since it is easy to draft the observation texts. If non-zero X value cannot be agreed immediately, FL would propose to go with X=0 to move forward. Another solution is NOT to use relative difference for drawing observation. 
Note that the decision of X does not impact on the discussion in section 2.2.5]

      - Option A-2: apply the following 3-step approach [FL note: companies’ views on this option have not been collected yet.]

        - Identify bottleneck channels in RACH procedure, i.e., compare coverage of msg1(PRACH), msg2 (PDCCH/PDSCH), msg3(PUSCH), and msg4(PDCCH/PDSCH).

        - Identify bottlenecks in control channel coverage, i.e., compare coverage of broadcast PDCCH vs PUCCH. This gives us an idea of control coverage that is independent of service rate requirements.

        - Identify bottlenecks in data channel coverage. Compare control channel coverage with data channel coverage.

          - [FL note: it seems only one company supports option A-2. A compromise would be to capture an additional observation on, e.g. bottleneck analysis among RACH related channels, but companies’ opinion need to be collected first]

      - Option A-3: Other approach

    - Relative differential value is applicable to all scenarios

      - [FL note: this bullet is merged into the new 2nd bullet]

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | Option A-1b: X=0 should be straightforward and For Option A-1a, we need also consider whether enhancement technology can reach this X when we determine X value. |
| InterDigital | We support Option A-1b mainly because an agreeable value for X may be difficult to find. If the choice of X is based on expected performance out of enhancements, due to modular structures of enhancements (i.e., multiple enhancements can be combined to get more performance gain) the amount gain expected out of enhancements can be large. Thus, typical values for X, based on the current list of enhancements, are not clear. We are open to other perspectives to determine X, but based on the current situation X=0 seems to be a reasonable choice. |
| China Telecom | The final goal is to decide which channels are identified as the bottleneck channels. In that sense, in our understanding, for relative metric, maybe we do not need to discuss which option is adopted in this proposal. And we can directly discuss how many worst channels are identified as the bottleneck channels. Regarding the amount of enhancements, there may be two options: Option 1: The gap between target MPL derived from target ISD and baseline MPL. Option 2: The specific value may not be needed, i.e. in a manner of best efforts. |
| Huawei, HiSilicon | Option A-1a is preferred because it seems clearer. |
| Intel | We slightly prefer Option A-1a. For Option A-1b with X=0, it is simply to identify the worst channel among all the evaluated channels, and may not be used to determine amount of enhancement for the bottleneck channels. This seems contradictory to the main bullet “relative difference is used to estimate the necessary amount of enhancements for potential bottleneck channel(s) in dB”? |
| ZTE | For Option A-1, we prefer A-1a, while we agree FL that it would be controversial to agree on an accurate X value. On the other hand, Option A-2 seems easier if it is simplified to define two different targets for initial access channels and non-initial access channels. In that case, we are also fine with Option A-2. |
| Samsung | We prefer Option A-1b. Do we need to estimate the necessary amount of enhancements for potential bottleneck channel(s)? This is commented in next section too. |
| vivo | Option A-1b is preferred |
| CMCC | Option A-1b is slightly preferred. The value X should be derived from the observation of the performance of enhancement collected. And the X value is hard to determine and may induce a long and hard debate. Another point is that 3 or 6 dB enhancements maybe too optimistic. |

Update after the GTW session on 10th November:

It was confirmed that the discussion on relative difference is still controversial. In the meantime, it is not clear from FL perspective if relative difference is essential for FR1 to finalize this SI. Given this reason, FL would like to check companies position.

Updated FL proposal:

For Relative difference between channels

  - MIL is used to derive relative differential values.

  - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

  - For each channel, relative differential value is defined by the following formula for at least FR1

    - Alt 1: (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + 3 dB)

    - Alt 2: (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + 0 dB)

    - Alt 3: (MIL of the channel) – (MIL of the worst channel among the non-data channels that have more than 3 samples)

If RAN1 cannot reach consensus on the proposal above by 3:00pm UTC on 11th November, RAN1 will not spend time to discuss relative difference between channels at least for FR1

  - If the decision is further delayed, FL cannot prepare the analysis and/or observation

Companies are invited to provide their views on the alternatives. If we can see majority support without strong concern, FL would like to adopt it as a FL proposal.

| Company | Alt for support | Alt for concern | Comments |
| --- | --- | --- | --- |
| CATT | Alt.2 | Alt.3 | It is hard to determine X value at this moment so we prefer to X=0. For Alt.3, we think the motivation of MIL of the worst channel among the non-data channels as reference channel isn’t clear. |
| Apple | Alt.1 |  | As commented during the online discussion, if X=0dB, then we still need another baseline value to determine the other bottleneck channel(s) except the worst channel. Checking the results collected by the FL, identified bottleneck channels with X=3 is almost the same as Alt.3. For Alt.1, PUSCH is always the worst channel in any scenarios, the worst non-data channel in Alt.3 is changing for different scenarios. |
| vivo | Alt.2 |  | Increasing the relative gap by 3dB, and do not change the relative relationship, therefore, we think 0dB is enough to acquire the relative relationship, no need to define other value. |
| Qualcomm | Alt 1 and Alt 3 |  | @CATT, Apple as we explained during the online session, the concern with Alt 1 is that it is unduly influenced by PUSCH data rate requirements (1 Mbps in urban cases, and 5 Mbps in FR2 are too high when determining cell coverage). The idea is to view these data rate requirements as “soft targets” while also paying more attention to more basic cell-coverage that is determined based on non-data channels. You can think of this as a bigger cell radius that determines the overall network footprint to offer some rudimentary or basic services/coverage. Alt 3 aims to identify bottlenecks from this perspective as well. I hope this helps explain our motivation better. We hope you can reconsider your position. @Vivo, I suspect there may be some misunderstanding. With Alt 2 only the worst channel is identified. We need a margin to identify other potential bottlenecks. |
| Intel | Alt. 1 | Alt. 3 | If the intention of relative difference is only to identify the worst channel, we are also fine with Alt. 2. For alt. 3, we share similar view as CATT that it is unclear the motivation. In some cases, using MIL of non-data channel as reference may over-compensate some channels. |
| Ericsson | Alt 2. (first choice), Alt 1 (second choice) |  | Alt3 has the merit that it addresses the fundamental limit of coverage, since that will be set by the control channels. However, Alt 3 essentially says that we will attempt to improve a control channel, since the baseline is a control channel, not the worst overall channel. We can still choose to do this with Alt 1 or Alt 2, however, as can be seen in section 2.5.2. While Alt 1 or Alt 2 are both reasonable, Alt 2 actually seems pragmatic from the perspective of determining the amount of effort in a WI. For Alt1, if some channel is 3.1 dB worse than the bottleneck according to the representative value and we use X=3, then we may debate for some long period of time on if 3.1 dB is an accurate number, or it is actually 2.9 dB. With Alt2, we identify the N worst channels, indicate how much gain is needed, and then leave it to the WI to see how much these N bottlenecks can be improved, whether the specification impacts, complexity etc., while deciding which channels should be enhanced. A question on the details of Alt2, similar I think to Intel’s comment in the last round: For the case where one channel is allowed, is the value X1=(MIL of the channel) – (MIL of the second worst channel among the channels that have more than 3 samples + X dB)? Or is there another step and/or factor needed to determine X1? Identifying the exact order of the bottlenecks may be challenging given the variation we have in our results. Please find below a chart comparing the MIL channels in all scenarios for FR1. The red error bars are +/- 1 standard deviation high. (The data are a bit outdated, but the trends should be reasonable) Here we can see that the standard deviation is larger than the differences between many channels. So we propose that the uncertainty in our link budget estimates be taken into account when reporting bottlenecks, and that some measure of uncertainty be reported along with the differential MIL values. |
| ZTE |  |  | If the intention is only to calculate the relative differential value instead of identifying the bottleneck channels, we are fine with either alternative here. If it is also for bottleneck identification, we are fine with Alt2 with identifying the N worst channels as mentioned by Ericsson (we may even no need to agree on a specific value for N if we just list an order of all channels in terms of MIL value ). Another way is we can go with Alt 3 by updating to (MIL of the target channel) – (MIL of PUCCH F1), which aligns the proposal in FR2. |
| Samsung | Alt.1 |  | We can live with Alt. 2 as well. We acknowledge Ericsson comment on the uncertainty of link budge estimate. |
| OPPO | Alt 1 and Alt 3 |  | For Alt.1, X=3dB is determined through the maximum gain by the combination of new functionalities. It is based on the dB that we can actually enhance to identify bottleneck channels. As Apple says, “For Alt.1, PUSCH is always the worst channel in any scenarios, the worst non-data channel in Alt.3 is changing for different scenarios.” In other words, Alt.1 ignores the enhancement of non-data channel, while Alt.3 takes into account the enhancement of non-data channel though (MIL of the non-data channel) – (MIL of the worst channel among the channels that have more than 3 samples + 3 dB) > 0. We prefer the Alt.1 and Alt.3. |
| InterDigital | Alt. 2 |  | By setting X=0, the bottleneck channel can be identified. Following the identification, the bottleneck channel can be improved to match the coverage of the reference channel. An example of the reference channel can be the second worst channel (if one channel is to be enhanced) or control channel. We think the purpose of the relative analysis to identify the bottleneck and imbalance in coverage among channels/signals. Thus, matching the coverage of the bottleneck channel to one of the channels with better coverage compared to the bottleneck channel seems to partially improve the imbalance in coverage among the channels/signals. |
| CMCC | Alt 2(1st preference) Alt 1 (2nd ) | Alt 3 | Alt 2 is 1st preference from our side, since the value of X is hard to be determined. And we also think 3dB enhancement may also be challenging for the worst channel and some other channels, unless higher power UE (26dBm) is considered. We may understand that QC’s intention is to enhance the channels other than the data channel and provide a more robust coverage. But the enhancement of data channel is also important. And we are also fine with Alt 2 with N worst channels, which could also cover Alt 3. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Alt 2 |  | Same comment as Interdigital. |

Summary of the discussion:

Alt 1: (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + 3 dB

  - Support: Apple, Intel, Ericsson (2nd), (ZTE), Samsung, CMCC(2nd)

  - Concern: No company

Alt 2: (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + 0 dB)

  - Support: CATT, vivo, Ericsson, (ZTE), InterDigital, CMCC(1st), Samsung(2nd), Intel(2nd), IITH et al,

  - Concern: : No company

Alt 2bis

Alt 3: (MIL of the channel) – (MIL of the worst channel among the non-data channels that have more than 3 samples)

  - Support: (ZTE)

  - Concern: CATT, Intel, CMCC

Alt 1 and Alt 3

  - Support: Qualcomm, Oppo

Note: new alternative was proposed by a company, but FL did not capture it because RAN1 has not time to discuss it.

It seems that Alt .2 got the largest supports while no concern was raised. Therefore, FL would like to conclude that Alt.2 is the most appropriate way to go.

On the other hand, a related issue was brought up during the email discussion, i.e. determination of how many channels (N) can be identified as bottleneck by using this metric. It is obvious that no time is left for this discussion, thus N should be discussed on case-by-case basis. FL would also like to point out that observation and bottleneck identification can be done without explicit decision on N.

Furthermore, it was pointed out the necessity to consider uncertainty when reporting bottlenecks. This can be taken into account in section 2.3.X (observation) and 2.5.2 (bottleneck identification), and hence FL has no plan to discuss this issue in this section.

Given the summary above, FL would like to propose the following.

FL proposal:

For Relative difference between channels

  - MIL is used to derive relative differential values.

  - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

  - For each channel, relative differential value is defined by the following formula for FR1

    - (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples)

If any concern is raised for this proposal, FL assumes that RAN1 has no consensus on relative difference between channels for FR1.

Conclusion:

No concern was raised in the email thread for this proposal. Hence, FL assumes the proposal is stable now.

| Company | Comments |
| --- | --- |
| Qualcomm | - Could you please outline how bottleneck identification happens after the relative diffrerences are captured? How many bottleneck channels do we identify per scenario? |

Update on 13th November

The question by Qualcomm was addressed during the GTW session. Therefore, no issue is left for this section.

[Closed] Interaction among three target metrics for bottleneck identification

While three targets can be used to draw observations, it is not preferable to keep so many options for bottleneck identification, which has already been pointed out by a company in RAN1#102-e. This issue was not discussed yet due to the lack of time. Some companies showed their preference to use a single target while no company proposed to use multiple target metrics.

From FL perspective, it would be good to apply an approach to use all the three targets as a compromise. This is the motivation why FL made the following proposal.

FL Proposal

  - For the interaction among Scenario dependent targets, Service dependent targets and Relative difference for bottleneck identification,

    - Option B-1: use scenario dependent targets only

    - Option B-2: use relative difference between channels only

    - Option B-3: use all the 3 targets, e.g. [FL note: this is FL preference]

      - Absolute targets (i.e. scenario dependent targets and service dependent target) are used to filter the channels/signals, i.e. can be a bottleneck or not.

      - Apply relative difference to the filtered channels/signals, which do not meet the absolute targets, to estimate the necessary amount of enhancements (in dB)

    - Other options are not precluded

Companies are invited to provide their views on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | - We prefer to Option B-3 |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | - Option B-1 |
| Interdigital | - We prefer Option B-3. For example, a bottleneck channel can be identified for each target (scenario target, relative, if applicable service dependent target). Then, the bottleneck channel that is identified across all 3 targets can be labeled as the global bottleneck channel for which enhancements can be considered at higher priority, e.g., if PUSCH VoiP is identified as the bottleneck channel in 4GHz TDD, using the service dependent target, scenario dependent target and relative analysis, it will be labeled as the global bottleneck channel for which enhancement is prioritized. Relative analysis can be conducted by each company since companies simulated different sets of channels/signals. |
| Apple | - Option B-3 is preferred. As commented above, different performance will identify different coverage bottleneck channel. At least we can identify the minimum set of bottleneck channels with three target metrics. |
| China Telecom | - We prefer Option B-3. |
| Qualcomm | - Option B-3. Good to capture observations from all 3 methodologies. |
| Samsung | - We are fine with Option B-3 in principal. Both absolute targets and relative difference can be used to identify bottleneck channels. In addition, we don’t need to estimate the necessary amount of enhancements (in dB). So option B-3 can be simplified such as: - Modified Option B-3: use all the 3 targets |
| Ericsson | - We are OK with option B-3, but think that it should be refined a bit if it is to help with the decision making process. Given that the relative values should be more accurate than the absolute values, we think they should be prioritized. If there is a clearly defined operator request for an absolute value, this should also be prioritized, although the limits of our methodologies for coverage determination should be taken into account when determining if the requirement is met. The scenario dependent values also seem a bit difficult to determine good ISDs for, as seen by the discussion above where we try to pick ISDs that fit all of the different scenarios. So our proposal is: - Option B-3a - Use all 3 targets, prioritizing both a) relative values and b) absolute coverage targets requested by operators. |
| Intel | - We prefer Option B-3. We think the approach mentioned by Interdigital would be a good way forward. |
| ZTE | - We are fine to use all three targets, while we don’t see the reasoning to mix them up together. In other words, we could make observations/bottleneck identification independently based on these three independent targets. So, the sub-bullets under Option B-3 are not needed. |
| vivo | - Option B-3 is preferred. - Option B-2 can not be used alone, since it may identify some “bottleneck channels” which can fulfill the absolute target, further enhancements on “bottleneck channels” identified through option B-2 may be not necessary. |
| CMCC | - We are fine with option B-3 in principle. - Firstly, we use the absolute targets to identify the bottleneck channels, namely the 1st bullet. Then base on the observations from the 1st bullet, we could discuss how many channels and how much gaps needs enhancements. |
| Huawei, Hisilicon | - We prefer Option B-1, but Option B-3 with CMCC’s clarification is OK for us. |

Summary of the email discussion

Option B-1: use scenario dependent targets only

  - Supported by 2 companies (one companies is OK for B-3 with some clarification)

Option B-2: use relative difference between channels only

  - No support

Option B-3: use all the 3 target metrics

  - Supported/preferred by 12 companies in principle. However, companies has different preference in detail. FL made 3 sub-options below by roughly combining companies’ view.

  - Option B-3a (based on FL proposal)

    - Absolute target metrics (i.e. scenario dependent targets and service dependent target) are used to filter the channels/signals, i.e. identify bottleneck channels.

      - [Prioritize both relative values and absolute coverage targets requested by operators (FL note: this will be discussed in section 2.2.2)]

    - Apply relative difference to the filtered channels/signals, which do not meet the absolute targets, to estimate the necessary amount of enhancements (in dB)

      - This includes the discussion how many channels and how much gaps needs enhancements.

  - Option B-3b (based on InterDigital proposal)

    - All target metrics are used to identify bottleneck channels/signals, and label as “global bottleneck channel(s)/signal(s)”

      - [Prioritize both relative values and absolute coverage targets requested by operators (FL note: this will be discussed in section 2.2.2)]

    - Apply relative difference analysis [to estimate the necessary amount of enhancements (in dB)]

      - This includes the discussion how many channels and how much gaps needs enhancements.

  - Option B-3c (based on Samsung & ZTE proposal)

    - All target metrics are independently used to identify bottleneck channels/signals

      - [Prioritize both relative values and absolute coverage targets requested by operators (FL note: this will be discussed in section 2.2.2)]

    - Do not estimate the necessary amount of enhancements

Given the companies view above, clear majority of companies are fine with Option B-3 in principle. Therefore, FL would like to propose the following:

- FL proposal:

Use all the 3 target metrics for identification of coverage bottleneck channel(s) and estimation for necessary amount of enhancements in dB (if applicable)

  - At least absolute metrics (i.e. scenario dependent and service dependent) are used for the identification of coverage bottleneck channel(s)

  - At least relative difference is used for the estimation of necessary amount of enhancements in dB (if applicable)

  - The other details are FFS, e.g.

    - Whether or not relative difference is used for identification of coverage bottleneck channel(s)

      - Note: this means RAN1 may or may not define criteria for this metric

      - (FL note: FL recommendation is “no” to skip the discussion on criteria)

    - Whether or not RAN will perform the estimation of the necessary amount of enhancements (in dB)

      - (FL note: if not, the discussion will have an impact on the WID drafting)

      - (FL note: see related question below. Companies are invited to share their view)

    - How many channels and how much gaps can be enhanced

      - Note: this means that RAN1 does not guarantee the achievement of scenario depended and service dependent targets.

      - (FL note: case-by-case basis discussion necessary. FL plans to discuss this issue under section 2.5.2 and 2.5.3)

Companies are encouraged to share your views on this proposal.

| Company | Comments |
| --- | --- |
| CATT | - Regarding description of relative difference, we prefer the original description “Apply relative difference to the filtered channels/signals, which do not meet the absolute targets, to estimate the necessary amount of enhancements (in dB)” |
| InterDigital | - We support the FL’s proposal. Regarding treatment of relative analysis, we provide our view as an answer for Question 1. |
| China Telecom | From our perspective, absolute metric is more appropriate for identifying bottleneck channels and target performance. If relative metric is indeed necessary, in our understanding, the final goal is to decide which channels are identified as the bottleneck channels. In that sense, we can directly discuss how many worst channels are identified as the bottleneck channels. Regarding the amount of enhancements, there may be two options: Option1: The gap between target MPL derived from target ISD and baseline MPL. Option 2: The specific value may not be needed, i.e. in a manner of best efforts. |
| Nokia/NSB | Agree with CATT. We think that introducing an “order” between absolute and relative metrics may be beneficial, and arguably yield a more pragmatic approach in which we first select a number of candidate bottlenecks and then we restrict the focus to the most important direction(s), e.g., (but not necessarily only) PUSCH. |
| Huawei, HiSilicon | We agree with the FL’s proposal that at least absolute value is used for identifying coverage bottleneck channel and relative difference may be additionally used for estimation of coverage enhancement target. Also, same with FL’s view that we don’t think using the relative difference to identify the coverage bottleneck channel is an appropriate method. |
| Qualcomm | We must perform coverage bottleneck identification using relative comparisons as well. Absolute comparisons require a large number of artificial assumptions, and its not clear this has any practical relevance. We propose the following: Use all the 3 target metrics for identification of coverage bottleneck channel(s) and estimation for necessary amount of enhancements in dB (if applicable) - At least absolute metrics (i.e. scenario dependent and service dependent) and relative metrics are used for the identification of coverage bottleneck channel(s) - At least relative difference is used for the estimation of necessary amount of enhancements in dB (if applicable) The amount of enhancement necessary need not be part of this exercise. This should save us some time and effort. RAN1 can also not guarantee the achievement of the identified targets. A large number of variables are at play and we cannot commit to meeting these targets across all scenarios and services. |
| Intel | We are fine with FL’s proposal that at least absolute target is used for bottleneck identification. We also share similar view as CATT and Nokia that relative difference is only applied for the bottleneck channels which are identified based on absolute target. |
| NTT DOCOMO | We support to use relative comparisons for the bottleneck identification, as well, since the absolute value may not reflect the real environment / deployments that have large difference over scenarios and variability. And for the absolute values (MPL), the specific values for the parameters related to MPL were not defined, so that identification using MPL seems very complicated to take the difference of the values into account, especially for FR2. |
| ZTE | It seems the main bullet (saying to use all three targets for bottleneck identification) and sub-bullets (saying only consider the absolute targets for bottleneck identification) is kind of contradictory. We prefer to consider both absolute target and relative target for bottleneck identification. On the other hand, we are also fine to first discuss absolute target as high priority. |
| Samsung | We are fine with FL’s proposal in principal. As commented above, we want to clarify whether the necessary amount of enhancements in dB is needed and how to define the proper values. |
| vivo | We support proposal from FL |

Furthermore, FL would like to ask the following question to go to the next step

Question 1: Whether relative difference can/should be used for bottleneck identification alone?

This issue is pointed out by vivo. FL agrees this concern because more additional information (i.e. criteria) is necessary to use relative difference for bottleneck identification. As you can see in section 2.2.4 (for example, how many channels and how much gaps needs enhancements etc.), this discussion will consume large amount of time. If bottleneck identification can be done by absolute metric(s) only, we can save much time for GTW/email discussion.

Please provide your answer in the table below.

| Company | Comments |
| --- | --- |
| CATT | - Relative difference should be used for bottleneck identification alone by MIL. It is easy to identify bottleneck channel by relative difference method. |
| InterDigital | - Relative analysis should be used jointly with other targets. From our point of view the main purpose of the relative analysis is to identify and resolve imbalance in coverage performance among channels/signals. The imbalance needs to be identified with aligned simulation assumptions among channels/signals. - If the discussion in 2.2.4 cannot be resolved, one option is to reserve the use of relative analysis until work item phase during which the gain obtained from enhancements is clarified. If the relative analysis is to be used in the work item phase, the conclusion of the study item (identification of bottleneck channels) may be based on service & scenario dependent targets. Then during the work item phase, additional channels/signals that need enhancements (they can be non-bottleneck channels) can be identified by the relative analysis. |
| China Telecom | Firstly, we also think bottleneck identification can be done by absolute metric(s) only. And we do not think relative difference can be used for bottleneck identification alone. If relative metric is indeed necessary, in our understanding, the final goal is to decide which channels are identified as the bottleneck channels. In that sense, we can directly discuss how many worst channels are identified as the bottleneck channels. Regarding the amount of enhancements, there may be two options: Option1: The gap between target MPL derived from target ISD and baseline MPL. Option 2: The specific value may not be needed, i.e. in a manner of best efforts. |
| Nokia/NSB | Relative analysis alone does not seem aligned with most operators’ preferences and inputs. We think it can be used to identify bottlenecks after absolute targets have been used to down-select a pool of candidate bottleneck channels. We think we should focus on what FL’s charts are already telling us and be pragmatic. For instance, assuming to apply a first filter based on absolute targets to reduce the number of candidate channels, we can then focus on a target channel and derive relative differences w.r.t. it. According to what has been observed above, one such channel could be PUCCH F1. Other channels could be considered as well, of course, provided pragmatism is applied. |
| Huawei, HiSilicon | We prefer not to use relative difference for bottleneck channel identification alone. |
| Qualcomm | Relative differences alone should be allowed to identify coverage bottlenecks. There is nothing wrong with this approach. In fact, this is more robust and simpler compared to the amount of reverse engineering we are having to do with absolute comparisons --- we are now picking ISDs that let us draw conclusions that fit a certain narrative. |
| NTT DOCOMO | Relative difference can be used for bottleneck identification alone, and we think it’s simple way to identify the bottleneck, e.g. identified with the worst channel. |
| ZTE | We are fine to prioritize absolute target first. |
| Samsung | Relative difference can be used for bottleneck identification with absolute targets. |
| vivo | No, it may identify some as “bottleneck channels” which can fulfill the absolute target, further enhancements on “bottleneck channels” identified through this method may be not necessary. |

Question 2: Whether or not RAN will perform the estimation of the necessary amount of enhancements (in dB)?

It seems at least one company doesn’t see the necessity of such estimation. Our workload in this SI can be reduced if this procedure can be skipped. However, FL is afraid that lack of such information will lead to more complicated discussion when drafting the WID. FL would like to ask companies if Ran1 should perform this estimation or not.

Please provide your view in the table below.

| Company | Comments |
| --- | --- |
| CATT | - RAN1 need firstly execute the estimation of the necessary amount of enhancements - (in dB) and based on estimation result, RAN1 can give some observations and - potential conclusion in TR. |
| InterDigital | - As we commented in 2.2.4, given a list of technical enhancements and modular nature of the enhancements, necessary amount of enhancements may be difficult to compute. RA1 should not perform such estimation at this point. |
| China Telecom | We think it is RAN1 task. Regarding the amount of enhancements, there may be two options: Option 1: The gap between target MPL derived from target ISD and baseline MPL. Option 2: The specific value may not be needed, i.e. in a manner of best efforts. |
| Nokia/NSB | From our perspective it is more important to identify the bottlenecks, and report in the TR which channel/value has been used to identify bottlenecks. Stating how many dBs are necessary for each bottleneck channels may be a false good idea. RAN1 has a big opportunity in the upcoming WI, i.e., enhancing performance of bottleneck channels to experience gains at system level. Why would RAN1 want to enhance MCL/MIL/MPL by a minimum number of dBs instead of aiming at the maximum number of dBs instead? RAN1 should do its best to specify the enhancements which guarantee the maximum performance increase, regardless of a specific minimum number of dBs. |
| Huawei, HiSilicon | Yes if time permits. |
| Qualcomm | RAN1 does not need to specify the amount of enhancement required. It suffices to identify the bottleneck channels. Among the proposed enhancements, we can then pursue the ones that offer the best means to close the coverage gaps. |
| Intel | We share similar view as other companies that this could be best efforts to perform the estimation of X dBs for enhancement. In our view, the most important thing is to identify the bottleneck channels. |
| ZTE | Similar view as other companies, we need to first identify the bottleneck channels. |
| Samsung | Like other companies already commented, identifying bottleneck channels is most important. WID may not necessarily require the estimation of the necessary amount of enhancements. |
| vivo | Similar to comments from other companies, main goal in RAN1 is to identify bottleneck channel (this is already tough!), and achievable enhancement in terms of X dB is based on best effort. |

Summary of the discussion

Given the situation in the GTW session on 10th November, it is not good idea to take a step-by-step approach to conclude this SI in a timely manner. FL would like to close the discussion on this section, and directly go to the bottleneck identification discussion in section 2.5.2.

[Closed] Handling of 2nd priority scenarios

The following proposal was not intensively discussed because of the lack of companies’ interest

Rural with long distance 700MHz FDD LOS O2O – 12, 20 and 30km for observation and bottleneck identification is not performed for this scenario

Rural with long distance 4GHz TDD LOS O2O – 12, 30km for observation and bottleneck identification is not performed for this scenario

Companies are encouraged to provide their views, if any. Please note that the discussion will take place on a best effort basis.

| Company | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | - As mentioned by us several times during the discussions, 4 GHz is an important deployment scenario. We want the coverage of this scenario to be improved. Having no analysis on this practically relevant scenario is not encouraging. - We have shown how 30km coverage is possible in our tdocs. However, most companies seem to agree that 12 km is a reasonable target. In the interest of progress, we would like to revive the old proposal from FL and intorduce bottleneck identification for 12km ISD. |
| Qualcomm | - We should analyze and capture conclusions for at least 1 long distance scenario, including bottleneck identification. ATT did mention deployments in Australia with very large ISDs. So there is some merit in addressing this scenario. |

Summary of the email discussion

Given the companies request above, FL would like to propose the following:

FL proposal:

Rural with long distance 4GHz TDD LOS O2O – 12km for observation and bottleneck identification

Companies are invited to provide their views, if they have a concern.

| Company | - Comments |
| --- | --- |

Final summary:

The proposal was agreed in the GTW session on 13th November. The discussion in this session is closed.

Observation from evaluation results

[Closed] How to aggregate evaluation results from different companies and limit number of scenarios of interest for bottleneck identification (common for FR1 and FR2)

Related to the discussion in section 2.2, it is necessary to define a single value from the evaluation results submitted by companies, which is used for the comparison with the target value(s). However, RAN1 has not decided how to define such value. We need to discuss how the single value (hereafter referred to as “representative value” in this document) can be derived from the evaluation results with different simulation assumptions/parameters. However, FL thinks this is not an easy exercise because of the following issues:

Issue 1. Diversity of companies’ evaluation results

As of Oct. 23rd, the word document of submission template reaches 90 pages for FR1 (and 40 pages for FR2, please check the FL document for the details), respectively. This huge amount of data makes our analysis very complicated. Here is an example below (these figures and table are made from the submission template version 034) explaining why it is not so easy:

Figure 2.3.1-1. An example of companies’ evaluation result

Table 2.3.1-1. Summary of companies’ evaluation results for Urban 4GHz TDD (DDDSU)

|  | MCL |  |  | MIL |  |  | MPL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mean | median | range | mean | median | range | mean | median | Range |
| PUSCH eMBB | 130.0 | 130.5 | 18.6 | 139.4 | 139.3 | 14.4 | 110.7 | 110.0 | 16.9 |
| PUSCH VoIP | 142.7 | 142.5 | 21.5 | 152.2 | 151.9 | 24.8 | 119.7 | 120.8 | 11.4 |
| PUSCH CSI | 146.0 | 146.0 | 4.0 | 155.4 | 155.4 | 4.0 | 130.3 | 130.3 | 3.9 |
| PUCCH F1 2bit | 147.4 | 147.9 | 22.0 | 158.3 | 156.7 | 13.8 | 123.2 | 122.8 | 5.6 |
| PUCCH F3 11bit | 145.0 | 146.4 | 19.3 | 155.8 | 155.2 | 8.1 | 123.4 | 121.3 | 17.2 |
| PUCCH F3 22bit | 141.9 | 143.7 | 27.6 | 154.5 | 152.2 | 17.3 | 119.3 | 118.4 | 10.8 |
| SSB | 151.3 | 151.4 | 16.8 | 160.5 | 160.8 | 11.3 | 129.5 | 129.4 | 19.9 |
| PRACH | 144.5 | 144.8 | 20.7 | 155.4 | 153.6 | 12.3 | 124.8 | 121.8 | 21.0 |
| PDCCH Msg2 | 146.5 | 147.5 | 13.0 | 159.0 | 159.3 | 17.0 | 128.6 | 125.5 | 25.3 |
| PDSCH Msg2 | 150.4 | 151.0 | 18.8 | 159.8 | 160.5 | 16.8 | 131.8 | 130.0 | 22.4 |
| PUSCH Msg3 | 142.6 | 143.5 | 19.0 | 153.6 | 152.4 | 8.5 | 124.9 | 122.9 | 12.1 |
| PDSCH Msg4 | 148.4 | 148.5 | 22.0 | 157.6 | 157.5 | 18.8 | 129.2 | 129.2 | 24.5 |
| PUCCH w/ HARQ-ACK Msg 4 | 140.7 | 140.7 | 18.6 | 151.7 | 151.7 | 8.4 | 121.3 | 121.3 | 0.6 |
| PDCCH | 152.8 | 153.6 | 23.5 | 164.1 | 164.2 | 13.1 | 131.9 | 130.4 | 20.1 |
| PDSCH eMBB | 150.1 | 147.8 | 22.1 | 159.1 | 158.3 | 19.8 | 131.0 | 127.5 | 28.7 |

Note: Column “range” mans the gap between the best and the worst value submitted by companies.

Figure 2.3.1-2. Summary of companies’ evaluation result for Urban 4GHz TDD (DDDSU only)

Originally, it was expected that MCL could achieve a good alignment among companies because of the reduced flexibility of parameter/value choice. It seems that most of the companies have a good alignment, but there are exceptional results far from the average value (peak-to-peak gap reaches 20dB!). This tendency is common for MIL and MPL. Ideally, the reason for this divergence can be clarified by checking the detailed simulation assumptions, and we can decide if the value can be used for the statistical analysis. However, this approach will not be realistic because we might need to repeat this approach for approx. 100 times (#of channels * #of scenarios *#of categories) while the available time at RAN1#103-e is limited.

FL observation 1

Evaluation results provided by companies display very large variance

  - There might be a risk to draw unreasonable conclusion if the representative value is derived an inappropriate way

On the other hand, it is quite challenging to discuss and clarify all the details of simulation results over email and/or GTW during the meeting period.

Issue 2. Many scenarios for evaluation

Ideally, we should look at all the scenario and frame structure to identify the bottleneck channels scenario by scenario. However, it was found from the latest evaluation template that the number of submissions is quite different depending on scenario.

Table 2.3.1-2. Number of contributors

|  | Urban 4GHz |  | Urban 2.6GHz | Rural 4GHz |  | Rural 2.6GHz | Rural 2Ghz | Rural 700Mhz | Rural L.D 700MHz | Rural L.D. 4GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DDDSU | DDDSU DDSUU | DDDDD DDSUU | DDDSU | DDDSU DDSUU | DDDDD DDSUU | UUUUU | UUUUU | UUUUU | DDDSU | DDDSU DDSUU |
| PUSCH for eMBB | 13 | 12 | 8 | 11 | 10 | 6 | 9 | 11 | 8 | 5 | 3 |
| PUSCH for VoIP | 9 | 10 | 6 | 9 | 8 | 5 | 9 | 9 | 8 | 3 | 3 |
| PDSCH for eMBB | 10 | 9 | 6 | 7 | 7 | 5 | 9 | 11 | 7 | 3 | 3 |

To accelerate the discussion in RAN1, it might be good to limit the scenario for bottleneck identification. One idea is to choose a representative scenario and/or frame structure, which has sufficient number of evaluation results.

FL observation 2

Identification of bottleneck channel should be done by focusing on a limited number of scenarios/frame structures to accelerate the discussion in RAN1.

There is a risk to make a wrong decision if we use the evaluation results with small number of samples.

Issue 3. Need of categorization

Similarly, we should also discuss if some categorization is necessary for deriving a representative value. For example, we have an agreement to perform evaluations for LOS/NLOS and O2I/O2O and companies are requested to submit the result separately. The result on PUSCH for eMBB (DDDSU) under Rural 4GHz TDD scenario is summarized in the table 2.3.1-1 below.

Table 2.3.1-3. Evaluation results on PUSCH (DDDSU) for eMBB under rural 4GHz scenario

|  | MCL |  |  | MIL |  |  | MPL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mean | median | range | mean | median | range | mean | median | Range |
| NLOS O2I | 134.52 | 138.79 | 20.23 | 141.38 | 143.29 | 32.87 | 123.59 | 125.66 | 18.30 |
| NLOS O2O | 135.81 | 138.63 | 15.96 | 141.51 | 142.98 | 22.63 | 126.45 | 127.37 | 8.52 |
| LOS O2I | 133.65 | 133.10 | 18.69 | 141.37 | 143.37 | 24.67 | 123.84 | 126.11 | 7.49 |
| LOS O2O | 136.17 | 138.16 | 18.67 | 144.97 | 143.51 | 30.61 | 128.12 | 129.32 | 5.16 |

Note: Column “range” mans the gap between the best and the worst value submitted by companies.

This table shows that the difference between NLOS/LOS and O2I/O2O (~5dB) is marginal compared with the difference among companies (~20dB). In addition, the number of contributors is different, i.e. 11 results for NLOS O2I while 4 results for LOS O2O. Ideally, it would always be good if a representative value is derived by categorizing NLOS/LOS and O2I/O2O, but it is not clear if the categorization is so meaningful for this case from workload and statistical point of view. One solution would to focus on only one scenario (e.g. NLOS O2I for rural, and LOS O2O for rural with long distance) for coverage bottleneck analysis.

Other parameters such as delta value, UE speed, PRACH format and Tx power can potentially be used for the categorization. However, too many categories will lead to the huge workload and less number of samples for each category. FL thinks that RAN1 should be careful if we introduce categorization for the evaluation results.

FL observation 3

RAN1 needs to take into account whether and how the evaluation results are categorized on top of scenario, frame structure, channel (including PUCCH format)

  - Candidate categories are LOS/NLOS, O2O/O2I, UE speed, RACH format, DL Tx power etc.

    - It should be kept in mind that number of samples is quite deferent depending on the parameters. Hence, it is not clear if the categorization works well from statistical point of view.

  - Smaller number of categories can help reduce the workload, while it is not so logical to apply averaging for the values derived using different parameters.

    - FL recommends to have a discussion on this issue at the beginning of RAN1#103e

Given the observations above, FL thinks the following approach can be considered at RAN1#103e.

Define a representative value for each channel/scenario/frame_structure/category from the submitted evaluation results, which is used for the comparison with target value(s).

  - The following scenario/frame structure pairs, which have a sufficient number of evaluation results, are used for the analysis of bottleneck identification

    - For FR1:

      - Urban 4GHz TDD (DDDSU, [DDDSUDDDSUU – FL note: FL doesn’t think this format give us important information on top of DDDSU given that UL channels will be the bottleneck] )

      - [Urban 2.6GHz TDD – FL note: it should be discussed if it can give us different observation from 4GHz]

      - Rural 4GHz TDD (DDDSU, [DDDSUDDDSUU] )

      - [Rural 2.6GHz TDD – FL note: it should be discussed if it can give us different observation from 4GHz]

      - [Rural 2GHz FDD - FL note: it should be discussed if it can give us different observation from 700MHz]

      - Rural 700MHz FDD

      - Rural with long distance 700MHz FDD

      - [Rural with long distance 4GHz TDD (DDDSU, DDDSUDDSUU) – FL note: the number of samples is smaller than other scenarios]

    - For FR2:

      - To be discussed under A.I.8.8.1.2

  - For each scenario, representative value(s) is/(are) derived for each channel/format, i.e.

    - PUSCH for eMBB

    - PUSCH for VoIP

    - [PUSCH for CSI: FL note – need more discussion because the number of data is not sufficient from statistical point of view]

    - PUCCH Format 1 with 2bits

    - [PUCCH Format 3 with 4bits – FL note: only one company submitted the results]

    - [PUCCH with 3-HARQ-ACK bits + SR - FL note: only one company submitted the results]

    - PUCCH Format 3 with 11bits

    - PUCCH Format 3 with 22bits

    - [PUCCH with HARQ-ACK for Msg.4 – FL note: only two companies submitted the results]

    - SSB

    - PRACH format 0

    - PRACH format B4

    - [PRACH format C2 – FL note: the number of samples is quite small]

    - PDCCH for Msg.2

    - PDSCH for Msg.2

    - PUSCH for Msg.3

    - PDSCH for Msg.4

    - PDCCH

    - PDSCH for eMBB

  - For each channel/format of each scenario, apply the following approach for categorization to derive a representative value:

    - Option A-1: based on UE speed

      - FL note – it may not be easy because “key assumptions” do not always include this information.

    - Option A-2: based on NLOS/LOS + O2O/O2I (for rural)

      - FL note - If categorization is not introduced, RAN1 needs to chose one combination for analysis (or merge everything)

    - Option A-3: based on delta value (zero vs non-zero)

      - FL note – it may not be easy because “key assumptions” do not always include this information.

    - Option A-4: transmission power

      - For UL: normal UE (23dBm) and HPUE (26dBm)

      - For DL: normal BS and low power BS for urban 4GHz

      - FL note: the number of samples for low power BS and HPUE may not be sufficient

    - Option A-5: no further categorization

  - The means to derive a single representative value:

    - Option B-1: use mean value

      - For this case, it is also necessary to discuss how to handle the result(s) that is far from the average.

    - Option B-2: use median value

  - To derive a single representative value for MCL/MPL/MIL from the performance evaluation:

    - Take the mean value

      - Excluding the highest & the lowest values when the number of samples is more than 2

Companies are encouraged to provide their views on the FL perspective above.

| Company | Comments |
| --- | --- |
| CATT | For FR1, evaluation performance of both 4GHz and 2.6GHz performance evaluation need be used for the analysis of bottleneck identification because both 2.6GHz and 4GHz are the important frequency band of the operator NW deployment and evaluation performance of both 4GHz and 2.6GHz are a good reference to the operator for Network planning and network optimization in the future. For deriving a single representative value for MCL/MPL/MIL from the performance evaluation, we prefer to OptionA-5 because current category is enough. In addition, if we use mean value to drive the representative value from the performance evaluation, we need consider how many samples for performance evaluation. For channels with only one company or two companies’ input, the evaluation result is not enough to be persuasive. So we suggest no any conclusion on those channels with only one company or two companies’ input is made. |
| NTT DOCOMO | We are fine with the FL approach. For the categorization, we prefer Option A-4 since the Tx power difference is large for FR1 4GHz DL (24 dBm/MHz or 33 dBm/MHz), and FR2 DL (23 dBm for Indoor, and 40 dBm for Outdoor), and FR2 UL (23 dBm or 12 dBm). |
| ZTE | For scenario/frame structure pairs, we prefer to not consider ‘Rural with long distance 4GHz TDD’, and it’s better to keep other scenarios there because the antenna configuration for different carrier frequencies is different and would lead to different observations. To relieve the work load, we can consider only O2O case for 700MHz rural and rural with long distance, and O2I case for all other scenarios. For channel/signal, we suggest to at least consider one format for each channel/signal to get a full investigation. For categorization, our thinking is: - Option A-1 is not preferred: Performance would not make big differences for different UE speeds. - Option A-2 is not preferred. For absolute MPL target, we can use O2I, which causing more server coverage issue, for results comparison. For relative target, MIL could be used and there is no much difference between O2O and O2I, one of them could be chosen for results comparison. - Option A-3 is good categorization. The non-zero value reported could be very large for some of the scenarios, and causing a large deviation. - Option A-4 is fine for us, while it seems not so much samples. |
| Panasonic | We are fine with FL approach. For the categorization, if MIL is used as target metric, at least Option A-2 is not necessary. |
| OPPO | For the approach to derive a representative value, we prefer option A-5: no further categorization. Maybe we can reduce the influence of different values of UE speed/ delta value/ transmission power by regarding relative MIL as the target metrics. |
| vivo | We suggest to limit the number of categories, and some of the scenarios can be down prioritized for drawing conclusions if there few samples. Besides, if the bottleneck channels observed from different scenarios are same, e.g. 4GHz and 2.6GHz, limited scenarios can be used to draw the conclusion. Agree with FL that a single representative value is derived by excluding the highest and lowest values. Besides, to make the conclusion more reliable, we suggest to only consider the scenarios/channels with samples more than X, e.g. X=4. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | We agree with the motivation to reduce the load of the group. 4 GHz TDD with rural deployments is an important deployment scenario for India and mostly developing countries with rural regions. We earnestly request that this be considered in any further studies. If down selection is mandatory, we suggest 4 GHz be chosen as any solution improving the coverage here would automatically improve coverage at 2.6 GHz. Also, UL data and control channels are bottleneck for the coverage. Excluding the lowest and highest value may not be the correct way to go forward. As mentioned during online discussion, may be these are indeed the true results. Some discussion may be held as to which values must be eliminated among the companies. |
| CMCC | We share a similar view that the group cannot bear the risk to draw unpersuasive conclusions based on limited contributed samples of specific scenarios. Both 2.6GHz and 4GHz are important frequencies for the deployment and should be analysed for the bottleneck channels. And the observations could provide a valuable reference for operators to evaluate whether and which channel needs enhancements. And observations of the bottleneck channels could not exactly same for those two frequencies. On the other side, we also have an agreement in #102e which is quoted in section 2.2 and pasted below. If we cannot provide observations for specific frequencies, how can we strives for satisfying the appropriate targets? Agreements: - RAN1 strives for satisfying appropriate targets identified by companies particularly operators - The targets may be in the form of one or more of the following: - 1. Scenario dependent targets, e.g., ISD/MPL - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP; - 3. Relative difference between channels, e.g, MIL(/[MCL]) - Further values and details of such targets will be clarified at RAN1#103-e - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets. |
| Nokia/NSB | We have strong concerns about the feasibility of any categorization which is not related to pathloss/penetration assumption (O2I/O2O). Additionally, our view is that priority should be given to statistically relevant scenarios to identify bottlenecks. It should be noted that we are not proposing to ignore channels with only 1 or 2 results, but to consider them as lower-priority items to discuss about after discussion on more statically relevant channels is concluded. Thus, we think that: - ideally no further categorization is necessary; - if a categorization must occur, then we should consider the typical distribution UEs have in urban scenarios and focus on O2I. Coincidentally, this is also the most challenging scenario for coverage; - At least one Rural scenario should be considered to address concerns related to more rural deployments. However, given the significant difference of the two deployments, both 700 MHz and 4GHz Rural could be considered. Categorizing by delta could be a good approach as well. However, we fear this could make the aggregation of the results more difficult. Finally, we would like to add a couple of comments in response to IITH, IITM, CEWIT, Reliance Jio and Tejas Networks: - Deployments at 4 GHz and 2.6 GHz often use different numbers of SSB beams, e.g., 8 and 4. Performance of both broadcast and RRC-idle channels may differ quite a lot in the two cases. - It is not clear to us how RAN1 may decide which is result is the “true” one. As far as reliability goes, all results are equally reliable a priori. In this contest, it seems natural to use a conventional approach to minimize the standard deviation of the considered set such as eliminating the outlier. Of course, alternative approaches could be used, e.g., eliminating all results outside of the support [-σ+μ,μ+σ], however this may result in even smaller samples and likely further hinder the statistical relevance of the results. Having said this, we are open to consider any reasonable alternative which does not rely on discussions to identify the “true” value, which may be long and possibly ineffective. |
| Intel | One editorial comment: in Table 2.3.1-1, it seems MPL and MIL are identical, which may not be accurate. :) We are fine with FL's proposal to define a representative value for each channel/scenario/frame structure/category, including the selected scenarios, frame structure and channel/format given the companies' input and collected results. Regarding the categorization, for Option A-2, our view is that this is tightly coupled with the target performance metric. If MPL based performance metric is used for analysis, it is very clear that we need to distinguish the O2O and O2I scenarios as the penetration loss is substantially different between these two. We do not prefer Option A-1 and A-4. Regarding the single representative value for MCL/MPL/MIL, we are fine to take mean value and the number of samples should be >= 4 in order to have meaningful results. |
| Qualcomm | Support FL proposal to focus on a subset of scenarios. Taking 4GHz and 700 MHz into account should provide sufficient insight. Relative gaps in coverage seen in these two cases should provide sufficient insight for the other bands. We are okay with the list of channels/formats to prioritize. No further categorization is preferred. For consistency, okay to choose a particular scenario to focus on (i.e., NLOS, O2I, etc) as in A-2. In some cases, such as gNB tx power, a separate category may be warranted --- we would like to accommodate this as it reflects a real world network. We are okay to use the method proposed to compute the representative value but have some concerns. One aspect that is unclear is whether to compute this mean in the linear domain or dB domain. One other concern is that as a group it might be preferable for us to be conservative on our coverage estimates, so while outliers on the extreme high end might be okay to drop, its not clear if it is safe for us to drop a result that points to limited coverage. A little more consideration may be warranted here to better understand the outliers. For example, if the outlier points to very low coverage due to high interference assumption based on SLS, then this is valuable insight that may be missing from other company reports that do not uses SLS results. |
| Apple | Any categorization will reduce the input for statistic, which make the statics less trust. Just like the FL summary on the eMBB PUSCH in Urban TDD 4GHz case in Figure 2.3.1-1, there are total input from 13 companies, if we perform categorization, then one part could have 6 results, and remove the best/worst results, we will use the 4 results to make the decision, this input seems not enough. If the majority believe categorization is necessary, from our side, the A-2 and A-3 can be considered. For the simulated scenarios, such as Urban 4GHz TDD, both TDD frame structure DDDSU and DDDSUDDSUU have similar number of results, so both configurations can be used for bottleneck identification, at least the results can be cross checked. |
| Ericsson | As discussed above, regarding the single representative value, in order to identify bottlenecks, it is best for companies to compare within their own simulations since these will be consistent. Otherwise, we will be averaging results with different simulation assumptions and for which there are different numbers of reporting companies, etc. So, we suggest that at least relative MIL be used to compare a target channel to some baseline channel, and then the single representative value can be calculated on these relative values. Regarding categorization, focussing on O2I results should be sufficient to identify bottlenecks, as indoor UEs are the most coverage limited ones according to our study. So we are OK with option A-1. While we think option A-2 would be quite enlightening, we may not have time to do so. If results are refined and therefore there is more time, this may be considered. Taking the mean as proposed is fine for us, except that we may wish to change to when the number of samples is more than 4. |
| Xiaomi | For each channel/format of each scenario, we prefer Option A-2 for categorization to derive a representative value. Because if MPL based performance metric is used for analysis, shadowing and penetration loss are different between O2O/O2I and NLOS/LOS scenarios, which need to be distinguished. |

Summary of the 1st round of email discussion

Down selection of the scenarios

  - Majority of companies support to limit the number of scenarios for further analysis.

  - 1 company proposed not to consider Rural with long distance 4GHz TDD, while one company thinks it important.

    - From FL perspective, the problem is that number of simulation results for this evaluation is not sufficient for Rural with long distance 4GHz TDD, hence the statistical relevance may not be justified.

  - 1 company proposed to apply O2O for rural & rural with L.D. , and O2I for other cases. Another company also points out that O2I can be considered because it is the most challenging scenario.

    - From FL perspective, this proposal makes sense, and number of the simulation results by companies also reveals this tendency.

  - One company mentioned that RAN1 could focus on 4GHz and 700MHz for rural. Another company thinks 4GHz and 700MHz are sufficient, too.

  - 2 companies believe 2.6GHz is an important scenario from real deployment point of view.

  - 1 company mentioned that both TDD frame structure DDDSU and DDDSUDDSUU can be used for bottleneck identification.

Categorization

  - 5 companies prefer A-5 (no further categorization), which is aligned with FL proposal.

  - 3 companies see the necessity to introduce categorization by A-4 (Tx power) while one company doesn’t prefer this categorization.

    - From “FR1” FL perspective, the problem is that number of simulation results for low power Tx may not be sufficient in some cases. As long as FL checked, only two results are submitted to 4GHz urban scenario, which may be treated as low priority anyway.

  - 2 companies do not prefer A-1 (UE speed), while 1 company is OK (assuming we can focus on O2I)

  - 1 company does not prefer A-2 (NLOS/NLOS – O2O/O2I). One company pointed out it this not necessary if MPL is not used for the performance metric. 2 companies pointed out that O2O/O2I differentiation may be needed because of the difference of its characteristics. 1 company can accept it, if majority believe it necessary.

    - From FL point of view, separation by O2O/O2I is feasible from workload point of view. For NLOS/LOS separation, we have very small number of simulation results for rural LOS, and similar tendency for rural L.D. NLOS. Therefore, FL thinks separation by NLOS/LOS is also feasible.

  - 1 company supports A-3 (delta) , while one company thinks it is challenging. 1 company can accept it, if majority believe it necessary.

    - From FL perspective, categorization by delta is quite challenging because most of the companies doesn’t show the delta value in “key assumptions” column. Even when companies use different values for delta, averaging with sufficient number of samples can help to derive good statistical results.

Derivation of representative value

  - 2 companies agreed the necessity of excluding outliers (highest and lowest) while 2 companies has a concern for this approach

    - FL agrees that the outlier is also “true”. However, we can guess that the result may be derived by “minor” assumption/parameters. As discussed below, many companies think the channels/signals with lower number of samples should be treated as “low priority”. FL thinks the same rule should apply to the outlier. Of course, one-by-one discussion would be the ideal approach, but it is clear that we don’t have enough time to do so. From FL perspective, applying an automatic approach such as “excluding outliers” is useful and it can drastically reduce FL’s workload.

  - 1 company proposed the following

      - Excluding the highest & the lowest values when the number of samples is more than 4 2

  - 1 company wonders which approach is used for averaging, log domain or linear domain.

    - FL prefers log domain averaging because the submitted result is express in log domain, and hence it can reduce the workload by FL.

  - 1 company proposes to calculate relative values with MIL by each company’s result first, and then aggregate them to derive a representative value.

    - The benefit claimed by the proponent is that the relative value between two channels is reliable because the same assumptions are used.

    - FL wonders this approach makes any difference if averaging operation among companies is used in the end. In FL’s understanding, the difference is the order of averaging operation and subtraction operation, but it will not create any difference in theory. The difference may arise when e.g. there is a company who submit a result for only one channel. The proposed approach will exclude such a result from making observation. This is an extreme case, but it is true that all the companies are required to submit an evaluation result for the reference channel, which may require additional efforts for companies at this late stage. Additionally, this approach will drastically increase the workload of FLs (for FR1 and FR2) because the current submission template is not optimized for sorting by companies.

Handling of the data with less number of samples

  - 3 companies suggest to make no conclusion by using the simulation results with less number of samples

    - Two company suggest more than 3 samples is necessary. FL found that there are many evaluation results with 3 samples. RAN1 should discuss if “3” is acceptable or not.

  - 1 company suggests the result with less samples (1 or 2) should be handled as low priority, and can be discussed later.

Editorial comment

  - It was pointed out that the values for MIL and MPL in Table 2.3.1-3 are identical. This typo was corrected.

Given the summary above, FL would like to propose the following as a way forward:

FL proposal

Representative values of [MCL, MIL and MPL] are used for bottleneck identification

  - Further down-selection of MCL/MIL/MPL may be performed depending on the decision of target performance metric(s)

  - Representative values are computed for the following scenarios for FR1

    - 1st priority

      - Urban 4GHz TDD

      - Urban 2.6GHz TDD

      - Rural 4GHz TDD NLOS O2I

      - Rural 700MHz FDD NLOS O2I

    - 2nd priority

      - Rural 2.6GHz TDD NLOS O2I

      - [Rural 2GHz FDD NLOS O2I – FL note: can be dropped? Nobody talked about it during the 1st round]

      - Rural 700MHz with long distance FDD LOS O2O

    - Note: the difference between 1st priority and 2nd priority is as follows

      - 1st priority scenarios have sufficient number of samples and strong support by companies, while insufficient number of samples scenarios or less number of supporting companies for 2nd priority scenarios

      - RAN1 discussion will focus on 1st priority scenarios for making observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

        - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

  - Representative values are computed for the following channels or signals for FR1

    - PUSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

    - PUSCH for VoIP (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

    - PUCCH Format 1 with 2bits

    - PUCCH Format 3 with 11bits

    - PUCCH Format 3 with 22bits

    - SSB

    - PRACH format 0

    - PRACH format B4

    - PDCCH of Msg.2

    - PDSCH for Msg.2

    - PUSCH of Msg.3

    - PDSCH of Msg.4

    - PDCCH

    - PDSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - No categorization by other simulation parameters (such as UE speed, antenna gain collection factors, Tx power) will be introduced for FR1.

  - If the number of samples used to compute a representative value is less than X for each scenario, this representative value is not used for bottleneck identification

    - Option 1: X = 3 (i.e. representative values with 1 and 2 samples is regarded as invalid)

    - Option 2: X = 4 (i.e. representative values with 1,2 and 3 samples is regarded as invalid)

  - A representative value is derived by taking the mean value from companies’ evaluation results

      - Excluding the highest & the lowest values when the number of samples is more than [2 or 4]

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification, can be used to make additional observations to be captured in the TR.

  - The following scenarios/channels are the candidate for the discussion for FR1

    - PUSCH for CSI, PUCCH with HARQ-ACK for Msg4, and Low Tx power for 4GHz BS etc.

  - This discussion will be held in RAN1#103-e on a low priority basis.

- Updated on 29th October

RAN1 discussed this issue on 28th October in the GTW session, and the methodology for representative value calculation was agreed (please see section 4.1 for more information). However, RAN1 needs to further agree on the scenarios, channels and categorization to complete a complete set of representative values.

Therefore, FL would like to propose the following.

- FL proposal:

Representative values are computed for the following scenarios for FR1

  - 1st priority

    - Urban 4GHz TDD

    - Urban 2.6GHz TDD

    - Rural 4GHz TDD NLOS O2I

    - Rural 700MHz FDD NLOS O2I

  - 2nd priority

    - Rural 2.6GHz TDD NLOS O2I

    - Rural 2GHz FDD NLOS O2I

    - Rural 700MHz with long distance FDD LOS O2O

  - Note: the difference between 1st priority and 2nd priority is as follows

    - 1st priority scenarios have sufficient number of samples and strong support by companies, while insufficient number of samples scenarios or less number of supporting companies for 2nd priority scenarios

    - RAN1 discussion will focus on 1st priority scenarios for making observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

    - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

Representative values are computed for the following channels or signals for FR1

  - PUSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for VoIP (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - PDCCH of Msg.2

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - PDCCH

  - PDSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for CSI (if necessary number of evaluation results for representative value is submitted)

  - PUCCH with HARQ-ACK for Msg.4 (if necessary number of evaluation results for representative value is submitted)

No categorization by other simulation parameters (such as UE speed, antenna gain collection factors, Tx power) will be introduced for FR1.

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification, can be used to make additional observations to be captured in the TR.

  - The following scenarios/channels are the candidate for the discussion for FR1

  - PUSCH for CSI, PUCCH with HARQ-ACK for Msg4, and Low Tx power for 4GHz BS, and Rural with long distance for 4GHz TDD LOS O2O scenario etc.

  - This discussion will be held in RAN1#103-e on a low priority basis.

Companies are invited to provide your view on the FL proposal above.

| Company | Comments |
| --- | --- |
| Vivo | We are generally fine with FL’s proposal. For number of samples to derive the representative value, X=4 is preferred. For the channels above, there are ‘MSG2 PDCCH’ and ‘PDCCH’. Shall we revise the wording to ‘broadcast PDCCH’ and ‘unicast PDCCH’, which seems more straightforward. |
| China Telecom | We are fine with FL’s proposal. |
| CATT | We agree with FL’s proposals. |
| Intel | We are fine with FL’s proposal. One minor comment: it seems that “PUSCH for CSI, PUCCH with HARQ-ACK for Msg4,” would not be used to derive the representative values. However, the list of channels still includes these two channels, which may seem contradictory. It would be good to clarify. |
| ZTE | As discussed in Section 2.2, if we cannot reach consensus on the target ISD of ‘Rural 700MHz FDD NLOS O2I’, we suggest to take ‘Rural 2GHz FDD NLOS O2I’ as the 1st priority instead. Note that, we have sufficient samples for this scenario. Another way is we don’t make prioritization among these suggested scenarios (including 1st and 2nd priority scenarios in FL proposal), since the statistics of the evaluation results for these scenarios are all available now (Thanks for FL’s great effort on this!). It would not take much time for drawing observations once the target performance is defined. |
| NTT DOCOMO | We are generally fine with FL’s proposal. We prefer to take the same approach for the two BS Tx power for 4GHz as in the following FR2 agreement. Agreements: The amount of available results for UL channels in FR2 should be considered as given by the total number of available results for both 23 dBm and 12 dBm, given that they can be derived one from the other by simple subtraction, and where each company is counted only once. |
| InterDigital | We support the proposal from the feature lead. |
| Qualcomm | Similar comments as vivo. Since the low tx power BS in 4 GHz was required by an operator based on real-world deployment, we think some observations based on this scenario would be valuable to capture. On a different note, there appears to be an attempt to add additional constraints on baseline PUCCH (Format 3) performance characterization. Will be good to get some clarity on this before we proceed too far along. |
| Ericsson | OK with FL proposal. |
| Samsung | FL proposal is fine with additional comments as following: 1) “Rural 2GHz FDD NLOS O2I” (seems to have sufficient number of samples) would be added in the 1st priority group such that various frequency bands can be covered (i.e., 4GHz, 2.6GHz, 700MHz, and 2GHz) 2) As the prioritization for scenario is based on the sufficient number of samples, we suggest to apply the same criteria (i.e., sufficient number of samples) for the prioritization of channels/signals. Current FL proposal looks like this criteria only applicable for ‘PUSCH for CSI’ and ‘PUCCH with HARQ-ACK for Msg4’ with the explicit text in the parenthesis. |
| Huawei, HiSilicon | OK with FL proposal. |
| CMCC | Generally fine with FL’s proposal. As discussed in section 2.2, we propose to keep the ‘Rural 700MHz FDD NLOS O2I’ in the 1st priority and more discussion of ISD are needed. If no consensus for ISD is reached, the ISD should remain 1732m and observations should be drawn based on that. Share a similar view with vivo. Broadcast and unicast PDCCH is more straightforward. The Msg 2 PDCCH could be put into a round bracket behind the broadcast PDCCH or vice versa. Examples are like this, “Broadcast PDCCH (Msg 2 PDCCH)” or “Msg 2 PDCCH (Broadcast PDCCH)”. Either one is fine to us. |
| Nokia/NSB | Agree with both Intel and Samsung. Concerning “Rural 2GHz FDD NLOS O2I”, ZTE’s suggestion could also be considered as a viable option. Switching the focus to the “sufficient number of samples” (assuming this is what FL refers to as “necessary number of evaluation results”) we would agree with vivo, i.e., X=4 seems to be a good option. Finally, we would also agree with NTT Docomo’s suggestion on low transmit power gNB. Indeed, the suggested approach would give as a possibility to extrapolate additional results accurately. |

Summary of the companies’ views:

Priority of the scenarios

  - Rural 2GHz FDD NLOS O2I can be the 1st priority – ZTE, Samsung, Nokia

  - Rural 700MHz FDD NLOS O2I can be kept in the 1st priority - CMCC

  - Rural 700MHz FDD NLOS O2I can be the 2nd priority if the target value cannot be decided – ZTE

  - No prioritization over the scenarios (listed in FL proposal) is necessary – ZTE

  - Rural w/ long distance scenario for 4GHz should be included in the list - IITH

Handling of PUSCH for CSI and PUCCH with HARQ-ACK for Msg4

  - It looks the handling of PUSCH for CSI and PUCCH with HARQ-ACK for Msg4 is contradicting. Clarification needed. - Intel

Categorization

  - For DL Tx power,

    - The same approach as FR2 UE Tx power below can be used – DOCOMO, Nokia

      - The amount of available results for UL channels in FR2 should be considered as given by the total number of available results for both 23 dBm and 12 dBm, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

    - One company see the necessity to draw an observation – Qualcomm

  - For PUCCH formant 3

    - One company see the necessity for categorization or discussion how to handle the difference of simulation results due to the different assumption - Qualcomm

Value of X (where X is necessary number of samples to derive a representative value)

  - X=4 is proposed – vivo

  - X=4 is preferred for each channel/signal – Nokia

  - It should be clarified that the prioritization criteria (based on number of companies) should be applicable to the prioritization of channels/signals. - Samsung

  - FL perspective: Since we have already agreed during the GTW session, it is not necessary to continue the discussion for scenarios. Meanwhile, it would be good if the same principle could apply to channels/signals to determine the 1st/2nd priority for each scenario. FL would like to make an additional proposal for this aspect.

    - Regarding the proposal by Nokia, X=4 for each channel/signal X=3 looks nicer from FL perspective.

Other proposals

  - Renaming of “PDCCH” and “PDCCH of Msg.2” used in the template – vivo, CMCC

    - “PDCCH of Msg 2” should be “broadcast PDCCH” while “PDCCH” should be “unicast PDCCH” to avoid the confusion

    - FL perspective: Agree

    - FL perspective: Agree in principle. This will be clarified in the next update.

Given the companies’ views and FL perspective above, The FL proposals are updated as follows:

Updated FL Proposal:

Rephrase the following terminologies, which is used in e.g. link budget template:

  - “PDCCH of Msg.2” is referred to as “broadcast PDCCH (PDCCH of Msg.2)”

  - “PDCCH” is referred to as “unicast PDCCH”

Representative values are computed for the following channels or signals for FR1, and they are used to draw observation and/or to identify bottlenecks if the number of samples for each scenario is more than Y (Y=[2 or 3] – FL recommendation is 2)

  - PUSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for VoIP (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for CSI (if necessary number of evaluation results for representative value is submitted)

    - (FL note: the parenthesis is not necessary anymore because the main bullet has covered the intention)

  - PUCCH with HARQ-ACK for Msg.4 (if necessary number of evaluation results for representative value is submitted)

    - (FL note: the parenthesis is not necessary anymore because the main bullet has covered the intention)

Representative values are computed for t The following scenarios are used for drawing observations and/or bottleneck identification for FR1

  - 1st priority (i.e. companies showed their interests and number of available channels/signals is greater than Z, where Z=11)

    - Urban 4GHz TDD (16 available channels/signals)

    - Urban 2.6GHz TDD (12 available channels/signals)

    - Rural 4GHz TDD NLOS O2I (16 available channels/signals for Y=2 and 14 for Y=3)

    - Rural 700MHz FDD NLOS O2I (13 available channels/signals)

  - 2nd priority (i.e. companies showed their interests but number of available channels/signals is no greater than Z)

    - Rural 2GHz FDD NLOS O2I (10 available channels/signals for Y=2 and 9 for Y=3 )

    - Rural 2.6GHz TDD NLOS O2I (10 available channels/signals for Y=2 and 9 for Y=3 )

    - Rural 700MHz with long distance FDD LOS O2O (11 available channels/signals for Y=2 and 8 for Y=3)

      - FL note: It is fine to put 1st priority for 2GHz and 2.6GHz by choosing appropriate value for Y and Z (Y=2 and Z=9). If we do so, 700MHz w/ L.D. will also be the 1st priority. However, the necessity for 700MHz is not so clear given less interest by companies. The best solution would be to remove all parentheses and just put 1st priority for the selected scenarios (Rural 2.6GHz and 2GHz).

    - [Rural 4GHz with long distance FDD LOS O2O (6 available channels/signals for Y=2 and 2 for Y=3)

      - FL note: 6 channels/signals may not enough to draw observation. More discussion necessary]

  - Note: the difference between 1st priority and 2nd priority is as follows

    - 1st priority scenarios have sufficient number of samples and strong support by companies, while insufficient number of samples scenarios or less number of supporting companies for 2nd priority scenarios

    - RAN1 discussion will focus on 1st priority scenarios for drawing observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

    - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

No categorization by other simulation parameters (such as UE speed, antenna gain collection factors, Tx power) will be introduced for FR1

  - The amount of available results for DL channels in FR1 4GHz scenario should be considered as given by the total number of available results for both 33 dBm/MHz and 24 dBm/MHz, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

  - In order to address the misalignment issue of the companies’ evaluation results due to no categorization and/or different simulation assumptions,

    - Number of samples and standard deviation is shown together with a representative value, and

      - [FL note, this bullet is to address a concern by Ericsson (proposed in section 2.2)]

    - A description on the potential fluctuation due to no categorization and/or different simulation assumptions can be added in the obersation

    - [FL note, this bullet is to address a concern Qualcomm (for PUCCH F3) ]

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification due to the lack of number of samples etc., can be used to make additional observations to be captured in the TR.

  - The following scenarios/channels are the candidate for the discussion for FR1

  - PUSCH for CSI, PUCCH with HARQ-ACK for Msg4, and Low Tx power for 4GHz BS, and Rural with long distance for 4GHz TDD LOS O2O scenario etc.

  - This discussion will be held in RAN1#103-e on a low priority basis.

Please note that the figures in the proposal is not subject to the agreement. They are a part of FL note and to be removed when it is captured in the chairman’s note.

Companies are invited to provide their views on the FL proposals:

| Company | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Rural with long distance 4 GHz is a valid deployment scenario to be considered. We have observed that UL data and control channels are a bottleneck in this setting. |

Summary of the discussion

Given the agreements made in the GTW session on 3rd November, there are no remaining issues to be discussed in this section. This section is closed.

[Closed] Approval of representative values

Since RAN1 has not made the final decision on the target value yet, it is not possible to make a solid observation for the evaluation results. Meanwhile, it would be good if company could take a look at it for the quick agreements.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/4th_round/FR1_eval_result_v062_r1.pdf

Since all the discussions on representative values and target metrics are finalized, their values are now read for capturing in the TR. FL would like to propose the following:

FL proposal:

Capture the following tables in Annex of the TR

  - Note: Even when the channel has less than 4 samples, it can be included in the table.

  - Note: The excelsheet for this table is found below:

  - https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

  - Urban 4GHz scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Urban 2.6GHz scenario

  - Rural 4GHz TDD NLOS O2I scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Rural 2.6GHz TDD NLOS O2I scenario

  - Rural 2GHz Rural 2GHz FDD NLOS O2I scenario

  - Rural 700MHz FDD NLOS O2I scenario

  - Rural 4GHz with long distance TDD NLOS O2O scenario

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| Qualcomm | Can we capture the results for 4 GHz with 24 dBm/MHz in a separate table, if it is not too much work for the FL? I feel the insight for this scenario is otherwise hard to see visually. Also, bottlenecks for this case will be identified and reported separately, correct? FL response: This is a fair comment, and reflected in the tables above. |
| CATT | We are fine with FL proposal |
| Ericsson | Looks good. Thanks. |
| CMCC | General fine with current version. |

Final status at the end of the meeting:

The FL proposal above looks stable.

[Closed] Observation for Urban 4GHz TDD scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Urban 4GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

      - PUSCH for eMBB with frame structure DDDSUDDSUU:  4.68 dB

  - In order to achieve 350m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 5.86 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  4.83 dB

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 8.12 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  7.09 dB

    - If low transmit power BS (i.e 24dBm/MHz) is assumed,

      - Broadcast PDCCH (PDCCH of Msg.2): 2.17dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 11.91 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  10.88 dB

    - PUSCH for VoIP with frame structure DDDSU: 0.03 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

    - PUSCH for VoIP with frame structure DDDSUDDSUU:  0.86 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22 bit payload: 2.62 dB

    - PUSCH of Msg.3: 1.11 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - If low transmit power BS (i.e 24dBm/MHz) is assumed,

      - SSB: 2.85dB

      - Broadcast PDCCH (PDCCH of Msg.2): 5.95dB

      - PDSCH for eMBB with frame structure DDDSUDDSUU: 1.09 dB

  - Achievement of 700m ISD is not easy because it requires enhancements for almost of channels.

    - Especially, PUSCH for eMBB with frame structure DDDSU requires huge amount of enhancements of 17.62dB

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | For urban 4G TDD scenario, we think SSB, Broadcast PDCCH and PDSCH for eMBB aren’t bottleneck channel under low transmit power BS (i.e 24dBm/MHz) based on our performance evaluation. |
| ZTE | Based on the results in the Figure, it seems 0.48dB enhancement to PRACH is needed in case of 500m ISD. |

Summary of the discussion

It was pointed out that the observation by representative values does not correspond to the result by own company.

  - FL perspective: It is obvious that each company’s result is not always same as that by representative values. Therefore, FL proposes not to address this concern.

It was pointed out 0.48dB enhancement to PRACH is missing for 500m ISD

  - FL response: it should be a typo. Need correction

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Urban 4GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 10.26 dB can be achieved at maximum. [FL note: if numbers is not captured, this sub-bullet itself isn’t necessary]

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 8.12 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  7.09 dB

    - If low transmit power BS (i.e 24dBm/MHz) is assumed,

      - Broadcast PDCCH (PDCCH of Msg.2): 2.17dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 11.91 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  10.88 dB

    - PUSCH for VoIP with frame structure DDDSU: 0.03 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

    - PUSCH for VoIP with frame structure DDDSUDDSUU:  0.86 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22 bit payload: 2.62 dB

    - PRACH Format B4: 0.48dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUSCH of Msg.3: 1.11 dB

  - If low transmit power BS (i.e 24dBm/MHz) is assumed, the following DL channel(s) needs to be enhanced, additionally.

    - For 400m ISD

      - Broadcast PDCCH (PDCCH of Msg.2)

    - For 500m ISD

      - SSB: 2.85dB

      - Broadcast PDCCH (PDCCH of Msg.2): 5.95dB

      - PDSCH for eMBB with frame structure DDDSUDDSUU: 1.09 dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| Qualcomm | Can we drop the gap numbers at the end of each bullet? Given the standard deviation, we don’t want to over-emphasize these numbers. FL reply: fair comment. The numbers are removed. Also, we prefer a separate set of observations for the low power case. Don’t wish to nest that case under a single observation. Separating them at the top level might also suffice. FL reply: Can do somewhat. However, it is not good idea to completely separate since the UL parts are completely identical. Thus the TR would be redundant. Maybe we can add some more text to improve the readability. |
| Ericsson | Support the revised FL proposal. Agree with Qualcomm’s comment on standard deviation. Also, the ISDs are pretty constrained, and so this contributes to overestimation of needed enhancement if we only look at MPL. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] Observation for Urban 2.6GHz TDD scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Urban 2.6GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, no other channel needs to be enhanced to maximize its benefit:

  - In order to achieve 350m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 2.86 dB

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 5.13 dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 8.92 dB

  - In order to achieve 700m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 14.63 dB

    - PUSCH for VoIP with frame structure DDDDDDDSUU: 4.59 dB

    - PUCCH format 1: 0.07 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 11bit payload: 2.16 dB

    - PUCCH format 3 with 22bit payload: 3.78 dB

    - PRACH format 0: 1.25 dB

    - PRACH format B4: 4.98 dB

    - PRACH format C2: 2.00 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 0.66 dB

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |

Summary of the discussion

No companies raised a concern.

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Urban 2.6GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 10.00 dB can be achieved at maximum.

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 5.13 dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 8.92 dB

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| Qualcomm | Same request as earlier, can we drop the gap numbers? |
| Ericsson | Support the revised FL proposal; same comments as the section above. |
| CMCC | Fine with currant version. |

[Closed] Observation for Rural 4GHz TDD NLOS O2I scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Rural 4GHz TDD NLOS O2I scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

      - PUSCH for eMBB with frame structure DDDSUDDSUU:  4.03 dB

      - PUSCH for VoIP with frame structure DDDSU: 0.21dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

      - PUSCH for VoIP with frame structure DDDSUDDSUU:  0.38 dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

      - PUCCH format 3 with 22bit payload: 0.54dB

      - PRACH format B4: 1.77dB

      - PUSCH of Msg.3: 0.14 dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

      - If low transmit power BS (i.e. 24dBm/MHz) is assumed,

        - Broadcast PDCCH (PDCCH of Msg.2): 0.59dB

          - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

        - PDSCH for Msg.2: 1.05dB

          - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

        - PDSCH of Msg.4: 0.58dB

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced for BS with 33dBm/MHz transmit power:

    - PUSCH for eMBB with frame structure DDDSU: 7.05 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 5.39 dB

    - PUSCH for VoIP with frame structure DDDSU: 1.89 dB

    - PUSCH for VoIP with frame structure DDDSUDDSUU:	 1.80 dB

    - PUCCH Format 1:	 1.23 dB

    - PUCCH Format 3 with 11bit payload: 2.53 dB

    - PUCCH Format 3 with 22bit payload: 5.49 dB

    - PRACH Format 0: 2.24 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH Format B4: 5.72 dB

    - Broadcast PDCCH(PDCCH of Msg.2) : 1.11 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 1.90 dB

    - PUCCH w/ HARQ-ACK for Msg.4: 1.47 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - Achievement of 3000m ISD is not easy because it requires enhancements for all the channels.

    - Especially, PUSCH for eMBB with frame structure DDDSU requires huge amount of enhancements of 16.27 dB

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | For Rural 4G TDD scenario, we think PRACH format 0, PRACH format B4, and Broadcast PDCCH aren’t bottleneck channel based on our performance evaluation. |
| Apple | For this scenario, as commented by Ericsson and Intel during GTW, too many channels could not reach the target coverage, it means we select the non-realistic ISD. Maybe we consider more realistic ISD to identify the bottleneck channels for this scenario. |
| Intel | As commented during GTW online session, we think the target ISD would be quite challenging for this particular scenario, i.e., all uplink channels need to be enhanced. It would be good to consider some realistic target as Apple mentioned. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | 1732 m is a target we must achieve using the coverage enhancement study. Support the observations with 1732 m as a target. |

Summary of the discussion

It was pointed out that the observation by representative values does not correspond to the result by own company.

  - FL perspective: It is obvious that each company’s result is not always same as that by representative values. Therefore, FL proposes not to address this concern.

Two companies showed concern on the reality of ISD=1732m

  - FL perspective: Since this section focuses on observation only, FL thinks this comment is not valid in this section. The issues on bottleneck identification can be discussed under section 2.5.2.

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Rural 4GHz TDD NLOS O2I scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 4.23 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced for BS with 33dBm/MHz transmit power:

    - PUSCH for eMBB with frame structure DDDSU: 7.05 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 5.39 dB

    - PUSCH for VoIP with frame structure DDDSU: 1.89 dB

    - PUSCH for VoIP with frame structure DDDSUDDSUU:	 1.80 dB

    - PUCCH Format 1:	 1.23 dB

    - PUCCH Format 3 with 11bit payload: 2.53 dB

    - PUCCH Format 3 with 22bit payload: 5.49 dB

    - PRACH Format 0: 2.24 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH Format B4: 5.72 dB

    - Broadcast PDCCH(PDCCH of Msg.2) : 1.11 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 1.90 dB

    - PUCCH w/ HARQ-ACK for Msg.4: 1.47 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - Achievement of 3000m ISD is not easy because it requires enhancements for all the channels

    - Especially, PUSCH for eMBB with frame structure DDDSU requires huge amount of enhancements of 16.27 dB

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
|  | - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 4.23 dB can be achieved at maximum. |
| CATT | We are fine with FL proposal. |
| Ericsson | Support FL proposal. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] Observation for Rural 2.6GHz TDD NLOS O2I scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Rural 2.6GHz TDD NLOS O2I scenario

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

      - PUSCH for VoIP with frame structure DDDDDDDSUU: 0.81dB

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: -3.86 dB

    - PUCCH format 3 with 11bit payload: -0.50 dB

    - PUCCH format 3 with 22bit payload: -1.60 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |

Summary of the discussion

No companies raised a concern.

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Rural 2.6GHz TDD NLOS O2I scenario

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 5.44 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 3.86 dB

    - PUCCH format 3 with 11bit payload: 0.50 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22bit payload: 1.60 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Ericsson | Support FL proposal. |
| Qualcomm | - Request to remove the sub-bullet under PUCCH Format 3 11bits as it seems to take a rather narrow view of coverage. Our take on coverage is a little different. We think its valuable to close the control channel imbalance irrespective of PUSCH data rate targets. - FL response OK from FL point of view. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] Observation for Rural 2GHz FDD NLOS O2I scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Rural 2GHz FDD NLOS O2I scenario

  - PRACH format B4 is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

      - PUSCH for eMBB: 3.15 dB

      - PUSCH for VoIP: 1.01 dB

      - PUSCH with SIP invite:	5.43 dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

      - PUCCH Format 3 with 22bit payload: 1.39 dB

      - PUSCH of Msg.3: 1.21 dB

      - PUCCH w/ HARQ-ACK for Msg.4: 8.40 dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH with SIP invite: 2.09dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH format B4: 3.10dB

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | For Rural 2 GHz FDD scenario, PRACH format 0 and PRACH format B4 aren’t bottleneck channel based on our performance evaluation. |

Summary of the discussion

It was pointed out that the observation by representative values does not correspond to the result by own company.

  - FL perspective: It is obvious that each company’s result is not always same as that by representative values. Therefore, FL proposes not to address this concern.

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Rural 2GHz FDD NLOS O2I scenario

  - PRACH format B4 is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - Since the number of samples to derive the representative value is 4, further analysis is advisable for more accurate MIL estimation on this channel as necessity. It has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 2.85 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH with SIP invite: 2.09dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH format B4: 3.10dB

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Ericsson | PRACH B4 being the worst channel, even worse that 1 Mbps PUSCH eMBB, is hard to understand. Stranger still, PRACH is not the bottleneck at 2.6 GHz (its MIL is 10 dB better than the worst there). I see that there are 4 results for 2 GHz, while there are 6 for 2.6. Should we say something like this for PRACH in this scenario: “while PRACH is observed to be the worst channel in this scenario, there are a limited number results, and PRACH is substantially better than the worst channel at 2.6 GHz, where more PRACH results are available. Consequently, it is recommended to consider the next worst channel, PUSCH eMBB as the bottleneck for this scenario.” FL Comment: FL understands the result is controversial. The proposal by Ericsson looks a bit strong and will cause more discussion, which is not good considering the deadline. FL would like to propose more “general” description. FL proposal is as follows: “Since the number of samples to derive the representative value is 4, further analysis is advisable for more accurate MIL estimation on this channel as necessity. It has not been pursued in this study item.” |
| Qualcomm | Support FL revision. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] Observation for Rural 700MHz FDD NLOS O2I scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Rural 700MHz FDD NLOS O2I scenario

  - For the service based requirement for VoIP, which is defined by 139.2dB MCL, the following channels can be bottlenecks for the given scenario.

    - PUSCH for VoIP: 5.01 dB

    - PUSCH with SIP invite: 4.20 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 11bit payload: 1.45 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 22bit payload: 2.95 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH Format 1:	1.76 dB

    - PUCCH Format 3 with 11bit payload:  2.49 dB

    - PRACH Format B4: 6.51 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3 	4.49 dB

  - PUCCH format 3 with 22 bit payload is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

      - PUSCH for eMBB: 5.78 dB

      - PUSCH for VoIP: 3.60 dB

      - PUSCH with SIP invite:	2.24 dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

      - PUSCH for CSI with 22bit payload: 0.94dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

      - PUCCH Format 3 with 11bit payload: 0.79 dB

      - PRACH Format B4: 8.35 dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

      - PUSCH of Msg.3: 3.04 dB

      - PUCCH w/ HARQ-ACK for Msg.4: 8.13 dB

        - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - No enhancement is necessary to achieve 1732m ISD

  - In order to achieve 3000m ISD, the following channel(s) needs to be enhanced:

    - PUCCH Format 3 with 22 bit payload: 2.28dB

    - PRACH format B4: 4.48dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH w/ HARQ-ACK for Msg.4: 4.43dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - In order to achieve 4000m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB: 3.05 dB

    - PUSCH for VoIP: 1.22 dB

    - PUCCH Format 1: 1.09 dB

    - PUCCH Format 3 with 11bit payload: 0.25 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH Format 3 with 22bit payload: 7.11 dB

    - PRACH Format B4: 9.31 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 0.60 dB

    - PUCCH w/ HARQ-ACK for Msg.4: 9.26 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Companies are invited to provide their view on the potential FL proposal.

| Company | Comments |
| --- | --- |
| CATT | For Rural 700MHz FDD scenario, PRACH format B4 isn’t bottleneck based on our performance evaluation results. |

Summary of the discussion

It was pointed out that the observation by representative values does not correspond to the result by own company.

  - FL perspective: It is obvious that each company’s result is not always same as that by representative values. Therefore, FL proposes not to address this concern.

Others

  - RAN1 achieved the offline consensus on the definition of relative reference. The observation based on this metric should also be added.

  - Proposal on the representative value table is moved to section 2.3.2.

Given the companies view and FL perspective, the proposal is updated as follows:

FL proposal:

Capture the following observation for Rural 700MHz FDD NLOS O2I scenario

  - For the service based requirement for VoIP that is defined by 139.2dB MCL, the following channels can be bottlenecks for the given scenario.

    - PUSCH for VoIP: 5.01 dB

    - PUSCH with SIP invite: 4.20 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 11bit payload: 1.45 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 22bit payload: 2.95 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH Format 1:	1.68 dB

    - PUCCH Format 3 with 11bit payload:  3.35 dB

    - PRACH Format B4: 6.51 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3 	4.49 dB

  - PUCCH format 3 with 22 bit payload is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - As the table of representative values (in annex) shows, the gap between the worst and the 2nd worst channel (PUSCH for eMBB) is relatively small, i.e. in the range of standard deviation. Thus more analysis is necessary for the accurate bottleneck channel identification. However, it has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 0.22 dB can be achieved at maximum.

  - In order to achieve 3000m ISD, the following channel(s) needs to be enhanced:

    - PUCCH Format 3 with 22 bit payload: 2.28dB

    - PRACH format B4: 4.48dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH w/ HARQ-ACK for Msg.4: 4.43dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - In order to achieve 4000m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB: 3.05 dB

    - PUSCH for VoIP: 1.22 dB

    - PUCCH Format 1: 1.01 dB

    - PUCCH Format 3 with 11bit payload: 2.58 dB

    - PUCCH Format 3 with 22bit payload: 7.11 dB

    - PRACH Format B4: 9.31 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 0.60 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH w/ HARQ-ACK for Msg.4: 9.26 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Companies are invited to provide their view on the FL proposal above.

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |
| Ericsson | It seems hard to say if PUSCH eMBB is the bottleneck for MIL or if PF3 is the bottleneck given the 0.22 dB difference. This is far less than the 1.92 and 2.45 dB standard deviations of the PF3 and PUSCH numbers. Can we say that it is not clear if PF3 or PUSCH eMBB is the bottleneck for this scenario? FL response Maybe we should be a bit faithful on the representative value, since we have already decided to use it officially (as RAN1) . Maybe we can solve this by adding a sub-bullet saying: As the table of representative values (in annex) shows, the gap between worst and 2nd worst channel (PUSCH for eMBB) is relatively small, i.e. in the range of standard deviation. Thus more analysis is necessary for the accurate bottleneck channel identification. However, it has not been pursued in this study item. |
| Qualcomm | We can go with FL revision. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] 2nd priority: Rural 700MHz with long distance FDD LOS O2O scenario

As of 12th November, no company request to draw observation for this scenario. Therefore, the discussion is temporary closed.

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

Potential FL proposal:

Capture the following table in the TR(Note 1) – when the final version is ready

  - Note1: In the chairman’s note, the table is captured as an image file. However, the table will be captured as a text based table, which is available from the following link (xxx: to be incorporated later).

Capture the following observation for Rural 700MHz with long distance FDD LOS O2O scenario

  - PUCCH format 3 with 22bit payload is the worst channel in terms of MIL, and the coverage bottleneck for the given scenario.

    - If enhancement of 6dB for this channel is achieved by Rel-17 new functionalities, the following channel(s) needs to be enhanced to maximize its benefit:

        - PUSCH for eMBB: 4.39 dB

        - PUSCH for VoIP: 1.22 dB

        - PUCCH Format 1: 0.01 dB

          - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

        - PUCCH Format 3 11bit: 3.13 dB

        - PUCCH Format 3 22bit : 6.00 dB

        - PRACH Format 0: 0.76 dB

          - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

        - PRACH Format B4: 6.48 dB

          - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

        - PUSCH of Msg.3: 3.47 dB

  - No enhancement is necessary to achieve 12km ISD

  - In order to achieve 20km ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB UUUUU: 2.44 dB

    - PUCCH Format 1: 0.53 dB

    - PUCCH Format 3 with 11bit payload: 3.38 dB

    - PUCCH Format 3 with 22bit payload: 5.91 dB

    - PRACH Format 0:	 1.70 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH Format B4: 6.74 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 1.14 dB

  - Achievement of 30km ISD is not easy because it requires enhancements for all the channels, except SSB.

    - Especially, PUCCH format 3 with 22bit payload requires huge amount of enhancements of 12.95 dB

The discussion on this scenario is put on hold because we have no agreement on the target ISD.

[Closed] 2nd priority: Rural 4GHz with long distance TDD LOS O2O scenario

To be added after the discussion on section 2.2.X settles down. However to accelerate the discussion in RAN1, FL would like to show the potential FL proposal, which is applicable when all the FL proposals in section 2.2.X are approved.

Please note the FL does not intend to have a GTW discussion on this section because it is expected that the observation will not be the blocking factor.

FL proposal:

Capture the following observation for Rural 4GHz with long distance TDD LOS O2O scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL. However, due to the lack of the number of samples for each channel, the statistical analysis by relative difference between channels could not be performed.

  - In order to achieve 12km ISD, at least the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 3.15 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 2.42 dB

    - This does not mean that other channels do not require any enhancements. RAN1 did not perform any analysis for this point because of the lack of the number of samples for each channel for this scenario.

Companies are invited to provide their view on the potential FL proposal. Note that the discussion on this scenario is valid when RAN1 agrees to include this scenario in the TR (see section 2.2.6)

| Company | Comments |
| --- | --- |
| CATT | We are fine with FL proposal. |
| ZTE | Fine with the proposal. |
| Ericsson | Support. |

Status at the end of the e-meeting

The FL proposal above looks stable.

[Closed] 2nd priority, Observation not using representative values

Companies are requested to provide observations not covered by the discussion using representative values in section 2.3.3 – 2.3.10. The example is as below:

- Observation from SLS evaluation results

- Use of qam64-LowSE MCS (R1-2008701)

| Company | Proposals on additional observations |
| --- | --- |
| ZTE | Based on our SLS results in R1-2007741, we propose to discuss the following observation. One source (ZTE, R1-2007741) evaluated the target performance, i.e. the 5th percentile geometry SINR value, based on system-level simulation. Compared to the baseline performance, i.e., required SNR based on link level simulation, the following is observed for FR1. In Urban 4 GHz O2I scenario, PUSCH eMBB, PUSCH VoIP, Msg3, PRACH B4, PUCCH 2 bits, PUCCH 11 bits, PUCCH 22 bits, requires coverage compensation, and the compensation gap is 14.65 dB, 3.86 dB, 2.1 dB, 5.2 dB, 4.26 dB, 4.29 dB and 4.64 dB respectively. In Rural 4 GHz O2I scenario, PUSCH eMBB, Msg3, PRACH B4, PUCCH 2 bits, PUCCH 11 bits, PUCCH 22 bits, requires coverage compensation, and the compensation gap is 5.89 dB, 3.08 dB, 4.14 dB, 4.19 dB, 4.31 dB and 4.56 dB respectively. In Rural 2.6 GHz O2I scenario, PUSCH eMBB, Msg3, PRACH B4, PUCCH 2 bits, PUCCH 11 bits, PUCCH 22 bits, requires coverage compensation, and the compensation gap is 5.11 dB, 3.35 dB, 4.11 dB, 4.3 dB, 4.28 dB and 4.56 dB respectively. In Rural 700MHz O2O scenario, no channel requires coverage compensation. In Rural with long distance 700MHz O2O scenario, no channel requires coverage compensation. |

At the next round of email discussion, FL will ask other companies whether or not such observations should be captured or not. Please note that the proposals will be treated on best effort basis.

Update on 12th November:

The discussion is still open but no input from companies was provided.

Update on 13th November:

Given the proposal by ZTE, FL would like to propose the following observation:

FL proposal

Capture the following observation in the TR

  - One source (ZTE, R1-2007741) evaluated the target performance, i.e. the 5th percentile geometry SINR value, based on system-level simulation. Compared to the baseline performance, i.e., required SNR based on link level simulation, the following is observed for FR1.

    - In Urban 4 GHz O2I scenario with ISD 500m, PUSCH eMBB, PUSCH VoIP, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 14.65 dB, 3.86 dB, 2.1 dB, 5.2 dB, 4.26 dB, 4.29 dB and 4.64 dB respectively.

    - In Rural 4 GHz O2I scenario with ISD 1732m, PUSCH eMBB, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.89 dB, 3.08 dB, 4.14 dB, 4.19 dB, 4.31 dB and 4.56 dB respectively.

    - In Rural 2.6 GHz O2I scenario with ISD 1732m, PUSCH eMBB,  Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits and  PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.11 dB, 3.35 dB, 4.11 dB, 4.3 dB, 4.28 dB and 4.56 dB respectively.

    - In Rural 700MHz O2O scenario with ISD 1732m, no channel requires coverage compensation.

    - In Rural with long distance 700MHz O2O scenario with ISD 12km, no channel requires coverage compensation.

Status at the end of the e-meeting

NO comment was provided by companies. The FL proposal above looks stable.

Collection of simulation results

The collection of the simulation results is on-going under another email discussion, i.e. [103-e-CovEnh-EvaluationResults]. Companies are encouraged to monitor this email thread, and input the updated simulation results when available.

Note that the 1st deadline of the submission aiming at observation/bottleneck identification is 23:59 UTC of 11/06.

The second deadline of the submission aiming at capturing in the Annex of TR has not been clarified yet as of 11/06 UTC.

[Closed] Approval of LLS based simulation results submitted by companies

It has not been clarified yet as of 11/06 when is the deadline of the submission of simulation results, which does not aim at the observation/bottleneck identification but capturing in Annex of the TR. Since it is expected that Chairman will announce the deadline of the final submission, the final proposal, i.e. which version of the template to be captured in the TR, will be made after that.

Update on 12th November

Rapporteur will capture the final version of the template in the TR. So no discussion is necessary under this A.I. This section is closed.

[Closed] Approval of SLS based simulation results submitted by companies

FL expects that the same approach LLS can be taken for SLS.

Update on 12th November

Rapporteur will capture the final version of the template in the TR. So no discussion is necessary under this A.I. This section is closed.

Identification of coverage bottleneck(s)

[Closed] Summary of the contributions submitted to RAN1#103-e

Most of the companies think PUSCH and PUCCH need enhancements while the amount (how many dBs) is not clear because the target metric and value are not decided yet. In addition, companies has different views on the details:

- which instance of PUSCH (eMBB, VoIP, CSI etc.) can be the bottleneck

- which format and bit size of PUCCH (Format 1,3 / bit size of 2, 11, 22/ UE-specific, Cell-specific) need to be enhanced

Furthermore, some companies think that PRACH, PDSCH, PDCCH should also be enhanced. It depends on the target value and the scenarios. This requires more discussion in RAN1. The detailed views from companies are summarized as below:

| Channels for enhancements | # of supporting companies | Details |
| --- | --- | --- |
| PUSCH | 17 | Huawei/HiSilicon vivo ZTE(including Msg.3) CATT(including Msg.3) Intel China Telecom (eMBB and VoIP) CMCC(eMBB , VoIP, Msg.3) Samsung OPPO Ericsson Panasonic(including Msg.3) Sharp(eMBB) MediaTek NTT DOCOMO Qualcomm(to ensure minimum service requirements) Apple InterDigital IITH, IITM, CEWIT, Reliance Jio and Tejas Networks |
| PUCCH | 15 | Huawei/HiSilicon (Large payload size) vivo ZTE(including carrying HARQ) CATT Intel(Format 3) China Telecom (Format 1 and 3) CMCC (Format 3 22bit and 11bit) Samsung OPPO Ericsson Panasonic(Format 3) Sharp (Format 3 22bit and Cell-specific) NTT DOCOMO Qualcomm Apple (format 3) IITH, IITM, CEWIT, Reliance Jio and Tejas Networks |
| PRACH | 3 | vivo ZTE Intel |
| PDCCH | 3 | ZTE (broadcast) NTT DOCOMO Qualcomm (broadcast) |
| PDSCH | 2 | Panasonic NTT DOCOMO |

At this stage, it is not easy to make a conclusion without the outcome of section 2.2 and 2.3. The discussion will be initiated after 2.2 and 2.3 is concluded.

Companies are encouraged to check if the table above correctly captures the companies view.

Update on 10th November:

FL would like to ask companies to take the table into consideration for further study. The discussion in this section is closed.

Bottleneck identification by using representative values

The representative values for absolute metrics is shown below:

The graph in red shows the channel, which does not satisfy the target MPL derived from the ISD value. The results are summarized in the table below.

| Scenario | Metric | Bottleneck Channels |
| --- | --- | --- |
| Urban 4GHz | ISD 400m | PUSCH for eMBB DDDSU (Worst) PUSCH for eMBB DDDSUDDSUU Broadcast PDCCH(PDCCH of Msg.2) (24dBm/MHz) |
| Urban 2.6GHz | ISD 400m | PUSCH for eMBB DDDDDDSUU |
| Rural 4GHz NLOS O2I | ISD 1732m, 33dBm/MHz BS Tx | PUSCH for eMBB DDDSU PUSCH for eMBB DDDSUDDSUU PUSCH for VoIP DDDSU PUSCH for VoIP DDDSUDDSUU PUCCH format 1 PUCCH format 3 11bit PUCCH formant 3 22bit PRACH format B4 PUSCH of Msg.3 |
| Rural 2.6GHz NLOS O2I | ISD 1732m | PUSCH for eMBB DDDDDDSUU PUCCH format 3 11bit |
| Rural 2GHz NLOS O2I | ISD 1732m | PRACH format B4 |
| Rural 700MHz NLOS O2I | Service based Metric | PUSCH for VoIP PUCCH format 1 PUCCH format 3 11bit PUSCH of Msg.3 |
| Rural 700MHz NLOS O2I | ISD 4000m | PUSCH for eMBB PUSCH for VoIP PUCCH format 1 PUCCH format 3 11bit PUCCH format 3 22bit PUSCH of Msg.3 |
| Rural 700MHz L.D. LOS O2O | N/A | N/A |
| Rural 4GHz MHz L.D. LOS O2O | N/A | PUSCH |

On top of this table, the additional analyses can be made using MIL, which is expected to have less divergent results among companies.

- The result above showed that Broadcast PDCCH wouldn’t be a serious bottleneck compared with PUSCH for eMBB.

- The results showed that PUSCH for eMBB could be the bottleneck.

- The results showed the enhancement for PUCCH format 1 and 3 with 11bit may not be needed because PUSCH for eMBB need to be enhanced around 10dB, which may not be feasible from technical point of view.

- The results showed the enhancement for PUCCH format 3 with 11bit may not be needed because PUSCH for eMBB need to be enhanced around 7dB, which may not be feasible from technical point of view.

- The results showed the enhancement for PRACH format B4 is helpful to extend the coverage.

- The result showed that enhancement for PUSCH for eMBB, PUSCH for VoIP, PUCCH format 3 with 22bit, PUSCH of Msg.3 is helpful to extend the coverage. The necessity of PUCCH format 1 and Format 3 with 11bit needs more discussion because it requires more than 6dB enhancements for PUCCH.

Please note that relative difference in the right column may not be captured in the TR depending on the discussion in section 2.2.4.

Given the analyses above, FL would like to propose the following for the identification of bottleneck channels.

- FL perspective:

The following channels are identified as the potential bottleneck channels derived from absolute metrics (i.e. service dependent metric and scenario dependent metrics) and MIL.

  - 1st priority – useful for many scenarios

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

    - PUCCH format 3 with 22bit

  - 2nd priority – useful some scenarios

    - PRACH format B4

      - Useful for Rural 2GHz

    - PUSCH of Msg.3

      - Useful for Rural 700MHz and Rural 4GHz

  - 3rd priority – useful when large (e.g. more than 6dB) enhancement is available for the worst bottleneck channel to enjoy the benefit

    - PUCCH format 1

    - PUCCH format 3 with 11bit

FFS: bottleneck identification derived from relative metric

Companies are encouraged to provide their view on the analysis and the FL perspective above:

| Company | Proposals on additional observations |
| --- | --- |
| CATT | - In general, we are fine with FL proposal and regarding PRACH B4, based on our performance evaluation result, it isn’t bottleneck channel in Rural 2G Hz. If PRACH B4 is only bottleneck in rural 2GHz, we can use other PRACH format such as format 0-3. So we suggest PRACH B4 needn’t be made as bottleneck channel for FR1. |
| vivo | - Generally fine with this proposal. - Agree with CATT that for PRACH format B4 in rural 2GHz scenario, we can use longer PRACH format to enhance PRACH coverage performance, e.g. format 0, format 2, therefore we propose to remove PRACH. |
| Qualcomm | - For the absolute metrics, can include the table you presented above as part of this proposal? We found it to be very insightful and informative. - Using this table as a reference, rather than priorities, can we simply state that certain channels are a bottlenecks in many/some scenarios? Suggest the following: - The following channels are identified as a bottleneck for many scenarios - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU) - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU) - PUCCH format 3 with 22bit - The following channels are identified as a bottleneck for some scenarios - PRACH format B4 - For the third priority, can we merge with second sub-bullet? We don’t think that large enhancements are necessary in the 700MHz case. We can identify that these channels come up in the context of a 700 MHz deployment. - We prefer to leave out the 6dB requirement as we have some concerns on the representative values for PUCCH Format 3 with 11 bits. This channel’s performance seems too close to the performance of PUCCH Format 1 with 2 bits. - In particular, we are a little concerned about the MIL and MPL values of PUCCH Format 3 11 bits in the rural 700Mhz scenario. Its far too close to PUCCH Format 1 performance (< 1 dB). We are expecting it to fall right between PUCCH Format 3 with 22 bits and PUCCH Format 1 with 2 bits, so about 3 dB gap between these 3 channels. It may be an artifact of how we arrived at the representative value. |
| Intel | - We are fine with the FL’s proposal in principle, especially considering the priorities for different channels. |
| China Telecom | - We support the bottleneck channels in the 1st priority. And PUSCH of Msg.3 should also be considered. |
| Ericsson | - Our understanding is that the PRACH coverage is driven by the missed detection requirement of 1% selected for the study. as we describe in R1-2008422. A reduced requirement of say 10% could improve coverage by e.g. 5.6 dB for 700 MHz FDD. We therefore suggest that this be clarified, e.g.: - 2nd priority – useful some scenarios - PRACH format B4 (scenarios where each PRACH must be detected 99% of the time) - PUCCH carrying only CSI has a similar issue: using a 1% (rather than say 10%) BLER target drives the coverage limit. While PUCCH carrying A/N naturally needs lower error rates, CSI tends to be more robust to error. This could be clarified as follows: - 1st priority – useful for many scenarios - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU) - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU) - PUCCH format 3 with 22bit (scenarios where each PUCCH with CSI must be received 99% of the time) - 3nd priority – useful when large (e.g. more than 6dB) enhancement is available for the worst bottleneck channel to enjoy the benefit - PUCCH format 1 - PUCCH format 3 with 11bit (scenarios where each PUCCH with CSI must be received 99% of the time) |
| ZTE | Our understanding is that, what we should we do in this agenda is to list bottleneck channels for each scenario. There is no need to further list the priority. Using a table like you made for absolute target seems sufficient. |
| Samsung | We are fine with FL’s proposal. Priority based grouping gives us more insight. |
| OPPO | - Generally fine with this proposal, but we may not set the priority. It would be difficult to wait for one channel after other channels. Further, we can consider different category for enhancement, e.g. the data channel and other channels. The following channels are identified as the potential bottleneck channels derived from absolute metrics (i.e. service dependent metric and scenario dependent metrics) and MIL. - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU) - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU) - PUCCH format 1 - PUCCH format 3 with 11bits and 22bit - PRACH format B4 - Useful for Rural 2GHz - PUSCH of Msg.3 Useful for Rural 700MHz and Rural 4GHz |
| CMCC | Generally fine with FL’s proposal. And we are also fine with this prioritized method. For the PRACH format 4, we are a little curious why Format 0 cannot be used in those scenarios. But we could stay open for this. - We share a similar view that Msg 3 needs enhancements. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support the FL proposal. |
| Nokia/NSB | While we can be fine with FL’s proposal in principle, we have the following observations: - Does RAN1 think that if a channel is considered a bottleneck for a specific configuration, then a specific enhancement targeting that configuration will be specified at the end of the WI (if applicable)? We are not sure this is a very likely outcome, with few exceptions (e.g., eMBB vs. VoIP). Easy examples in this sense are PUCCH and PRACH: - Both PF3 11 bits and PF3 22 bits were part of the baseline RAN1 agreed for this SI. If PF3 with 22 bits is a bottleneck for some scenarios, and PF3 11 bits is as well but for fewer scenarios, is RAN1 really going to enhance PF3 only in case of specific payload size, or rather PF3 in general? From our perspective, the latter case is more likely. - Both PRACH 0 and PRACH B4 were part of the baseline RAN1 agreed for this SI. If PRACH B4 is a bottleneck for some scenario, and PRACH 0 is not, is RAN1 really going to enhance PRACH only in case a specific preamble is used or rather PRACH in general? Again, from our perspective, the latter case is more likely - RAN1 could not test all the possible channel/signal configurations. We think it is just natural to consider a channel a bottleneck (or not) in general and not just for a specific configuration. The specific configurations RAN1 selected are very useful to characterize the amount of dBs any enhancements should bring to the considered channel/signal in general. Other approaches would not seem very reasonable, and probably more complex from spec perspective. As such, we would be in favour of saying, e.g., PUCCH F3 is a bottleneck channel more than PUCCH F3 22 bits is a bottleneck channel, and use the performance for 22 bits to calculate how many dBs we need…Same logic would apply to any other candidate channels. Prioritization can be a good idea in general, however we are not sure it is necessary in this case. Either way, if a channel is a bottleneck and enhancements are needed, wouldn’t RAN1 simply spend time resources to work on it? |

Summary of the discussion

Necessity of PRACH enhancement

  - The analyses showed that PRACH format B4 could be a bottleneck, some companies pointed out if other format can be used or not. If yes, PRACH wouldn’t be a bottleneck.

  - If RAN1 agrees to enhance PRACH, only for specific format or PRACH in general?

Necessity of PUCCH enhancement

  - If RAN1 agrees to enhance PUCCH F3, only for specific payload size or PUCCH F3 in general?

Necessity of priority

  - Some companies is OK to introduce priority for enhancement

  - Some companies think prioritization is not necessary.

  - One company just want to list all the bottleneck channels for each scenario

Other

  - Representative value for PUCCH format 1 and 3 with 11bits, i.e. these values are too close

    - FL Perspective: Understand the concern, but it is the nature of representative value. The correctness of the values can be checked but FL doesn’t recommend to do something further than that.

  - 6dB requirement

    - FL Perspective: This is just example. Another criteria can be used to filter the potential bottleneck channel(s)

  - Clarification of the simulation assumption, i.e. target BLER

    - FL perspective: Concern on this approach. If we mention about simulation assumption, more and more information eventually need to be capture. FL proposal is to add a generic description.

Most companies are OK with the direction proposed by FL. Companies has different preference on the detailed direction.

- FL Proposal:

Capture the following table as a result of bottleneck identification by using absolute metrics [FL note: Firstly, RAN1 should agree if such a table should be captured in the TR]

Capture the following observation [ FL note: Firstly, RAN1 should agree if such observations are necessary, then the discussion can be continued via email]

  - The bottleneck channels identified by absolute metrics can be further filtered by using relative difference between channels in MIL

    - If only one channel for each scenario/target can be enhanced, the following channels can be chosen as the candidate for enhancements

      - PUSCH for eMBB – value XX dB

        - [FL note: the might a company who want to capture a value. However, FL does not recommend doing so because such discussion is so controversial. Table above is sufficient]

      - PRACH Format B4

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

        - In the evaluation, the scenario where each PRACH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

        - In addition, it was not identified through the evaluation that other PRACH format (such as 0) will not be the bottleneck. If other format is available, the enhancement for this PRACH format may not be required.

      - PUSCH for VoIP

      - PUCCH format 3 with 22bit payload

        - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

    - If two channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - Broadcast PDSCH for BS with 24dBm/MHz Tx Power

        - The enhancement may be useful if large enhancement for PUSCH for eMBB is available or the service requirement for PUSCH for eMBB can be relaxed

      - PUCCH format 3 with 11bit payload

        - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

      - PUSCH of Msg.3

    - If more than 3 channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - PUCCH format 1

        - The enhancement may be useful if large enhancement for the worst channel (such as PUSCH for eMBB) is available or the service requirement can be relaxed

The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - [1st priority – ] useful for many scenarios

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

    - PUCCH format 3 with 22bit  [(scenarios where each PUCCH with CSI must be received 99% of the time)]

      - [FL note: proposal by Ericsson. If observation in the above main bullet is agreed, this description is not necessary, and FL prefers to do so]

  - [2nd priority – ] useful some scenarios

    - [PRACH format B4 (scenarios where each PRACH must be detected 99% of the time)

      - Useful for Rural 2GHz

      - [FL note: CATT, CMCC, vivo see no necessity ]

    - PUSCH of Msg.3

      - Useful for Rural 700MHz and Rural 4GHz

      - [FL note: China Telecom want to give 1st priority]

  - [3rd priority –] useful depending on the enhancements for other bottleneck channel(s) when large (e.g. more than 6dB) enhancement is available for the worst bottleneck channel to enjoy the benefit and/or requirement for other bottleneck channel(s) can be relaxed

    - PUCCH format 1

    - PUCCH format 3 with 11bit [ (scenarios where each PUCCH with CSI must be received 99% of the time)]

  - [FL note: Disucssion on the necessity of prioritization. Some companies don’t want to describe the priority]

FFS: bottleneck identification derived from relative metric

Update after the GTW session on 12th November

In order to finalize all the remaining issues, FL proposes to continue the discussion in each subsection.

[Closed] Confirmation of the working assumption on 1st step bottleneck identification

In the GTW session on 12th November, the following was agreed with working assumption.

- Agreements:

- Capture the following table (working assumption) showing the result of bottleneck identification by using absolute metrics

Please note that the reason for the WA is as follows:

- To encourage companies to check the details of the table.

- Ask if the addition of Rural 4GHz with long distance TDD LOS O2O scenario for bottleneck identification is acceptable.

Also, FL found a typo in the table. So the table is updated as follows:

The excelsheet of this table is available from the following link: 
https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

Companies are invited to provide their views on the working assumption.

| Company | Proposals on additional observations |
| --- | --- |
| CATT | - We are fine with FL proposal. |
| ZTE | - Fine with the update. |
| CMCC | - General fine with current version. |

Status at the end of the e-meeting

All the companies are OK to adopt the update table. The following can be the final proposal.

FL proposal:

- Regarding the working assumption on the result of bottleneck identification by using absolute metrics, the working assumption is confirmed by applying the following table.

Note: the excelsheet of this table is available from the following link: 
https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

[Closed] Details of 2nd step bottleneck identification

In the GTW session on 12th November, the following was agreed but the details are still FFS.

- Agreements:

- For FR1, the potential bottleneck channels identified by absolute metrics can be further filtered by using relative difference between channels in MIL

  - FFS details, including the possibility of applying the relative difference for a limited set of scenario(s)

For the FFS point, FL has prepared the example text for approval:

FL proposal

If only one channel for each scenario/target can be enhanced, the following channels can be chosen as the candidate for enhancements

  - PUSCH for eMBB

    - Urban 4GHz, Urban 2.6GHz, Rural 4GHz TDD NLOS O2I, Rural 2.6GHz TDD NLOS O2I, [ and Rural 4GHz with long distance TDD LOS O2O] scenarios

    - Potentially useful for Rural 700MHz FDD NLOS O2I scenario depending on the enhancements for other channel(s)

  - PRACH Format B4

    - Rural 2GHz FDD NLOS O2I scenario

    - Potentially useful for Rural 4GHz TDD NLOS O2I scenario depending on the enhancements for other channel(s)

    - [However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required. / FL note: it seems controversial, remove?]

    - [In the evaluation, the scenario where each PRACH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed. / FL note: it seems controversial, remove?]

    - [In addition, it was not identified through the evaluation that other PRACH format (such as 0) will not be the bottleneck. If other format is available, the enhancement for this PRACH format may not be required. / FL note: it seems controversial, remove?]

  - PUSCH for VoIP

    - Rural 700MHz FDD NLOS O2I scenario with Service dependent target

    - Potentially useful for Rural 4GHz TDD NLOS O2I, Rural 700MHz FDD NLOS O2I with ISD 4000m and Rural 4GHz with long distance TDD LOS O2O scenarios depending on the enhancements for other channel(s)

  - PUCCH format 3 with 22bit payload

    - Rural 700MHz FDD NLOS O2I with ISD 4000m

    - Potentially useful for Rural 4GHz TDD NLOS O2I scenarios depending on the enhancements for other channel(s)

    - [In the evaluation, the scenario where each CSI on PUCCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed. / FL note: it seems controversial, remove?]

If two channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

  - Broadcast PDSCH

    - Urban 4GHz with 24dBm/MHz BS Tx Power

    - [The enhancement for this channel may be useful if large enhancement for PUSCH for eMBB is available or the service requirement for PUSCH for eMBB can be relaxed / FL note: this may be removed to align the description level with other parts]

    - [It is, however, noted that the gap between the worst channel and this channel reaches 10.53dB. This means that a large enhancement for the bottleneck channel is required to maximize the benefit of the improvements on broadcast PDSCH. /FL note: number is included. Not consistent with other observations , remove?]

  - PUCCH format 3 with 11bit payload

    - Rural 2.6GHz TDD NLOS O2I scenario

    - Potentially useful for Rural 4GHz TDD NLOS O2I and Rural 700MHz FDD NLOS O2I scenarios depending on the enhancements for other channel(s)

    - [In the evaluation, the scenario where each CSI on PUCCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed. / FL note: it seems controversial, remove?]

  - PUSCH of Msg.3

    - Rural 700MHz FDD NLOS O2I scenario with Service dependent target

    - Potentially useful for Rural 4GHz TDD NLOS O2I and Rural 700MHz FDD NLOS O2I with ISD 4000m scenarios depending on the enhancements for other channel(s)

If more than 3 channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

  - PUCCH format 1

    - Rural 4GHz TDD NLOS O2I and Rural 700MHz FDD NLOS O2I scenarios

    - [The enhancement may be useful if large enhancement for the worst channel (such as PUSCH for eMBB) is available or the service requirement can be relaxed / FL note: for the consistency with other parts, this can be removed]

As the table with MIL values shows, the number of samples is different for each channel for each scenario and the evaluation results submitted by companies have certain level of divergence. Even though more simulation study is necessary to achieve an accurate estimation of bottleneck channels, it has not been pursued in this study item.

- Please note that the following are potential issue for further discussion:

Whether or not any values can be included

  - FL has a concern to do so because it is clearly controversial. The table agreed in section 2.5.2.1 will give sufficient information.

Which scenario the bottleneck identification by relative difference apply to

  - FL does not see a strong necessity to limit the scenario. MIL is useful to double-check the validity of representative value in general.

Companies can refer to the following table in section 2.5.2.1 to check the correctness of this observation.

Companies are invited to provide their views on the FL proposal above.

| Company | Proposals on additional observations |
| --- | --- |
| Qualcomm | - This statement is saying something about PUSCH but is captured under PUCCH. Doesn’t seem relevant, suggest removing it. - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed. - Even if this is a typo, we see no reason to add this sub-bullet. - We prefer to not introduce any further filtering. It doesn’t seem necessary. Many of the gaps are too small or within margin of error. Prefer to leave the list as is. |
| CATT | - We suggest deleting PRACH B4 in the1st bullet because PRACH b4 isn’t bottleneck channel based on our performance evaluation in Rural 2GHz FDD NLOS O2I and other PRACH format (such as 0) will not be the bottleneck Rural 2GHz FDD NLOS O2I. |
| ZTE | As discussed in GTW session, there is no need to list the sub-bullets under each channel. Take the following one for instance, it would be sufficient to capture standard deviation values as in the table. Is there any evidence/reasoning to have such conclusion? It could be one or two samples which make the standard deviation relatively large. But we cannot make any statement based on one or two samples. Otherwise, we need to use the sample with the largest MIL value to compare with the target and make corresponding observation. - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required. In addition, it seems current formulation cannot reflect PRACH Format B4 as the second worst channel in Rural 4GHz TDD NLOS O2I? |
| Qualcomm | We think all the sub-bullets in yellow can be dropped. |
| CMCC | General fine to us. And the yellow parts could also be removed. |

Final status at the end of e-meeting.

The email discussion before the closure of e-meeting did not solve the controversial part of the proposal. Thus, no consensus achieved. This section is closed without any FL proposal

[Closed] Conclusion of bottleneck identification

It was concluded during the discussion on 12th November that (1) introduce priories for FR1, and (2) do not separate the bottleneck identification by scenarios.

FL proposal

The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - 1st priority – useful for many scenarios

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

    - PUCCH format 3 with 22bit (scenarios where each PUCCH with CSI must be received 99% of the time)

  - 2nd priority – useful for some scenarios

    - PRACH format B4 (scenarios where each PRACH must be detected 99% of the time)

      - Useful for Rural 2GHz

    - PUSCH of Msg.3

      - Useful for Rural 700MHz and Rural 4GHz

  - 3rd priority – useful depending on the enhancements for other bottleneck channel(s)

    - PUCCH format 1

    - PUCCH format 3 with 11bit (scenarios where each PUCCH with CSI must be received 99% of the time)

The issues identified during the 1st email discussion are summarized below.

Issue 1. Whether or not PRACH (format B4) is a bottleneck

Issue 2. Mapping between priority and channel

  - Can PUCCH format 1 and 3 with 11bit can be 2nd priority ?

  - Can PUSCH of Msg.3 be 1st priority?

Issue 3. Additional note for PUCCH format 3 and PRACH

  - i.e. scenarios where each xxxx must be received 99% of the time

  - FL perspective: This description is correct. However, FL thinks adding so many disclaimers like this should be avoided. If there is no such concern, FL is fine to introduce this note.

Companies are invited to provide their views on the FL proposals

| Company | Proposals on additional observations |
| --- | --- |
| Qualcomm | - Don’t support adding “(scenarios where each PUCCH with CSI must be received 99% of the time)”. Its not necessary. It was the default simulation metric (also required by RAN4) and we don’t need to capture it here. - Agree with FL that such sub-bullets are not necessary. - Regarding issue 2, PF3 11 bits appears as bottleneck in 4 scenarios. Suggest including this in first or second priority. Rules for prioritization aren’t very clear. |
| CATT | - We suggest deleting PRACH B4 from 2nd priority channel lists because PRACH b4 isn’t bottleneck channel based on our performance evaluation in Rural 2GHz FDD NLOS O2I and other PRACH format (such as 0) will not be the bottleneck Rural 2GHz FDD NLOS O2I. |

[Closed] Estimation of necessary amount of enhancements

Even though RAN1 cannot achieve an explicit agreement on the necessity of Estimation of necessary amount of enhancements, it is clear that RAN1 will not have time to perform such an exercise, and no company thinks this has a high priority. Therefore, FL would like to propose to temporary close the discussion in this section for the time being to focus on the critical issues. FL may open this discussion again considering the progress of overall discussions.

Update on 12th November

Given the number of remaining issues, RAN1 will not have time to discuss this issue. Therefore, FL would like to close the discussion in this section.

Final Summary

Given the agreement in the GTW session on 13th November. There is no remaining issue handled in this section. Therefore, the discussion in this section is closed.

[Closed] Other issues

Update on 12th November

Since no more issue is foreseen on top of the open issues on the table, the discussion of this section can be closed.

Proposals for GTW sessions

Proposals in the GTW session on 28th October

FL proposal

Representative values of [MCL, MIL and MPL] are used for bottleneck identification

  - Further down-selection of MCL/MIL/MPL may be performed depending on the decision of target performance metric(s)

  - Representative values are computed for the following scenarios for FR1

    - 1st priority

      - Urban 4GHz TDD

      - Urban 2.6GHz TDD

      - Rural 4GHz TDD NLOS O2I

      - Rural 700MHz FDD NLOS O2I

    - 2nd priority

      - Rural 2.6GHz TDD NLOS O2I

      - [Rural 2GHz FDD NLOS O2I – FL note: can be dropped? Nobody talked about it during the 1st round]

      - Rural 700MHz with long distance FDD LOS O2O

    - Note: the difference between 1st priority and 2nd priority is as follows

      - 1st priority scenarios have sufficient number of samples and strong support by companies, while insufficient number of samples scenarios or less number of supporting companies for 2nd priority scenarios

      - RAN1 discussion will focus on 1st priority scenarios for making observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

        - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

  - Representative values are computed for the following channels or signals for FR1

    - PUSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

    - PUSCH for VoIP (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

    - PUCCH Format 1 with 2bits

    - PUCCH Format 3 with 11bits

    - PUCCH Format 3 with 22bits

    - SSB

    - PRACH format 0

    - PRACH format B4

    - PDCCH of Msg.2

    - PDSCH for Msg.2

    - PUSCH of Msg.3

    - PDSCH of Msg.4

    - PDCCH

    - PDSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - No categorization by other simulation parameters (such as UE speed, antenna gain collection factors, Tx power) will be introduced for FR1.

  - If the number of samples used to compute a representative value is less than X for each scenario, this representative value is not used for bottleneck identification

    - Option 1: X = 3 (i.e. representative values with 1 and 2 samples is regarded as invalid)

    - Option 2: X = 4 (i.e. representative values with 1,2 and 3 samples is regarded as invalid)

  - A representative value is derived by taking the mean value from companies’ evaluation results

      - Excluding the highest & the lowest values when the number of samples is more than [2 or 4]

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification, can be used to make additional observations to be captured in the TR.

  - The following scenarios/channels are the candidate for the discussion for FR1

    - PUSCH for CSI, PUCCH with HARQ-ACK for Msg4, and Low Tx power for 4GHz BS etc.

  - This discussion will be held in RAN1#103-e on a low priority basis.

Proposals in the GTW session on 3rd November

FL Proposal:

Rephrase the following terminologies, which is used in e.g. link budget template:

  - “PDCCH of Msg.2” is referred to as “broadcast PDCCH (PDCCH of Msg.2)”

  - “PDCCH” is referred to as “unicast PDCCH”

Representative values are computed for the following channels or signals for FR1, and they are used to draw observation and/or to identify bottlenecks if the number of samples for each scenario is more than Y (Y=[2 or 3] – FL recommendation is 2)

  - PUSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for VoIP (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for CSI (if necessary number of evaluation results for representative value is submitted)

  - PUCCH with HARQ-ACK for Msg.4 (if necessary number of evaluation results for representative value is submitted)

Representative values are computed for t The following scenarios are used for drawing observations and/or bottleneck identification for FR1

  - 1st priority (i.e. companies showed their interests and number of available channels/signals is greater than Z, where Z=11)

    - Urban 4GHz TDD (16 available channels/signals)

    - Urban 2.6GHz TDD (12 available channels/signals)

    - Rural 4GHz TDD NLOS O2I (16 available channels/signals for Y=2 and 14 for Y=3)

    - Rural 700MHz FDD NLOS O2I (13 available channels/signals)

  - 2nd priority (i.e. companies showed their interests but number of available channels/signals is no greater than Z)

    - Rural 2GHz FDD NLOS O2I (10 available channels/signals for Y=2 and 9 for Y=3 )

    - Rural 2.6GHz TDD NLOS O2I (10 available channels/signals for Y=2 and 9 for Y=3 )

    - Rural 700MHz with long distance FDD LOS O2O (11 available channels/signals for Y=2 and 8 for Y=3)

      - [FL note: It is fine to put 1st priority for 2GHz and 2.6GHz by choosing appropriate value for Y and Z (Y=2 and Z=9). If we do so, 700MHz w/ L.D. will also be the 1st priority. However, the necessity for 700MHz is not so clear given less interest by companies. The best solution would be to remove all parentheses and just put 1st priority for the selected scenarios (Rural 2.6GHz and 2GHz).]

    - [Rural 4GHz with long distance FDD LOS O2O (6 available channels/signals for Y=2 and 2 for Y=3)

      - [FL note: 6 channels/signals may not enough to draw observation. More discussion necessary]

  - Note: the difference between 1st priority and 2nd priority is as follows

    - RAN1 discussion will focus on 1st priority scenarios for drawing observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

    - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

No categorization by other simulation parameters (such as UE speed, antenna gain collection factors, Tx power) will be introduced for FR1

  - The amount of available results for DL channels in FR1 4GHz scenario should be considered as given by the total number of available results for both 33 dBm/MHz and 24 dBm/MHz, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

  - In order to address the misalignment issue of the companies’ evaluation results due to no categorization and/or different simulation assumptions,

    - Number of samples and standard deviation is shown together with a representative value, and

    - A description on the potential fluctuation due to no categorization and/or different simulation assumptions can be added in the observation

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification due to the lack of number of samples etc., can be used to make additional observations to be captured in the TR.

  - The following scenarios/channels are the candidate for the discussion for FR1

  - PUSCH for CSI, PUCCH with HARQ-ACK for Msg4, and Low Tx power for 4GHz BS, and Rural with long distance for 4GHz TDD LOS O2O scenario etc.

  - This discussion will be held in RAN1#103-e on a low priority basis.

FL proposal:

  - For, Scenario dependent targets, e.g., ISD/MPL

      - MPL is used to compute ISD values computed from ISD will be used for the bottleneck identification.

      - The following formula is used to convert the ISD value to MPL value:

        - For urban scenarios,

          - Target MPL dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97-0.6∙hUT-1.5, where W=20m is the average street width, h=20m is the average building height, hBS=25m is the BS antenna height, hUT=1.5m is the UT antenna height, fc is the carrier frequency and d3D is the target range calculated by ISD/sqrt(3).

        - For rural scenarios,

          - Target MPL dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97,where W=20m, h=5m, hBS=35m and hUT=1.5m.

        - For rural with long distance scenarios,

          - Target MPL dB=20∙log1040∙π∙dBP∙fc3+min0.03∙h1.72,10∙log10dBP-min0.044∙h1.72,14.77+0.002∙log10h∙dBP+40∙log10d3DdBP,where W=20m, h=5m, dBP=2∙π∙hBS∙hUT∙fcc, hBS=35m, hUT=1.5m and c=3∙108m/s.

      - Scenarios and target ISD below can be used to draw observation

        - Urban 4GHz TDD - 400m

        - Urban 2.6GHz TDD – 400m

        - Rural 4GHz TDD NLOS O2I – [1732] m - [FL note: discussion needed if 1732m is agreeable from the feasibility point of view]

        - Rural 2.6 GHz TDD NLOS O2I – 1732m

        - Rural 2 GHz FDD NLOS O2I – 1732m

        - Rural 700MHz FDD NLOS O2I – [3000m and 4000m] – [FL note: need confirmation by companies if 4000m is acceptable]

        - [Rural with long distance 700MHz FDD LOS O2O – FL note: operators’ input is highly encouraged. Otherwise, no ISD target can be set for this scenario]

        - [Rural with long distance 4GHz TDD LOS O2O – FL note: this issue should be discussed under section 2.3.1 first]

      - Further discussion is necessary how to capture the evaluation results with SLS

  - For Service dependent targets, e.g., [MCL=147] dB for VoIP

    - 139.2dB is used for Rural 700MHz scenario for drawing observation

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - [PUCCH format 3 with 22bits]

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

FL proposal:

- All the parameters/values/configurations related to FR1 modelling for which an agreement has not been reached among companies prior to RAN1 #103-e, will be henceforth treated according to the “reported by companies” principle. RAN1 will not spend further time during RAN1 #103-e on the resolution of these issues.

Proposals in the GTW session on 4th November

FL Proposal

PUSCH for CSI with 11 bits and PUSCH for CSI with 22 bits are considered two separate channels.

FL Proposal

When coverage of 2nd priority channels is discussed by RAN1, observations can be drawn and bottleneck identification can occur.

- Note: If RAN1 cannot reach consensus on 2nd priority channels, the channel(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

- Note: 2nd priority channel refers to the channel which number of samples is smaller than 4 for each scenario.

Proposals in the GTW session on 10th November

FL proposal 1

  - For, Scenario dependent targets, e.g., ISD/MPL

    - For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

    - Target ISD values for each scenario are as follows:

        - Urban 4GHz TDD – 350 400, 500 and 700m for observation and 400m for bottleneck identification

          - [FL note: China telecom proposed to remove 350 and 700m for observation. We need to check if everyone is OK for this approach]

        - Urban 2.6GHz TDD – 350, 400, 500 and 700m for observation and 400m for bottleneck identification

          - [FL note: if 350m and 700m are removed from Urban 4GHz, similar exercise might be applicable to Urban 2.6GHz as well. Need the confirmation by the group ]

        - Rural 4GHz TDD NLOS O2I – 1732 and 3000 m for observation and [1732m with 33dBm/MHz BS transmit power for bottleneck identification]

          - [FL note: Intel showed a concern to adopt 1732m for bottleneck identification]

        - Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 700MHz FDD NLOS O2I – [3000, and ] 4000m for observation and 4000m for bottleneck identification

          - [FL note: 4000m can be adopted considering the request from operators. China telecom proposed to remove 3000m for observation. We need to check if it is OK for everyone]

        - Rural with long distance 700MHz FDD LOS O2O – 12, 20 and 30km for observation and bottleneck identification is not performed for this scenario

          - [FL note: 12km for bottleneck identification was proposed by CATT, but no strong support from operators]

        - Rural with long distance 4GHz TDD LOS O2O – 12, 30km for observation and bottleneck identification is not performed for this scenario

FL proposal 2

  - For Service dependent targets, e.g., [MCL=147] dB  for VoIP

    - MCL of 139.2dB is used for Rural 700MHz scenario for drawing observation and can be used to identify bottlenecks (if applicable)

      - Captured the following table, which shows how the target value (i.e. 139.2dB) is derived, in Annex of the TR.

| Transmitter |  |
| --- | --- |
| (2a) Number of transmit chains modelled in LLS | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 3840000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: For FR1 downlink, (3b) should satisfy the following: For 4GHz frequency, 24 and 33 For 2.6 GHz frequency, 33 For 700MH and 2GHz frequency, 36 Note: For FR2 downlink, the following should be satisfied: 40 dBm for 100 MHz Urban scenario, 23 dBm for 100 MHz Indoor scenario. Note: no PSD constraint for uplink | 17.16 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. for uplink, (3a) = (3c) | 3840000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 |
| Receiver |  |
| (10a) Number of [receive TxRUs] Note: this row is void (empty) for downlink | 2 |
| (10b) Number of receive chains modelled in LLS | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 |
| (13) Receiver noise figure (dB) | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -98.2 |
| (19) Required SNR (dB) | 5 |
| (20) Receiver implementation margin (dB) | 2 |
| (21) H-ARQ gain (dB) or Process gain for UMTS Note: Only applicable if HARQ is not considered in LLS, | 25 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -116.2 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 139.2 |

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - [PUCCH format 3 with 11bits]

        - [FL note: final confirmation in GTW is recommended, one company think it is not necessary for VoIP]

      - [PUCCH format 3 with 22bits]

        - [FL note: final confirmation in GTW is recommended. Majority supports to keep it, while two companies think it is not necessary for VoIP]

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

FL Proposal 3

  - For Relative difference between channels

    - MIL is used to derive relative differential values.

    - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

    - Relative difference between channels is used to estimate the necessary amount of enhancements for potential bottleneck channel(s) in dB (if applicable)

    - For each channel, relative differential value is defined by the following formula

      - (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples + X dB)

        - Option A-1a: X=3 or 6 for FR1

        - Option A-1b: X=0 for FR1

        - Note: the necessity to introduce different X value for FR2 can be discussed under A.I. 8.8.1.2.

        - [FL note: FL preference is option A-1a since it is easy to draft the observation texts. If non-zero X value cannot be agreed immediately, FL would propose to go with X=0 to move forward. No companies proposed specific value for X, though. 
Note that the decision of X does not impact on the discussion in section 2.2.5, i.e. X is used for draw observation]

      - [Apply the following 3-step approach

        - [Identify bottleneck channels in RACH procedure, i.e., compare coverage of Msg.1 (PRACH), Msg.2 (PDCCH/PDSCH), Msg.3 (PUSCH), and Msg.4 (PDCCH/PDSCH/PUCCH with HARQ-ACK).

        - Identify bottlenecks in control channel coverage, i.e., compare coverage of broadcast PDCCH vs PUCCH. This gives us an idea of control coverage that is independent of service rate requirements.

        - Identify bottlenecks in data channel coverage. Compare control channel coverage with data channel coverage.]

        - [FL note: This is a proposal by Qualcomm (former option A-2) also supported by ZTE, but not many companies showed their interest. We need final check on the preference of the group on this approach, including the compromise by the proponents]

      - [(MIL of the RACH related channel) - (MIL of the worst RACH related channel among the channels that have more than 3 samples)

        - RACH related channel refers to Msg.1 (PRACH), Msg.2 (PDCCH/PDSCH), Msg.3 (PUSCH), and Msg.4 (PDCCH/PDSCH/PUCCH with HARQ-ACK).

        - This formula is applicable only for drawing observation]

        - [FL note: this is a compromise proposal considering benefit of former option A-2 (Qualcomm proposal). We need to check if group can accept this approach]

- FL proposal 4

For FR1 and FR2, use all the 3 target metrics for identification of coverage bottleneck channel(s) and estimation for necessary amount of enhancements in dB (if applicable)

    - At least absolute metrics (i.e. scenario dependent and service dependent) are used for the identification of coverage bottleneck channel(s)

      - If relative difference is used for identification of coverage bottleneck channel(s), it is limited to supplement of absolute metrics (i.e. relative difference cannot be used alone for bottleneck identification)

        - [FL note: this would be the best approach considering the companies view, but there are companies who don’t want to use relative difference even for the supplemental use. This should be discussed in the GTW session.]

    - At least relative difference is used for the estimation of necessary amount of enhancements in dB (if applicable)

For FR1 and FR2, the following procedure applies to coverage bottleneck channel(s) and estimation for necessary amount of enhancements in dB

  - Step 1: Use absolute targets (i.e. scenario dependent targets and service dependent target) to filter the channels, i.e. can be a bottleneck or not

  - Step 2: Apply relative difference to the filtered channels which do not meet the absolute targets, to further filter the bottleneck channels

    - The applicability and the detailed procedure of this step will be discussed on case-by-case basis, e.g. depending on FR or scenario.

    - At least, how many channels and how much gaps needs enhancements should be taken into account in this step.

    - [FL note: it seems there is no clear majority whether to introduce this step. FL believes the description above is the good way forward]

  - Step 3: perform the estimation of the necessary amount of enhancements in dB by using relative difference

    - This step can be done on best effort basis.

    - The detailed procedure needs to be discussed during this step.

      - [FL note: this is a request from Samsung, which FL thinks reasonable. On the other hand, RAN1 will not have enough time for this in the next GTW. Thus, FL proposes to leave it open for now]

Proposals in the GTW session on 12th November

- FL Proposal:

Capture the following table showing the result of bottleneck identification by using absolute metrics [FL note: Firstly, RAN1 should agree if such a table should be captured in the TR]

Capture the following observation for further bottleneck identification [FL note: Firstly, RAN1 should agree if such observations are necessary, then the discussion can be continued via email]

  - The bottleneck channels identified by absolute metrics can be further filtered by using relative difference between channels in MIL

    - If only one channel for each scenario/target can be enhanced, the following channels can be chosen as the candidate for enhancements

      - PUSCH for eMBB – value XX dB

        - [FL note: the might a company who want to capture a value. However, FL does not recommend doing so because such discussion is so controversial. Table above is sufficient]

      - PRACH Format B4

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

        - In the evaluation, the scenario where each PRACH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

        - In addition, it was not identified through the evaluation that other PRACH format (such as 0) will not be the bottleneck. If other format is available, the enhancement for this PRACH format may not be required.

      - PUSCH for VoIP

      - PUCCH format 3 with 22bit payload

        - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

    - If two channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - Broadcast PDSCH for BS with 24dBm/MHz Tx Power

        - The enhancement may be useful if large enhancement for PUSCH for eMBB is available or the service requirement for PUSCH for eMBB can be relaxed

      - PUCCH format 3 with 11bit payload

        - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

      - PUSCH of Msg.3

    - If more than 3 channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - PUCCH format 1

        - The enhancement may be useful if large enhancement for the worst channel (such as PUSCH for eMBB) is available or the service requirement can be relaxed

The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - [FL note: Discussion on the necessity of prioritization. Some companies don’t want to describe the priority]

  - [1st priority – ] useful for many scenarios

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

    - PUCCH format 3 with 22bit  [(scenarios where each PUCCH with CSI must be received 99% of the time)]

      - [FL note: proposal by Ericsson and CMCC is OK for this proposal. If observation in the 2nd main bullet is agreed, this description is not necessary to be captured here, and FL prefers to do so]

  - [2nd priority – ] useful for some scenarios

    - [PRACH format B4 (scenarios where each PRACH must be detected 99% of the time)

      - Useful for Rural 2GHz]

      - [FL note: CATT, CMCC, vivo see no necessity ]

    - PUSCH of Msg.3

      - Useful for Rural 700MHz and Rural 4GHz

      - [FL note: China Telecom want to give 1st priority]

  - [3rd priority –] useful depending on the enhancements for other bottleneck channel(s) when large (e.g. more than 6dB) enhancement is available for the worst bottleneck channel to enjoy the benefit and/or requirement for other bottleneck channel(s) can be relaxed

    - PUCCH format 1

    - PUCCH format 3 with 11bit [ (scenarios where each PUCCH with CSI must be received 99% of the time)]

Proposals in the GTW session on 13th November

FL proposal 1

For Rural with long distance 4GHz TDD LOS O2O scenario, 12km is applied for observation and bottleneck identification

FL proposal 2

The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - 1st priority [FL note: everyone agree to be important]

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

  - 2nd priority  [FL note: companies have different understanding on the priority & necessity]

    - PRACH format B4 [(scenarios where each PRACH must be detected 99% of the time) ]

    - PUSCH of Msg.3

    - PUCCH format 1

    - PUCCH format 3 with 11bit [(scenarios where each PUCCH with CSI must be received 99% of the time)]

    - PUCCH format 3 with 22bit [(scenarios where each PUCCH with CSI must be received 99% of the time)]

    - Broadcast PDCCH

FL proposal 3

Capture the following observation in the TR

  - The following observation can be drawn when the potential bottleneck channels identified by absolute metrics is further filtered by using relative difference between channels ([FL note: QC has a concern on further filtering])

    - If only one channel for each scenario/target can be enhanced, the following channels can be chosen as the candidate for enhancements

      - PUSCH for eMBB – for Urban 4GHz, Urban 2.6GHz, Rural 4GHz TDD NLOS O2I, Rural 2.6GHz TDD NLOS O2I, [Rural 4GHz L.D. TDD LOS O2O]

      - [PRACH Format B4 - Rural 2GHz FDD NLOS O2I

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

        - In the evaluation, the scenario where each PRACH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

        - In addition, it was not identified through the evaluation that other PRACH format (such as 0) will not be the bottleneck. If other format is available, the enhancement for this PRACH format may not be required. ]

        - [FL note: Flagged by CATT, they think this explanation is unnecessary]

      - PUSCH for VoIP - Rural 700MHz FDD NLOS O2I with Service dependent target

      - PUCCH format 3 with 22bit payload - Rural 700MHz FDD NLOS O2I ISD 4000m

        - In the evaluation, the scenario where each PUCCH with CSI must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

          - [FL note: Flagged by Qualcomm, they think this explanation is unnecessary]

    - If two channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - Broadcast PDSCH for Urban 4GHz with 24dBm/MHz Tx Power BS

        - The enhancement for this channel may be useful if large enhancement for PUSCH for eMBB is available or the service requirement for PUSCH for eMBB can be relaxed

        - It is, however, noted that the gap between the worst channel and this channel reaches 10.53dB. This means that a large enhancement for the bottleneck channel is required to maximize the benefit of the improvements on broadcast PDSCH.

      - PUCCH format 3 with 11bit payload - Rural 2.6GHz TDD NLOS O2I and potentially Rural 700MHz FDD NLOS O2I (depending on the enhancements for other channel(s) )

        - In the evaluation, the scenario where each CSI on PUSCH must be received 99% of the time is assumed. If this requirement is relaxed, the necessity of enhancement can also be relaxed.

      - PUSCH of Msg.3 - Rural 700MHz FDD NLOS O2I

    - If more than 3 channels for each scenario/target can be enhanced, the following channels can be additionally chosen as the candidate for enhancements

      - PUCCH format 1 - Rural 4GHz TDD NLOS O2I and Rural 700MHz FDD NLOS O2I

        - The enhancement may be useful if large enhancement for the worst channel (such as PUSCH for eMBB) is available or the service requirement can be relaxed

Proposals at the end of the e-meeting

FL proposal:

  - Confirm the Working Assumption on the pathloss formula for rural with long distance scenarios.

  - TR editor will take care of the detailed description regarding which model in the reference document is used for these formulae.

FL proposal:

For Relative difference between channels

  - MIL is used to derive relative differential values.

  - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

  - For each channel, relative differential value is defined by the following formula for FR1

    - (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples)

FL proposal:

Capture the following tables in Annex of the TR

  - Note: Even when the channel has less than 4 samples, it can be included in the table.

  - Note: The excelsheet for this table is found below:

  - https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

  - Urban 4GHz scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Urban 2.6GHz scenario

  - Rural 4GHz TDD NLOS O2I scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Rural 2.6GHz TDD NLOS O2I scenario

  - Rural 2GHz Rural 2GHz FDD NLOS O2I scenario

  - Rural 700MHz FDD NLOS O2I scenario

  - Rural 4GHz with long distance TDD NLOS O2O scenario

FL proposal:

Capture the following observation for Urban 4GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 10.26 dB can be achieved at maximum.

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 8.12 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  7.09 dB

    - If low transmit power BS (i.e 24dBm/MHz) is assumed,

      - Broadcast PDCCH (PDCCH of Msg.2): 2.17dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 11.91 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  10.88 dB

    - PUSCH for VoIP with frame structure DDDSU: 0.03 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

    - PUSCH for VoIP with frame structure DDDSUDDSUU:  0.86 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22 bit payload: 2.62 dB

    - PRACH Format B4: 0.48dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUSCH of Msg.3: 1.11 dB

  - If low transmit power BS (i.e 24dBm/MHz) is assumed, the following DL channel(s) needs to be enhanced, additionally.

    - For 400m ISD

      - Broadcast PDCCH (PDCCH of Msg.2)

    - For 500m ISD

      - SSB: 2.85dB

      - Broadcast PDCCH (PDCCH of Msg.2): 5.95dB

      - PDSCH for eMBB with frame structure DDDSUDDSUU: 1.09 dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

FL proposal:

Capture the following observation for Urban 2.6GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 10.00 dB can be achieved at maximum.

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 5.13 dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 8.92 dB

FL proposal:

Capture the following observation for Rural 4GHz TDD NLOS O2I scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 4.23 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced for BS with 33dBm/MHz transmit power:

    - PUSCH for eMBB with frame structure DDDSU: 7.05 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 5.39 dB

    - PUSCH for VoIP with frame structure DDDSU: 1.89 dB

    - PUSCH for VoIP with frame structure DDDSUDDSUU:	 1.80 dB

    - PUCCH Format 1:	 1.23 dB

    - PUCCH Format 3 with 11bit payload: 2.53 dB

    - PUCCH Format 3 with 22bit payload: 5.49 dB

    - PRACH Format 0: 2.24 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH Format B4: 5.72 dB

    - Broadcast PDCCH(PDCCH of Msg.2) : 1.11 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 1.90 dB

    - PUCCH w/ HARQ-ACK for Msg.4: 1.47 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - Achievement of 3000m ISD is not easy because it requires enhancements for all the channels

    - Especially, PUSCH for eMBB with frame structure DDDSU requires huge amount of enhancements of 16.27 dB

FL proposal:

Capture the following observation for Rural 2.6GHz TDD NLOS O2I scenario

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 5.44 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 3.86 dB

    - PUCCH format 3 with 11bit payload: 0.50 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22bit payload: 1.60 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

FL proposal:

Capture the following observation for Rural 2GHz FDD NLOS O2I scenario

  - PRACH format B4 is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - Since the number of samples to derive the representative value is 4, further analysis is advisable for more accurate MIL estimation on this channel as necessity. It has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 2.85 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH with SIP invite: 2.09dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH format B4: 3.10dB

FL proposal:

Capture the following observation for Rural 700MHz FDD NLOS O2I scenario

  - For the service based requirement for VoIP that is defined by 139.2dB MCL, the following channels can be bottlenecks for the given scenario.

    - PUSCH for VoIP: 5.01 dB

    - PUSCH with SIP invite: 4.20 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 11bit payload: 1.45 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 22bit payload: 2.95 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH Format 1:	1.68 dB

    - PUCCH Format 3 with 11bit payload:  3.35 dB

    - PRACH Format B4: 6.51 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3 	4.49 dB

  - PUCCH format 3 with 22 bit payload is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - As the table of representative values (in annex) shows, the gap between the worst and the 2nd worst channel (PUSCH for eMBB) is relatively small, i.e. in the range of standard deviation. Thus more analysis is necessary for the accurate bottleneck channel identification. However, it has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 0.22 dB can be achieved at maximum.

  - In order to achieve 3000m ISD, the following channel(s) needs to be enhanced:

    - PUCCH Format 3 with 22 bit payload: 2.28dB

    - PRACH format B4: 4.48dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH w/ HARQ-ACK for Msg.4: 4.43dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - In order to achieve 4000m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB: 3.05 dB

    - PUSCH for VoIP: 1.22 dB

    - PUCCH Format 1: 1.01 dB

    - PUCCH Format 3 with 11bit payload: 2.58 dB

    - PUCCH Format 3 with 22bit payload: 7.11 dB

    - PRACH Format B4: 9.31 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 0.60 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH w/ HARQ-ACK for Msg.4: 9.26 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

FL proposal:

Capture the following observation for Rural 4GHz with long distance TDD LOS O2O scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL. However, due to the lack of the number of samples for each channel, the statistical analysis by relative difference between channels could not be performed.

  - In order to achieve 12km ISD, at least the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 3.15 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 2.42 dB

    - This does not mean that other channels do not require any enhancements. RAN1 did not perform any analysis for this point because of the lack of the number of samples for each channel for this scenario.

FL proposal

Capture the following observation in the TR

  - One source (ZTE, R1-2007741) evaluated the target performance, i.e. the 5th percentile geometry SINR value, based on system-level simulation. Compared to the baseline performance, i.e., required SNR based on link level simulation, the following is observed for FR1.

    - In Urban 4 GHz O2I scenario with ISD 500m, PUSCH eMBB, PUSCH VoIP, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 14.65 dB, 3.86 dB, 2.1 dB, 5.2 dB, 4.26 dB, 4.29 dB and 4.64 dB respectively.

    - In Rural 4 GHz O2I scenario with ISD 1732m, PUSCH eMBB, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.89 dB, 3.08 dB, 4.14 dB, 4.19 dB, 4.31 dB and 4.56 dB respectively.

    - In Rural 2.6 GHz O2I scenario with ISD 1732m, PUSCH eMBB,  Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits and  PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.11 dB, 3.35 dB, 4.11 dB, 4.3 dB, 4.28 dB and 4.56 dB respectively.

    - In Rural 700MHz O2O scenario with ISD 1732m, no channel requires coverage compensation.

    - In Rural with long distance 700MHz O2O scenario with ISD 12km, no channel requires coverage compensation.

FL proposal:

- Regarding the working assumption on the result of bottleneck identification by using absolute metrics, the working assumption is confirmed by applying the following table.

Note: the excelsheet of this table is available from the following link: 
https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

Agreements

Agreements in the GTW session on 28th October

Agreements:

- Representative values of the absolute values of [MCL, MIL and MPL] are used for bottleneck identification

  - Further down-selection one or more of MCL/MIL/MPL may be performed depending on the decision of target performance metric(s)

  - Companies can also report their individual observations of the bottleneck based on individual simulation results

  - How to use the respresentive values is FFS

  - A representative value is derived by taking the mean value (in dB domain) from companies’ evaluation results

    - Excluding the highest & the lowest values when the number of samples is more than 3

    - If the number of samples used to compute a representative value is less than 4 for each scenario, this representative value is not used for bottleneck identification

      - In this case, observations may still be drawn

Agreements in the GTW session on 3rd November

Agreements:

Rephrase the following terminologies, which is used in e.g. link budget template:

  - “PDCCH of Msg.2” refers to as “broadcast PDCCH (PDCCH of Msg.2)”

  - “PDCCH” refers to as “unicast PDCCH”

- Agreements:

Representative values are computed for the following channels or signals for FR1, and they are used to draw observation and to identify bottlenecks (if any) if the number of samples for each scenario is more than Y (Y=3)

  - PUSCH for eMBB (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for VoIP (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH with HARQ-ACK for Msg.4

Agreements:

The following scenarios are used for drawing observations and bottleneck identification for FR1

  - 1st priority

    - Urban 4GHz TDD

    - Urban 2.6GHz TDD

    - Rural 4GHz TDD NLOS O2I

    - Rural 700MHz FDD NLOS O2I

    - Rural 2GHz FDD NLOS O2I

    - Rural 2.6GHz TDD NLOS O2I

  - 2nd priority

    - Rural 700MHz with long distance FDD LOS O2O

    - Rural 4GHz with long distance TDD LOS O2O

  - Note: the difference between 1st priority and 2nd priority is as follows

    - RAN1 discussion will focus on 1st priority scenarios for drawing observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

    - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

- Agreements

No categorization by other simulation parameters (such as UE speed, antenna gain correction factors) will be introduced for FR1 for deriving representative values

  - The amount of available results for DL channels in FR1 4GHz scenario should be considered as given by the total number of available results for both 33 dBm/MHz and 24 dBm/MHz, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

  - In order to address the misalignment issue of the companies’ evaluation results due to no categorization and/or different simulation assumptions,

    - Number of samples and standard deviation is shown together with a representative value, and

    - A description on the potential fluctuation due to no categorization and/or different simulation assumptions can be added in the observation

The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification due to the lack of number of samples etc., can be used to make additional observations to be captured in the TR.

  - This discussion will be held in RAN1#103-e on a low priority basis.

Agreements:

  - For, Scenario dependent targets, e.g., ISD/MPL

      - The following formula is used to convert an ISD value to a target MPL value (to add the reference when capturing into TR):

        - For urban scenarios,

        - For rural scenarios,

        - For rural with long distance scenarios (working assumption)

Agreements:

- All the parameters/values/configurations related to FR1 modelling for which an agreement has not been reached among companies prior to RAN1 #103-e, will be henceforth treated according to the “reported by companies” principle. RAN1 will not spend further time during RAN1 #103-e on the resolution of these issues.

Agreements in the GTW session on 4th November

No agreements

Agreements in the GTW session on 10th November

Agreements:

  - For, Scenario dependent targets, e.g., ISD/MPL

    - For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

    - Target ISD values for each scenario are as follows:

        - Urban 4GHz TDD –400, 500m for observation and 400m for bottleneck identification

        - Urban 2.6GHz TDD –400, 500m for observation and 400m for bottleneck identification

        - Rural 4GHz TDD NLOS O2I – 1732 and 3000 m for observation and 1732m with 33dBm/MHz BS transmit power for bottleneck identification

        - Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 700MHz FDD NLOS O2I –3000m, 4000m for observation and 4000m for bottleneck identification

Agreements:

  - For Service dependent targets for VoIP

    - MCL of 139.2dB is used for Rural 700MHz scenario for drawing observation and can be used to identify bottlenecks (if applicable)

      - Captured the following table, which shows how the target value (i.e. 139.2dB) is derived, in Annex of the TR.

| Transmitter |  |
| --- | --- |
| (2a) Number of transmit chains modelled in LLS | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 3840000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: For FR1 downlink, (3b) should satisfy the following: For 4GHz frequency, 24 and 33 For 2.6 GHz frequency, 33 For 700MH and 2GHz frequency, 36 Note: For FR2 downlink, the following should be satisfied: 40 dBm for 100 MHz Urban scenario, 23 dBm for 100 MHz Indoor scenario. Note: no PSD constraint for uplink | 17.16 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. for uplink, (3a) = (3c) | 3840000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 |
| Receiver |  |
| (10a) Number of [receive TxRUs] Note: this row is void (empty) for downlink | 2 |
| (10b) Number of receive chains modelled in LLS | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 |
| (13) Receiver noise figure (dB) | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -98.2 |
| (19) Required SNR (dB) | 5 |
| (20) Receiver implementation margin (dB) | 2 |
| (21) H-ARQ gain (dB) or Process gain for UMTS Note: Only applicable if HARQ is not considered in LLS, | 25 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -116.2 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 139.2 |

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

Agreements in the GTW session on 12th November

- Agreements:

- Capture the following table (working assumption) showing the result of bottleneck identification by using absolute metrics

- Agreements:

- For FR1, the potential bottleneck channels identified by absolute metrics can be further filtered by using relative difference between channels in MIL

  - FFS details, including the possibility of applying the relative difference for a limited set of scenario(s)

Agreements in the GTW session on 13th November

Agreement

For Rural with long distance 4GHz TDD LOS O2O scenario, 12km is applied for observation and bottleneck identification

Agreement

The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - 1st priority

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

  - 2nd priority

    - PRACH format B4

    - PUSCH of Msg.3

    - PUCCH format 1

    - PUCCH format 3 with 11bit

    - PUCCH format 3 with 22bit

    - Broadcast PDCCH

Agreements at the end of the e-meeting

Agreement:

  - Confirm the Working Assumption on the pathloss formula for rural with long distance scenarios.

  - TR editor will take care of the detailed description regarding which model in the reference document is used for these formulae.

Agreement:

For Relative difference between channels

  - MIL is used to derive relative differential values.

  - Relative difference between channels is used to draw observation for the 1st and 2nd priority scenarios, and can be used to identify bottlenecks (if applicable)

  - For each channel, relative differential value is defined by the following formula for FR1

    - (MIL of the channel) – (MIL of the worst channel among the channels that have more than 3 samples)

Agreement:

Capture the following tables in Annex of the TR

  - Note: Even when the channel has less than 4 samples, it can be included in the table.

  - Note: The excelsheet for this table is found below:

  - https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

  - Urban 4GHz scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Urban 2.6GHz scenario

  - Rural 4GHz TDD NLOS O2I scenario

    - For BS with 33dBm/Hz Tx power

    - For BS with 24dBm/Hz Tx power

  - Rural 2.6GHz TDD NLOS O2I scenario

  - Rural 2GHz Rural 2GHz FDD NLOS O2I scenario

  - Rural 700MHz FDD NLOS O2I scenario

  - Rural 4GHz with long distance TDD NLOS O2O scenario

Agreement:

Capture the following observation for Urban 4GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 10.26 dB can be achieved at maximum.

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 8.12 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  7.09 dB

    - If low transmit power BS (i.e 24dBm/MHz) is assumed,

      - Broadcast PDCCH (PDCCH of Msg.2): 2.17dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 11.91 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU:  10.88 dB

    - PUSCH for VoIP with frame structure DDDSU: 0.03 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required.

    - PUSCH for VoIP with frame structure DDDSUDDSUU:  0.86 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22 bit payload: 2.62 dB

    - PRACH Format B4: 0.48dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUSCH of Msg.3: 1.11 dB

  - If low transmit power BS (i.e 24dBm/MHz) is assumed, the following DL channel(s) needs to be enhanced, additionally.

    - For 400m ISD

      - Broadcast PDCCH (PDCCH of Msg.2)

    - For 500m ISD

      - SSB: 2.85dB

      - Broadcast PDCCH (PDCCH of Msg.2): 5.95dB

      - PDSCH for eMBB with frame structure DDDSUDDSUU: 1.09 dB

        - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

FL note: On 16th Dec, it was point out that the agreement above includes typo, i.e. “For the observation of Urban 4GHz TDD scenario with 500m ISD, it is clarified that PUSCH of Msg.3 does not need to be enhanced.”.  This typo will be addressed during TP phase.

Agreement:

Capture the following observation for Urban 2.6GHz TDD scenario in the TR

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 10.00 dB can be achieved at maximum.

  - In order to achieve 400m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 5.13 dB

  - In order to achieve 500m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 8.92 dB

Agreement:

Capture the following observation for Rural 4GHz TDD NLOS O2I scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced for frame structure DDDSU, coverage enhancement of 4.23 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced for BS with 33dBm/MHz transmit power:

    - PUSCH for eMBB with frame structure DDDSU: 7.05 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 5.39 dB

    - PUSCH for VoIP with frame structure DDDSU: 1.89 dB

    - PUSCH for VoIP with frame structure DDDSUDDSUU:	 1.80 dB

    - PUCCH Format 1:	 1.23 dB

    - PUCCH Format 3 with 11bit payload: 2.53 dB

    - PUCCH Format 3 with 22bit payload: 5.49 dB

    - PRACH Format 0: 2.24 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH Format B4: 5.72 dB

    - Broadcast PDCCH(PDCCH of Msg.2) : 1.11 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 1.90 dB

    - PUCCH w/ HARQ-ACK for Msg.4: 1.47 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - Achievement of 3000m ISD is not easy because it requires enhancements for all the channels

    - Especially, PUSCH for eMBB with frame structure DDDSU requires huge amount of enhancements of 16.27 dB

Agreement:

Capture the following observation for Rural 2.6GHz TDD NLOS O2I scenario

  - PUSCH for eMBB with frame structure DDDDDDDSUU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - If only one channel can be enhanced, coverage enhancement of 5.44 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDDDDDSUU: 3.86 dB

    - PUCCH format 3 with 11bit payload: 0.50 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH format 3 with 22bit payload: 1.60 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Agreement:

Capture the following observation for Rural 2GHz FDD NLOS O2I scenario

  - PRACH format B4 is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - Since the number of samples to derive the representative value is 4, further analysis is advisable for more accurate MIL estimation on this channel as necessity. It has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 2.85 dB can be achieved at maximum.

  - In order to achieve 1732m ISD, the following channel(s) needs to be enhanced:

    - PUSCH with SIP invite: 2.09dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PRACH format B4: 3.10dB

Agreement:

Capture the following observation for Rural 700MHz FDD NLOS O2I scenario

  - For the service based requirement for VoIP that is defined by 139.2dB MCL, the following channels can be bottlenecks for the given scenario.

    - PUSCH for VoIP: 5.01 dB

    - PUSCH with SIP invite: 4.20 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 11bit payload: 1.45 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH for CSI with 22bit payload: 2.95 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH Format 1:	1.68 dB

    - PUCCH Format 3 with 11bit payload:  3.35 dB

    - PRACH Format B4: 6.51 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3 	4.49 dB

  - PUCCH format 3 with 22 bit payload is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario.

    - As the table of representative values (in annex) shows, the gap between the worst and the 2nd worst channel (PUSCH for eMBB) is relatively small, i.e. in the range of standard deviation. Thus more analysis is necessary for the accurate bottleneck channel identification. However, it has not been pursued in this study item.

    - If only one channel can be enhanced, coverage enhancement of 0.22 dB can be achieved at maximum.

  - In order to achieve 3000m ISD, the following channel(s) needs to be enhanced:

    - PUCCH Format 3 with 22 bit payload: 2.28dB

    - PRACH format B4: 4.48dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUCCH w/ HARQ-ACK for Msg.4: 4.43dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

  - In order to achieve 4000m ISD, the following channel(s) needs to be enhanced:

    - PUSCH for eMBB: 3.05 dB

    - PUSCH for VoIP: 1.22 dB

    - PUCCH Format 1: 1.01 dB

    - PUCCH Format 3 with 11bit payload: 2.58 dB

    - PUCCH Format 3 with 22bit payload: 7.11 dB

    - PRACH Format B4: 9.31 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

    - PUSCH of Msg.3: 0.60 dB

      - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required.

    - PUCCH w/ HARQ-ACK for Msg.4: 9.26 dB

      - However, due to the lack of the number of samples, the statistical correctness of this estimation has not been pursued in this study item.

Agreement:

Capture the following observation for Rural 4GHz with long distance TDD LOS O2O scenario in the TR

  - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL. However, due to the lack of the number of samples for each channel, the statistical analysis by relative difference between channels could not be performed.

  - In order to achieve 12km ISD, at least the following channel(s) needs to be enhanced:

    - PUSCH for eMBB with frame structure DDDSU: 3.15 dB

    - PUSCH for eMBB with frame structure DDDSUDDSUU: 2.42 dB

    - This does not mean that other channels do not require any enhancements. RAN1 did not perform any analysis for this point because of the lack of the number of samples for each channel for this scenario.

Agreement:

Capture the following observation in the TR

  - One source (ZTE, R1-2007741) evaluated the target performance, i.e. the 5th percentile geometry SINR value, based on system-level simulation. Compared to the baseline performance, i.e., required SNR based on link level simulation, the following is observed for FR1.

    - In Urban 4 GHz O2I scenario with ISD 500m, PUSCH eMBB, PUSCH VoIP, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 14.65 dB, 3.86 dB, 2.1 dB, 5.2 dB, 4.26 dB, 4.29 dB and 4.64 dB respectively.

    - In Rural 4 GHz O2I scenario with ISD 1732m, PUSCH eMBB, Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits, and PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.89 dB, 3.08 dB, 4.14 dB, 4.19 dB, 4.31 dB and 4.56 dB respectively.

    - In Rural 2.6 GHz O2I scenario with ISD 1732m, PUSCH eMBB,  Msg3, PRACH B4, PUCCH Format 1 with 2 bits, PUCCH Format 3 with 11 bits and  PUCCH Format 3 with 22 bits, requires coverage compensation, and the compensation gap is 5.11 dB, 3.35 dB, 4.11 dB, 4.3 dB, 4.28 dB and 4.56 dB respectively.

    - In Rural 700MHz O2O scenario with ISD 1732m, no channel requires coverage compensation.

    - In Rural with long distance 700MHz O2O scenario with ISD 12km, no channel requires coverage compensation.

Agreement:

- Regarding the working assumption on the result of bottleneck identification by using absolute metrics, the working assumption is confirmed by applying the following table.

Note: the excelsheet of this table is available from the following link: 
https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx

References

- R1-2007581 Evaluation on the baseline performance for FR1	Huawei, HiSilicon

- R1-2007678 Evaluation on NR coverage performance for FR1	vivo

- R1-2007741 Discussion on baseline coverage performance for FR1	ZTE

- R1-2007872 Baseline coverage performance for FR1	CATT

- R1-2007904 Baseline coverage performance for uplink in FR1	Indian Institute of Tech (H)

- R1-2007931 FR1 PUSCH Baseline Coverage Performance	Sierra Wireless, S.A.

- R1-2007952 On baseline coverage performance for FR1	Intel Corporation

- R1-2007993 Updated baseline performance for NR coverage enhancements for FR1	China Telecom

- R1-2008024 Discussion on the baseline performance in FR1	CMCC

- R1-2008089 Baseline coverage performance for FR1	Xiaomi

- R1-2008179 Baseline coverage performance using LLS for FR1	Samsung

- R1-2008269 Evaluation on NR coverage performance for FR1	OPPO

- R1-2008343 Link and System Evaluation of Coverage for FR1	Ericsson LM

- R1-2008377 Baseline coverage performance analysis in FR1	Panasonic Corporation

- R1-2008380 Target value for FR1 voice coverage enhancements	SoftBank Corp.

- R1-2008398 Link budget analysis for FR1	Sharp

- R1-2008478 Evaluation on FR1 coverage performance	Apple

- R1-2008481 FR1 baseline coverage performance using LLS	InterDigital, Inc.

- R1-2008515 Discussion on scenarios for FR1 baseline performance evaluation	MediaTek Inc.

- R1-2008557 Baseline coverage performance for FR1	NTT DOCOMO, INC.

- R1-2008624 Baseline FR1 coverage performance	Qualcomm Incorporated

- R1-2008701 Baseline coverage evaluation of UL and DL channels – FR1	Nokia, Nokia Shanghai Bell

- R1-2007683 Considerations on Parameters for Coverage Evaluation	vivo

- R1-2007746 Discussion on target performance for NR coverage enhancements	ZTE

- R1-2007877 Discussion on remaining issues for coverage enhancement	CATT

- R1-2007957 On simulation assumptions for NR coverage enhancement	Intel Corporation

- R1-2008274 Functionality of Coverage Enhancement and other WI	OPPO

- R1-2008422 Coverage Parameter Sensitivity and Network Enhancement	Ericsson

- R1-2008487 Discussion on simulation assumptions for VoIP	InterDigital, Inc.

- R1-2008629 Other coverage enhancement aspects	Qualcomm Incorporated

- R1-2008706 Evaluation assumptions for NR coverage enhancement evaluation	Nokia, Nokia Shanghai Bell

Annex 1 – Agreements at RAN1#101e

Update on 6/1: to check 6/2

Update from 6/4 GTW:

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

- Urban scenario: DL 10Mbps, UL 1Mbps

- Rural scenario: DL 1Mbps, UL 100kbps

- Rural with long distance scenario: DL 1Mbps, UL 100kbps, [30kbps] (optional)

Agreements:

- For VoIP performance evaluation based on link-level simulation for FR1.

- A packet size of [320] bits with 20ms data arriving interval is adopted.

- FFSTBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point.

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1.

- Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Step 2: Obtain the baseline performance based on required SINR and link budget template.

- Note: asepcts related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately

- FFS: The evaluation methodology based on system-level simulation is optional for FR1.

- Note: The simulation assumptions for SLS are up to companies’ reports.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. [CDL] |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Frequency hopping | w/ or w/o Intra-slot frequency hopping for PUSCH w/ frequency hopping for PUCCH is enabled. |

- FFS whether there are any additional simulation considerations for the extreme coverage scenarios (e.g., rural)

Update on 6/5:

Agreement:

- Down selection on the following options for the link budget template for FR1 in next meeting.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

  - FFS: The template provided by FL in Tdoc R1-2005005.

- Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

- Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

Agreement:

Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

- Option 1: Antenna array gain is included in the link budget template.

- FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs)

- FFS: For TDL channel model

- FFS: Values reflective of realistic implementation and network operation.

- Option 2: Antenna array gain is included in LLS.

- FFS: For CDL channel model

Agreement:

- For link level simulation, adopt the following table for PDSCH for FR1.

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS |
| Other parameters | FFS |

Agreements:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

Agreements:

- For link level simulation, the packet size of VoIP for FR2 is the same as FR1.

Agreements:

- For link level simulation, TBS of Msg3 for FR2 is the same as FR1.

Agreements:

- The evaluation methodology for FR2 is the same as FR1.

Agreements:

- The link budget template for FR2 is the same as FR1.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) Other frame structures can be reported by companies. |
| Subcarrier Space | 120kHz |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, (optional: 120km/h) for outdoor. |
| Occupied channel bandwidth for | 100MHz, [400MHz] |
| Frequency hopping for PUSCH | w/ or w/o frequency hopping |

Final summary in R1-2005004.

//Update on 6/7, post e-Meeting additional email approval

[101-e-Post-NR-Cov-Enh] Email discussion/approval focusing on remaining  evaluation assumptions till 6/17 – Jianchi (CT)

- Focusing on high priority proposals first, target 6/11 for early approvals

- Followed by medium priority/low priority proposals

Update on 6/11: check on 6/12 for potential agreements

Update on 6/12:

Agreements

- For link level simulation, adopt the following table for PUSCH for eMBB data or VoIP for FR1.

| Parameters | Values |
| --- | --- |
| BLER for PUSCH | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains for PUSCH | 1，2 (optional) |
| DMRS configuration for PUSCH | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform for PUSCH | DFT-s-OFDM, CP-OFDM (optional) |
| Repetitions for PUSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B |
| HARQ configuration for PUSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| Latency requirements for voice | 50ms/100ms |
| PUSCH duration | 14 OS |

Agreements

- For link level simulation, adopt the following table for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | FFS: number of DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of receive TxRUs for BS | TBD gNB architectures to study for TDL: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz [gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS (as starting point). FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation.] [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain, obtained from mapping antenna elements to TXRU, is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR2.

- Indoor: DL: 25Mbps, UL:5Mbps

- Urban: DL: 25Mbps, UL: 5Mbps

- Suburban: FFS: (DL: 1Mbps, UL: 50kbps)

Other proposals?

- # Number of receive TxRUs for BS – 6/15

- Others?

Update on 6/17

Regarding # Number of receive TxRUs for BS – see the update of the agreement above.

Agreements:

- For link level simulation, adopt the following table for SSB for FR1.

| Parameters | Values |
| --- | --- |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. |
| Other parameters | Reported by companies. |

- For link level simulation, adopt the following table for Msg.3 for FR1.

| Parameters | Values |
| --- | --- |
| Number of PRBs | 2 |
| Waveform | DFT-s-OFDM |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| PUSCH duration | 14 OS |
| Other parameters | Reported by companies. |

Other proposals 6/18

Update on 6/18:

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR1.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for SSB for FR1.

| Parameters | Values |
| --- | --- |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR1.

| Parameters | Values |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability FFS: 10% missed detection. |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for PDSCH of Msg.4 for FR1.

  - Reuse the following simulation assumption for PDSCH

    - Waveform, [PDSCH duration]

  - FFS: Payload size: [3000bits].

  - Other parameters: Reported by companies.

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 and PDSCH for FR1.

  - Reuse following simulation assumptions agreed for PUSCH.

    - Scenario and frequency, frame structure, SCS, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS.

  - The number of UE receive chains: is 2.

    - 4 for 4GHz/2.6GHz

    - 2 or 4 for 2GHz

    - 2 for 700MHz

  - For PDSCH, reuse DM-RS configuration, BLER, HARQ, Latency requirements for voice agreed for PUSCH.

    - Reuse DM-RS configuration agreed for PUSCH except that 3 DMRS symbols is used for Msg2.

- For link level simulation, for PRACH and Msg.3 for FR1.

  - Reuse following simulation assumptions agreed for PUSCH

    - Scenario and frequency, frame structure, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS and Number of UE transmit chains.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping agreed for PUSCH.

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report rBLER. w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| DMRS configuration | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH FFS: CP-OFDM for PUSCH |
| Repetitions for PUSCH/PDSCH | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B for PUSCH. |
| HARQ configuration for PUSCH/PDSCH | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH/PDSCH duration | 14 OS for PUSCH, 12 OS for PDSCH |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2 Note: Analog beamforming is assumed. |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| Channel model for link-level simulation | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |
| Latency requirements for voice | 50ms/100ms |
| PRBs/TBS/MCS for eMBB for PUSCH/PDSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30] PRBs for 5Mbps for PUSCH and full bandwidth for 25Mbps for PDSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH/PDSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PDSCH/PUSCH Optional: pi/2 BPSK for PUSCH |

Agreements:

- For link level simulation, adopt the following simulation assumption for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2), FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2) |

Agreements:

- For link level simulation, adopt the following table for PUCCH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI FFS: Format 0, 2 |
| BLER for PUCCH | The same as FR1 |
| Number of PRBs for PUCCH | The same as FR1 |
| Number of UE transmit chains for PUCCH | The same as FR1 |
| Number of repetitions for PUCCH | The same as FR1 |
| PUCCH duration | 14 OFDM symbols FFS: 4 OFDM symbols |
| DMRS configuration for PUCCH | FFS: [4] DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for PDCCH for FR2.

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48PRBs |
| Tx Diversity | Reported by companies |
| BLER for PDCCH | 1% BLER. FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreements:

- For link level simulation, adopt the following table for PRACH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format B4, (Optional: Format C2) |
| SCS | Reported by companies. |
| Performance metric | 0.1% false alarm, 1% miss-detection FFS: 10% missed detection. |
| Number of SSB beams | Reported by companies |
| Other parameters | Reported by companies. |

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 for FR2.

  - Reuse following simulation assumptions for PDSCH

    - Scenario and frequency, frame structure, SCS, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE Tx/Rx chains and UE antenna elements.

- For link level simulation, for PUCCH, PRACH and Msg.3 for FR2.

  - Reuse following simulation assumptions for PUSCH

    - Scenario and frequency, frame structure, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS, number of UE antenna elements for PUSCH.

  - For PRACH and Msg.3, reuse number of UE Tx chains for PUSCH.

  - For PUCCH, reuse SCS for PUSCH.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping for PUSCH.

Final summary in R1-2005192.

Annex 2 – Agreements at RAN1#102e

Agreements:

- TDL models are used to generate results in the link budget templates for FR1

  - This does not preclude companies from performing the link-level simulations using CDL

Agreements (for both FR1 & FR2):

- For the definition of antenna array gain, adopt option 1, i.e. Antenna array gain is included in the link budget template, where there are four antenna gain components

  - Note: the four components are illustrated below – the figure is for illustration purpose only

  - FFS which component(s) are NOT part of the definition of antenna array gain

Agreements:

- For TDL Option 1

  - Definition of MCL

    - Total transmit power - Receiver sensitivity + gNB antenna gain (component 2)

  - Definition of MIL

    - Total transmit power - Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain

  - Definition of MPL

    - Further discussion offline the definition using below as a starting point:

      - Total transmit power - Receiver sensitivity + gNB antenna array gain (component 2+3+4 for TDL option 1) + UE antenna gain - (8) Cable, connector, combiner, body losses (Tx side) - (20) Receiver implementation margin + (21a/b) H-ARQ gain - (25a/b) Shadow fading margin + (26) BS selection/macro-diversity gain - (27) Penetration margin + (28) Other gains – (12) Cable, connector, combiner, body losses (Rx side)

  - Note: whether/how to use the above definitions is to be discussed

Update on 8/20: to check on 8/21

Update on 8/21: to check on 8/24

Update from GTW on 8/24

Agreements:

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters

  - [For LLS based methodology, ]coverage bottleneck(s) identification is performed using at least [MCL and] MIL.

  - [MCL values can also be considered to compare channels with similar antenna (and antenna array) gain]

Agreements:

- MPL can be used as supplemental information for coverage bottleneck(s) identification

- The results based on MPL are to be captured in TR

  - Note: this is uself to show the achievable ISD.

- The definition of MPL shall be determined in RAN1

- RAN1 will not further discuss on specific values for the parameters related to MPL

  - IMT-2020 values are as a starting point, but:

    - companies may use other values, and

    - for the parameters that companies think IMT-2020 self-evaluation does not clearly define the values for some scenarios, it is up to companies to report

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

Agreements:

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters

  - For LLS based methodology, coverage bottleneck(s) identification is performed using at least MIL or MCL (assuming the set of simuation assumptions)

    - Even when SLS is used to obtain some components of MIL or MCL, it is categorized as LLS based methodology.

    - MCL values can also be used to identify the coverage bottleneck(s) when applicable

      - “applicable” above means the following situation:

        - [comparing channels with similar antenna (and antenna array) gain, and/or

        - the simulation results with MIL from companies are diverse, and the comparison with MIL is not easy]

Update on 8/27:

Agreements:

- for SIP invite message

  - Payload of 1500 bytes can be a starting point.

  - The assumptions (TB size, time period etc.) are reported by companies.

  - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation.

    - In addition, 1 second time period can also be considered.

Agreements:

For PDSCH, other parameters are reported by companies.

Agreements:

- Confirm the working assumption on DMRS configuration for PUSCH:

  - For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data.

- The number of DMRS symbols is reported by companies

Agreements:

- Update the description on Repetitions for PUSCH as follows:

  - For VoIP, w/ type A repetition. (optional for type B repetition)
The actual number of repetitions is reported by companies.
FFS: Repetition type B

Agreements:

- Update the row for BLER for PUCCH as follows:

  - FFS: BLER for CSI (10% or 1%, (optional for 10%) )

Agreements:

| Number of TxRUs for BS | gNB modelling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS. FFS: - Optional Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation. - Companies can report if and how correlation is modelled |
| --- | --- |

Agreements:

- Remove the whole bullets about gNB architectures to study for CDL and gNB modelling in LLS for CDL

- Note: if CDL is used for link level simulation for a certain purpose, the assumption for the number of TxRUs for BS is reported by companies, which implies that the assumption will be captured in the TR.

Agreements:

- The same PDSCH duration as PDSCH is used for Msg.4 PDSCH (i.e. remove the square bracket)

  - Note: this does not preclude Msg4 with retransmission as a baseline.

Agreements:

- Update the BLER for PDCCH as follows:

| BLER for PDCCH | 1% BLER FFS: (optional for 10% BLER) |
| --- | --- |

Agreements:

- The agreement at RAN1#101-e remains: the simulation assumptions for SLS are up to companies’ reports

- The target performance of SLS based methodology, it is recommended to refer the agreements for LLS based methodology as much as possible.

- Note: these proposals are not necessary to be captured in the chairman’s note.

Update from 8/28 GTW

Agreements:

Update the agreements as follows:

- For VoIP performance evaluation based on link-level simulation for FR1

A packet size of [320bits] with 20ms data arriving interval is adopted, which component is as follows:

|  | Size (bits) |
| --- | --- |
| Payload | 256 |
| CRC | 16 (TBS size lower than 3824 bits) |
| MAC | 16 (with 12 bits SN size) |
| RLC | 8 (with 6 bits SN size) |
| PDCP | 16 |
| RTP/UDP/IP | 24 (w RoHC) |

The following packet component for AMR-WB 12.65 (kbit/s) is optionally adopted.

|  | Size (bits) |
| --- | --- |
| Payload | 264 |
| CRC | 16 (TBS size lower than 3824 bits) |
| MAC | 16 (with 12 bits SN size) |
| RLC | 8 (with 6 bits SN size) |
| PDCP | 16 |
| RTP/UDP/IP | 32 (w RoHC) |

[A packet size of 160 bits with 20ms data arriving interval is optionally adopted for rural scenario with long distance]

If applicable, companies report TB size assumed in evaluation

Agreements:

- For the evualation, it is assumed that Msg. 4 PDSCH payload size is 1040 bits.

Agreements:

- For receiver interference density

  - Up to each company to report for all scenarios as baseline

    - E.g. obtained by SLS, the ones for ITU self-evulation, etc.

Agreements:

Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows:

- For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below)

  - The gain of antenna gain component 1 is included in LLS results

  - The gain of antenna gain component 2 is included in link budget template

    - The gain is expressed by 10 * log 10( N/k ) - 1

    - For TDL option 2 & CDL, the gain is 0 dB

  - The gain of antenna gain component 3 is included in link budget template

  - The gain of antenna gain component 4 is included in link budget template

    - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2

    - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4)

    - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11)

    - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated.

Agreements:

- Define PSD for DL Tx power, which is depend on deployment scenario

  - For 4GHz frequency,

    - For rural with long distance scenario, PSD is 24 and 33 dBm/MHz

    - For rural scenario, PSD is 24 and 33 dBm/MHz

    - For urban scenario, PSD is 24 and 33 dBm/MHz

  - For 2.6 GHz frequency,

    - For rural with long distance scenario, PSD is 33 dBm/MHz

    - For rural scenario, PSD is 33 dBm/MHz

    - For urban scenario, PSD is 33 dBm/MHz

  - For 700MHz, 2GHz frequency

    - For rural with long distance scenario, PSD is 36 dBm/MHz

    - For rural scenario, PSD is 36 dBm/MHz

    - For urban scenario, PSD is 36 dBm/MHz

- Modify the description of row(s) of link budget template:

  - Keep the meaning of Total transmit power (row (3) ) and adding a new row (3 bis):

    - (3bis) means the transmit power for occupied channel bandwidth for control channel (17a) or data channel (17b)

- Companies are requested to set appropriate values for parameters, which is used to determine total transmit power ( row (3) and/or (3bis) ), to satisfy the PSD value above

- Note: RAN1 will further check the consistency of the definition of row(s) in link budget table when the IMT-2020 based link budget tale is updated

Agreements:

For FR1 and FR2:

- Further clarify the Definition of MCL for downlink

  - Total transmit power – Receiver sensitivity + gNB antenna gain (component 2), where

    - Total transmit power corresponds to row No.(3) + {(6) or -(7)} (for control & data channels)

    - Receiver sensitivity corresponds to row No.(22a/22b)

- Further clarify the Definition of MIL for downlink

  - Total transmit power – Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain, where

    - Total transmit power + gNB antenna gain (component 2 + 3 + 4) corresponds to row No.(9a/9b), i.e.

      - (3) + (4) + (5) + (6) – (8) for control channel

      - (3) + (4) + (5) – (7) – (8) for data channel

      - Note: the derivation of (9a/9b) will be modified depending on the discussion on antenna gain & antenna gain correction

    - Receiver sensitivity corresponds to row No.(22a/22b)

    - (Working assumption for FR2) UE antenna gain corresponds to row No.(11)+No(11bis)

- Note: further refinement/definition of (3) and/or (22a/22b) can be discussed when link budget table is updated.

Agreements:

Definition of MPL for TDL option 1

- MPL = MIL + [(21a/b) H-ARQ gain] – [ (25a/b) Shadow fading margin – (27) Penetration margin ] + [(26) BS selection/macro-diversity gain ] + [(28) Other gains] – [(12) Cable, connector, combiner, body losses (Rx side) ]

- Note1: (8) is not necessary because it is included in the definition of MIL

- Note2: (20) is not necessary because it is included in receiver sensitivity, which is used to derive MIL

Update on 8/28:

Agreements:

·         As for the agreement on antenna gain and antenna gain components including antenna gain correction factors, Table A and Table B are defined as below

Table A. antenna gain components for TDL option 1

Table B. antenna gain components for TDL option 2 and CDL

Agreements:

- Latency requirements assumed in VoIP evaluation for TDD and FDD are reported by companies

Agreements:

- For link level simulations in FR2, only PUCCH format 1 and format 3 are considered for baseline performance evaluation.

- For link level simulations in FR2, only PUCCH duration of 14 OFDM symbols is considered for baseline performance evaluation.

- For link level simulations in FR2, consider 4 DMRS symbol for PUCCH Format 3.

- Consider only one panel at the UE in link budget in FR2.

- For link budget calculation in FR2, downlink transmit power is scaled by the occupied bandwidth. The following downlink transmit power vs occupied bandwidth values are considered as baseline for the calculations:

  - 40 dBm for 100 MHz Urban scenario,

  - 23 dBm for 100 MHz Indoor scenario.

- For link budget calculation in FR2, an uplink transmit power of 23dBm is considered for baseline performance evaluations. Other values can be reported by companies.

- Confirm the target throughput values of the REL-17 SID for the suburban scenario:

  - DL: 1 Mbps, UL: 50 kbps

- Study performance of PUSCH in FR2 only for DFT-s-OFDM.

- For link level simulations, only 1% BLER should be considered for baseline performance evaluation of PDDCH in FR2.

- For link level simulations in FR2, only PUSCH repetition type A is considered for baseline performance evaluation.

  - Note: companies are not precluded to report results for repetition type B.

- Suburban scenario is deprioritized for NR coverage enhancement SI.

- Baseline performance evaluation of msg1 transmission is studied for 1% missed detection probability in FR2.

- Only 1% BLER target should be considered for baseline performance evaluation of PUCCH in FR2, regardless of whether UCI includes CSI feedback or not.

- Simulation assumptions for SLS in FR2 are up to companies’ reports, i.e., no more clarification is needed, as per agreement during RAN1#101-e.

Annex 3 – Agreements at post-email discussion of RAN1#102e

Agreement:

- Antenna array gain at a UE for FR1 and FR2 is clarified as follows:

  - The meaning of k, N and M:

    - k is the number of Tx/Rx chains, e.g., number of SRS/CSI-RS ports to be simulated in LLS.

    - M is the number of antenna elements used both for transmission and reception, i.e., M2 xpol antenna elements.

    - A formal definition of N is not necessary for UE antenna array gain modeling.

  - The values for k and the relationship between k and M are clarified as follows:

    - For FR1, k = M is assumed for the simulations, and

      - k=1 for Tx (optional k = 2)

      - k∈{2,4} for Rx

    - For FR2, there are two possibilities for simulations:

      - k∈{1,2}; for Tx and k=2 for Rx; or

      - k=M.

  - Antenna array gain in transmission/reception to input in link budget template is given by

    - Antenna Element Gain+10*log10Mk-Δ3, where

      - Δ3(≥0 dB) is a correction factor to account for various non-idealities impacting the actual antenna array gain, if any

        - For FR1, Δ3=0.

        - For FR2, 3 is channel procedure/dependent, and reported by companies.

- The values for antenna element gain:

  - 0 dBi for FR1

  - 5 dBi for FR2

Agreement:

- The working assumption for FR2 is updated as follows:

  - UE receive antenna gain corresponds to row is given by row No.(11) + row No. (11bis) - Δ3

- UE transmit antenna gain is given by row No. (4) + row No. (5) - Δ3

Agreement

- The agreement on the definition of MIL for downlink is updated by adding Rx loss as follows:

  - Total transmit power – Receiver sensitivity – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain, where

    - Rx loss corresponds to row No. (12)

- MPL = MIL – (25a/b) Shadow fading margin + (26) BS selection/macro-diversity gain – (27) Penetration margin + (28) Other gains [– (12) Cable, connector, combiner, body losses (Rx side) ]

- It is confirmed that H-ARQ gain is included in sensitivity

  - H-ARQ gain should be included in LLS. In this case, “(21a/b) H-ARQ gain” is set to zero

  - If not, “(21a/b) H-ARQ gain” can be used for companies report

- Note: as per the former agreement, the values for rows (25a/b) (26) (27) (28) and (12) are left to companies’ report, which includes the values for IMT-2020 self evaluation and/or using 0 dB

- Note:  (12) Cable, connector, combiner, body losses (Rx side) is not included in MCL, but included in MIL and MPL

- The definition of MCL, MIL and MPL for TDL Option 2 & CDL is the same as that for TDL option 1

- Note: The agreements on MIL, MCL and MPL definition is used to show which components of link budget template are included / not included. The sophistication of MIL, MCL and MPL formula will be discussed under [102-e-Post-NR-CovEnh-02] email discussion by using draft link budget template prepared by the FL.

- Note: Companies are encouraged to further check the values for (12) Rx losses proposed by a company, in addition to the values used for IMT-2020 self-evaluation

  - feeder loss at gNB (1dB for 700MHz, 0dB for 4GHz with AAS)

  - 0dB for the loss at UE

proposal 1:

- Agree the following link budget template

Transmitter

(1) Number of transmit antenna elements.

(2) Number of [(transmit TxRUs) or (modelled transmit chains)] Note: this row is void (left empty) for uplink

(2a) Number of transmit chains modelled in LLS

(3) Total transmit power (dBm) Note: total transmit power for system bandwidth

(3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz)

(3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: For FR1 downlink, (3b) should satisfy the following: For 4GHz frequency, 24 and 33 For 2.6 GHz frequency, 33 For 700MH and 2GHz frequency, 36 Note: For FR2 downlink, the following should be satisfied: 40 dBm for 100 MHz Urban scenario, 23 dBm for 100 MHz Indoor scenario. Note: no PSD constraint for uplink

(3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c)

(3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm)

(4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB)

(4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink

(4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB)

(4c) Gain of antenna element (dBi)

(5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink

(5a) Antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink

(5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink

(8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink)

(9) EIRP = (3bis) + (4) + (5) – (8) dBm

Receiver

(10) Number of receive antenna elements

(10a) Number of [ (receive TxRUs) or (modelled receive chains)] Note: this row is void (empty) for downlink

(10b) Number of receive chains modelled in LLS

(11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB)

(11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink

(11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB)

(11c) Gain of antenna element (dBi)

(11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink

(11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for downlink

(11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink

(12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink)

(13) Receiver noise figure (dB)

(14) Thermal noise density (dBm/Hz)

(15) Receiver interference density (dBm/Hz)

(16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz)

(18) Effective noise power = (16) + 10 log ((3c)) (dBm)

(19) Required SNR (dB)

(20) Receiver implementation margin (dB)

(21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS

(22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm)

(22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB)

(23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12)

Calculation of available pathloss

(25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB)

(26) BS selection/macro-diversity gain (dB)

(27) Penetration margin (dB)

(28) Other gains (dB) (if any please specify)

(29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB)

Range/coverage efficiency calculation

(30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m)

proposal 2:

- The values for following parameters are provided together with the link budget template

  - The details how to provide the values (i.e. by introducing rows in the same/different tab, by different excel file, by different tabs, etc. ) is up to rapporteur and feature leads.

Scenarios

Carrier frequency (GHz)

BS antenna heights (m)

UT antenna heights (m)

Cell area reliability (%)

Lognormal shadow fading std deviation (dB)

Pathloss model (select from LoS or NLoS)

UE speed (km/h)

Channel for evaluation

UL-DL configuration for TDD

Subcarrier Spacing

Channel model for link level simulation

Frequency hopping

Number of PRBs, TBS and MCS

BWP size

DMRS configuration

Waveform

Repetition

HARQ configuration

Latency requirements for voice

PUCCH format type

Tx Diversity

Target error rate (BLER, miss detection, false alarm, etc.)

PRACH format

Number of SSB

Correlation for TxRU at BS

Description on how the value in antenna gain correction factor in (4b) is derived

Description on how the value in antenna gain correction factor in (5b) is derived

Description on how the value in antenna gain correction factor in (11b) is derived

Description on how the value in antenna gain correction factor in (11bis-b) is derived

Description on how the value in (8) is derived

Description on how the value in (12) is derived

Other parameters

proposal 3:

- For (13) Receiver noise figure, the following values are adopted

  - For FR1: 7dB for UE and 5dB for BS

  - For FR2: 10dB for UE and 7dB for BS

- For other parameters/values with FFS, square bracket or no agreement, interested companies are encouraged to continue their assessment aiming at the final resolution at RAN1#103-e.

  - Detailed information can be find in the attached excel spreadsheet, i.e. link-budget-template-v013

  - Note: link-budget-template-v013 is found from the following link:
https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_102-e/Inbox/drafts/8.8.1.1/post_meeting/102-e-Post-NR-CovEnh-02/1-link_budget_template/fine_tuning/link-budget-template-v013.xlsx