3GPP TSG RAN WG1 Meeting #103-e	R1-20xxxxx

E-Meeting, October 26 – November 13, 2020

Agenda Item:	5

Source:	Moderator (Huawei)

Title:	Summary on [103-e-AI5-LS-01] on Incomplete LTE Physical Layer Capabilities

Document for:	Discussion and Decision

Introduction

In RAN1#103-e, an LS from RAN2 was received on incomplete LTE physical layer capabilities [1].

This documents provides the proposals and summary of discussions of the following second phase email discussion regarding the reply to the RAN2 LS.

[103-e-AI5-LS-01] For LS in R1-2007518, a reply LS may be necessary – email discussion/approval till 10/29 (to be handled under AI5) – Huawei (name TBD)

Discussion

### Issue 1: On FDD/TDD differentiation of some capabilities.

- RAN1 is asked to provide feedback regarding the FDD/TDD differentiation as listed in the table in [1]:

To provide feedback on whether RAN1 has any concern with updating FDD/TDD differentiation for the listed physical layer capabilities.

- In [2], the following feedback is proposed:

RAN1 has no concern on the intended changes.

Please input your comments to the proposed feedback:

| Companies | Comments |
| --- | --- |

### Issue 2: On value ranges of nMaxResource.

- RAN1 is asked to provide feedback regarding the value ranges for nMaxResource in both nzp-CSI-RS-AperiodicInfo-r14 and nzp-CSI-RS-PeriodicInfo-r14 [1]:

To provide feedback on the correct value range to use for nMaxResource in both nzp-CSI-RS-AperiodicInfo-r14 and nzp-CSI-RS-PeriodicInfo-r14.

MIMO-UE-ParametersPerTM-v1430 ::=		SEQUENCE {

nzp-CSI-RS-AperiodicInfo-r14			SEQUENCE {

nMaxProc-r14							INTEGER(5..32),

nMaxResource-r14						ENUMERATED {ffs1, ffs2, ffs3, ffs4}

}	OPTIONAL,

nzp-CSI-RS-PeriodicInfo-r14				SEQUENCE {

nMaxResource-r14						ENUMERATED {ffs1, ffs2, ffs3, ffs4}

}	OPTIONAL,

- In [2], the following feedback is proposed:

The nMaxResource-r14 in both nzp-CSI-RS-AperiodicInfo-r14 and nzp-CSI-RS-PeriodicInfo-r14 can be one value of {1, 2, 4, 8}.

Please input your comments on the question and proposed feedback:

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | Regarding the Rel-14 capabilities related to MIMO that have not been captured in 36.306, propose to capture the following into the LS: In addition, RAN1 noticed that the following Rel-14 capabilities have not been captured in TS 36.306. Capabilities; Description. nzp-CSI-RS-AperiodicInfo-r14; Indicates the support of aperiodic NZP CSI-RS transmission, separately per transmission mode (TM9, TM10). nMaxProc indicates the maximal number of updated CSI process for aperiodic NZP CSI-RS. nMaxResource indicates the maximum number of CSI-RS resources which can be activated by MAC CE for aperiodic NZP CSI-RS.. nzp-CSI-RS-PeriodicInfo-r14; Indicates the support of periodic NZP CSI-RS transmission, separately per transmission mode (TM9, TM10). nMaxResource indicates the maximum number of CSI-RS resources which can be activated by MAC CE for periodic NZP CSI-RS.. Action to RAN2: RAN1 kindly ask RAN2 to capture the listed capabilities into TS 36.306. |

Summary

To be added.

- References

- R1-2007518, “LS on Incomplete LTE Physical Layer Capabilities”, Source: RAN2, To: RAN1, RAN1#103-e, 2020.

- R1-2008342, “[DRAFT] Reply LS on Incomplete LTE Physical Layer Capabilities”, Huawei, RAN1#103-e, 2020.