# 3GPP TSG RAN WG1 #103-e															R1-2008523

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | DRAFT | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power sharing for LTE DAPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm Incorporated) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#102-e, the following was agreed regarding power sharing for inter-band DAPS: Agreements: - UL power sharing is introduced for inter-band DAPS. - The following optional UE capabilities are introduced to indicate support of power sharing (per band combination): - Support of power sharing for sync DAPS. - Support of power sharing for async DAPS. - NOTE: These capabilities are separate from the capability indicating DAPS support (i.e., a UE may indicate support of DAPS but no power sharing in a band combination) - An RRC configuration parameter is introduced to configure DAPS power sharing with the same contents as PowerCoordinationInfo-r12 (up to RAN2 whether to reuse PowerCoordinationInfo-r12 or define a new IE) - p-MeNB-r12 INTEGER (1..16): power sharing parameter for target eNB. - p-SeNB-r12 INTEGER (1..16): power sharing parameter for source eNB. - powerControlMode-r12: Indicates the power sharing mode. - NOTE: Support for UL cancellation is not introduced in Rel-16 - NOTE: Behavior of UE’s not supporting power sharing is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implement the agreement from RAN1#102-e - Specify that if the UE does not support power sharing, or for intra-band DAPS, the UE does not expect channels in source and target eNB to overlap in time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Power sharing is not supported for DAPS. - UE behavior for intra-band DAPS and when power sharing is not supported is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.4a(new)	Power allocation for dual active protocol stack

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG and a source MCG in different bands, and the UE is configured with DAPS-PowerCoordinationInfo, the UE shall apply the procedures described in clause 5.1.4 with the following modifications

- Consider the target MCG as the MCG and the source MCG as the SCG.

- Replace p-MeNB and p-SeNB by p-DAPS-Target and p-DAPS-Source, respectively.

- Replace “(a)synchronous dual connectivity” by “(a)synchronous DAPS”.

- “Dual connectivity power control mode” is replaced by “DAPS power control mode”, and is given by higher layer parameter uplinkPowerSharingDAPS-Mode”.

- The UE assumes only Pcell exists in each MCG.

If UE does not indicate a capability for power sharing between source and target MCGs in DAPS handover, or if a UE is configured with a target MCG and a source MCG in the same band, the UE does not expect the transmissions on the target and source cell to overlap in time domain.