3GPP TSG-RAN WG1 Meeting #103-e											R1-200xxxx

e-Meeting, Oct 26th – Nov 13th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			FL summary on the maintenance of 2-step RACH

Document for:	Discussion

# Introduction

This document contains the summary of issues related to the maintenance of Rel-16 2-step RACH WI in RAN1#103-e meeting.

# Maintenance issues

The following 5 issues are identified based on the submitted contributions in RAN1#103-e.

| Issue # | Description | Related TDoc # |
| --- | --- | --- |
| 1 | Capture a missing agreement in 38.213 on the MsgA configuration for unlicensed band | R1-2007963, TP#1 |
| 2 | Editorial change on the “HARQ Feedback Timing Indicator” in 38.213 | R1-2008418, TP#1 |
| 3 | Correction on the mistaken implementation of a previously agreed TP in 38.211 | R1-2008785, TP#1 |
| 4 | Correction on the PUSCH occasion validation rule in 38.213 | R1-2008785, TP#2 |
| 5 | Alignment on the RRC parameter names in RAN1 specs | R1-2008660, TP#1~4 |

FL comments:

Regarding #Issue 1, although the previous agreement was made in 2-step RACH WI, it was still been discussing in the NR-U session (7.2.2.1.1) in the last meeting, so it is preferably handled in NR-U session.

#Issue 5 has been covered by Editors’ draft CRs, i.e. R1-2007794 for 38.211, R1-2008124 for 38.213 and R1-2008292 for 38.214.

#Issue 2,3,4 can be considered for email discussions.

# Summary

The following Email discussion is proposed (will be updated based on companies’ feedback, if any).

Proposed Email thread #1:

Editorial and corrections including the following issues:

- TP#1 in R1-2008418 (editorial)

- TP#1 in R1-2008785 (correction)

- TP#2 in R1-2008785 (correction/clarification)

Any comments?

| Company | Comment |
| --- | --- |

# References

- R1-2007963	Text proposal on the MsgA configuration in unlicensed band	ZTE, Sanechips

- R1-2008418	Maintenance of Two step RACH	Ericsson

- R1-2008660	Miscellaneous corrections for 2-step RACH	vivo

- R1-2008785	Remaining issues for 2-step RACH	Huawei, HiSilicon

- R1-2007794	Alignment of RRC parameter names	Ericsson

- R1-2008124	Alignment CR for TS 38.213	Samsung

- R1-2008292	Alignment of RRC parameter names for 38.214	Nokia, Nokia Shanghai Bell

# Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2007963, ZTE | Proposal 1: - Adopt the following TP#1 to TS38.213 Reasons for change To capture the agreement in the specification Summary of changes All msgA PUSCH occasions and the associated msgA RACH occasions are confined within a single RB set Specs/Sections impacted TS 38.213, Section 8.1A Consequences if not approved: MsgA PRACH or PUSCH may across multiple RB sets ================ Start of TP1 for TS 38.213 ======================== 8.1A PUSCH for Type-2 random access procedure *** Unchanged text omitted *** A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrofMsgA-PO-FDM. For operation with shared spectrum channel access, all PUSCH occasions and the associated RACH occasions are confined within a single RB set. *** Unchanged text omitted *** ==================== End of TP1 ===============================. |
| R1-2008418, Ericsson | To avoid inconsistency with the MAC specification, use “HARQ Feedback Timing Indicator” in 38.213 section 8.2A for indication of the PUCCH timing for MsgB HARQ feedback, according to text proposal TP1. --------------------------start of TP1 for 38.213 section 8.2A--------------------------------- -----------------------------unchanged text omitted------------------------------------------ ## 8.2A Random access response - Type-2 random access procedure If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer - an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or - transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where - a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon - a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback HARQ Feedback tTiming iIndicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆, where n is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214] - the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214] - for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR - the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission ---------------------------------unchanged text omitted-------------------------------------- ---------------------------------------end of TP1------------------------------------------------- |
| R1-2008660, vivo | Proposal 1: Adopt the text proposal #1 for 38.213. -----------------start of TP #1 for 38.213------------------------- #### 7.1.1 UE behaviour <Unchanged part omitted> j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by preambleReceivedTargetPower and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c ----------------end------------------------- -----------------start of TP #1 for 38.213------------------------- #### 7.1.1 UE behaviour <Unchanged part omitted> j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by preambleReceivedTargetPower and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c ----------------end------------------------- Proposal 2: Adopt the text proposal #2 for 38.211. -----------------start of TP #2 for 38.211------------------------- #### 6.3.1.1 Scrambling <Unchanged part omitted> cinit=nRNTI∙216+nRAPID∙210+nIDfor msgA on PUSCHnRNTI∙215+nIDotherwise where - equals the higher-layer parameter dataScramblingIdentityPUSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, SP-CSI-RNTI or CS-RNTI, and the transmission is not scheduled using DCI format 0_0 in a common search space; - nID∈0,1,…,1023 equals the higher-layer parameter msgA-DataScramblingIndex if configured and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213]; - otherwise ----------------end------------------------- -----------------start of TP #2 for 38.211------------------------- #### 6.3.1.1 Scrambling <Unchanged part omitted> cinit=nRNTI∙216+nRAPID∙210+nIDfor msgA on PUSCHnRNTI∙215+nIDotherwise where - equals the higher-layer parameter dataScramblingIdentityPUSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, SP-CSI-RNTI or CS-RNTI, and the transmission is not scheduled using DCI format 0_0 in a common search space; - nID∈0,1,…,1023 equals the higher-layer parameter msgA-DataScramblingIndex if configured and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213]; - otherwise ----------------end------------------------- Proposal 3: Adopt the text proposal #3 for 38.211. -----------------start of TP #3 for 38.211------------------------- #### 6.3.3.2 Mapping to physical resources <Unchanged part omitted> Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610 if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured. ----------------end------------------------- -----------------start of TP #3 for 38.211------------------------- #### 6.3.3.2 Mapping to physical resources <Unchanged part omitted> Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610 if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex if configured. ----------------end------------------------- Proposal 4: Adopt the text proposal #4 for 38.214. -----------------start of TP #4 for 38.214------------------------- #### 6.1.4.1 Modulation order and target code rate determination <Unchanged part omitted> - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. ----------------end------------------------- -----------------start of TP #4 for 38.214------------------------- #### 6.1.4.1 Modulation order and target code rate determination <Unchanged part omitted> - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. ----------------end------------------------- |
| R1-2008785, Huawei | ## Text proposal #1 for TS 38.211: Reasons for change To capture the previous decision into the TS 38.211 Summary of changes Implement the above update Specs/Sections impacted TS 38.211, Section 6.4.1.1.1.1 ---------------------Text proposal #1 starts for TS 38.211 ------------------------ ## 6.4 Physical signals ### 6.4.1 Reference signals #### 6.4.1.1 Demodulation reference signal for PUSCH ##### 6.4.1.1.1 Sequence generation 6.4.1.1.1.1 Sequence generation when transform precoding is disabled <Unchanged Text Omitted> The quantity nSCID∈0,1 is - indicated by the DM-RS initialization field, if present, either in the DCI associated with the PUSCH transmission if DCI format 0_1 or 0_2, in [4, TS 38.212] is used; - indicated by the higher layer parameter dmrs-SeqInitialization, if present, for a Type 1 PUSCH transmission with a configured grant; - determined by the mapping between preamble(s) and a PUSCH occasion and the associated DMRS resource for a PUSCH transmission of Type-2 random access process in [5, TS 38.213]; - otherwise nSCID=0. <Unchanged Text Omitted> ---------------------- Text proposal #4 ends for TS 38.211 -------------------------. ## Text proposal #2 for TS 38.213: Reasons for change To correct the PUSCH occasion validation rule Summary of changes Implement the above update Specs/Sections impacted TS 38.213, Section 6.4.1.1.1.1 ----------------------Text proposal #2 starts for TS 38.213 ----------------------- ## 8.1A PUSCH for Type-2 random access procedure <Unchanged Text Omitted> A PUSCH occasion is valid if it does not overlap in time and frequency with any valid contention based PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon <Unchanged Text Omitted> ---------------------- Text proposal #5 ends for TS 38.213 --------------------------. |