3GPP TSG RAN WG1 #103-e								R1-20xxxxx

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.3

Source:	Moderator (AT&T)

Title:	Feature Lead Summary of [103-e-NR-IAB-01] Corrections for DCI Format 2_5 monitoring

Document for:	Discussion/Approval

Introduction

This contribution provides a summary of the following email discussion:

[103-e-NR-IAB-01] Corrections for DCI Format 2_5 monitoring – Thomas (AT&T)

-       Harmonize proposals in R1-2008328, R1-2008409, R1-2008742 to approve a single CR for 38.213.

-       Discussion and decision by 10/29, TPs by 11/4

Corrections for DCI Format 2_5 monitoring

Source: R1-2008328, R1-2008409, R1-2008742

Background: It is currently not captured in 38.213 that an IAB-MT monitors PDCCH candidates in Type-3-PDCCH CSS (all cell types) and USS sets for DCI formats with CRC scrambled by AI-RNTI in addition to the candidates/formats already captured in Section 10.1 of 38.213. This was previously discussed in RAN1#102-e but no CR was approved. Three different proposals are put forward in contributions (R1-2008328, R1-2008409, R1-2008742) so there is a need for discussion during the meeting to harmonize and approve a single CR.

Alt 1 (R1-2008328):

- 14	Integrated access-backhaul operation

*** Unchanged text is omitted ***

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot. An IAB-MT monitors PDCCH candidates in a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common and a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by AI-RNTI.

Alt 2 (R1-2008409)

10.1   UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-     a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-     a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-     a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-     a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-     a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, PS-RNTI, or AI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-     a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL-L-CS-RNTI, or AI-RNTI.

--------------- Unchanged parts omitted -------------

Alt 3 (R1-2008742):

14 	Integrated access-backhaul operation

<text omitted>

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot.

The IAB-MT monitors PDCCH candidates in one or more search space sets as described in Clause 10.1, and additionally monitors PDCCH candidates for a DCI format 2_5 with CRC scrambled by AI-RNTI in one or both of the following search spaces sets

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific.

<text omitted>

FL Observation 2.1: All three alternative candidate TPs make the necessary changes, but while Alt 2 has the smallest spec impact, it makes the change in a section outside of Section 14, which goes against the principle of keeping all IAB-specific functionality self-contained in a single section of 38.213. Alt 1 and Alt 3 are almost identical, but Alt 1 is slightly more concise and only references monitoring for 2_5 which is the context of the paragraph where the text is added. So the FL has a slight preference for Alt 1, but discussion is welcome on the alternatives and any changes which may be needed to capture the issue correctly in a single CR.

FL Proposal 2.1: Take Alt 1 (R1-2008328) as the starting point for the 38.213 CR to address necessary corrections for DCI Format 2_5 monitoring.

Discussion:

| Company | Do you agree with FL Proposal 2.1? | Comments |
| --- | --- | --- |
| Ericsson | Slight preference to Alt. 3. | Both Alt. 1 and Alt. 3 are acceptable. We have a preference to Alt. 3 since it refers to 10.1 and lists the two search spaces in bullet form making it a bit clearer. |
| Qualcomm | Slight preference for Alt. 3. | Same view as Ericsson. Both Alt. 1 and Alt 3 are agreeable, however Alt. is in our view a clearer version. |
| Huawei | Yes | Both Alt.1 and Alt.3 are okay. Similar to the moderator, we have a slight preference to Alt.1 since it is more concise. The reference to section 10.1 is not necessary since it is already stated at the beginning of section 14. “Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4 through 13 are equally applicable to the IAB-MT of an IAB node.” |
| LG | Have preference on Alt. 2 | Since Alt2 has the smallest spec. impact (as commented by FL), our first preference is Alt 2. However, if LG is the only supporter for Alt2, we can live with Alt 1 or 3. Among them, we have preference on Alt 3. |

Summary

TBD