3GPP TSG RAN WG1 #103-e								R1-20xxxxx

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.3

Source:	Moderator (AT&T)

Title:	Feature Lead Summary of [103-e-NR-IAB-02] Issue with IAB-DU “serving” cell terminology

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the following email discussion:

[103-e-NR-IAB-02] Issue with IAB-DU “serving” cell terminology – Thomas (AT&T)

-       Determine whether the term “serving cells” should be removed for IAB-DUs in 38.213

-       Discussion and decision by 10/27, TPs (if needed) by 10/29

# Issue with IAB-DU “serving” cell terminology

Source: R1-2008742

Background: In 38.213 Section 14, multiple references to a “serving cell” in relation to an IAB-DU are made, which are consistent with the terminology used for UEs and IAB-MTs. However, this is inconsistent with other specifications where gNB-DUs do not use the term “serving” for associated cells, including for IAB-DUs (see 38.331 and 38.401 for examples from RAN2 and RAN3 respectively). Unless a clear motivation is made for keeping the term “serving cells,” since the IAB-DU hosts gNB-DU functionality, it seems more appropriate to remove the instances of “IAB-DU serving cell(s)” or “serving cell(s) of an IAB-DU.” Since there does not appear to be any functional impact, an editorial correction of the specification could be used to make the update instead of a CR.

FL Proposal 2.1: Remove all references to “serving” in relation of IAB-DU cell(s) in TS 38.213 Section 14.

Note: This will be done via editorial update of the alignment CR

Discussion:

| Company | Do you agree with FL Proposal 2.1? | Comments |
| --- | --- | --- |
| Ericsson | Yes | A serving cell is a cell from a UE or IAB-MT perspective. It has no bearing on the IAB-DU side. |
| Qualcomm | Yes | None. |
| LG | Yes | None |
| Huawei | Yes | This was agreed before. |
| Samsung | Yes | None |
| Nokia | Yes | None |
| ZTE, Sanechips |  | After checking 38.331, the term “serving” is also used for a IAB DU related cell in the following text: ---------------------------------------------------- - AvailabilityCombinationsPerCell The IE AvailabiltyCombinationsPerCell is used to configure the AvailabiltyCombinations applicable for a serving cell of the IAB-node DU (see TS 38.213 [13], clause 14). ----------------------------------------------------- The inconsistent issue may still there if just RAN1 chooses to remove all references to “serving” in relation of IAB-DU cell(s) in TS 38.213. Given both RAN1 and RAN2 spec have used term “serving” for IAB DU, the following either way is OK for us: Option 1: Remove all references to “serving” in relation of IAB-DU cell(s) in TS 38.213 Section 14, also inform RAN2 to update its spec accordingly. Option 2: Do nothing, since current text does not cause any misunderstanding. |
| Vivo | Yes | As commented by ZTE, RAN1 and RAN2 should align the name. |
| Intel |  | Agree with ZTE’s comments. Either Option 1 or Option 2 is Ok. |

FL Proposal 2.2: Adopt the following TP to remove all references to “serving” in relation of IAB-DU cell(s) in TS 38.213 Section 14:

Note 1: This will be done via editorial update of the alignment CR

Note 2: It is RAN1’s understanding that the description of AvailabilityCombinationsPerCell  in 38.331 should also remove a reference to a “serving” cell of the IAB-node DU and recommends the following update:

“The IE AvailabiltyCombinationsPerCell is used to configure the AvailabiltyCombinations applicable for a serving cell of the IAB-node DU (see TS 38.213 [13], clause 14).”

TP for 38.213 Section 14

14 Integrated access-backhaul operation <text omitted> If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that NTA/2+Ndelta+Tdelta⋅Gstep⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when NTA/2+Ndelta+Tdelta⋅Gstep>0, where NTA is obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell and Ndelta and Gstep are determined as - Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1, - Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2 The IAB-node may use the time difference to determine a DU transmission time. A slot format for an IAB-DU or an IAB-MT includes downlink symbols, uplink symbols, and flexible symbols. For each serving cell of an IAB-DU, the IAB-DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration. <text omitted> With reference to slots of an IAB-DU serving cell, a symbol in a slot of an IAB-DU serving cell can be configured to be of hard, soft, or unavailable type. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU serving cell can respectively transmit, receive, or either transmit or receive in the symbol. When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive in the symbol, or - the IAB-MT would transmit or receive in the symbol, and the transmission or reception in the symbol is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available When a symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the symbol. A symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSize-AI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH. For each serving cell of an IAB-DU in a set of serving cells of the IAB-DU, the IAB-DU can be provided: - an identity of the IAB-DU serving cell by iabDuCellId-AI - a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI - a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU serving cell, and - a mapping for the soft symbol availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId The IAB-DU can assume a same SCS configuration for availabilityCombinations for a serving cell as an SCS configuration provided by IAB-DU-Resource-Configuration-TDD-Config for the serving cell. An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3. <text omitted>

# Summary

TBD