3GPP TSG RAN WG1 #103-e							    R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	FL summary#1 of issues for V2X Mode 2 in 7.2.4

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution provides a summary of proposed correction to Mode-2 sidelink resource allocation for NR-V2X communication, based on review of the submitted contributions [1]-[74].

Identification of email discussions

It is proposed to organize the following email discussions:

Alt. 1 (if two threads are allocated to M2)

- Thread M2-1

  - Issue 1

- Thread M2-2

  - Issue 2

  - Issue 3

Alt. 2 (if one thread is allocated to M2)

- Thread M2-1

  - Issue 1

There are several tdocs resolving the RAN2 LS questions from R1-2007521. It is assumed that there will be a dedicated LS AI5 thread allocated for the LS.

Issue list

Open issue list:

- Issue 1: Fix undefined UE behaviour for the case of re-evaluation performed during periodic reservation process

- Issue 2: Handle cases of infinite execution of step 1)-7) when X% could not be reached

- Issue 3: Handling of resource selection for multiple TB / MAC PDU

- Issue 4: Logical slot period calculation clarification in section 8.1.7 of 38.214

- Issue 5: Whether to apply Tproc,0 offset in step 6)-c) of 8.1.4 of 38.214 when calculating slots for exclusion. More details in R1-2008081

- Issue 6: Logical slot index vs physical slot index for slots where candidate set is determined

There are a couple issues with respect to capturing existing RAN1 agreements:

- Capturing issue 1: According to understanding in R1-2007986, MAC still does not implement RAN1 request to provide period equal to 0 in the identified cases

- Capturing issue 2: According to understanding in R1-2008667, MAC does not implement RAN1 agreement of time instances for pre-emption and re-evaluation checking

There are several editorial issues. Those can be captured once CRs are prepared.

Analysis of Draft Corrections

| Tdoc# | Issue within the tdoc | Issue index | FL comment |
| --- | --- | --- | --- |
| R1-2007612 Huawei, HiSilicon | Backward indication |  | Optimization |
|  | Clarification on exclusion during pre-emption and re-evaluation |  | Optimization |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Infinite loop / Candidate set starvation due to step 5) exclusion | 2 | Issue needs checking |
|  | Handling of resource selection for multiple TB | 3 | Open issue |
|  | For pre-emption check, step 5) does not consider current period |  | Optimization |
|  | Allow exclusion based on decoding result for unicast |  | Optimization |
|  | In 8.1.7, 38.214 clarification on logical slot period conversion | 4 | Issue needs checking |
| R1-2007774 LG Electronics | Re-selection of a dropped resource related to RAN2 LS R1-2007521, Q1. |  | Open issue Treat as part of LS reply activity |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
| R1-2007811 CATT | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Additional RSRP threshold incrementation stopping condition for pre-emption |  | Optimization |
|  | Further capture maximum gap between ReTX resource to be 32 in spec |  | No need |
|  | Optimization of exclusion of slots related to non-monitored slots |  | Optimization |
|  | Backward indication |  | Optimization |
|  | Re-selection of a dropped resource related to RAN2 LS R1-2007521, Q1. |  | Open issue Treat as part of LS reply activity |
| R1-2007923 ZTE, Sanechips | Clarification that sets r’ and r’’ may not be provided |  | Editorial |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Step 5) skipping for re-evaluation / pre-emption |  | Optimization |
|  | Correction to {tySL} in 16.3, 38.213 |  | Editorial |
|  | Delete “can” for description of slots which a UE shall monitor |  | Optimization / Editorial |
|  | Handling of resource selection for multiple TB | 3 | Open issue |
| R1-2007935 Intel Corporation | Handling of resource selection for multiple TB | 3 | Open issue |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
| R1-2007986 ETRI | Capture the case of setting reservation period to 0 |  | Capturing issue |
| R1-2008081 NEC | Correction to step 6) to include slots within Tproc0 | 5 | Issue needs checking |
| R1-2008096 Spreadtrum Communications | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Optimization of the sensing window size |  | Optimization |
|  | Restrict S_A by X*Mtotal |  | Optimization |
| R1-2008131 Samsung | In 8.1.4 correct t'iSL instead of tiSL | 6 | Issue needs checking |
|  | Remove step 5) |  | Optimization |
| R1-2008132 Samsung | In 8.1.7 clarify meaning of N, as an average number of SL slots | 4 | Issue needs checking |
| R1-2008236 OPPO | Infinite loop / Candidate set starvation due to step 5) exclusion | 2 | Issue needs checking |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
| R1-2008389 Sharp | Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions |  | Optimization |
| R1-2008431 Apple | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Define sensing window in logical slots | 6 | Issue needs checking |
| R1-2008531 NTT DOCOMO, INC. | Re-selection of a dropped resource related to RAN2 LS R1-2007521, Q1. |  | Open issue Treat as part of LS reply activity |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
| R1-2008606 Qualcomm Incorporated | Infinite loop / Candidate set starvation due to step 5) exclusion | 2 | Issue needs checking |
|  | Re-evaluation with periodic reservation | 1 | Open issue |
| R1-2008633 ASUSTeK | Optimization of exclusion of slots related to non-monitored slots |  | Optimization |
| R1-2008667 vivo | Re-evaluation with periodic reservation | 1 | Open issue |
|  | Restrict how much resources a UE can reserve in total after pre-emption with periodic reservation |  | Optimization |
|  | Capture pre-emption/re-evaluation checking instances in MAC as per agreements |  | Capturing issue |
|  | Restrict the maximum total number of reserved resources |  | Optimization |
|  | Align sidelink grant terminology with MAC |  | Editorial |
| R1-2008750, R1-2008750 Ericsson | Editorial correction for references/descriptions of SCI fields |  | Editorial |

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2007612	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2007774	Discussion on essential corrections in resource allocation for Mode 2	LG Electronics

- R1-2007811	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2007923	Remaining issues in mode 2	ZTE, Sanechips

- R1-2007935	Corrections related to Mode-2 resource allocation	Intel Corporation

- R1-2007986	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2008081	Maintenance for mode 2 resource allocation	NEC

- R1-2008096	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2008131	Draft CR on Mode 2 for NR Sidelink	Samsung

- R1-2008132	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2008236	Remaining open issues and corrections for mode 2 RA	OPPO

- R1-2008389	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2008431	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2008531	Maintenance for resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2008606	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2008633	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2008667	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson

- R1-2008752	Draft_CR_TS38.212	Ericsson

Other Rel.16 NR V2X contributions

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007611	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-2007613	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

- R1-2007775	Discussion on essential corrections in sidelink synchronization mechanism	LG Electronics

- R1-2007776	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2007779	A remaining issue on UE procedures for reporting HARQ-ACK on uplink	Fujitsu

- R1-2007780	A remaining issue on simultaneous transmissions of uplink and PUSCH carrying sidelink HARQ-ACK	Fujitsu

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

- R1-2007812	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

- R1-2007813	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007922	Remaining issues in Mode-1	ZTE, Sanechips

- R1-2007924	Remaining issues of synchronization	ZTE, Sanechips

- R1-2007925	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

- R1-2007936	Corrections related to Mode-1 resource allocation	Intel Corporation

- R1-2007987	Physical layer procedures for sidelink	ETRI

- R1-2008095	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-2008097	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008130	Draft CR on PUCCH Power Control for NR Sidelink Mode 1 Scheduling	Samsung

- R1-2008133	Draft CR on Physical Layer Procedures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008231	Text proposal of mode 1 for NR sidelink	OPPO

- R1-2008232	Text proposal of physical layer procedure for NR sidelink	OPPO

- R1-2008237	Corrections for FDM-based semi-static power split for in-device coexistence	OPPO

- R1-2008334	Correction on sidelink timing definition	Huawei, HiSilicon

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008388	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

- R1-2008390	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2008391	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2008428	Remaining Issues of Physical Layer Procedures	Apple

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure	Apple

- R1-2008430	Remaining Issues of Mode 1 Resource Allocation	Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008497	Remaining issues on sidelink power control	ASUSTeK

- R1-2008498	Miscellaneous issues of SL HARQ-ACK reporting on PUCCH	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2008532	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2008533	Maintenance for sidelink-related collision	NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure	Qualcomm Incorporated

- R1-2008605	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008666	Remaining issues on mode 1 resource allocation mechanism	vivo

- R1-2008668	Remaining issues on sidelink synchronization mechanism	vivo

- R1-2008669	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2008721	Remaining issues on physical layer procedures for sidelink	KT Corp.

- R1-2008751	Draft_CR_TS38.211	Ericsson

- R1-2008753	Draft_CR_TS38.213	Ericsson

- R1-2008754	Draft_CR_TS38.214	Ericsson