#3GPP TSG RAN WG1 #103-e						R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

______________________________________________________________________Agenda item: 7.2.4

Source: Moderator (LG Electronics)

Title: Feature lead summary#1 for physical layer procedure aspects in AI 7.2.4

Document for: Discussion and decision

- Potential issues for email discussions

- Physical layer procedure

  - Thread 1

    - Issue #2: Prioritization rule between PSCCH/PSSCH and PUCCH without SL HARQ reports/PUSCH without UL-SCH/SRS

    - Issue #3: Prioritization rule between PSFCH/S-SSB reception(s) and UL transmission(s)

    - Issue #4: MsgA PUSCH is prioritized over SL transmission

  - Thread 2

    - Issue #7: Clarification on the logical slot set for resource allocation procedure

- Physical layer procedure

Issue #1: Prioritization rule between PUSCH carrying SL HARQ-ACK reports and SL transmission [Huawei,4] [LGE,9] [Apple,48] [vivo,68] [KT,69] [Ericsson,70]

- Option 1: Reuse prioritization between PSFCH/S-SS/PSBCH block transmission and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR, or a PUCCH with sidelink HARQ-ACK information report [LGE,9]

  - No spec change is needed

- Option 2: SL transmission is prioritized when the SL transmission is prioritized over both UL-SCH and SL HARQ-ACK reporting [Huawei,4] [Apple,48] [vivo,68] [KT,69] [Ericsson,70]

  - Relevant TP can be found in [Huawei,4] [Apple,48] [vivo,68]

Issue #2: Prioritization rule between PSCCH/PSSCH and PUCCH without SL HARQ reports/PUSCH without UL-SCH/SRS

- Reuse prioritization between PSFCH/S-SS/PSBCH block transmission and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR, or a PUCCH with sidelink HARQ-ACK information report [LGE,9] [ZTE,21]

  - Relevant TP can be found in [LGE,9] [ZTE,21]

Issue #3: Prioritization rule between PSFCH/S-SSB reception(s) and UL transmission(s)

- Reuse prioritization between PSFCH/S-SS/PSBCH block transmission and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR, or a PUCCH with sidelink HARQ-ACK information report [LGE,9] [ZTE,21] [Ericsson,70]

  - Relevant TP can be found in [LGE,9] [ZTE,21]

Issue #4: MsgA PUSCH is prioritized over SL transmission

- Support: [Apple,48] [vivo,68] [Ericsson,70]

  - Relevant TP can be found in [Apple,48] [vivo,68]

Issue #5: Prioritization rule between PSFCH TX and PSFCH RX or between PSFCH TX and PSFCH TX for tie-break

- Up to UE implementation [LGE,9] [Apple,48]

- Additional rule based on HARQ-ACK feedback option and HARQ-ACK state [CATT,16] [KT,69] [Ericsson,70]

  - Relevant TP can be found in [CATT,16]

Issue #6: Prioritization rule between PUCCH carrying SL HARQ-ACK reporting and SL transmission for tie-break

- Up to UE implementation [LGE,9] [ZTE,21] [Apple,48]

- Additional rule [Ericsson,70]

Issue #7: Clarification on the logical slot set for resource allocation procedure [Huawei,4] [LGE,5] [OPPO,32] [Apple,48]

- Option 1: For TRIV interpretation and resource reservation period, the logical slots belonging to a resource pool. [Huawei,4] [LGE,5] [Apple,48]

  - Relevant TP can be found in [Huawei,4] [LGE,5]

- Option 2: For TRIV interpretation and resource reservation period, the logical slots can belong to a resource pool [OPPO,32]

  - Relevant TP can be found in [OPPO,32]

Issue #8: Capture constraint on total SL TX power in case of simultaneous transmission of SL and UL

- Support: [LGE,9] [Spreadtrum,31]

  - Relevant TP can be found in [LGE,9] [Spreadtrum,31]

References

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007611	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-2007612	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2007613	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

- R1-2007774	Discussion on essential corrections in resource allocation for Mode 2	LG Electronics

- R1-2007775	Discussion on essential corrections in sidelink synchronization mechanism	LG Electronics

- R1-2007776	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2007779	A remaining issue on UE procedures for reporting HARQ-ACK on uplink	Fujitsu

- R1-2007780	A remaining issue on simultaneous transmissions of uplink and PUSCH carrying sidelink HARQ-ACK	Fujitsu

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

- R1-2007811	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2007812	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

- R1-2007813	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007922	Remaining issues in Mode-1	ZTE, Sanechips

- R1-2007923	Remaining issues in mode 2	ZTE, Sanechips

- R1-2007924	Remaining issues of synchronization	ZTE, Sanechips

- R1-2007925	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

- R1-2007935	Corrections related to Mode-2 resource allocation	Intel Corporation

- R1-2007936	Corrections related to Mode-1 resource allocation	Intel Corporation

- R1-2007986	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2007987	Physical layer procedures for sidelink	ETRI

- R1-2008081	Maintenance for mode 2 resource allocation	NEC

- R1-2008095	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-2008096	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2008097	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008130	Draft CR on PUCCH Power Control for NR Sidelink Mode 1 Scheduling	Samsung

- R1-2008131	Draft CR on Mode 2 for NR Sidelink	Samsung

- R1-2008132	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2008133	Draft CR on Physical Layer Procedures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008231	Text proposal of mode 1 for NR sidelink	OPPO

- R1-2008232	Text proposal of physical layer procedure for NR sidelink	OPPO

- R1-2008236	Remaining open issues and corrections for mode 2 RA	OPPO

- R1-2008237	Corrections for FDM-based semi-static power split for in-device coexistence	OPPO

- R1-2008334	Correction on sidelink timing definition	Huawei, HiSilicon

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008388	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

- R1-2008389	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2008390	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2008391	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2008428	Remaining Issues of Physical Layer Procedures	Apple

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure	Apple

- R1-2008430	Remaining Issues of Mode 1 Resource Allocation	Apple

- R1-2008431	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008497	Remaining issues on sidelink power control	ASUSTeK

- R1-2008498	Miscellaneous issues of SL HARQ-ACK reporting on PUCCH	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2008531	Maintenance for resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2008532	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2008533	Maintenance for sidelink-related collision	NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure	Qualcomm Incorporated

- R1-2008605	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2008606	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2008633	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008666	Remaining issues on mode 1 resource allocation mechanism	vivo

- R1-2008667	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2008668	Remaining issues on sidelink synchronization mechanism	vivo

- R1-2008669	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2008721	Remaining issues on physical layer procedures for sidelink	KT Corp.

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson

- R1-2008751	Draft_CR_TS38.211	Ericsson

- R1-2008752	Draft_CR_TS38.212	Ericsson

- R1-2008753	Draft_CR_TS38.213	Ericsson

- R1-2008754	Draft_CR_TS38.214	Ericsson