[103-e-NR-Rel-16-V2X-06]: Email discussion/approval regarding remaining issues in prioritization between SL and UL

- Issue PP-2: Prioritization rule between PSCCH/PSSCH and PUCCH without SL HARQ reports and SR/PUSCH without UL-SCH/SRS

- Issue PP-3: Prioritization rule between PSFCH/S-SSB reception(s) and UL transmission(s)

- Issue PP-4: PRACH and MsgA PUSCH are prioritized over SL transmission

till 10/30, with a potential CR by 11/4 – Hanbyul (LGE)

Issue PP-2: Prioritization rule between PSCCH/PSSCH and PUCCH without SL HARQ reports and SR/PUSCH without UL-SCH/SRS

Q1: Do you agree following the proposal?

Proposal:

For prioritization between PSCCH/PSSCH TX and UL TX,

- When the overlapping UL TX is PUCCH with neither SL HARQ reports nor SR, PUSCH without UL-SCH, or SRS,

  - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case)

    - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized)

    - Otherwise, UL TX is prioritized

  - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case

| Company | Answer | Comment |
| --- | --- | --- |
| LGE | Yes |  |

Issue PP-3: Prioritization rule between PSFCH/S-SSB reception(s) and UL transmission(s)

Q2: Do you agree the following proposal?

Proposal:

For prioritization between PSFCH/S-SSB RX and UL TX,

- The priority of PSFCH RX is the highest priority of the associated PSCCH/PSSCH

- The priority of S-SSB RX is equal to the (pre-)configured priority introduced for in-device coexistence.

- When the overlapping UL TX other than PUCCH carrying SL HARQ reporting, PRACH, PUSCH scheduled by RAR UL grant, and PUSCH for Type-2 random access procedure (i.e. MsgA),

  - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case)

    - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized)

    - Otherwise, UL TX is prioritized

  - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case

| Company | Answer | Comment |
| --- | --- | --- |
| LGE | Yes |  |

Issue PP-4: PRACH and MsgA PUSCH are prioritized over SL transmission

Q3: Do you agree the following proposal?

Proposal:

- PRACH, PUSCH scheduled by RAR UL grant, and PUSCH for Type-2 random access procedure (i.e. MsgA) are always prioritized over SL transmissions or receptions

| Company | Answer | Comment |
| --- | --- | --- |
| LGE | Yes |  |

Others

Q4: If there are any other aspects that need to be considered in the scope of this email discussion, please specify them.

| Company | Comment |
| --- | --- |