[103-e-NR-Rel-16-V2X-07]: Email discussion/approval regarding clarification on the sidelink slot index

- Issue PP-7: Interpretation of sidelink slot for TRIV and resource reservation period

- Issue M2-6: Interpretation of sidelink slot for sensing and resource selection procedure

till 10/29, with a potential CR by 11/4 – Hanbyul (LGE)

Issue PP-7: Interpretation of sidelink slot for TRIV and resource reservation period

Q1: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Time resource assignment” in the SCI format 1-A indicates ti, what is UE’s understanding?

- Option 1: ti is counted in t'0SL, t'1SL, t'2SL,… (i.e. The number of slots in the resource pool between slot n and slot n+ti is always the same as ti (including slot n+ti itself).

- Option 2: ti is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+ti can be less than ti).

| Company | Option | Comment |
| --- | --- | --- |

Q2: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P, what is UE’s understanding?

- Option 1: P is counted in t'0SL, t'1SL, t'2SL,… (i.e. The number of slots in the resource pool between slot n and slot n+P is always the same as P (including slot n+P itself)).

- Option 2: P is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+P can be less than P).

| Company | Option | Comment |
| --- | --- | --- |

Issue M2-6: Interpretation of sidelink slot for sensing and resource selection procedure

Q3: When a UE performs PSSCH resource selection procedure in sidelink resource allocation mode 2, over which slots does the UE monitor SCIs for sensing operation?

- Option 1: t'0SL, t'1SL, t'2SL,… in the sensing window.

- Option 2: t0SL,t1SL,t2SL,... in the sensing window.

| Company | Option | Comment |
| --- | --- | --- |

Others

Q4: If there are any other aspects that need to be considered in the scope of this email discussion, please specify them.

| Company | Comment |
| --- | --- |