[103-e-NR-Rel-16-V2X-08]: Email discussion/approval of CRs for the agreements from previous meetings (Physical layer structure) by 10/30 - Jeongho (Samsung)

List of changes

- Draft CR#1: [38.213 16] Periodicity of resource pool configuration and bitmap [OPPO]

- Editorial CRs

  - #2 [38.211, 8.4.1.2.2] Typos in clause of PT-RS mapping [CATT]

  - #3 [38.211 8.3.1.2] Table number for 211 [Intel]

  - #4 [38.212 8.4.1.1/2 and 38.214 8.1] name of HARQ process number to sidelink process ID [vivo]

  - #5 [38.214 8.1.3] RV description [ZTE, Sanechips]

Draft CR#1. Periodicity of resource pool bitmap

| Clauses affected and spec | Clause 16 of TS38.213 |
| --- | --- |
| Reason for change: | In RAN1#101-e, it was agreed that the periodicity of resource pool bitmap is 10240 ms. The higher layer parameter ‘periodResourcePool’ is not defined and not needed. |
| Summary of change: | The higher layer parameter ‘periodResourcePool’ is replaced by ‘10240 ms’. |
| Consequences if not approved: | It is unclear what ‘periodResourcePool’ and the periodicity of resource pool bitmap mean. |

----------------------------------------- Begin Text proposal of TS38.213 ------------------------------------- 16 UE procedures for sidelink A UE is provided by locationAndBandwidth-SL a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS38.211]. For a resource pool within the SL BWP, the UE is provided by numSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by subchannelsize. The first RB of the first sub-channel in the SL BWP is indicated by startRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined. <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.213 -------------------------------------

Please provide your views and reason.

| Company | Views |
| --- | --- |

Draft Editorial CRs

Draft CR#2. Typos in clause of PT-RS mapping in Clause 8.4.1.2.2 of TS38.211

----------------------------------------- Begin Text proposal of TS38.211 ------------------------------------- 8.4.1.2.2 Mapping to physical resources <Unchanged text is omitted> The set of time indices l defined relative to the start of the PSSCH allocation is defined by 1. set i=0 and lref=0 2. if any symbol in the interval max lref+i-1LPT-RS+1, lref,…,lref+iLPT-RS overlaps with a symbol used for DM-RS according to clause 8.4.1.1.2 - set i=1 - set lref to the symbol index of the DM-RS symbol - repeat from step 2 as long as lref+iLPT-RS is inside the PSSCH allocation <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.211 -------------------------------------

Draft CR#3. Table number in Clause 8.3.1.2 of TS38.211

----------------------------------------- Begin Text proposal of TS38.211 ------------------------------------- #### 8.3.1.2 Modulation For the single codeword q=0, the block of scrambled bits shall be modulated, resulting in a block of complex-valued modulation symbols d(q)0,…,d(q)Msymb(q)-1 where Msymb(q)=Msymb,1(q)+Msymb,2(q). Modulation for 0≤i<Mbit,SCI2(q) shall be done as described in clause 5.1 using QPSK, where Msymb,1(q)=Mbit,SCI2(q)2. Modulation for Mbit,SCI2(q)≤i< Mbit(q) shall be done as described in clause 5.1 using one of the modulation schemes in Table 8.3.1.2-1 where Msymb,2(q)=Mbit,data(q)Qm. Table 8.3.1.2-1: Supported modulation schemes. Modulation scheme; Modulation order Qm. QPSK; 2. 16QAM; 4. 64QAM; 6. 256QAM; 8. <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.211 -------------------------------------

Draft CR#4. name of HARQ process number to sidelink process ID in TS38.212 and TS38.214

----------------------------------------- Begin Text proposal of TS38.212 ------------------------------------- #### 8.4.1.1 SCI format 2-A SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information. The following information is transmitted by means of the SCI format 2-A: - sidelink process ID – 4 bits as defined in clause 16.4 of [5, TS 38.213]. <Unchanged text is omitted> #### 8.4.1.2 SCI format 2-B SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information. The following information is transmitted by means of the SCI format 2-B: - sidelink process ID – 4 bits as defined in clause 16.4 of [5, TS 38.213]. <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.212 -------------------------------------

----------------------------------------- Begin Text proposal of TS38.214 ------------------------------------- ## 8.1 UE procedure for transmitting the physical sidelink shared channel <Unchanged text is omitted> The UE shall set the contents of the SCI format 2-A as follows: - the UE shall set value of the "sidelink process ID" field as indicated by higher layers. <Unchanged text is omitted> The UE shall set the contents of the SCI formats 2-B as follows: - the UE shall set value of the "sidelink process ID" field as indicated by higher layers. <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.214 -------------------------------------

Draft CR#5. RV description in TS38.214

----------------------------------------- Begin Text proposal of TS38.214 ------------------------------------- ### 8.1.3 Modulation order, target code rate, redundancy version and transport block size determination The redundancy version is given by the “Redundancy version field” field in SCI format 2-A or 2-B. <Unchanged text is omitted> -------------------------------------------- End Text proposal of TS38.214 -------------------------------------

Please provide your views and reason including whether the change is needed or not.

| Company | Views |
| --- | --- |
|  | On Draft CR#2: On Draft CR#3: On Draft CR#4: On Draft CR#5: |

Any others?

If there is any other editorial changes needed, or text proposal that needs to be captured from the previous meetings, please provide issues and text proposals.

| Company | Views |
| --- | --- |

# Reference

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design		Intel Corporation

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure		Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure		NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure		Qualcomm Incorporated

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson