3GPP TSG RAN WG1 Meeting #103-E						     R1- 200xxxx

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	Discussion on [103-e-NR-Rel-16-V2X-10]

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution provides discussion on critical issues for the thread [103-e-NR-Rel-16-V2X-10].

[103-e-NR-Rel-16-V2X-10]: Email discussion/approval of CRs for the agreements from previous meetings (Mode 2) by 10/30 – Sergey (Intel)

Inputs on potential TPs

Please indicate which agreements are not yet captured in specification, including editorial issues. So far, the following has been identified in contributions:

Editorial #1: Clarification that sets r’ and r’’ may not be provided simultaneously [ZTE, R1-2007923]

Editorial #2: {tySL} should be replaced by {t'ySL} in 38.213 section 16.4 (TDRA/FDRA setting in SCI 1-A) [ZTE, R1-2007923]

Editorial #3: TP to clarify that the configured sidelink grant in 8.1.5 of 38.214 refers to a selected sidelink grant defined in 38.321 (i.e. Mode-2 UE-autonomous scheduling) [vivo, R1-2008667]

Editorial #4: Correction for references/descriptions of SCI fields in section 8.3.1.1, TS 38.212 [Ericsson, R1-2008750, R1-2008752]

Q1: which of the above editorial corrections should be agreed? If agreed, are any modifications needed for the TPs provided in the referred contributions?

| Source | Comments |
| --- | --- |
| Qualcomm | #1 is not needed, at least r’_0 need to be presented all the time since it will be the next resource that a transmission will happen on. For #2, this is related to M-7 and we should wait for the conclusion of that issue. |

Q2: Any other omissions/editorials?

| Source | Comments |
| --- | --- |

- Annex - TPs presented in contributions for the identified issues

Editorial #1

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts are omitted> - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer may provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and may provide another set of resources (r0',r1',r2',…) which may be subject to pre-emption. - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , if any provided, and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. <Unchanged parts are omitted>

Editorial #2

## 16.4 UE procedure for transmitting PSCCH <Unchanged parts are omitted> - the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where: - N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserve - each resource, from the set of Ry resources, corresponds to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels available for PSSCH/PSCCH transmission in a slot - t'0SL,t'1SL,t'2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214] - y0 is an index of a slot where the PSCCH with SCI format 1-A is transmitted. <Unchanged parts are omitted>

Editorial #3

If a set of sub-channels in slot tmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the configured selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots tm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t0SL,t1SL,t2SL,... is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

Editorial #4

<Unchanged parts omitted>

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 23.287].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213].

-	Resource reservation interval –log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

<Unchanged parts omitted>

#### 8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number – log2Nprocess bits as defined in clause 8.1 of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214]..

-	Redundancy version – 2 bits as defined in clause 8.1 of [6, TS 38.214]..

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214]..

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

<Unchanged parts omitted>

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2007612	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2007774	Discussion on essential corrections in resource allocation for Mode 2	LG Electronics

- R1-2007811	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2007923	Remaining issues in mode 2	ZTE, Sanechips

- R1-2007935	Corrections related to Mode-2 resource allocation	Intel Corporation

- R1-2007986	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2008081	Maintenance for mode 2 resource allocation	NEC

- R1-2008096	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2008131	Draft CR on Mode 2 for NR Sidelink	Samsung

- R1-2008132	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2008236	Remaining open issues and corrections for mode 2 RA	OPPO

- R1-2008389	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2008431	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2008531	Maintenance for resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2008606	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2008633	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2008667	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson

- R1-2008752	Draft_CR_TS38.212	Ericsson

Other Rel.16 NR V2X contributions

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007611	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-2007613	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

- R1-2007775	Discussion on essential corrections in sidelink synchronization mechanism	LG Electronics

- R1-2007776	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2007779	A remaining issue on UE procedures for reporting HARQ-ACK on uplink	Fujitsu

- R1-2007780	A remaining issue on simultaneous transmissions of uplink and PUSCH carrying sidelink HARQ-ACK	Fujitsu

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

- R1-2007812	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

- R1-2007813	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007922	Remaining issues in Mode-1	ZTE, Sanechips

- R1-2007924	Remaining issues of synchronization	ZTE, Sanechips

- R1-2007925	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

- R1-2007936	Corrections related to Mode-1 resource allocation	Intel Corporation

- R1-2007987	Physical layer procedures for sidelink	ETRI

- R1-2008095	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-2008097	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008130	Draft CR on PUCCH Power Control for NR Sidelink Mode 1 Scheduling	Samsung

- R1-2008133	Draft CR on Physical Layer Procedures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008231	Text proposal of mode 1 for NR sidelink	OPPO

- R1-2008232	Text proposal of physical layer procedure for NR sidelink	OPPO

- R1-2008237	Corrections for FDM-based semi-static power split for in-device coexistence	OPPO

- R1-2008334	Correction on sidelink timing definition	Huawei, HiSilicon

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008388	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

- R1-2008390	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2008391	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2008428	Remaining Issues of Physical Layer Procedures	Apple

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure	Apple

- R1-2008430	Remaining Issues of Mode 1 Resource Allocation	Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008497	Remaining issues on sidelink power control	ASUSTeK

- R1-2008498	Miscellaneous issues of SL HARQ-ACK reporting on PUCCH	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2008532	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2008533	Maintenance for sidelink-related collision	NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure	Qualcomm Incorporated

- R1-2008605	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008666	Remaining issues on mode 1 resource allocation mechanism	vivo

- R1-2008668	Remaining issues on sidelink synchronization mechanism	vivo

- R1-2008669	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2008721	Remaining issues on physical layer procedures for sidelink	KT Corp.

- R1-2008751	Draft_CR_TS38.211	Ericsson

- R1-2008753	Draft_CR_TS38.213	Ericsson

- R1-2008754	Draft_CR_TS38.214	Ericsson