3GPP TSG RAN WG1 Meeting #103-E						     R1- 200xxxx

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	Discussion on [103-e-NR-Rel-16-V2X-10]

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution provides discussion on critical issues for the thread [103-e-NR-Rel-16-V2X-10].

[103-e-NR-Rel-16-V2X-10]: Email discussion/approval of CRs for the agreements from previous meetings (Mode 2) by 10/30 – Sergey (Intel)

Inputs on potential TPs

Please indicate which agreements are not yet captured in specification, including editorial issues. So far, the following has been identified in contributions:

Editorial #1: Clarification that sets r’ and r’’ may not be provided simultaneously [ZTE, R1-2007923]

Editorial #2: {tySL} should be replaced by {t'ySL} in 38.213 section 16.4 (TDRA/FDRA setting in SCI 1-A) [ZTE, R1-2007923]

Editorial #3: TP to clarify that the configured sidelink grant in 8.1.5 of 38.214 refers to a selected sidelink grant defined in 38.321 (i.e. Mode-2 UE-autonomous scheduling) [vivo, R1-2008667]

Editorial #4: Correction for references/descriptions of SCI fields in section 8.3.1.1, TS 38.212 [Ericsson, R1-2008750, R1-2008752]

Q1: which of the above editorial corrections should be agreed? If agreed, are any modifications needed for the TPs provided in the referred contributions?

| Source | Comments |
| --- | --- |
| Qualcomm | #1 is not needed, at least r’_0 need to be presented all the time since it will be the next resource that a transmission will happen on. For #2, this is related to M-7 and we should wait for the conclusion of that issue. |
| ETRI | For #2, agree with Qualcomm. |
| vivo |  |
| Sharp | Agree with Qualcomm on #1 and #2. |
| ZTE | We accept all the above editorial changes. Consider to keep align with the conclusion of pp-7, it is also acceptable for us to postpone #2 and wait for the conclusion of pp-7. |
| OPPO | Agree with #1. For #2, agree with Qualcomm and wait for the outcome of M-7 discussion. OK with #3. For #4, we don’t agree with changing resource reservation interval, because the original wording “period” has been used throughout other specs including at least RAN1 and RAN2. Also it has been always used in past agreements and discussions in both RAN1 and RAN2. If changing now it may cause questions and confusion as to any difference between them. In terms of meaning of these two words, they are very similar. We don’t see the need to change the terminology/wording. |
| Samsung | Agree with #1. Agree with #2 but this issue can be handled in email thread#7. O.K with #3 and #4. |
| CATT | For issue #1, No need to further clarify. both the re-evaluation and pre-emption resource set are provided by higher layer, RAN1 only need to perform re-evaluation or pre-emption according to the provided resource set type. For issue #2, agree to wait for the conclusion of M-7. |
| Huawei/HiSilicon | #1: The “if condition”, i.e., “if the higher layer requests the UE …”, means higher layer already requests to the UE to do re-evaluation or pre-emption check, so it is more accurate to use “the higher layer provides a set of resources” instead of “the higher layer may provide a set of resources”. Because “the higher layer may provide a set of resources” leads to an interpretation that it’s also possible the “if condition” is satisfied, and higher layer does not provide a set of resources. To clarify that sets r’ and r’’ may not be provided simultaneously, the following change is suggested: == - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation andor a set of resources (r0',r1',r2',…) which may be subject to pre-emption, respectively. - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) andor (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. == #2: agree. #3: agree. But “sidelink grant” should be in black color. #4: we share similar view with OPPO that there is no need to change the terminology since “period” is already widely used in current specifications and previous agreements. There is no ambiguity. |

Q2: Any other omissions/editorials?

| Source | Comments |
| --- | --- |
| ETRI | We have one more capturing issue regarding period equal to 0 [ETRI, R1-2007986]. For FL, do you have a plan to deal with the issue even though it is related with MAC specification? |
| vivo | The essential agreement about re-evaluation and pre-emption have not been captured in spec. we agree on some mandatory timing for re-evaluation and pre-emption, and also the mandatory resources for re-evaluation/pre-emption check. The intention is to gurantee system performance. Based on current 38.321 and 38.213, the trigger for re-evaluation/pre-emption is totally up to implementation, the resource set r’ and r’’ are decided by MAC via implementation as well. We suggests two alternatives to capture the agreement. First alternative is to send LS to RAN2 to trigger related spec. edition. Second alternative, we add restriction on r’ and r’’ to reflect the mandatory re-evaluation and pre-emption check. |
| Sharp | As discussed in [R1-2008389, Sharp], we think to further clarify actual resource number N=1 for non-monitored case is to align the agreement in RAN1#99 which is to reuse LTE sensing procedure step 5. |
| NEC | As pointed out in our contribution R1-2008081, I would like to invite FL and companies to consider whether to apply Tproc,0 offset in step 6)-c) of 8.1.4 of 38.214 when calculating slots for exclusion. Initially, we thought it’s an optimization issue, but after checking previous agreements and LTE V2X design, we think it's actually a Rel.16 agreement wrongly captured in TS. In RAN1#99, in terms of SPS resources, we agreed to reuse LTE design: - (working assumption) Procedure of mapping of periodic semi-persistent resources into the resource selection window is reused from LTE - By reusing TS 36.213, section 14.1.1.6, steps 5 and 6 of non-partial sensing, as applicable And in LTE V2X, the related agreement is agreed in RAN1#87 as: R1-1613655 WF on Support of Small Transmission Periods for V2V Communication Intel, Qualcomm Inc. Agreement: - Confirm that reselection UE scales the number of reservations of other UE within selection window by 1/i when 0<i<1 for the SCI received in the last i*P_step logical subframes in the sensing window. Here i denotes the resource reservation interval in the received SCI. Hence, the design in LTE is to scale SPS reservation from SCI received in the sensing window, this agreement was captured in TS as endorsed in RP-170622 (TS 36.213-e20). When we develop NR spec, we should respect the LTE design if we already agreed to reuse LTE procedure but not only copy LTE spec wording. Back to this issue, as Tproc,0 offset doesn’t belong to sensing window in NR, in the step 6)-c) of 8.1.4 of 38.214 to scale SPS reservation, we should not use n'-m≤Prsvp_RX', but to use n'-Tproc,0SL-m≤Prsvp_RX' to correctly reuse LTE procedure as agreed. Thank you. |

- Annex - TPs presented in contributions for the identified issues

Editorial #1

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts are omitted> - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer may provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and may provide another set of resources (r0',r1',r2',…) which may be subject to pre-emption. - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , if any provided, and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. <Unchanged parts are omitted>

Editorial #2

## 16.4 UE procedure for transmitting PSCCH <Unchanged parts are omitted> - the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where: - N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserve - each resource, from the set of Ry resources, corresponds to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels available for PSSCH/PSCCH transmission in a slot - t'0SL,t'1SL,t'2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214] - y0 is an index of a slot where the PSCCH with SCI format 1-A is transmitted. <Unchanged parts are omitted>

Editorial #3

If a set of sub-channels in slot tmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the configured selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots tm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t0SL,t1SL,t2SL,... is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

Editorial #4

<Unchanged parts omitted>

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 23.287].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213].

-	Resource reservation interval –log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

<Unchanged parts omitted>

#### 8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number – log2Nprocess bits as defined in clause 8.1 of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214]..

-	Redundancy version – 2 bits as defined in clause 8.1 of [6, TS 38.214]..

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214]..

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

<Unchanged parts omitted>

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2007612	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2007774	Discussion on essential corrections in resource allocation for Mode 2	LG Electronics

- R1-2007811	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2007923	Remaining issues in mode 2	ZTE, Sanechips

- R1-2007935	Corrections related to Mode-2 resource allocation	Intel Corporation

- R1-2007986	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2008081	Maintenance for mode 2 resource allocation	NEC

- R1-2008096	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2008131	Draft CR on Mode 2 for NR Sidelink	Samsung

- R1-2008132	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2008236	Remaining open issues and corrections for mode 2 RA	OPPO

- R1-2008389	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2008431	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2008531	Maintenance for resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2008606	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2008633	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2008667	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson

- R1-2008752	Draft_CR_TS38.212	Ericsson

Other Rel.16 NR V2X contributions

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007611	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-2007613	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

- R1-2007775	Discussion on essential corrections in sidelink synchronization mechanism	LG Electronics

- R1-2007776	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2007779	A remaining issue on UE procedures for reporting HARQ-ACK on uplink	Fujitsu

- R1-2007780	A remaining issue on simultaneous transmissions of uplink and PUSCH carrying sidelink HARQ-ACK	Fujitsu

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

- R1-2007812	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

- R1-2007813	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007922	Remaining issues in Mode-1	ZTE, Sanechips

- R1-2007924	Remaining issues of synchronization	ZTE, Sanechips

- R1-2007925	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

- R1-2007936	Corrections related to Mode-1 resource allocation	Intel Corporation

- R1-2007987	Physical layer procedures for sidelink	ETRI

- R1-2008095	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-2008097	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008130	Draft CR on PUCCH Power Control for NR Sidelink Mode 1 Scheduling	Samsung

- R1-2008133	Draft CR on Physical Layer Procedures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008231	Text proposal of mode 1 for NR sidelink	OPPO

- R1-2008232	Text proposal of physical layer procedure for NR sidelink	OPPO

- R1-2008237	Corrections for FDM-based semi-static power split for in-device coexistence	OPPO

- R1-2008334	Correction on sidelink timing definition	Huawei, HiSilicon

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008388	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

- R1-2008390	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2008391	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2008428	Remaining Issues of Physical Layer Procedures	Apple

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure	Apple

- R1-2008430	Remaining Issues of Mode 1 Resource Allocation	Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008497	Remaining issues on sidelink power control	ASUSTeK

- R1-2008498	Miscellaneous issues of SL HARQ-ACK reporting on PUCCH	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2008532	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2008533	Maintenance for sidelink-related collision	NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure	Qualcomm Incorporated

- R1-2008605	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008666	Remaining issues on mode 1 resource allocation mechanism	vivo

- R1-2008668	Remaining issues on sidelink synchronization mechanism	vivo

- R1-2008669	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2008721	Remaining issues on physical layer procedures for sidelink	KT Corp.

- R1-2008751	Draft_CR_TS38.211	Ericsson

- R1-2008753	Draft_CR_TS38.213	Ericsson

- R1-2008754	Draft_CR_TS38.214	Ericsson