3GPP TSG-RAN WG1 Meeting #103-e	R1-20xxxxx

Online, October 26th – November 13th, 2020

Agenda Item:	7.2.4

Source:	Moderator (Ericsson)

Title:	Thread 14 on the reply to the LS in R1-2007522

Document for:	Discussion, Decision

1	List of issues for discussion

[103-e-NR-Rel-16-V2X-14]: For LS in R1-2007522, a reply LS is necessary – target 10/29 for email approval. To be handled under 7.2.4 – Ricardo (Ericsson).

2	Discussion

The LS in R1-2007522 includes the following question:

1. Overall Description: RAN2 discussed the sidelink configured grant handling and would like to ask RAN1 the following question. Question： Is it possible to use the retransmission opportunities for initial transmission for a sidelink configured grant in case when the data was not available for the transmission opportunity for initial transmission? 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to take the above information into account and provide feedback on the question.

This LS is addressed by contributions submitted to AI 5 as well as to AI 7.2.4. All of them but one propose to reply to RAN2 that, from RAN1 perspective it is possible to use the retransmission opportunities for initial transmission for a sidelink configured grant in case when the data was not available for the transmission opportunity for initial transmission.

Several contributions include (at least) the following two agreements to support their conclusion:

Agreements: - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB. - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. - UE decides which TB to transmit in each of the occasions indicated by a given configured grant. - FFS: whether different transmissions of a TB can take place across multiple configured grants. - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2. Agreements: - Only one new TB can be transmitted in one period of the configured grant. - FFS any issue with retransmission spanning multiple periods - The DCI scheduling the retransmissions uses the HARQ process ID corresponding to the first transmission of the TB, as agreed for Q2.

Proposal:

- Reply to RAN2 that from RAN1 perspective it is possible to use the retransmission opportunities for initial transmission for a sidelink configured grant in case when the data was not available for the transmission opportunity for initial transmission

Is the proposal acceptable? If not, please state which part of the specification or which of the existing agreements are violated, or whether this is a new agreement:

| Company | View |
| --- | --- |
| Ericsson | Agree |
| OPPO | Agree |

References

AI 5

R1-2007522	LS to RAN1 on sidelink configured grant handling	RAN2, Huawei

R1-2007920	[DRAFT] Reply LS to RAN1 on sidelink configured grant handling	ZTE, Sanechips

R1-2008120	Draft reply LS to RAN1 on sidelink configured grant handling	Samsung

R1-2008593	Draft reply LS on LS to RAN1 on sidelink configured grant handling	Qualcomm Incorporated

R1-2008648	Draft Reply LS on sidelink configured grant handling	vivo

R1-2008746	Discussion on RAN2 LS on SL CG handling	Ericsson

R1-2008747	[Draft] LS reply on SL CG handling	Ericsson

R1-2008781	On sidelink configured grant handling	Huawei, HiSilicon

AI 7.2.4

R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.