3GPP TSG RAN WG1 Meeting #102-E					       R1-20zzzzz

E-meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	Outcome of RAN WG1 E-mail Discussion [103-e-NR-Pos-01]

Agenda item:		7.2.8

Document for:		Discussion and Decision

- Introduction

In this contribution, we provide summary of the RAN WG1 e-mail discussion [103-e-NR-Pos-01]. This e-mail discussion is organized to resolve remaining open aspects identified in the submitted contributions [1]-[4] for NR Positioning Maintenance agenda item as reported in [5]:

- [103-e-NR-Pos-01] Email discussion/approval on DL PRS on aspects 2, 3, 13 (to capture RAN1 agreement only), 14, 16, in the FL summary until 10/29 with potential CRs by 11/4– Alexey (Intel)

- List of Remaining Opens on NR Positioning

## Aspect #2: On DL PRS periodicity and DL PRS processing capability

In [Huawei, [1]], it is discussed how to select K considering the RAN4 definition of DL PRS report delay which is dependent on UE reported capability N, T and the calculated K. The following is proposed:

K is selected to be the maximum PRS periodicity among the DL PRS resource sets in a positioning frequency layer.

- Capture it in RAN4 spec or in RAN1 spec

- In case it is required to be captured in the RAN4 spec, send an LS to RAN4.

- In case it is required to be captured in the RAN1 spec, agree the draft CR in R1-2008789, where the following reasons for change are provided:

  - The P-msec window selection is undefined in the specification, which results in ambiguity of K-msec PRS duration calculation, as a different P corresponds to a different K. It will further result in ambiguity in calculation of the PRS measurement latency requirement defined in TS 38.133. The selection of P-msec is non-trivial as PRS resource (sets) on a positioning frequency layer can have various periodicities, and it should be clarified which periodicity should be used for the selection of P.

  - The specification number referenced for PRS processing capability is not correct, as LPP capabilities are not captured in TS 38.306, but in TS 37.355.

#### 5.1.6.5 PRS reception procedure ============================== Unchanged parts ============================== For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS37.355]. For the purpose of DL PRS processing capability, the duration K ms of DL PRS symbols within any P ms window corresponding to the maximum PRS periodicity in a positioning frequency layer, is calculated by - Type 1 duration calculation with UE symbol level buffering capability K=s∈SKs Ks=Tsend-Tsstart - Type 2 duration calculation with UE slot level buffering capability K=12μS ============================== Unchanged parts ==============================

## Aspect #2 – Discussion Round #1

Companies are invited to provide views on the text proposal above to address discussion aspect #2:

| Company Name | Comments |
| --- | --- |

## Aspect #3: On handling DL PRS periodicity which is not LCM

In [Huawei,[1]], it is also proposed that “UE is not expected to handle the case that the maximum PRS periodicity in a positioning frequency layer is not the least common multiple of the periodicities in the positioning frequency layer”.

## Aspect #3 – Discussion Round #1

Companies are invited to provide views on proposal below to address discussion aspect #3:

Proposal:

UE is not expected to handle the case that the maximum PRS periodicity in a positioning frequency layer is not the least common multiple of the periodicities in the positioning frequency layer.

| Company Name | Comments |
| --- | --- |

## Aspect #13: DL PRS Processing Priority

The following TPs was provided in [LGE, [3]] aiming to reflect RAN1 agreements on DL PRS processing order.

#### 5.1.6.5 PRS reception procedure ---- Unchanged parts omitted ---- When a UE is configured with a number of PRS resources beyond its capability, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo-r16 being the highest priority for measurement, and the following priority is assumed. - The 64 TRPs per frequency layer are sorted according to priority, - The 2 sets per TRP of the frequency layer are sorted according to priority,

## Aspect #13 – Discussion Round #1

Text proposal above for the TS 38.214 aims to reflect the following RAN WG1 agreements

Agreement: - When a UE is configured in the assistance data of a positioning method with a number of PRS resources beyond its capability (FG 13-2,13-3,13-4 for AoD, TDOA, MRTT respectively), the UE assumes the DL-PRS Resources in the assistance data are sorted in a decreasing order of measurement priority. Specifically, according to the current RAN2 structure of the assistance data, the following priority is assumed: - FFS: the 4 frequency layers are sorted according to priority, - The 64 TRPs per frequency layer are sorted according to priority, - The 2 sets per TRP of the frequency layer are sorted according to priority, - FFS: The 64 resources of the set per TRP per frequency layer are sorted according to priority. - The reference indicated by nr-DL-PRS-ReferenceInfo-r16 for each frequency layer has the highest priority at least for DL-TDOA

Companies are invited to provide views on the text proposal above to address discussion aspect #13:

| Company Name | Comments |
| --- | --- |

## Aspect #14: Reference Correction in the TS 38.211

In [Ericsson, [4]], it was identified that “In 38.211, the clause referenced for description of how a DL PRS resource is transmitted in incorrect. Change the incorrect reference to clause 5.1.6.4 in 38.214 to 5.1.6.5.”

##### 7.4.1.7.4 Mapping to slots in a downlink PRS resource set ---------------------------------------------Unchanged parts are omitted---------------------------------------------------- For a downlink PRS resource in a downlink PRS resource set configured, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.5 of [6, TS 38.214]. ---------------------------------------------Unchanged parts are omitted----------------------------------------------------

## Aspect #14 – Discussion Round #1:

Companies are invited to provide views on TP above to address discussion aspect #14:

| Company Name | Comments |
| --- | --- |

## Aspect #16: RSTD Measurement on Multiple DL PRS Resources

In [OPPO, [2]], it is proposed to change the text when UE performs multiple measurements on DL PRS resources with the following reasoning:

“There was an agreement as below, which is not captured in the spec TS 38.214 correctly. The condition (highlighted by Yellow) is for the case of different DL PRS resource ID(s) in the agreement. However, the conditioned is misplaced for the case of a different DL PRS resource set.”

Agreement: The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference

The following TP is provided to correct the existing text.

In TS 38.214 Section 5.1.6.5 <omitted text> The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo-r16. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-expectedRSTD-r16 and nr-DL-PRS-expectedRSTD-uncerainty-r16. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo-r16 may include an dl-PRS-ID-r16, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs. The UE may use different DL PRS resources measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID-r16, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. <omitted text>

## Aspect #16 – Discussion Round #1:

Companies are invited to provide views on TP above to address discussion aspect #16:

| Company Name | Comments |
| --- | --- |

- Outcome of E-Mail Discussion

TBD

- References

- R1-2007574	Rel-16 positioning corrections	Huawei, HiSilicon

- R1-2008215	Text Proposals on RS for Positioning	OPPO

- R1-2008414	Discussions on remaining issues on Rel-16 NR positioning	LG Electronics

- R1-2008760	Corrections to 38.211 for NR positioning	Ericsson

- R1-2009239	Feature Lead Summary for NR Positioning Maintenance AI 7.2.8, Moderator (Intel Corporation, CATT, Ericsson, Qualcomm)