FDD CSI

Study following alternatives, down-select one or more alternatives in RAN1 104e:

Proposal 3-0: Alt 0: Based on  W=W1W2 or  W=W1W2Wf,  W1 can be an identity matrix

Proposal 3-1: Based on  W=W1W2,  study following detailed design of matrices W1, at least for rank 1.

- Alt1: W1∈N PCSI-RS×K1(K1≤ PCSI-RS) is a port selection matrix in order to freely select K1 ports out of PCSI-RS CSI-RS ports or  K12 ports out of  PCSI-RS2 CSI-RS ports  (FFS polarization-common/specific selection) whereas each column of  W1 has only one element of “1”

- Alt2: W1∈N PSD-FD×K2(K2≤ PSD-FD = OfPCSI-RS, , Of≥1) is a SD-FD basis selection matrix in order to freely select  K2 bases out of PSD-FD bases or  K22 bases out of  PSD-FD2 bases (FFS polarization-common/specific selection) whereas each column of  W1 has only one element of “1”

  - FFS the mechanism of conveying more than one SD-FD beamforming bases per CSI-RS port which is to be discussed in Proposal 4

- Note that W1 can be an identity matrix for above Alternative

Proposal 3-2: Based on  W=W1W2WfH,  study following detailed design of matrices  W1 and  Wf , at least for rank 1.

- Alt3: W1∈N PCSI-RS×K1(K1≤ PCSI-RS) is a port selection matrix in order to freely select K1 ports out of PCSI-RS CSI-RS ports or  K12 ports out of PCSI-RS2 CSI-RS ports  (FFS polarization-common/specific selection) whereas each column of  W1 has only one element of “1”, i.e. PSD-FD = OfPCSI-RS and Of≥1

  - Alt3-0 (one SD-FD/SD pair per port):Wf∈CN3× Mv( Mv ≤N3) is a DFT based compression matrix (FFS: configured/indicated to the UE and/or selected/reported by the UE), whereas N3 = NCQISubband*R and  Mv≥1.

  - Alt3-1 (Multi-SD-FD pairs per port):Wf∈CN3× Mv( Mv≤N, N ≤N3) is a DFT matrix selected by the UE from N pre-configured/pre-defined DFT vectors, whereas N3 = NCQISubband*R and  Mv≥1.

    - FFS the mechanism of conveying more than one SD-FD beamforming bases per CSI-RS port which is to be discussed in Proposal 4

    - Note that  Mv=N is not excluded by gNB/codebook configuration.

  - Alt3-2  (Multi-SD-FD/SD pairs per port): Wf∈NK3×M(M≤K3) is a selection matrix in order to select M SD-FD basis whereas each column of  Wf has only one element of “1”,

    - FFS the mechanism of conveying more than one SD-FD beamforming bases per CSI-RS port which is to be discussed in Proposal 4

    - Note that Wf can be an identity matrix

- Alt4: W1∈N Pgroup×K4 (K4 ≤ Pgroup) is a port-group selection matrix to freely select K4  groups out of Pgroup port groups or K4/2  groups out of Pgroup/2 port groups (FFS polarization-common/specific selection) whereas PCSI-RS CSI-RS ports in a resource are divided into Pgroup groups with K5 ports per group, and each port group corresponding to the same SD basis, i.e. PSD-FD = OfPCSI-RS and Of≤1

  - Wf∈NK5×M(M≤K5) is a selection matrix to select the same M ports across all port groups each column of  Wf has only one element of “1”.

- Alt5: W1∈N PSD-FD×K2(K2≤ PSD-FD = OfPCSI-RS, , Of≥1) is a SD-FD basis selection matrix in order to freely select  K2 bases out of PSD-FD bases or  K22 bases out of  PSD-FD2 bases (FFS polarization-common/specific selection) whereas each column of  W1 has only one element of “1”

  - Wf∈CN3× Mv( Mv≤N, N ≤N3) is a DFT based compression matrix (FFS: configured/indicated to the UE and/or selected/reported by the UE), whereas N3 = NCQISubband*R and  Mv≥1..

  - FFS the mechanism of conveying more than one SD-FD beamforming bases per CSI-RS port which is to be discussed in Proposal 4

- Note that W1 can be an identity matrix for above Alternative