#3GPP TSG RAN WG1 #103-e						R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

______________________________________________________________________Agenda item: 8.11.2.2

Source: Moderator (LG Electronics)

Title: Feature lead summary for AI 8.11.2.2 Feasibility and benefits for mode 2 enhancements

Document for: Discussion and information

- Email discussion

As per Chairman’s guideline, the following email discussion was allocated for AI 8.11.2.2. Please provide your view on the questions in Section 1.1/1.2/1.3/1.4/1.5/1.6 by November 4th, 3:00am UTC. Based on the collected view, I’ll make a set of proposals that will be discussed and finalized by November 5th.

      - [103-e-NR-Sidelink-Enh-03] Email discussion/approval for feasibility and benefits for mode 2 enhancements– Seungmin (LGE)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

# 1.1	How to define “A set of resources”?

      - Q1: Do you agree the following proposal?

- When a set of resources determined at UE-A is sent to UE-B, at least the following types of “a set of resources” are supported.

  - Type 1: Resource set which is preferred for UE-B’s transmission

  - Type 2: Resource set which is preferred not to be used by UE-B’s transmission

| Company | Answer | Comment |
| --- | --- | --- |

      - Q2: Which option(s) is(are) used for UE-A to determine “a set of resources”?

- Option 1: The set of candidate resources obtained after Step 7) in TS 38.214

- Option 2: The set of selected resources for UE-A’s PSCCH/PSSCH transmissions

- Option 3: The set of slots where UE-A cannot receive NR sidelink channel/signal

- Option 4: Others (please specifiy it)

| Company | Option | Comment |
| --- | --- | --- |

      - Q3: What is the detailed form to indicate the set of resources?

- Contiguous frequency domain resource vs. Non-contiguous frequency domain resource

- Contiguous time domain resource vs. Non-contiguous time domain resource’

- Reference point to indicate the time domain resource

- Etc

| Company | Comment |
| --- | --- |

# 1.2	When does UE-A transmits “A set of resources” to UE-B?

      - Q1: When does UE-A send “a set of resources” to UE-B?

- Option 1: Based on signaling of triggering or requesting

- Option 2: Based on a pre-defined or (pre)configured triggering condition(s)

- Option 3: Others (please specifiy it)

| Company | Option | Comment |
| --- | --- | --- |

      - Q2: For Option 1 in Q1, how does UE-B transmit the triggering/requesting message to UE-A in terms of container, contents, condition for transmitting it, resource used for its transmission, etc?

| Company | Comment |
| --- | --- |

      - Q3: For Option 2 in Q1, what is the detailed condition when UE-A sends the set of resources to UE-B?

| Company | Comment |
| --- | --- |

# 1.3	How does UE-A transmit “A set of resources” to UE-B?

      - Q1: Which option(s) is(are) used as the container when UE-A transmits “a set of resources” to UE-B?

- Option 1: MAC CE

- Option 2: PC5-RRC signaling

- Option 3: New 2nd SCI format

- Option 4: PSCCH

- Option 5: PSFCH

- Option 6: Others (please specifiy it)

| Company | Option | Comment |
| --- | --- | --- |

      - Q2: How does UE-A transmit the set of resources to multiple UE-Bs?

- Option 1: UE-A individualy transmits the set of resources to each UE-B

- Option 2: UE-A can simultaneously transmit the same set of resources to multiple UE-Bs

- Option 3: Others (please specifiy it)

| Company | Option | Comment |
| --- | --- | --- |

# 1.4	When “A set of resources” provided by UE-A is valid at UE-B side?

      - Q1: When does UE-B determine that “A set of resources” provided by UE-A is valid?

- Option 1: When the SL RSRP measurement based on RS used for transmitting the set of resources is larger than a certain threshold

- Option 2: When the geographical distance between UE-A and UE-B is smaller than a certain threshold

- Option 3: When the received set of resources is not outdated

- Option 4: When the ID(s) associated with the set of resources is matched

- Option 5: When the target cast type is matched

- Option 6: When UE-B has a packet to transmit

- Option 7: Others (please specifiy it)

| Company | Option | Comment |
| --- | --- | --- |

# 1.5	How does UE-B take “A set of resources” into account in the resource selection for its own transmission?

      - Q1: Which option(s) is(are) supported for UE-B to take “A set of resources” into account in the resource selection for its own transmission?

- Option 1: UE-B triggers the resource re-selection

- Option 2: UE-B determines the candidate resources that can be used for its PSCCH/PSSCH transmission, based on its sensing results by using Rel-16 Mode 2 procedure and the set of resource provided by UE-A

- Option 3: UE-B determines the candidate resources that can be used for its PSCCH/PSSCH transmission, based on the set of resource provided by UE-A

- Option 4: Up to UE-B’s implementation

| Company | Option | Comment |
| --- | --- | --- |

# 1.6	Other aspects for “Inter-UE coordination”

      - Q1: In case when it is necessary to discuss/specify additional aspects other than those in Section 1.1/1.2/1.3/1.4/1.5, please specify it in details.

| Company | Comment |
| --- | --- |

- Appendix A - Summary of contributions

      - Q1: When does UE-A trigger or transmit the assistance information?

- Explicit Trigger-based based coordination procedures [2] [3] [4] [6] [7] [11] [12] [13] [16] [18] [20] [21] [22] [36] [23] [24] [27] [28] [30] [14]

  - Explicit triggering message includes the parameters associated with the sensing procedure of the Tx UE [2] [6] [7] [21] [30] [14]

- Event-trigger based coordination procedures [2] [3] [6] [10] [11] [12] [16] [18] [20] [32] [23] [24] [27] [28] [30] [14]

  - Based on observed collisions [3] [6] [10] [12] [18] [32]

  - Based on observed half-duplex problem [10] [12] [32]

  - Based on interference measurement [6] [10] [11] [18]

  - UE-A triggers its resource (re)selection procedure [10]

  - When UE-A transmitting the inter-UE coordination is in proximity [11] [20]

      - Q2: How does UE-A generate the assistance information?

- Definition of “a set of resources”

  - Resource set which is preferred for UE-B’s transmission [1] [2] [3] [4] [6] [7] [10] [11] [13] [15] [16] [18] [20] [21] [32] [36] [23] [24] [27] [28] [30] [37] [38] [14]

  - Resource set which is preferred not to be used by UE-B’s transmission [1] [3] [4] [6] [7] [10] [11] [15] [16] [18] [20] [32] [35] [36] [23] [24] [27] [28] [30] [31] [37] [14]

- Other information in the assistance information

  - Type indicator for a set of resources [4] [10] [16] [18] [23]

  - Associated RSRP [6] [10] [16] [30]

  - Associated RX priority [6] [10] [16] [30]

  - Triggering resource (re)selection [29] [36] [24]

- Consideration aspects when determining the set of resources

  - Hidden-node problem [1] [2] [3] [4] [6] [7] [10] [11] [15] [21] [32] [35] [36] [23] [28] [30] [31] [38] [14]

  - Half-duplex problem [1] [2] [3] [4] [6] [7] [10] [11] [32] [35] [36] [23] [30] [31] [14]

  - Exposed-node problem [1] [2] [3] [6] [11] [15] [14]

    - Objection: [10] [33] [31]

  - Persistent collision [1] [2] [3] [6] [10] [11] [23] [14]

  - Avoiding PSFCH TX/TX or PSFCH TX/RX collisions [4] [10] [35]

  - Near-far problem [35]

      - Q3: How does UE-A transmit the assistance information?

- Container

  - SCI format 1-A [1] [11] [32] [25]

  - New 2nd SCI format [5] [6] [10] [11] [13] [18] [32] [25] [28] [14]

  - MAC CE [6] [10] [32] [24] [25] [37]

  - PC5-RRC [21] [33] [24] [37] [14]

  - PSFCH [24] [25] [36] [29]

- Retransmission of the inter-UE coordination information [10] [32]

      - Q4: When does UE-B use the received assistance information for resource (re)selection procedure?

- Target cast type

  - Unicast [2] [3] [4] [5] [6] [7] [10] [12] [13] [19] [20] [21] [22] [29] [33] [24] [27] [30] [34] [37] [14]

  - Groupcast [3] [4] [6] [10] [12] [13] [19] [20] [21] [22] [29] [33] [24] [27] [30] [34] [37]

  - Broadcast [3] [4] [6] [10] [12] [19] [20] [22] [29] [33] [30] [37]

- When UE-A transmitting the inter-UE coordination is in proximity [6] [10] [18] [35]

  - Based on geographical distance

  - Based on RSRP measurement

- When UE-B triggers resource (re)selection procedure [10]

      - Q5: How does UE-B use the received assistance information for resource (re)selection procedure?

- Combine UE-B’s sensing results and resource set provided from UE-A [2] [3] [6] [9] [10] [11] [16] [17] [18] [29] [24] [28] [30] [34] [14]

  - During determination of candidate resource set [2] [4] [6] [9] [10] [11] [18] [14]

    - Union of preferred resource set based on inter-UE coordination from UE-A and UE-B’s candidate resource set [2]

    - Intersection of preferred resource set based on inter-UE coordination from UE-A and UE-B’s candidate resource set [2] [6] [10]

  - During determination of selected resource set [4] [9] [10] [18] [14]

  - After determination of selected resource set [9]

- Use resource set provided from UE-A directly [2] [3] [4] [16] [17] [18] [30] [34] [14]

  - This is out of scope [10] [29]

- Update excluded resources associated with non-monitored slot at UE-B side based on the inter-UE coordination information [6] [10]

- It is up to UE-B how to use it [13] [21] [37]

      - Further consideration on skipping the transmission of inter-UE coordination information [6] [10] [32]

      - Further consideration on validity check for the inter-UE coordination information [10] [13] [18]

      - Further consideration on how to guarantee the minimum ratio of candidate resources over a resource selection window [10]

      - Further consideration on dedicated resources for inter-UE coordination information and its request [36]

      - Mode 2 enhancements other than inter-UE coordination

- Further consideration that UE-A transmits SL HARQ-ACK feedback on behalf of another UE [6] [36] [23]

- Further consideration that UE-A forwards TB(s) received from another UE [6] [23]

- Further consideration on mixture of blind retransmission and HARQ-ACK feedback-based retransmission [7] [35] [36]

- Further consideration on interference sum to decide the excluded resources [10]

- Further consideration on TX power of UE performing a resource (re)selection procedure [10]

- Further consideration on prioritizing earlier resources for resource selection [26] [35] [36]

- Further consideration on handling half-duplex problem based on the SCI transmitted by the intended RX UE [26]

- Further consideration on groupcast HARQ-ACK feedback enhancement [36]

- Reference

- R1-2007554	Views on resource allocation enhancements for sidelink communication	FUTUREWEI

- R1-2007616	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2007623	Discussion of feasibility and benefits for mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2007689	Discussion on mode-2 enhancements	vivo

- R1-2007771	Inter-UE Coordination Mode 2	Kyocera Corporation

- R1-2007788	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2007834	Discussion on feasibility and benefits for mode 2 enhancements	CATT

- R1-2007880	Enhancement of Mode 2 Latency Performance	ITRI

- R1-2007893	Feasibility and benefits for mode 2 enhancements	TCL Communication Ltd.

- R1-2007896	Discussion on feasibility and benefits for mode 2 enhancements	LG Electronics

- R1-2008032	Discussion on reliability and latency enhancements for mode-2 resource  allocation	CMCC

- R1-2008099	Discussion on feasibility and benefit of mode 2 enhancements	Spreadtrum Communications

- R1-2008190	On Feasibility and Benefits for Mode2 Enhancements	Samsung

- R1-2008240	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2008374	Discussion on reliability and latency enhancements for mode 2	Sony

- R1-2008447	Discussion on Inter-UE Coordination for Mode 2 Resource Allocation	Apple

- R1-2008499	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2008757	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2008820	Views on inter-UE coordination for mode 2 enhancements	Zhejiang Lab

- R1-2008861	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2008879	Inter-UE coordination in mode-2	ZTE Corporation, Sanechips

- R1-2008892	Inter-UE coordination for mode 2 enhancement	ITL

- R1-2008918	Sidelink resource allocation for Reliability enhancement	Lenovo, Motorola Mobility

- R1-2008951	Discussion on feasibility and benefits for mode 2 enhancements	NEC

- R1-2008975	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2008999	Analysis of potential sidelink enhancements targeting Mode 2 reliability and latency	Intel Corporation

- R1-2009022	Discussion on feasibility and benefits for mode 2 enhancements	ETRI

- R1-2009038	Considerations on Mode 2 enhancement for enhanced reliability and reduced latency	Xiaomi

- R1-2009073	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2009122	NR SL Mode 2 enhancement for reliability improvement	InterDigital, Inc.

- R1-2009126	Mode 2 enhancements in sidelink	Panasonic Corporation

- R1-2009127	Discussion on sidelink mode-2 resource allocation enhancements	ROBERT BOSCH GmbH

- R1-2009139	Enhancements of resource allocation Mode 2 for NR sidelink	Sharp

- R1-2009162	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless

- R1-2009194	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2009273	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2009291	Discussion on feasibility and benefits for NR Sidelink mode 2 enhancements	CEWiT

- R1-2009297	Views on feasibility and benefits for mode 2 enhancements	KT Corp.