3GPP TSG-RAN WG1 Meeting #103-e	draft R1- 200NNNN

e-Meeting, October 26th– November 13th, 2020

Agenda Item:	8.5.1

Source:	Moderator (Ericsson)

Title:	Output of  [103-e-NR-ePos-01] Email discussion on additional scenarios for evaluation

Document for:	Discussion, Decision

- Introduction

This document present the outcome of the email discussion triggered under agenda 8.5.1 of RAN1#103e:

[103-e-NR-ePos-01] Email discussion/approval on additional scenarios for evaluation and TR updates until 11/4; address any remaining aspects by 11/12 – Florent (Ericsson)

The proposals in the contributions centered on the following topics, for which discussion tracks are provided:

- Definition of UE based latency

- Requirements for commercial and IIOT use cases

- UE efficiency

- Network Efficiency

- Further modelling assumptions

- Remaining issues

  - Definition of UE based latency

Summary and initial proposal

During RAN1#102e, 3 alternatives where given for the definition of the starting time of physical layer latency for UE based positioning:

- Alt. 1: transmission of the PUSCH carrying the MG Request from the UE.

- Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data

- Alt. 3: Start of the Reception of DL PRS

The following support was expressed in contributions:

- Alt. 1: [8] 13] (UE in RRC connected state) [14]

- Alt. 2: [1] [3] (with modification) [9][12]

- Alt. 3: [13] (UE in RRC idle/inactive state)

Additionally, [1] proposed including the gap request in the physical latency analysis when it is needed. Additional gap are proposed to be taken into account when the RRM measurement gap does not cover the need for PRS measurement gaps [1].

| source | - proposal |
| --- | --- |
| [1] | - Observation 1: The following definition of physical layer latency for UE-based positioning methods is more suited. UE-based; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, transmission of the PDSCH from the gNB carrying the message containing the assistance data; Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE. Observation 4: The following assumptions are suited for evaluating the latency with respect to measurement gaps. Case; PRS periodicity; Whether a gap is needed; CSSF for PRS. No existing RRM gap; Any; Yes; 1. With existing gap; <=20ms; Yes; 2. >=160ms; No; 1. |
| [3] | Proposal 2: Alt.2 is used to define Physical Layer Latency Start time for UE based method, i.e. Method; Start; End. UE-based; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data; Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE. |
| [8] | Proposal 1: For physical layer latency of UE-based method, select Alt1 as the start time when PDSCH carrying LPP request location is not appliable. |
| [9] | Proposal 2: For UE based methods, set the start of the PHY layer latency as the moment of transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data (Alt. 2 from prior agreement). |
| [12] | Proposal 1 : For UE-based positioning, support Alt. 2, “Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data”, as the start time for the physical layer latency analysis |
| [13] | Proposal 1: With regards to the “start time” of the PHY-layer latency for UE-based method initiated by an internal client, - For a UE in RRC connected state, Alt 1 should be considered as the start time - For a UE in RRC Idle/inactive state, Alt. 3 should be considered as the start time |
| [14] | For UE based positioning, the start time for physical latency evaluation is defined asthe transmission of the PUSCH carrying the MG Request from the UE (Alt. 1). |

Based on the submitted contributions, there is no consensus yet on on what alternative to use for the definition of the starting time for physical latency of UE based methods.  It should be noted, however, that the meeting is the last one scheduled for the SI. Hence we should not expect companies to be able to impact their TR results based on the chosen alternative. Thus we propose the following way forward:

Feature lead proposal: For UE based positioning solutions, the definition of physical layer latency starting time is:

- Option1: up companies to chose between alt 1, alt 2 or alt 3 in the agreement from RAN1#102e

- Option 2: to be further discussed during RAN1#103e

Companies are encouraged to provide their views in the table below and state their preference in terms of alt1, alt2, alt3 in case option 2 is chosen.

| Company | Comment |
| --- | --- |
| Futurewei | Option 1 |
| Nokia/NSB | Option 1 |
| OPPO | It is preferred to have a unified definition. We have clear definition of starting time for all the other scenarios, why leaving companies to choose only for this scenaro? |
| ZTE | Option 2. Unified definition is preferred. |
| CATT | Option 2. We prefer to have a common definition of physical latency starting time for UE based positioning method. Alt-2 is our preference (i.e., Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data). |
| Huawei/HiSilicon | We can accept Option 1. In addition, we had the observation on how gap request and gap sharing are connected for latency analysis. We assume that it is also up to each company to report. |

  - Requirements for commercial and IIOT use cases

Summary and initial proposal

During RAN1#101e, an agreement was made on requirements for commercial and IIOT use cases, with open issues to resolve regarding the accuracy and latency requirements as well as the applicable percentile points for these requirements.  During RAN1#102e, the discussion continued but did not converge.

The following views was expressed for the requirements:

Commercial use cases:

- Horizontal accuracy

  - 1m @ 90% of UEs: [2], [3], [8][10] [11] (SL1,2) )[14]

  - 1m @ 85% of UEs: [4]

  - 1m @ 80% of UEs: [6]

- Vertical accuracy

  - 3m @ 90% of UEs: [2], [3], [8], [10], [11] (SL2)

  - 3m @ 85% of UEs: [4]

  - 3m @ 80% of UEs: [6]

  - 2m @90% of UEs [11] (SL1)

- End to end latency

  - 100ms: [2] ,[3], [4], [6], [8] )[14]

- Physical layer latency

  - 10ms: [2] ,[3], [4],

  - 100ms [6] )[14]

  - 25ms [8]

IIOT use cases:

- Horizontal accuracy

  - 0.2m @ 90% of UEs: [2] [3], [4], [6], [7] (for SH), [10], [11] (SL1,2)[14]

  - 0.5m @90% of UEs: [3], [6], [7](for DH), [8], [11] (SL3)

- Vertical accuracy

  - 1m @ 90% of UEs: [2], [4], [6], [7](for DH), [8], [10], [11] (SL2,3) )[14]

  - 0.2m @ 90% of UEs:  [3], [6], [7] (for SH), [11] (SL1)

- End to end latency

  - 100ms: [2] , [3], [4], [7], [8] [11] (SL3) )[14]

  - 10ms: [6], [11] (SL1)

  - 25ms [8]

  - 20ms [11] (SL2)

- Physical layer latency

  - 10ms: [2] , [3], [4], [6], [7], [11] (SL1,2,3) )[14]

| source | proposal |
| --- | --- |
| [2] | Proposal 1: - In Rel-17 the target positioning requirements should be independent of use cases and service level. Proposal 2: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [90%] of UEs - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs - End-to-end latency for position estimation of UE (< [100 ms]) - FFS: Physical layer latency for position estimation of UE (< [10 ms]) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X m) for [90%] of UEs - X = [0.2 or 0.5] m - Vertical position accuracy (< Y m) for [90%] of UEs - Y = [0.2 or 1] m - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms]) - FFS: Physical layer latency for position estimation of UE (< [10ms]) Note: Target positioning requirements may not necessarily be reached for all scenarios. |
| [3] | Proposal 1: Adopt following target accuracy requirements for Rel-17 - For commercial use cases: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - For IIoT use cases: - Horizontal position accuracy (< X m) for 90% of UEs - X = 0.2 or 0.5 m - Vertical position accuracy (< Y m) for 90% of UEs - Y = 0.2 or 1 m |
| [4] | Proposal 1: We prefer the following numbers for Rel-17 target positioning requirements: In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for X1 of UEs - X1 = 85% - Vertical position accuracy (< 3 m) for X2 of UEs - X2 = 85% - End-to-end latency for position estimation of UE (< X3) - X3 = 100ms - Physical layer latency for position estimation of UE (< X4) - X4 = 10ms In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X5) for 90% of UEs - X5 = 0.2m - Vertical position accuracy (< X6) for 90% of UEs - X6 = 1m - End-to-end latency for position estimation of UE (< X7) - X7 = 100ms - Physical layer latency for position estimation of UE (< X8) - X8 = 10ms |
| [6] | - For the accuracy and latency requirements, we suggest defining X1 = 80 %, X2 = 80 %, X3 = 100 ms, X4 = (0.2 m or 0.5 m), X5 = (0.2 m or 1 m), X6 = 10 ms, X7 = 10 ms |
| [8] | Proposal 2: Propose to adopt the following performance requirement for Rel-17 positioning: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< 25 ms) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.5 m) for 90% of UEs - Vertical position accuracy (< 1 m) for 90% of UEs - End-to-end latency for position estimation of UE (<100ms) - FFS: Physical layer latency for position estimation of UE (< 25ms). |
| [10] | Proposal 1: Our preferred positioning accuracy requirements: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90%of UEs - Vertical position accuracy (<3m) for 90% of UEs - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.2 m) for 90%of UEs - Vertical position accuracy (< 1 m) for 90% of UEs Proposal 2: Support different service level in which each level represent both positioning accuracy and latency. Proposal 3: Define the end-to-end positioning latency after receiving the associated LS response from RAN2 as this requirement is beyond RAN1 scope. |
| [11] | Proposal 1: - RAN 1 should adopt following table for accuracy and latency requirements: Table 1. Target positioning requirements for commercial and IIOT use cases Service level Requirement; Commercial– SL1; Commercial– SL2; IIoT - SL1; IIoT - SL2; IIoT – SL3. Horizontal position accuracy; < 1 m for 90% of UEs; < 1 m for: 90% of UEs; <0.2m for 90% of UEs; <0.2m for 90% of UEs; <0.5m for 90% of UEs. Vertical position accuracy; < 2 m for 90% of UEs; < 3 m for 90% of UEs; <0.2m for 90% of UEs; <1m for 90% of UEs; <1m for 90% of UEs. End-to-end latency; <100ms; <100ms; <10ms; <20ms; <100ms. Physical layer latency; <10ms; <10ms; <10ms; <10ms; <10ms. - Note 1: All positioning techniques do not need to achieve the target positioning requirements over all scenarios. - Note 2: End-to-end latency requirements can be changed depending on a response LS from other WGs. |

Based on the submitted contributions, there is no complete consensus on the values to use for requirements. There are however quite a lot of companies agreeing for most of the requirements.

The following proposal is based on the majority of the companies:

Feature lead proposal:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for 90% of UEs

    - X = 0.2 or 0.5 m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y =0.2 or 1 m

  - End-to-end latency for position estimation of UE (<100ms)

  - FFS: Physical layer latency for position estimation of UE (<10ms)

- Note: Target positioning requirements may not necessarily be reached for all scenarios

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Nokia/NSB | We should not rewrite the SID (which has very clear requirements in our view). We already have the note that these may not be reached for all scenarios so don’t see the benefit of having multiple accuracies. |
| OPPO | Considering the different maximal BW in FR1 and FR2, one wayforward is to set the position accuracy as follows: - For FR1: X = 0.5m - For FR2: X = 0.2m |
| ZTE | Agree with the proposal. |
| CATT | We can agree the FL proposal with X=0.2m and Y=1m. It looks like we don’t need multiple values for accuracy, since target positioning requirements may not necessarily be reached for all scenarios. |
| Huawei/HiSilicon | For IIOT horizontal accuracy, we do not think 0.2m is feasible when the product impairment (sync error) is taken into account, and thus 0.5m should be more appropriate to provide guidance on achievable accuracy to the industry. |

  - UE efficiency

Summary and initial proposal

In [1], the UE power model for UE efficiency evaluation is discussed.  A measurement period assumption for RRM is proposed and power components are listed. In [2], a power consumption model is proposed.  [5] underlines that the study of UE power consumption is up to each company.

| source | proposal |
| --- | --- |
| [1] | Observation 5: The following parameters are suited for PRS RRM power evaluation N: Number of TRPs for intra-frequency measurement & search; Synchronous case. FR1; FR2. N=8; 200; 320. Observation 6: Power consumption in RRC_CONNECTED state should consider C-DRX configuration. Observation 7: Power consumption in RRC_IDLE/INACTIVE state should consider the following power components - Synchronization to SSB, and hold-time in micro-sleep to maintain time/frequency synchronicity for further Rx/Tx - Reception of paging DCI in 2 two consecutive slots (Assuming FR1 SSB/RMSI multiplexing pattern 1 with paging search space being SS#0) - Data/RS Rx/Tx in RRC_IDLE/INACTIVE state - DCI Rx following data Tx in INACTIVE state |
| [2] | Proposal 4: - The power consumption model below for PRS measurement should be considered. For frequency layer i, the power of PRS measurement is represented as: Ei=PPRS*NPRS+EMG_switch For Nf frequency layers, the total power is ENf=i=0Nf-1Ei where - Ei is total power over slots over which measurements are carried out in frequency layer i - PPRS is the slot average power for PRS measurements in frequency layer i - NPRS is the number of slot over which measurements are carried out - EMG_switch is the power for measurement gap switching - ENf is total power for Nf frequency layers. |
| [5] | Proposal 1: The study of the UE power consumption should allow flexibility for each company to detail their methodology for evaluation. |

There are several proposals to define power consumption in more details. Given the fact that this is the last meeting in the SI, it will be however difficult to reflect further agreements regarding evaluation assumptions. It is thus proposed to decide whether to detail the assumption for power consumption evaluation further, or leave it to companies to detail their evaluation.

Feature lead proposal: For the study of UE power consumption, the methodology is:

Option 1: use the following power consumption model:

- For frequency layer i, the power of PRS measurement is represented as:

Ei=PPRS*NPRS+EMG_switch

- For Nf frequency layers, the total power is

ENf=i=0Nf-1Ei

where

-    Ei is total power over slots over which measurements are carried out in frequency layer i

-    PPRS is the slot average power for PRS measurements in frequency layer i

-    NPRS is the number of slot over which measurements are carried out

-    EMG_switch is the power for measurement gap switching

-    ENf is total power for Nf frequency layers

- For RRC connected, include C_DRX configuration

- For RRC idle, the following components are considered:

- Synchronization to SSB, and hold-time in micro-sleep to maintain time/frequency synchronicity for further Rx/Tx

- Reception of paging DCI in 2 two consecutive slots (Assuming FR1 SSB/RMSI multiplexing pattern 1 with paging search space being SS#0)

- Data/RS Rx/Tx in RRC_IDLE/INACTIVE state

- DCI Rx following data Tx in INACTIVE state

Option 2: up to each company

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Futurewei | Option 2 |
| Nokia/NSB | We already have the following from last meeting: Conclusion: For power consumption evaluation, it is up to each company to detail their methodology (including power model) for evaluation. We suggest no more discussion on this proposal in this meeting. |
| OPPO | Just as Nokia mentioned, we did already reach a conclusion on the UE efficiency evaluation in RAN1#102e. Why do we discuss it again? |
| ZTE | Option 2. |
| CATT | Option 2. |
| Huawei/HiSilicon | Option 2. |

  - Network Efficiency

Summary and initial proposal

In [2], it is proposed to provide the PRS configuration (PRS comb size, # of symbols, number of TRPs, periodicity, repetition factor, muting pattern, numerology and beam sweeping assumptions) to assess resource utilization.  In [4], a model for the PRS and SRS resource utilization is proposed adding the PRS resources over resource sets, and TRPs for a given gNB.  In [8] it is proposed to use the measurement gap length to periodicity ratio for PRS and the SRS utilization as the definition for resource utilization. In [9], it is proposed to leave it to companies to define the utilization. In [13],[14] it is also proposed to take the measurement gap configuration into account for PRS resource utilization.

| source | proposal |
| --- | --- |
| [2] | Observation 1: - It may not be appropriate to have MG configuration in the evaluation of PRS/SRS resource utilization as a metric of network efficiency considering it is a UE-Specific configuration and does not affect network scheduling. Proposal 3: - For the network efficiency based on PRS, at least the comb size, number of symbols, TRP number, PRS periodicity, resource repetition factor, muting pattern, numerology and beam sweeping assumptions of PRS should be included in resource utilization. - For the network efficiency based on SRS, at least the comb size, number of symbols, UE number, SRS periodicity, numerology and beam sweeping assumptions of SRS should be included in resource utilization. |
| [4] | Proposal 2: The PRS/SRS resource utilization can be defined according to the configuration parameters of PRS/SRS resource as shown in the section 2.2.1and 2.2.2. |
| [8] | Proposal 3: The resource overhead for DL PRS resource reception is calculated as the length of measurement gap / the repetition period of measurement gap. Proposal 4: The resource overhead for SRS resource positioning is calculated as the ratio of symbols used to transmit SRS resource for positioning. |
| [9] | Proposal 1: It is left to individual companies to define the resource utilization for positioning. |
| [13] | Proposal 5: With regards to PRS resource utilization, for PRS processing with MG, the overhead caused by the chosen MG configuration should be considered, e.g. for a given PRS configuration, define the PRS usage as the smallest supported MG length which contains all the PRS resources of an instance. |
| [14] | Network efficiency is reported in the form of the total resource utilization for positioning, including RS signal configuration and MG configuration. Aligned Tx/Rx beams is assumed. |

The contributions submitted propose various way to define efficiency. As with other proposal in this AI, there are several proposals to define network efficiency in more details. Given the fact that this is the last meeting in the SI, it will be however difficult to reflect further agreements regarding evaluation assumptions. It is thus proposed to decide whether to detail the assumption for power consumption evaluation further, or leave it to companies to detail their evaluation. The following options are proposed:

Feature lead summary:  when reporting network efficiency:

Alt1:

For DL PRS efficiency:

- Option1a: comb size, number of symbols, TRP number, PRS periodicity, resource repetition factor, muting pattern, numerology and beam sweeping assumptions

- Option 2a: The resource overhead for DL PRS resource reception is calculated as the length of measurement gap / the repetition period of measurement gap.

- Option 3a: define the PRS usage as the smallest supported MG length which contains all the PRS resources of an instance.

- Option 4a: according to equation 1 below

For UL SRS efficiency:

- Option 1b:  comb size, number of symbols, UE number, SRS periodicity, numerology and beam sweeping assumptions

- Option 2b:The resource overhead for SRS resource positioning is calculated as the ratio of symbols used to transmit SRS resource for positioning.

- Option 3b: according to equation 1 below

Alt2: : It is left to individual companies to define the resource utilization for positioning.

Equation 1:

DL_PRS_RUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain

DL_PRS_RUPRS resource set i, time-domain= NumOfSymbolsPRS resourcePeriodicityPRS resource set i×NumOfSymbolsPer millisecond×NumofPRSResourcesPRS resource set i×RepetitionFactorPRS resource set i

DL_PRS_RUPRS resource set i, frequency-domain=BandWidthPRS resource set iBandWidthSystem×1CombSizeNPRS resource set i

DL_PRS_RUTRP j = i=1NDL_PRS_RUPRS resource set i

DL_PRS_RUgNB x = j=1MDL_PRS_RUTRP j

Equation 2:

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain

SRS_Pos_RUSRS resource set i, time-domain= NumOfSymbolsSRS resourcePeriodicitySRS resource set i×NumOfSymbolsPer millisecond×NumofSRSResourcesSRS resource set i

SRS_Pos_RUSRS resource set i, frequency-domain=BandWidthSRS resource set iBandWidthSystem×1CombSizeNSRS resource set i

SRS_Pos_RUUE j = i=1NSRS_Pos_RUSRS resource set i

SRS_Pos_RUgNB x = j=1MSRS_Pos_RUUE j

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Futurewei | Alt 2 |
| Nokia/NSB | Alt 2 |
| ZTE | Alt 2 |
| CATT | Alt1: For DL PRS efficiency: Option 4a. For UL SRS efficiency: Option 3b. |
| Huawei/HiSilicon | Alt 2 |

  - Further modelling assumptions

Clutter model

Summary and initial proposal

In [2] , it is propose to add an additional clutter parameter. In [5], it is observed that no further modification is required.

| source | proposal |
| --- | --- |
| [2] | Proposal 5: - The clutter parameter {60%, 6m, 2m} should be evaluated to identify the performance gap with NLOS conditions. |
| [5] | Observation 1: There is no further need to modify the existing agreement on the high clutter density scenarios in TR38.857. |

Given the fact that the SI is now on its last meeting, there is no possibility for the SI to be impacted by a decision on the clutted model. It is thus proposed not to further discuss it.

Feature lead proposal: do not discuss further the clutter model assumption for NR positioning enhancements.

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Futurewei | Agree with FL proposal |
| Nokia/NSB | Agree with FL/Futurewei |
| ZTE | Agree with FL’s proposal |
| CATT | Support FL proposal. |
| Huawei/HiSilicon | Support. |

UE and gNB Rx/Tx

Summary and initial proposal

In [5], it is proposed to finalize the model with X and Y values as follow

| source | proposal |
| --- | --- |
| [6] | - Define the tentative timing error parameter X = 5 ns at the gNB side and Y = 10 ns at the UE side - Send LS to RAN4 group and ask for feedback on the proposed tentative values for X and Y |

Since this is the last meeting for this SI, it will be difficult to send an LS to RAN4 and get feedback in time for the conclusion of the meeting. Similarly, defining tentative parameters will not impact the simulation results since companies will not have the time to update results.

Feature lead proposal: timing error parameter X  at the gNB side and Y  at the UE side are provided by companies when evaluating UE  and gNB  RxTx errors.

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Nokia/NSB | Support. |
| ZTE | Support. |
| CATT | Support FL proposal. |
| Huawei/HiSilicon | Support. |

Phase model for PRS and SRS stitching

Summary and initial proposal

In [13], further details are provided to take into account the RF impact on frequency domain stitching for PRS and SRS. It is proposed to model the phase error using a random phase offset for each additional PFL used in the PRS aggregation.

| source | Proposal |
| --- | --- |
| [13], | Proposal 2: At least of the purpose of modelling the effect of phase offset between two PFLs, consider a uniformly distributed phase offset being added in the 2nd PFL with respect to the 1st PFL. Proposal 3: At least of the purpose of modelling the effect of channel spacing between two PFLs, - consider simulating a range of gaps starting from ~1 MHz, to even up to tenths of MHz. It is up to each company to provide the values of gaps being considered. - No need to have an agreed/common algorithm on how to process the scenarios of PFL aggregation with frequency-domain gaps; companies are encouraged to provide a description of the algorithm used. Proposal 4: At least for the purpose of modelling the effect of time drifts between two PFLs, consider defining a phase slope parameter that controls the amount of phase ramp being added in the subcarriers of the 2nd PFL with respect to the 1st PFL. |

As there are no further meetings after RAN1#103 for the SI, it will not be possible for companies to submit new simulations. However, discussing what should be assumed in the error models for PRS and SRS stiching is useful in order to derive meaningful conclusions in the enhancements.

Feature lead proposal: for SRS bandwidth stiching and PRS bandwidth stiching, when modelling the effect of phase offset between two PFLs,  a uniformly distributed phase offset is applied the 2nd PFL with respect to the 1st PFL.

- Companies to provide how the phase offest is modeled in their evaluations.

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Futurewei | Agree with FL proposal |
| Nokia/NSB | Okay. |
| ZTE | Okay. |
| CATT | Support FL proposal. BTW: It looks like there are several typos in the proposal: stiching may be stitching, and offest may be offset. |
| Huawei/HiSilicon | We would like to note that that both zero-phase offset and uniformly distributed phase offset should be considered to model different RF architectures and to model intra-band and inter-band. In addition, we would like to clarify the understanding that each company can still report different values of the range of the uniform distribution. |

Hand blockage losses and transmit power reduction

Summary and initial proposal

In [5], the proposal is confirming an earlier conclusion to leave it to companies to detail their methodology regarding hand blockage and transmit power reduction.

| source | Proposal |
| --- | --- |
| [5] | Proposal 2: For hand blockage losses and/or transmit power reduction, it is up to each company to detail their methodology (including power model) for evaluation. |

Since the proposal from the contribution is confirming an earlier agreement, it is the FL opinion that no further discussion is needed on this aspect.

Companies are encouraged to provide their views in the table below

| Company | Comment |
| --- | --- |
| Nokia/NSB | Agree with FL. |
| CATT | Agree to FL’s opinion. |
| Huawei/HiSilicon | OK |

Conclusion

TBD

References

R1-2007575 Scenarios and evaluation assumption for IIoT positioning, Huawei, HiSilicon

R1-2007664, Discussion on additional scenarios for NR positioning evaluation, vivo

R1-2007753, Remaining issues on evaluation assumptions, ZTE

R1-2007858, Remaining issues on additional scenarios for evaluation of NR Positioning Enhancements, CATT

R1-2007907, Remaining Issues on Evaluation Assumptions, FUTUREWEI

R1-2007944, NR positioning scenarios, Intel Corporation

R1-2008014, Remaining issues on target performance requirement of IIoT use case, CMCC

R1-2008224, Discussion on Additional Scenarios for Evaluation of NR Positioning, OPPO

R1-2008299, Additional scenarios for evaluation of NR positioning, Nokia, Nokia Shanghai Bell

R1-2008363, Considerations on Additional Scenarios for Evaluation, Sony

R1-2008415, Discussions on additional scenarios for evaluation for NR positioning, LG Electronics

R1-2008488, Discussion on evaluation assumptions for latency, InterDigital, Inc.

R1-2008617, Considerations on Additional Scenarios for Evaluation, Qualcomm Incorporated

R1-2008763, Remaining details on additional scenarios for evaluation, Ericsson

RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 2019

[Post111-e][625][POS] End-to-end latency analysis (Intel), RAN2 email discussion