3GPP TSG RAN WG1 #103-e	R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.6.3

Source: 		Moderator (Qualcomm Inc.)

Title: 	FL summary #9 on Coverage Recovery and Capacity Impact for RedCap

Document for:	Discussion and Decision

# Introduction

This contribution summarizes the contributions submitted to AI 8.6.3 (Study on NR reduced capability devices – coverage recovery and capacity impact).

This document captures the following RAN1#103e RedCap email discussion.

[103-e-NR-RedCap-04] Email discussion for coverage recovery and capacity impact– Chao (Qualcomm) - Recovery: email extension till 11/17, focusing on “TR clause 9.1: TP for coverage recovery observation for FR2 indoor scenario”

In this round of the email discussion, please check the proposals/questions tagged ‘FL9’ (search for ‘FL9’).

# Coverage Recovery

Agreements: - For FR2 indoor scenario, the representative value is derived based on results for max TRP 12 dBm. The aggregated value for UL channels has then been obtained by considering - Results presented by companies assuming max TRP 12 dBm; and - Results presented by companies assuming max TRP 23 dBm, where corresponding MIL values have been reduced by 11 dB, and each company is counted only once (no double value is considered, if any). Agreements: - Capture the following observations for FR2 coverage recovery to the TR 38.875 - For FR2, there is no assumption of reduced antenna efficiency for RedCap UE and the MIL of the UL channels is the same as the reference NR UE and coverage recovery for UL channels is not needed. - [For RedCap UE with 100 MHz BW and 1Rx, although there is performance loss from reducing the number of Rx branches to 1, the MIL(s) of all the DL channels is better that that of the bottleneck channel for the reference NR UE and coverage recovery for DL channels is not needed. ] - For RedCap UE with 50MHz BW and 1Rx, coverage recovery may be needed for PDSCH when the same target data rate as the reference NR UE is assumed, and the amount of coverage recovery to be considered is approximately [2-3 dB] - The tradeoff between data rate and coverage can be considered and the amount of coverage recovery may depend on this choice. - The determination of which channels require coverage recovery and the amount of coverage recovery depend on the choice of the target for coverage recovery - E.g. coverage recovery may not be needed for FR2 indoor scenario when the target is based on an MPL value from a target ISD of 20m - E.g. a large amount of coverage recovery may be needed for the initial access channels if the target is to achieve the same coverage for the initial access channels between RedCap UE and the reference NR UE

[FL8] Proposal 3.4-2:

- Observations for FR2 coverage recovery can be drawn separately for max TRP 12 dBm and max TRP 23 dBm

| Company | Y/N | Comments |
| --- | --- | --- |
| vivo | N | We prefer to use the same way as in CE, i.e. based on TRP 12dBm, otherwise, it may be inconsistent between different items. |
| Samsung | Y | Separate observations seem reasonable. |
| Futurewei | N | Prefer to have one assumption on max TRP |
| Intel | Y | The main reason to go with single observation based on 12dBm is to align with CE. However, RedCap is quite different from CE. - For CE, it is to enhance the coverage of UE using maximum 12dBm TX power, while the coverage of UE with 23dBm max TX power is maintained (i.e. at least not worse for reference UE). - On the contrary, if we only consider RedCap UE with max 12dBm Tx power, it means such UE can be enhanced to be no worse than reference UE. However, a RedCap UE with max 23dBm TX power is still worse than reference UE. Such a problem should be avoided. |
| Ericsson | Y | We are fine either way. |
| Qualcomm | Y |  |
| Nokia, NSB |  | We have no strong view here and are fine either way. |
| MediaTek | Y |  |
| Huawei, HiSilicon | N | Prefer to align with CovEnh SI. |

Based on the received responses for [FL8] Proposal 3.4-2, there is no consensus to support separate observations for max TRP 12 dBm and 23 dBm. From FL’s understanding, the max TRP value will also affect the reference NR UE since we use the same Tx power for both RedCap and the reference NR UE. We will observe a different bottleneck channel for the reference NR UE with max 12 dBm and 23 dBm TX power and the corresponding MIL value for coverage recovery target is also different. That is, the current observation that the DL channels of the RedCap UE is not coverage limited as compared to the bottleneck channel of the reference NR UE is due to max 12 dBm Tx power used for the reference NR UE. However, we have not captured such information clearly in the observations.

Considering that we have agreed to use max TRP 12 dBm, the FL’ suggestion is to adopt the TP for max TRP 12dBm but update the observation to clarify the max TRP assumption used for the reference NR UE.

[FL9] Proposal:

- Adopted the updated TP in section 3.4 of R1-2009722 as baseline text for TR clause 9.1

- Adopt the following update to observations for FR2 indoor coverage recovery

- Capture the following observations for FR2 coverage recovery to the TR 38.875 - For FR2, there is no assumption of reduced antenna efficiency for RedCap UE and the MIL of the UL channels is the same as the reference NR UE and coverage recovery for UL channels is not needed. - [For RedCap UE with 100 MHz BW and 1Rx in FR2 indoor scenario, although there is performance loss from reducing the number of Rx branches to 1, the MIL(s) of all the DL channels is better that that of the bottleneck channel for the reference NR UE, for which max TRP 12dBm is assumed, and coverage recovery for DL channels is thus not needed.] - For RedCap UE with 50MHz BW and 1Rx, coverage recovery may be needed for PDSCH when the same target data rate as the reference NR UE is assumed, and the amount of coverage recovery to be considered is approximately [2-3 dB] - The tradeoff between data rate and coverage can be considered and the amount of coverage recovery may depend on this choice. - The determination of which channels require coverage recovery and the amount of coverage recovery depend on the choice of the target for coverage recovery and/or max TRP for the reference NR UE - E.g. coverage recovery may not be needed for FR2 indoor scenario when the target is based on an MPL value from a target ISD of 20m - E.g. a large amount of coverage recovery may be needed for the initial access channels if the target is to achieve the same coverage for the initial access channels between RedCap UE and the reference NR UE - E.g. coverage recovery for DL channels may be needed for RedCap UE with 100 MHz BW and 1Rx when max TRP 23 dBm is assumed for the reference NR UE

If not, what modification is needed?

| Company | Y/N | Comments |
| --- | --- | --- |

# References

- R1-2008865	Coverage recovery and capacity impact for RedCap	Ericsson

- R1-2007536	Coverage recovery for RedCap	FUTUREWEI

- R1-2008813	Functionality for coverage recovery, Huawei, HiSilicon

- R1-2007670	Discussion on coverage recovery, capacity and spectrum efficiency impact, vivo, Guangdong Genius

- R1-2007717	Discussion on coverage recovery for RedCap UE	ZTE

- R1-2007864	Coverage recovery for reduced capability NR devices	CATT

- R1-2007889	Coverage recovery and capacity impact	TCL Communication Ltd.

- R1-2007949	On coverage recovery for RedCap UEs	Intel Corporation

- R1-2009217	Coverage Recovery and Capacity Impact	Panasonic Corporation

- R1-2008018	Discussion on coverage recovery for RedCap UEs	CMCC

- R1-2008050	Discussion on the coverage recovery of reduced capability NR devices	LG Electronics

- R1-2008070	Functionality for coverage recovery	Nokia, Nokia Shanghai Bell

- R1-2008086	Discussion on coverage recovery for reduced capability device	Xiaomi

- R1-2008102	Discussion on coverage recovery and capacity impact	Spreadtrum Communications

- R1-2008172	Coverage recovery for low capability device	Samsung

- R1-2008262	Discussion on coverage recovery issues and evaluation	OPPO

- R1-2009173	Coverage recovery for RedCap	Lenovo, Motorola Mobility

- R1-2008367	Coverage recovery for Redcap devices	Sony

- R1-2008396	Coverage recovery for reduced capability UEs	Sharp

- R1-2008472	Functionality for Coverage Recovery for RedCap	Apple

- R1-2008512	Discussion on coverage recovery for NR RedCap UEs	MediaTek Inc.

- R1-2008518	On coverage recovery for reduced capability UEs 	Convida Wireless

- R1-2008553	Discussion on coverage recovery for RedCap	NTT DOCOMO, INC.

- R1-2009310	Coverage Recovery for RedCap Devices	Qualcomm Incorporated

- R1-2008686	Coverage recovery for reduced capability NR devices	InterDigital, Inc.

- R1-2008728	Discussion on Coverage Recovery for RedCap UE	WILUS Inc.

- R1-2008740	Coverage recovery for RedCap UE	Sequans Communications

# Appendix