3GPP TSG RAN WG1 Meeting #103-E					    	 R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Intel Corporation)

Title:	Moderator summary #3 on RedCap - Others

Agenda item:	8.6.5

Document for:	Discussion and Decision

- Introduction

This document presents a summary of submitted contributions to AI 8.6.5 (Other aspects of RedCap) and some contributions to AI 8.6.4 that have discussions relevant to RedCap UE identification and access control.

Based on the submitted contributions to RAN1 #102-E meeting, the discussion points are categorized into the following topics:

- Access control

- Identification of RedCap UEs by the NW

- RRM relaxations for power savings

In this version of the summary, a new sub-section (3.1) has been added with new questions marked in Turquoise (e.g., Questions 3.1.1-F/N/P, etc.). To better focus the discussion, the analysis in Sub-section 3.1.4 on Opt. 4 will be updated further once discussion on Options 1 through 3 are a bit stable.

- Access control

The SID on RedCap lists the following objectives:

Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

During RAN1 #102-e meeting, the following was discussed in RAN1 without conclusion [29]:

Updated FL Proposal 1 (from RAN1 # 102-e) - Further study the options to realize cell barring for RedCap UEs, including at least the following indication methods: - Implicit or explicit indication (as may apply): - Alt. A: Via separate SSB and/or CORESET 0. - Alt. B: Via indication in MIB. - Alt. C: Via indication in DCI format scheduling SIB1. - Alt. D: Via indication in SIB1. - Other methods are not precluded. Note: This study intends to establish feasibility of, and pros and cons for the identified methods from RAN1 perspective, without any intention of down-selection without guidance from RAN2.

In the meantime, RAN WG2 decided on the following:

- An indication in system information is needed to indicate whether a REDCAP UE can camp on the cell. FFS whether the indication is explicit or implicit. - UAC mechanism also apply to REDCAP UEs. - System information indicates whether REDCAP operation is allowed/barred on a frequency. FFS reuse the legacy intraFreqReselection or introduce separate flag - Further discuss enhancement of UAC for REDCAP UEs, including e.g.: - define new Access Identity for REDCAP UEs - define new Access Categories for REDCAP UEs (for any final decision we need to check with SA1 and/or CT1)

Some contributions ([5], [9], [10], [17]) have presented discussions on access control mechanisms beyond the above-quoted RAN WG2 agreement, highlighting potential benefits from additional/supplementary mechanisms beyond RAN2 agreement of indication of support of RedCap UEs in a cell via SIB1 – specifically, via use of PDCCH/DCI format scheduling SIB1 PDSCH. However, given the agreement in RAN WG2, whether additional mechanisms are necessary or warranted is not obvious at this point (e.g., Proposal 2 in [19]).

The issue of access control is being discussed further in RAN WG2, and there exists various dependencies on decisions related to, e.g., configuration of separate DL/UL initial BWPs for RedCap UEs, support of a separate SIB1 for RedCap UEs, etc., some of which needs to rely on decisions in RAN WG2. Thus, at this point, it may be prudent to wait for RAN WG2 to make further progress, especially considering the related discussions during RAN1 #102-e meeting. This view (to wait for further progress in RAN WG2) is expressed by several contributions as well (e.g., [2], [6], [19]).

FL Proposal 1

- On the issue of access control for RedCap UEs, RAN1 to wait for further progress in RAN2.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| vivo | Y |  |
| Panasonic | Y |  |
| LG | Y | RAN1 specific solution such as using DCI could be discussed in RAN1 as well. |
| Futurewei | Y |  |
| Qualcomm | Y |  |
| Convida | Y | Based on our understanding, RAN2 agreements do not preclude considering RMSI-PDCCH. Therefore, we think that RMSI-PDCCH can be still discussed in RAN1 pending RAN2 progress. |
| ZTE |  | Use of PDCCH/DCI format scheduling SIB1 PDSCH can bring additional power saving. Since this is the last meeting in RAN1, as proposed, waiting further progress in RAN2 means RAN1 related solution is precluded even though it is beneficial. Since use of PDCCH/DCI format scheduling SIB1 (Alt C) is within RAN1 scope and has no impact on RAN2 spec, we hope the feasibility and benefit of Alt C can be discussed in RAN1 in this meeting. |
| CATT | Y |  |
| Huawei, HiSilicon | N | RAN1 can continue the discussion from RAN1 perspective. There is no need for any specific conclusion. Particularly one thing we may be decide within SI is that whether the network capability for supporting RedCap UE accessing should be indicated to UEs or not. With this high level conclusion it would help RAN2 signalling design in later WI phase. Moderator: This seems already agreed now according to RAN2 decision quoted above. |
| OPPO | Y |  |
| Samsung | Y |  |
| CMCC | Y | Same view with LG, some RAN1 specific solution can be discussed parallel with RAN2 |
| DOCOMO | Y |  |
| Lenovo, Motorola Mobility | Y | To us, the RAN2 agreement is clear. There is an indication in system information to indicate if RedCap UEs could access the cell. |
| Nokia | N | Similar opinion to Huawei, CMCC, LG. RAN1 can continue the discussion of access control from the RAN1 perspective. |
| WILUS | Y |  |
| Xiaomi | N | RAN1 could discuss this issue in parallel |
| Spreadtrum | Y |  |
| Ericsson | Y | In our view, RAN1 should continue to prioritize study of other aspects of the study item and leave the study of access control to RAN2. |
| Apple | Y | Support FL proposal as access control procedure is typically handled by RAN2 and maintained in RAN2 specification. Nothing is special for Redcap study item. Should leave to RAN2. |

Summary (FL Prop 1):

- 16 companies indicate that they are fine for RAN1 to wait for RAN2 to make further progress on access control.

  - However, 3 out of these 16 also indicate that they think RAN1 can also discuss any Layer 1 signaling solutions in parallel.

- 4 companies do not agree with the proposal and would like to study Layer 1 signaling solutions.

The RAN2 agreement says:

An indication in system information is needed to indicate whether a REDCAP UE can camp on the cell. FFS whether the indication is explicit or implicit

Given that there is an FFS on whether the indication is explicit or implicit, and the fact that in the recent RAN2 email discussion (RAN2 email discussion #914) almost all companies seem to have indicated that details beyond the above agreement should be considered during WI phase, the option of using DCI format/PDCCH scheduling system information is not precluded and can still be considered during the normative phase.  For reference, the RAN2 email discussion rapporteur’s recommendation based on received feedback is quoted below:

Proposal 8: Postpone the discussion on the camping indicator for RedCap UEs to the WI phase.

At this point, from RAN1’s perspective, no solution is being ruled out, and given that at least 13 companies feel that we can work out the next level of details during the WI phase, and this is not critical to the conclusion of the SI, it is recommended to postpone discussion of exact signaling solutions associated with system information to a latter phase.

The proposal is now updated as below to better align with the proposal being tabled in RAN2 and to clarify the scope further in light of the RAN2 decisions.

FL Proposal 1A

- For access control for RedCap UEs, detailed signaling options associated with system information are postponed to the WI phase.

- Identification of RedCap UEs

The SID on RedCap lists the following objectives:

Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

As can be seen from the above, it is imperative that RedCap Ues can be identified by the network without ambiguity. Here, it is noted that such identification is relevant primarily in the context of accessing a cell (as against merely camping on a cell).

In contributions [2], [4], [5], [8], [10], [12], [13], [14], [15], [16], [17], [18], [19], [20], [21], [22] views on realizing the objective of access control for RedCap Ues have been presented. Please refer to Appendix A for list of observations and proposals from these contributions related to identification for RedCap Ues by the NW.

During RAN1 #102-e meeting, the following was agreed [29]:

Agreements: - Further study the options for identification of RedCap Ues, including at least the following indication methods: - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning. - Opt. 2: During Msg3 transmission. - Opt. 3: Post Msg4 acknowledgment. - E.g., during Msg5 transmission or part of UE capability reporting. - Opt. 4: During MsgA transmission (subject to support of if 2-step RACH) - Other options are not precluded. - Note: This study intends to establish feasibility of, and pros and cons for the identified options from RAN1 perspective, without any intention of down-selection without guidance from RAN2.

In general, how early such identification needs to be made depends on the physical layer procedures for RedCap Ues regarding random access and whether there may be differences for RedCap Ues compared to regular NR Ues. In case different handling of RedCap Ues is required for random access, then early identification either via Msg1 or Msg3 may be necessary.

Some cited motivations for early identification of RedCap Ues include:

- Coverage: Different coverage performance for RAR and/or Msg4 for RedCap Ues compared to regular NR Ues  this may necessitate different scheduling approaches for Msg2/Msg4 and Msg3 (use of repetitions, etc.).

  - Even if Msg2/Msg3/Msg4 may not necessarily need coverage enhancements/recovery, the knowledge that a UE initiating a random-access procedure is a RedCap UE can be beneficial in realizing appropriate DL and UL link adaptation for Msg2/Msg3/Msg4 scheduling as well. However, the trade-off against increased OH, RACH capacity loss, etc.

- Initial UL BWP congestion: Limitations to non-RedCap Ues by limiting max BW for initial UL BWP (e.g., 20 MHz in FR1, 50 MHz in FR2 (if supported))

  - Separation of initial UL BWPs for RedCap Ues from non-RedCap Ues could alleviate the restrictions that may otherwise impact non-RedCap Ues.

  - However, since the initial UL BWP could be limited to random access related (Msg1, Msg3) transmissions, the overall adverse impact to non-RedCap Ues may not be significant as the non-RedCap Ues could be switched to larger UL BWPs upon RRC connection establishment.

- Initial DL BWP congestion: Limitations to non-RedCap Ues by limiting max BW for initial DL BWP (e.g., 20 MHz in FR1, 50/100 MHz in FR2)

  - This impact is primarily limited to Msg4 scheduling for non-RedCap Ues, and UE identification via Msg1 or Msg3 could facilitate more flexible resource allocation for Msg4 scheduling for non-RedCap Ues.

- Minimum UE requirements impacting RA procedure: It may be necessary to identify RedCap Ues at Msg1 transmission if minimum UE processing times for RedCap Ues are relaxed compared to Capability #1 values or requirements on UL waveform are relaxed (e.g., SC-FDM only) for RedCap Ues, etc.

  - In the absence of such knowledge upon reception of Msg1, the gNB may be forced to schedule RAR/Msg3/etc. conservatively in terms of minimum UE processing times or waveform support, thereby potentially causing adverse impact to non-RedCap UE’s control plane (CP) latency.

On the other hand, if RedCap Ues can perform random access procedure like regular NR Ues, it may be sufficient if RedCap Ues are identified via Msg3 or even via Msg5 (upon connection establishment).

In summary, the necessity of early identification may not be clearly established until further clarity is achieved on related design choices for RedCap UE cost/complexity reduction and whether any new mechanisms related to initial access and system information acquisition are introduced (e.g., separate SIB1 for RedCap Ues, separate initial DL/UL BWPs for RedCap Ues, etc.). This is further complicated by the fact that such UE identification may not be limited to identifying RedCap Ues from non-RedCap ones, but also further identification of RedCap Ues.

Further, it has also been pointed out in several contributions that configurability of some of the features, e.g., separate RACH preambles, separate RACH resources, or separate initial UL BWP for RedCap Ues, may be considered further to realize some of the above-listed benefits for deployments that may merit from such enhancements. In other words, early identification of RedCap Ues (prior to UE capability reporting) may not be mandated but could be optionally configured by the gNB towards achieving the best trade-off between access latency, user capacity, and system OH, congestion control, and resource utilization in both DL and UL. This also implies that at least RedCap UE identification as part of UE capability reporting should be supported.

Thus, at this point, RAN1 could focus on establishing the feasibility of and identifying the pros and cons for the various options for RedCap UE identification while further details on RedCap UE capabilities/minimum requirements and related system design details attain further clarity.

FL Proposal 2

- As a next step, during RAN1 #103-e meeting, RAN1 to focus on establishing feasibility and identifying pros and cons for the following schemes:

  - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning.

  - Opt. 2: During Msg3 transmission.

  - Opt. 3: Post Msg4 acknowledgment.

    - E.g., during Msg5 transmission or part of UE capability reporting.

  - Opt. 4: During MsgA transmission

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| vivo | Y |  |
| Panasonic | Y |  |
| LG | Y |  |
| Futurewei | Y | Proposal 2 can include necessity as well. Per RAN2 decision, minimum fragmentation is needed, so for this discussion we should have only one option per band (no mixing of RedCap Ues using different initial access methods in the same band). Market fragmentation needs to be avoided. |
| Qualcomm | Y |  |
| Convida | Y |  |
| ZTE |  | Considering that Option 1 and Option 4 are mostly RAN1 related, RAN1 needs to focus on the study of necessity of identification of RedCap UE by Msg1/MsgA. |
| CATT | Y | We also suggest separating ‘via separate initial UL BWP’ from Opt.1 (maybe another new option is needed). When discussing whether Msg1/3/5 is used to identify RedCap UE from normal NR UE, it is more common to assume that they share the same initial UL BWP. Some companies may prefer separate initial UL BWP but not using Msg1 division, and vice versa. BTW it seems a typo above the listed agreement? It was reached in RAN1#102-e, but not RAN1 #103-e. Moderator: |
| Huawei, HiSilicon | Almost | As discussed during GTW, it is desirable to have some narrow down. Even not to down select to a single option but can still be helpful to narrow the scope for WI if there are multiple. However, in any case we don’t think the discussion here would directly lead to conclusion related to UE type – which would need to be used mainly for the purpose of differentiation from normal Ues as required in SID rather than differentiation among RedCap use cases. We suggest - As a next step, during RAN1 #103-e meeting, RAN1 to focus on establishing feasibility and identifying pros and cons, and strive for narrowing down the possible solution(s) for early identification of RedCap Ues from for the following schemes: - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning. - Opt. 2: During Msg3 transmission. - Opt. 3: Post Msg4 acknowledgment. - E.g., during Msg5 transmission or part of UE capability reporting. - Opt. 4: During MsgA transmission |
| OPPO | Y | Consider the early identification of RedCap Ues, the options can be further down-selected in RAN1. |
| Samsung | Y |  |
| CMCC | Y | Same view as CATT to separate initial UL BWP from option 1, because if the separate initial UL BWP is used for RedCap UE accessing, there is no need to use separate PRACH/preamble as early identification. |
| DOCOMO | Y |  |
| Lenovo, Motorola Mobility | Y | We need list the motivations specific for each option, and see if any conclusions could be drawn for the motivations. |
| Nokia | Almost | As per comments made in the GTW session, we should be striving for more than just feasibility and pros and cons, i.e. a recommended option. |
| WILUS | Y |  |
| Xiaomi | Y | Further down-selection can be performed to narrow down the candidates |
| Spreadtrum | Y |  |
| Ericsson | Y | In our view, the study of feasibility already includes the study of necessity (i.e., if the system does not work when the RedCap UE indication is in MsgX, RedCap UE indication in MsgX is not a feasible solution), but it is fine to spell it out in the proposal. Note that the necessity of a RedCap UE indication option may not be a binary question, since it may be scenario dependent. As mentioned in the background text above the proposal, it may be desired to let the network decide which option to use. |
| Apple | Y |  |

Summary (FL Prop 2):

- There seems to be reasonable consensus on the intent of the proposal.

- Some companies suggest to explicitly capture “necessity/motivation” in addition to “feasibility and identifying pros and cons”.

  - Necessity was always intended to be included as part of the proposed study but it is now reflected explicitly in the updated version of the proposal.

- Couple of companies suggest that we should at least strive for some level of pruning of the list of options during the SI phase itself.

  - We could strive to narrow down the list of solutions. However, this may not be quite practicable given that at least Options 2 and 3 cannot be decided by RAN1 without RAN2 consideration.

  - However, RAN1 should certainly aim for eventual down-selection that could even be captured as observations from RAN1 perspective, based on a well-rounded summary of the different options.

- Couple of companies suggest to split the option of using separate UL BWP from Option 1 in order to not confuse with Msg1 partitioning options

  - Here, it should be noted that Option 1 is not saying “Msg1 partitioning”, but says “During Msg1 transmission”, and for the sub-option of separate UL BWP configuration, Msg1 partitioning could indeed be avoided.

  - Although not spelled out, it should be clear that variants, similar to the sub-options for Option 1, are also applicable for Option 4.

The proposal is revised in consideration of the above points and with addition of an important context that got left out in the original proposal.

FL Proposal 2A

- As a next step, for the study on the options for RedCap UE identification during RAN1 #103-e meeting, RAN1 to focus on establishing feasibility, necessity, and identifying pros and cons for the following schemes:

  - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning.

  - Opt. 2: During Msg3 transmission.

  - Opt. 3: Post Msg4 acknowledgment.

    - E.g., during Msg5 transmission or part of UE capability reporting.

  - Opt. 4: During MsgA transmission.

FL Proposal 3

- Under the assumption that RedCap UEs may be identified at least via UE capability reporting:

  - Optional configurability of separate resources for Msg1/MsgA transmissions for RedCap UEs via use of separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning for early identification of RedCap UEs is considered further.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| vivo |  | Main bullet is fine but not sure about the intention of the sub-bullet, seems already covered by option 1 of FL proposal 2? |
| Panasonic |  | This is basically Opt.1. At this moment, it is too early to prioritize any of the options before concluding proposal 2 and discussion on that. |
| Moderator |  | To clarify the distinction w.r.t. Option 1 in FL proposal 2, while FL proposal 2 talks about the general applicability of each of the options for RedCap UE identification, FL Proposal 3 is highlighting a specific point that the mechanism for identification of RedCap UEs via Option 1 can be considered as a supplementary mechanism that can be turned ON/OFF by the gNB. In other words, it may be up to gNB to decide whether to use Option 1/4 or Option 3 (Msg5+). The feasibility of such optionality relies on the main bullet in FL Proposal 3, and there seems to be interest in such consideration at least for Options 1 and 4 (MsgA now added to FL Proposal 3 above for completeness), while that for Option 3 may be a bit less clear. |
| LG |  | We think that early indication to REDCAP UE in MSG1/A is used for gNB to understand REDCAP UE during initial access. Meanwhile, UE capability reporting is normally used for gNB to understand capability of connected UEs. For the time being, it is unclear whether gNB can understand REDCAP UEs without such early indication during initial access. We propose to discuss optionality of early indication during WI phase. |
| Futurewei | Y | This solution has the advantage of being similar to the LTE solution |
| Qualcomm |  | Thanks for drafting this proposal. We are fine with the general description of “Optional configurability of separate resources for Msg1/MsgA transmissions.” It is ok to include “via use of separate PRACH resource, or PRACH preamble partitioning for early identification of RedCap UEs.” However, the configuration of separate initial UL BWP has not been discussed yet for RedCap UE. Therefore, we think it is not necessary to include “via use of separate initial UL BWP” in the FL proposal. Moderator: To be fair, all of these options have been discussed by multiple companies in their contributions. Thus, we Regarding the use of separate PRACH resources, could FL clarify if it covers different PRACH formats (e.g. RedCap UEs use long PRACH preamble of length 839, and eMBB UEs use short PRACH preamble of length 139) and/or different PRACH occasions ? Moderator: Yes, separate PRACH resources can include both occasions and formats. |
| Convida |  | It may be too early to discuss the possibility of allowing the gNB to turn On/Off the UE identifications vis Msg1/MsgA. Moreover, when using MsgA, it seems that sub-bullet precludes other mechanisms that depends on MsgA-PUSCH and only mentions aspects related MsgA-PRACH. |
| ZTE |  | We think this proposal is related to the feasibility of Option 1. RAN1 can study potential solutions to support identification of RedCap UE by Msg1. |
| CATT |  | See also our comments in Proposal 2. We have similar views with Qualcomm that ‘separate initial UL BWP’ is different from other methods in Option1. Configuring and indicating another initial UL BWP is not as simple as configuring separated Msg1 resource (no matter by preambles or by time-frequency resource). |
| Huawei, HiSilicon | Almost | We understand the intention of the FL proposal 3. Following the FL proposal 2 this can be a good starting point for next step discussion - i.e. firstly identify the feasibility of Option 1 and pros and cons. Similar observations for other options can be made in line with this from proponents. We don’t think the main bullet is necessary and actually may lead to different understanding of early identification – which is supposed to be before RRC connection. - For early identification of RedCap UEs Under the assumption that RedCap UEs may be identified at least via UE capability reporting: - At least for Option 1, Optional configurability of separate resources for Msg1/MsgA transmissions for RedCap UEs via use of separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning for early identification of RedCap UEs is considered further. |
| OPPO |  | Proposal 3 is only related to option 1 of proposal 2. This should be clarified in the proposal. And it can be deprioritized before the options in proposal 2 are down-selected. |
| Samsung |  | It would be better to discuss the options in Proposal 2 and conclude if all/some options should be considered further. Then we can discuss turning ON/OFF the identification mechanism with option 1. |
| CMCC | Y |  |
| DOCOMO | Y | Assuming that Proposal 2 would be agreeable, Proposal 3 can be a starting point for further discussion on the feasibility and pros/cons. In that sense, we agree with that proposal. Similar discussion can be held for Opt. 2: During Msg3 transmission in the next step. |
| Lenovo, Motorola Mobility | Y |  |
| Nokia |  | Though we favour a msg1/msgA approach to enable the network to initially identify REDCAP devices, we have a similar view to OPPO. Let’s fairly downselect from proposal 2 first. |
| WILUS |  | We need to discuss all of options in proposal 2 at the same level. This proposal prioritizes option 1 in proposal 2. |
| Xiaomi | Y |  |
| Spreadtrum |  | It may be too early to discuss this. |
| Ericsson |  | Is the intention with the proposal to say that a potential approach for study is to leave it to the network to configure whether to use RedCap UE identification via Msg1/MsgA or whether to use RedCap UE identification later than Msg1/MsgA? In that case, we are fine with the proposal. If the intention is something else, then perhaps it is a premature proposal before we know more about what UE complexity reduction techniques (if any) that will be recommended by RAN1. Moderator: Yes, that is indeed the intention of the proposal (“leave it to the network to configure whether to use RedCap UE identification via Msg1/MsgA or whether to use RedCap UE identification later than Msg1/MsgA”)! |
| Apple |  | We think it goes a bit far at this stage especially considering the dependency with Option.1. We may first progress on FL proposal 2 and further progress on this 2nd level details if Option.1 is selected. |

Summary (FL Prop 3):

- The group seems divided on FL Proposal 3 with many thinking that it may be “too early” to discuss this.

- Some companies see this as implying prioritization of Option 1 from FL Proposal 2.

  - This was not the intention; see next response below.

- Another observation made by several companies is that Msg3 and MsgA-PUSCH should also be considered in the same light.

  - The proposal was based on views expressed in company contributions, wherein examples involving UE identification during Msg1 transmission have been cited. However, technically, such configurability can also be envisioned for Option 2 and MsgA-PUSCH.

Considering the feedback above, FL Proposal 3 is revised as below, and should be interpreted as an extension of the study in FL Proposal 2A.

FL Proposal 3A

- Further study the configurability of methods for early identification of RedCap UEs:

  - during Msg1/MsgA-PRACH transmissions,

  - during Msg3/MsgA-PUSCH transmissions.

  - Analysis of options for UE identification

During Msg1 transmission (Opt. 1)

Feasibility

Early identification of RedCap UE type during transmission of Msg1 can be achieved via one or more of:

- Separation of PRACH resources (occasions and/or formats) or PRACH preambles between RedCap and non-RedCap UEs

- Separation of initial UL BWP for RedCap and non-RedCap UEs.

  - PRACH resource configuration may be separately or commonly provided for each initial UL BWP configuration

Question 3.1.1-F(easability)

- Are the above observations on feasibility of early identification of RedCap UE types during Msg1 transmission accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG | Y | gNB may configure a separate initial UL BWP for RedCap UE with PRACH resource configuration provided for the initial UL BWP. If this configuration is included in legacy SIB1 or new SIB1, gNB can identify RedCap UE via the initial UL BWP. |

Necessity

Early identification of RedCap UE type during transmission of Msg1 could be necessary in the following events:

- Relaxation of UE minimum processing times for PDSCH processing and PUSCH preparation

  - Strictly necessary? - No; gNodeB may need to schedule conservatively, always assuming RedCap UE, at the expense of slower initial access procedure for non-RedCap UEs

- Coverage recovery for one or more of Msg2, Msg3, or Msg4 scheduling or the associated PDCCH

  - Strictly necessary?  - Depends on outcome of discussion in AI 8.6.3 on Coverage Recovery and if Msg2 and/or Msg3 and/or broadcast PDCCH are identified as requiring coverage recovery.

- Appropriate link adaptation for Msg2/Msg4 PDSCH or Msg3 PUSCH, and associated PDCCH

  - Strictly necessary? – No; gNodeB may need to schedule conservatively, i.e., always assuming RedCap UE, at the expense of increased system OH (from using conservative link adaptation even for non-RedCap UEs).

Question 3.1.1-N(ecessity)

- Are the above observations on necessity of early identification of RedCap UE types during Msg1 transmission accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | Regarding processing time relaxation, we have no consensus on how much RedCap UEs relax UE minimum processing times during RACH procedure. Thus, it seems too early to say that gNodeB can always schedule conservatively assuming RedCap UEs by sacrificing non-RedCap UEs. We do not know how much non-RedCap UEs should sacrifice themselves at a cell where RedCap UEs may access. Regarding coverage recovery, it would depends on outcome of discussion in AI 8.6.3 on Coverage Recovery as indicated by FL. Regarding link adaptation, it is unclear whether gNB can always schedule conservatively assuming RedCap UEs. |

Pros and Cons

Early identification of RedCap UE type during transmission of Msg1

| Pros | Cons |
| --- | --- |
| Enables efficient handling of different UE minimum processing times between RedCap and non-RedCap UEs for: minimum timing between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDSCH carrying Ms4 and the corresponding HARQ-ACK feedback; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission. | Reduction in PRACH user capacity (for the options based on separation of PRACH preambles), impacting both RedCap and non-RedCap UEs. |
| Enables coverage recovery for any one or more of: broadcast PDCCH, PDSCH associated with Msg2, PDSCH associated with Msg4, and PUSCH associated with Msg3, if coverage recovery is needed for these channels. | Increase in UL OH from PRACH (for the options based on separation of PRACH resources), impacting both RedCap and non-RedCap UEs. |
| Enables separate link adaptation for Msg2/Msg4 PDSCH and/or Msg3 PUSCH and/or their corresponding PDCCH between that for RedCap and non-RedCap UEs, and potentially, also between RedCap UEs with different numbers of DL Rx branches, etc. | Increase in UL OH and complexity in configuration and maintenance of multiple initial UL BWP for the gNodeB. |
| The option of configuring separate initial UL BWPs enables address congestion in the initial UL BWP that may otherwise need to be restricted to the mandatory required BW for RedCap UEs in the band/FR. | Scales poorly - the indication mechanisms in this category may be limiting in terms of distinguishing between further sub-categories within RedCap device type. |
|  | The option of configuring separate initial UL BWP is only available for FDD deployments and thus, may be limited in its effectiveness. |
|  | Higher impact to RAN1 and RAN2 specifications as well as increased SIB signaling OH compared to other options. |

Question 3.1.1-P(ros&Cons)

- Are the above observations on pros and cons for early identification of RedCap UE types during Msg1 transmission accurate and complete? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | Regarding ‘scales poorly’, it is not clear whether sub-categories within RedCap device type is really essential from RAN1 perspective. This aspect seems not a drawback depending on need for sub-categories. Regarding initial UL BWP, we think that for TDD, separate initial UL/DL BWP could be configured for RedCap UEs. Thus, this aspect seems not a drawback. |

During Msg3 transmission (Opt. 2)

Feasibility

Early identification of RedCap UE type during transmission of Msg3 can be achieved via one or more of:

- Using the spare bit in existing Msg3 definition

- Defining a new TBS value for Msg3 PUSCH to carry additional one or more bits, indicating RedCap UE type

Question 3.1.2-F

- Are the above observations on feasibility of early identification of RedCap UE types during Msg3 transmission? accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | The feasibility of spare bit should be studied in RAN2, not in RAN1. |

Necessity

Early identification of RedCap UE type during transmission of Msg3 could be necessary in the following events:

- Coverage recovery for one or more of Msg4 PDSCH

  - Strictly necessary?  - Depends on outcome of discussion in AI 8.6.3 on Coverage Recovery if Msg4 are identified as requiring coverage recovery.

- Appropriate link adaptation for Msg2/Msg4 PDSCH or Msg3 PUSCH, and associated PDCCH

  - Strictly necessary? – No; gNodeB may need to schedule conservatively, i.e., always assuming RedCap UE, at the expense of increased system OH (from using conservative link adaptation even for non-RedCap UEs).

Question 3.1.2-N

- Are the above observations on necessity of early identification of RedCap UE types during Msg3 transmission accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | If early identification is included in an additional bit of Msg 3, it would be in a RRC message. If it is the case, we wonder if gNB-DU should always decode this indication because gNB-DU does not have RRC. Meanwhile, Msg4 UE Contention Resolution Identity MAC CE is created by gNB-DU. gNB-DU may send Msg4 PDSCH without detecting early identification. Regarding link adaptation, it is unclear whether gNB can always schedule conservatively assuming RedCap UEs. |

Pros and Cons

Early identification of RedCap UE type during transmission of Msg3

| Pros | Cons |
| --- | --- |
| Enables coverage recovery for PDSCH associated with Msg4 (and scheduling of Msg5), if coverage recovery is needed for these channels. | If new TBS for Msg3 is not defined, this would consume the single spare bit currently available in Msg3 payload, and this may not be most desirable. |
| Enables separate link adaptation for Msg4 PDSCH between that for RedCap and non-RedCap UEs, and potentially, also between RedCap UEs with different numbers of DL Rx branches, etc. | If new TBS for Msg3 PUSCH is defined, additional mechanisms are likely necessary to allow UE to use a particular Msg3 PUSCH TBS (and/or resource allocation) in response to a RAR, and associated blind decoding by the gNB on the Msg3 PUSCH TBS (and/or resource allocation). Considerable impact to RAN1 and RAN2 specifications related to Msg3 scheduling and transmission. |
| Limited impact to RAN1 specifications if the spare bit in Msg3 payload is utilized. | The option of using the spare bit in Msg3 scales poorly - limiting to a single-bit indication may not be sufficient if intending to distinguish between further sub-categories within RedCap device type. |
| The option of new TBS for Msg3 PUSCH may offer good scalability in the number of bits for such UE identification; e.g., if sub-categories of RedCap device types are to be indicated in Msg3. | Cannot facilitate additional coverage recovery (if needed) or separate link adaptation for broadcast PDCCH and/or Msg2 PDSCH, and/or Msg3 PUSCH for RedCap UEs. |
|  | If UE minimum processing times are relaxed, cannot facilitate scheduling with separate minimum timing relationships for RedCap UEs (compared to non-RedCap UEs) between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDSCH carrying Ms4 and the corresponding HARQ-ACK feedback; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission. This could result in increased initial access latency for non-RedCap UEs. |

Question 3.1.2-P

- Are the above observations on pros and cons for early identification of RedCap UE types during Msg3 transmission accurate and complete? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | Regarding Msg4, if early identification is included in an additional bit of Msg 3, it would be in a RRC message. If it is the case, gNB-DU may send Msg4 PDSCH without detecting early identification as we explained in 3.1.2-N. Accordingly, Msg3 could not be used to enable coverage recovery and separate link adaptation for PDSCH associated with Msg4. Regarding RAN1 impact, it is not clear whether RAN1 impact is limited until RAN2 confirms that a spare bit can be utilized. |

During Msg5 transmission or in UE capability report (Opt. 3)

Feasibility

Identification of RedCap UE type during transmission of Msg5 or as part of UE capability reporting are feasible from RAN1 perspective, with the indication being carried by the PUSCH. The key details pertain to RAN WG2.

Question 3.1.3-F

- Are the above observations on feasibility of identification of RedCap UE types during Msg5 transmission or in UE capability report accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG | Y |  |

Necessity

The options of RedCap UE identification during Msg5 transmission or in the UE capability report would be essential if earlier identification is not supported/configured.

From the perspective of RAN1, there is no functional difference between the options of UE identification during Msg5 transmission and in UE capability report.

Note that depending on exact definition and types of RedCap devices, some or all information regarding RedCap UE identification may be provided as part of UE capability reporting, irrespective of whether early indication of RedCap UE type is also provided during Msg1 or Msg3 transmission.

Question 3.1.3-N

- Are the above observations on necessity of identification of RedCap UE types during Msg5 transmission or in UE capability report accurate and agreeable? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG |  | RAN1 may need to focus on whether identification earlier than Msg5 is necessary or not. Necessity of RedCap UE identification during Msg5 transmission could be discussed in RAN2 later. |

Pros and Cons

Identification of RedCap UE type during transmission of Msg5 or in UE capability report

| Pros | Cons |
| --- | --- |
| At least the option of UE capability reporting offers the simplest and most scalable option for indication of RedCap UE type with full flexibility. | Cannot facilitate additional coverage recovery (if needed) or separate link adaptation for broadcast PDCCH and/or Msg2 and/or Msg4 PDSCH, and/or Msg3 PUSCH for RedCap UEs. Too conservative scheduling and link adaptation for all UEs imply increased system OH for initial access in the initial DL and UL BWPs. |
| Limited or no impact to RAN1 specifications. | If UE minimum processing times are relaxed, cannot facilitate scheduling with separate minimum timing relationships for RedCap UEs between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDSCH carrying Ms4 and the corresponding HARQ-ACK feedback; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission. This could result in increased initial access latency for non-RedCap UEs. |

Question 3.1.3-P

- Are the above observations on pros and cons for early identification of RedCap UE types during Msg5 transmission or in UE capability report accurate and complete? Please share your views, including any further considerations, below.

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |
| LG | Y |  |

During MsgA transmission (Opt. 4)

This set of options include:

- During MsgA-PRACH transmission

- During MsgA-PUSCH transmission

In general, most of the observations for Msg1 and Msg3 may apply to the options involving MsgA-PRACH and MsgA-PUSCH respectively, except that there may not be any difference between the MsgA-PRACH and MsgA-PUSCH options in terms of UE identification for scheduling of PDCCH and PDSCH for MsgB.

Other details for MsgA to be added further when the discussions in the previous sub-sections for Options 1 through 3 are a bit more stable.

| Company | Comments |
| --- | --- |
| LG | Option 4 could be discussed later after down-selecting to one of Option 1, 2 and 3. |

- RRM relaxations for UE power savings

In contributions [4] and [13], views on RRM relaxations for RedCap UEs that may be stationary to enable reduced power consumption have been presented (see Appendix B for summary of proposals).

Following the earlier conclusion from RAN1 #102-e meeting (see below), it is recommended that these discussions be deprioritized until RAN WG2 makes further progress.

From RAN1 #102-e [29]:

Conclusion: - RAN1 to defer to RAN2 for further progress on studies regarding RRM relaxations and E-DRx for RedCap UEs to facilitate reduced UE power consumption.

References

- RP-201386, Revised SID on Study on support of reduced capability NR devices.

- R1-2007533, UE identification and access restriction for RedCap	Ericsson

- R1-2007538, Identification for RedCap UEs	FUTUREWEI

- R1-2007672, RRM relaxation for Reduced Capability NR devices	vivo, Guangdong Genius

- R1-2007719, Access control and identification for Reduced Capability NR devices	ZTE

- R1-2007866, Identification and access restriction for reduced capability NR devices	CATT

- R1-2007951, On identification of and access control for RedCap UEs	Intel Corporation

- R1-2008020, Discussion on identification and access control for Reduced Capability NR devices	CMCC

- R1-2008052, Other aspects for reduced capability NR devices	LG Electronics

- R1-2008072, Initial access for RedCap UEs	Nokia, Nokia Shanghai Bell

- R1-2008075, Procedure of identification for Reduced Capability NR devices	TCL Communication Ltd.

- R1-2008088, Discussion on the access control and configuration for reduced capability device	Xiaomi

- R1-2008106, Consideration on power saving for reduced capability NR devices	Spreadtrum Communications

- R1-2008174, UE identification and access barring	Samsung

- R1-2008264, Other considerations for reduced UE capability	OPPO

- R1-2008291, On RedCap device identification	Panasonic

- R1-2008329, Other aspects for reduced capability devices	Huawei, HiSilicon

- R1-2008337, Narrowband operation and identification of RedCap UE	Lenovo, Motorola Mobility

- R1-2008397, Identification and access restriction for reduced capability UEs	Sharp

- R1-2008556, Discussion on UE identification for RedCap	NTT DOCOMO, INC.

- R1-2008585, Discussion on identification of reduced capability UE	ASUSTeK

- R1-2008688, Device identification and access restriction for RedCap	InterDigital, Inc.

- R1-2007671, Framework and Principles for Reduced Capability	vivo, Guangdong Genius (from AI 8.6.4)

- R1-2008087, Framework and Principles for Reduced Capability	Xiaomi

- R1-2008101, Discussion on Framework and Principles for Reduced Capability	Spreadtrum Communications

- R1-2008296, Framework and Principles for RedCap 	Lenovo, Motorola Mobility

- R1-2008513, On the framework for RedCap UEs	MediaTek Inc.

- R1-2008623, Standardization Framework and Design Principles for RedCap Devices	Qualcomm Incorporated

- R1-2007283, Summary on [102-e-NR-RedCap-05]	Moderator (Intel Corporation), RAN1#102-e.

Appendix A

List of observations/proposals on access control for and identification of RedCap UEs:

R1-2007533	UE identification and access restriction for RedCap	Ericsson

Proposal 1 In the TR, capture feasibility/pros/cons for different options from RAN1 perspective, considering the input in the table in this contribution. Do not make further attempts to down-select between the options in this RAN1 meeting. Option; Feasibility/pros/cons from RAN1 perspective. Option 1: Msg1; Feasibility: - Yes, assuming enough PRACH resources are available for grouping Pros: - Msg2/3/4/5 transmission parameters can be optimized for RedCap and non-RedCap UEs, respectively - Does not require any additional bits in Msg3 (from RAN1 perspective) Cons: - Increased PRACH resource consumption/fragmentation; the fragmentation could multiply if Msg1 indication is further used for RedCap UE identification, Small Data transmission and slicing, possibly leading to resource depletion. Option 2: Msg3; Feasibility: - Yes, assuming Msg2/3 transmission does not become an issue without earlier indication Pros: - Msg4/5 transmission parameters can be optimized for RedCap and non-RedCap UEs, respectively - No increased PRACH resource consumption/fragmentation Cons: - Msg2/3 transmission parameters cannot simultaneously be optimized for RedCap and non-RedCap UEs - Requires at least one bit in Msg3 and potentially a larger Msg3 TBS. Option 3: Msg5+; Feasibility: - Yes, assuming Msg2/3/4/5 transmission does not become an issue without earlier indication Pros: - No increased PRACH resource consumption/fragmentation - Does not require any additional bits in Msg3 (from RAN1 perspective) Cons: - Msg2/3/4/5 transmission parameters cannot simultaneously be optimized for RedCap and non-RedCap UEs. Option 4: MsgA; Feasibility: - Yes, assuming 2-step RACH is used Pros: - MsgB and Msg5 transmission parameters can be optimized for RedCap and non-RedCap UEs, respectively - Pros for UE indication in MsgA preamble part are similar as for Option 1 - Pros for UE indication in MsgA PUSCH part are similar as for Option 2 Cons: - Cons for UE indication in MsgA preamble part are similar as for Option 1 - Cons for UE indication in MsgA PUSCH part are similar as for Option 2. Proposal 2 RAN1 continues to prioritize study of other aspects of the study item and leave the study of access restriction to RAN2.

R1-2007538	Identification for RedCap UEs	FUTUREWEI

Observation 1: it is better to pool RACH resources between RedCap and non-RedCap UEs. Proposal 1: Similar to LTE-M, PRACH preamble partitioning is used for RedCap identification Proposal 2: RedCap identification in Msg 3 can be considered if RAN1 decides that no compensation for coverage is needed for Msg 3 Proposal 3: If two-step RACH is supported, RedCap identification in Msg A can be considered

R1-2007672	RRM relaxation for Reduced Capability NR devices	vivo, Guangdong Genius

Observation 1: IDLE mode power saving is critical for wearable devices. Observation 2: 13.4% power saving gain in IDLE mode can be observed if serving cell RRM relaxation is introduced for high SINR UE. Observation 3: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles. Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state should be supported for RedCap UE.

R1-2007719	Access control and identification for Reduced Capability NR devices	ZTE

Observation 1: For reduced capability NR devices, an access control indication carried in DCI scheduling SIB1 is beneficial for UE’s power saving. Observation 2: Early identification of RedCap UEs as early as possible is beneficial for coverage recovery and dedicated scheduling of messages during initial access. Proposal 1: For reduced capability NR devices, an access control indication is carried in DCI scheduling SIB1. Proposal 2: From RAN1 perspective, identification of RedCap UEs during Msg1/MsgA transmission is feasible.

R1-2007866	Identification and access restriction for reduced capability NR devices	CATT

Observation 1: cellBarred in MIB already provides overall barring information, which is determined comprehensively from network perspective. Observation 2: the motivation of introducing RedCap-specific barring indication before acquiring SIB1 need further study. Observation 3: Redcap identification during Msg1 may benefit the scheduling of largest number of channels before RRC setup, at the cost of PRACH partitioning. Observation 4: Redcap identification during Msg3 may benefit the scheduling of small number of channels before RRC setup, at the cost of future Msg3 extendibility. Observation 5: Redcap identification after Msg4 is considerable, if the current RACH procedure can be reused for RedCap. Proposal1: Leave the access restriction of RedCap UE to RAN2 discussion

R1-2007951	On identification of and access control for RedCap UEs	Intel Corporation

Observation 1: - Identification of RedCap UEs at least after Msg4 Ack (i.e., via Msg5 or UE capability reporting) is necessary as a baseline mechanism. Observation 2: - Early identification of RedCap UEs during RA procedure may not always be necessary. Such mechanisms, if introduced, could be optionally configured by the gNB via SIB signaling. Observation 3: - Early identification of RedCap UEs via Msg1 or MsgA could be beneficial, although not strictly necessary, if relaxed minimum UE processing times are introduced for RedCap UEs. Observation 4: - Early identification of RedCap UEs during the RA procedure may not be necessary from perspective of coverage recovery needs for Msg2 and/or Msg4. Observation 5: - Early identification of RedCap UEs during RA procedure may be necessary if additional or separate initial DL or UL (respectively) BWPs may be configured for Msg2/Msg4 or Msg1/Msg3 (respectively).

R1-2008020	Discussion on identification and access control for Reduced Capability NR devices	CMCC

Proposal 1: Msg.3 can be used for early identification of RedCap UEs. Proposal 2: Separate initial BWP can be used for early identification of RedCap UEs. Proposal 3: Early access control by PBCH, type0-PDCCH, SIB1, PRACH procedure can be further studied.

R1-2008052	Other aspects for reduced capability NR devices	LG Electronics

Observation 1: It is beneficial to introduce a new SIB1 for REDCAP UEs e.g. for support of potential REDCAP specific common channel configuration and transmissions without any concern on SIB1 size. Proposal 1: Study a mechanism for scheduling new SIB1 (e.g. SIB1-R) used by REDCAP UEs. Observation 2: In LTE, MIB provides scheduling of SIB1-BR. However, it seems difficult for NR MIB to accommodate scheduling information of a new SIB1-R. Proposal 2: If CORESET0 can be shared by REDCAP UEs and normal UEs, the DCI format 1_0 with CRC scrambled by SI-RNTI can be used to schedule both legacy SIB1 and new SIB1-R. Observation 3: If the DCI schedules new SIB1-R, REDCAP UEs can identify whether the cell supports REDCAP UEs based on the DCI. Proposal 3: The DCI format 1_0 with CRC scrambled by SI-RNTI explicitly or implicitly indicates whether the cell supports REDCAP UEs Observation 4-1: gNB currently transmit other system information within the size of CORESET 0. However, REDCAP UEs may or may not receive legacy transmission of other system information depending on RAN1 discussion. In addition, gNB could possibly provide different configurations in other system information to REDCAP UEs and normal UEs. Observation 4-2: It is beneficial for gNB to know whether on-demand SI is requested by REDCAP UEs or normal UEs. Proposal 4: REDCAP specific RACH resources can be configured for gNB to transmit on-demand SI message decodable by REDCAP UEs. Observation 5: A best beam RS would not frequently change for a certain stationary UE for which some aspects could be optimized. Proposal 5: Study relaxed operation of RLM/BFD as well as RRM for a stationary UE.

R1-2008072	Initial access for RedCap UEs	Nokia, Nokia Shanghai Bell

Observation 1: The NR MIB “cellBarred” bit applies to all UEs (RedCap and non-RedCap) attempting to access the cell. Observation 2: The NR MIB does not support enough spare bits to indicate RedCap device specific barring. Observation 3: The DCI format 1-0 variant used to schedule SIB1 and other SI messages has 15 reserved bits. Observation 4: The enhancement of UAC for REDCAP is pending SA1 discussions. Proposal 1: A REDCAP UE can determine if a cell supports REDCAP without the need to read SIB1. Proposal 2: A REDCAP UE can determine if a REDCAP capable cell is barring specific REDCAP capability types without the need to read SIB1. Note: This level of “hard” barring compliments the SIB1 based UAC level of “soft” barring. Proposal 3: RAN1 and RAN2 to determine if a separate SIB1 for REDCAP devices, R-SIB1, is specified. Proposal 4: If a separate R-SIB1 is specified for REDCAP devices, then a new DCI RNTI is defined to scramble a DCI that shares the same search space as used by the current SIB1 DCI, but which schedules the new R-SIB1. Proposal 5: If the current SIB1 is to be reused by REDCAP devices, then some of the reserved bits in the existing DCI used to schedule the SIB1 are used to indicate cell REDCAP capability. Proposal 6: If RAN1 determines that additional coverage recovery is needed on msg2/msgB, the network can identify REDCAP UEs at the msg1/msgA stage (options 1 and 4) of the RACH procedures. FFS: The specific method(s) of identification, preamble partitioning/separate UL BWP/etc

R1-2008075	Procedure of identification for Reduced Capability NR devices	TCL Communication Ltd.

Proposal 1: Study the methods to partition preambles to minimize the impact on the current specifications. Proposal 2: Study the method of identifying UE type by the location of PRACH occasion Proposal 3: The specific identification of the reduced capability UE type post-RAR shall be considered. Proposal 4: Further study to determine which method to be adopted.

R1-2008088	Discussion on the access control and configuration for reduced capability device	Xiaomi

Proposal 1: Network access control to RedCap UE should be explicitly indicated as early as possible. Proposal 2: Mechanism for identification of RedCap UEs During Msg1 transmission should be supported.

R1-2008106	Consideration on power saving for reduced capability NR devices	Spreadtrum Communications

Proposal 1: RRM measurement relaxation for neighbour cells including frequency layers with higher/equal/lower priority can be considered for stationary RedCap UEs. Proposal 2: RRM measurement relaxation for serving cell can be considered for stationary RedCap UEs.

R1-2008174	UE identification and access barring	Samsung

Proposal #1: Support early access for identification for RedCap UEs during random access.

R1-2008264	Other considerations for reduced UE capability	OPPO

Observation: Identification of RedCap UEs by Msg1 will reduce the utilization efficiency of PRACH resource. Proposal 1: Separate SSB and CORESET 0 are not used for cell barring indication. Proposal 2: Leagacy Access Control mechanism is reused for RedCap UE as much as possible. The details are discussed in RAN2. Proposal 3: Msg1 or Msg3 can be used for the identification of RedCap UE. Msg3 is slightly preferred.

R1-2008291	On RedCap device identification 	Panasonic

Observation 1: RedCap device type identification during Msg1 transmission allows network to compensate the coverage performance in early stage and the efficiency of Msg2 and larter could be improved. Observation 2: In general, it is feasible to support RedCap device type identification by configuring separate PRACH resource in SIB with some specification impact. Observation 3: Compared with separate PRACH resource in time and frequency domain, PRACH preamble partitioning may involve additional standardization efforts in RAN1 and lead to more complicated gNB implementation. Observation 4: It would be a challenge to PRACH resource and preamble capacity for UE identification via Msg1 if more than one UE types for RedCap need to be identified. Observation 5: Device type identification during Msg3 transmission is attractive when Msg1 resource is limited like narrower system bandwidth. There is no RAN1 specification impact. Observation 6: For device type identification during Msg3 transmission, the network is not able to compensate the coverage performance for Msg2 and Msg3. There could also be scheduling limitation for normal UEs, although it could be regarded as the acceptable cost compared with the resource overhead of Msg1 sharing to RedCap devices. Observation 7: The pros and cons for device type identification via Msg3 and post Msg4 transmission are quite similar. The more detailed differentiation of RedCap devices needs to use this option using the existing framework of UE capability reporting, if supported. Observation 8: Device type identification during MsgA transmission requires additional standardization efforts. Observation 9: The pros and cons for device type identification via MsgA is similar with that of Msg1 and Msg3, except earlier identification by MsgA than Msg3.

R1-2008329	Other aspects for reduced capability devices	Huawei, HiSilicon

Observation 1: For FR1, spare bits in MIB are possible for restricting the access of RedCap UEs. Observation 2: Compared with access restriction via SIB1, access restriction via DCI associated with SIB1 is beneficial for RedCap UE’s power saving and with minor specification impact. Observation3: The earlier the network identify the RedCap UEs, the better for the gNB to separately schedule the data transmission and make network control on RedCap UEs. Proposal 1: It is necessary for network to indicate its capability of whether it supports RedCap UEs accessing or not. Proposal 2: Consider to restrict the access of RedCap UEs via DCI associated with SIB1. Proposal 3: Consider to identify RedCap UEs via Msg1/MsgA.

R1-2008337	Narrowband operation and identification of RedCap UE	Lenovo, Motorola Mobility

- Proposal 1: Specify a RedCap UE category(ies), each category defining a particular set of mandatory RedCap UE features and/or capabilities. - Proposal 2: Support indication of a RedCap UE category based on active applications or traffic types. - Proposal 3: For a reduced capability UE, support operating with a bandwidth smaller than a bandwidth of an active BWP. - Proposal 4: Study mechanisms exploiting the frequency diversity and realizing interference randomization within an active BWP, for a case that a reduced capability UE operates with a bandwidth smaller than a bandwidth of the active BWP. - Proposal 5: Support dynamic switching between narrowband and wideband operations within a BWP.

R1-2008397	Identification and access restriction for reduced capability UEs	Sharp

Proposal 1: RedCap UEs are identified by Msg1 transmission. Proposal 2: Indication of whether a RedCap UE can camp on a cell, as already agreed in RAN2, is in SIB. Observation 1: The RedCap specific cell-barring indication in system information is only used when the “cellBarred” bit in MIB indicates “notBarred”.

R1-2008556	Discussion on UE identification for RedCap	NTT DOCOMO, INC.

Observation 1: - It is beneficial to identify RedCap UEs through Msg1 so that gNB can schedule RedCap UEs appropriately using the coverage recovery techniques for Msg2/3/4

R1-2008585	Discussion on identification of reduced capability UE	ASUSTeK

Observation 1: There is a trade off in the base station whether it performs the coverage recovery technique when it has not identified the RedCap UE. Observation 2: RedCap UE may fail to decode RAR and Msg4 in Opt. 2 and Opt. 3 if the base station does not deploy the coverage recovery technique when it has not identified the RedCap UE. Proposal 1: Select Opt. 1, Opt. 4 as methods for identification of RedCap UE during random access procedure. Observation 3: Configuring a separated initial uplink bandwidth part of RedCap UE improves the load balance and removes the restriction on configuring the legacy initial uplink bandwidth part. Observation 4: Configuring separated RACH resource on time for RedCap UE may delay the downlink scheduling. Proposal 2: Consider configuring a separated initial uplink bandwidth of RedCap UE for identification in random access procedure.

R1-2008688	Device identification and access restriction for RedCap	InterDigital, Inc.

Observation-1: Using Msg1/A for RedCap UE identification could avoid unnecessary PRACH collision increase for the other type of devices. Observation-2: With a limited number of PRACH resource configured for the RedCap UEs, the access could be restricted naturally. Observation-3: The use of Msg1/A for RedCap UE identification could be up to gNB configuration. Proposal 1: Support dedicated resources of Msg1/A for the RedCap UE identification and it is up to the network whether to configure the dedicated Msg1/A resources for RedCap UEs.

Proposals/observations relevant to access control and UE identification from AI 8.6.4:

R1-2007671	Framework and Principles for Reduced Capability	vivo, Guangdong Genius

Observation 2: repetitions for initial access channels are not specific for RedCap UEs, if coverage enhancements SI/WI decide the solutions for differentiating the UEs requiring repetitions and UEs not requiring repetitions, it is desirable to reuse the solutions. Proposal 4: further study following two options for identification of RedCap UEs: - Opt. 1: During Msg1 transmission via separate initial UL BWP. - Opt. 5: During UE capability reporting.

R1-2008087	Framework and Principles for Reduced Capability	Xiaomi

Proposal 4: Early identification of RedCap capability by RACH procedure can be considered.

R1-2008101	Discussion on Framework and Principles for Reduced Capability	Spreadtrum Communications

Proposal 3: Study the early indication of RedCap UE capability or the solutions that can free the configuration of the uplink initial BWP of normal UEs from the limitation of the maximum bandwidth of RedCap UEs.

R1-2008296	Framework and Principles for RedCap	Lenovo, Motorola Mobility

Observation 3: The gNB could control the access of RedCap UEs in different stages during initial access, depending on different design. Proposal 3: Study the feasibility of UE access control during initial access, through - Cellbarred in dedicated SSB - Cellbarred introduced in SIB1 - RACH procedure Proposal 4: Study UE type identification through either Msg1/MsgA or Msg3.

R1-2008513	On the framework for RedCap UEs	MediaTek Inc.

- A RedCap UE only camps on a cell that indicates support of RedCap operation. - Support of RedCap operation in a cell is broadcasted by the network. - A RedCap UE that is registered to a network is identified by the network at msg5.

R1-2008623	Standardization Framework and Design Principles for RedCap Devices	Qualcomm Incorporated

Proposal 4: Support early indication of RedCap device type by Type-1 and Type-2 RACH procedures.

Appendix B

List of observations/proposals on RRM relaxations for RedCap UEs:

R1-2007672	RRM relaxation for Reduced Capability NR devices	vivo, Guangdong Genius

Observation 1: IDLE mode power saving is critical for wearable devices. Observation 2: 13.4% power saving gain in IDLE mode can be observed if serving cell RRM relaxation is introduced for high SINR UE. Observation 3: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles. Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state should be supported for RedCap UE.

R1-2008106	Consideration on power saving for reduced capability NR devices	Spreadtrum Communications

Proposal 1: RRM measurement relaxation for neighbour cells including frequency layers with higher/equal/lower priority can be considered for stationary RedCap UEs. Proposal 2: RRM measurement relaxation for serving cell can be considered for stationary RedCap UEs.