3GPP TSG RAN WG1 Meeting #103-e                                   R1-20xxxxx

E-meeting, October 26th - November 13th, 2020

Agenda Item:			8.8

Source:			Moderator (China Telecom)

Title:			[103-e-NR-CovEnh-01] Email discussion for TR38.830 update

Document for:		Discussion and Decision

- Introduction

In RAN #86 meeting, a new Rel-17 study item on NR coverage enhancements was approved [1]. The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

This contribution is a summary of email discussion on TR38.830 update.

[103-e-NR-CovEnh-01] Email discussion for TR38.830 update – Jianchi (CT)

- 1st check point: 10/29 (particularly to endorse x7992 if possible)

- 2nd check point: 11/12 further update based on the progress during this e-meeting.

- Discussion on R1-2007992 [3] (10/29)

2.1 Discussion on section 4.1

Companies are invited to provide views on section 4.1.

| Companies | Comments |
| --- | --- |

2.2 Discussion on section 4.2

Companies are invited to provide views on section 4.2.

| Companies | Comments |
| --- | --- |

2.3 Discussion on section 4.3

Companies are invited to provide views on section 4.3.

| Companies | Comments |
| --- | --- |

2.4 Discussion on section 6.1

Companies are invited to provide views on section 6.1.

| Companies | Comments |
| --- | --- |

2.5 Discussion on section 6.2

Companies are invited to provide views on section 6.2.

| Companies | Comments |
| --- | --- |

2.6 Discussion on section 6.3

Companies are invited to provide views on section 6.3.

| Companies | Comments |
| --- | --- |

2.7 Discussion on section A.1

Companies are invited to provide views on A.1.

| Companies | Comments |
| --- | --- |

2.8 Discussion on section A.2

Companies are invited to provide views on A.2.

| Companies | Comments |
| --- | --- |

2.9 Discussion on section A.3

Companies are invited to provide views on A.3.

| Companies | Comments |
| --- | --- |

- Discussion on further TR update (11/12)

To be added later

- Reference

- 3GPP RP-200861, “Revised SID on Study on NR coverage enhancements”, China Telecom, RAN#88e, June 29th – July 3rd, 2020.

- “3GPP TSG RAN WG1 RAN1 #102e Chairman’s Notes”, e-Meeting, Aug. 17th – 28th, 2020.

- 3GPP R1-2007992 TR 38.830 v0.0.3 Study on NR coverage enhancements China Telecom