3GPP TSG RAN WG1 Meeting #102-e	R1-20xxxxx

E-Meeting, October 26 – November 13, 2020

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for NB-IoT and LTE-MTC [1] includes an objective to support 16-QAM for unicast in UL and DL in NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

This documents provides the proposals and summary of discussions of the following email discussion according to the inputs [2-10]

[103-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Issues

Support of 16QAM for NB-IoT downlink

### Issue 1: The maximum TBS to support 16-QAM for unicast in DL for standalone and guardband.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 1: 5736 bits with ISF = 7 (i.e. option 3) is the maximum TBS to support 16-QAM for unicast in DL for standalone and guard-band deployments. |
| [3] | Proposal 4: For stand-alone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits for both 1-HARQ and 2-HARQ UEs. |
| [4] | Observation 1: For guard-band/standalone deployment, the TBS of 5736 bits cannot be applied for 2Tx antennas since the code rate of 5736 exceeds the upper limit of 0.932 when 2 NRS ports are configured. Proposal 1: 4968 bits with ISF=7 can be defined as the maximum TBS for DL 16QAM in guard-band/standalone deployment. |
| [5] | Proposal 2: For all operation modes, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7. |
| [6] | Observation 1 On the new max TBS to be supported for 16-QAM in DL, “Option 1: 4968 bits with ISF =7” seems to be a better choice as to avoid using a TBS that has not been previously used in the standard, or going beyond twice the max TBS in Rel-16. Proposal 1 The maximum TBS to support 16-QAM for unicast in DL for stand-alone and guard-band deployments is “Option 1: 4968 bits with ISF =7”. |
| [8] | Proposal 1: The maximum TBS for DL 16-QAM is 5736. - Target a maximum code rate of ~0.9 for all cases (deployment scenarios and N_SF) |
| [9] | Proposal 5: For all deployment scenarios, the max DL TBS should be 4968. |

For the following options:

- Option 1: 4968 bits with ISF=7

  - Nokia, Nokia Shanghai Bell, ZTE, Lenovo, Motorola Mobility, Ericsson, Sierra Wireless

- Option 2: 5072 bits with ISF=7

- Option 3: 5736 bits with ISF=7

  - Huawei, HiSilicon, Qualcomm,

Based on the intpus, the following is proposed:

Proposal 1: At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is down-selected from following options in RAN1#103e:

- Option 1: 4968 bits with ISF=7

- Option 3: 5736 bits with ISF=7

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 2: The maximum TBS to support 16-QAM for unicast in DL for inband.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 5: For in-band deployments, the maximum TBS to support 16-QAM in DL is 3624 bits. |
| [3] | Proposal 5: For in-band deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits for both 1-HARQ and 2-HARQ UEs. |
| [4] | Proposal 6: In-band uses the same DL TBS table as guard-band/standalone for DL 16QAM. - TBS configured for in-band should be less than or equal to TBS16 i.e. 3240 bits with ISF=7. |
| [6] | Proposal 3 For in-band deployments, the TBS/MCS Table to support 16-QAM is a sub-case of the TBS/MCS Table used for stand-alone and guard-band deployments. • The entries for 16-QAM ranges from a TBS = 176 bits to TBS = 3624 as to keep below 0.88 the achievable code rates in in-band deployments. |

Based on the inputs, there are following options for maximum TBS for inband deployment:

- Option 1: 3624 bit

  - Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

- Option 2: 3240 bit

  - ZTE,

Based on the majority view, the following is proposed:

Proposal 2: For inband deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 3: The TBS table.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Table 1 An example of TBS table for DL 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. 22; 520; 1064; 1608; 2152; 2664; 3240; 4264; 5352. 23; 552; 1128; 1736; 2280; 2856; 3496; 4584; 5736. Proposal 3: Adopt table 1 as the TBS design to support 16-QAM in DL. Proposal 2: I_SF>7 for 16-QAM is not supported. |
| [3] | Table 1. DL TBS table with 16-QAM. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. Proposal 6: Keep all existing MCS values in the TBS table and extend the TBS table to support 16-QAM. The TBS table for DL 16-QAM is given in Table 1. |
| [4] | Table 2: Extended TBS entries for DL 16QAM 0; 1; 2; 3; 4; 5; 6; 7. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. Proposal 2: TBS table for DL 16QAM can be expanded to 0~21 based on Table 16.4.1.5.1-1 of [2]. - Reuse the existing TBS 0~13 in Table 16.4.1.5.1-1 - Reuse TBS 14~21 in LTE TBS table |
| [6] | Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 16-QAM only; 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2536; 2984; 4008; 4968. Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 16-QAM only; 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 536; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; -; -; -; -; -; -; -; -. 19; -; -; -; -; -; -; -; -. 20; -; -; -; -; -; -; -; -. 21; -; -; -; -; -; -; -; -. |
| [9] | Proposal 1: TBS table(s) should be designed to support data rates of at least 180 kbps for all deployment scenarios (i.e. in-band, guard band, stand-alone) Proposal 2: The same TBS table should be used for all deployment scenarios Proposal 3: For the SA/GB scenario, the TBS table should be designed such that the code rate is always <= 0.85. Proposal 4: To support the in-band scenario, the maximum number of SFs (NSF) per TBS should be 15 (i.e. up from 10). ITBS; ISF (NSF). 0 (1); 1 (2); 2 (4); 3 (6); 4 (8); 5 (10); 6 (12); 8 (15). 0; 16; 32; 88; 152; 208; 256; 328; 392. 1; 24; 56; 144; 208; 256; 344; 424; 520. 2; 32; 72; 176; 256; 328; 424; 520; 648. 3; 40; 104; 208; 328; 440; 568; 680; 872. 4; 56; 120; 256; 408; 552; 696; 840; 1064. 5; 72; 144; 328; 504; 680; 872; 1032; 1320. 6; 328; 176; 392; 600; 808; 1032; 1224; 1544. 7; 104; 224; 472; 712; 968; 1224; 1480; 1800. 8; 120; 256; 536; 808; 1096; 1384; 1672; 2088. 9; 136; 296; 616; 936; 1256; 1544; 1864; 2344. 10; 144; 328; 680; 1032; 1384; 1736; 2088; 2664. 11; 176; 376; 776; 1192; 1608; 2024; 2408; 2984. 12; 208; 440; 904; 1352; 1800; 2280; 2728; 3368. 13; 224; 488; 1000; 1544; 2024; 2536; 3112; 3880. 14; 256; 552; 1128; 1736; 2280; 2856; 3496; 4264. 15; 280; 600; 1224; 1800; 2472; 3112; 3624; 4584. 16; 328; 632; 1288; 1928; 2600; 3240; 3880; 4968. 17; 336; 696; 1416; 2152; 2856; 3624; 4392. 18; 376; 776; 1544; 2344; 3112; 4008; 4776. 19; 408; 840; 1736; 2600; 3496; 4264. 20; 440; 904; 1864; 2792; 3752; 4584. 21; 488; 1000; 1992; 2984; 4008; 4968. Proposal 6: Adopt the above TBS table to support max TBS =4968 for all deployment scenarios (in-band, standalone, guard band) |

Based on the inputs, the following is proposed:

Proposal 3: to support 16QAM for NB-IoT DL, at least the following TBS indices are introduced

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS for I_SF > 7

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 4: The switching point from QPSK to 16QAM.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [3] | Proposal 7: For stand-alone and guard-band deployments, the break-point for 16-QAM in DL is I_TBS ≥ 14 based on Table 1. Proposal 8: For in-band deployment, the break-point for 16-QAM in DL is I_TBS ≥ 11 based on Table 1. |
| [4] | Observation 2: For NPDSCH in guard-band/standalone deployment, 16QAM performance is slightly better than QPSK performance at TBS= 2024 with ISF=7, i.e. TBS 11. Proposal 5: TBS 11 or 12 could be adopted as DL 16QAM switching point for guard-band/standalone deployment. Observation 3: For NPDSCH in in-band deployment, 16QAM performance is better than QPSK performance at TBS=1736 with ISF=7, i.e. TBS 10. Proposal 7: TBS 10, i.e. 1736 bits with ISF=7, could be adopted as DL 16QAM switching point for in-band deployment. |
| [8] | Observation 1: The breakpoint between QPSK and 16-QAM for DL is at approximately at 1.8 (total bits)/#REs (code rate of 0.9 for QPSK) Observation 2: For a given TBS, the optimum modulation scheme is different for different deployment scenarios. Proposal 2: For downlink, specify different MCS/TBS tables for different deployment scenarios (to optimize the switching point between 16-QAM and QPSK). The switching point between QPSK and 16-QAM is at approximately at (total bits)/#REs ≈1.65 Consider at least the following scenarios: - In-band with [3] symbol control, 2/4 port CRS - Guard-band/standalone with 0 symbol control |

On the switching point from QPSK to 16QAM for standalone and guardband deployments, there are following options:

  - The TBS entries of 14 (TBS of 2856 for I_SF=7) and above are used for 16QAM

    - Nokia, Nokia Shanghai Bell, Ericsson, MediaTek

  - The TBS entries of [11 (TBS of 2024 for I_SF=7) or 12 (TBS of 2280 for I_SF=7)] and above are used for 16QAM

    - ZTE

For inband deployment, there are following options:

  - The TBS entries of 11 (TBS of 2024 for I_SF=7) and above are used for 16QAM

    - Nokia, Nokia Shanghai Bell, Ericsson

  - The TBS entries of 10 (TBS of 1736 for I_SF=7) and above are used for 16QAM

    - ZTE,

In addition, [8] also proposed that the switching point if at approximately at (total bits)/#REs≈1.65.

As not many companies provides Thus it is proposed:

Proposal 4: Different switching points are used for standalone/guradband and inband deployments.

  - FFS the details of the switching point.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 5: Applicability

The following are proposed on scheduling of TBS and modulation:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 4: Repetition is not supported for 16-QAM in DL. |
| [3] | Proposal 11: Support 16-QAM with repetition in the DL. Proposal 12: Support modulation adjustment to QPSK when 16-QAM is scheduled with repetition in the DL. |
| [4] | Observation 4: For DL, 16QAM with 2 repetitions can still provide a higher peak data rate compared to QPSK. But 16QAM may show worse data rate after the number of repetitions increases to 4. Proposal 8: At least 2 repetitions should be supported for DL 16QAM. |
| [7] | Observation: Repetition for 16QAM has much worse performance under AWGN. |
| [8] | Proposal 5: DL 16-QAM is only applicable for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI. - At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS. Proposal 6: 16-QAM NPDSCH is only supported for R=1. - FFS whether to support the new TBSs with QPSK and increased RU |

As there are diverse views, the following is proposed to collect more views

Proposal 5: Further study on the following to support 16QAM for NB-IoT DL:

  - Whether repetition can be used for 16QAM in DL.

  - Whether DL 16QAM is only used for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI from USS.

  - Whether to support the new TBS with QPSK and increased RU.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Support of 16QAM for NB-IoT uplink

### Issue 6: The TBS/MCS table to support 16-QAM for unicast in UL.

There are following proposals on TBS design of 16-QAM for UL unicast

| Sourcing | proposals |
| --- | --- |
| [2] | Table 4 An example of TBS table for UL 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1000. 7; 104; 224; 328; 472; 584; 712; 1000; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1000; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 1000; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2472. 16; 328; 632; 968; 1288; 1608; 1928; 2536. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152; 2536. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2536. Proposal 6: Adopt table 4 as the TBS design to support 16-QAM in UL. |
| [3] | Table 2. UL TBS table with 16-QAM. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2472. 16; 328; 632; 968; 1288; 1608; 1928. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2472. Proposal 13: Keep all existing MCS values in the TBS table and extend the TBS table to support 16-QAM. The TBS table for UL 16-QAM is given in Table 2. |
| [4] | Proposal 10: For UL 16QAM, TBS table can be extended to up to 2536 bits for each IRU. Table 6: TBS table for UL 16QAM 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 696. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 328; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 712; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1000; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2536. 16; 328; 632; 968; 1288; 1608; 1928. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152; 2536. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2536. |
| [5] | Proposal 8: Support 16QAM for NPUSCH needs further study: - Option1: Extend TBS table and generate modulation, TBS and MCS table. - Option2: Reinterpret the number of resource unit for modulation order of 16QAM. |
| [6] | Proposal 8 The TBS/MCS Table to support 16-QAM in UL consists of: - All legacy QPSK entries. - The entries for 16-QAM are from legacy LTE Table 7.1.7.2.1-1 in TS 36.213 ranging from a TBS = 280 bits to TBS = 4968, with only two changes: o In the above-mentioned TBS range, the TBS > 2536 are excluded as to be compliant with the objective of preserving for UL the max TBS for Rel-16. o To transmit the max Rel-16 TBS with half of the time domain resources replacing TBS = 2472 bits by TBS = 2536 bits. |
| [8] | Proposal 12: RAN1 to discuss whether to introduce one or more “implicit MCS” for retransmissions in the MCS table for UL 16-QAM. |

Based on the inputs, the following can be proposed:

Proposal 6: to support 16QAM for NB-IoT UL, at least the following TBS indices are introduced

| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472,2536] | 2984 | 4008 | 4968 |

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 7: The crossing point from QPSK to 16QAM.

There are following proposals on TBS design of 16-QAM for UL unicast

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 15: The break-point for 16-QAM in UL is I_TBS ≥ 14 based on Table 2. |

As not many companies provide input, the following is proposed:

Proposal 7: further study on the switching point from QPSK to 16QAM for NB-IoT UL.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 8: Applicability

The following are proposed on scheduling of TBS and modulation:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 7: Repetition is not supported for 16-QAM in UL. Proposal 8: 16-QAM can be used for NPUSCH with at least multi-tone 3, 6 and 12 subcarriers. |
| [3] | Proposal 16: FFS whether to Support 16-QAM with repetition in the UL. Proposal 17: 16-QAM is not supported for sub-PRB allocation. |
| [4] | Proposal 9: UL 16QAM should be supported for 3/6/12 subcarriers allocation. |
| [6] | Observation 2 Although 16-QAM requires a high SNR and resource allocations < 12 subcarriers are mainly targeted towards low SNR regimes (especially single-tone allocations), still there are scenarios (e.g., NPRACH and NPUSCH coexistence) where multi-tone allocations could benefit from a higher order modulation. Proposal 7 The support of 16-QAM in UL is only for NPUSCH Format 1 using both full-PRB allocations and multi-tone allocations consisting of 6 and 3 allocated subcarriers. |
| [8] | Proposal 15: UL 16-QAM is applicable for NPUSCH scheduled from a DCI with CRC scrambled by C-RNTI. - At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS. - FFS: Applicability of 16-QAM for PUR or EDT. Proposal 16: UL 16-QAM is applicable at least to NPUSCH with full-PRB allocations. FFS NPUSCH with sub-PRB allocations. |

On the application of 16QAM to full PRB/sub-PRB allocations, there are following options:

  - Multi-tone transmission with 3, 6 and 12 subcarriers

    - Huawei, HiSilicon, ZTE, Ericsson,

  - Multi-tone transmission with only 12 subcarriers

    - Nokia, Nokia Shanghai Bell,

  - FFS on sub-PRB allocation

    - Qualcomm

Therefore, the following is proposed based on the input:

Proposal 8: RAN1 to further discuss on the applicability to different number of subcarriers.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

DCI

### Issue 9: MCS field.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 9: The introduction of 16-QAM shall not increase the NPDCCH blind decodes. Proposal 10: The introduction of 16-QAM shall avoid increasing DCI size. |
| [3] | Proposal 9: The size of the MCS field in DCI N1 in UE-specific search space is increased to 5 bits. Proposal 14: The size of the MCS field in DCI N0 in UE-specific search space is increased to 5 bits. |
| [4] | Proposal 3: 4-bit MCS table could be defined for DL 16QAM in guard-band/standalone deployment. Table 3: Possible 4-bit MCS table for DL 16QAM MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 2. 2; 2; 4. 3; 2; 6. 4; 2; 8. 5; 2; 10. 6; 4; 12. 7; 4; 13. 8; 4; 14. 9; 4; 15. 10; 4; 16. 11; 4; 17. 12; 4; 18. 13; 4; 19. 14; 4; 20. 15; 4; 21. Proposal 4: 4-bit MCS table for DL 16QAM can be defined based on the MCS entries of Rel-16 NB-IoT. - Remove the existing 6 MCS entries - Add new 8 MCS entries Proposal 11: 5-bit MCS table could be used for UL 16QAM. - MCS 0~13 correspond to TBS 0~13 with QPSK modulation - MCS 14~24 correspond to TBS 11~21 with 16QAM modulation Table 7: Possible 5-bit MCS table for UL 16QAM MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 1. 2; 2; 2. 3; 2; 3. 4; 2; 4. 5; 2; 5. 6; 2; 6. 7; 2; 7. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12; 2; 12. 13; 2; 13. 14; 4; 11. 15; 4; 12. 16; 4; 13. 17; 4; 14. 18; 4; 15. 19; 4; 16. 20; 4; 17. 21; 4; 18. 22; 4; 19. 23; 4; 20. 24; 4; 21. 25~31; reserved; reserved. Proposal 12: The most significant bit of the subcarrier indication field can be used for UL 5-bit MCS indication. |
| [5] | Proposal 3: To support 16QAM of NPDSCH, the MCS field in DCI format N1 is enlarged, optimized or reinterpreted, which needs further discussion. |
| [7] | MCS Index; Modulation Order; TBS Index. 0; 2; 0. 1; 2; 1. 2; 2; 2. 3; 2; 3. 4; 2; 4. 5; 2; 5. 6; 2; 6. 7; 2; 7. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12; 2; 12. 13; 2; 13. 14; 4; 13. 15; 4; 14. 16; 4; 15. 17; 4; 16. 18; 4; 17. 19; 4; 18. 20; 4; 19. 21; 4; 20. 22; 4; 21. Table 2: DL MCS table for NB-IOT 16QAM Proposal: 5-bit MCS should be adopted and Table 2 should be DL MCS table |
| [9] | Proposal 7: Increase the Modulation and coding scheme DCI field from 4 to 5 bits |

On the size of the MCS field, there are following options:

  - 4 bits as legacy

    - Huawei, HiSilicon, ZTE (for DL),

  - 5 bits

    - Nokia, Nokia Shanghai Bell, ZTE (for UL), MediaTek, Sierra Wireless, Ericsson

Based on the majority view, the following is proposed

Proposal 9: The MCS field to support 16QAM is increased from 4bits to 5 bits.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Downlink power allocation to support 16QAM

### Issue 10: Power allocation.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 11: The ratio of NPDSCH EPRE to NRS EPRE should be signaled for the following cases: - NPDSCH in symbols without NRS and CRS - NPDSCH in symbols with CRS (only for “In-band” deployment) - NPDSCH in symbols with NRS |
| [4] | Proposal 13: The same total transmit power is set for each OFDM symbol for Rel-17 DL power allocation. Proposal 14: Only a new higher layer parameter Pn which indicates the ratio of NPDSCH EPRE and NRS EPRE in symbols with NRS needs to be introduced for Rel-17 DL power allocation. |
| [5] | Proposal 5: Network should semi-statically configure three types of NPDSCH EPRE separately. |
| [6] | Proposal 4 The data-to-power ratios for 16-QAM in DL, uses as baseline the DL power control definitions in LTE  Stand-alone and Guard-band deployments: - Type A refers to the NPDSCH symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB] - Type B refers to the NPDSCH symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB]  In-band deployments: - Type A refers to the NPDSCH symbols without NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB] - Type B refers to the NPDSCH symbols with NRS and without CRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB] - Type C refers to the NPDSCH symbols without NRS and with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB] Where: ρ_a = PA [dB] PB is the index that refers to the linear ratio between ρ_b and ρ_a PC is the index that refers to the linear ratio between ρ_c and ρ_a FFS: signaling details of ρ_a, ρ_b, ρ_c. or PB, PC. |
| [8] | Observation 3: In NB-IoT, the power level change of NPDSCH relative to NRS does not have impact on legacy NPDSCH with QPSK. This does not hold anymore with 16-QAM NPDSCH. Proposal 8: Define three different levels ρA,ρB,ρC of EPRE of NPDSCH with respect to EPRE of NRS: - ρA: Applicable to NPDSCH in symbols with NRS. - ρB: Applicable to NPDSCH in symbols with CRS (required for in-band NB-IoT only). - ρC: Applicable to NPDSCH in symbols without NRS and CRS. Proposal 9: RAN1 to decide among the following alternatives: - Alt1: Rel-16 NRS power levels are kept (ρA=ρC). - Alt2: An additional “power boost” value for NRS is introduced (ρA≠ρC). Proposal 10: The UE derives the values of ρA, ρB, ρC implicitly based on - Power boost value for NRS (if introduced) - NRS and CRS relative power level. - Number of NRS and CRS ports. |

Based on the input, the following is proposed:

Proposal 10: The signal of ration of NPDSCH EPRE to NRS EPRE for following cases explicitly or implicitly are supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Configuration and capability

### Issue 11: capability

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 1: Support of 16-QAM by UE is indicated by UE capability signalling. |
| [5] | Proposal 1: Introduce UE capability signaling for the support of 16QAM for unicast NPDSCH. Proposal 6: Introduce UE capability signaling for the support of 16QAM for unicast NPUSCH. |
| [8] | Proposal 4: Introduce a unicast RRC parameter to enable 16-QAM for NPDSCH. Introduce an optional UE capability indicating support of DL 16-QAM. Proposal 11: Introduce a unicast RRC parameter to enable 16-QAM for NPUSCH. Introduce an optional UE capability indicating support of UL 16-QAM. |

Based on the input, the following is proposed:

Proposal 11: Support of 16QAM of DL and UL are indicated by an optional UE capability signaling separately.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

### Issue 12: configuration

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 2: 16-QAM feature is configured (e.g. enabled/disabled) via UE-specific RRC signalling. Proposal 3: Consider separate 16-QAM capability signalling and configuration for UL and DL. |
| [5] | Proposal 4: The configuration of 16QAM for NPDSCH can be enabled/disabled by eNB through RRC signaling. Proposal 7: The configuration of 16QAM for NPUSCH can be enabled/disabled by eNB through RRC signaling. |
| [8] | Proposal 4: Introduce a unicast RRC parameter to enable 16-QAM for NPDSCH. Introduce an optional UE capability indicating support of DL 16-QAM. Proposal 11: Introduce a unicast RRC parameter to enable 16-QAM for NPUSCH. Introduce an optional UE capability indicating support of UL 16-QAM. |

Based on the input, the following is proposed:

Proposal 12: 16QAM for UL and DL are configured by UE-specific RRC signaling separately.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Channel quality reporting

### Issue 13: Channel quality reporting

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 10: Study further channel quality reporting to support 16-QAM in DL. In the WID, one of the objective is to extend the NB-IoT channel quality reporting based on the framework of Rel-14-16, to support 16-QAM in DL. We see two potential areas here – - Extend the channel quality report to cover the relevant spectral efficiency range for 16-QAM. - Extend the channel quality report to UE in CONNECTED mode (in addition to Msg3). |
| [4] | Proposal 15: For Rel-17 channel quality report, the CQI table needs to be discussed in RAN1. |
| [6] | Proposal 5 The CQI reporting definition to support 16-QAM in DL is as in clause 7.2.3 of TS 36.213 for LTE-MTC with the corresponding updates to adapt it to NB-IoT. Proposal 6 The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.213 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL. - The NPDCCH and NPDSCH repetition level is equal to 1. o candidateRep-M is reported when the SINR is suitable for 16-QAM with ITBS =A/D.  FFS: A for stand-alone and guard-band deployments, and D for in-band deployments. o candidateRep-N is reported when the SINR is suitable for 16-QAM with ITBS =B/E.  FFS: B for stand-alone and guard-band deployments, and E for in-band deployments. o candidateRep-O is reported when the SINR is suitable for 16-QAM with ITBS =C/F.  FFS: C for stand-alone and guard-band deployments, and F for in-band deployments. |
| [10] | Observation 1: In current NB-IoT, the channel quality reporting in Msg3 and connected mode are NPDCCH repetition level reporting. Observation 2: For UE in good coverage and hence not needing repetition, the repetition-level based channel quality reporting does not convey sufficiently fine-grained channel quality information. Proposal 1: Finer NB-IoT channel quality reporting is supported to provide sufficient channel quality information in good coverage, particularly for 16-QAM. Proposal 2: Re-purpose the channel quality reporting field in Msg3 and MAC CE to support finer channel quality reporting. |

As the channel quality report depends on the discussion of max DL TBS, application on repetition etc, the following is proposed:

Observation 1: The channel quality report depends on discussion of DL TBS and needs further discussion.

Please input your comments in the following table

| Companies | Comments |
| --- | --- |

Others

### Issue 16: Others

There are also other proposals as below:

| Sourcing | proposals |
| --- | --- |
| [3] | Proposal 18: 16-QAM can be supported together with DL/UL multi-TB scheduling in unicast. Proposal 19: 16-QAM can be supported together with PUR. Proposal 20: 16-QAM is not supported for UL EDT. |
| [4] | Proposal 16: Soft buffer size needs to be specified based on maximum TBS for DL 16QAM. |
| [8] | Proposal 7: Do not introduce LBRM for 16-QAM. The soft buffer size is doubled with respect to QPSK. Proposal 14: RAN1 to consider adding an additional power control parameter to allow for increased power with 16-QAM (e.g. similar to ΔTF) |

Please input your comments if you think any proposed listed in this section (2.7) or any other issue can be discussed in this meeting:

| Companies | Comments |
| --- | --- |

Summary

- References

- RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.

- R1-2007618	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2008073	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2008697	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2008920	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2008930	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2008969	Further considerations on support of 16QAM for NB-IOT	MediaTek Inc.

- R1-2009112	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2009125	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

- R1-2007620	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon