3GPP TSG RAN meeting #90e										RP-20xxxx

Electronic Meeting, December 7-11, 2020

Status Report to TSG

Agenda item:			10.7.1

| WI / SI Name | Additional enhancements for NB-IoT and LTE-MTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |
| Unique ID | 860044 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201306 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2021 | Performance part: 03/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 25% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG 1 |
| --- | --- | --- |
| Rapporteur | Name | Yubo YANG Emre YAVUZ |
|  | Company | Huawei Ericsson |
|  | Email | yangyubo1@huawei.com emre.yavuz@ericsson.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

In RAN1#103-e meeting, 23 contributions ([1]-[23]) were submitted, and the following were achieved.

For NB-IoT 16QAM:

Agreement

At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7.

Agreement

For inband deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits (ISF=7).

Agreement

Different breaking points (QPSK16QAM) are used for standalone/guardband and inband deployments.

- FFS the details of the breaking point.

Agreement

Explicit or implicit signaling of power ratios of NPDSCH EPRE to NRS EPRE for the following cases is supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Agreement

For 16-QAM in NB-IoT, separate optional UE capabilities for UL and DL are supported:

- The support of 16QAM in DL is indicated by an optional UE capability signaling.

- The support of 16QAM in UL is indicated by an optional UE capability signaling.

Agreement

For 16-QAM in NB-IoT, separate UE-specific RRC signaling for UL and DL are supported:

- 16QAM for UL is configured by UE-specific RRC signaling.

- 16QAM for DL is configured by UE-specific RRC signaling.

Working Assumption

The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

Working Assumption

The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Working Assumption

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

Agreement

Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

Agreement

16QAM can be used at least for multi-tone transmission with 12 subcarriers.

- FFS: 3 and 6 subcarriers.

For support of additional PDSCH scheduling delay for introduction of 14-HARQ processes for eMTC:

Agreement

The following working assumption is confirmed

Introduce a new optional UE capability to support 14 HARQ processes

Agreement

The design of the 14 HARQ processes feature accounts for the presence of non-BL/CE UL and DL subframes in the PUCCH non-repetition case.

- FFS: PDSCH scheduling delays

- FFS: HARQ-ACK delays

- FFS: Configurable/dynamic set of PDSCH delays/HARQ-ACK delays

Agreement

For the support of 14 HARQ processes, the solution to assign PDSCH scheduling delays should be able to minimize unnecessary waste of subframes derived from the presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- The following solutions will be further investigated:

  - The indication of subframe types for the PDSCH scheduling delay of 7 are:

    - 1 BL/CE DL subframe + 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 1 BL/CE DL subframe.

    - 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 2 BL/CE DL subframes.

  - Configurable delays including other values than 2 and 7.

- Other solutions are not precluded.

Agreement

For the support of 14 HARQ processes, the solution to assign HARQ-ACK delays should aim to maximize the number of HARQ processes that can be scheduled in presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- Different percentages of presence of non-BL/CE subframes can be analyzed as to represent typical scenarios and determine which HARQ-ACK delays should be included.

2.1.2	Remaining Open issues

  - For NB-IoT, confirmation and down-selection of TBS tables for UL and DL, and details of MCS tables for UL and DL. [NB-IoT]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT]

  - Investigation and down-selection of solutions to support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC]

  - Add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only. [LTE-MTC]

2.2	RAN2

Contributions [24] – [44] were submitted to RAN2#112-e meeting. The list of agreements made in the meeting is captured in [45].

2.2.1	Agreements

NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

RAN2 discussed NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF and made the following agreements:

RAN2#112-e agreements: Enhancements to the random-access procedure are not considered. The solution includes reduction of the time between declaration of RLF and the start of the random-access procedure (points C and D) FFS whether the solution includes reduction of the time between out-of-sync detection and declaration of RLF (points B and C)

NB-IoT carrier selection based on the coverage level and associated carrier specific configuration

RAN2 discussed NB-IoT carrier selection based on the coverage level and associated carrier specific configuration and made no agreements.

2.2.2	Remaining Open issues

- For NB-IoT, support of NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

- For NB-IoT, support of NB-IoT carrier selection based on the coverage level and associated carrier specific configuration

- For NB-IoT, RAN2 aspects of support of 16QAM.

- For eMTC, RAN2 aspects of support of additional PDSCH scheduling delay to support 14-HARQ processes in DL.

- For eMTC, RAN2 aspects of support of DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A.

2.3	RAN3

2.3.1	Agreements

There was no discussion in RAN3.

2.3.2	Remaining Open issues

- Support of NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.)

2.4	RAN4

2.4.1	Agreements

There was no discussion in RAN4.

2.4.2	Remaining Open issues

- For NB-IoT, specify 16-QAM for unicast in UL and DL. [NB-IoT]

- For NB-IoT, specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT]

- For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC]

- Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements. [NB-IoT][LTE-MTC]

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2007618	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2008073	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2008697	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2008920	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2008930	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2008969	Further considerations on support of 16QAM for NB-IOT	MediaTek Inc.

- R1-2009112	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2009125	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

- R1-2009477	Feature lead summary #1 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]	Moderator (Huawei)

- R1-2009658	Feature lead summary #2 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]	Moderator (Huawei)

- R1-2009730	Feature lead summary #3 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]	Moderator (Huawei)

- R1-2007619	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2008074	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2008698	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC	ZTE

- R1-2008931	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson, Telefónica, Verizon, SoftBank, AT&T, Telstra

- R1-2009113	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2009124	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

- R1-2009513	Feature Lead Summary [103-e-LTE-Rel17_NB_IoT_eMTC-02]	Moderator (Ericsson)

- R1-2009514	Feature Lead Summary#2 [103-e-LTE-Rel17_NB_IoT_eMTC-02]	Moderator (Ericsson)

- R1-2009515	Feature Lead Summary#3 [103-e-LTE-Rel17_NB_IoT_eMTC-02]	Moderator (Ericsson)

- R1-2007620	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

- R1-2008699	DL TBS increase for eMTC	ZTE

- R1-2008932	On the support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson

- R2-2008937	Impact on Static devices	THALES

- R2-2009058	Further consideration on measurement in connected mode	ZTE Corporation, Sanechips

- R2-2009059	Further consideration on multi carriers configuration and selection	ZTE Corporation, Sanechips

- R2-2009146	Discussion on the corresponding measurement before RLF	Spreadtrum Communications

- R2-2009147	Discussion on enhanced paging carrier selection and multi carrier configuration	Spreadtrum Communications

- R2-2009180	NB-IoT carrier selection and configuration based on coverage level	Ericsson

- R2-2009268	Enhancements for Re-establishment time reduction	Nokia, Nokia Shanghai Bell

- R2-2009269	Analysis on carrier selection options for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-2009731	Neighbour cell measurements in RRC_CONNECTED	Huawei, HiSilicon

- R2-2009732	Paging carrier selection based on CEL and on DRX	Huawei, HiSilicon

- R2-2009788	Report for [Post111-e][923][NBIOT R17] RLF Enhancements (Qualcomm)	Qualcomm Incorporated

- R2-2009789	Way forward for connected mode neighbour cell measurement in NB-IoT	Qualcomm Incorporated

- R2-2009790	Support for NB-IoT carrier selection based on the coverage level	Qualcomm Incorporated

- R2-2009876	Neighbor cell measurements triggering before RLF	Lenovo, Motorola Mobility

- R2-2010076	Reducing time taken for reestablishment procedures in NB-IoT	Ericsson

- R2-2010077	NB-IoT carrier selection and configuration based on coverage level	Ericsson

- R2-2010249	Discussion on Total Interruption Time	ETRI

- R2-2010460	Measurement before radio link failure	MediaTek Inc.

- R2-2010470	Carrier selection enhancement	MediaTek Inc.

- R2-2010905	[draft] LS on NB-IoT connected mode neighbour cell measurements	Qualcomm

- R2-2010906	[AT112-e][302][NBIOT R17] Carrier selection (Ericsson)	Ericsson

- R2-2010911	RAN2 agreements for Rel-17 NB-IoT and LTE-MTC	Document Rapporteur (Ericsson)

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template