3GPP TSG RAN WG1 Meeting #103-e 	R1-2007617

E-meeting, October 26th – November 13th, 2020

Agenda Item:	8.16

Source:	Huawei, HiSilicon

Title:	On support of DL 1024QAM for NR FR1

Document for:	Discussion and Decision

Introduction

In RAN#89 e-meeting, a new work item as introduction of DL 1024-QAM for NR FR1 was approved [1]. The objective is to specify downlink 1024-QAM for NR PDSCH operation in FR1, together with related procedures, signaling and necessary RF requirements, with the main objectives listed below.

- Specify high order modulation for PDSCH [RAN1] - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH - Specify corresponding MCS table with 1024QAM entries as defined in E-UTRA - Note: DCI overhead for MCS indication should be the same as in Rel-15 - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA - Specify corresponding RRC signalling and UE capabilities [RAN2] - Note: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability - Specify corresponding UE and BS RF core requirements [RAN4] - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

The cell size(s) and type of stationary wireless scenarios that supports 1024-QAM are still open for discussion. Although it will be studied and decided by RAN4, it is beneficial for RAN1 to provide system-level evaluations as a reference for RAN4 to reach the final conclusion. In this contribution, the system-level evaluation results for scenarios of different cell sizes are provided. And the details on RAN1 specification issues for supporting 1024-QAM including the design of MCS and CQI tables as well as constellation mapping are also discussed.

Discussion

On cell size(s) and type of scenarios

As defined in [2], there are three different BS classes for BS type 1-C including wide area BS, medium range BS and local area BS, which are of different limits for rated output power. It should be noted that, when deciding the cell size(s) that supports 1024-QAM, verifying the system-level performance improvement provided by 1024-QAM should be of the highest priority. Usually, such kind of system-level evaluation is conducted by RAN1 rather than RAN4, as in the introduction of 1024-QAM for LTE/E-UTRA [4]. Thus, it is more thoughtful and deliberating for RAN1 to identify the cell size(s) in which supporting 1024-QAM can provide notable performance gain. It is not necessary to support 1024-QAM for the cell size that no gain or only marginal gain can be obtained.

Proposal 1: RAN1 should conduct system-level evaluation to identify the cell size(s) in which supporting 1024-QAM can provide notable performance gain.

The discussion is focused on the class of wide area BS, which corresponds to the macro BS whose typical application scenarios include dense urban and urban macro. Thus in the following part, the performance comparison between schemes with and without 1024-QAM for these two scenarios are provided. The detailed simulation assumptions are listed in Table A.1 of Appendix A. Regarding that the MCS table supporting 1024-QAM is still under discussion, it is assumed that 6 MCS entries listed in Table III for 1024-QAM are added to the current MCS index table 2 (Table 5.1.3.1-2) in [3] in order to fully take advantage of 1024-QAM, without removing any legacy entries. The evaluation results in terms of gain of cell average spectral efficiency (SE) and the SE of cell centered UEs (95%-tile UEs) with full buffer traffic are provided in the following Table I.

Table I. Evaluation results with and without 1024-QAM of full buffer

| Scenario | Dense Urban |  | Urban macro |  |
| --- | --- | --- | --- | --- |
| Scheme | w/o 1024 | with 1024 | w/o 1024 | with 1024 |
| Gain of cell average SE | - | -0.02% | - | 0.09% |
| Gain of 95%-tile UE SE | - | 1.93% | - | 0.71% |

It can be observed from the above table that supporting 1024-QAM can at most provide 1.93% gain in terms of cell centered SE under the traffic model of full buffer, while there is no gain for cell average SE.

Then in the following Table II, the evaluation results in terms of gain of cell average user perceived throughput (UPT) and 95%-tile UE UPT with FTP traffic are given.

Table II. Evaluation results with and without 1024-QAM of FTP traffic

| Scenario | Dense Urban |  | Urban macro |  |
| --- | --- | --- | --- | --- |
| Scheme | w/o 1024 | with 1024 | w/o 1024 | with 1024 |
| RU | ~25% |  | ~40% |  |
| Gain of average UPT | - | 2.40% | - | 2.40% |
| Gain of 95%-tile UE UPT | - | 2.86% | - | 2.77% |
| RU | ~60% |  | ~90% |  |
| Gain of average UPT | - | 2.33% | - | 0.83% |
| Gain of 95%-tile UE UPT | - | 2.17% | - | 0.00% |

According to the above table, it is observed that in the case of 25-40% RU, the performance gain from 1024-QAM for both average and 95% UPT are less than 3%. And in the case of 60%-90% RU, the corresponding gain is less than 2.5% and less than 1% for dense urban and urban macro, respectively.

Please note that the above results are based on the assumption without transmit power reduction at the BS side. Actually, the macro BS needs to additionally reduce the transmit power in some scale in order to satisfy the Tx EVM requirement of 1024-QAM. By considering such power reduction, the corresponding performance are given below.

Table III. Evaluation results with different values of additional transmit power reduction

| Scenario | Dense Urban |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scheme | w/o 1024 (baseline) | with 1024 |  |  |  |
| Transmit power reduction | 0 dB | 1 dB | 2 dB | 3 dB | 4 dB |
| RU | ~60% |  |  |  |  |
| Gain of average UPT | - | 2.65% | 1.41% | -1.14% | -1.48% |
| Gain of 95%-tile UE UPT | - | 2.17% | 0.00% | -2.08% | -4.08% |

It can be observed that when additional power reduction reaches 3 dB, supporting 1024-QAM will result in 1.14% loss for average UPT and 2.08% loss for 95% UPT. The performance will be further degraded with larger value of transmit power reduction.  To conclude, introducing 1024-QAM for wide range BS will lead to performance degradation if additional 3 dB or more transmit power reduction is required at the BS side. In accordance with above results, the following observations can be obtained.

Observation 1: For wide area BS without transmit power reduction and with full buffer traffic, supporting 1024-QAM can at most provide 1.93% gain for cell centered SE, while no gain can be obtained for cell average SE.

Observation 2: For wide area BS without transmit power reduction and with FTP traffic, the performance gain provided by 1024-QAM for both average and 95% UPT are less than 3%.

Observation 3: For wide area BS, supporting 1024-QAM will lead to performance degradation when more than 3 dB transmit power reduction is needed.

On MCS index table with 1024-QAM

First of all, it is necessary to stress that the motivation of introducing 1024-QAM to NR downlink is to further enhance the network throughput by improving the spectral efficiency for UE with good channel condition, especially for fixed wireless access (FWA) users and small cell users. Due to the very low mobility environment, FWA user is more likely to have higher spectral efficiency (SE) than ordinary mobile UE. Thus, when concerning the specification design for supporting 1024-QAM, it is more appropriate to follow the principle of improving the performance of UE under good coverage as much as possible.

In current specification, three different MCS index tables are specified for PDSCH. Note that the MCS index table supporting 256-QAM, i.e., MCS index table 2, was designed based on MCS index table 1 by removing several legacy entries of low SE and replacing them with 256-QAM entries of the same number, including the entry for retransmission indication. Such a design can avoid to introduce different code rates from those in MCS index table 1 for 64QAM and lower order modulations, which is beneficial to reduce the implementation complexity for channel encoding and decoding. For the same reason, similar principle is preferred to be adopted in designing the MCS index table supporting 1024-QAM, which can be obtained based on MCS index table 2 by removing a few legacy entries and adding 1024-QAM entries.

Proposal 2: The MCS index table supporting 1024-QAM should be designed based on MCS index table 2 by removing a few legacy entries and replacing them with 1024-QAM entries.

To specify the MCS entries for 1024-QAM, the crossover point with 256-QAM needs to be determined. As the coding scheme and reference signals of NR are different from those of LTE, evaluations for NR are needed. In Figure 1, the SNR for 10% BLER versus spectral efficiency is illustrated for both 256-QAM and 1024-QAM, which is based on the assumption in Table A.2. It can be shown that the crossover point is around the SE of 7.4, which is very close to the highest SE for 256-QAM.

Figure 1. Illustration of crossover point between 256-QAM and 1024-QAM

Observation 4: The crossover point from 256-QAM to 1024-QAM is at spectral efficiency of 0.74.

Regarding the upper limit of SE, it can be directly obtained by setting target code rate as 948/1024, which is the maximum value of the code rate for NR LDPC channel coding. Therefore, the highest SE is determined as 9.2578 bps/Hz.

Based on the above discussion, the SE values of other MCS entries for 1024-QAM can be calculated by linear interpolation between 7.4063 bps/Hz and 9.2578 bps/Hz.

Proposal 3: The lowest and highest spectral efficiency in MCS entries for 1024-QAM are 7.4063 bps/Hz and 9.2578 bps/Hz respectively, and the spectral efficiencies of other MCS entries for 1024-QAM are obtained by linear interpolation from 7.4063 bps/Hz to 9.2578 bps/Hz.

1024-QAM has already been specified for E-UTRA in Release-15, and the corresponding MCS index table supporting 1024-QAM only contains four or five MCS entries of 1024-QAM, while most of legacy entries of low SE are kept. The MCS entries with low SE may seldom be used for FWA/small cell users due to the good channel quality. Keeping too many low SE entries would decrease the granularity for high SE entries, which may decrease the scheduling efficiency and thus data rates for UEs with good channel quality. One reason for E-UTRA to not specify too many 1024-QAM entries is that E-UTRA uses TBS instead of target code rate for MCS, and too many 1024-QAM entries will lead to a lot of specification work. While for NR, the specification work load will not be increased in a large scale with specifying more 1024-QAM entries. Hence, it is proposed to specify more MCS entries for 1024-QAM than that in E-UTRA.

To support URLLC, a separate MCS index table with low SE values, i.e., MCS index table 3, was introduced in NR. To fulfill the reliability requirement, the number of MCS entries of low SE (lowest modulation, i.e., QPSK) accounts for half of the total entries within the entire table in order to decrease the granularity between low SE entries. This principle can be applied to 1024-QAM as well. Specifically, the number of high SE entries, i.e., entries for 256-QAM and 1024-QAM, can account for at least half of the total number of MCS entries in the MCS index table. As there have been eight entries for 256-QAM in MCS index table 2, and thus at least 6 MCS entries for 1024-QAM can be introduced for the new MCS index table. An example with 6 entries for 1024-QAM is given in the following table.

Table IV. MCS entries for 1024-QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| … | … | ... | … |
| 21 | 10 | 790 | 7.7149 |
| 22 | 10 | 822 | 8.0235 |
| 23 | 10 | 853 | 8.3321 |
| 24 | 10 | 885 | 8.6407‬ |
| 25 | 10 | 916 | 8.9493‬ |
| 26 | 10 | 948 | 9.2578 |
| … | … | reserved |  |

Proposal 4: At least 6 MCS entries for 1024-QAM are supported in the MCS index table for PDSCH.

On CQI table with 1024-QAM

There are only two entries for 1024-QAM in the CQI table specified in E-UTRA, while most entries of lower SE are kept which may be seldom used by FWA/small cell users with good coverage. Keeping too many low SE entries would increase the granularity of high SE entries, which would decrease the accuracy of CQI feedback. It is beneficial to specify more entries for 1024-QAM than that in E-UTRA so as to improve the accuracy of CQI feedback for UEs with higher SE. Similar to the design principle of MCS index table, the number of high SE entries, i.e., entries for 256-QAM and 1024-QAM can also account for at least half of the total number of CQI entries in the corresponding table. In current CQI table 2, there are four 256-QAM entries and thus at least 4 entries for 1024-QAM can be introduced. As usual, the CQI entries for 1024-QAM can be a sub-set of MCS entries in MCS index table supporting 1024-QAM.

Proposal 5: At least 4 CQI entries for 1024-QAM are supported in the CQI table.

On constellation mapping

The specified constellation mapping in Rel-15 E-UTRA follows the criterion of Gray mapping, which obviously has better performance than non-Gray mapping. We don’t see the necessity to introduce any other mapping scheme. Therefore, reusing the same constellation mapping approach in E-UTRA is straightforward.

Proposal 6: The constellation mapping for 1024-QAM of E-UTRA is reused to NR.

Conclusions

This contribution discussed RAN1 specification issues for 1024-QAM including the design of MCS and CQI tables as well as constellation mapping, and also provided system-level evaluation results to verify the effectiveness of 1024-QAM for wide area BS. The following observations and proposals are given:

Observation 1: For wide area BS without transmit power reduction and with full buffer traffic, supporting 1024-QAM can at most provide 1.93% gain for cell centered SE, while no gain can be obtained for cell average SE.

Observation 2: For wide area BS without transmit power reduction and with FTP traffic, the performance gain provided by 1024-QAM for both average and 95% UPT are less than 3%.

Observation 3: For wide area BS, supporting 1024-QAM will lead to performance degradation when more than 3 dB transmit power reduction is needed.

Observation 4: The crossover point from 256-QAM to 1024-QAM is at spectral efficiency of 0.74.

Proposal 1: RAN1 should conduct system-level evaluation to identify the cell size(s) in which supporting 1024-QAM can provide notable performance gain.

Proposal 2: The MCS index table supporting 1024-QAM should be designed based on MCS index table 2 by removing a few legacy entries and replacing them with 1024-QAM entries.

Proposal 3: The lowest and highest spectral efficiency in MCS entries for 1024-QAM are 7.4063 bps/Hz and 9.2578 bps/Hz respectively, and the spectral efficiencies of other MCS entries for 1024-QAM are obtained by linear interpolation from 7.4063 bps/Hz to 9.2578 bps/Hz.

Proposal 4: At least 6 MCS entries for 1024-QAM are supported in the MCS index table for PDSCH.

Proposal 5: At least 4 CQI entries for 1024-QAM are supported in the CQI table.

Proposal 6: The constellation mapping for 1024-QAM of E-UTRA is reused to NR.

- References

RP-202044, “New WI: Introduction of DL 1024-QAM for NR FR1”, Ericsson, Electronic Meeting, September 14 - 18, 2020.

3GPP TS 38.104 v16.4.0, “Base station (BS) radio transmission and reception”.

3GPP TS 38.214 v16.2.0, “Physical layer procedures for data”.

3GPP TR 38.783 V15.0.0, “Introduction of 1024 Quadrature Amplitude Modulation (QAM) in LTE downlink”.

- Appendix A

Table A.1. System-level evaluation assumptions

| Scenario | Dense Urban | Urban Macro |
| --- | --- | --- |
| Layout | Single layer: 7 sites, 3 sector per site | 7 sites, 3 sector per site |
| Inter-BS distance | 200m | 500m |
| Carrier frequency | 4GHz |  |
| Simulation bandwidth | 20MHz |  |
| Numerology | 30KHz |  |
| Duplex configuration | TDD: DDDSU |  |
| Channel model | 3D UMa |  |
| BS Tx power | 44dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 44dBm | 49dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm |
| BS antenna configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |  |
| BS antenna height | 25m |  |
| BS antenna element gain + connector loss | 8dBi |  |
| BS antenna element gain pattern | According to TR36.873 |  |
| UE Tx power | Maximum 23dBm |  |
| UE antenna configuration | 2Tx4Rx |  |
| UE antenna height | 1.5m |  |
| UE antenna gain | 0dBi |  |
| UE antenna element gain pattern | Omnidirectional |  |
| UE receiver noise figure | 9dB |  |
| UE distribution | 10 UEs per sector; 80% indoor+20% outdoor |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Real |  |
| Traffic model | 1) Full buffer; 2) FTP model with packet size 0.5Mbytes |  |
| MIMO scheme | SU-MIMO with rank adaptation and MU-MIMO |  |
| CSI scheme | Reciprocity based; 2T SRS |  |

Table A.2. Link-level evaluation assumptions

| Parameter | Value |
| --- | --- |
| Channel model | AWGN |
| Antenna configuration | 1T1R |
| Subcarrier spacing | 30 kHz |
| Rank | 1 |
| Channel estimation | ideal |
| Tx EVM | 0% |