3GPP TSG RAN WG1 Meeting #103-E				R1-2007720

e-Meeting, October 26th – November 13th, 2020

Source:	BUPT

Title:	Evaluation of achievable positioning accuracy

Agenda item:	8.5.2

Document for:	Discussion

1	Discussion

The NR positioning study item will address positioning solutions. The outcome of the study will be documented in the technical report 38.857.

In this contribution, we update the NR positioning evaluation results. Furthermore, we propose to include the results in TR 38.857.

Proposal 1 Add the text proposal in Appendix A to the TR 38.857.

2	Conclusion

We have the following proposal:

Proposal 1 Add the text proposal in Appendix A to the TR 38.857.

Appendix

A. Text proposal to the TR 38.857

8	Performance evaluations for R17 performance targets

8.1	Performance analysis of Rel-16 positioning solutions

8.1.1	Positioning accuracy analysis

8.1.1.1	Results from source [1]

8.1.1.1.1	Description of evaluation scenarios

Table 8.1.1.1.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 1], [InF-SH], [FR1], [R 16 DL-TDOA] | [Case 2], [InF-DH], [FR1], [R 16 DL-TDOA] | [3], [InF-SH], [FR2], [R 16 DL-TDOA] | [4], [InF-DH], [FR2], [R 16 DL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline | baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100M | 100M | 400M | 400M |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | TS38.211 R16 PRS comb-12 pattern | TS38.211 R16 PRS comb-12 pattern | TS38.211 R16 PRS comb-12 pattern | TS38.211 R16 PRS comb-12 pattern |
| Reference signal (type of sequence, number of ports, …) | TS38.211 R16 PRS | TS38.211 R16 PRS | TS38.211 R16 PRS | TS38.211 R16 PRS |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 12 symbols | 12 symbols | 12 symbols | 12 symbols |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 1 | 1 | 1 | 1 |
| Uplink power control (applied/not applied) | not applied | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Phase tracking | Phase tracking | Phase tracking | Phase tracking |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Chan | Chan | Chan | Chan |
| Network synchronization assumptions | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal alignment | Ideal alignment | Ideal alignment | Ideal alignment |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | nrof antenna elements used | nrof antenna elements used | nrof antenna elements used | nrof antenna elements used |
| Additional notes, if any |  |  |  |  |

8.1.1.1.2	Positioning accuracy evaluation results

Table 8.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from [1]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR1], [R 16 DL-TDOA] | Convex UEs | 0.18778m | 0.25715m | 0.3927m | 0.56973m |
| [Case 2], [InF-DH], [FR1], [R 16 DL-TDOA] | Convex UEs | 0.17072m | 0.21744m | 0.2609m | 0.29708m |
| [Case 3], [InF-SH], [FR2], [R 16 DL-TDOA] | Convex UEs | 0.026979m | 0.044699m | 0.10266m | 0.4465m |
| [Case 4], [InF-DH], [FR2], [R 16 DL-TDOA] | Convex UEs | 0.044432m | 0.62978m | 0.94494m | 0.17867m |

8.1.1.1.3	Observations on Rel-16 NR positioning accuracy

Table 8.1.1.1.3-1: Rel.16 NR positioning – horizontal accuracy performance summary [1]

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR1], [R 16 DL-TDOA] | Yes | No 0.36973m | No 0.06973m |
| [Case 2], [InF-DH], [FR1], [R 16 DL-TDOA] | Yes | No 0.09708m | Yes |
| [Case 3], [InF-SH], [FR2], [R 16 DL-TDOA] | Yes | No 0.2465m | Yes |
| [Case 4], [InF-DH], [FR2], [R 16 DL-TDOA] | Yes | Yes | Yes |