3GPP TSG RAN WG1 #103-e	R1-2008180

# e-Meeting, Oct. 27th – Nov. 12th, 2020

Agenda Item:		8.8.1.2

Source:				Samsung

Title:					Baseline coverage performance using LLS for FR2

Document for:	Discussion

Introduction

This contribution provides baseline coverage performance for FR2 based on the simulation assumptions agreed in RAN1#102-e meeting and follow-up RAN1 email discussion. Note the link budget excel sheet is attached in a separate file. Simulation results for FR1 are given in [1].

Simulation assumptions

In RAN1#102-e meeting (and post e-mail discussion), simulation parameters were agreed for several scenarios, services and channels. In this contribution, we give the simulation results for various scenarios, such as indoor, urban, and suburban, for uplink and downlink data and control channels, such as PUSCH, PUCCH, PDSCH, and PDCCH. In detail, the values of the simulation parameters used in the evaluation for FR2 are summarized in the Appendix.

Baseline coverage performance for FR2

Link budget results are obtained based on the agreed link budget template [2]. In Table 1 below, required SNR, Maximum Coupling Loss (MCL), Maximum Isotropic Loss (MIL) and Maximum Path Loss (MPL) are summarized for each scenario and channel. The underlying assumption is that all of antenna gain correction factors (Δ1, Δ2, Δ3) are set to zero. Bottleneck channels are determined (marked in yellow) based on relative difference between channels. Overall, we observe that PUSCH and PUCCH need improvement in most evaluated cases.

The evaluated scenarios include indoor 28 GHz TDD, urban 28 GHz TDD and suburban 28 GHz TDD. For urban scenarios, we consider O2I with UE velocity 3kmph and O2O with UE velocity 30kmph. We also consider O2I with UE velocity 3kmph, O2O with UE velocity 30kmph and O2O with UE velocity 120kmph for suburban scenario.

Table 1. Baseline coverage performance

| Indoor 28 GHz TDD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel | The required SNR | MCL | MIL | MPL | Key assumptions |
| PUSCH for eMBB | 2.4 | 95.78 | 122.84 | 121.93 | 30 PRBs/ O2I with UE velocity: 3kmph |
| PUSCH for VoIP | -6.8 | 113.73 | 140.79 | 139.88 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type A/ O2I with UE velocity: 3kmph |
|  | -7.55 | 114.48 | 141.54 | 140.63 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type B/ O2I with UE velocity: 3kmph |
| PUCCH | -7.5 | 116.87 | 143.94 | 141.14 | UCI: 2 bits/ Format 1/ O2I with UE velocity: 3kmph |
|  | -2.55 | 111.92 | 138.99 | 136.19 | UCI: 11 bits/ Format 3/ O2I with UE velocity: 3kmph |
|  | -0.315 | 109.69 | 136.75 | 133.95 | UCI: 22 bits/ Format 3/ O2I with UE velocity: 3kmph |
| PDCCH | -7.73 | 113.72 | 143.79 | 140.99 | 48 PRBs/ O2I with UE velocity: 3kmph |
| PDSCH for eMBB | 2.1 | 104.39 | 134.46 | 133.55 | 50 PRBs/ O2I with UE velocity: 3kmph |
| Urban 28 GHz TDD |  |  |  |  |  |
| Channel | The required SNR | MCL | MIL | MPL | Key assumptions |
| PUSCH for eMBB | 3.2 | 94.98 | 125.05 | 90.28 | 30 PRBs/ O2I with UE velocity: 3kmph |
|  | 3.2 | 94.98 | 125.05 | 111.20 | 30 PRBs/ O2O with UE velocity: 30kmph |
| PUSCH for VoIP | -6.6 | 113.53 | 143.60 | 108.83 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type A/ O2I with UE velocity: 3kmph |
|  | -8.4 | 115.33 | 145.40 | 131.55 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type A/ O2O with UE velocity: 30kmph |
|  | -7.4 | 114.33 | 144.40 | 109.63 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type B/ O2I with UE velocity: 3kmph |
|  | -8.75 | 115.68 | 145.75 | 131.90 | The max # of HARQ tx: 32/ Latency: 51 ms/ Repetition type B/ O2O with UE velocity: 30kmph |
| PUCCH | -7.4 | 114.33 | 144.40 | 109.63 | UCI: 2 bits/ Format 1/ O2I with UE velocity: 3kmph |
|  | -7 | 116.37 | 146.45 | 129.38 | UCI: 2 bits / Format 1/ O2O with UE velocity: 30kmph |
|  | -0.35 | 109.72 | 139.80 | 101.95 | UCI: 11 bits/ Format 3/ O2I with UE velocity: 3kmph |
|  | -0.315 | 109.69 | 139.76 | 101.91 | UCI: 22 bits/ Format 3/ O2I with UE velocity: 3kmph |
|  | -2.2 | 111.57 | 141.65 | 124.58 | UCI: 11 bits/ Format 3/ O2O with UE velocity: 30kmph |
|  | -0.39 | 109.76 | 139.84 | 122.77 | UCI: 22 bits/ Format 3/ O2O with UE velocity: 30kmph |
| PDCCH | -7.88 | 130.87 | 163.95 | 126.10 | 48 PRBs / O2I with UE velocity: 3kmph |
|  | -7.88 | 130.87 | 163.95 | 146.88 | 48 PRBs/ O2O with UE velocity: 30kmph |
| PDSCH for eMBB | 3.05 | 120.44 | 153.52 | 118.75 | 50 PRBs / O2I with UE velocity: 3kmph |
|  | 3.05 | 120.44 | 153.52 | 139.67 | 50 PRBs/ O2O with UE velocity: 30kmph |
| Suburban 28 GHz TDD |  |  |  |  |  |
| Channel | The required SNR | MCL | MIL | MPL | Key assumptions |
| PUSCH for eMBB | 0.1 | 112.85 | 142.92 | 108.15 | 1 PRBs / O2I with UE velocity: 3kmph |
|  | -0.25 | 113.20 | 143.27 | 129.42 | 1 PRBs / O2O with UE velocity: 30kmph |
|  | -1.2 | 125.16 | 155.23 | 141.38 | 1 PRBs / O2O with UE velocity: 120kmph |
| PUSCH for VoIP | -6.6 | 113.53 | 143.60 | 108.83 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type A/ O2I with UE velocity: 3kmph |
|  | -8.40 | 115.33 | 145.40 | 131.55 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type A/ O2O with UE velocity: 30kmph |
|  | -6.76 | 113.69 | 143.76 | 129.91 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type A/ O2O with UE velocity: 120kmph |
|  | -7.4 | 114.33 | 144.40 | 109.63 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type B/ O2I with UE velocity: 3kmph |
|  | -8.75 | 115.68 | 145.75 | 131.90 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type B/ O2O with UE velocity: 30kmph |
|  | -6.8 | 113.73 | 143.80 | 129.95 | The max # of HARQ tx: 32 / Latency: 51 ms/ Repetition type B/ O2O with UE velocity: 120kmph |
| PUCCH | -7.04 | 116.41 | 146.48 | 108.63 | UCI: 2 bits / Format1/ O2I with UE velocity: 3kmph |
|  | -7.09 | 116.46 | 146.54 | 129.47 | UCI: 2 bits/ Format1/ O2O with UE velocity: 30kmph |
|  | -6.98 | 116.35 | 146.42 | 129.35 | UCI: 2 bits/ Format1/ O2O with UE velocity: 120kmph |
|  | -0.35 | 109.72 | 139.80 | 101.95 | UCI: 11 bits / Format3/ O2I with UE velocity: 3kmph |
|  | -2.26 | 111.63 | 141.71 | 124.64 | UCI: 11 bits/ Format3/ O2O with UE velocity: 30kmph |
|  | -2.7 | 112.07 | 142.15 | 125.08 | UCI: 11 bits/ Format3/ O2O with UE velocity: 120kmph |
|  | -0.32 | 109.69 | 139.76 | 101.91 | UCI: 22 bits / Format3/ O2I with UE velocity: 3kmph |
|  | -0.39 | 109.76 | 139.84 | 122.77 | UCI: 22 bits / Format3/ O2O with UE velocity: 30kmph |
|  | -0.518 | 109.89 | 139.96 | 122.89 | UCI: 22 bits / Format3/ O2O with UE velocity: 120kmph |
| PDCCH | -7.88 | 130.87 | 163.95 | 126.10 | 48 PRBs/ O2I with UE velocity: 3kmph |
|  | -7.88 | 130.87 | 163.95 | 146.88 | 48 PRBs / O2O with UE velocity: 30kmph |
|  | -7.93 | 130.92 | 164 | 146.93 | 48 PRBs / O2O with UE velocity: 120kmph |
| PDSCH for eMBB | 4.5 | 118.99 | 152.07 | 117.30 | 50 PRBs/ O2I with UE velocity: 3kmph |
|  | 4.9 | 118.59 | 151.67 | 137.82 | 50 PRBs / O2O with UE velocity: 30kmph |
|  | 4.15 | 119.34 | 152.42 | 138.57 | 50 PRBs / O2O with UE velocity: 120kmph |

For each scenario, the bottleneck channels are summarized as following:

- Indoor (28 GHz, TDD): PUSCH for eMBB

- Urban (28 GHz, O2I with UE velocity: 3kmph, TDD): PUSCH for eMBB

- Urban (28 GHz, O2O with UE velocity: 30kmph, TDD): PUSCH for eMBB, PUCCH with 11 UCI bits and 22 UCI bits

- Suburban (28 GHz, O2I with UE velocity: 3kmph, TDD): PUCCH with 11 UCI bits and 22 UCI bits

- Suburban (28 GHz, O2O with UE velocity: 30kmph, TDD): PUCCH with 22 UCI bits

- Suburban (28 GHz, O2O with UE velocity: 120kmph, TDD): PUCCH with 22 UCI bits

Based on the results, it is observed that coverage enhancements for PUSCH and PUCCH are needed for FR2.

Observation 1. In terms of coverage enhancement, PUSCH and PUCCH are bottleneck compared to other channels for FR2.

Proposal 1. Support coverage enhancements for PUSCH and PUCCH for FR2.

Conclusion

This contribution presents FR2 simulation results to identify baseline coverage performance of several UL and DL channels in FR2. The following observations and proposals are made:

Observation 1. In terms of coverage enhancement, PUSCH and PUCCH are bottleneck compared to other channels for FR2.

Proposal 1. Support coverage enhancements for PUSCH and PUCCH for FR2.

- References

- R1-2008179, Baseline coverage performance using LLS for FR1, Samsung

- R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of Phase 1 - link budget template, Moderator (SoftBank)

- Appendix

Table A-1. General simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Subcarrier Space | 120kHz |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, 120km/h for outdoor. |
| Occupied channel bandwidth for | 100MHz |
| Frequency hopping for PUSCH | w/ or w/o frequency hopping |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| Channel model for link-level simulation | TDL-A |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |

Table A-2. Channel specific parameters for PUSCH and PDSCH

| Parameters | Values |
| --- | --- |
| BLER | For eMBB, w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| DMRS configuration | For 30km/h and 120km/h: Type I, 2 DMRS symbol, no multiplexing with data. For 3km/h: Type I, 1 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH |
| Repetitions for PUSCH/PDSCH | For eMBB, w/o repetition as baseline For VoIP, w/ repetition and # of actual repetitions = 2 (2/14 symbols in DDDSU with repetition type B) |
| HARQ configuration for PUSCH/PDSCH | For eMBB, no HARQ transmission. For VoIP, w/ HARQ and the maximum number of HARQ transmission = 32 due to latency requirements. |
| PUSCH/PDSCH duration | 14 OS for PUSCH, 12 OS for PDSCH |
| PRBs/TBS/MCS for eMBB for PUSCH | 30 PRBs for 5Mbps and 1 PRBs for 50kbps QPSK modulation |
| PRBs/MCS for VoIP for PUSCH | 4 PRBs for VoIP QPSK modulation |
| PRBs/MCS for eMBB for PDSCH | 50 PRBs for 25Mbps and 2 PRBs for 1Mbps QPSK modulation |
| Frequency hopping | w/o frequency hopping for PUSCH |

Table A-3. Channel specific parameters for PUCCH

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, 11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: ACK missed detection probability: 1%. For PUCCH format 3: BLER: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/o repetition for PUCCH. |
| PUCCH duration | 14 OFDM symbols |
| DMRS configuration for PUCCH | Additional 3 DMRS symbols for PUCCH Format 3. |
| Frequency hopping | w/ frequency hopping for PUCCH |

Table A-4. Channel specific parameters for PDCCH

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48PRBs |
| Tx Diversity | Precoder cycling |
| BLER for PDCCH | 1% BLER. |