3GPP TSG RAN WG1 #103-e		R1-2008225

e-Meeting, October 26th – November 13th

Source:	OPPO

Title:	Evaluation of NR positioning in IIOT scenario

Agenda Item:	8.5.2

Document for:	Discussion and Decision

- Introduction

New SI on NR positioning enhancements was approved in RAN#86[1]

This study item includes the following objectives:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases

- Note: Modifications to parameters in the InF-DH models will be discussed separately.

- Note: Target performance and performance gap identification will be discussed separately.

- Note: Individual companies may consider additional InF models in TR 38.901 as complementary evaluation scenarios in their simulation investigation and the evaluation results can be considered to be captured in the TR 38.857.

- Note: Target positioning requirements may not necessarily be reached for all scenarios.

Agreement:

Clutter parameters {density , height ,size } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m } for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Agreement:

CDFs of positioning errors are used as performance metrics in NR positioning evaluation with at least the following percentiles 50%, 67%, 80%, 90%.

- Note: In addition to overall positioning accuracy performance, companies are encouraged to report the estimation accuracy of UE/gNB measurements (e.g., RSTD) for performance comparison.

Agreement:

Absolute-time-of arrival model defined in TR 38.901 without modification is considered in the evaluation of all scenarios.

Agreement:

Blockage model is not considered in the evaluation of all scenarios

Agreement: (Proposal 5.1-4, Revision 3, in Section 5.1 of R1-2004961)

- Adopt the parameters defined in the Table below as the baseline parameters for all InF scenarios in the evaluation of positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

Agreement:

Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

In this contribution, we discuss evaluation assumptions and present evaluation results for InF scenarios.

- Evaluation of Positioning Accuracy

Positioning Accuracy in IIOT scenario

To evaluate NR positioning for IIOT use cases, we conducted evaluation in the scenarios of InF-SH and InF-DH.  For detailed simulation parameters, please see the appendix, which follows the assumption agreed in last meeting. DL-TDOA method is evaluated in those scenarios. In the evaluation, positioning method is based on Chan algorithm with equally weighted TOA covariance. We used maximum-likelihood detection to obtain 1/4Ts resolution and good quality of TOA measurement.

Absolute time of arrival defined in TR38.901 is modelled in our evaluation as agreed. The TOA estimated from a NLOS path could cause severe error to positioning estimate. NLOS probability is higher in DH scenario than SH, and in last meeting it was agreed to modify clutter parameter for more number of LOS channel.  It is observed that, for the scenario of InF-DH, parameter {density, height, size}= {40%, 2m, 2m } and  {40%, 3m, 5m } have similar performance, and both of new parameter combination can greatly improve the performance over the conventional parameter {60%, 6m, 2m } specified for InF-DH. Another parameter affecting the LOS probability is the ISD (value of D). In scenario of InF-DH with D = 50 meters, a portion of UEs have less than 3 sites with LOS channel, and thus the positioning accuracy is poor in this case.  The positioning performance in InF-SH with D = 20m and D = 50m is shown in Figure 1.

Figure 1 positioning errors in InF-SH scenarios

The positioning performance in InF-DH with D = 20m and D = 50m is shown in Figure 2.

Figure 2 positioning errors in InF-DH scenarios

Accordingly, the positioning error at 50%, 67%, 80% and 90% UEs in InF-DH and InF-SH scenarios with D = 20m or 50m are summarize in table 1:

Table 1 summary of positioning error for InF scenarios

| Positioning error | InF-DH with D = 50m | InF-DH with D=20m | InF-SH with D = 50m | InF-SH with D= 20m |
| --- | --- | --- | --- | --- |
| 50% | 3.64 | 0.21 | 0.16 | 0.14 |
| 67% | 5.78 | 0.29 | 0.20 | 0.18 |
| 80% | 8.87 | 0.75 | 0.24 | 0.22 |
| 90% | 13.19 | 2.47 | 0.31 | 0.27 |

From the evaluation results, we can observe:

- In the InF-SH scenario, < 1meter positioning accuracy for 90% of UEs is achievable. Specifically, InF-SH with D = 20m has better performance than InF-SH with D = 50m.

- In the InF-DH scenario, D=20m have 2~3m error for 90% of UEs. With D=50m, positioning error is 13.19m for 90% of UEs and the large ambiguity is caused by NLOS channel.

Thus, we can make the following observation:

Observation1: The performance of DL-TDOA in InF scenarios are:

- In InF-SH scenario, < 1m accuracy for 90% of UEs is achievable.

- In InF-DH scenarios, < 1m accuracy for 90% of UEs is not achievable.

  - D = 20m can achieve 2.47 m accuracy for 90% of UEs.

  - D = 50m can achieve 13.19m accuracy for 90% of UEs.

Effect of multipath mitigation

One issue discussed in last meeting is whether to support multipath mitigation. We evaluated the following two methods to mitigate the NLOS paths:

- The method of LOS classification: LOS/NLOS channel are estimated, the decision of LOS/NLOS is the estimated K factor larger than 0 dB in simulation;

- The method of NLOS mitigation: RSTD are pruned by implementation, the quality of RSTD are verified by triangle inequality in simulation;

For comparison, we also evaluate the performance of the following two cases

- ALL TRP: RSTD of all the TRPs are used for positioning calculation, which can be considered as the worst case for NLOS;

- LOS classification (perfect) : here we assume we know the LOS/NLOS perfectly for positioning;

Figure 3 positioning errors in InF-DH scenarios

Figure 4 positioning errors in InF-SH scenarios

Perfect LOS classification is considered as performance upper bound. The estimation error of LOS/NLOS classification decrease performance severely. By gNB/UE implementation, NLOS channel can be detected from  RSTD, which may achieve better performance than LOS classification only. When NLOS mitigation and LOS classification are considered together, performance is close to the perfectly case. For InF-SH, the LOS classification does not improve performance obviously when NLOS mitigation is implemented. When advance NLOS mitigation is implemented, the gain of explicitly LOS classification may be marginal.

Observation2: Regarding the NOLS mitigation, we can observe:

- Implementing NLOS mitigation can improve positioning accuracy,

- Implementing LOS classification method can improve the positioning accuracy, but the errors in LOS classification may decrease performance.

- In InF-SH scenario, gain from the method of LOS classification is marginal.

Evaluation of physical layer latency

UE-assisted DL-based method

We have agreed that to evaluate the physical layer latency of UE-assisted DL-based method, the start time is the transmission of the PDSCH from the gNB carrying the LPP Request Location Information message and the end time is the successful decoding of the PUSCH carrying the LPP Provide Location Information message. Accordingly, the physical layer procedure for UE-assisted DL-based method is illustrated in Figure 5.

Figure 5 procedure of UE-assisted DL-based method

As shown In Figure 5, the steps included in physical layer procedure of UE-assisted DL-based method includes:

- The system sends LRR location request information, that is the trigger time.

- After the UE receives the LRR location request information, the UE sends RRC location indication message to request configuration of measurement gap.

- To respond the UE, the gNB configures measurement gap for the UE to measure DL PRS resources.

- Based on the configuration, the UE receives the DL PRS resource during measurement gap and then conduct the required measurement, such as RSTD or RSRP.

- After the positioning measurement is ready, the UE sends SR to request uplink grant, the gNB schedule PUSCH grant and then the UE reports the positioning measurement through PUSCH grant.

- Finally, the gNB decode the PUSCH carrying the positioning measurement.

Based on the procedure described above, the Physical layer latency for UE-assisted DL-based method is analyzed as follows:

Table 2 Physical layer latency for UE-assisted DL-based method

| Component | Description | Value range |
| --- | --- | --- |
| 0 | Start trigger: gNB sending PDSCH carrying LPP location request message |  |
| 1 | UE process LPP location request information message | 10ms |
| 2 | UE requests MG and gNB configures MG | 10ms+10ms: RRC reconfiguration delay |
| 3 | DL PRS reception and measurement | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or10240 ms MG periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability: Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms |
| 4 | UE sends PUSCH carrying location measurement report and gNB decode PUSCH | 12.5ms for 15KHz 6.25ms for 30KHz 3.125ms for 60KHz 1.56 ms for 120KHz UE sends PUCCH SR to request UL grant UE receives DCI scheduling PUSCH UE sends PUSCH carrying measurement report PUSCH decodes PUSCH |
|  | Total latency: | For FR1 (60kHz): 54.125ms For FR2 (120KHz): 52.56ms |

UE-assisted UL-based method

For UE-assisted UL-based method, we have agreed that for evaluation of Physical layer latency, the start time is the reception by the gNB of the NRPPa measurement request message and the end time is he transmission by the gNB of the NRPPa measurement response message. Given that, the procedure of physical layer procedure for UE-assisted UL-based method is shown in Figure 6.

Figure 6 procedure for UE-assisted UL-based method

As shown in Figure 6, the procedure of UE-assisted UL-based method includes the following steps:

- The gNB receives NPPPa measurement request from the NW, that is treated as the start time of this procedure

- The gNB sends the configuration of SRS for positioning.

- If the SRS is semi-persistent or aperiodic, the gNB sends MAC CE or DCI to activate or trigger the transmission of SRS for positioning. This step is optional for latency analysis.

- Then the UE transmits the SRS for positioning according to the configuration

- The gNB receives the SRS for positioning and conduct corresponding measurement.

- Finally, the gNB reports the uplink measurement results to the server.

Based on that procedure, the physical layer latency for UE-assisted UL-based method is analyzed:

Table 2 Physical layer latency for UE-assisted UL-based method

| Component | Description | Value range |
| --- | --- | --- |
| 0 | Start trigger: gNB receives NPPPa measurement request |  |
| 1 | gNB process the NPPPa measurement request | 10ms: |
| 2 | gNB configures SRS | 10ms: RRC reconfiguration delay |
| 3 | UE transmits SRS for positioning | Assume periodic SRS resource. The periodicity of SRS can be 1, 2, 4, 5, 8, 10, 16, 20 .. slots. So the minimum latency is: - 1ms for 15KHz - 0.25ms for 60KHz. - 0.125ms for 120KHz |
| 4 | gNB measures SRS for positioning and reports the NPPPa measurement response | Assume that 3ms is needed by gNB to measure SRS and process the NPPPa measurement response |
|  | Total latency: | For FR1 (60kHz): 23.25ms For FR2 (120KHz): 23.125ms |

UE-based Methods

For UE-based method, we have two options for start time and end time. In the first option is for the use case where NW sends LPP request location information. Thus, the start time in the first option is when the gNB sends PDSCH carrying the LPP request location information and the end time is the successful decoding PUSCH carrying the LPP measurement report. The physical layer latency of the first option would be same to that of the UE-assisted DL-based method.

The second option is for the use cases where the NW does not send LPP request location information and the UE calculates the location by itself. For the second option, we had three alternatives for the start time. As proposed in our companion paper, Alt1 is preferred. Therefore, to evaluate the physical layer latency for the second option, the transmission of measurement gap request is assumed to be start time.

Figure 7 Physical layer procedure of UE-based method

A physical layer procedure of UE-based method is shown in Figure 7, which includes the following steps:

- The UE sends LPP request assistance data message to request the configuration of DL PRS from the NW. And the NW configures the DL PRS resource by sending the LPP provide assistance data. In our view, we do not need count this step in the physical layer latency calculation because the configuration of DL PRS is semi-static and the UE only requests it once.

- For receiving and measuring DL PRS, the UE needs measurement gap. The UE sends RRC location measurement location to the gNB to request configuration of measurement gap.

- The gNB responds to the UE and sends the RRC configuration of measurement gap.

- Within the measurement gap, the UE receives the DL PRS and then conducts corresponding measurement.

- Finally, the UE calculates the location.

The physical latency of UE-based method is analyzed as follows:

Table 4 Physical layer latency of UE-based method

| Component | Description | Value range |
| --- | --- | --- |
| 1 | Start trigger: UE sends RRC Location measurement indication | 10ms RRC processing delay |
| 2 | gNB sends RRC measurement gap configuration | 10ms: RRC reconfiguration delay |
| 3 | DL PRS reception and measurement | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or 10240ms. However, the measurement gap periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability. Assume extra 1ms is needed for the UE to finish measurement |
| 4 | UE completes location calculation | 3 ms |
| Total latency: |  | 44ms |

Based on the discussion above, we made observation:

Observation3: The physical layer latency are:

- For UE-assisted DL-based method, the total latency is 54.125ms for FR1 (60KHz), 52.56ms for FR2 (120KHz);

- For UE-assisted UL-based method, the total latency is 23.25ms for FR1 (60KHz SCS), 23.125ms for FR2 (120KHz);

- For UE-based method, the total latency is 44ms

- Conclusion

In this contribution, we presented our simulation result on NR positioning. Based on the evaluation, the following observation are provided:

Observation1: The performance of DL-TDOA in InF scenarios are:

- In InF-SH scenario, < 1m accuracy for 90% of UEs is achievable.

- In InF-DH scenarios, < 1m accuracy for 90% of UEs is not achievable.

  - D = 20m can achieve 2.47 m accuracy for 90% of UEs.

  - D = 50m can achieve 13.19m accuracy for 90% of UEs.

Observation2: Regarding the NOLS mitigation, we can observe:

- Implementing NLOS mitigation can improve positioning accuracy,

- Implementing LOS classification method can improve the positioning accuracy, but the errors in LOS classification may decrease performance.

- In InF-SH scenario, gain from the method of LOS classification is marginal.

Observation3: The physical layer latency are:

- For UE-assisted DL-based method, the total latency is 54.125ms for FR1 (60KHz), 52.56ms for FR2 (120KHz);

- For UE-assisted UL-based method, the total latency is 23.25ms for FR1 (60KHz SCS), 23.125ms for FR2 (120KHz);

- For UE-based method, the total latency is 44ms

- Reference

RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

Appendix: Evaluation Assumptions

Table: Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values | Comments (to each of the parameter) |
| --- | --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |  |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |  |
|  | Room height | 10m |  |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |  |
| Peneteration loss |  | 0dB |  |  |  |
| Number of floors |  | 1 |  |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: See Proposal 5.1-7 |  |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |  |

| Parameter | [Source 1, scenario, FRx] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb4 |
| Reference signal (type of sequence, number of ports, …) | Downlink PRS, one port |
| Number of sites | 18 sites |
| Number of symbols used per slot per positioning estimate | 4 symbols |
| Number of slots per positioning estimate | single |
| Power-boosting level | none |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | ML |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Chan |
| Network synchronization assumptions | Perfect synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beam operation |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No precoding |
| Additional notes, if any |  |