3GPP TSG RAN WG1 Meeting #103-e	R1-2008391

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Remaining issues on physical layer procedures for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views about remaining issues on physical layer procedures for NR sidelink.

Discussion

Simultaneous PSFCH transmission/reception

The following changes were introduced in RAN1#100bis-e,

#### 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

The corresponding RAN1 agreements are as follows,

Agreements: For the prioritization between PSFCH TX and PSFCH RX, - When the UE is required to transmit more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH transmission. - When the UE is required to receive more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH reception.

As can be seen above, the intention of the RAN1 agreements for the yellow highlighted text was that the decision on whether to transmit one set of PSFCHs or receive another set of PSFCHs is based on which set contains a PSFCH with highest priority. However this was unfortunately not correctly reflected in the spec text, which says the selected “set of PSFCHs” should correspond to the smallest priority field value, e.g. if each PSFCH in a set “A”={PSFCH1, PSFCH2, PSFCH3, PSFCH4} for transmission has respective priority field values {2, 2, 3, 5} and each PSFCH in a set “B”={PSFCH5, PSFCH6, PSFCH7, PSFCH8} for reception has respective priority field values {3, 4, 5, 6}, then the spec text says the UE should transmit a subset of “A” = {PSFCH1, PSFCH2}, as they correspond to the smallest priority field value i.e. 2. However, the intention of the RAN1 agreements was actually that since set “A” for transmission contains a PSFCH with a smallest priority field value, the UE shall transmit (PSFCHs in set “A”) rather than receive (PSFCHs in set “B”).

Proposal 1: For simultaneous PSFCH transmission/reception, clarify that it is “a PSFCH” that corresponds to the smallest priority field value, rather than “a set of PSFCHs” that correspond to the smallest priority field value.

The following TP is proposed to clarify the intended UE behaviour in the RAN1 agreements:

#### 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs containing a PSFCH that corresponds to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

Conclusion

In this contribution, we discuss remaining issues on physical layer procedures for NR sidelink, and provide corresponding proposal/TP.

Proposal 1: For simultaneous PSFCH transmission/reception, clarify that it is “a PSFCH” that corresponds to the smallest priority field value, rather than “a set of PSFCHs” that correspond to the smallest priority field value.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.