| 3GPP TSG-RAN WG1 Meeting #103e e-Meeting, October 26th – November 13th, 2020 | R1-2008624 |
| --- | --- |

Agenda item:	8.8.1.1

Source: 	Qualcomm Incorporated

Title: 	Baseline FR1 coverage performance

Document for:	Discussion/Decision

# Introduction

In this contribution, we first discuss remaining aspects of link-level simulation assumption for baseline FR1 coverage evaluation. Based on the proposed assumption, we present our results on coverage of 5G NR using link level simulations, followed by a detailed link budget analysis. A thorough link budget analysis is presented for two distinct scenarios -- a dense urban deployment and a rural deployment. The basic parameters assumed in this study are in line with the agreements made in the previous RAN1 meeting. FR1 link budget analysis presented in R1-102e is available in [1]. Our FR2 link budget analysis is presented in [2].

# Remaining Aspects on Baseline Coverage Assumption

In this section we address some of our assumptions on certain key aspects on coverage analysis that were identified in the previous RAN1 meeting. Three key aspects include (a) assumptions on gNB modelling and antenna gain, (b) link budget template and (c) methodology to identify performance bottlenecks. We address each of the above in the discussion below.

## gNB Modelling and antenna gain assumptions

We made the following two agreements in R1-101e:

____________________________________

Agreement:

Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting.

- Option 1: Antenna array gain is included in the link budget template.

- FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs)

- FFS: For TDL channel model

- FFS: Values reflective of realistic implementation and network operation.

- Option 2: Antenna array gain is included in LLS.

- FFS: For CDL channel model

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of receive TxRUs for BS | TBD gNB architectures to study for TDL: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz [gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS (as starting point). FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation.] [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain, obtained from mapping antenna elements to TXRU, is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

____________________________________

We made the following agreements in R1-102e:

We believe it is important that the LLS capture accurately the number of gNB TXRUs. Simulating gNBs with only 1, 2 or 4 TXRUs is an over-simplification and may not accurately reflect the performance of a real gNB with a lot more TXRUs. We also think that antenna gain components 3 and 4 take a fixed value across all antenna uplink and downlink PHY channels and therefore do not play a significant role in determine the coverage bottleneck when making relative comparisons. We therefore make the following remarks:

Remark 1: The number of gNB receive chains is equal to the number of TXRUs in LLS. Antenna array gains due to digital beamforming or combining are included in LLS.

Remark 2: Analog antenna gains are computed assuming UE is in boresight. The same analog antenna gains are applies across all uplink and downlink channels. No correction factors are applied to analog antenna gains.

## Link budget template

With regard to which the link budget template should be used for coverage analysis, we made the following agreement in R1-101e:

---------------------------

Agreement:

- Down selection on the following options for the link budget template for FR1 in next meeting.

- Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters.

  - FFS: The template provided by FL in Tdoc R1-2005005.

- Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824.

- Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters.

---------------------------

In R1-102e, we further agreed to adopt a single link budget template:

We use the link budget table agreed over email discussions for our link budget analysis. We use this template to compute MCL, and MIL. For completeness, we also present MPL calculations using the IMT-2020 parameters as a starting point. In particular, interference margins, shadow fading margins and penetration margins are as per IMT-2020 assumptions.

## Methodology to identify performance bottleneck

In R1-101e, we made the following agreement on the overall evaluation methodology to be adopted for this study.

Agreements:

- The basic evaluation methodology is based on link-level simulation for FR1.

- Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Step 2: Obtain the baseline performance based on required SINR and link budget template.

- Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately

- The evaluation methodology based on system-level simulation is optional for FR1.

- Note: The simulation assumptions for SLS are up to companies’ reports.

In R1-102e, we made the following agreements on the overall evaluation methodology to be adopted for this study:

With these agreements in mind, we propose the following guidelines for identifying the coverage bottleneck. Coverage analysis has two key components, the first focused on coverage of control channels that are devoid of any minimum service requirements and the second component focused on data channels and minimum service requirements.

Figure 1 Methodology to identify coverage bottlenecks

Control channels in uplink and downlink are fundamental to a link --- with control coverage a link failure is declared. Typically, a higher level of reliability is required of control channels. Interference margins, shadow fading margins, and BLER targets are different and more stringent for control channels compared to data channels. For these reasons, we make the following proposal:

Proposal 1: Use relative MCL, MIL, and/or MPL as a measure of coverage in different scenarios. Control coverage gaps are determined by the difference in MCL, MIL, or MPL of control channels in uplink and downlink. RAN1 strives to pursue enhancements that close the control coverage gaps in various deployment scenarios.

The second component focuses on the data channels and the discussion here depends on the various levels of service requirement. Data rates specified for minimum service requirements are used to determine data channel coverage. This can then be compared against control channel coverage to determine whether any enhancements for data channels are necessary. Using this approach, we make the following proposal:

Proposal 2: Data channel coverage is determined based on the service data rate requirements. When data channel coverage is smaller than control channel coverage, RAN1 strives to pursue enhancements that improve the data channel coverage.

# Parameters for Link Budget Analysis

In R1-101e and R102e, we made several agreements on the general parameters and channel-specific parameters for link-level simulations. All results presented here are in line with these agreements. We list some of our assumptions for clarity.

Table 1 Common link-level parameters

| Parameter | Rural | Urban |
| --- | --- | --- |
| Carrier Frequency | 700 MHz | 4GHz |
| Multiplexing | FDD | TDD (DDDSU) |
| System BW | 20MHz | 100MHz |
| gNB Tx power | 49 dBm for 20 MHz bandwidth | 53 dBm and 44 dBm for 100 MHz |
| UE power class | 23 dBm | 23 dBm |
| Numerology | 15 kHz SCS | 30 kHz SCS |
| BS Noise Figure | 5 dB | 5 dB |
| UE Noise Figure | 7 dB | 7 dB |
| BS antenna gain (dB) | 8 dB | 8 dB |
| UE antenna gain (dB) | 0 dB | 0 dB |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Channel | TDL-C 300ns | TDL-C 300ns |
| UE speed | 3km/h & 120 km/h | 3km/h |
| # of gNB TxRUs | 4 | 64 |
| # of Tx at UE | 1 | 1/2 |
| # of Rx at UE | 2 | 4 |
| # of cells | 1 | 1 |
| TDD pattern | N/A | DDDSU |
| UL data rate target | 100kbps | 1Mbps |
| DL data rate target | 1Mbps | 10Mbps |

Table 2 PUCCH Parameters for link-level simulations and link budget analysis

| PUCCH format | PUCCH format 1 | PUCCH format 3 |
| --- | --- | --- |
| Waveform | Low PAPR sequence | DFT-s-OFDM |
| UCI payload (bit) | 2 bits | 11, 22 bits |
| PUCCH duration (symbols) | 14 | 14 |
| # RBs for long PUCCH | 1 RB | 1 RB |
| Repetition | disabled | disabled |
| Frequency hopping | Enabled | Enabled |
| Performance | 1% false alarm and 1% mis-detection |  |

Table 3 PUSCH (EMBB, VoNR)  Parameters for link-level simulations and link budget analysis

| PUSCH Parameter | Value |
| --- | --- |
| UE antenna configuration | 1 Tx |
| UE power class | Power class 3 |
| Slot structure | 12 data symbols and 2 DMRS symbols |
| Waveform | DFT-S-OFDM |
| PUSCH rank | 1 |
| PUSCH RBs | Variable 1-32 RBs |
| PUSCH DMRS | Type 1 with no data on DMRS symbols |
| HARQ | Enabled |
| Performance metric | 10% iBLER after 1st tx for eMBB and VoNR |

Table 4 VoNR parameters

| TDD Config (DDDSU) |  |
| --- | --- |
| Payload segments per 20 ms | 1 |
| Payload/TB size (no segmentation) | 320 bits (256 bits from voice codec; 3 byte RoHC; 2 byte PDCP hdr; 1 byte RLC hdr ; 2 byte MAC hdr) |
| RB, MCS | 4 RB, MCS4 (64QAM table) |
| HARQ | enabled; max retransmissions = 7 |
| FDD config |  |
| Payload segments per 20ms | 4 |
| Payload/TB size(with 4 segments) | (296)/4 + 24 = 98 bits |
| RB, MCS | 4 RB, MCS 0 (64QAM table) |
| HARQ | enabled; max retransmissions = 4 |

Table 5 Parameters related to RACH procedure

| RACH |  |
| --- | --- |
| Format | Format 0 for rural, B4 for urban |
| Msg 2 PDSCH |  |
| Payload | 9 bytes |
| RB, MCS | MCS 0, TB scale factor 0.25, 12 PRBs |
| HARQ | N/A |
| Slot structure | 9 data symbols, 3 DMRS symbols |
| Precoding | Precoder cycling |
| Performance target | 10% iBLER |
| Msg 3 PUSCH |  |
| Payload | 56 bits |
| RB, MCS | 2 RBs, MCS 0 (64QAM table) |
| Slot structure | 11 data symbols, 3 DMRS symbols |
| HARQ | disabled |
| Performance target | 10% iBLER |
| Msg 4 PDSCH |  |
| Payload | 130 bytes |
| Slot structure | 10 data symbols, 2 DMRS symbols |
| RB, MCS | 36 RB, MCS 0 (64QAM table) |
| HARQ | disabled |
| Precoding | Precoder cycling |
| Performance target | 10% rBLER |

Table 6 Broadcast PDCCH Parameters

| Parameter | Value |
| --- | --- |
| BW | 48 RBs |
| # of symbols | 2 |
| PDCCH aggregation level | 16 |
| DCI size | 40 (+ 24 bits CRC) |
| REG bundle size | 6 |
| Beam forming | Precoder cycling |

Table 7 PDSCH Parameters

| Parameter | Value |
| --- | --- |
| Slot structure | 10 PDSCH data symbols, 2 DMRS symbols |
| Precoding | Closed-loop precoding |
| PDSCH rank | 1 |
| Frequency Allocation | Full band allocation |

# Baseline eMBB Coverage

In the following, with a focus on eMBB data traffic we present our results on the link budget analysis driven by the results obtained in our link level simulations.

## Rural Scenario

Result of the link budget analysis for downlink and uplink are presented in Table 8 and Table 9 for downlink and uplink channels in rural scenarios, respectively.

Table 8 Link budget results for in rural pedestrian scenarios

| Rural pedestrian (1Tx4Rx, TDL-C 300ns, 700MHz FDD, 3 kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 100kbps Unicast (wHARQ) | 56 bits Msg3 (noHARQ) | VoNR (wHARQ 2% rBLER) | Format 0 |
| MCL | 139.91 | 136.41 | 133.91 | 136.46 | 135.47 | 135.46 | 139.35 |
| MIL | 152.93 | 149.43 | 146.93 | 149.48 | 148.49 | 148.48 | 152.37 |
| MPL | 131.98 | 128.48 | 125.98 | 131.85 | 130.86 | 130.85 | 131.42 |

| Rural Pedestrian (700MHz FDD, 20MHz, 4T2R, TDL-C 300ns, 3 kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (no HARQ) | Unicast PDSCH 1Mbps (wHARQ) |  |  |
| MCL | 151.78 | 147.98 | 148.98 | 144.98 | 149.98 |  |  |
| MIL | 164.80 | 161.00 | 162.00 | 158.00 | 163.00 |  |  |
| MPL | 143.85 | 140.05 | 144.37 | 140.37 | 145.37 |  |  |

Table 9 Link budget results in rural vehicular scenarios

| Rural Vehicular (1Tx4Rx, TDL-C 300ns, 700MHz FDD, 120 kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 100kbps Unicast (wHARQ) | 56 bits Msg3 (noHARQ) | VoNR (wHARQ 2% rBLER) | Format 0 |
| MCL | 139.91 | 136.41 | 133.91 | 136.46 | 135.47 | 135.46 | 139.35 |
| MIL | 152.93 | 149.43 | 146.93 | 149.48 | 148.49 | 148.48 | 152.37 |
| MPL | 133.48 | 129.98 | 127.48 | 133.87 | 132.88 | 132.87 | 132.92 |

| Rural Vehicular (700MHz FDD, 20MHz, 4T2R, TDL-C 300ns, 120 kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (no HARQ) | Unicast PDSCH 1Mbps (wHARQ) |  |  |
| MCL | 151.78 | 147.98 | 148.98 | 144.98 | 149.98 |  |  |
| MIL | 164.80 | 161.00 | 162.00 | 158.00 | 163.00 |  |  |
| MPL | 145.35 | 141.55 | 146.39 | 142.39 | 147.39 |  |  |

Based on the above results, we make the following observations and proposals:

Observation 1: Large coverage asymmetry is observed between uplink and downlink control channel coverage in the rural scenarios.

Observation 2: PUSCH coverage is also a bottleneck in ensuring minimum service requirements for eMBB and VoNR.

Proposal 3: Consider enhancements to PUCCH performance to reduce the uplink-downlink control channel coverage imbalance in rural scenarios. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

Our proposals on improving the performance of PUCCH and PUSCH are provided in [3] and [4].

## Urban Scenario

Result of the link budget analysis for downlink and uplink are presented in Table 10 and Table 11 for the downlink and uplink channels in urban scenarios, respectively.

Table 10 Link budget results in urban scenarios with downlink power of 53 dBm

| Urban Pedestrian (TDD, 4GHz, DDDSU, 1Tx64Rx, TDL-C 300ns, 3kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 1Mbps Unicast (wHARQ) | 56 bits Msg3 (noHARQ) | VoNR (wHARQ 2% rBLER) | Format B4 |
| MCL | 146.90 | 144.40 | 142.90 | 127.92 | 142.96 | 141.45 | 144.76 |
| MIL | 159.67 | 157.17 | 155.67 | 140.69 | 155.73 | 154.22 | 157.53 |
| MPL | 125.86 | 123.36 | 121.86 | 109.96 | 125.00 | 123.49 | 123.72 |

| Urban (100MHz, TDD 4G, DDDSU, 64T4R, TDL-C 300ns, 11Hz Doppler) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (no HARQ) | Unicast PDSCH 10Mbps (wHARQ) |  |  |
| MCL | 151.39 | 147.49 | 148.99 | 144.49 | 145.99 |  |  |
| MIL | 163.16 | 159.26 | 160.76 | 156.26 | 157.76 |  |  |
| MPL | 129.35 | 125.45 | 130.03 | 125.53 | 127.03 |  |  |

Based on the above results, we make the following observations and proposals:

Observation 3: In urban scenarios with downlink power of 53 dBm, large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). PUCCH emerges as a bottleneck.

Observation 4: In urban scenarios with downlink power of 53 dBm, PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Table 11 Link budget results in urban scenarios with downlink power of 44 dBm

| Urban Pedestrian (TDD, 4GHz, DDDSU, 1Tx64Rx, TDL-C 300ns, 3kmph) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUCCH |  |  | PUSCH (14 OS) |  |  | PRACH |
|  | PUCCH PF1 2 bits | PUCCH PF3 11 bits | PUCCH PF3 22 bits | 1Mbps Unicast (wHARQ) | 56 bits Msg3 (noHARQ) | VoNR (wHARQ 2% rBLER) | Format B4 |
| MCL | 146.90 | 144.40 | 142.90 | 127.92 | 142.96 | 141.45 | 144.76 |
| MIL | 159.67 | 157.17 | 155.67 | 140.69 | 155.73 | 154.22 | 157.53 |
| MPL | 125.86 | 123.36 | 121.86 | 109.96 | 125.00 | 123.49 | 123.72 |

| Urban (100MHz, TDD 4G, DDDSU, 64T4R, TDL-C 300ns, 11Hz Doppler) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PBCH | Broadcast PDCCH (40-bit, AL16) | Broadcast PDSCH msg2 (no HARQ) | Broadcast PDSCH msg4 (no HARQ) | Unicast PDSCH 10Mbps (wHARQ) |  |  |
| MCL | 142.39 | 138.49 | 139.99 | 135.49 | 136.99 |  |  |
| MIL | 154.16 | 150.26 | 151.76 | 147.26 | 148.76 |  |  |
| MPL | 120.35 | 116.45 | 121.03 | 116.53 | 118.03 |  |  |

Based on the above results, we make the following observations and proposals:

Observation 5: In urban scenarios with downlink power of 44 dBm,  large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). Broadcast PDCCH emerges as a bottleneck in downlink.

Observation 6: In urban scenarios with downlink power of 44 dBm, PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Based on the above observations, we make the following proposals

Proposal 4: Consider enhancements aimed at reducing the control coverage gap in urban scenarios. Enhancements to both broadcast PDCCH and PUCCH are necessary. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

Our proposals on improving the performance of broadcast PDCCH are provided in [5].

# Conclusion

This contribution presents our views on remaining aspects of simulation assumptions and our baseline coverage analysis for FR1. In particular, the following observations and proposals have been made:

On remaining aspects of simulation assumption

Remark 1: The number of gNB receive chains is equal to the number of TXRUs in LLS. Antenna array gains due to digital beamforming or combining are included in LLS.

Remark 2: Analog antenna gains are computed assuming UE is in boresight. The same analog antenna gains are applies across all uplink and downlink channels. No correction factors are applied to analog antenna gains.

On identifying coverage bottlenecks:

Proposal 1: Use relative MCL, MIL, and/or MPL as a measure of coverage in different scenarios. Control coverage gaps are determined by the difference in MCL, MIL, or MPL of control channels in uplink and downlink. RAN1 strives to pursue enhancements that close the control coverage gaps in various deployment scenarios.

Proposal 2: Data channel coverage is determined based on the service data rate requirements. When data channel coverage is smaller than control channel coverage, RAN1 strives to pursue enhancements that improve the data channel coverage.

On baseline coverage analysis for FR1

Observation 1: Large coverage asymmetry is observed between uplink and downlink control channel coverage in the rural scenarios.

Observation 2: PUSCH coverage is also a bottleneck in ensuring minimum service requirements for eMBB and VoNR.

Proposal 3: Consider enhancements to PUCCH performance to reduce the uplink-downlink control channel coverage imbalance in rural scenarios. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

Observation 3: In urban scenarios with downlink power of 53 dBm, large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). PUCCH emerges as a bottleneck.

Observation 4: In urban scenarios with downlink power of 53 dBm, PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Observation 5: In urban scenarios with downlink power of 44 dBm, large coverage asymmetry observed between uplink control coverage (PUCCH) and downlink control coverage (broadcast PDCCH). Broadcast PDCCH emerges as a bottleneck in downlink.

Observation 6: In urban scenarios with downlink power of 44 dBm, PUSCH coverage to meet minimum service requirements is a significant bottleneck.

Proposal 4: Consider enhancements aimed at reducing the control coverage gap in urban scenarios. Enhancements to both broadcast PDCCH and PUCCH are necessary. Also consider enhancements to PUSCH performance to ensure minimum service requirements.

# Reference

- R1-2006818, Baseline FR1 coverage performance, 3GPP TSG RAN1 WG1, Meeting 102-e, August 17th – 28th, 2020.

- R1-2008625, Baseline FR2 coverage performance, 3GPP TSG RAN1 WG1, Meeting 103-e, October 26th -November 13th, 2020.

- R1-2008626, Potential coverage enhancement techniques for PUSCH, 3GPP TSG RAN1 WG1, Meeting 103-e, October 26th -November 13th, 2020.

- R1-2008627, Potential coverage enhancement techniques for PUCCH, 3GPP TSG RAN1 WG1, Meeting 103-e, October 26th -November 13th, 2020.

- R1-2008628, Potential coverage enhancement techniques for other channels, 3GPP TSG RAN1 WG1, Meeting 103-e, October 26th -November 13th, 2020.

- RP-193240, New SID on NR coverage enhancement, 3GPP TSG RAN Meeting #86, Sitges, Spain, Dec. 9-12, 2019.