3GPP TSG RAN WG1 Meeting #103-e	                                                                          R1-2009410

Oct 26th –  Nov 13th, 2020

Agenda item:    7.2.2

Source:              Moderator (Qualcomm Incorporated)

Title:                  NR-U email discussion thread 07

Document for:  Discussion and Decision

# Introduction

This paper summarized the following email discussion

[103-e-NR-NRU-07] Email discussion/approval on editorial changes for issues IA-C, DL-Z1, DL-Z2, DL-Z3, UL-02, and WB04 in R1-2008888 until 10/29 – Jing (Qualcomm)

Agreement:

The following TPs in R1-2009410 (FL summary) are endorsed (to be forwarded to editors):

TP 2-1 in section 2.

TP 3-2a in section 3.

TP 4-1 in section 4.

TP 5-2 in section 5

TP 6-1a in section 6.

TP 7-1 in section 7.

TP 7-2b in section 7

TP 5-1a in section 5

# Issue IA-C (On PRACH signal generation)

In R1-2007978 (Ericsson) and R1-2008125 (Samsung), a bug in PRACH signal generation in 38.211 section 5.3.2 is identified. The TP from R1-2007978 is listed below, which covers the TP from R1-2008125.

--------------------------------------- Text Proposal 2-1 for 38.211, Section 5.3.2 ---------------------------------------

Reason for changes

Error in equation for PRACH signal generation for the case of PRACH sequence lengths 571/1151.

Summary of changes

- Addition of the variable NscRB (number of subcarriers per RB) to the equation for PRACH signal generation

- Clarification to resolve differences in notation between 38.211 and 38.214.

Specs/Sections impacted

38.211 Section 5.3.2

Consequences if not approved

PRACH signal generation will be incorrect if sequence length 571 or 1151 is configured.

--------------------------------------- Start of Text Proposal for 38.211, Section 5.3.2 ---------------------------------------

*** Unchanged text omitted ***

5.3.2	OFDM baseband signal generation for PRACH

The time-continuous signal  on antenna port p for PRACH is defined by

sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA
K=ΔfΔfRA
k1=k0μ+NBWP,istart-Ngridstart,μNscRB-Ngridsize,μNscRB2+nRAstartNscRB+nRANRBRANscRBif LRA∈139, 839NRB,UL,n0+nRAstart,μ-NRB,UL,n0start,μNscRBif LRA∈571, 1151
k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ

where  and

-	 is given by clause 6.3.3;

-	 is the subcarrier spacing of the initial uplink bandwidth part during initial access. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	 is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise,  is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

-	nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213];

-	 is the PRACH transmission occasion index in frequency domain for a given PRACH transmission occasion in one time instance as given by clause 6.3.3.2;

-	 is the number of resource blocks occupied and is given by the parameter allocation expressed in number of RBs for PUSCH in Table 6.3.3.2-1.

-	NRB,UL,nstart,μ is the start CRB index of uplink RB set n corresponding to the quantity RBn,ULstart,μ in [6, TS 38.214]

-	n0 is the index of the RB set which contains the lowest PRACH transmission occasion in frequency domain indicated by nRAstart. The UE may assume that nRAstart is configured such that each PRACH transmission occasion is fully contained within an RB set.

-	 and  are given by clause 6.3.3

*** Unchanged text omitted ***

-------------------------------------------------------- End Text Proposal -----------------------------------------------------------

Please provide your view below:

| Company | View |
| --- | --- |
| OPPO | Fine with the TP |
| Nokia, NSB | support |
| LG Electronics | Fine with the TP |
| Qualcomm | Support the TP |
| Ericsson | Support the TP |
| Sharp | Support the TP |
| Samsung | Support the TP |
| ZTE, Sanechips | Agree |
| Huawei, HiSilicon | Support the adding NscRB Is it possible to unify the notation of NRB,UL,nstart,μ and RBn,ULstart,μ between 211 and 214? |
| Fujitsu | Support the TP |

# Issue DL-Z1 (On CSI-RS measurements and averaging)

R1-2008041, P2 [LG]:

TP 3-2 for TS 38.214 section 5.2.1.4.2.

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive: - the CSI parameters without averaging instances of any nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement or interference measurement located in different DL transmissions, where - the nzp-CSI-RSResource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatIndicator or CO-DurationList-r16; or - the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats, including indicating PDCCH(s), overlapped with nzp-CSI-RSResource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatIndicator - the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

R1-2008204, P3 [Nokia]:

TP 3-3 for  38.214 sub-clause 5.2.1.4.2

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive: - the CSI parameters without averaging instances of any nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement that do not occur located in different DL transmission bursts (defined in [16, TS 37.213]). - within remaining channel occupancy, or - in a time duration for which all OFDM symbols are occupied by a set of PDSCH and/or CSI-RS(s) that are scheduled/triggered to the UE including the associated PDCCH(s). - the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

R1-2008664, P1 [vivo]:

TP 3-4 for section 5.2.1.4.2 of TS 38.214.

----------------------------------------TP1: Start TP for Section 5.2.1.4.2 of TS 38.214 ------------------------------------- For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive: - the CSI parameters without averaging instances of any periodic/semi-persistent nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement located in different DL transmissions bursts (defined in [16, TS 37.213])., where - the nzp-CSI-RS Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or - the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with nzp-CSI-RSResource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell. - the interference measurements for computing CSI value - based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213]; and - without averaging instances of any periodic/semi-persistent CSI-IM-Resource in the corresponding CSI-IM-ResourceSet for interference measurement located in different DL transmissions, where - the CSI-IM-Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or - the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with CSI-IM-Resource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell. ----------------------------------------TP1: End TP for Section 5.2.1.4.2 of TS 38.214 --------------------------------------

R1-2007960 [ZTE]

TP3-5 for 5.2.1.4.2 of TS 38.214

Reasons for change: Capture the following agreements of channel measurement and interference measurement reached in RAN1 #102 e-meeting into specification. Summary of changes: Add two exceptions for a UE to average measurements of two or more instances of a periodic or semi-persistent NZP CSI-RS for channel measurement or for interference measurement. Specs/clauses affected: TS 38.214 Clause 5.2.1.4.2. Consequences if not approved: Incomplete specification for channel measurement and for interference measurement. --------------------------------------- < Start of TP#1 for 38.214 V16.3.0 [1]> ----------------------------------------- 5.2.1.4.2 Report Quantity Configurations < Unchanged parts are omitted > For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive: - the CSI parameters without averaging instances of any nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement or interference measurement, where the instances do not occur in the same remaining channel occupancy duration indicated by DCI format 2_0 if the UE is provided at least one of CO-DurationList-r16 or SlotFormatCombinationsPerCell, or the instances occur in a time duration for which not all OFDM symbols are occupied by a set of PDSCH and/or CSI-RS(s) that are scheduled/triggered to the UE and/or the scheduling/triggering PDCCH(s) if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell. - the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213]. -------------------------------------------------- < End of text proposal> -----------------------------------------------------

Please provide your view below for the above TPs:

| Company | View |
| --- | --- |
| Nokia, NSB | From these three TPs on the subject, we prefer the one which is technically correct (in terms of the RAN1#102 agreement) and the most compact |
| LG Electronics | Prefer the first TP as the proposing proponent. However, vivo’s one does not seem to correctly reflect previous agreement since the second bullet is for CSI-IM based interference measurement and is out of scope of the corresponding previous agreement. |
| Qualcomm | Prefer TP 3-2, which is more complete and concise |
| Ericsson | Prefer TP 3-2; however, I had trouble parsing it, particularly the 2nd sub-bullet. I have proposed an amendment below (TP 3-2a) that is hopefully easier to read. Also, it captures that this applies to periodic and semi-persistent CSI-RS corresponding to the agreements from last meeting. |
| Sharp | Fine with going with TP3-2. But, for the 1st sub-bullet point in TP 3-2, we prefer the text proposed in TP3-3 which covers not only DCI 2_0 basis but also FBE-COT. |
| Samsung | Fine with TP3-2 |
| ZTE, Sanechips | Seems ZTE’s TP has not been missed, I added it in the above summary part. Our TP(TP3-5) or TP3-2 is Okey for us. |
| Huawei, HiSilicon | Nokia’s TP reflects same meaning as LG’s in TP3-2. LG’s text reads more complete but the 2nd and 3rd bullet should be subbullet of the first bullet. The 2nd part of VIVO’s TP on CSI-IM is not necessary. The measurement on ZP CSI-RS is not required to be restricted in a burst. |
| Fujitsu | Fine with TP3-2. |
| Ericsson 2 | Perhaps I was not clear in my first comment. We are not fine with TP 3-2: - The agreements are for periodic and semi-persistent CSI-RS, and this need to be clarified - The 2nd paragraph of TP 3-2 is very hard to understand, and can easily lead to misunderstanding Please see TP 3-2a below that corrects these problems. |

================TP 3-2a for TS 38.214 section 5.2.1.4.2.==============

Reason for changes

Capture the agreement on CSI-RS averaging validation

Summary of changes

Clarify the CSI-RS can be averaged only if in the same COT or in continuous symbols covered by PDSCH and AP-CSI-RS and corresponding grants.

Specs/Sections impacted

38.214 Section 5.2.1.4.2

Consequences if not approved

Agreement not captured

=============Start of TP for TS 38.214 section 5.2.1.4.2==============

===Unchanged part omitted============

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging two or more instances of any periodic or semi-persistant nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement or for interference measurement located in different DL transmissions, where

-	the instances of the nzp-CSI-RSResource are not in the same channel occupancy duration indicated by DCI format 2_0, if the UE is provided at least one of SlotFormatIndicator or CO-DurationList-r16; or

-	the instances of the nzp-CSI-RSResource occur in a set of symbols which are not all occupied by PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats and the corresponding PDDCH(s), if the UE is neither provided with CO-DurationPerCell-r16 nor SlotFormatIndicator, but is provided with CSI-RS-ValidationWith-DCI-r16

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

===========end of TP 3-2a====================

# Issue DL-Z2 (On introduction of new PDSCH Mapping Type B Durations)

R1-2007979, P1 [Ericsson]:

TP4-1 for 38.214 Section 5.1.6.2

Add missing procedure text related to UE assumptions on default DMRS positions for the case of PDSCH scheduled by DCI 1_0 for the new Type B PDSCH mapping lengths {3,5,6,8,9,10,11,12,13}.

------------------------------------ Text Proposal (TP 4-1) for 38.214, Section 5.1.6.2 -----------------

Reason for changes

Add default type B PDSCH DMRS positions for new type B PDSCH lengths introduced in Rel.16.

Summary of changes

Add missing procedure text related to UE assumptions on default DMRS positions for the case of PDSCH scheduled by DCI 1_0 for the new Type B PDSCH mapping lengths {3,5,6,8,9,10,11,12,13}.

Specs/Sections impacted

38.214 Section 5.1.6.2

Consequences if not approved

No default DMRS position

------------------------------------ Start of Text Proposal (TP 4-1) for 38.214, Section 5.1.6.2 -----------------

*** Unchanged text omitted ***

When receiving PDSCH scheduled by DCI format 1_0 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A and type B, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 7 symbols for normal CP or 6 symbols for extended CP with mapping type B, the UE shall assume one additional single-symbol DM-RS present in the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of the PDSCH allocation duration, otherwise the UE shall assume that the additional DM-RS symbol is not present, and

-	For PDSCH with allocation duration of 4 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

*** Unchanged text omitted ***

----------------------------------------------------- End Text Proposal -----------------------------------------------------------

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| Nokia, NSB | We support |
| LG Electronics | Support the TP. |
| Qualcomm | Support the TP |
| Ericsson | Support the TP (as proponent). In case some companies wonder why some text is struck out in the TP, it is because this text duplicates what is already written in 38.211 Clause 7.4.1.1 which is now referenced for both Type A and Type B PDSCH mapping. |
| Sharp | Support the TP. |
| Samsung | Support the TP |
| ZTE, Sanechips | agree |
| Huawei, HiSilicon | Support |
| Fujitsu | Support the TP. |
| Lenovo, Motorola Mobility | Support the TP. |

# Issue DL-Z3 (Search space set switching behaviour)

R1-2008204, P2 [Nokia]:

TP 5-1 for 38.213 sub-clause 10.4. Follow Interpretation #1 for cross-carrier scheduling

If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, onfor the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set - if the UE monitors PDCCH onfor thea serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, onfor the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| OPPO | Leave for editor to decide |
| Nokia, NSB | Again, our motivation is to clarify cross-carrier scheduling with Search space groups of scheduled cell on the scheduling cell |
| LG Electronics | Support the TP. |
| Qualcomm | Support the TP |
| Ericsson | Support the TP |
| Sharp | Support the TP |
| Samsung | Support the TP |
| ZTE, Sanechips | Support the TP |
| Huawei, HiSilicon | The SS set switch feature of search space switching is for UE to save power when gNB acquired COT. The intention of the TP is to switch SS set group on the CC where PDSCH locates. However, if the PDCCH is to schedule a PUSCH on another carrier, UE should not switch SS set group on the CC where PUSCH locates because gNB may not acquire that CC when UL grant is transmitted. The existing word “ON” is more accurate. |
| Lenovo, Motorola Mobility | Similar to Huawei, we think the feature of switching was about on which serving cell the UE needs to monitor PDCCH, regardless of whether this is a cross-carrier or self-carrier scheduling. Therefore we think the current wording as accurate. |

After email discussion, it was identified the issue is for the cross carrier scheduling case, where the search space set group switching is for each scheduled cell (both implicit switching and explicity switching). The TP is updated as follows

============TP 5-1a for 38.213 10.4 and 11.1.1 =====================

Reason for changes

Clarify the search space set group switching behaviour when cross carrier scheduling is used

Summary of changes

Clarify the search space set group switching behaviour when cross carrier scheduling is used

Specs/Sections impacted

38.213 Section 10.4 and Section 11.1.1

Consequences if not approved

When cross carrier scheduling is applied, the search space group for scheduling cell will be switching instead of the search space group for scheduled cell.

==============Start of TP5-1a =========================

10.4	Search space set group switching

=====Unchanged text omitted==========

If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set group switching flag field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1;

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, onfor the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, onfor the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16

-	if the UE monitors PDCCH onfor a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH onfor the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, onfor the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, onfor the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH onfor thea serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH onfor the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, onfor the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

=====Unchanged part omitted===========

11.1.1 UE procedure for determining slot format

=====Unchanged part omitted===========

- a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on for the serving cell as described in Clause 10.4.

=====Unchanged part omitted===========

=================End of TP 5-1a=======================

R1-2008247, P1 [OPPO]:

TP 5-2 for 38.213 11.1.1.

Avoid the inconsistency in TS38.213 for the cell group configured cases.

----------------TP5-2 for TS 38.213 section 11.1.1--------------------

Reason for changes

Avoid the inconsistency in TS38.213 for the cell group configured cases.

Summary of changes

Avoid the inconsistency in TS38.213 for the cell group configured cases.

Specs/Sections impacted

38.213 Section 11.1.1

Consequences if not approved

Inconsistency in TS38.213 for the cell group configured cases

----------------TP5-2 for TS 38.213 section 11.1.1--------------------

11.1.1 UE procedure for determining slot format

<omit unrelated text>

- a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell or the set of serving cells, provided by CellGroupsForSwitching-r16, as described in Clause 10.4.

----------------------END of the TP5-2-------------------------------------------

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| OPPO | As explained in our Tdoc, there are some parts in TS 38.213 that explicitly write ‘one the serving cell or the set of serving cells’. To make them consistency, we support this TP. |
| Nokia, NSB | We are fine, but depending on above also here “PDCCH monitoring for scheduling on the serving cell…” would be more appropriate |
| LG Electronics | Support the TP. |
| Qualcomm | Support the TP and Nokia change |
| Ericsson | Support the TP and Nokia change. Suggest adjusting the commas to clarify that CellGroupsForSwitching-r16 only applies to the case when there is a set of serving cells. … PDCCH monitoring for scheduling on the serving cell, or the set of serving cells, provided by CellGroupsForSwitching-r16, as described in Clause 10.4. |
| Sharp | Support the TP. |
| Samsung | Support the TP |
| ZTE, Sanechips | Support TP and Ericsson modification |
| Huawei, HiSilicon | Support the TP |
| Fujitsu | Support the TP. |
| Lenovo, Motorola Mobility | Fine with the original TP, but it seems Ericsson's suggestion changes the scope of the monitoring "for the serving cell" instead of "on the serving cell" (see our comment to TP5-1). Since we don't agree to that change of scope, we don't agree to the scope change here either. |

# Issue UL-02 (Starting SRS symbol index)

In Rel-15, it was only possible to configure an SRS resource to start within the last 6 symbols of the slot. In Rel-16, it was agreed to support an SRS resource starting at any symbol of the slot. This feature is optional with per-band UE capability signaling (FG 10-11). CATT (R1-2008521) proposes that 38.214 be corrected to capture that the configuration is subject to UE capability

=======TP 6-1 for 38.214==============

6.2.1	UE sounding procedure

<Unrelated part omitted>

Subject to UE’s capability, The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot or at any symbol location within the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16 the higher layer parameter resourceMapping in SRS-PosResource-r16 with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

<Unrelated part omitted>

===End of TP 6-1=========================

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| OPPO | Agree with the intention, but we propose the following amendments. =======TP 6-1 for 38.214============== 6.2.1 UE sounding procedure <Unrelated part omitted> When the SRS is configured with the higher layer parameter SRS-Resource, the UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16 the higher layer parameter resourceMapping-r16 in SRS-PosResource-r16 with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. ===End of TP 6-1========================= |
| Nokia, NSB | We are ok with the proposal |
| Qualcomm | Support the TP |
| Ericsson | Agree with the intention of the original TP; also agree with some of OPPO’s amendments. They two TPs can be merged as follows: =======TP 6-1a for 38.214============== 6.2.1 UE sounding procedure <Unrelated part omitted> The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16 the higher layer parameter resourceMapping in SRS-PosResource-r16 with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. <Unrelated part omitted> ===End of TP 6-1========================= |
| Samsung | Support the TP |
| ZTE, Sanechips | Support the TP |
| Huawei, HiSilicon | Support the TP |
| CATT | Support the TP. Ericsson’s revision is fine to us. |
| Fujitsu | Support the TP |

=======TP 6-1a for 38.214==============

Reason for changes

Clarify SRS positions in slot is UE capability

Summary of changes

Specs/Sections impacted

38.214 Section 6.2.1

Consequences if not approved

UE capability information missing

=======Start of TP 6-1a for 38.214==============

6.2.1	UE sounding procedure

<Unrelated part omitted>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16 the higher layer parameter resourceMapping in SRS-PosResource-r16 with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

<Unrelated part omitted>

===End of TP 6-1=========================

# Issue WB-04

From R1-2007777 (Fujitsu),

------------------------------------ Text Proposal 7-1 for Clause 7 of TS 38.214------------------------

Reason for changes

Use the right RRC parameter name for BWP

Summary of changes

Use the right RRC parameter name for BWP

Specs/Sections impacted

38.214 Section 7

Consequences if not approved

Wrong RRC parameter name

------------------------------------ Start of Text Proposal 7-1 for Clause 7 of TS 38.214------------------------

*** Unchanged text omitted ***

For a carrier, the UE expects  N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1  where 0≤s0≤s1≤NRB-set,x-1 for a BWP i configured by BWP-DownlinkCommon or BWP-DownlinkDedicated initialDownlinkBWP or BWP-Downlink for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated initialUplinkBWP or BWP-Uplink for the UL BWP. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1  where NRB-set,xBWP  is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier.

*** Unchanged text omitted ***

---------------------------- End of TP 7-1 -------------------------

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| OPPO | Left for editor to decide |
| Nokia, NSB | I think it would be sufficient to keep only “BWP-DownlinkCommon”, since this IE contains generic parameters for any BWP (initial or dedicated), such as location BWP-DownlinkCommon ::= SEQUENCE { genericParameters BWP, pdcch-ConfigCommon SetupRelease { PDCCH-ConfigCommon } OPTIONAL, -- Need M pdsch-ConfigCommon SetupRelease { PDSCH-ConfigCommon } OPTIONAL, -- Need M ... } BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED { extended } OPTIONAL -- Need R } |
| Qualcomm | Support the TP |
| Ericsson | I don't think this TP is needed; the orginal wording was sufficient to cover all cases of SpCell configuration and SCell addition for both SA and NSA deployments. If BWP-Downlink/UplinkDedicated is removed, then SpCell configuration by dedicated signaling is not covered. |
| Sharp | Support the TP. |
| ZTE,Sanechips | Left for editor to decide |
| Huawei, HiSilicon | Support the TP. The name of IE should be used. |
| Fujitsu | Support the TP. Correction is needed, as a BWP cannot be configured by BWP-Downlink/UplinkDedicated alone but the original wording is misleading. Although we agree with Nokia’s comment that keeping only“… linkCommon” is technically sufficient, it seem better to use the parameter names as in the TP for alignment with Clause 12(Bandwidth part operation) of TS 38.213. |

From R1-2008386 (Sharp), it is proposed to align RRC parameter names.

The latest TS38.331 specifies RRC parameters for intra-cell guard bands as follows [1].

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

As above, intra-cell guard band configuration is updated to include configuration per SCS (IntraCellGuardBandsPerSCS-r16). The structure of IntraCellGuardBandsPerSCS-r16 is as follows.

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

Section 7 of TS38.214V16.3,0 doesn’t reflect this change. Therefore, we propose to update the spec. as follows. In addition, intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 are replaced by intraCellGuardBands-r16. This change should also be reflected.

TP 7-2

--------- beginning of text proposal for TS 38.214-----------------------

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBands-r16 for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with  NRB-set,x-1    intra-cell guard bands on a carrier with SCS configuration μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ  and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to SCS configuration μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as

RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise

and

RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise

The RB set with index s consists of RBs,xsize,μ resource blocks where  RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBands-r16 for SCS configuration μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to SCS configuration μ and carrier size Ngrid,xsize,μ. μNgrid,xsize,μFor either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1.

For a carrier with SCS configuration μ, the UE expects   N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1  where 0≤s0≤s1≤NRB-set,x-1 for a BWP I configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP I, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1  where NRB-set,xBWP  is the number of RB sets contained in the BWP I and RB set 0 within the BWP I corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP I corresponds to RB set s1 in the carrier.

When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier with SCS configuration μ, the UE is indicated that no intra-cell guard-bands are configured for the carrier and expects NRB-set,x>1. For SCS configuration μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For SCS configuration μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.

-------- Unchanged contents are omitted

--------- end of text proposal

Please provide your view below for the above TP:

| Company | View |
| --- | --- |
| OPPO | Agree in principle, further discussing TP content is needed. |
| Nokia, NSB | We are fine with the TP, which generalizes text for DL and UL, and such simplifies the specification. |
| LG Electronics | Agree to - Change GB s,xstartsize,μ - Add SCS configuration μ For other changes, modification made by spec editor as in R1-2008292 seems sufficient. |
| Qualcomm | Support the TP |
| Ericsson | Agree to the change GB s,xstartsize,μ For the rest of the TP, we prefer to just substitute the proper RRC parameter names for DL and UL separately rather than trying to achieve more compaction of the text. Besides the first paragraph is not correct anymore; “any of” doesn’t make sense if only a single parameter is used. It is not necessary to write everywhere “SCS configuration μ.” This can be handled once in the first paragraph as follows. Then all other changes can be removed. There is no risk of confusion, since all relevant variables have superscript μ. We propose the following simpler TP: TP 7-2a *** Unchanged text omitted *** For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandsUL-List-r16 for an UL carrier and intraCellGuardBandsDL-List-r16 for a DL carrier, where each list entry corresponds to an SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. *** Unchanged text omitted *** |
| Sharp | - For the change “GB s,xstartsize,μ” - It should be OK since it’s purely editorial. - For the change “intraCellGuardBands-r16 for SCS configuration μ” - It corresponds to the cralification of SCS specific intra-cell guard bands, confirmed in RAN2. It should be OK. - For the change “For a carrier with SCS configuration μ” - This clarification is based on our understanding that the carrier in Clause 7 of TS38.214 corresponds to a SCS specific carrier provided by higher layer parameter SCS-specificCarrier. - For addition of “SCS configuration” in each μ - Our intention is to align other parts of TS38.214. On the other hand, we are fine with Ericsson’s proposal. Regarding Ericsson’s TP, We are not fine with the change “intraCellGuardBandsUL-List-r16 for an UL carrier”, because intraCellGuardBandsUL-List-r16 doesn’t corresponds to an UL carrier. In our understanding, each SCS-specific guard band configuration intraCellGuardBands-r16 corresponds to an UL carrier. Sharp’s updated TP7-2a TP 7-2a --------- beginning of text proposal for TS 38.214----------------------- 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBands-r16 for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with SCS configuration μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise and RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise The RB set with index s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBands-r16 for μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. μNgrid,xsize,μFor either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1. For a carrier with μ, the UE expects N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1 for a BWP I configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP I, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP I and RB set 0 within the BWP I corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP I corresponds to RB set s1 in the carrier. When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier with μ, the UE is indicated that no intra-cell guard-bands are onfigure for the carrier and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs. -------- Unchanged contents are omitted --------- end of text proposal |
| Samsung | Fine with the updated TP from Sharp |
| ZTE, Sanechips | Share same views with Ericsson. |
| Huawei, HiSilicon | The IE name for intra cell guard band for UL carrier and DL carrier are intraCellGuardBandsDL-List-r16 and intraCellGuardBandsUL-List-r16. The structure of intraCellGuardBands-r16 do not include SCS itself. The clarification on the parameter μ is fine. |
| Fujitsu | Fine with the updated TP from Sharp. However, considering the concern of Huawei, maybe the IE name could be IntraCellGuardBandsPerSCS-r16 rather than intraCellGuardBands-r16 |
| Sharp | According to Fujitsu’s proposal, Sharp’s TP is further updated. One more redundant “SCS configuration” is also deleted. TP 7-2b --------- beginning of text proposal for TS 38.214----------------------- 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of IntraCellGuardBandsPerSCS-r16 for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise and RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise The RB set with index s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with IntraCellGuardBandsPerSCS-r16 for μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. μNgrid,xsize,μFor either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1. For a carrier with μ, the UE expects N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1 for a BWP I configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP I, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP I and RB set 0 within the BWP I corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP I corresponds to RB set s1 in the carrier. When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier with μ, the UE is indicated that no intra-cell guard-bands are onfigure for the carrier and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs. -------- Unchanged contents are omitted --------- end of text proposal |
| Ericsson 2 | We are fine with TP 7-2b from Sharp. |

--------- TP 7-2b for TS 38.214-----------------------

Reason for changes

SCS dependent intra-cell guard band configuration

Summary of changes

SCS dependent intra-cell guard band configuration. RRC parameter name consistency

Specs/Sections impacted

38.214 Section 7

Consequences if not approved

Did not capture SCS dependency configuration of RRC parameters

--------- beginning of text proposal 7-2b for TS 38.214-----------------------

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access, when the UE is configured with any of IntraCellGuardBandsPerSCS-r16 for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with  NRB-set,x-1    intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ  and GB s,xstartsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as

RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise

and

RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise

The RB set with index s consists of RBs,xsize,μ resource blocks where  RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with IntraCellGuardBandsPerSCS-r16 for μ, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. μNgrid,xsize,μFor either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1.

For a carrier with μ, the UE expects   N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1  where 0≤s0≤s1≤NRB-set,x-1 for a BWP I configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP. Within the BWP I, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1  where NRB-set,xBWP  is the number of RB sets contained in the BWP I and RB set 0 within the BWP I corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP I corresponds to RB set s1 in the carrier.

When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier with μ, the UE is indicated that no intra-cell guard-bands are onfigure for the carrier and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.

-------- Unchanged contents are omitted

--------- end of text proposal