3GPP TSG RAN WG1 #104-e		R1-2100127

e-Meeting, January 25th – February 5th, 2021

Source:	OPPO

Title:	Text Proposals on NR Positioning

Agenda Item:	7.2.8

Document for:	Discussion and Decision

Text Proposal on DL PRS reception

Reason for changes:

During measurement gap, the UE can measure DL PRS outside the active DL BWP or a DL PRS with a numerology different from the one of the active DL BWP. It is specified in TS 38.214 that if the UE is expected to measure DL PRS resource outside the active BWP, the UE can request a measurement gap. But in contrast, the current specification has not specified that the UE can request a measurement gap when the UE is expected to measure expected to measure DL PRS resource with a different numerology. That is a critical issue. The consequence is that the UE is not able to measure a DL PRS resource if the UE is configured with DL PRS resource with a numerology different from the numerology of the active BWP with no measurement gap. Then the UE would have to skip the measurement. A change in TS 38.214 is needed to resolve the issue.

Summary of changes:

In section 5.1.6.5 of TS 38.214, add specification that the UE can request measurement gap when the UE is expected to measure DL PRS with numerology different from the active BWP.

Specs/Sections impacted:

TS 38.214 V16.4.0 /5.1.6.5

Consequences if not approved:

The UE is not able to measure the DL PRS when the numerology of DL PRS is different from that of the active BWP.

The text proposal for TS 38.214 V16.4.0 is:

#### 5.1.6.5 PRS reception procedure ## *** Unchanged text is omitted *** The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP, it may request a measurement gap in higher layer parameter MeasGapConfig [12, TS 38.331]. The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell. ## *** Unchanged text is omitted ***