3GPP TSG RAN WG1 Meeting #104-e	R1-2100509

e-Meeting, January 25th – February 5th, 2021

Agenda item:		8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Support of a maximum DL TBS of 1736 bits for eMTC

Document for:		Discussion and Decision

# Introduction

The revised Rel-17 WI for additional enhancements for NB-IoT and LTE-MTC includes an objective to add optional UE capability to support a maximum DL TBS of 1736 bits as below [1]:

- Add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only. [LTE-MTC] [RAN1, RAN2]

  - Determine soft buffer size [RAN1]

  - Capability signaling without introducing a new UE category [RAN2]

  - There shall be no changes to: DCI formats, TBS tables, CQI tables

  - This objective begins work from RAN#90, i.e. December 2020

In this contribution, we discuss support of maximum DL TBS of 1736 bits for HD-FDD Cat-M1 UEs in CE mode A.

# Discussion

According to the objective in the WID, a maximum DL TBS of 1736 bits shall be supported for HD-FDD Cat-M1 UEs in CE mode A only as an optional UE capability. Therefore, after the UE indicates this capability to the eNB, the eNB should enable the UE to use this capability through higher layer configuration.

Proposal 1: A capability is introduced for Rel-17 Cat-M1 UEs (BL/CE UEs with DL PDSCH bandwidth of 1.4 MHz) supporting a maximum DL TBS of 1736 bits in CEModeA.

Proposal 2: The use of a maximum DL TBS of 1736 bits for a Rel-17 Cat-M1 UE with the corresponding capability is enabled through higher layer configuration.

The objective for adding support for maximum DL TBS of 1736 bits also clarifies that there shall be no changes to DCI formats, TBS tables, and CQI tables. We first consider support for the larger TBS with 16-QAM and 64-QAM. A BL/CE in CEModeA uses the Table 7.1.7.1-1 in [2] as the modulation and coding scheme (MCS) table, reproduced Table 1 below. If the UE is not configured with the higher layer parameter ce-PDSCH-64QAM-Config-r15, the maximum supported order is 16-QAM and the UE reads a 4-bit MCS field in the DL DCI (DCI format 6-1A). In this case, the UE is not expected to receive DCI indicating IMCS>15. Therefore, the first 16 rows of the table (shown shaded) are used. It is apparent that the maximum TBS index then corresponds to ITBS=14. The corresponding TBS is obtained from Table 7.1.7.2.1-1 in [2], the relevant portion of which is shown in Table 2 below. Thus, for 16-QAM, the first 15 rows (shown shaded) are used. Since there shall be no changes to DCI formats, TBS tables, or CQI tables, the increased maximum DL TBS of 1736 bits can only be used with ITBS=14 and NPRB=6.

Observation 1: The existing TBS table enables the use of the increased maximum DL TBS of 1736 bits with 16-QAM only with ITBS=14 and NPRB=6.

Table 1. Table 7.1.7.1-1 in [2]: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 1 |
| 2 | 2 | 2 | 2 |
| 3 | 2 | 2 | 3 |
| 4 | 2 | 2 | 4 |
| 5 | 2 | 4 | 5 |
| 6 | 2 | 4 | 6 |
| 7 | 2 | 4 | 7 |
| 8 | 2 | 4 | 8 |
| 9 | 2 | 4 | 9 |
| 10 | 4 | 6 | 9 |
| 11 | 4 | 6 | 10 |
| 12 | 4 | 6 | 11 |
| 13 | 4 | 6 | 12 |
| 14 | 4 | 6 | 13 |
| 15 | 4 | 6 | 14 |
| 16 | 4 | 6 | 15 |
| 17 | 6 | 6 | 15 |
| 18 | 6 | 6 | 16 |
| 19 | 6 | 6 | 17 |
| 20 | 6 | 6 | 18 |
| 21 | 6 | 6 | 19 |
| 22 | 6 | 6 | 20 |
| 23 | 6 | 6 | 21 |
| 24 | 6 | 6 | 22 |
| 25 | 6 | 6 | 23 |
| 26 | 6 | 6 | 24 |
| 27 | 6 | 6 | 25 |
| 28 | 6 | 6 | 26/26A |
| 29 | 2 | 2 | reserved |
| 30 | 4 | 4 |  |
| 31 | 6 | 6 |  |

Table 2. From Table 7.1.7.2.1-1 in [2]: Transport block size table

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 |
| 22 | 520 | 1064 | 1608 | 2152 | 2664 | 3240 |
| 23 | 552 | 1128 | 1736 | 2280 | 2856 | 3496 |
| 24 | 584 | 1192 | 1800 | 2408 | 2984 | 3624 |
| 25 | 616 | 1256 | 1864 | 2536 | 3112 | 3752 |
| 26 | 712 | 1480 | 2216 | 2984 | 3752 | 4392 |
| 26A | 632 | 1288 | 1928 | 2600 | 3240 | 3880 |

If the UE is configured with the higher layer parameter ce-PDSCH-64QAM-Config-r15, the maximum supported modulation order is 64-QAM and the UE reads a 5-bit MCS field in the DCI. In this case, the UE may receive DCI indicating an MCS index corresponding to IMCS≤26. While the rows of the TBS table shown in Table 2 corresponding to 14≤ITBS≤26 are relevant for 64-QAM, the DL TBS should now be limited to a maximum size of 1736 bits instead of the existing limit of 1000 bits. As seen from the table, the TBS of 1736 bits can be used corresponding to the TBS index range 14≤ITBS≤26 with a valid NPRB depending on the TBS index.

Observation 2: The increased maximum DL TBS of 1736 bits can be used with 64-QAM with all TBS indices in the range 14≤ITBS≤26.

To derive the benefit of higher peak data rates, the increased maximum TBS of 1736 bits should be supported with both 16-1QAM and 64-QAM.

Proposal 3: A maximum DL TBS of 1736 bits is supported both with and without configuration of 64-QAM for PDSCH.

Support for the increased maximum DL TBS of 1736 bits would require an update to the specification in TS 36.213. The relevant part of the current specification reads as follows.

if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Clause 7.1.7.2.1,

-	TBS=minTBS',1000

This is based on support of a maximum TBS of 1000 bits (when the UE is configured with 64-QAM) and must be modified corresponding to a maximum TBS of 1736 bits when a UE is also configured with this larger maximum TBS.

The soft buffer size for different UE categories is specified in [3]. For the DL category M1, we have the following entry from Table 4.1A-1 in [3].

Table 3. Entry for Cat-M1 UE from Table 4.1A-1 in [3]

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category M1 | 1000 | 1000 | 25344 | 1 |

According to Table 3, the soft buffer size for a Cat-M1 UE is 25344. While the soft buffer size was originally determined for a Rel-13 UE with a maximum TBS of 1000 bits for 8 HARQ processes, the same soft buffer size was retained when the number of HARQ processes was increased to 10 in Rel-14 to avoid increasing the complexity. For a UE supporting the increased TBS of 1736 bits, however, a new soft buffer size must be determined.

A simple approach is to simply scale the currently specified soft buffer size based on the increase in the maximum TBS, which would yield 25344·1736/1000=43988 soft channel bits.

Observation 3: The total number of soft channel bits for supporting an increased maximum TBS of 1736 bits based on scaling the total number of soft channel bits specified for supporting a maximum TBS of 1000 bits is 43988.

Alternatively, we can calculate the total number of soft channel bits with Full Buffer Rate Matching (FBRM) for the increased maximum TBS of 1736 bits using N∙96∙X+28/32 for a Turbo encoder using a mother coding rate of 1/3, where N is the number of HARQ processes and X is the maximum TBS. With the support for 14 HARQ processes also being added in Rel-17, the soft buffer size for this case may be considered as well although there is no consideration to increase the soft buffer size for supporting for 14 HARQ processes with the legacy maximum TBS capability of 1000 bits. Thus, the total number of soft channel bits can be calculated to be as follows.

- 10 HARQ processes → 53760

- 14 HARQ processes → 75264

These correspond to a soft buffer size of a factor of ~2.12 increase and a factor of ~2.97 increase, respectively, compared to that of 25344 soft channel bits for a legacy UE. On the other hand, the number of soft channel bits calculated for a Rel-17 UE not supporting the increased maximum DL TBS (i.e., supporting a maximum DL TBS of 1000 bits) is 44352, which is 1.75 times that of the legacy UE.

Observation 4: The total number of soft channel bits calculated based on a Turbo encoder with mother coding rate of 1/3 as

- 53760 for a maximum DL TBS of 1736 bits and 10 HARQ processes → a factor of ~2.12 increase compared to a legacy UE;

- 75264 for a maximum DL TBS of 1736 bits and 14 HARQ processes → a factor of ~2.97 increase compared to a legacy UE;

- 44352 for a maximum DL TBS of 1000 bits and 14 HARQ processes → a factor of 1.75 increase compared to a legacy UE;

While supporting the larger number of HARQ processes in Rel-17 with the increased TBS has the potential to further increase the peak data rate, it comes at the expense of higher UE complexity in the form of the soft buffer size. One possibility to limit the complexity is to determine the soft buffer size based on 10 HARQ processes and not increase it for 14 HARQ processes even when supported together with the increased maximum TBS. The reduction in soft buffer size for 14 HARQ processes can be achieved, for example, through Limited Buffer Rate Matching (LBRM). This may come at some cost to the DL performance, however.

Proposal 4: Further study the tradeoffs between cost and benefits for different soft buffer size candidates for the increased maximum DL TBS of 1736 bits.

# Conclusions

In this contribution, we discuss supporting a maximum DL TBS of 1736 bits for HD-FDD Cat-M1 UEs in CE mode A as an optional UE capability and make the following observations and proposals.

Observation 1: The existing TBS table enables the use of the increased maximum DL TBS of 1736 bits with 16-QAM only with ITBS=14 and NPRB=6.

Observation 2: The increased maximum DL TBS of 1736 bits can be used with 64-QAM with all TBS indices in the range 14≤ITBS≤26.

Observation 3: The total number of soft channel bits for supporting an increased maximum TBS of 1736 bits based on scaling the total number of soft channel bits specified for supporting a maximum TBS of 1000 bits is 43988.

Observation 4: The total number of soft channel bits calculated based on a Turbo encoder with mother coding rate of 1/3 as

- 53760 for a maximum DL TBS of 1736 bits and 10 HARQ processes → a factor of ~2.12 increase compared to a legacy UE;

- 75264 for a maximum DL TBS of 1736 bits and 14 HARQ processes → a factor of ~2.97 increase compared to a legacy UE;

- 44352 for a maximum DL TBS of 1000 bits and 14 HARQ processes → a factor of 1.75 increase compared to a legacy UE;

Proposal 1: A capability is introduced for Rel-17 Cat-M1 UEs (BL/CE UEs with DL PDSCH bandwidth of 1.4 MHz) supporting a maximum DL TBS of 1736 bits in CEModeA.

Proposal 2: The use of a maximum DL TBS of 1736 bits for a Rel-17 Cat-M1 UE with the corresponding capability is enabled through higher layer configuration.

Proposal 3: A maximum DL TBS of 1736 bits is supported both with and without configuration of 64-QAM for PDSCH.

Proposal 4: Further study the tradeoffs between cost and benefits for different soft buffer size candidates for the increased maximum DL TBS of 1736 bits.

# References

- RP-201306, “Additional enhancements for NB-IoT and LTE-MTC,” Rel-17 WID, Huawei, HiSilicon, RAN #88e.

- 3GPP TS 36.213, v16.4.0

- 3GPP TS 36.306, v16.3.0.