3GPP TSG RAN WG1 Meeting #104-e  		R1-2100580

e-Meeting, January 25th – February 5th, 2021

Agenda Item: 7.1

Source: MediaTek Inc.

Title: Clarification on back-to-back PUSCHs scheduling restriction in Rel-15

Document for: Discussion and Decision

# Introduction

In NR Rel-15, there is a restriction on scheduling the UE with another dynamic PUSCH before the first PUSCH with the same HARQ process ID has been transmitted. The restriction is captured in Clause 6.1 of TS38.214 (V15.11.0) as follows:

The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0 or 0_1 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

In this contribution, we discuss some of the ambiguity and the cases of dynamic PUSCH scheduling that are not properly covered in the above paragraph.

# Ambiguity of the Specs’ text

The TP relevant to the above restriction was agreed in RAN1#94bis:

Agreements: - RAN1 clarifies operation by adopting the TP to 6.1 of 38.214 below, which corresponds to updating a previous agreement (copied below) A UE shall upon detection of a PDCCH with a configured DCI format 0_0 or 0_1 transmit the corresponding PUSCH as indicated by that DCI. For any two HARQ process IDs in a given cell, if the UE is scheduled to start a PUSCH transmission in symbol j by a PDCCH in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than symbol j by a PDCCH starting later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0 or 0_1 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process. Copy of previous agreements as in RAN1#88: For UE configured with K repetitions for a TB transmission with/without grant, the UE can continue repetitions (FFS can be different RV versions, FFS different MCS) for the TB until one of the following conditions is met - If an UL grant is successfully received for a slot/mini-slot for the same TB - FFS: How to determine the grant is for the same TB - FFS: An acknowledgement/indication of successful receiving of that TB from gNB - The number of repetitions for that TB reaches K - FFS: Whether it is possible to determine if the grant is for the same TB Note that this does not assume that UL grant is scheduled based on the slot whereas grant free allocation is based on mini-slot (vice versa)

As it can been seen from the feature lead summary [1], the intention of the restriction is to simplify the UE implementation by excluding a “back-to-back” PUSCHs scheduling with the same HARQ process ID. By back-to-back scheduling, it meant that the UE doesn’t expect another DCI scheduling a PUSCH for a given HARQ process ID unless the last PUSCH of that HARQ process has been transmitted. The relevant section from the feature lead summary [1] is copied below. More background on the motivation for the restriction can be found in [2][3] as well.

2.2 Back-to-back uplink transmissions Currently in specification, a PDSCH is not expected to be transmitted for the same HARQ process until after the HARQ-ACK has been transmitted. The provides some reasonable constraint on dynamic scheduling that helps simplify implementation and testing. It was noted by [Intel] that the equivalent limitation for the uplink has not been captured in specification but should be this meeting. The following proposal is provided from [Intel]. Proposal (from [Intel]): - For each HARQ process ID, the UE is not expected to receive a scheduling DCI for a unicast PUSCH transmission with the same HARQ process ID until - The time after the end of the expected transmission of the PUSCH, including any repetition of the PUSCH, of an earlier transmission on the same HARQ process ID. Proposal (offline consensus): RAN1 clarifies operation by adopting the TP below, which corresponds to updating a previous agreement for a condition associated with grant-based repetition of a TB which was not captured in specification. --------------------------------------------------- Start of Text Proposal for 38.214 ------------------------------------------------- < Unchanged parts are omitted > A UE shall upon detection of a PDCCH with a configured DCI format 0_0 or 0_1 transmit the corresponding PUSCH as indicated by that DCI. For any two HARQ process IDs in a given cell, if the UE is scheduled to start a PUSCH transmission in symbol j by a PDCCH in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than symbol j by a PDCCH starting later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0 or 0_1 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process. --------------------------------------------------- End of Text Proposal for 38.214 -------------------------------------------- The previous agreement is noted below. RAN1#88 Agreements: For UE configured with K repetitions for a TB transmission with/without grant, the UE can continue repetitions (FFS can be different RV versions, FFS different MCS) for the TB until one of the following conditions is met - If an UL grant is successfully received for a slot/mini-slot for the same TB - FFS: How to determine the grant is for the same TB - FFS: An acknowledgement/indication of successful receiving of that TB from gNB - The number of repetitions for that TB reaches K - FFS: Whether it is possible to determine if the grant is for the same TB Note that this does not assume that UL grant is scheduled based on the slot whereas grant free allocation is based on mini-slot (vice versa) Also for reference, following wording in specification for the downlink from 38.214 is provided. “The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Subclause 9.2.3 of [6].”

However, the current text of TS38.214 doesn’t properly reflect the intention of the TP. As it was highlighted in [4] in RAN1#103e meeting, the text can be interpreted in two ways:

- Interpretation 1: DCI scheduling another PUSCH for a given HARQ process shall not occur before the end of the last PUSCH.

- Interpretation 2: Another PUSCH for a given process shall not occur before the end of the last PUSCH.

In our view, the first interpretation is the intention of the TP. Hence, the specs need to be modified to correctly reflect the first interpretation.

# Conflict with CG-PUSCH repetitions termination

According to Clause 6.1.2.3.1 in TS38.214 (V15.11.0), the repetitions of CG-PUSCH can be terminated if the UE receives a DCI scheduling a PUSCH with the same HARQ process.

Clause 6.1.2.3.1 in TS38.214 (V15.11.0): For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0 or 0_1, whichever is reached first.

This implies that the UE can receive a DCI that schedules a PUSCH for a given HARQ process before the end of the expected transmission of the last repetition of the CG-PUSCH for that HARQ process, which conflicts with the above mentioned restriction. The conflict can be summarized as follows:

- Clause 6.1.2.3.1 says: a DCI with C-RNTI (scheduling a PUSCH for a given HARQ process) can be received before the end of the last PUSCH for that HARQ process (because the initial PUCH is CG).

- Clause 6.1 says: a DCI with C-RNTI (scheduling a PUSCH for a given HARQ process) cannot be received before the end of the last PUSCH for that HARQ process.

Hence, this conflict in the spec should be resolved by updating the text in Clause 6.1 to exclude the case of “DG overriding CG” (that is described in Clause 6.1.2.3.1) from the restriction.

# Other cases of dynamic PUSCH scheduling

It can noticed from the FL summary [1] that the TP meant to cover the dynamic PUSCH scheduling case (referred to it as “grant-based”). However, the existing text of the restriction didn’t include all the cases of dynamic PUSCH scheduling, it listed only the PUSCHs that are scheduled with DCIs scrambled by C-RNTI or MCS-C-RNTI. The following two cases should be included in the restriction:

Case#1: DCI scrambled with TC-RNTI, which is used for scheduling the initial transmission and retransmission of Msg3, are not currently included in the restriction. These are dynamically scheduled PUSCHs, and the UE behaviour is identical to PUSCHs scheduled with DCIs scrambled by C-RNTI.

Case#2: DCI scrambled by CS-RNTI when used for the second (or later) retransmission of the CG-PUSCH, as illustrated in Figure 1. Similar to the first case, the subsequent retransmissions of a CG-PUSCH are considered dynamic PUSCHs. Hence, the mentioned restriction should be applicable to this case as well.

Figure 1: Scheduling multiple retransmissions of CG-PUSCH using DCIs scrambled by CS-RNTI.

Hence, the current specs need to be updated to capture the other cases of dynamically scheduled PUSCHs. Therefore, we have the following proposal:

- Adopt the following TP for TS38.214;

6 Physical uplink shared channel related procedure 6.1 UE procedure for transmitting the physical uplink shared channel < Unchanged parts are omitted > A UE shall upon detection of a DCI format scheduling a PUSCH transmit the corresponding PUSCH unless the UE does not generate a transport block as described in [10, TS38.321] and there is no PUCCH with CSI/HARQ-ACK that overlaps in time with the PUSCH. In this release of the specification, the UE behavior is undefined if there would be a PUCCH with CSI/HARQ-ACK overlapping in time with a PUSCH scheduled by a DCI format and if the UE does not generate a transport block as described in [10, TS38.321] when skipUplinkTxDynamic provided by higher layers is set to true. Upon detection of a DCI format 0_1 with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to receive a DCI format 0_0 scrambled by TC-RNTI schedules a PUSCH for a given HARQ process until after the end of the transmission of the last PUSCH for that HARQ process. If the UE receives a DCI scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI schedules a PUSCH for a given HARQ process, the UE is not expected to receive another DCI scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI scheduling a PUSCH with the same HARQ process until after the end of the transmission of the last PUSCH for that HARQ process. < Unchanged parts are omitted >

# References

- R1-1811891, “Summary for Rel-15 DL/UL data scheduling and HARQ procedure”, Qualcomm, RAN1#94bis, Oct. 2018.

- R1-1810756, “Remaining issues on NR scheduling & HARQ”, Intel, RAN1#94bis, Oct. 2018.

- R1-1807364, “Remaining Issues on DL/UL Scheduling, Processing Time and HARQ management,” Qualcomm, RAN1#93, May 2018.

- R1-2008767, “Clarification on the PUSCH scheduling constraint in Rel-15”, Apple, RAN1#103e, Oct. 2020.