3GPP TSG RAN WG1 #104-e		R1-2100787

e-Meeting, January 25th – February 5th, 2021

Agenda Item:     8.1.2.4

Source:	Spreadtrum Communications

Title:	             Discussion on enhancements on HST-SFN deployment

Document for:	Discussion and decision

Introduction

In RAN1#102e, the following agreement on HST-SFN deployment was achieved: [1]

Agreement

For the discussion purpose consider the following categorization of the enhanced DL transmission schemes

- Scheme 1:

  - TRS is transmitted in TRP-specific / non-SFN manner

  - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner

- Scheme 2:

  - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner

  - PDSCH from TRPs is transmitted in SFN manner

Agreement

Study the following aspects of the enhanced transmission schemes:

- For scheme 1:

  - Target DL physical channels, i.e., PDSCH only or PDSCH + PDCCH

  - Whether more than 2 QCL/TCI states are required and corresponding signaling details

  - Whether and how to indicate scheme 1 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states

  - QCL relationship between TRS and DMRS ports

  - Note: Other schemes/aspects are not precluded

- For scheme 2:

  - Association of each MIMO layer of PDSCH to DM-RS antenna ports

  - Whether more than 2 QCL/TCI states are required and corresponding signaling details

  - Whether and how to indicate scheme 2 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states

  - Note: Other schemes/aspects are not precluded

Agreement

For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme:

- 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation

- 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step

- 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step

- Note: A second set of TRS resource(s) may be transmitted at 3rd step.

Agreement

Study TRP-based frequency offset pre-compensation including the following aspects:

- Aspects related to indication of the carrier frequency determined based on the received TRS resource(s) in the 1st step

  - Option 1: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step

    - Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step

    - Type of the uplink reference signals / physical channel used in the 2nd step, necessity of new configuration and corresponding signaling details

  - Option 2: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework

    - FFS: Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step

    - CSI reporting aspects, configuration, quantization, signalling details, etc.

- New QCL types/assumption for TRS with other RS (e.g., SS/PBCH), when TRS resource(s) is used as target RS in TCI state

- New QCL types/assumptions for TRS with other RS (e.g., DM-RS), when TRS resource(s) is used as source RS in the TCI state

- Target physical channels (e.g., PDSCH only or PDSCH/PDCCH) and reference signals that should be supported for pre-compensation

- Signaling/procedural details on whether/how the pre-compensation is applied to target channels

- Whether multiple sets of TRS and pre-compensation on TRS is needed in 3rd step.

- Note: Other aspects/schemes are not precluded

Furthermore, in RAN1#103e, the following agreement on HST-SFN deployment was achieved: [2]

Agreement

Support at least the following configuration for HST scenario in Rel-17

- The same DMRS port(s) can associate with multiple TCI states

  - FFS other details

Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

Agreement

At most two TCI states are supported for HST scenario in Rel-17

- FFS: Whether to support more than two TCI states for FR2

- FFS configuration/signalling details of the TCI states

Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

Agreement

When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations

- Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI state can be associated with {delay spread}  and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties

- Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD)

- Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting

- Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation.

- This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

Agreement For PDCCH reliability enhancements, support SFN scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

In this paper, we will present our opinions on this issue.

Discussion

High speed train scenario is one important 5G NR scenario captured in TR38.913. For this scenario, generally in current real network, SFN based transmission has been adopted to avoid frequent handover and improve user experience. However, Doppler shifts from different RRHs along the trajectory of the train is different, even sometimes it will be opposite. One illustration is shown as Fig.1.

Figure 1: HST-SFN Scenario

In order to overcome the above issue, one possible approach is to perform Doppler shift estimation and pre-compensation at gNB to realize the alignment between RRHs for one cell. In RAN1#102e meeting, three steps for TRP-based frequency offset pre-compensation scheme have been listed for consideration. To realize aligned PDSCH transmission for step 3, the reference RRH should be common understanding between gNB and UE. Considering TRS from non-reference RRH could not be as the source RS providing Doppler related parameter, QCL/TCI relation should be rethought.

In RAN1#103e meeting, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, some candidate QCL variants, i.e., Variant A/B/C/E has been listed. For Variant E, both TCI states provides integrated channel large scale parameters per RRH. Naturally, it can be applied for scheme 1 without gNB pre-compensation. But it also can be used for gNB pre-compensation scheme while additional spec work is needed to indicate meaninglessness of Doppler spread/Doppler shift parameters from one TCI state for non-reference RRH. Variant A/B/C aims to be used for pre-gNB compensation scheme. In current spec QCL-TypeB is only used for CSI acquisition. Therefore, we don’t prefer Variant B for within it QCL-TypeB is used for PDSCH. We have not seen the use case for Variant C. Relatively, Variant A is one simple and clean solution. Thus, we have the following the proposal:

Proposal 1: For Scheme 1 without gNB pre-compensation, Variant E could be supported.

Proposal 2: For gNB pre-compensation, at least Variant A could be supported.

In Re-16, SDMed, FDMed, and TDMed PDSCH transmission across multiple TRP are all supported, and DMRS ports from different TRPs are associated with different TCI states. High layer signaling and DMRS configuration are used to differentiate these transmission schemes. For SDMed PDSCH transmissions, DMRS ports are from different CDM groups, while for TDMed and FDMed PDSCH transmissions, DMRS ports are from the same CDM group. High layer signaling is used to differentiate TDM and FDM PDSCH transmissions. For scheme 1, all DMRS ports are associated with both TCI states. One simple scheme is to reuse the high layer signaling to differentiate scheme 1 and Rel-16 TDM/FDM schemes, and DMRS port(s) for scheme1 also could be confined into one CDM group.

Proposal 3: For scheme 1, DMRS port(s) should be from the same DMRS CDM group.

Proposal 4: For scheme 1, reuse Rel-16 high layer signaling to differentiate with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states.

In Rel-16, MAC-CE activates one TCI state for each CORESET. For SFN-based PDCCH enhancement, a straightforward method is to activate up to two TCI states per CORESET by MAC CE.

In Rel-16, the TCI state of CORESET with lowest ID UE monitoring in the latest slot can be used as default QCL assumptions for other channels/signals, e.g. PDSCH. In Rel-17, one CORESET can be configured with more than one TCI states. If the CORESET with lowest ID UE monitoring in the latest slot is configured with two active TCI states, the default QCL assumptions are ambiguous for UE that which TCI state can be used to determine the default QCL assumptions. To avoid ambiguity, a pre-defined rule e.g. the first TCI state or the TCI state with lowest ID can be considered to determine the default QCL assumptions.

Proposal 5: For default QCL assumptions ambiguous issue, support the default QCL assumptions based on a pre-defined rule, e.g. the first TCI state or the TCI state with lowest ID.

Conclusion

In this contribution, we provide our opinions on HST-SFN deployment scenario:

Proposal 1: For Scheme 1 without gNB pre-compensation, Variant E could be supported.

Proposal 2: For gNB pre-compensation, at least Variant A could be supported.

Proposal 3: For scheme 1, DMRS port(s) should be from the same DMRS CDM group.

Proposal 4: For scheme 1, reuse Rel-16 high layer signaling to differentiate with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states.

Proposal 5: For default QCL assumptions ambiguous issue, support the default QCL assumptions based on a pre-defined rule, e.g. the first TCI state or the TCI state with lowest ID.

Reference

- Draft Report of 3GPP TSG RAN WG1 #102e

- Draft Report of 3GPP TSG RAN WG1 #103e