3GPP TSG RAN WG1 #104-e 	                                                    	      R1-2101211

# e-Meeting, January 25th – February 5th, 2021

Agenda Item:		8.5.2

Source:				Samsung

Title:					Discussion on accuracy improvements for UL-AoA positioning solutions

Document for:	Discussion

# Introduction

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

Note: RAN1 will discuss the candidate solutions and provide updates for this objective, with status to be reviewed in RAN#91e.

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

Note: RAN1 will discuss the candidate solutions and provide updates for this objective, with status to be reviewed in RAN#91e.

In the approved WI [1] for positioning enhancement, the angle based method have been recognized as one important direction to improve the positioning accuracy.

In this paper, we will discuss the accuracy improvements for UL-AoA positioning solutions by using differential beamforming.

# Discussion

In a brief summary of differential beamforming based method, the different beam patterns, namely, normal (sum) beam pattern and differential beam pattern, are used by the differential beamforming technique. The normal beam uses the conventional beamforming coefficients while for differential beam pattern, the first half of coefficients are the same with that of conventional one while the other half are the opposite. After receiving the sequences with different patterns, receiver can deduce the best transmit beam direction for transmitter according to the received signal ratio of normal beam pattern and differential beam pattern. The detailed introduction of the differential beam forming can be found in our companion contribution [2].

## Differential beamforming for UL-AOA

In the normal UL-AOA based method, each of the planned TRP (gNB) needs to derive the AOA information based on the measurement of the SRS-pos, which the related configuration is delivered by LMF. The derived UL-AOA method will be reported to LMF, and it will be used to calculate the UE positioning together with the location information of the TRPs. For some of the UL-AOA method (like the beam direction), it is simple but the accuracy is not enough, because the direction of beam is relatively fixed thus is not suitable for more accurate positioning requirement. For some other methods (like MUSIC, ESPRIT etc), the complexity of the solutions will be relatively high; thus a quite amount of effort or time will be consumed.

Observation 1: the normal UL-AOA methods may not provide good enough estimation accuracy as well as latency.

However, by using the differential beamforming techniques as introduced in [2] for the signal reception and as shown in Fig.1, with estimated angle error between actual direction and beam direction, and also the known beam direction information, the angle information of the actual angle of arrival can be derived.

The following aspects can be improved:

- With the continuity of the relation between ratio of received signals of normal beam and differential beam and the angle direction error, the accuracy can be guaranteed as long as the SNR level is above certain level;

- By reception of the signal from same UL Tx beam with both sum beam and differential beam, gNB could

  - Save some resource to schedule the UL SRS-positioning resource;

  - Save some measurement for many UL SRS-positioning purpose from single UE;

  - Achieve shorter latency;

Fig.1 – illustration of differential beamforming based UL-AOA positioning solution.

Observation 2: the differential beamforming technique is beneficial for improving UL-AOA method to better satisfy the positioning requirement.

Proposal 1: Support differential beamforming technique for UL-AOA positioning methods.

## Aspects to be enhanced

In order to support the differential beamforming for UL-AOA positioning methods, the gNB should receive the UL signals (from same UL Tx beam) by using the differential beam as well normal beam, and calculate the ratio for the received signal. Then gNB derives the angle error relative to the beam direction and based on the known beam direction angle, the actual angle from UE could be derived.

SRS-positioning resource configuration

Based on above discussion, it can see that if gNB is only equipped with single antenna panel, which implies only one Rx beam could be used at one time. The gNB (LMF) will need to configure two TDMed SRS-pos resource to facilitate the differential beam reception (but no need many SRS-pos for beam sweeping). If the gNB is equipped with multiple antenna panels, thus FDMed SRS-pos or even in some cases the single SRS-Pos can be used. For example if we consider the distance between gNB antenna panel and UE is larger enough or the distance for gNB antenna panels are small enough, so that one panel received the UL signals with normal bum and the other panel received the same UL signals with differential beam. These two received signals can be used to calculate the ratio and then eventually derive the angle difference as well. But the gNB scheduling should guarantee what is necessarily needed at gNB side for the receptions.

Observation 3: the gNB scheduling could ensure the proper SRS-pos configuration in order to support reception based on differential beam techniques.

Power setting for SRS-pos

In some cases that the SRS-pos will be configured to be received to multiple gNB, among which there could be neighbouring base stations or TRPs relatively far away from UE. If UE still determine the Tx power for the SRS-pos transmission based on it’s configuration or measurement in the serving cell only, the power eventually reaches to the  neighbouring base stations or TRPs might not be enough. Thus the measurement accuracy by these base stations could be degraded and impacts the final positioning accuracy. This issue is not only for reception by differential beam but to general UL-AOA based method which involves the receiving UL signals for multiple cells.

Observation 4: the power setting for the SRS-pos might be not appropriate for neighbouring cell measurement.

Proposal 2: the power control enhancement for SRS-pos to improve UL-AOA based solution should be considered.

# Conclusion

This contribution discusses the accuracy improvements for DL-AoD positioning solutions by using differential beamforming. Observations and proposals are summarized as follows:

Observation 1: the normal UL-AOA methods may not provide good enough estimation accuracy as well as latency.

Observation 2: the differential beamforming technique is beneficial for improving UL-AOA method to better satisfy the positioning requirement.

Observation 3: the gNB scheduling could ensure the proper SRS-pos configuration in order to support reception based on differential beam techniques.

Observation 4: the power setting for the SRS-pos might be not appropriate for neighbouring cell measurement.

Proposal 1: Support differential beamforming technique for UL-AOA positioning methods.

Proposal 2: the power control enhancement for SRS-pos to improve UL-AOA based solution should be considered.

# References

- RP-202900, New WID on NR Positioning Enhancements, RAN#90-e, Dec, 2020.

- R1-2101213, “Discussion on differential beamforming for positioning enhancement”, Samsung, Jan, 2021.