3GPP TSG RAN WG1 #104-e	R1-2101535

e-Meeting, January 25th – February 5th, 2021

Source:	Sharp

Title:	Correction on inconsistence between TS 38.213 and TS 38.331 in terms of the dci-FormatsExt for NR URLLC

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss a remaining issue on inconsistence between TS 38.213 and TS 38.331 in terms of the dci-FormatsExt for NR URLLC.

Discussions

- 2.1 Inconsistence between TS 38.213 and TS 38.331 in terms of the dci-FormatsExt

In Rel-16, DCI format 0_2 and DCI format 1_2 have been introduced for Rel-16 URLLC operation. A dci-FormatsExt is used to indicate whether a UE to monitor the DCI format 0_2 and DCI format 1_2 in the USS. To be more specific, in TS 38.331 [1] as below, the dci-FormatsExt is used to indicate whether a UE to monitor PDCCH candidates for the DCI format 0_2 and DCI format 1_2, or for the DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2 in a USS.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

On the other hand, in TS 38.213 [2] as below, it seems that the dci-FormatsExt can be also used to indicate a UE to monitor PDCCH candidates for the DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, which are not allowed in the TS 38.331.

TS 38.213 V16.4.0 (2020-12) ## 10.1 UE procedure for determining physical downlink control channel assignment - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

Proposal: Adopt the following TP relating to dci-FormatsExt in TS 38.213 to keep insistent with the description of the dci-FormatsExt in TS 38.331.

TP ### TS 38.213 V16.4.0 (2020-12) ## 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1 < Unchanged parts are omitted >

Conclusion

In this contribution, we have discussed one remaining issue on inconsistence between TS 38.213 and TS 38.331 in terms of description of dci-FormatsExt for NR URLLC and have the following proposal:

Proposal: Adopt the following TP relating to dci-FormatsExt in TS 38.213 to keep insistent with the description of the dci-FormatsExt in TS 38.331.

TP ### TS 38.213 V16.4.0 (2020-12) ## 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1 < Unchanged parts are omitted >

References

3GPP TS38.331 V16.3.1 (2021-01).

3GPP TS38.213 V16.4.0 (2020-12).