3GPP TSG RAN WG1 #104e			R1-2101616

e-Meeting, January 25th – February 5th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on timing relationship enhancements for NTN

Agenda Item:	8.4.1

Document for: 	Discussion and Decision

- Introduction

According to the WID [1], the following objectives are listed to be specified in Rel-17 NTN.

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821): - Timing relationship enhancements[RAN1,RAN2] - Enhancements on UL time and frequency synchronization [RAN1,RAN2] - HARQ - Number of HARQ process [RAN1] - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2] In addition, the following topics should be specified if beneficial and needed - Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2]. - Feeder link switch [RAN2,RAN1] - Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2] - Including signalling of polarization mode

Also, at the RAN1#103-e meeting, the following was agreed [2].

Agreement: Introduce K_offset (may or may not be the same as the K_offset value in other timing relationships) to enhance the timing relationship of HARQ-ACK on PUCCH to MsgB. Agreement: - For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported. - FFS: Beam specific K_offset configured in system information and used in initial access. Agreement: Denote by K_mac a scheduling offset other than K_offset: - If downlink and uplink frame timing are aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - If downlink and uplink frame timing are not aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac.

In this document, we discuss on timing relationship enhancements for NTN.

- Discussion on Timing relationship

  - On the signaling of Koffset in initial access

For Koffset configured in system information and used in initial access, at least a cell specific  Koffset configuration, which is used in all beams of a cell, has been agreed [2].

One of the remaining issues is how to signal Koffset for initial access. As agreed in [3], for the transmission timing of RAR grant scheduled PUSCH (i.e. Msg3), the slot allocated for the PUSCH is determined by Koffset in addition to K2 and Δ. To achieve this, we consider that Koffset should be signaled before Msg3 and the following options can be considered as in which system information the Koffset should be sent.

- Option 1: Koffset is signaled in MIB.

- Option 2: Koffset is signaled in SIB1.

- Option 3: Koffset is signalled in SIB following SIB1.

Since MIB has only 1 bit left in the current NR specification, it is necessary to recreate MIB for NTN to realize option 1. We do not see any benefits of signaling Koffset in MIB compared to options 2 and 3. Option 2 is a reasonable option because SIB1 is received before Msg3 transmission and resources are surplus compared to MIB. SIB1 can be scheduled in the same way as PDSCH. Option 3 is also reasonable when SIB1 resources are insufficient. According to TS 38.331 [4], if the concerned SI message was not received in the current modification period, handling of SI message acquisition is left to UE implementation. Therefore, if it is clear that Koffset is included SIB following SIB1, it is natural for the NTN UE to receive it before starting random access.

Proposal 1: Koffset is signaled in SIB1 or in SIB following SIB1.

When considering beam-specific Koffset, it is necessary to clarify the relationship between the satellite beam and the terrestrial cell. As shown in Figure 1, the NW implementation can choose whether (a) mapping one satellite beam to one terrestrial SSB or (b) mapping one satellite beam to one terrestrial cell. In the implementation of (a), if the Koffset is carried via SIB, the definition of the NR cell may change unless SIB notifies all Koffset for each SSB. Moreover, the idea of notifying all Koffset for each SSB in the SIB is not preferred from payload perspective.

At least SIB1 is cell-specific in current NR because if SIB1 is beam-specific, UE has to reread SIB1 every time the UE moves the SSB beam, which becomes a problem in the initial access. It is unclear if there is any merit in the implementation of (a) by changing the definition of NR cell. An example of a possible benefit might be that cell size in NTN is huge and the appropriate Koffset value for each beam is different, but in that case the cell size should be reconsidered.

On the other hand, the operation of beam-specific Koffset in system information can be realized easily by the implementation of (b) because the beam-specific Koffset looks cell-specific system information from the terrestrial cell, and there is no need to change the definition of the NR cell. For these reasons, we recommend the implementation of (b).

Figure 1. Relationship between the satellite beam and the terrestrial cell

Observation 1: Beam-specific Koffset can be achieved by mapping one satellite beam to one terrestrial cell by NW implementation.

Proposal 2: Koffset in initial access is a cell-specific parameter. Beam-specific Koffset is not supported.

  - Relationship of Koffset and K1/K2 in initial access

Regarding K1/K2 in initial access, whether K1/K2 candidate set needs to be extended or not should be discussed with configured Koffset value. Firstly, let us confirm possible delays in NTN cell. The maximum satellite beam size can be up to 3500 km for GEO and 1000 km for LEO, resulting in up to 3.2 ms maximum differential delay for LEO within a satellite beam [6]. As shown in Figure 3, even with one beam mapped to one cell, the maximum RTT value difference between UEs in the same cell is up to 20.6 ms for GEO and 6.4 ms for LEO.

- 20.6 ms is equivalent to 20.6 / 41.2 / 82.4 / 164.8 slots for SCS 15 / 30 / 60 / 120 kHz, respectively.

- 6.4 ms is equivalent to 6.4 / 12.8 / 25.6 / 51.2 slots for SCS 15 / 30 / 60 / 120 kHz, respectively.

Figure 2. Illustration of differential delay within a cell/beam for transparent satellite

Even in this scenario, we believe that all UEs in the cell/beam can establish consistency of the timing relationship while keeping the existing K1/K2 range for initial access by setting the value of Koffset according to the maximum delay of the cell/beam. As shown in Figure 3, UE on the farthest side from a satellite can establish consistency because the timing difference between PDCCH and PUSCH at the UE is the same as the K2 range. At this time, for the near side UE from a satellite, though there are slots that cannot be scheduled from PDCCH reception to the PUSCH transmission timing instructed by K2 due to the large Koffset for this UE, there is no problem from a system perspective because it is possible to schedule other UEs in these slots. Lower latency scheduling for near side UEs would not be needed at least for initial access. Therefore, a value of Koffset derived from the maximum delay of NTN cell/beam should be supported while there is no necessity to enhance K1/K2 candidate set. The same argument applies to K1.

Observation 2: In NTN initial access, all UEs in the cell/beam can establish consistency of the timing relationship while keeping the existing K1/K2 range by setting the value of Koffset according to the maximum delay of the cell/beam.

Figure 3. Timing relationship for PDCCH to the scheduled PUSCH in initial access

Note that setting Koffset as a value other than that based on the maximum delay does not guarantee the consistency of the timing relationship unless K1/K2 candidate set is significantly expanded due to the large cell size. In the current NR specification, K1 is up to 8 slots in DCI format 1_0, and K2 is similar range, which is determined in DCI format 0_0 from Table 6.1.2.1.1-2 in TS 38.214 for PUSCH scheduled by RAR (e.g., 3 to 6 for 15kHz SCS) [8]. Based on the above possible delays and the current K1/K2, both GEO and LEO scenarios will require K1/K2 expansion. If it corresponds to the worst case, K1/K2 needs to be expanded up to 164.82=83 taking into account the scenario of GEO with 120kHz SCS. Such an expansion of K1/K2 candidate set means large DCI overhead or controversial discussions to reduce the overhead. We believe that NW side should set Koffset as an appropriate value, and enhancement on K1/K2 candidate set is undesirable and unnecessary.

Observation 3: In NTN initial access, when Koffset is set to a value other than that based on the maximum delay in the cell/beam, the consistency of the timing relationship is not guaranteed unless K1/K2 range is  significantly expanded due to the large cell size. Large overhead or controversial discussions to reduce the overhead is expected.

Proposal 3: Support the value of Koffset in initial access which corresponds to the largest delay in the cell.

Proposal 4: Keep the existing K1/K2 range in initial access.

  - Updating Koffset or extending K1/K2 after initial access

When Koffset is set according to the maximum cell/beam delay in initial access, there remain issues on e.g. scheduling efficiency, latency performance, and HARQ process management, as 1) Appropriate Koffset might be changed due to high-speed movement of LEO; 2) Koffset is uselessly large for the near side UE from a satellite after initial access. For these issues, it is effective to update Koffset to UE-specific value  via RRC signaling, MAC CE, or PHY after RRC connection setup. Besides, extending K1/K2 range and providing a set of K1/K2 values optimal for each NTN UE are also possible options to solve the above issues. Whether to update Koffset in UE-specific manner and whether to extend K1/K2 should be discussed and the motivation would be the same in our understanding; therefore it is important to consider whether to update Koffset in UE-specific manner and/or extend K1/K2 candidate set together after RRC connection setup.

Observation 4: After RRC connection setup, there remain issues (e.g. scheduling efficiency) as 1)  Appropriate Koffset might be changed due to high-speed movement of LEO; 2) Koffset determined in initial access is uselessly large for the near side UE from a satellite, and updating Koffset in UE-specific manner or extending K1/K2 candidate set are possible options to solve these issues.

As organized in feature lead summary on timing relationship enhancements [5], 5 or more options in total were discussed regarding updating Koffset. One of the factors that divided the discussion is the influence of the high-speed movement of LEO. To make progress in this discussion, we consider the frequency of updating Koffset due to LEO movement. As shown in Figure 4, we assume LEO at an altitude of 600 km with fixed footprint with reference to the parameters assumed by TR 38.821 [6]. When Koffset is signaled in SIB1 or in SIB following SIB1, cell-specific Koffset can be updated within 160 ms since the maximum transmission cycle of SIB1 is 160 ms in the current NR specification [4]. While LEO moves about 1.2 km in 160 ms, the distance from the satellite to the farthest UE changes only about 0.2 m. Furthermore, the distance difference from a satellite between the UE farthest from the satellite and the UE closest to the satellite also changes by only about 0.2 m. Therefore, the SIB cycle is sufficient for the frequency of updating Koffset due to the movement of LEO, and no additional RRC parameters are required from perspective of LEO’s movement.

Observation 5: The SIB cycle is sufficient for the frequency of updating Koffset due to the movement of LEO.

Figure 4. Illustration of LEO (at 600 km altitude) movement with fixed footprint

Since we found that any additional RRC parameters are not required for updating Koffset due to the movement of LEO, we consider the remaining issue of different optimal Koffset for each UE to reduce time offset from PDCCH reception to UL transmission. There are two possible options to solve this issue as follows:

- Option 1: Update Koffset in UE-specific manner.

- Option 2: Extend K1/K2 candidate values.

In case of option 1, Koffset would be updated after initial access via RRC configuration. If an UE is in near side from a satellite and smaller Koffset is available, UE-specific RRC parameter can be provided to the UE.

On the other hand, in case of option 2, the same effect as option 1 can be obtained by allowing negative values as candidates for K1/K2 and setting several values from the candidates in the RRC configuration. As shown in Figure 5, when the cell-specific value of Koffset corresponds to the largest delay in the cell, scheduling efficiency can be improved by allowing negative values to be set as K2 candidate values. The same argument applies to K1.

Figure 5. Improving scheduling efficiency by allowing negative values for K2 candidates

Observation 6: After RRC connection setup, either updating Koffset in UE-specific manner or introducing negative values to K1/K2 candidate set  improves scheduling efficiency.

Proposal 5: After RRC connection setup, RAN1 to support either updating Koffset in UE-specific manner or introducing negative values to K1/K2 candidate set via RRC configuration.

- Conclusion

In this contribution, we discuss timing relationship enhancements for NTN. Based on the discussion we made following observations and proposals.

Observation 1: Beam-specific Koffset can be achieved by mapping one satellite beam to one terrestrial cell by NW implementation.

Observation 2: In NTN initial access, all UEs in the cell/beam can establish consistency of the timing relationship while keeping the existing K1/K2 range by setting the value of Koffset according to the maximum delay of the cell/beam.

Observation 3: In NTN initial access, when Koffset is set to a value other than that based on the maximum delay in the cell/beam, the consistency of the timing relationship is not guaranteed unless K1/K2 range is  significantly expanded due to the large cell size. Large overhead or controversial discussions to reduce the overhead is expected.

Observation 4: After RRC connection setup, there remain issues (e.g. scheduling efficiency) as 1)  Appropriate Koffset might be changed due to high-speed movement of LEO; 2) Koffset determined in initial access is uselessly large for the near side UE from a satellite, and updating Koffset in UE-specific manner or extending K1/K2 candidate set are possible options to solve these issues.

Observation 5: The SIB cycle is sufficient for the frequency of updating Koffset due to the movement of LEO.

Observation 6: After RRC connection setup, either updating Koffset in UE-specific manner or introducing negative values to K1/K2 candidate set  improves scheduling efficiency.

Proposal 1: Koffset is signaled in SIB1 or in SIB following SIB1.

Proposal 2: Koffset in initial access is a cell-specific parameter. Beam-specific Koffset is not supported.

Proposal 3: Support the value of Koffset in initial access which corresponds to the largest delay in the cell.

Proposal 4: Keep the existing K1/K2 range in initial access.

Proposal 5: After RRC connection setup, RAN1 to support either updating Koffset in UE-specific manner or introducing negative values to K1/K2 candidate set via RRC configuration.

References

- RP-201256, “WID: Solutions for NR to support non-terrestrial networks (NTN),” Thales, June 2020.

- 3GPP, RAN1#103e, RAN1 Chairman’s Notes, Nov. 2020.

- 3GPP, RAN1#102e, RAN1 Chairman’s Notes, Sep. 2020.

- 3GPP TS 38.331 V16.3.1, “NR; Radio Resource Control (RRC); Protocol specification,” Jan. 2021.

- R1-2009733, “Feature lead summary on timing relationship enhancements”, Ericsson, Nov. 2020.

- 3GPP TR 38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN),” Jan. 2020.

- 3GPP TS 38.214 V16.4.0, “NR; Physical layer procedures for data,” Jan. 2021.