3GPP TSG RAN WG1 Meeting #104b-e R1-2102429 e-Meeting, April 12th –20th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 15.13.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on interaction between SFI and data repetition. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2021-4-12. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; PDSCH/PUSCH repetition is supported for SPS and configured grant. But the interaction between PDSCH/PUSCH repetition for SPS/configured grant and semi-static slot configuration is missing in current specification.. -. Summary of change:; The same interaction between PDSCH/PUSCH repetition and semi-static slot configuration is shared by dynamically-scheduled PDSCH/PUSCH and SPS/configured-grant transmission, i.e., a slot is excluded from the repetition if it contains at least one symbol direction confliction.. Consequences if not approved:; The specification is not clear on interaction between PDSCH/PUSCH repetition for SPS/configured grant and semi-static slot configuration.. Clauses affected:; 11.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR is aiming to clarify interaction between PDSCH/PUSCH repetition for SPS/configured grant and semi-static uplink-downlink configuration. The proposed clarification is common understanding, and it is expected UEs and networks are implemented with the same understanding and therefore no change is required on the current UE and network implementation.. This CR's revision history:; This is the first version for this CR..

<Unchanged part omitted>

## 11.1	Slot configuration

<Unchanged part omitted>

If a UE is scheduled by a DCI format 1_1 or configured to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot.

If a UE is scheduled by a DCI format 0_1 or configured to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

<Unchanged part omitted>