3GPP TSG RAN WG1 #104b-e		   R1-2102457

e-Meeting, April 12th – April 20th, 2021

Agenda item:	8.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on intra-UE multiplexing/prioritization

Document for:	Discussion and decision

Introduction

During RAN 88e meeting, a revised WID of Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR was approved. One objective is to further enhance the multiplexing and prioritization scheme in URLLC, the objective is shown below.

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

    - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH

This contribution provides some considerations on intra-UE multiplexing/prioritization for URLLC.

Discussion

- Overlapping between high priority HARQ-ACK and low priority HARQ-ACK

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2.

- FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- FFS in case the total number of LP and HP HARQ-ACK bits is 2.

- FFS details

It has been supported to multiplex a HP HARQ-ACK and a LP HARQ-ACK onto a PUCCH resource configured for HP HARQ-ACK. Considering that maximum 16 resources can be configured in each PUCCH-resource-set, and the reliability of scheduling DCI for HP HARQ-ACK is generally high enough to avoid miss detection, we do not see much necessity to configure dedicated PUCCH resources for multiplexing.

In addition, in order to have a unified solution, it is also reasonable to also support to multiplex on the HP PUCCH resources if a 1 bit LP HARQ-ACK overlaps with a 1 bit HP HARQ-ACK.

- Not support to configure dedicate PUCCH resources for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- Support to multiplex on the HP PUCCH resources if a 1 bit LP HARQ-ACK overlaps with a 1 bit HP HARQ-ACK

With respect to coding scheme for the case the total number of LP and HP HARQ-ACK bits being more than 2, separate coding is slightly preferred due to the reason that the total resource can be saved and the HP HARQ-ACK bits can get better protection if LP HARQ-ACK bits are error detected.

- Support separate coding for multiplexing of HP HARQ-ACK and LP HARQ-ACK for total bit number being larger than 2.

- Enable/disable multiplexing of PUCCH

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing.

- FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration

- FFS: Interaction between the enable/disable mechanism and other multiplexing conditions

PUCCH-config （priority）configuration?

- FFS for other types of UCI.

For a mechanism to enable/disable the multiplexing between PUCCHs, compared with the semi-static RRC configuration, dynamically indication in DCI would cost more DCI resources and may not be feasible for SPS transmission. So we support to use semi-static RRC configuration at least, and dynamical indication schemes can be further considered.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, support RRC configuration to enable/disable the multiplexing as a baseline.

- Overlapping of more than two PUCCHs

In case a PUCCH overlaps with more than one PUCCH with different priorities, Rel-16 behaviour can be employed at first, i.e. multiplexing/dropping of overlapping PUCCHs with the low priority assuming PUCCHs with high priority do not exist. After that, a LP PUCCH including any intermediate scheduled LP transmission should be dropped if it overlaps with any HP PUCCH. Thirdly, multiplexing/overriding of HP PUCCHs is performed as if LP channels do not exist. Finally, a final HP PUCCH is multiplexed with a final LP PUCCH if they overlap, after performing multiplexing of HP PUCCHs.

- In case a PUCCH overlaps with more than one PUCCH with different priorities, perform multiplexing/dropping of overlapping PUCCHs with the same priority first, and then deal with multiplexing/dropping of resulted PUCCHs with different priorities in general.

- Overlapping between HARQ-ACK and PUSCH

Agreements:

For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1.

- FFS value(s)

- FFS to additionally support beta-offset =0 or a value disabling the multiplexing

- Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

From our perspective, gNB can set smaller beta_offset values, such as 0.8, 0.5, to decrease the occupied resources of LP HARQ-ACK and maintain the reliability of HP UL-SCH when multiplexing is enabled. In addition, beta-offset=0 can also be supported to disable the multiplexing of LP HARQ-ACK.

- gNB can set the smaller beta_offset values for multiplexing LP HARQ-ACK in a HP PUSCH, such as 0.8, 0.5 and 0.

- Overlapping between high priority SR and low priority HARQ-ACK.

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  Opt.1a: The UE does not transmit negative SR.

  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  FFS: whether with power boost to transmit multiplexed payload or not.

  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

  Opt.3: No enhancement over Rel-16.

  Other options not excluded.

  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, to reuse current spec as much as possible, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. Thus we prefer Opt. 2c here. In addition, to distinguish between HP SR and LP SR on HARQ-ACK with PF0, Opt. 2b can also be considered.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- To distinguish between HP SR and LP SR on HARQ-ACK with PF0, Opt. 2b can also be considered.

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):

  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  Opt.1a: The UE does not transmit negative SR.

  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  FFS: whether with power boost to transmit multiplexed payload or not.

  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  FFS on conditions of multiplexing.

  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

  Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

  Opt.5: No enhancement over Rel-16.

  Other options not excluded.

  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, for a positive SR, extra CS needs to be reserved if multiplexed on SR resources, and the performance of SR would be affected. On the other side, if HARQ-ACK resource is used for multiplexing, the latency of HP SR cannot be maintained. Overall, Opt.4 is preferred.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, for positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

Agreements:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

  Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

  Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource

  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

  Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.

  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

  Opt.4: No enhancement over Rel-16.

  Other options not excluded.

  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Rel-15 rule can be reused, thus we support Opt.2c here. To additional differentiate HP SR and LP SR, Opt.2b can also be considered.

- If a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Conclusion

In this contribution, we made the following observations and proposals.

- Not support to configure dedicate PUCCH resources for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- Support to multiplex on the HP PUCCH resources if a 1 bit LP HARQ-ACK overlaps with a 1 bit HP HARQ-ACK

- Support separate coding for multiplexing of HP HARQ-ACK and LP HARQ-ACK if total bit number is larger than 2.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, support RRC configuration to enable/disable the multiplexing as a baseline.

- In case a PUCCH overlaps with more than one PUCCH with different priorities, perform multiplexing/dropping of overlapping PUCCHs with the same priority first, and then deal with multiplexing/dropping of resulted PUCCHs with different priorities in general.

- gNB can set the smaller beta_offset values for multiplexing LP HARQ-ACK in a HP PUSCH, such as 0.8, 0.5 and 0.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- To distinguish between HP SR and LP SR on HARQ-ACK with PF0, Opt. 2b can also be considered.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, for positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- If a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

References

RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR.

Draft Report of 3GPP TSG RAN WG1 #104e.